*******************************************************************************

Johnson

*******************************************************************************

CUPL(WM)        5.0a Serial# MW-10400000

Device          f1508ispplcc84  Library DLIB-h-40-2

Created         Sun Jun 02 10:38:51 2013

Name            Johnson

Partno          Vitural

Revision        03

Date            May01

Designer        Licensed for use of Joe Designer, at Widgets Inc

Company         c jmg 2013

Assembly        Johnson Counters and Edge Pulse

Location        Web Example

===============================================================================

Expanded Product Terms

===============================================================================

John_D =>

!Qc1 & !Qc2

Qc =>

Qc6 , Qc5 , Qc4 , Qc3 , Qc2 , Qc1 , Qc0

Qc0.d  =>

John_D & SRn

Qc0.ck  =>

CLKi

Qc1.d  =>

Qc0 & SRn

Qc1.ck  =>

CLKi

Qc2.d  =>

Qc1 & SRn

Qc2.ck  =>

CLKi

Qc3.d  =>

Qc2 & SRn

Qc3.ck  =>

CLKi

Qc4.d  =>

Qc3 & SRn

Qc4.ck  =>

CLKi

Qc5.d  =>

Qc4 & SRn

Qc5.ck  =>

CLKi

Qc6.d  =>

Qc5 & SRn

Qc6.ck  =>

CLKi

RAS_ =>

RASe0 & RASe1

# !RASe0 & !RASe1

RASe =>

RASe1 , RASe0

RASe0.d  =>

RASei

RASe0.ck  =>

CLKi

RASe1.d  =>

RASe0

RASe1.ck  =>

CLKi

===============================================================================

Symbol Table

===============================================================================

Pin Variable                                    Pterms   Max     Min

Pol   Name              Ext     Pin     Type     Used   Pterms  Level

--- --------            ---     ---     ----    ------  ------  -----

CLKi                        0        V        -       -       -

John_D                      0        V        1       0       1

OEN                         0        V        -       -       -

Qc                          0        F        -       -       -

Qc0                         0        V        -       -       -

Qc0                 d       0        X        1       0       1

Qc0                 ck      0        X        1       0       1

Qc1                         0        V        -       -       -

Qc1                 d       0        X        1       0       1

Qc1                 ck      0        X        1       0       1

Qc2                         0        V        -       -       -

Qc2                 d       0        X        1       0       1

Qc2                 ck      0        X        1       0       1

Qc3                         0        V        -       -       -

Qc3                 d       0        X        1       0       1

Qc3                 ck      0        X        1       0       1

Qc4                         0        V        -       -       -

Qc4                 d       0        X        1       0       1

Qc4                 ck      0        X        1       0       1

Qc5                         0        V        -       -       -

Qc5                 d       0        X        1       0       1

Qc5                 ck      0        X        1       0       1

Qc6                         0        V        -       -       -

Qc6                 d       0        X        1       0       1

Qc6                 ck      0        X        1       0       1

RAS_                        0        V        2       0       1

RASe                        0        F        -       -       -

RASe0                       0        V        -       -       -

RASe0               d       0        X        1       0       1

RASe0               ck      0        X        1       0       1

RASe1                       0        V        -       -       -

RASe1               d       0        X        1       0       1

RASe1               ck      0        X        1       0       1

RASei                       0        V        -       -       -

SRn                         0        V        -       -       -

LEGEND    D : default variable         F : field      G : group

I : intermediate variable    N : node       M : extended node

U : undefined                V : variable   X : extended variable

T : function