WRD william robert drury Rev06

bob_drury@hotmail.com

Abstract

Spin2 Example Programs

##### HELLO Propeller II EXAMPLES

User Notes

# Foreward

The notes  are a summation of various documents produced either by Parllax or Forum Topic submissions (thanks) while the Propeller II was being developed.

Sections 1.0-11.0 are Hardware product related. Sections 12.0-17.0 are programming related. The Appendix are either details of Propeller function or general support information.

If there are any suggestions for clarification and improvements(or outright mistakes). Please forward this too: bob_drury@hotmail.com Your input would be appreciated.

Propeller II has smart pins allowing any pin to be either digital In,digital Out, analog In or analog Out. This feature sets the propeller apart from most microprocessors.

The smart pins also have built in logic allowing functions to run independent of the processor(s) and to request servicing.

The 8 independent cores with the same clock ,can run separately or co-operatively with “OR” digiatal busing makes a unique hardware configuration.

Propeller firmware has custom “debug” routime which facilitates learning and debugging programs which is another unique feature from other microprocessors.

Propeller assembly language allows self modifying operands , again this unique feature allows for implementing pointers . Normally self modifying code is considered risky in the programming world.

# Smart Pin Block Diagram

Each of the 64 I/O pins in a Propeller-2 microcontroller can operate as a Smart Pin. In brief, every Smart Pin provides access to internal functions such as analog-to-digital converters, digital-to-analog converters, signal generators, PWM controllers, and so on. The Propeller-2 architecture lets these functions operate independent of the cogs so they don't rely on software interactions to "micromanagement" their control and operation. Normally, a DIR bit controls an I/O pin's output enable, while the IN bit returns the pin's state. In Smart Pin modes, though, these bits serve different purposes. The DIR bit controls an active-low (logic-0) reset signal for the selected Smart Pin's circuitry, while a configuration bit controls the pin's output enable state. In some modes, the Smart-Pin circuit directly controls the pin's output state, in which case the OUT bit gets ignored. The IN bit serves as a "finished" flag that indicate to a cog(s) that the Smart Pin has completed some function, or an event has occurred. Depending on the operation, software might need to acknowledge a set IN flag (and reset it?). The block diagram below shows the main functions for a Smart Pin. At first this information might seem complicated, but later explanations of the functions, registers, and instructions clarify their use.

## 1.1) Smart Pin Schematic

Every I/O pin features versatile digital and analog capabilities as well as autonomous state machine functions that would otherwise require processor time to perform. The combination provides adept functionality for application design, increasing the Propeller 2 potential beyond what multi-core architecture alone provides. There are 24 low-level 'pin' modes and 34 high-level 'smart' modes. Pin Modes Each I/O pin has 13 low-level pin mode configuration bits which determine the operation of its 3.3 V circuit.

The pin mode is set using the WRPIN instruction, where the 13 %MMMMMMMMMMMMM bits within the instruction's D operand go directly to these bits. Note though that in some smart pin modes, these bits are partially overwritten to set things like DAC values.

Note: Upon startup or reset, all I/O pins default to input (high impedance), meaning each cog's direction registers are initialized to zero. Each cog's output registers are initialized to zero as well, but this low (ground) state is not reflected on the pin until the pin is set to the output direction (via the direction register).
Pins to avoid for starndard configuration:

SP58: MISO (connection to SPI Flash Data Out pin or Micro SD MISO pin) 
P59: MOSI (connection to SPI Flash Data In pin or Micro SD MOSI pin) 
P60: CLK / CS (connection to SPI Flash CLK pin or Micro SD CS pin) 
P61: CS / CLK (connection to SPI Flash CS pin or Micro SD CLK pin) 
P62: Serial Tx (connection to host's Serial Rx) 
P63: Serial Rx (connection to host's Serial Tx)

## 1.2) Smart Pin WRPIN,WXPIN and WYPIN

Smart Modes Each I/O pin has built-in 'smart pin' circuitry which, when enabled, performs an autonomous function on the pin. Smart pins free the cogs from the need to micromanage many I/O operations by providing high-bandwidth concurrent hardware functions that cogs could otherwise not perform as well through I/O pin manipulating instructions. In normal operation, an I/O pin's output enable is controlled by its DIR bit, its output state is controlled by its OUT bit, and its IN bit returns the pin's read state. With smart pin mode enabled, its DIR bit is used as an active-low reset signal to the smart pin circuitry, while the output enable state is controlled by a configuration bit. In some modes, the smart pin circuit takes over driving the output state, in which case the OUT bit gets ignored. Its IN bit serves as a flag to indicate to the cog(s) that the smart pin has completed some function or an event has occurred, and acknowledgment is perhaps needed. To configure a smart pin, first set its DIR bit to low (holding it in reset) then use WRPIN, WXPIN, and WYPIN to establish the mode and related parameters. Once configured, DIR can be raised high and the smart pin will begin operating. After that, depending on the mode, you may feed it new data via WXPIN/WYPIN or retrieve results using RDPIN/RQPIN. These activities are usually coordinated with the IN signal going high; explained later. Note that while a smart pin is configured, the %TT bits (of the WRPIN instruction's D operand) will govern the pin's output enable, regardless of the DIR state. Smart pins have four 32-bit registers inside of them:

Note:  S/# indicates a literal 9-bit pin number (0..63) or a symbol such as LED_pin, you defined earlier.  D/# indicates a literal 9-bit value or a symbol such as sensor_12A, you defined earlier.  {WC} indicates the operation affects the carry flag.

These four registers are written and read via the following PASM 2-clock instructions, in which S/# is used to select the pin number (0..63) and D/# is the 32-bit data conduit:

WRPIN D/#,S/# - Set smart pin S/# mode to D/#, ack pin

WXPIN D/#,S/# - Set smart pin S/# parameter X to D/#, ack pin

WYPIN D/#,S/# - Set smart pin S/# parameter Y to D/#, ack pin

RDPIN D,S/# {WC} - Get smart pin S/# result Z into D, flag into C, ack pin

RQPIN D,S/# {WC} - Get smart pin S/# result Z into D, flag into C, don't ack pin

AKPIN S/# - Acknowledge pin S/#

The format of the D (pin setup) operand value is:

D = %AAAA_BBBB_FFF_MMMMMMMMMMMMM_TT_SSSSS_0

● A = PINA input selector

● B = PINB input selector

● F = PINA and PINB input logic/filtering (after PINA and PINB input selectors)

● M = pin mode

● T = pin DIR/OUT control (default = %00)

● S = smart mode

Each smart pin has a 34-bit input bus and a 33-bit output bus that connect it to the cogs. To configure and control smart pins, each cog writes data and acknowledgement signals to the smart pin input bus. Each smart pin OR's all incoming 34-bit buses from the collective of cogs in the same way DIR and OUT bits are OR'd before going to the pins. Therefore, if you intend to have multiple cogs execute WRPIN / WXPIN / WYPIN / RDPIN / AKPIN instructions on the same smart pin, you must be sure that they do so at different times, in order to avoid clobbering each other's bus data. Reading a smart pin with RDPIN can cause the same conflict; however, any number of cogs can read a smart pin simultaneously without bus conflict by using RQPIN ('read quiet'), since it does not utilize the smart pin input bus for acknowledgement signalling (like RDPIN does). Each smart pin writes to it's output bus to convey its Z result and a special flag. The RDPIN and RQPIN multiplex and read these buses, so that a pin's Z result is read into D and its special flag can be read into C. C will be either a mode-related flag or the MSB of the Z result. When a mode-related event occurs in a smart pin, it raises its IN signal to alert the cog(s) that new data is ready, new data can be loaded, or some process has finished. A cog can test for this signal via the TESTP instruction and can acknowledge a smart pin by executing a WRPIN, WXPIN, WYPIN, RDPIN, or AKPIN instruction for it. This acknowledgement causes the smart pin to lower its IN signal so that it can be raised again on the next event. After a WRPIN/WXPIN/WYPIN/RDPIN/AKPIN, it takes two clocks for IN to drop, before it can be polled again. A smart pin can be reset at any time, without the need to reconfigure it, by clearing and then setting its DIR bit. To return a pin to normal mode, do a 'WRPIN #0,pin'

Pin Modes corresponding to the 13-bit M field are described by this table (?)

What is DAC Level

## 1.3) SSSSS Smart Pin Mode Setting

## 1.4) Smart Pin Symbol Names

| Smart Pin Symbol Value | Symbol Name | Details |
| --- | --- | --- |
| A Input Polarity | (pick one) |  |
| %0000_0000_000_0000000000000_00_00000_0 | P_TRUE_A (default) | True A input |
| %1000_0000_000_0000000000000_00_00000_0 | P_INVERT_A | Invert A input |
| A Input Selection | (pick one) |  |
| %0000_0000_000_0000000000000_00_00000_0 | P_LOCAL_A (default) | Select local pin for A input |
| %0001_0000_000_0000000000000_00_00000_0 | P_PLUS1_A | Select pin+1 for A input |
| %0010_0000_000_0000000000000_00_00000_0 | P_PLUS2_A | Select pin+2 for A input |
| %0011_0000_000_0000000000000_00_00000_0 | P_PLUS3_A | Select pin+3 for A input |
| %0100_0000_000_0000000000000_00_00000_0 | P_OUTBIT_A | Select OUT bit for A input |
| %0101_0000_000_0000000000000_00_00000_0 | P_MINUS3_A | Select pin-3 for A input |
| %0110_0000_000_0000000000000_00_00000_0 | P_MINUS2_A | Select pin-2 for A input |
| %0111_0000_000_0000000000000_00_00000_0 | P_MINUS1_A | Select pin-1 for A input |
| B Input Polarity | (pick one) |  |
| %0000_0000_000_0000000000000_00_00000_0 | P_TRUE_B (default) | True B input |
| %0000_1000_000_0000000000000_00_00000_0 | P_INVERT_B | Invert B input |
| B Input Selection | (pick one) |  |
| %0000_0000_000_0000000000000_00_00000_0 | P_LOCAL_B (default) | Select local pin for B input |
| %0000_0001_000_0000000000000_00_00000_0 | P_PLUS1_B | Select pin+1 for B input |
| %0000_0010_000_0000000000000_00_00000_0 | P_PLUS2_B | Select pin+2 for B input |
| %0000_0011_000_0000000000000_00_00000_0 | P_PLUS3_B | Select pin+3 for B input |
| %0000_0100_000_0000000000000_00_00000_0 | P_OUTBIT_B | Select OUT bit for B input |
| %0000_0101_000_0000000000000_00_00000_0 | P_MINUS3_B | Select pin-3 for B input |
| %0000_0110_000_0000000000000_00_00000_0 | P_MINUS2_B | Select pin-2 for B input |
| %0000_0111_000_0000000000000_00_00000_0 | P_MINUS1_B | Select pin-1 for B input |
| A, B Input Logic | (pick one) |  |
| %0000_0000_000_0000000000000_00_00000_0 | P_PASS_AB (default) | Select A, B |
| %0000_0000_001_0000000000000_00_00000_0 | P_AND_AB | Select A & B, B |
| %0000_0000_010_0000000000000_00_00000_0 | P_OR_AB | Select A ¦ B, B |
| %0000_0000_011_0000000000000_00_00000_0 | P_XOR_AB | Select A ^ B, B |
| %0000_0000_100_0000000000000_00_00000_0 | P_FILT0_AB | Select FILT0 settings for A, B |
| %0000_0000_101_0000000000000_00_00000_0 | P_FILT1_AB | Select FILT1 settings for A, B |
| %0000_0000_110_0000000000000_00_00000_0 | P_FILT2_AB | Select FILT2 settings for A, B |
| %0000_0000_111_0000000000000_00_00000_0 | P_FILT3_AB | Select FILT3 settings for A, B |
| Low-Level Pin Modes | (pick one) |  |
| Logic/Schmitt/Comparator Input Modes |  |  |
| %0000_0000_000_0000000000000_00_00000_0 | P_LOGIC_A (default) | Logic level A → IN, output OUT |
| %0000_0000_000_0001000000000_00_00000_0 | P_LOGIC_A_FB | Logic level A → IN, output feedback |
| %0000_0000_000_0010000000000_00_00000_0 | P_LOGIC_B_FB | Logic level B → IN, output feedback |
| %0000_0000_000_0011000000000_00_00000_0 | P_SCHMITT_A | Schmitt trigger A → IN, output OUT |
| %0000_0000_000_0100000000000_00_00000_0 | P_SCHMITT_A_FB | Schmitt trigger A → IN, output feedback |
| %0000_0000_000_0101000000000_00_00000_0 | P_SCHMITT_B_FB | Schmitt trigger B → IN, output feedback |
| %0000_0000_000_0110000000000_00_00000_0 | P_COMPARE_AB | A > B → IN, output OUT |
| %0000_0000_000_0111000000000_00_00000_0 | P_COMPARE_AB_FB | A > B → IN, output feedback |
| %xxxx_xxxx_xxx_xxxxSIOHHHLLL_xx_xxxxx_x |  | Sync mode, IN/output polarity, high/low drive |
| ADC Input Modes |  |  |
| %0000_0000_000_1000000000000_00_00000_0 | P_ADC_GIO | ADC GIO → IN, output OUT |
| %0000_0000_000_1000010000000_00_00000_0 | P_ADC_VIO | ADC VIO → IN, output OUT |
| %0000_0000_000_1000100000000_00_00000_0 | P_ADC_FLOAT | ADC FLOAT → IN, output OUT |
| %0000_0000_000_1000110000000_00_00000_0 | P_ADC_1X | ADC 1x → IN, output OUT |
| %0000_0000_000_1001000000000_00_00000_0 | P_ADC_3X | ADC 3.16x → IN, output OUT |
| %0000_0000_000_1001010000000_00_00000_0 | P_ADC_10X | ADC 10x → IN, output OUT |
| %0000_0000_000_1001100000000_00_00000_0 | P_ADC_30X | ADC 31.6x → IN, output OUT |
| %0000_0000_000_1001110000000_00_00000_0 | P_ADC_100X | ADC 100x → IN, output OUT |
| %xxxx_xxxx_xxx_xxxxxxOHHHLLL_xx_xxxxx_x |  | O = output polarity, HHH/LLL = high/low drive |
| DAC Output Modes |  | DIR enables output, OUT enables ADC |
| %0000_0000_000_1010000000000_00_00000_0 | P_DAC_990R_3V | DAC 990Ω, 3.3V peak, ADC 1x → IN |
| %0000_0000_000_1010100000000_00_00000_0 | P_DAC_600R_2V | DAC 600Ω, 2.0V peak, ADC 1x → IN |
| %0000_0000_000_1011000000000_00_00000_0 | P_DAC_124R_3V | DAC 123.75Ω, 3.3V peak, ADC 1x → IN |
| %0000_0000_000_1011100000000_00_00000_0 | P_DAC_75R_2V | DAC 75Ω, 2.0V peak, ADC 1x → IN |
| %xxxx_xxxx_xxx_xxxxxDDDDDDDD_xx_xxxxx_x |  | DDDDDDDD = 8-bit DAC value |
| Level-Comparison Modes |  | DIR enables output (1.5kΩ drive) |
| %0000_0000_000_1100000000000_00_00000_0 | P_LEVEL_A | A > Level → IN, output OUT |
| %0000_0000_000_1101000000000_00_00000_0 | P_LEVEL_A_FBN | A > Level → IN, output negative feedback |
| %0000_0000_000_1110000000000_00_00000_0 | P_LEVEL_B_FBP | B > Level → IN, output positive feedback |
| %0000_0000_000_1111000000000_00_00000_0 | P_LEVEL_B_FBN | B > Level → IN, output negative feedback |
| %xxxx_xxxx_xxx_xxxxSLLLLLLLL_xx_xxxxx_x |  | S = Synchronous, LLLLLLLL = 8-bit Level |
| Low-Level Pin Sub-Modes |  |  |
| Sync Mode | (pick one) | (for Logic/Schmitt/Comparator/Level modes) |
| %xxxx_xxxx_xxx_xxxxSxxxxxxxx_xx_xxxxx_x |  | Sync mode bit |
| %0000_0000_000_0000000000000_00_00000_0 | P_ASYNC_IO (default) | Select asynchronous I/O |
| %0000_0000_000_0000100000000_00_00000_0 | P_SYNC_IO | Select synchronous I/O |
| IN Polarity | (pick one) | (for Logic/Schmitt/Comparator modes) |
| %xxxx_xxxx_xxx_xxxxxIxxxxxxx_xx_xxxxx_x |  | IN polarity bit |
| %0000_0000_000_0000000000000_00_00000_0 | P_TRUE_IN (default) | True IN bit |
| %0000_0000_000_0000010000000_00_00000_0 | P_INVERT_IN | Invert IN bit |
| Output Polarity | (pick one) | (for Logic/Schmitt/Comparator/ADC modes) |
| %xxxx_xxxx_xxx_xxxxxxOxxxxxx_xx_xxxxx_x |  | Output polarity bit |
| %0000_0000_000_0000000000000_00_00000_0 | P_TRUE_OUTPUT (default) | Select true output |
| %0000_0000_000_0000001000000_00_00000_0 | P_INVERT_OUTPUT | Select inverted output |
| Drive-High Strength | (pick one) | (for Logic/Schmitt/Comparator/ADC modes) |
| %xxxx_xxxx_xxx_xxxxxxxHHHxxx_xx_xxxxx_x |  | Drive-high selector bits |
| %0000_0000_000_0000000000000_00_00000_0 | P_HIGH_FAST (default) | Drive high fast (30mA) |
| %0000_0000_000_0000000001000_00_00000_0 | P_HIGH_1K5 | Drive high 1.5kΩ |
| %0000_0000_000_0000000010000_00_00000_0 | P_HIGH_15K | Drive high 15kΩ |
| %0000_0000_000_0000000011000_00_00000_0 | P_HIGH_150K | Drive high 150kΩ |
| %0000_0000_000_0000000100000_00_00000_0 | P_HIGH_1MA | Drive high 1mA |
| %0000_0000_000_0000000101000_00_00000_0 | P_HIGH_100UA | Drive high 100μA |
| %0000_0000_000_0000000110000_00_00000_0 | P_HIGH_10UA | Drive high 10μA |
| %0000_0000_000_0000000111000_00_00000_0 | P_HIGH_FLOAT | Float high |
| Drive-Low Strength | (pick one) | (for Logic/Schmitt/Comparator/ADC modes) |
| %xxxx_xxxx_xxx_xxxxxxxxxxLLL_xx_xxxxx_x |  | Drive-low selector bits |
| %0000_0000_000_0000000000000_00_00000_0 | P_LOW_FAST (default) | Drive low fast (30mA) |
| %0000_0000_000_0000000000001_00_00000_0 | P_LOW_1K5 | Drive low 1.5kΩ |
| %0000_0000_000_0000000000010_00_00000_0 | P_LOW_15K | Drive low 15kΩ |
| %0000_0000_000_0000000000011_00_00000_0 | P_LOW_150K | Drive low 150kΩ |
| %0000_0000_000_0000000000100_00_00000_0 | P_LOW_1MA | Drive low 1mA |
| %0000_0000_000_0000000000101_00_00000_0 | P_LOW_100UA | Drive low 100μA |
| %0000_0000_000_0000000000110_00_00000_0 | P_LOW_10UA | Drive low 10μA |
| %0000_0000_000_0000000000111_00_00000_0 | P_LOW_FLOAT | Float low |
| DIR/OUT Control | (pick one) |  |
| %0000_0000_000_0000000000000_00_00000_0 | P_TT_00 (default) | TT = %00 |
| %0000_0000_000_0000000000000_01_00000_0 | P_TT_01 | TT = %01 |
| %0000_0000_000_0000000000000_10_00000_0 | P_TT_10 | TT = %10 |
| %0000_0000_000_0000000000000_11_00000_0 | P_TT_11 | TT = %11 |
| %0000_0000_000_0000000000000_01_00000_0 | P_OE | Enable output in smart pin mode |
| %0000_0000_000_0000000000000_01_00000_0 | P_CHANNEL | Enable DAC channel in non-smart pin DAC mode |
| %0000_0000_000_0000000000000_10_00000_0 | P_BITDAC | Enable BITDAC for non-smart pin DAC mode |
| Smart Pin Modes | (pick one) |  |
| %0000_0000_000_0000000000000_00_00000_0 | P_NORMAL (default) | Normal mode (not smart pin mode) |
| %0000_0000_000_0000000000000_00_00001_0 | P_REPOSITORY | Long repository (non-DAC mode) |
| %0000_0000_000_0000000000000_00_00001_0 | P_DAC_NOISE | DAC Noise (DAC mode) |
| %0000_0000_000_0000000000000_00_00010_0 | P_DAC_DITHER_RND | DAC 16-bit random dither (DAC mode) |
| %0000_0000_000_0000000000000_00_00011_0 | P_DAC_DITHER_PWM | DAC 16-bit PWM dither (DAC mode) |
| %0000_0000_000_0000000000000_00_00100_0 | P_PULSE | Pulse/cycle output |
| %0000_0000_000_0000000000000_00_00101_0 | P_TRANSITION | Transition output |
| %0000_0000_000_0000000000000_00_00110_0 | P_NCO_FREQ | NCO frequency output |
| %0000_0000_000_0000000000000_00_00111_0 | P_NCO_DUTY | NCO duty output |
| %0000_0000_000_0000000000000_00_01000_0 | P_PWM_TRIANGLE | PWM triangle output |
| %0000_0000_000_0000000000000_00_01001_0 | P_PWM_SAWTOOTH | PWM sawtooth output |
| %0000_0000_000_0000000000000_00_01010_0 | P_PWM_SMPS | PWM switch-mode power supply I/O |
| %0000_0000_000_0000000000000_00_01011_0 | P_QUADRATURE | A-B quadrature encoder input |
| %0000_0000_000_0000000000000_00_01100_0 | P_REG_UP | Inc on A-rise when B-high |
| %0000_0000_000_0000000000000_00_01101_0 | P_REG_UP_DOWN | Inc on A-rise when B-high, dec on A-rise when B-low |
| %0000_0000_000_0000000000000_00_01110_0 | P_COUNT_RISES | Inc on A-rise, optionally dec on B-rise |
| %0000_0000_000_0000000000000_00_01111_0 | P_COUNT_HIGHS | Inc on A-high, optionally dec on B-high |
| %0000_0000_000_0000000000000_00_10000_0 | P_STATE_TICKS | For A-low and A-high states, count ticks |
| %0000_0000_000_0000000000000_00_10001_0 | P_HIGH_TICKS | For A-high states, count ticks |
| %0000_0000_000_0000000000000_00_10010_0 | P_EVENTS_TICKS | For X A-highs/rises/edges, count ticks / Timeout on X ticks of no A-high/rise/edge |
| %0000_0000_000_0000000000000_00_10011_0 | P_PERIODS_TICKS | For X periods of A, count ticks |
| %0000_0000_000_0000000000000_00_10100_0 | P_PERIODS_HIGHS | For X periods of A, count highs |
| %0000_0000_000_0000000000000_00_10101_0 | P_COUNTER_TICKS | For periods of A in X+ ticks, count ticks |
| %0000_0000_000_0000000000000_00_10110_0 | P_COUNTER_HIGHS | For periods of A in X+ ticks, count highs |
| %0000_0000_000_0000000000000_00_10111_0 | P_COUNTER_PERIODS | For periods of A in X+ ticks, count periods |
| %0000_0000_000_0000000000000_00_11000_0 | P_ADC | ADC sample/filter/capture, internally clocked |
| %0000_0000_000_0000000000000_00_11001_0 | P_ADC_EXT | ADC sample/filter/capture, externally clocked |
| %0000_0000_000_0000000000000_00_11010_0 | P_ADC_SCOPE | ADC scope with trigger |
| %0000_0000_000_0000000000000_00_11011_0 | P_USB_PAIR | USB pin pair |
| %0000_0000_000_0000000000000_00_11100_0 | P_SYNC_TX | Synchronous serial transmit |
| %0000_0000_000_0000000000000_00_11101_0 | P_SYNC_RX | Synchronous serial receive |
| %0000_0000_000_0000000000000_00_11110_0 | P_ASYNC_TX | Asynchronous serial transmit |
| %0000_0000_000_0000000000000_00_11111_0 | P_ASYNC_RX | Asynchronous serial receive |

## 1.1_Example_WRD_SmartSerial_Demo

{{1.1_Example_WRD_SmartTerminal_Demo}}

{{ Parallax Serial Terminal}}

HOME     =  1 ,  CRSR_XY  =  2,  CRSR_LF  =  3,  CRSR_RT  =  4,  CRSR_UP  =  5

CRSR_DN  =  6 ,  BELL     =  7,  BKSP     =  8,  TAB      =  9,  LF       = 10

CLR_EOL  = 11 ,  CLR_DN   = 12,  CR       = 13,  CRSR_X   = 14,  CRSR_Y   = 15

CLS      = 16

con { timing }

_clkfreq = 200_000_000  'debug requires 10MHZ or greater

baud = 230_400

obj

com : "1.1_Example_WRD_SmartSerial"

1.1_Example pub main()|byte byteVar, wordVar,byte char

com.start(baud)

waitms(4000)

send := @com.tx

recv := @com.rx

send(CLS)

send(BELL)

send("Smart Terminal Demo",CR,"Enter Character")

repeat

send(BELL)

char := recv()

send(char)

repeat

## 1.1_Example_WRD_SmartSerial

{{1.1_Example_WRD_SmartSerial}}'

' SmartSerial.spin2

' simple smart pin serial object for P2 eval board

' implements a subset of FullDuplexSerial functionality

'

' Written by Eric R. Smith

' Copyright 2020 Total Spectrum Software Inc.

' Distributed under the MIT License (See LICENSE.md)

'

' methods:

' start(baud):

'   start serial on pins 63 and 62 at given baud rate

' startx(rxpin, txpin, flags, baud):

'   start on pins rxpin,txpin and given baud rate; `flags` is ignored

' tx(c)

'   send character `c` (must call start or startx first)

' rxcheck()

'   see if a character is available; returns -1 if no character, otherwise

'   returns the character

' rx()

'   waits for a character to become available, then returns it

'

con

_txmode       = %0000_0000_000_0000000000000_01_11110_0

'async tx mode, output enabled for smart output

_rxmode       = %0000_0000_000_0000000000000_00_11111_0

'async rx mode, input  enabled for smart input

var

long rx_pin, tx_pin

' start with default settings

pub start(baud)

startx(63, 62, 0, baud)

'

' start up on a specific set of pins

' `mode` is for compatibility with FullDuplexSerial and is ignored

'

pub startx(rxpin, txpin, mode, baudrate) | bitperiod, bit_mode

bitperiod := (clkfreq / baudrate)

' save parameters in the object

rx_pin := rxpin

tx_pin := txpin

' calculate smartpin mode for 8 bits per character

bit_mode := 7 + (bitperiod << 16)

' set up the transmit pin

pinf(txpin)

wrpin(txpin, _txmode)

wxpin(txpin, bit_mode)

pinl(txpin)   ' turn smartpin on by making the pin an output

' set up the receive pin

pinf(rxpin)

wrpin(rxpin, _rxmode)

wxpin(rxpin, bit_mode)

pinl(rxpin)  ' turn smartpin on

pub tx(val) | z

wypin(tx_pin, val)

repeat

z := pinr(tx_pin)

while z == 0

' check if byte received (never waits)

' returns -1 if no byte, otherwise byte

pub rxcheck() : rxbyte | rxpin, z

rxbyte := -1

rxpin := rx_pin

z := pinr(rxpin)

if z

rxbyte := rdpin(rxpin)>>24

' receive a byte (waits until one ready)

pub rx() : v

repeat

v := rxcheck()

while v == -1

# Digitial Pin Operation (Smart Pin Off)

The “Drive” Logic block is broken into the following components:

The resistors/current sources are a kind of drive strength. To get a pullup, you need to set the pin to output with a drive strength of 1.5k or 15k or 150k Ohm for HHH and you need to output a High (OUTx=1). Also if the pin is set to output, you still can read the input.
If the pin is in smartmode, the red labels are valid, DIRx and OUTx do no longer control the IO signals.

D = %AAAA_BBBB_FFF_MMMMMMMMMMMMM_TT_SSSSS_0

AAAA                                       = 0000                      “x000 this pin read state”

BBBB                                        = 0000                      “x000 this pin read state”

FFF                                            =  000                       “A,B default filter

MMMMMMMMMMMMM = 000000000000    “Fast”

TT                                              = 00                           “can’t figure out what table says

SSSSS                                        = 00000                    “smart pin off”

By default Propeller II starts with ( 'WRPIN #0,pin') Fast Logic Mode

D = %0000_0000_000_0000000000000_00_00000_0

If you're familiar with the assembly-language input-output instructions for the Propeller-1 microcontroller you will recognize the following six instructions a Propeller-2 program also may use these registers. These registers give you direct access to I/O pins:

DIRA    direction register pins P0..P31, 1= output, 0 = disable output

DIRB    direction register pins P63-P32, 1= output, 0 = disable output

OUTA    output register bits for pins P0..P31

OUTB    output register bits for pins P32..P63

INA     input register bits for pins P0..P31

INB     input register bits for pins P32..P63

Propeller II does not have instructions to above registers as propeller 1  DIRA[0]~~ but the registers can be accessed with:

mov dira,#0
mov reg1,ina
mov outa,#1

Definition of PinField

{#}D = PinField = 11 bits   %LLLLL_PPPPPP     %extrapins_basepins

The above direction and output registers can be affected using special instructions which operate on 1 to 32 bits within each register.

In the following lists, {#}D denotes an 11-bit value,(PinField) with the 6 lower bits pointing to a base pin and the next upper 5 bits expressing an additional number of pins within the same I/O register.

PinField = 11 bits   %LLLLL_PPPPPP     %extrapins_basepins

11 bits for PinField LLLLL 5bits for number of additional pins PPPPPP 6 bits for base pin number

%00011_000101  base pin 5 plus 3 pins P3 P4 P5 P6 total 4 pins

%11111_0010000 base pin 8 plus 31 pins wrapping occurs P8-P31 plus P0-P7 (P0-P31)

PinField ≔ BasePin addpins Add_pins eg. PinField ≔ 0 addpins 7 ‘ P0-P7 assigned

In these instructions, bit 5 of {#}D selects between DIRA/DIRB or OUTA/OUTB.

10-09-08-07-06__05-04-03-02-01-00

16 08 04 02 01 __32  16 08 04 02  01

The ADDPINS operator can be used to set the additional-bits field in {#}D as follows:

DIRH    #8			‘Drive P8 high

DIRH    #10 ADDPINS 7		'Drive P10..P17 high {#}D = 00111_001010 = LLLLL_PPPPPP

Each cog has its own pairs of 32-bit I/O Direction Registers (DIRA & DIRB) and 32-bit I/O Output Registers (OUTA & OUTB) to influence the directions and output states of the Propeller 2’s 64 I/O pins. A cog's desired I/O directions and output states are communicated through the entire cog collective to ultimately become what is applied to the I/O pins.

The result of this I/O pin wiring configuration can easily be described in the following simple rules:
* A pin is an input only if no active cog sets it to an output. 
* A pin outputs low only if all active cogs that set it to output also set it to low. 
* A pin outputs high if any active cog sets it to an output and also sets it high.

The Propeller 2 is a CMOS device, so the I/O pin digital logic threshold is approximately 1/2 Vdd.
With the Propeller 2's I/O pins powered by 3.3 V (via the corresponding Vxxyy pins), the I/O pin digital logic threshold is about 1.65 V. 
An input pin will interpret a voltage below 1.65 V as a digital logic level low, and will interpret a voltage above 1.65 V as a digital logic level high. 
An output pin will produce 0 V for digital low and 3.3 V for digital high.

DIRL    {#}D	Set direction bit(s) to logic 0 (input)

DIRH    {#}D	Set direction bit(s) to logic 1 (output)

DIRC    {#}D	Set direction bit(s) to Carry flag

DIRNC   {#}D	Set direction bit(s) to inverse of Carry flag

DIRZ    {#}D	Set direction bit(s) to Zero flag

DIRNZ   {#}D	Set direction bit(s) to inverse of Zero flag

DIRRND  {#}D	Set direction bit(s) to random state(s)

DIRNOT  {#}D	Invert direction bit(s)

Example: DIRL #20	'Set P20 as an input pin

## 2.1) PASM Pin Digital Commands

### 2.1.1)Pin-Output Instructions

These Instructions change the associated OUT bit(s)

OUTL    {#}D	Set output bit(s) to logic 0

OUTH    {#}D	Set output bit(s) to logic 1

OUTC    {#}D	Set output bit(s) to Carry flag

OUTNC   {#}D	Set output bit(s) to inverse of Carry flag

OUTZ    {#}D	Set output bit(s) to Zero flag

OUTNZ   {#}D	Set output bit(s) to inverse of Zero flag

OUTRND  {#}D	Set output bit(s) to random state(s)

OUTNOT  {#}D	Invert output bit(s)

Example: OUTNOT $20	'Invert the logic state of the P20 output

### 2.1.2)Pin-Float Instructions

These instructions change the asscoiaed DIR bit(s) to logic-0(input float)

FLTL    {#}D	Set output bit(s) to logic 0

FLTH    {#}D	Set output bit(s) to logic 1

FLTC    {#}D	Set output bit(s) to Carry flag

FLTNC   {#}D	Set output bit(s) to inverse of Carry flag

FLTZ    {#}D	Set output bit(s) to Zero flag

FLTNZ   {#}D	Set output bit(s) to inverse of Zero flag

FLTRND  {#}D	Set output bit(s) to random state(s)

FLTNOT  {#}D	Invert output bit(s)

Example:  FLTC #20	'Make P20 input with its output bit set to C.

### 2.1.3) Pin-Drive Instructions

These instructions change the associated DIR bit(s) to logic-1 (output).

DRVL    {#}D	Set output bit(s) to logic-0

DRVH    {#}D	Set output bit(s) to logic-1

DRVC    {#}D	Set output bit(s) to Carry flag value

DRVNC   {#}D	Set output bit(s) to inverse of Carry flag

DRVZ    {#}D	Set output bit(s) to Zero flag

DRVNZ   {#}D	Set output bit(s) to inverse of Zero flag

DRVRND  {#}D	Set output bit(s) to random state(s)

DRVNOT  {#}D	Invert output bit(s)

Example: DRVZ #20		'Make P20 output the Z-flag state.

### 2.1.4)Input-Pin Instructions

Two instructions, TESTP and TESTPN can read the state of a single bit within an INA/INB register and either write that bit to the Carry (C) or Zero (Z) flag, or perform a logic operation on the flag. Again, {#}D (pinfield)represents a pin number.

TESTP    {#}D   WC/WZ		Get a pin's state and write it into the C or Z flag.

TESTP    {#}D   ANDC/ANDZ	Get a pin's state and AND it into the C or Z flag.

TESTP    {#}D   ORC/ORZ	Get a pin's state and OR it into the C or Z flag.

TESTP    {#}D   XORC/XORZ	Get a pin's state and XOR it into the C or Z flag.

TESTPN   {#}D   WC/WZ		Get a pin's NOT-state and write it into the C or Z flag.

TESTPN   {#}D   ANDC/ANDZ	Get a pin's NOT-state and AND it into the C or Z flag.

TESTPN   {#}D   ORC/ORZ	Get a pin's NOT-state and OR it into the C or Z flag.

TESTPN   {#}D   XORC/XORZ	Get a pin's NOT-state and XOR it into the C or Z flag.

Example: TESTP #10   ORZ	'Read P10 and or its state into Z.

### 2.1.5)Input-Output-Bit Timing

When an instruction changes a DIRx or OUTx bit, the processor needs three (3) additional system-clock cycles after the instruction before the pin starts to transition to its new state. The figure below shows the delay for a DRVH instruction:

When an instruction reads the contents of the IN register associated with a pin, the processor receives the state of the pins as they existed three (3) system-clock cycles before the start of the instruction. The figure below shows the timing for a the TESTB INA,#0 operation:

When a program uses a TESTP or TESTPN instruction to read the state of a pin, the processor receives the state of the pins as they existed two (2) system-clock cycles before the start of the instruction. So, the TESTP and TESTPN gather "fresher" INx data than is available via the INx registers. The figure below shows the timing for a TESTP instruction:

### 2.1.1_Example_WRD_PASM_DIRH_OUTH

{2.1.1_Example_WRD_PASM_DIRH_OUTH}

{{DIRH    {#}D    Set direction bit(s) to logic 1 (output)}}

{{OUTH    {#}D    Set output bit(s) to logic 1}}

'P0 used with 1k resitor to LED

CON

clk_freq = 200_000_000    'system freq as a constant used on boot of spin byte code interperter

P0 = 0                    'pin 0 constant P0 directive

PUB main() | x              'spin interperter runs in cog 0 runs first PUB method in program

org                     'org directive creates inline hub PASM code

SimpleOut  dirh #P0     'set direction register DIRA P0-P31 pin P0=1 for output enable

outh #P0     'set output register OUTA P0-P31 oub P0=1 for output high

end                     'ends inline PASM returns to spin interperter in cog0

waitms(2000)              'spin cog wait 2000 ms

org

SimpleOutPinField dirh #P0 addpins 7  'use PinField addpins function

outh #P0 addpins 7

end

repeat                     'keeps program looping else cog ends and output is off

'remember imediate # declaration if not present dirh P0 will load from register 0 of the 512 cog registers

### 2.1.2_Example_Wrd_PASM_DRVH_DRVL.spin2

{2.1.2_Example_WRD_PASM_SimpleDigitalOut}

'P0-P7 used with 1k resitor to LED

CON

clk_freq = 200_000_000    'system freq as a constant used on boot of spin byte code interperter

P0 = 0                    'pin 0 constant P0 directive

PUB main() | x                     'spin interperter runs in cog 0 runs first PUB method in program

org                                      'org directive creates inline hub PASM code

SimpleDRVH  drvh #P0    'set direction register DIRA P0 pin P0=1 and OUTA P0 = 1

end                                      'ends inline PASM returns to spin interperter in cog0

waitms(2000)                      'spin cog wait 2000 ms

org

SimpleDRVHPinField drvh #P0 addpins 7

'set direction DIRA register P0-P7 %11111111 and OUTA P0-P7 %11111111

end

repeat                     'keeps program looping else cog ends and output is off

'remember imediate # declaration if not present dirh P0 will load from register 0 of the 512 cog registers

'DrivePin DRVH command sets DIRA\DIRB output enable 1 and the OUTA\OUTB output to command L/H ‘according to addpins

### 2.1.3_Example_WRD_PASM_Cog_Assembly

{{2.1.3_Exammple_WRD_PASM_Cog_Assembly}}

''Debug must be enabled in propeller tool

{===================================================================================}

CON {Processor Timing}

_clkfreq  = 200_000_000

P0 = 0

PUB main()

COGINIT(COGEXEC_NEW,@blink01,0) 'returns cog number started 1 PTRA set to 0

repeat                                      'processor clock speed

DAT     ORG   0  'Cog1 Blink

blink01 OR             DIRA, #$FF                                         'set the direction of the first 8 pins to Output

OR             OUTA, #P0 + 1                                   'set P0 bit 0 equal 1 for High

GETCT        cog1CountValue                               'the counter value is now in in cog1CountValue

ADDCT1     cog1CountValue,cog1WaitTime   'add counter tick for delay

'the cog1CountValue + cog1WaitTime = CT1 result is placed in the CT1 event register

Loop01  WAITCT1                                                                 'wait for cog1 program counter to reach CT1

ADDCT1       cog1CountValue,cog1WaitTime    'add counter tick for delay

XOR              OUTA, #1                                            'toggle bit 1 exclusive or

AND              cog1MskINA,INA                               'mask of bits not being used

debug(ubin(cog1MskINA))                                    ‘debug interrupt window to display value

MOV              cog1MskINA,#%00000000_00000000_00000000_00000001

JMP             #Loop01

'------------------------------------------

cog1WaitTime       long      150_000_000

cog1CountValue    long      200_000_000

cog1MskINA           long      %00000000_00000000_00000000_00000001

### 2.1.4_Example_WRD_PASM_Cog_Assembly

{{2.1.4_Example_WRD_PASM_Cog_Assembly}}

''Debug must be enabled in propeller tool

{===================================================================================}

CON {Processor Timing}

_clkfreq  = 200_000_000

P0 = 0

P1 = 1

PUB main()

COGINIT(COGEXEC_NEW,@blink00,0) 'returns cog number started 1 PTRA set to 0

COGINIT(COGEXEC_NEW,@blink01,0) 'returns cog number started 1 PTRA set to 0

repeat

DAT     ORG   0  'Cog1 Blink

blink00 OR             DIRA, #$FF                    'Set the direction of the first 8 pins to Output

MOV          OUTA, #%01                'set P0 bit 0 equal 1 for High

GETCT         cog1CountValue         'the counter value is now in in cog1CountValue

ADDCT1       cog1CountValue,cog1WaitTime1   'add counter tick for delay

'the cog1CountValue + cog1WaitTime = CT1 result is placed in the CT1 event register

Loop01  WAITCT1                                        'wait for cog1 program counter to reach CT1

ADDCT1          cog1CountValue,cog1WaitTime1    'add counter tick for delay

XOR                 OUTA, #%01                      'toggle bit 1 exclusive or

JMP                 #Loop01

'------------------------------------------

cog1WaitTime1      long      100_000_000

cog1CountValue     long      200_000_000

'------------------------------------------

DAT     ORG   0  'Cog1 Blink

blink01 OR              DIRA, #$FF          'Set the direction of the first 8 pins to Output

MOV             OUTA, #%10           'set P0 bit 0 equal 1 for High

GETCT           cog2CountValue                'get cog counter value

'the counter value is now in in cog1CountValue

ADDCT1          cog2CountValue,cog2WaitTime1   'add counter tick for delay

'the cog1CountValue + cog1WaitTime = CT1 result is placed in the CT1 event register

Loop02                  WAITCT1                                        'wait for cog1 program counter to reach CT1

ADDCT1          cog2CountValue,cog2WaitTime1    'add counter tick for delay

XOR              OUTA, #%10                      'toggle bit 1 exclusive or

JMP             #Loop02

'------------------------------------------

cog2WaitTime1      long      200_000_000

cog2CountValue     long      200_000_000

## 2.2) SPIN I/O Digital Methods

PinField = 11 bits   %LLLLL_PPPPPP     %extrapins_basepins

11 bits for PinField LLLLL 5bits for number of additional pins PPPPPP 6 bits for base pin number

PinField = %00011_000101  base pin 5 plus 3 pins P3 P4 P5 P6 total 4 pins

%11111_0010000 base pin 8 plus 31 pins wrapping occurs P8-P31 plus P0-P7 (P0-P31)

PinField ≔ BasePin addpins Add_pins eg. PinField ≔ 0 addpins 7 ‘ P0-P7 assigned

addpins is a Propeller Tool directive to create PinField

The following are spin 2 commands:

| Pin Methods | Details |
| --- | --- |
| PINW ¦ PINWRITE(PinField, Data) | Drive PinField pin(s) with Data |
| PINL ¦ PINLOW(PinField) | Drive PinField pin(s) low |
| PINH ¦ PINHIGH(PinField) | Drive PinField pin(s) high |
| PINT ¦ PINTOGGLE(PinField) | Drive and toggle PinField pin(s) |
| PINF ¦ PINFLOAT(PinField) | Float PinField pin(s) |
| PINR ¦ PINREAD(PinField) : PinStates | Read PinField pin(s) |
| PINSTART(PinField, Mode, Xval, Yval) | Start PinField smart pin(s): DIR=0, then WRPIN=Mode, WXPIN=Xval, WYPIN=Yval, then DIR=1 |
| PINCLEAR(PinField) | Clear PinField smart pin(s): DIR=0, then WRPIN=0 |
| WRPIN(PinField, Data) | Write 'mode' register(s) of PinField smart pin(s) with Data |
| WXPIN(PinField, Data) | Write 'X' register(s) of PinField smart pin(s) with Data |
| WYPIN(PinField, Data) | Write 'Y' register(s) of PinField smart pin(s) with Data |
| AKPIN(PinField) | Acknowledge PinField smart pin(s) |
| RDPIN(Pin) : Zval | Read Pin smart pin and acknowledge, Zval[31] = C flag from RDPIN, other bits are RDPIN data |
| RQPIN(Pin) : Zval | Read Pin smart pin without acknowledge, Zval[31] = C flag from RQPIN, other bits are RQPIN data |

### 2.2.1_Example_WRD_SPIN_PINT_PINREAD_PINF_PINCLEAR

{{2.2.1_Example_WRD_SPIN_PINT_PINREAD_PINF_PINCLEAR}}

{{

PINT | PINTOGGLE(PinField)              Drive and toggle PinField pin(s)

PINR | PINREAD(PinField) : PinStates    Read PinField pin(s)

PINCLEAR(PinField)                      Clear PinField smart pin(s): DIR=0, then WRPIN=0

PINF | PINFLOAT(PinField)               Float PinField pin(s)

}}

{LED for P0-P7}

Con

_clkfreq = 200_000_000  'debug requires 10MHZ or greater

Pub toggle()|Pin,Status             'are local variables to method toggle

Pin := 0

repeat 20                         'code snipet will toggle 20 times

pintoggle(Pin)

Status := pinread(Pin)

debug(ubin(Status))            'debug command allows Propeller tool to display binary value

waitms(500)

pinclear(Pin)     'this clears DIR = 0 and WRPIN =0 No smart Mode

pinfloat(Pin)     'removes any pull dn or pull up resistor settings

### 2.2.2_Example_WRD_SPIN_PINW_PINL_PINH

{{2.2.2_Example_WRD_SPIN_PINW_PINL_PINH}}

{{

PINW | PINWRITE(PinField, Data)         Drive PinField pin(s) with Data

PINL | PINLOW(PinField)                         Drive PinField pin(s) low

PINH | PINHIGH(PinField)                        Drive PinField pin(s) high

PINR | PINREAD(PinField) : PinStates    Read PinField pin(s)

PINCLEAR(PinField)                                  Clear PinField smart pin(s): DIR=0, then WRPIN=0

PINF | PINFLOAT(PinField)                      Float PinField pin(s)

LED for P0-P7 1k resistor

}}

Con

_clkfreq = 200_000_000

Var

Long PinField,P[8]

Long Data

Pub toggle()|x,y

PinField := 0 addpins 7

debug("Sequence 1")

waitms(200)

repeat x from 0 to 7

P[x] := x

repeat x from 0 to 7

debug(udec(P[x]))

waitms(3000)

debug("Sequence 2")

repeat x from 0 to 10

pinwrite(PinField,%1010_1010)

repeat y from 0 to 7

P[y] := pinread(y)

debug(udec(P[0]),udec(P[1]),udec(P[2]),udec(P[3]),udec(P[4]),udec(P[5]),udec(P[6]),udec(P[7]),udec(x))

waitms(500)

pinwrite(PinField,%0101_0101)

repeat y from 0 to 7

P[y] := pinread(y)

debug(udec(P[0]),udec(P[1]),udec(P[2]),udec(P[3]),udec(P[4]),udec(P[5]),udec(P[6]),udec(P[7]),udec(x))

waitms(500)

waitms(3000)

debug("Sequence 3")

pinlow(0)

pinhigh(1)

pinlow(2)

pinhigh(3)

pinlow(4)

pinhigh(5)

pinlow(6)

pinhigh(7)

Data := pinread(pinfield)

debug(ubin(Data))

waitms(3000)

repeat x from 0 to 7

pinclear(P[x])              'this clears DIR = 0 and WRPIN =0 No smart Mode

pinfloat(P[x])              'removes any pull dn or pull up resistor settings

debug("End of Sequence")

repeat                         'keep cog0 active

# PWM Pulse Width Modulation with Smart Pin

The mechanism typically used to control the brightness of an LED is called PWM (Pulse Width Modulation). In our blink example the LED was either always on or always off. If we want an intermediate brightness, we need to have it partially on; this is the purpose of PWM.

In this figure the on-time portion of the waveform is 30% of the entire cycle (on-time plus off-time). The ratio of on-time to cycle-time is called the duty cycle.

pinstart(led, m, x, y)

m := P_PWM_SAWTOOTH | P_OE

| %0000_0000_000_0000000000000_00_01001_0 | P_PWM_SAWTOOTH | PWM sawtooth output |
| --- | --- | --- |
| %0000_0000_000_0000000000000_01_00000_0 | P_OE | Enable output in smart pin mode |

The first step is to select the PWM mode (sawtooth is the easiest to implement) and to make the smart pin an output with the P_OE constant. The output enable flag is required because in smart pin mode, the pin direction bit is used to enable or disable the smart pin.

If  the Common Cathode connection is used the output being high will turn on the LED if the Common Anode connection is used the output being high will turn off the LED.

The output can be inverted by setting the P_INVERT_OUTPUT bit in the mode register:

m |= P_INVERT_OUTPUT

| %0000_0000_000_0000001000000_00_00000_0 | P_INVERT_OUTPUT | Select inverted output |
| --- | --- | --- |

x.word[1] := 255

The high word of the smart pin X register holds the value that will set the output to 100% duty cycle. As discussed, we will use 255.

x.word[0] := 1 #> ((clkfreq / hz) / 255) <# $FFFF

Finally, the low word of the smart pin X register holds the number of system ticks in one unit for the desired PWM frequency. This takes a little bit of math, but, again, is fairly straightforward.

It works out like this: the system clock frequency (clkfreq) is divided by the desired PWM frequency (hz); this gives us the number of system ticks in one PWM period. That is divided by the number of units in 100% (255) to get the number of system ticks in one unit. The #> and <# operators constrain the value to a legal 16-bit number for the low word of X.

Y register which holds the current level; in our setup this will be 0 (0%) to 255 (100%). To change the LED brightness at any time we can write to the smart pin Y register like this:

wypin(LED, 128)   this sets 50% duty cycle

## 3.1_Example_ WRD_PWM_Demo.spin2

# Analog Out Smart Pin(DAC)

## 4.1) DAC  Digital to Analog Conversion

Figure 4.1.0

The above Schematic is the standard Method for Dac P2 Uses a Voltage Divider approach:

Figure 4.1.1

The above schematic is how the propeller II implements a Dac using a voltage divider

The main difference between a standard voltage divider and the  Prop2 the Pro2 uses a more robust and faster acting arrangement that doesn't have the buffering op-amp. Therefore the output is pure resistive, hence why the output resistance is specied. The side effect of better linearity may be a result.

Note: The first Figure 4.1.0 is called a "binary weighted resistor" DAC. Linearity of that type becomes a major issue at higher word sizes. Figure 4.1.1The "R-2R ladder" DAC is effectively the same but greatly improves on the linearity. Prop2 uses what's called a "Thermometer coded" or "Unary coded" DAC. It's pretty heavy on real-estate but does deliver high precision at speed.

Propeller smart pins each have 2 Dac’s , one DAC has 255 resistors of 252K and the other DAC has 255 resistors of 31.6k. The resistors are pulled high or low dependant on the magnitude  of the value essentially an 8 bit dac is created. If all resistors are pulled high the Voltage is 100% or value 255 (3.3v) if the value is 128 ($80) the voltage is 50% half resistors high half low. If 0 is used all resistors pulled low. 255 resistors in parallel would be 252K/255 Plus the driver circuit impedance aprox 990 ohm. For video on DAC see:  https://www.youtube.com/c/ParallaxInc/playlists The essence of this dac is a voltage divider network that is set by a clocked  Flip Flops.

1) Variable “pin” may be a  PinField need to make sure only 1 pin is configured.

PinField = lllll_pppppp    lllll = 5 bits for addpins     pppppp = 6 bits for P0-P63 0-63

let pin = lllll_pppppp

0000_0000_0000_0000_0000_0lll_llpp_pppp   =       pin

0000_0000_0000_0000_0000_0000_ 0011_1111 =  $3F

pin &= $3F   ‘include this instruction to clear llll upper addpins

0000_0000_0000_0000_0000_0000_00pp_pppp =   pin

2) Disable analog smart pin if previously configured

pinclear(Pin)                                                ' disable smart pin

3) Following Spin method sets Pin for Digital output

pinstart(pin, P_DAC_DITHER_PWM | P_DAC_990R_3V | P_OE, 256, 0)' 16-bit dac

-- https://docs.google.com/document/d/16qVkmA6Co5fUNKJHF6pBfGfDupuRwDtf-wyieh_fbqw/edit#heading=h.1h0sz9w9bl25

Built-In Symbols for Smart Pin Configuration; use  Ctrl+F for searching strings

| %0000_0000_000_0000000000000_00_00011_0 | P_DAC_DITHER_PWM | DAC 16-bit PWM dither (DAC mode) |
| --- | --- | --- |

| %0000_0000_000_1010000000000_00_00000_0 | P_DAC_990R_3V | DAC 990Ω, 3.3V peak, ADC 1x → IN |
| --- | --- | --- |

| %0000_0000_000_0000000000000_01_00000_0 | P_OE | Enable output in smart pin mode |
| --- | --- | --- |

## 4.1_Example_WRD_Analog_Out_Demo

PINSTART(PinField, Mode, Xval, Yval)

Start PinField smart pin(s): DIR=0, then WRPIN=Mode, WXPIN=Xval, WYPIN=Yval, then DIR=1

P_DAC_DITHER_PWM = %0000_0000_000_0000000000000_00_00011_0 DAC 16-bit PWM dither (DAC)

P_DAC_990R_3V          = %0000_0000_000_1010000000000_00_00000_0 DAC 990Ω, 3.3V peak

POE                                 = %0000_0000_000_0000000000000_01_00000_0 Enable    smart pin mode

MODE                             = %0000_0000_000_1010000000000_01_00011_0 above values or'ed

PINFIELD (Pin)    = 25 from calling 4.1_Example_WRD_Analog_Output_Demo

MODE                  = %0000_0000_000_1010000000000_01_00011_0 above values or'ed

WXPIN                 = 256 max unsigned 16 bit $FFFF

WYPIN                 = 0   min unsigned

Pinstart(pin,MODE,WXPIN,WYPIN)

pinstart(pin, P_DAC_DITHER_PWM | P_DAC_990R_3V | P_OE, 256, 0)         ' 16-bit dac selection

The pinstart method writes to the 3 registers WRPIN,WXPIN,WYPIN (below are the PASM instructions that can be used to write directly to these registers:

D = %AAAA_BBBB_FFF_MMMMMMMMMMMMM_TT_SSSSS_0

● A = PINA input selector

● B = PINB input selector

● F = PINA and PINB input logic/filtering (after PINA and PINB input selectors)

● M = pin mode

● T = pin DIR/OUT control (default = %00)

● S = smart mode

WRPIN D/#,S/# - Set smart pin S/# mode to D/#, ack pin

WXPIN D/#,S/# - Set smart pin S/# parameter X to D/#, ack pin

WYPIN D/#,S/# - Set smart pin S/# parameter Y to D/#, ack pin

{{4.1_Example_WRD_Analog_Out_Demo}}

CON { timing }

CLK_FREQ = 200_000_000                                        ' system freq as a constant

MS_001   = CLK_FREQ / 1_000                                   ' ticks in 1ms

US_001   = CLK_FREQ / 1_000_000                               ' ticks in 1us

_clkfreq = CLK_FREQ                                           ' set system clock

CON

analogOutPinP25 = 25  { 0 } 'P25 20k Load Resistor

analogOutP25_LO =   0    'Lo range analog out                  ' scaled range for pot

analogOutP25_HI = 3300   'Hi range analog out                  '16 bit dac ou 0-256

VAR

long  analogOutValueP25

OBJ

analogOut : "4.1_Example_WRD_analog_output"

PUB main() |t , ReturnHexValueP25

analogOut.start(analogOutPinP25,analogOutP25_LO, analogOutP25_HI)  'call analog.start(Pin,lo,Hi)

repeat

t :=getct()

repeat analogOutValueP25 from 0 to 3300

ReturnHexValueP25 := analogOut.write(analogOutValueP25)     ' round to whole, 0..100

debug(udec(analogOutValueP25))

waitct(t += (5*MS_001))

## 4.1_Example_WRD_Analog_Output

{{

{4.1_Example_WRD_analog_output

''   File....... jm_analog_out.spin2

''   Purpose.... Simple P2 analog output using smart pin

''   Author..... Jon "JonnyMac" McPhalen Copyright (c) 2020-2021 Jon McPhalen MIT Licenc

PUB start(Pin,lo,hi) pin for analog out lo range 0 and high range 3300 for 3.3v

PINSTART(PinField, Mode, Xval, Yval)

Start PinField smart pin(s): DIR=0, then WRPIN=Mode, WXPIN=Xval, WYPIN=Yval, then DIR=1

WXPIN                = 256 max unsigned 16 bit $FFFF

WYPIN                 = 0   min unsigned

PINFIELD (Pin)    = 25 from calling 4.1_Example_WRD_Analog_Output_Demo

P_DAC_DITHER_PWM  = %0000_0000_000_0000000000000_00_00011_0 DAC 16-bit PWM dither

P_DAC_990R_3V           = %0000_0000_000_1010000000000_00_00000_0 DAC 990Ω, 3.3V peak, ADC

POE                                  = %0000_0000_000_0000000000000_01_00000_0 Enable output smart mode

MODE                             = %0000_0000_000_1010000000000_01_00011_0 above values or'ed

}}

con { fixed io pins }

RX1    = 63  { I }                                            ' programming / debug

TX1    = 62  { O }

var

long  ap                                                      ' analog pin

long  urlo                                                    ' user output range, low

long  urhi                                                    ' user output range, high

long  aout                                                    ' analog out value (in user range)

long  setup

long codetest                                                  ' true when pin setup

pub null()

'' This is not a top-level object

pub start(pin, lo, hi)

'' Setup pin for analog output

'' -- lo and hi define user range (e.g. 0 and 3300 for millivolts)

stop()

pin &= $3F                                                    ' limit to one pin

pinstart(pin, P_DAC_DITHER_PWM | P_DAC_990R_3V | P_OE, 256, 0)' 16-bit dac

codetest :=   P_DAC_DITHER_PWM | P_DAC_990R_3V | P_OE         'get Mode value

debug(uhex(codetest),ubin(codetest)) 'test smart pin mode value

waitms(10000)

longmove(@ap, @pin, 3)                                        ' save setup

setup := true

pub stop()

'' Disable analog smart pin if previously configured

if (setup)

pinclear(ap)                                                ' disable smart pin

longfill(@ap, 0, 5)                                         ' mark disabled

pub write(value) : result

'' Convert value to 16-bit level and write to DAC

'' -- value is in user-defined lo..hi range

aout := urlo #> value <# urhi                                 ' constrain to user range

result := (aout - urlo) * $FFFF / (urhi - urlo) + urlo        ' convert to 16-bit dac value

wypin(ap, result)

pub level() : result

'' Return last user level written to analog output

return aout

# Analog Input Smart Pin (ADC)

## 5.1) ADC Analog Digital Conversion

Comparator Circuit

Figure 5.1.0

An analogue comparator such as the LM339N which has two analogue inputs, one positive and one negative, and which can be used to compare the magnitudes of two different voltage levels. A voltage input, (VIN) signal is applied to one input of the comparator, while a reference voltage, (VREF) to the other. A comparison of the two voltage levels at the comparator’s input is made to determine the comparators digital logic output state, either a “1” or a “0”.

The reference voltage, VREF is compared against the input voltage, VIN applied to the other input. For an LM339 comparator, if the input voltage is less than the reference voltage, (VIN < VREF) the output is “OFF”, and if it is greater than the reference voltage, (VIN > VREF) the output will be “ON”. Thus a comparator compares two voltage levels and determines which one of the two is higher.

2-bit ADC Using Diodes

Figure 5.1.1

Prop2's ADCs are what's called a "sigma-delta modulator" or "delta-sigma modulator". Just a single fast comparator with feedback, which oscillates in the MHz region producing a natural bit-stream known as pulse-density-modulation (PDM). This is subsequently vacuumed up by a counter configured as a "Sinc filter/decimator" from which you then get your typical PCM samples.

## 5.2_Example_WRD_Analog_Input_Demo

D = %AAAA_BBBB_FFF_MMMMMMMMMMMMM_TT_SSSSS_0

● A = PINA input selector

● B = PINB input selector

● F = PINA and PINB input logic/filtering (after PINA and PINB input selectors)

● M = pin mode

● T = pin DIR/OUT control (default = %00)

● S = smart mode

WRPIN D/#,S/# - Set smart pin S/# mode to D/#, ack pin

WXPIN D/#,S/# - Set smart pin S/# parameter X to D/#, ack pin

WYPIN D/#,S/# - Set smart pin S/# parameter Y to D/#, ack pin

## 5.2_Example_WRD_Analog_Input

PINCLEAR(PINFIELD) Clear PinField smart pin(s): DIR=0, then WRPIN=0

PINFLOAT(PinField) Float PinField pin(s)

LONGMOVE(Dest, Source, Count) Move Count longs from Source to Dest

LONGFILL(Dest, Value, Count) Fill Count longs at Dest with Value

RDPIN(Pin)  :Zval  Read Pin smart pin and acknowledge, Zval[31] = C flag from RDPIN, other bits are RDPIN Data

# AnalogIn and AnalogOut Demo

Using P24 as an analog Input fed from P25 as an analog output a 25 K load resistor is tied to Pins and ground. Two objects are included “4.1_Example_WRD_analog_output.spin2” and “5.1_Example_WRD_analog_input.spin2”. These programs were previously used in section 4.0 and 5.0.

## 6.1_Example_WRD_AnalogIn_AnalogOut_Demo

{6.1_Example_WRD_AnalogIn_AnalogOut_Demo}

CON

clk_freq = 200_000_000

_clkfreq = clk_freq              'system freq as a constant

mS_001   = clk_freq / 1_000      'ticks in 1ms

uS_001   = clk_freq / 1_000_000

analogInP24 =      24           'P24 analog pin input

analogInP24_LO =    0           'Lo range analog out

analogInP24_HI = 3300           'Hi range analog out

analogOutPinP25 =   25          'P25 20k Load Resistor

analogOutP25_LO =    0          'Lo range analog out

analogOutP25_HI = 3300          'Hi range analog out

VAR

long  analogInValueP24

long  analogOutValueP25

OBJ

analogOut : "4.1_Example_WRD_analog_output"

analogIn  : "5.1_Example_WRD_analog_input"

PUB main() |t , ReturnHexValueP25

analogOut.start(analogOutPinP25,analogOutP25_LO, analogOutP25_HI)

analogIn.start(analogInP24, analogInP24_LO, analogInP24_HI)

repeat

t :=getct()

repeat analogOutValueP25 from 0 to 3300

ReturnHexValueP25 := analogOut.write(analogOutValueP25)

analogInValueP24 := (analogIn.read())

debug(udec(analogOutValueP25),udec(analogInValueP24))

waitct(t += (10*mS_001))

# 7.0) P2 Eval PB/LED Control Add-on Board (64006-ES)

Led_P16 Led_P17 Led_P18 Led_P19           Pb_P20 Pb_21 Pb_22 Pb_23 Pb-24

wrpin (20 , P_LOW_15K)              'select P20 pull-down enable sets pin as a low

result0:=pinread(20)                     'read status of P20 button press

pinhigh(17)                                     'set P17 led High = 1 (out high enable on)

“7.1_Example_WRD_Control_Board_01” and “7.2_Example_WRD_Control_Board_02” illustrates how to press input button to turn on the corresponding LED using two different methods.

## 7.1_Example_WRD_Control_Board_01

## 7.2_Example_WRD_Control_Board_02

# 8.0) P2 Eval LED Matrix Add-on Board (#64006C)

## 8.1_Example_WRD_P2_LED_Matrix_Digits

# 9.0) P2 Eval Serial Device Add-on Board (SKU 64006F)

Two user controlled activity LEDs (red and blue) are located beside each microUSB-type socket. Copyright © Parallax Inc. P2 Eval Add-on Boards (#64006 Series)

Function

0 Blue LED with 1 kΩ series resistor. Assert high to light.

1 Red LED with 1 kΩ series resistor. Assert high to light.

2 Serial channel 1 : Data D-

3 Serial channel 1 : Data D+

4 Serial channel 2 : Data D-

5 Serial channel 2 : Data D+

6 Blue LED with 1 kΩ series resistor. Assert high to light.

7 Red LED with 1 kΩ series resistor. Assert high to light

# 10.0) P2 Eval Serial Host Add-on Board (SKU 64006B)

## 10.1) USB host overview

The Universal Serial Bus, or USB, is an external port that interfaces between external devices and a computer.

When a port is in USB host mode, it  powers the bus, and enumerates connected USB devices.

- Host:   The host is the computer or item that acts as the main element or controller for the USB system. The host has a hub contained within it and this is called the Root Hub.

- Hub:   The hub is a device that effectively expands the number of ports available - it will have one connection to the upstream connection, and several downstream. It is possible to plug one hub into another to expand the capability and connectivity further.

- Port:   This is the socket through which access to the USB network is gained. It can be on a host, or a hub.

- Function:   These are the peripherals or items to which the USB link is connected. Mice, keyboards, Flash memories, etc, etc.

- Device:   This term is collectively used for hubs and functions.

## 10.2) Selecting USB function destination

With data for all devices beings sent along the bus, it is necessary for the USB operation that the data is only accepted by the required function.

To achieve this, when a device is attached to the bus it is assigned a unique number or address by the host for the time it is connected.

In addition to the address, the device also contains endpoints. These are the actual sources and destinations for communications between the host and the device.

Endpoints can only operate in one direction, i.e. input or output, but not both, and devices can have up to 16, of which one each for the input and output must be reserved as the 'Zero Endpoint' for that direction. Although each device can have sixteen input and sixteen output endpoints, it is very rare for them all to be used.

The zero endpoints are used for a variety of activities including auto-detection and configuration of the device on the bus and the two zero endpoints are the only ones accessible until the device is properly connected on the bus.

## 10.3) USB data pipes

The communication within USB is based around the concept of using data pipes. These can be considered as being logical channels within the data flow on the bus.

In reality, a USB data pipe is a connection from the host controller to a logical entity within a device, i.e. the endpoint. Because pipes correspond to endpoints, the terms are sometimes used interchangeably.

The host then uses the concept of a data pipe to ensure the data to and from a device is correctly directed or the source is known. The data pipe uses a combination of the address, endpoint and also the direction to define it.

To communicate with the zero endpoints a special form of data pipe is needed because it needs to be used to establish the initial communication. It is called the Default Control Pipe and it can be used when the initial physical connection is made.

There are two types of USB pipe:

- Message Pipe :   This type is a bi-directional USB pipe and it is used for control data. Message pipes are typically used for short, simple commands to the device, and for status responses from the device. They can be used by the bus control pipe number 0.

- Stream Pipe:   This form of USB pipe is uni-directional and it is connected to a uni-directional endpoint that transfers data using an isochronous, interrupt, or bulk transfers (see below).

- h

## 10.4) USB signalling and data transfer basics

For USB 1 and 2 a four wire system is employed. As detailed elsewhere, the cables carry: power, ground and then there is a twisted pair for the differential data transfer.

The lines are designated Data+, D+ and Data-, D- for USB 1 and USB 2. For USB 3, new lines were introduced. For each port there are TX1+ & TX1- and TX2+ & TX2- to cover the transmitted data and then for the received data the lines are RX1+ & RX1- and RX2+ & RX2-.

The use of twisted pairs and differential signaling reduces the effects of external interference that may picked up. It also reduces the effect of any hum loops, etc that could cause issues. As it is not related to ground, but the difference between the two lines, the effects of hum are significantly reduced

The data uses an NRZI system, i.e. non-return to zero.In terms of operation, when the USB host powers up, it polls each of the slave devices in turn.

The USB host has address 0, and then assigns addresses to each device as well as discovering the slave device capabilities in a process called enumeration. [Enumeration takes place when a new device is connected].

Transactions between the host and device comprise a number of packets. As there are several different types of data that can be sent, a token indicating the type is required, and sometimes an acknowledgement is also returned.

Each packet that is sent is preceded by a sync field and followed by an end of packet marker. This defines the start and end of the packet and also enables the receiving node to synchronize properly so that the various date elements fall into place.

There are four basic types of data transaction that can be made within USB.

Control:   This type of data transaction within the overall USB protocol is used by the host to send commands or query parameters. The packet lengths are defined within the protocol as 8 bytes for Low speed, 8-64 bytes for Full, and 64 bytes for High Speed devices.

Interrupt:   The USB protocol defines an interrupt message. This is often used by devices sending small amounts of data, e.g. mice or keyboards. It is a polled message from the host which has to request specific data of the remote device

Bulk:   This USB protocol message is used by devices like printers for which much larger amounts of data are required. In this form of data transfer, variable length blocks of data are sent or requested by the Host. The maximum length is 64-byte for full speed Devices or 512 bytes for high speed ones. The data integrity is verified using cyclic redundancy checking, CRC and an acknowledgement is sent. This USB data transfer mechanism is not used by time critical peripherals because it utilises bandwidth not used by the other mechanisms.

Isochronous:   This form of data transfer is used to stream real time data and is used for applications like live audio channels, etc. It does not use and data checking, as there is not time to resend any data packets with errors - lost data can be accommodated better than the delays incurred by resending data. Packet sizes can be up to 1024 bytes.

The data transfer methodology and protocol for USB provides an effective method of transferring the data across the interface in an effective and reliable manner.

## 10.5) USB data packets

Within the USB system, there are four different types of data packets each used for different types of data transfer.

Token Packets:   Essentially a Token USB data packet indicates the type of transaction is to follow.

Data Packets:   The USB data packets carry the payload data, carrying the data as required.

Handshake Packets:   The handshake packets are used acknowledging data packets received or for reporting errors, etc.

Start of Frame Packets:   The Start of Frame packets used to indicate the start of a new frame of data.

Although USB has developed from USB 1 through USB 2 to USB 3 and now USB 4, it still utilizes the same basic approach to data transfer. There are many USB connectors and leads available, and these leads now have many more wires for higher rate data transfer. Accordingly the data transfer speeds have increased many fold over the first USB specification that was released and the devices that were available.

## 10.6) USB 3 capabilities

The USB 3.0, Superspeed and 3.1, Superspeed+ specifications enable much higher rates of data transfer. This is in keeping with requirements for downloading video and many other applications.

# 11.0) P2 Eval A/V (Audio/Video) Breakout Add-on Board (#64006H)

### 11.1)  VGA Object (ansi_vgatext_demo.spin2)

Using the library file from the “Propeller Tool”( ansi_vgatext_demo.spin2)  for VGA output (No resistor wire Pins directly to VGA Test Plug)

Set clock to 200_000_000 scren size to 600x800 set “CELL_SIZE = 4” call demo.spin2.

VGA Connector (No dropping resistors uses DAC to generate signal)

Pin- Function- VGA Pin -  Wirecolour (VGA Test Plug)

P48       H              13              Torquise-Yel

P49       B                3               RED-yel

P50       G                2               BROWN-yel

P51       R                1                BLACK-yel

P52       V               14              BLUE/White-yel

GND                5,6,7,8,10        purple blue green yellow grey –blk

# 12.0 COG Initiation

Any cog can start or stop any other cog, or restart or stop itself. Each of the eight cogs has a unique three-bit ID which can be used to start or stop it. It's also possible to start free (stopped or never started) cogs, without needing to know their ID's. This way, entire applications can be written which simply start free cogs, as needed, and as those cogs retire by stopping themselves or getting stopped by others, they return to the pool of free cogs and become available, again, for restarting.

In Spin2 the Cogspin(CogNum,Spin_Method(<(parameters)>,@stack) instruction is used to start cog running a “spin method”

In Spin2 the Coginit(CogID,AsmAddress,Paramater) instruction is used to start cog to run a “PASM” program

| COGSPIN(CogNum, Method({Pars}), StkAddr) | Start Spin2 method in a cog, returns cog's ID if used as an expression element, -1 = no cog free |
| --- | --- |
| COGINIT(CogNum, PASMaddr, PTRAvalue) | Start PASM code in a cog, returns cog's ID if used as an expression element, -1 = no cog free |
| COGSTOP(CogNum) | Stop cog CogNum |
| COGID() : CogNum | Get this cog's ID |
| COGCHK(CogNum) : Running | Check if cog CogNum is running, returns -1 if running or 0 if not |

PASM code can also initiate a cog to run.

RET {WC/WZ/WCZ} Return by popping stack (K). C = K[31], Z = K[30], PC = K[19:0].

REGLOAD and REGEXEC

The Spin2 instructions REGLOAD(HubAddress) and REGEXEC(HubAddress) are used to load or load-and-execute PASM code and/or data chunks from hub RAM into cog registers.

The chunk of PASM code and/or data must be preceded with two words which provide the starting register and the number of registers (longs) to load, minus 1.

PUB go() REGLOAD(@chunk) 'load self-defined chunk from hub into registers REPEAT CALL(#start) 'call program within chunk at register address WAITMS(100) DAT chunk WORD start,finish-start-1 'define chunk start and size-1 ORG $120 'org can be $000..$130-size start DRVRND #56 ADDPINS 7 'some code _RET_ DRVNOT #0 'more code + return finish

REGEXEC works like REGLOAD, but it also CALLs to the start register of the chunk after loading it.

In the example below, REGEXEC launches a chunk of code in upper register memory which sets up a timer interrupt and then returns to Spin2. Meanwhile, as the Spin2 method repeatedly randomizes pins 60..63 every 100ms, the chunk of code loaded into upper register memory perpetuates the timer interrupt and toggles pins 56..59 every 500ms. Note that registers $000..$127 are still free for other code chunks and interrupts 2 and 3 are still unused.

PUB go() REGEXEC(@chunk) 'load self-defined chunk and execute it 'chunk starts timer interrupt and returns REPEAT PINWRITE(60 ADDPINS 3, GETRND()) 'randomize pins 60..63 WAITMS(100) 'pins 56..59 toggle via interrupt DAT chunk WORD start,finish-start-1 'define chunk start and size-1 ORG $128 'org can be $000..$130-size start MOV IJMP1,#isr 'set int1 vector SETINT1 #1 'set int1 to ct-passed-ct1 event GETCT PR0 'get ct _ret_ ADDCT1 PR0,bigwait 'set initial ct1 target, return to Spin2 isr DRVNOT #56 ADDPINS 3 'interrupt service routine, toggle 56..59 ADDCT1 PR0,bigwait 'set next ct1 target RETI1 'return from interrupt bigwait LONG 20_000_000 / 2 '500ms second on RCFAST finish

## 12.1)Built-In Symbols for COGINIT Usage

| COGINIT Symbol Value | Symbol Name | Details |
| --- | --- | --- |
| %00_0000 | COGEXEC (default) | Use "COGEXEC + CogNumber" to start a cog in cogexec mode |
| %10_0000 | HUBEXEC | Use "HUBEXEC + CogNumber" to start a cog in hubexec mode |
| %01_0000 | COGEXEC_NEW | Starts an available cog in cogexec mode |
| %11_0000 | HUBEXEC_NEW | Starts an available cog in hubexec mode |
| %01_0001 | COGEXEC_NEW_PAIR | Starts an available eve/odd pair of cogs in cogexec mode, useful for LUT sharing |
| %11_0001 | HUBEXEC_NEW_PAIR | Starts an available eve/odd pair of cogs in hubexec mode, useful for LUT sharing |

## 12.2) Built-In Symbol for COGSPIN Usage

| COGINIT Symbol Value | Symbol Name | Details |
| --- | --- | --- |
| %01_0000 | NEWCOG | Starts an available cog |

## 12.3) PASM Propeller Assembly Machine Language

PASM stands for propeller assembly language program. PASM can be inline with spin2 code or called and loaded separately. There are two different languages PASM for propeller 1 (32 I/O) and PASM for propeller 2 (64 I/O). Most of the instructions are similar but are not 100% equivalent.

The boot procedure requires spin code to initiated. The spin interpreter then can be used to launch PASM code. The propeller 1 and propeller 2 do not operate in the same manner conderning assembly code and hub access.

## 12.4) In Line PASM Code

Spin2 methods can execute in-line PASM code by preceding the PASM code with an 'ORG {$000..$12F}' and terminating it with an END.

PUB go() ¦ x repeat org getrnd wc 'rotate a random bit into x rcl x,#1 end pinwrite(56 addpins 7, x) 'output x to the P2 Eval board's LEDs waitms(100)

Your PASM code will be assembled with a RET instruction added at the end to ensure that it returns to Spin2, in case no early _RET_ or RET executes.

Here's the internal Spin2 procedure for executing in-line PASM code:

- Save the current streamer address for restoration after the PASM code executes.

- Copy the method's first 16 long variables, including any parameters, return values, and local variables, from hub RAM to cog registers $1E0..$1EF.

- Copy the in-line PASM-code longs from hub RAM into cog registers, starting at the ORG address (default is $000).

- CALL the PASM code.

- Restore the 16 longs in cog registers $1E0..$1EF back to hub RAM, in order to update any modified method variables.

- Restore the streamer address and resume Spin2 bytecode execution.

Within your in-line PASM code, you can do all these things:

- Read and write the following register areas:

  - $000..$12F, which your PASM code loads into. You can even load different PASM programs at different addresses within this range and CALL them from Spin2.

  - $1D8..$1DF, which are general-purpose registers, named PR0..PR7, available to both PASM and Spin2 code.

  - $1E0..$1EF, which temporarily contain the method's first 16 long hub RAM variables and are temporarily assigned the same symbolic names.

  - $1F0..$1FF, which include IJMP3, IRET3, IJMP2, IRET2, IJMP1, IRET1, PA, PB, PTRA,  PTRB, DIRA, DIRB, OUTA, OUTB, INA, and INB.

  - Avoid writing to $130..$1D7 and LUT RAM, since the Spin2 interpreter occupies these areas. You can look in "Spin2_interpreter.spin2" to see the interpreter code.

- Use the streamer temporarily.

- Use up to 5 levels of the hardware stack for nested CALLs, including CALLs to hub RAM.

- Declare and reference regular and local symbols. These symbols will not be accessible outside of your PASM code.

- Declare BYTE, WORD, and LONG data.

- Use the RES, ORGF, and FIT directives. The directives ORG, ORGH, ALIGNW, ALIGNL, and FILE are not allowed within in-line PASM code.

- Establish an interrupt which executes your code remaining in cog registers $000..$12F. Spin2 accommodates interrupts and only stalls them briefly, when necessary.

- Return to Spin2, at any point, by executing an _RET_ or RET instruction.

## 12.5) Calling PASM from Spin2

You can do a CALL(address) in Spin2 to execute PASM code in either cog register space or hub RAM.

PUB go() ¦ x repeat call(@random) pinwrite(56 addpins 7, pr0) waitms(100) DAT orgh 'hub PASM program to rotate a random bit into pr0 random getrnd wc _ret_ rcl pr0,#1

Here's the internal Spin2 procedure for executing a CALL:

- Save the current streamer address for restoration after the PASM code executes.

- CALL the PASM code.

- Restore the streamer address and resume Spin2 bytecode execution.

Within code which you CALL, you can do all these things:

- Read and write the following register areas:

  - $000..$12F, which may contain PASM code and/or data which you previously loaded.

  - $1D8..$1DF, which are general-purpose registers, named PR0..PR7, available to both PASM and Spin2 code.

  - $1E0..$1EF, which are available for scratchpad use, but will likely be rewritten when Spin2 resumes.

  - $1F0..$1FF, which include IJMP3, IRET3, IJMP2, IRET2, IJMP1, IRET1, PA, PB, PTRA,  PTRB, DIRA, DIRB, OUTA, OUTB, INA, and INB.

  - Avoid writing to $130..$1D7 and LUT RAM, since the Spin2 interpreter occupies these areas. You can look in "Spin2_interpreter.spin2" to see the interpreter code.

- Use the streamer temporarily.

- Use up to 5 levels of the hardware stack for nested CALLs, including CALLs to hub RAM.

- Establish an interrupt which executes your code remaining in cog registers $000..$12F. Spin2 accommodates interrupts and only stalls them briefly, when necessary.

- Return to Spin2, at any point, by executing an _RET_ or RET instruction.

## 12.6) Launching Cogs with SpinMethod at StackPointer

Cogspin(CogNum,Spin_Method(<(parameters)>,@stack)

returns cogID if started and running or -1 if no cog free

Note: Followin Stack Space was for P1 assignments

Stack Space should be over estimated and then "Stack Space Tool" can be used to determine

actual size of stack required

2 longs for return address

1 long for return result

1 long for each method parameter

1 long for each local variable

1 long for each intermediate concurrent calculation

## 12.7) Launching Cogs with Assembly Program

The COGINIT instruction is used to start cogs from a PASM program:

COGINIT D/#,S/# {WC}

D/# = 	%0_x_xxxx	The target cog loads its own registers $000..$1F7 from the hub,

starting at address S/#, then begins execution at address $000.

%1_x_xxxx	The target cog begins execution at address S/#.

%x_0_CCCC	The target cog's ID is %CCCC.

%x_1_xxx0	If a cog is free (stopped), then start it.

To know if this succeeded, D must be a register and WC must be

used. If successful, C will be cleared and D will be over-

written with the target cog's ID. Otherwise, C will be set and D will be overwritten with $F.

%x_1_xxx1	If an even/odd cog pair is free (stopped), then start them.

To know if this succeeded, D must be a register and WC must be

used. If successful, C will be cleared and D will be over-

written with the even/lower target cog's ID. Otherwise, C will be

set and D will be overwritten with $F.

S/# = address		This value is either the hub address from which the target cog

Will load from, or it is the cog/hub address from which the target

Cog will begin executing at, depending on D[5]. This 32-bit value

Will be written into the target cog's PTRB register.

If COGINIT is preceded by SETQ, the SETQ value will be written into the target cog's PTRA register. This is intended as a convenient means of pointing the target cog's program to some runtime data structure or passing it a 32-bit parameter. If no SETQ is used, the target cog's PTRA register will be cleared to zero.

COGINIT #1,#$100		'load and start cog 1 from $100

COGINIT #%1_0_0101,PTRA	'start cog 5 at PTRA

SETQ    ptra_val		'ptra_val will go into target cog's PTRA register

COGINIT #%0_1_0000,addr	'load and start a free cog at addr

COGINIT #%1_1_0001,addr	'start a pair of free cogs at addr (lookup RAM sharing)

COGINIT id,addr WC		'(id=$30) start a free cog at addr, C=0 and id=cog if okay

COGID   myID			'reload and restart me at PTRB

COGINIT myID,PTRB

The COGSTOP instruction is used to stop cogs. The 4 LSB's of the D/# operand supply the target cog ID.

COGSTOP #0			'stop cog 0

COGID   myID			'stop me

COGSTOP myID

A cog can discover its own ID by doing a COGID instruction, which will return its ID into D[3:0], with upper bits cleared. This is useful, in case the cog wants to restart or stop itself, as shown above.

If COGID is used with WC, it will not overwrite D, but will return the status of cog D/# into C, where C=0 indicates the cog is free (stopped or never started) and C=1 indicates the cog is busy (started).

COGID   ThatCog  WC		'C=1 if ThatCog is busy

## 12.1_Example_WRD_COGINIT_COGEXEC_NEW

{{12.1_Example_WRD_COGINIT_COGEXEC_NEW}}

''Debug must be enabled

{-------------------------------------------------------------------------------------------------------------}

CON {Processor Timing}

_clkfreq  = 200_000_000  'processor clock speed

VAR

Byte cogStarted_COGEXEC_NEW 'cog ID started is returned or -1 if not started

PUB main()

cogStarted_COGEXEC_NEW := COGINIT(COGEXEC_NEW,@_blink01,$FFF_FFFF)

'Start next available cog which is 1 and load cog 1 memory with _blink01 PASM at Cog Memory $000

'PTRA will be loaded with $FFF_FFFF

repeat  'keep cog 0 running

DAT     ORG   0  'COGINIT(COGEXEC_NEW,@_blink,PTRAvalue)

_blink01

MOV     DIRA, #$FF           'Set the direction of the first 8 pins to Output

GETCT   cogCounterValue      'Get global system counter value

ADDCT1  cogCounterValue,PTRA 'set CT1 event to trigger on CT = countvalue + PTRA

._Loop  WAITCT1

ADDCT1  cogCounterValue,PTRA

XOR     OUTA, #1

NOP

debug(ubin(OUTA))             'send status to Debug Window

JMP     #._Loop                'JMP to Loop

cogCounterValue Long 0                'counter value CT storage

'------------------------------------------

## 12.2_Example_WRD_COGINIT_COGEXEC_CogID

{{12.1_Example_WRD_COGINIT_COGEXEC_CogID}}

''Debug must be enabled

{-------------------------------------------------------------------------------------------------------------}

CON {Processor Timing}

_clkfreq  = 200_000_000  'processor clock speed

VAR

Byte cogStarted_COGEXEC_CogID 'cog ID started is returned or -1 if not started

Byte Cog_ID

PUB main()

Cog_ID := 2    'the cog to be started is 2

cogStarted_COGEXEC_CogID := COGINIT(COGEXEC + Cog_ID,@_blink01,$FFF_FFFF)

debug(udec(cogStarted_COGEXEC_CogID))

'Start next available cog which is 1 and load cog 1 memory with _blink01 PASM at Cog Memory $000

'PTRA will be loaded with $FFF_FFFF

repeat  'keep cog 0 running

DAT     ORG   0  'COGINIT(COGEXEC + CogID,@_blink,PTRAvalue)

_blink01

MOV     DIRA, #$FF           'Set the direction of the first 8 pins to Output

GETCT   cogCounterValue      'Get global system counter value

ADDCT1  cogCounterValue,PTRA 'set CT1 event to trigger on CT = countvalue + PTRA

._Loop  WAITCT1

ADDCT1  cogCounterValue,PTRA

XOR     OUTA, #1

NOP

debug(ubin(OUTA))             'send status to Debug Window

JMP     #._Loop                'JMP to Loop

cogCounterValue Long 0                'counter value CT storage

'------------------------------------------

## 12.3_Example_WRD_COGINIT_HUBEXEC

{12.3_Example_WRD_COGINIT_HUBEXEC}

{{

COGINIT(CogNum, PASMaddr, PTRAvalue)    Start PASM code in a cog,

returns cog's ID if used as an expression element, -1 = no cog free

}}

CON

_clkfreq = 200_000_000     'debug must have a clock greater than 10MHZ

pub go()

coginit(HUBEXEC_NEW,@blink,0)               'launch hub-exe program in free cog

'%11_0000 = HUBEXEC_NEW  Starts an available cog in hubexec mode

'coginit(32+16,@blink,0)

DAT     orgh                            'being hub-exec program

blink   setbyte dira,#$FF,#0            'make LEDs outputs

debug(ubin(dira))

mov     pb,#0                   'clear pb

loop    loc     pa,#table               'get address of table (relative)

add     pa,pb                   'add pb

rdbyte  pa,pa                   'read pa table byte and put in pa

not     pa                      'NOT for LEDs

setbyte outa,pa,#0             'write to LEDs

debug(ubin(outa))

waitx   ##clkfreq_

debug(udec(#clkfreq_))

incmod  pb,#7                   'inc pb 0..7 and repeat

'INCMOD D,{#}S {WC/WZ/WCZ} Increment with modulus. If D = S then D = 0 and C = 1, else D = D + 1        ‘and C = 0. *

waitx   ##clkfreq_           'wait 1/10 second

'WAITX {#}D {WC/WZ/WCZ} Wait 2 + D clocks if no WC/WZ/WCZ. If WC/WZ/WCZ, wait 2 + (D & RND) ‘clocks. C/Z = 0.

jmp     #loop                   'loop

table   byte    $01,$02,$04,$08,$10,$20,$40,$80

## 12.4_Example_WRD_REGLOAD

## 12.5_Example_WRD_8CogSpin_Demo

This program does not have any looping it is in line code sequence. The following program 12.6 uses variables instead of constants and loops with repeat command to load cogspin programs. It would be of interest to see which program runs faster.

{{12.5_Example_WRD_8CogSpinDemo}}

'Cogstart

'COGSPIN(CogNum, Method({Pars}), StkAddr)

con

_clkfreq = 200_000_000

P0 = 0,P1 = 1,P2 = 2,P3 = 3,P4 = 4,P5 = 5,P6 = 6,P7 = 7

Cog1 = 1,Cog2 = 2,Cog3 = 3,Cog4 = 4,Cog5 = 5,Cog6 = 6,Cog7 = 7

x01 = 5,x02 = 10,x03 = 15,x04 = 20,x05 = 25,x06= 30,x07 = 35

var

long   Cog1Stack[32],Cog2Stack[32],Cog3Stack[32],Cog4Stack[32],Cog5Stack[32],Cog6Stack[32],Cog7Stack[32]

long   Cog1Status,Cog2Status,Cog3Status,Cog4Status,Cog5Status,Cog6Status,Cog7Status

long   NumCogsRunning

pub main()|x

NumCogsRunning := 0                            'test

repeat

if NumCogsRunning == 0

debug("Cogs OFF Re-Start Scan")

waitms(3000)

pintoggle(P0)

''cogspin(CogNum,Spin_Method(<(parameters)>,@stack)

cogspin(Cog1,blink(P1,x01),@Cog1Stack)

cogspin(Cog2,blink(P2,x02),@Cog2Stack)

cogspin(Cog3,blink(P3,x03),@Cog3Stack)

cogspin(Cog4,blink(P4,x04),@Cog4Stack)

cogspin(Cog5,blink(P5,x05),@Cog5Stack)

cogspin(Cog6,blink(P6,x06),@Cog6Stack)

cogspin(Cog7,blink(P7,x07),@Cog7Stack)

NumCogsRunning := 7

repeat while (NumCogsRunning > 0)

waitms(500)

Cog1Status := CogChk(Cog1)   'CogChk() returns -1 if running 0 if off

debug(udec(Cog1Status))      'Convert CogStatus -1 $FFFF_FFFF to 1 to allow add

if Cog1Status == 0

Cog1Status := 0

else

Cog1Status := 1

debug(udec(Cog1Status))

Cog2Status := CogChk(Cog2)

debug(udec(Cog2Status))

if Cog2Status == 0

Cog2Status := 0

else

Cog2Status := 1

debug(udec(Cog2Status))

Cog3Status := CogChk(Cog3)

debug(udec(Cog3Status))

if Cog3Status == 0

Cog3Status := 0

else

Cog3Status := 1

debug(udec(Cog3Status))

Cog4Status := CogChk(Cog4)

debug(udec(Cog4Status))

if Cog4Status == 0

Cog4Status := 0

else

Cog4Status := 1

debug(udec(Cog4Status))

Cog5Status := CogChk(Cog5)

debug(udec(Cog5Status))

if Cog5Status == 0

Cog5Status := 0

else

Cog5Status := 1

debug(udec(Cog5Status))

Cog6Status := CogChk(Cog6)

debug(udec(Cog6Status))

if Cog6Status == 0

Cog6Status := 0

else

Cog6Status := 1

debug(udec(Cog6Status))

Cog7Status := CogChk(Cog7)

debug(udec(Cog7Status))

if Cog7Status == 0

Cog7Status := 0

else

Cog7Status := 1

debug(udec(Cog7Status))

NumCogsRunning := Cog1Status + Cog2Status + Cog3Status + Cog4Status + Cog5Status + Cog6Status + Cog7Status

debug(udec(NumCogsRunning))

pub blink(pin,x)|z

repeat z from 0 to x

pintoggle(pin)                       '  Toggle I/O 56 (LED on P2 EVAL board)

waitms(500)

pinlow(pin)

## 12.6_Example_WRD_8CogSpin_Demo_Rev

{{12.6_Example_WRD_8CogSpinDemo}}

'COGSPIN(CogNum, Method({Pars}), StkAddr)

con

_clkfreq = 200_000_000

var

byte   Pin[8]                        'pin array 0..7

byte   Cog[8]                       'cog ID array 0..7

long   Delay[8]                     'delay array 0..7

long   CogStk[224]              'stack for 7 cogs 32 long each

long   CogStatus[8]             'individualcog status

long   NumCogsRunning     'number of cogs running

pub main()|x

NumCogsRunning := 0                            'test

repeat x from 0 to 7                           'loop to load

Pin[x],Cog[x] := x ,x                                'load pin numbersand load cog Id numbers

Delay[x] := 20 + (5 * x)                    'load delay numbers

repeat

if NumCogsRunning == 0

debug("Cogs OFF Re-Start Scan")

waitms(1000)

pintoggle(Pin[0])

''cogspin(CogNum,Spin_Method(<(parameters)>,@stack)

repeat x from 1 to 7        ‘'cogspin(CogNum,Spin_Method(<(parameters)>,@stack)

cogspin(Cog[x],blink(Pin[x],Delay[x]),@CogStk[(32*x -32)])

debug(udec(Cog[x]),udec(Pin[x]),udec(Delay[x]),udec(@CogStk[(32*x-32)]))

NumCogsRunning := 7

debug(udec(NumCogsRunning))

waitms(1500)

repeat while (NumCogsRunning > 0)

waitms(500)

repeat x from 1 to 7

CogStatus[x] := CogChk(Cog[x])   'CogChk() returns -1 if running 0 if off

debug(udec(x),udec(CogStatus[x]))        'Convert CogStatus -1 $FFFF_FFFF to 1 to allow add

if CogStatus[x] == 0

CogStatus[x] := 0

else

CogStatus[x] := 1

debug(udec(x),udec(CogStatus[x]))

NumCogsRunning := CogStatus[1] + CogStatus[2] + CogStatus[3] + CogStatus[4] + CogStatus[5] + CogStatus[6] + CogStatus[7]

debug(udec(NumCogsRunning))

pub blink(Ppin,Ddelay)|z

repeat z from 0 to Ddelay

pintoggle(Ppin)                       '  Toggle I/O 56 (LED on P2 EVAL board)

waitms(500)

pinlow(pin)

# 13.0) Debug for Testing and Troubleshooting

The Spin2 compiler contains a stealthy debugger program that can be automatically downloaded with your application. It uses the last 16KB of RAM plus a few bytes for each Spin2 DEBUG statement and one instruction for each PASM DEBUG statement. You place DEBUG statements in your application which contain output commands that will serially transmit the state of variables and equations as your application runs. Each time a DEBUG statement is encountered during execution, the debugger is invoked and it outputs the message for that statement. Debugging is initiated by adding the Ctrl key to the usual F10 to 'run' or F11 to 'program'. This compiles your application with all the DEBUG statements, adds the debugger to the download, and then brings up the DEBUG Output window which begins receiving messages at the start of your application. DEBUG can be used in Spin2 or PASM.

## 13.1) Things to know about the DEBUG system

- To use the debugger, you must configure at least a 10 MHz clock derived from a crystal or external input. You cannot use RCFAST or RCSLOW.

- The debugger occupies the top 16 KB of hub RAM, remapped to $FC000..$FFFFF and write-protected. The hub RAM at $7C000..$7FFFF will no longer be available.

- Data defining each DEBUG statement is stored within the debugger image in the top 16 KB of RAM, minimizing impact on your application code.

- In Spin2, each DEBUG statement adds three bytes, plus any code needed to reference variables and resolve run-time expressions used in the DEBUG statement.

- In PASM, each DEBUG statement adds one instruction (long).

- DEBUG statements are ignored by the compiler when not compiling for DEBUG mode, so you don't need to comment them out when debugging is not in use.

- If no DEBUG statements exist in your application, you will still get notification messages when cogs are started.

- Debugging is invoked by pressing the CTRL key before the usual F9..F11 keys, which compile, download, and program to flash.

- During execution, as DEBUG statements are encountered, text messages are sent out serially on P62 at 2 Mbaud in 8-N-1 format.

- DEBUG messages always start with "CogN  ", where N is the cog number, followed by two spaces, and they always end with CR+LF (new line).

- Up to 255 DEBUG statements can exist within your application, since the BRK instruction is used to interrupt and select the particular DEBUG statement definition.

- You can define several symbols to modify debugger behavior: DEBUG_COG, DEBUG_DELAY, DEBUG_PIN, DEBUG_TIMESTAMP, etc. See table.

- Each time a debug-enabled cog is started, a debug message is output to indicate the cog number, code address (PTRB), parameter (PTRA), and 'load' or 'jump' mode.

- For Spin2, DEBUG statements can output expression and variable values, hub byte/word/long arrays, and register arrays.

- For PASM, DEBUG statements can output register values/arrays, hub byte/word/long arrays, and constants. PASM syntax is used: implied register or #immediate.

- DEBUG output data can be displayed in decimal, hex, or binary, signed or unsigned, and sized to byte, word, long, or auto. Hub character strings are also supported.

- DEBUG output commands show both the source and value: "DEBUG(UHEX(x))" might output "x = $123".

- DEBUG commands which output data can have multiple sets of parameters, separated by commas: SDEC(x,y,z) and LSTR(ptr1,size1,ptr2,size2)

- Commas are automatically output between data: "DEBUG(UHEX_BYTE(d,e,f), SDEC(g))" might output "d = $45, e = $67, f = $89, g = -1_024".

- All DEBUG output commands have alternate versions, ending in "_" which output only the value: DEBUG(UHEX_BYTE_(d,e,f)) might output "$45, $67, $89".

- DEBUG statements can contain comma-separated strings and characters, aside from commands: DEBUG("We got here! Oh, Nooooo...", 13, 13)

- DEBUG statements may contain IF() and IFNOT() commands to gate further output within the statement. An initial IF/IFNOT will gate the entire message.

- DEBUG statements may contain a final DLY(milliseconds) command to slow down a cog's messaging, since messages may stream at the rate of ~10,000 per second.

- DEBUG serial output can be redirected to a different pin, at a different baud rate, for displaying/logging elsewhere.

- LOCK[15] is allocated by the debugger and used among all cogs during their debug interrupts to time-share the DEBUG serial-transmit pin.

- Command-line supports DEBUG-only mode: PNut -debug {CommPort if not 1} {BaudRate if not 2_000_000}

| DEBUG Statement (v=100, BYTE[a]=1,2,3,4,5) | DEBUG Message Output | Note |
| --- | --- | --- |
| DEBUG("`LOGIC MyDisplay SAMPLES ", SDEC_(v)) | Cog0 `LOGIC MyDisplay SAMPLES 100 | Regular DEBUG syntax can drive DEBUG displays, but it's not optimal. |
| DEBUG(`LOGIC MyDisplay SAMPLES 100) | `LOGIC MyDisplay SAMPLES 100 | DEBUG-display syntax is simpler and 'CogN' is omitted in the output. |
| DEBUG(`LOGIC MyDisplay SAMPLES `(v)) | `LOGIC MyDisplay SAMPLES 100 | Decimal numbers are output using `(value) notation. Short for SDEC_. |
| DEBUG(`LOGIC MyDisplay SAMPLES `$(v)) | `LOGIC MyDisplay SAMPLES $64 | Hex numbers are output using `$(value) notation. Short for UHEX_. |
| DEBUG(`LOGIC MyDisplay SAMPLES `%(v)) | `LOGIC MyDisplay SAMPLES %1100100 | Binary numbers are output using `%(value) notation. Short for UBIN_. |
| DEBUG(`LOGIC MyDisplay TITLE '`#(v)') | `LOGIC MyDisplay TITLE 'd' | Characters are output using `#(value) notation. |
| DEBUG(`MyDisplay `UDEC_BYTE_ARRAY_(@a,5)) | `MyDisplay 1, 2, 3, 4, 5 | Regular DEBUG commands can follow the backtick, as well. |

## 13.2) Simple DEBUG example in Spin2

CON _clkfreq = 10_000_000 'set 10 MHz clock (assumes 20 MHz crystal) PUB go() ¦ i REPEAT i FROM 0 TO 9 'count from 0 to 9 DEBUG(UDEC(i)) 'debug, output i

When run with Ctrl-F10, the Debug window opens and this is what appears:

Cog0 INIT $0000_0000 $0000_0000 load Cog0 INIT $0000_0D6C $0000_10BC jump Cog0 i = 0 Cog0 i = 1 Cog0 i = 2 Cog0 i = 3 Cog0 i = 4 Cog0 i = 5 Cog0 i = 6 Cog0 i = 7 Cog0 i = 8 Cog0 i = 9

In the first line of the report, you see Cog0 loading the Spin2 set-up code from $00000. In the second line, the Spin2 interpreter is launched from $00D58 with its stack space starting at $0101C. After that, the Spin2 program is running and you see 'i' iterating from 0 to 9.

If you change the "9" to "99" in the REPEAT, data will scroll too fast to read, but by adding a DLY command at the end of the DEBUG statement, you can slow down the output:

debug(udec(i), dly(250))     'debug, output i with a 250ms delay after each report

Let's say you want to limit the messages being output, so that only odd values of 'i' are shown. You could use an IF at the start of your DEBUG statement to check the least-significant bit of 'i'. When the IF is false, no message will be output, causing only the odd values of i to be shown:

debug(if(i & 1), udec(i), dly(250))     'debug, output only odd i values with a 250ms delay after each report

## 13.3) Simple DEBUG example in PASM

CON _clkfreq = 10_000_000 'set 10 MHz clock (assumes 20 MHz crystal) DAT ORG MOV i,#9 'set i to 9 loop DEBUG (UHEX_LONG(i)) 'debug, output i in hex DJNF i,#loop 'decrement i and loop if not -1 JMP #$ 'don't go wandering off, stay here i RES 1 'reserve one register as 'i'

When run with Ctrl-F10, the Debug window opens and this is what appears:

Cog0 INIT $0000_0000 $0000_0000 load Cog0 i = $0000_0009 Cog0 i = $0000_0008 Cog0 i = $0000_0007 Cog0 i = $0000_0006 Cog0 i = $0000_0005 Cog0 i = $0000_0004 Cog0 i = $0000_0003 Cog0 i = $0000_0002 Cog0 i = $0000_0001 Cog0 i = $0000_0000

In the first line of the report, you see Cog0 loading our PASM program from $00000. After that, the program runs and you see 'i' iterating from 9 down to 0.

If you change the "9" to "99" in the MOV instruction and you'd like to slow things down, add a DLY command to the DEBUG statement and be sure to express the milliseconds as #250, since a plain 250 would be understood as register 250:

debug   (uhex_long(i), dly(#250))      'debug, output i in hex and delay for 250ms after each report

There are two steps to using graphical DEBUG displays. First, they must be instantiated and, second, they must be fed:

| To Use a Display: | 1st | 2nd | 3rd | 4th | Note |
| --- | --- | --- | --- | --- | --- |
| First, instantiate it. | ` | display_type | unknown_symbol | keyword(s), number(s), string(s) | Unknown_symbol becomes instance_name. |
| Then, feed it. | ` | instance_name(s) | keyword(s), number(s), string(s) |  | Multiple displays can be fed the same data. |

To bring this all together, let's show a sawtooth wave on a SCOPE display:

| CON _clkfreq = 10_000_000 PUB go() ¦ i debug(`SCOPE MyScope SIZE 254 84 SAMPLES 128) debug(`MyScope 'Sawtooth' 0 63 64 10 %1111) repeat debug(`MyScope `(i & 63)) i++ waitms(50). |  |
| --- | --- |

Step 1 Instanitiate Graphic Display

debug(`SCOPE Scope_Name SIZE 254 84 SAMPLES 128)  ‘note the backtick “ ` ”

debug(`Scope_Name ‘Sawtooth’  0 63 64 10 %1111)   ‘note strings have single quote ‘Sawtooth’

Step 2 Feed Data to Window (Display)

Debug(`Scope_Name   ( i  & 63))

Debug(`Scope_Name

In the example above, a SCOPE is instantiated called MyScope that is 254 x 84 pixels and shows 128 samples. A width of 254 was chosen since samples are numbered 0..127 and I wanted them to be spaced at a constant two-pixel pitch (127 * 2 = 254). A height of 84 was chosen so that there would be 10 pixels above and below the waveform, which will have a height of 64 pixels.

A channel called "Sawtooth" is defined which, for the purpose of display, has a bottom value of 0 and a top value of 63, is 64 pixels tall within that range, and is elevated 10 pixels off the bottom of the scope window. The %1111 enables top and bottom legend values and top and bottom lines. Within the REPEAT block, the SCOPE is fed a repeating pattern of 0..63 which forms the sawtooth wave. The SCOPE updates its display each time it receives a value. If there were eight channels defined, instead of just one, it would update the display on every eighth value received, drawing all eight channels.

## 13.4) Commands for use in DEBUG statements

| Conditionals | Details |
| --- | --- |
| IF(condition) | If condition <> 0 then continue at the next command within the DEBUG statement, else skip all remaining commands and output CR+LF. If used as the first command in the DEBUG statement, IF will gate ALL output for the statement, including the "CogN "+CR+LF. This way, DEBUG messages can be entirely suppressed, so that you can filter what is important. |
| IFNOT(condition) | If condition = 0 then continue at the next command within the DEBUG statement, else skip all remaining commands and output CR+LF. If used as the first command in the DEBUG statement, IFNOT will gate ALL output for the statement, including the "CogN "+CR+LF. This way, DEBUG messages can be entirely suppressed, so that you can filter what is important. |

| String Output * | Details | Output |
| --- | --- | --- |
| ZSTR(hub_pointer) | Output zero-terminated string at hub_pointer | "Hello!" |
| LSTR(hub_pointer,size) | Output 'size' characters of string at hub_pointer | "Goodbye." |

| Decimal Output, unsigned * | Details | Min Output | Max Output |
| --- | --- | --- | --- |
| UDEC(value) | Output unsigned decimal value | 0 | 4_294_967_295 |
| UDEC_BYTE(value) | Output byte-size unsigned decimal value | 0 | 255 |
| UDEC_WORD(value) | Output word-size unsigned decimal value | 0 | 65_535 |
| UDEC_LONG(value) | Output long-size unsigned decimal value | 0 | 4_294_967_295 |
| UDEC_REG_ARRAY(reg_pointer,size) | Output register array as unsigned decimal values | 0 | 4_294_967_295 |
| UDEC_BYTE_ARRAY(hub_pointer,size) | Output hub byte array as unsigned decimal values | 0 | 255 |
| UDEC_WORD_ARRAY(hub_pointer,size) | Output hub word array as unsigned decimal values | 0 | 65_535 |
| UDEC_LONG_ARRAY(hub_pointer,size) | Output hub long array as unsigned decimal values | 0 | 4_294_967_295 |
| Decimal Output, signed * | Details | Min Output | Max Output |
| SDEC(value) | Output signed decimal value | -2_147_483_648 | 2_147_483_647 |
| SDEC_BYTE(value) | Output byte-size signed decimal value | -128 | 127 |
| SDEC_WORD(value) | Output word-size signed decimal value | -32_768 | 32_767 |
| SDEC_LONG(value) | Output long-size signed decimal value | -2_147_483_648 | 2_147_483_647 |
| SDEC_REG_ARRAY(reg_pointer,size) | Output register array as signed decimal values | -2_147_483_648 | 2_147_483_647 |
| SDEC_BYTE_ARRAY(hub_pointer,size) | Output hub byte array as signed decimal values | -128 | 127 |
| SDEC_WORD_ARRAY(hub_pointer,size) | Output hub word array as signed decimal values | -32_768 | 32_767 |
| SDEC_LONG_ARRAY(hub_pointer,size) | Output hub long array as signed decimal values | -2_147_483_648 | 2_147_483_647 |
| Hexadecimal Output, unsigned * | Details | Min Output | Max Output |
| UHEX(value) | Output auto-size unsigned hex value | $0 | $FFFF_FFFF |
| UHEX_BYTE(value) | Output byte-size unsigned hex value | $00 | $FF |
| UHEX_WORD(value) | Output word-size unsigned hex value | $0000 | $FFFF |
| UHEX_LONG(value) | Output long-size unsigned hex value | $0000_0000 | $FFFF_FFFF |
| UHEX_REG_ARRAY(reg_pointer,size) | Output register array as unsigned hex values | $0000_0000 | $FFFF_FFFF |
| UHEX_BYTE_ARRAY(hub_pointer,size) | Output hub byte array as unsigned hex values | $00 | $FF |
| UHEX_WORD_ARRAY(hub_pointer,size) | Output hub word array as unsigned hex values | $0000 | $FFFF |
| UHEX_LONG_ARRAY(hub_pointer,size) | Output hub long array as unsigned hex values | $0000_0000 | $FFFF_FFFF |
| Hexadecimal Output, signed * | Details | Min Output | Max Output |
| SHEX(value) | Output auto-size signed hex value | -$8000_0000 | $7FFF_FFFF |
| SHEX_BYTE(value) | Output byte-size signed hex value | -$80 | $7F |
| SHEX_WORD(value) | Output word-size signed hex value | -$8000 | $7FFF |
| SHEX_LONG(value) | Output long-size signed hex value | -$8000_0000 | $7FFF_FFFF |
| SHEX_REG_ARRAY(reg_pointer,size) | Output register array as signed hex values | -$8000_0000 | $7FFF_FFFF |
| SHEX_BYTE_ARRAY(hub_pointer,size) | Output hub byte array as signed hex values | -$80 | $7F |
| SHEX_WORD_ARRAY(hub_pointer,size) | Output hub word array as signed hex values | -$8000 | $7FFF |
| SHEX_LONG_ARRAY(hub_pointer,size) | Output hub long array as signed hex values | -$8000_0000 | $7FFF_FFFF |
| Binary Output, unsigned * | Details | Min Output | Max Output |
| UBIN(value) | Output auto-size unsigned binary value | %0 | %11111111_11111111_11111111_11111111 |
| UBIN_BYTE(value) | Output byte-size unsigned binary value | %00000000 | %11111111 |
| UBIN_WORD(value) | Output word-size unsigned binary value | %00000000_00000000 | %11111111_11111111 |
| UBIN_LONG(value) | Output long-size unsigned binary value | %00000000_00000000_00000000_00000000 | %11111111_11111111_11111111_11111111 |
| UBIN_REG_ARRAY(reg_pointer,size) | Output register array as unsigned binary values | %00000000_00000000_00000000_00000000 | %11111111_11111111_11111111_11111111 |
| UBIN_BYTE_ARRAY(hub_pointer,size) | Output hub byte array as unsigned binary values | %00000000 | %11111111 |
| UBIN_WORD_ARRAY(hub_pointer,size) | Output hub word array as unsigned binary values | %00000000_00000000 | %11111111_11111111 |
| UBIN_LONG_ARRAY(hub_pointer,size) | Output hub long array as unsigned binary values | %00000000_00000000_00000000_00000000 | %11111111_11111111_11111111_11111111 |
| Binary Output, signed * | Details | Min Output | Max Output |
| SBIN(value) | Output auto-size signed binary value | -%10000000_00000000_00000000_00000000 | %01111111_11111111_11111111_11111111 |
| SBIN_BYTE(value) | Output byte-size signed binary value | -%10000000 | %01111111 |
| SBIN_WORD(value) | Output word-size signed binary value | -%10000000_00000000 | %01111111_11111111 |
| SBIN_LONG(value) | Output long-size signed binary value | -%10000000_00000000_00000000_00000000 | %01111111_11111111_11111111_11111111 |
| SBIN_REG_ARRAY(reg_pointer,size) | Output register array as signed binary values | -%10000000_00000000_00000000_00000000 | %01111111_11111111_11111111_11111111 |
| SBIN_BYTE_ARRAY(hub_pointer,size) | Output hub byte array as signed binary values | -%10000000 | %01111111 |
| SBIN_WORD_ARRAY(hub_pointer,size) | Output hub word array as signed binary values | -%10000000_00000000 | %01111111_11111111 |
| SBIN_LONG_ARRAY(hub_pointer,size) | Output hub long array as signed binary values | -%10000000_00000000_00000000_00000000 | %01111111_11111111_11111111_11111111 |

| Delay to Pace Messages | Details |
| --- | --- |
| DLY(milliseconds) | Delay for some milliseconds to slow down continuous message outputs for this cog. DLY is only allowed as the last command in a DEBUG statement, since it releases LOCK[15] before the delay, permitting other cogs to capture LOCK[15] so that they may take control of the DEBUG serial-transmit pin and output their own DEBUG messages. |

*  These commands accept multiple parameters, or multiple sets of parameters. Alternate commands with the same names, but ending in "_", are also available for value-only output (i.e. ZSTR_, LSTR_, UDEC_).

## 13.5) Symbols you can define to modify DEBUG behavior

| CON Symbol | Default | Purpose |
| --- | --- | --- |
| DEBUG_COGS | %11111111 | Selects which cogs have debug interrupts enabled. Bits 7..0 enable debugging interrupts in cogs 7..0. |
| DEBUG_DELAY | 0 | Sets a delay in milliseconds before your application runs and DEBUG messages start appearing. |
| DEBUG_PIN | 62 | Sets the DEBUG serial output pin. For DEBUG windows to open, DEBUG_PIN must be 62. |
| DEBUG_BAUD | 2_000_000 | Sets the DEBUG baud rate. |
| DEBUG_TIMESTAMP | undefined | By declaring this symbol, each DEBUG message will be time-stamped with the 64-bit CT value. |
| DEBUG_LOG_SIZE | 0 | Sets the maximum size of the 'DEBUG.log' file which will collect DEBUG messages. A value of 0 will inhibit log file generation. |
| DEBUG_LEFT | (dynamic) | Sets the left screen coordinate where the DEBUG message window will appear. |
| DEBUG_TOP | (dynamic) | Sets the top screen coordinate where the DEBUG message window will appear. |
| DEBUG_WIDTH | (dynamic) | Sets the width of the DEBUG message window. |
| DEBUG_HEIGHT | (dynamic) | Sets the height of the DEBUG message window. |
| DEBUG_DISPLAY_LEFT | 0 | Sets the overall left screen offset where any DEBUG displays will appear (adds to 'POS' x coordinate in each DEBUG display). |
| DEBUG_DISPLAY_TOP | 0 | Sets the overall top screen offset where any DEBUG displays will appear (adds to 'POS' y coordinate in each DEBUG display). |
| DEBUG_WINDOWS_OFF | 0 | Disables any DEBUG windows from opening after downloading, if set to a non-zero value. |

## 13.6) Packed-Data Modes

Packed-data modes are used to efficiently convey sub-byte data types, by having the host side unpack them from bytes, words, or longs it receives. As well, bytes can be sent within words and longs, and words can be sent within longs for some efficiency improvement.

To establish packed-data operation, you must specify one of the modes listed below, followed by optional 'ALT' and 'SIGNED' keywords:

packed_mode {ALT} {SIGNED}

The ALT keyword will cause bits, double-bits, or nibbles, within each byte sent, to be reordered on the host side, within each byte. This simplifies cases where the raw data you are sending has its bitfields out-of-order with respect to the DEBUG display you are using. This is most-likely to be needed for bitmap data that was composed in standard formats.

The SIGNED keyword will cause all unpacked data values to be sign-extended on the host side.

| Packed-Data Modes | Descriptions | Final Values | Final Values if SIGNED |
| --- | --- | --- | --- |
| LONGS_1BIT | Each value received is translated into 32 separate 1-bit values, starting from the LSB of the received value. | 0..1 | -1..0 |
| LONGS_2BIT | Each value received is translated into 16 separate 2-bit values, starting from the LSBs of the received value. | 0..3 | -2..1 |
| LONGS_4BIT | Each value received is translated into 8 separate 4-bit values, starting from the LSBs of the received value. | 0..15 | -8..7 |
| LONGS_8BIT | Each value received is translated into 4 separate 8-bit values, starting from the LSBs of the received value. | 0..255 | -128..127 |
| LONGS_16BIT | Each value received is translated into 2 separate 16-bit values, starting from the LSBs of the received value. | 0..65,535 | -32,768..32,767 |
| WORDS_1BIT | Each value received is translated into 16 separate 1-bit values, starting from the LSB of the received value. | 0..1 | -1..0 |
| WORDS_2BIT | Each value received is translated into 8 separate 2-bit values, starting from the LSBs of the received value. | 0..3 | -2..1 |
| WORDS_4BIT | Each value received is translated into 4 separate 4-bit values, starting from the LSBs of the received value. | 0..15 | -8..7 |
| WORDS_8BIT | Each value received is translated into 2 separate 8-bit values, starting from the LSBs of the received value. | 0..255 | -128..127 |
| BYTES_1BIT | Each value received is translated into 8 separate 1-bit values, starting from the LSB of the received value. | 0..1 | -1..0 |
| BYTES_2BIT | Each value received is translated into 4 separate 2-bit values, starting from the LSBs of the received value. | 0..3 | -2..1 |
| BYTES_4BIT | Each value received is translated into 2 separate 4-bit values, starting from the LSBs of the received value. | 0..15 | -8..7 |

## 13.7) Graphical DEBUG Displays

DEBUG messages can invoke special graphical DEBUG displays which are built into the tool. These graphical displays each take the form of a unique window. Once instantiated, displays can be continuously fed data to generate animated visualizations. These displays are very handy for development and debugging, as various data types can be viewed in their native contexts. Up to 32 graphical displays can be running simultaneously.

Currently, the following graphical DEBUG displays are implemented, but more will be added in the future:

| Display Types | Descriptions |
| --- | --- |
| LOGIC | Logic analyzer with single and multi-bit labels, 1..32 channels, can trigger on pattern |
| SCOPE | Oscilloscope with 1..8 channels, can trigger on level with hysteresis |
| SCOPE_XY | XY oscilloscope with 1..8 channels, persistence of 0..512 samples, polar mode, log scale mode |
| FFT | Fast Fourier Transform with 1..8 channels, 4..2048 points, windowed results, log scale mode |
| SPECTRO | Spectrograph with 4..2048-point FFT, windowed results, phase-coloring, and log scale mode |
| PLOT | General-purpose plotter with cartesian and polar modes |
| TERM | Text terminal with up to 300 x 200 characters, 6..200 point font size, 4 simultaneous color schemes |
| BITMAP | Bitmap, 1..2048 x 1..2048 pixels, 1/2/4/8/16/32-bit pixels with 19 color systems, 15 direction/autoscroll modes, independent X and Y pixel size of 1..256 |
| MIDI | Piano keyboard with 1..128 keys, velocity depiction, variable screen scale |

When a DEBUG message contains a backtick (`) character (ASCII $60), a string, containing everything from the backtick to the end of the message, is sent to the graphical DEBUG display parser. The parser looks for several different element types, treating any commas as whitespace:

| Element Type | Example | Description |
| --- | --- | --- |
| display_type | LOGIC, SCOPE, PLOT, BITMAP | This is the formal name of the graphical DEBUG display type you wish to instantiate. |
| unknown_symbol | MyLogicDisplay | Each graphical DEBUG display Instance must be given a unique symbolic name. |
| instance_name | MyLogicDisplay | Once instantiated, a graphical DEBUG display instance is referenced by its symbolic name. |
| keyword | TITLE, POS, SIZE, SAMPLES | Keywords are used to configure displays. They might be followed by numbers, strings, and other keywords. |
| number | 1024, $FF, %1010 | Numbers can be expressed in decimal, hex ($), and binary (%). |
| string | 'Here is a string' | Strings are expressed within single-quotes. |

Before getting into how all this fits together, we need to go over some special DEBUG-display syntax that can be used for displays. This syntax is invoked when the first character in the DEBUG statement is the backtick. This causes everything in the DEBUG statement to be viewed as a string, except when subsequent backticks act as 'escape' characters to allow normal or shorthand DEBUG commands.

| DEBUG Statement (v=100, BYTE[a]=1,2,3,4,5) | DEBUG Message Output | Note |
| --- | --- | --- |
| DEBUG("`LOGIC MyDisplay SAMPLES ", SDEC_(v)) | Cog0 `LOGIC MyDisplay SAMPLES 100 | Regular DEBUG syntax can drive DEBUG displays, but it's not optimal. |
| DEBUG(`LOGIC MyDisplay SAMPLES 100) | `LOGIC MyDisplay SAMPLES 100 | DEBUG-display syntax is simpler and 'CogN' is omitted in the output. |
| DEBUG(`LOGIC MyDisplay SAMPLES `(v)) | `LOGIC MyDisplay SAMPLES 100 | Decimal numbers are output using `(value) notation. Short for SDEC_. |
| DEBUG(`LOGIC MyDisplay SAMPLES `$(v)) | `LOGIC MyDisplay SAMPLES $64 | Hex numbers are output using `$(value) notation. Short for UHEX_. |
| DEBUG(`LOGIC MyDisplay SAMPLES `%(v)) | `LOGIC MyDisplay SAMPLES %1100100 | Binary numbers are output using `%(value) notation. Short for UBIN_. |
| DEBUG(`LOGIC MyDisplay TITLE '`#(v)') | `LOGIC MyDisplay TITLE 'd' | Characters are output using `#(value) notation. |
| DEBUG(`MyDisplay `UDEC_BYTE_ARRAY_(@a,5)) | `MyDisplay 1, 2, 3, 4, 5 | Regular DEBUG commands can follow the backtick, as well. |

There are two steps to using graphical DEBUG displays. First, they must be instantiated and, second, they must be fed:

| To Use a Display: | 1st | 2nd | 3rd | 4th | Note |
| --- | --- | --- | --- | --- | --- |
| First, instantiate it. | ` | display_type | unknown_symbol | keyword(s), number(s), string(s) | Unknown_symbol becomes instance_name. |
| Then, feed it. | ` | instance_name(s) | keyword(s), number(s), string(s) |  | Multiple displays can be fed the same data. |

Note: The backtick is critical and the variables must have single quotes

debug(`BobTerm 0 '`udec(x)' 9 '`udec(y)')            ‘gives decimal value

debug(`BobTerm 0 '`uhex(x)' 9 '`uhex(y)')            ‘gives hex value

debug(`BobTerm 0 '`ubin(x)' 9 '`ubin(y)')             ‘gives binary value

debug(`BobTerm 0 'Var x = `(x)' 9 'Var y = `(y)')  ‘gives decimal value

debug(`BobTerm 0 '`$(x)' 9 '`$(y)')                         ‘gives hex value

debug(`BobTerm 0 '`%(x)' 9 '`%(y)')                       ‘gives binary value

CON _clkfreq = 10_000_000

PUB go() | i

debug(`SCOPE MyScope SIZE 254 84 SAMPLES 128)

debug(`MyScope 'Sawtooth' 0 63 64 10 %1111)

repeat

debug(`MyScope `(i & 63))

i++

waitms(50)

In the example above, a SCOPE is instantiated called MyScope that is 254 x 84 pixels and shows 128 samples. A width of 254 was chosen since samples are numbered 0..127 and I wanted them to be spaced at a constant two-pixel pitch (127 * 2 = 254). A height of 84 was chosen so that there would be 10 pixels above and below the waveform, which will have a height of 64 pixels. A channel called "Sawtooth" is defined which, for the purpose of display, has a bottom value of 0 and a top value of 63, is 64 pixels tall within that range, and is elevated 10 pixels off the bottom of the scope window. The %1111 enables top and bottom legend values and top and bottom lines. Within the REPEAT block, the SCOPE is fed a repeating pattern of 0..63 which forms the sawtooth wave. The SCOPE updates its display each time it receives a value. If there were eight channels defined, instead of just one, it would update the display on every eighth value received, drawing all eight channels.

## 13.8) Logic Analyzer Display

Logic analyzer with single and multi-bit labels, 1..32 channels, can trigger on pattern

| CON _clkfreq = 10_000_000 PUB go() ¦ i debug(`LOGIC MyLogic SAMPLES 32 'Low' 3 'Mid' 2 'High') debug(`MyLogic TRIGGER $07 $04 HOLDOFF 2) repeat debug(`MyLogic `(i & 63)) i++ waitms(25). |  |
| --- | --- |

| LOGIC Instantiation | Description | Default |
| --- | --- | --- |
| TITLE 'string' | Set the window caption to 'string'. | <none> |
| POS left top | Set the window position. | 0, 0 |
| SAMPLES 4_to_2048 | Set the number of samples to track and display. | 32 |
| SPACING 2_to_32 | Set the sample spacing. The width of the display will be SAMPLES * SPACING. | 8 |
| RATE 1_to_2048 | Set the number of samples (or triggers, if enabled) before each display update. | 1 |
| LINESIZE 1_to_7 | Set the line size. | 1 |
| TEXTSIZE 6_to_200 | Set the legend text size. Height of text determines height of logic levels. | editor text size |
| COLOR back_color {grid_color} | Set the background and grid colors *. | BLACK, GREY 4 |
| 'name' {1_to_32 {color}} | Set the first/next channel or group name, optional bit count, optional color *. | 1, default color |
| packed_data_mode | Enable packed-data mode. See description at end of this section. | <none> |
| LOGIC Feeding | Description | Default |
| TRIGGER mask match sample_offset | Trigger on (data & mask) = match. If mask = 0, trigger is disabled. | 0, 1, SAMPLES / 2 |
| HOLDOFF 2_to_2048 | Set the minimum number of samples required from trigger to trigger. | SAMPLES |
| data | Numerical data is applied LSB-first to the channels. |  |
| CLEAR | Clear the sample buffer and display, wait for new data. |  |
| SAVE {WINDOW} 'filename' | Save a bitmap file (.bmp) of either the entire window or just the display area. |  |
| CLOSE | Close the window. |  |

* Color is rgb24 value, else BLACK / WHITE or ORANGE / BLUE / GREEN / CYAN / RED / MAGENTA / YELLOW / GREY followed by an optional 0..15 for brightness (default is 8).

The LOGIC display can be used to display data that was captured at high speed. In the example below, the P2 is generating 8-N-1 serial at 333 Mbaud using a smart pin. This bit stream can be captured by the streamer. On every clock, the streamer will record the smart pin's IN signal and its output state, as read from an adjacent pin. Every time it gets four two-bit sample sets, it does an RFBYTE to save them to hub RAM, forming contiguous bytes, words, and longs. By invoking the LONGS_2BIT packed-data mode, we can have the LOGIC display unpack the two-bit sample sets from longs, yielding 16 sets per long.

## 13.9) Scope Display

SCOPE Display	Oscilloscope with 1..8 channels, can trigger on level with hysteresis

| CON _clkfreq = 100_000_000 PUB go() ¦ a, af, b, bf debug(`SCOPE MyScope) debug(`MyScope 'FreqA' -1000 1000 100 136 15 MAGENTA) debug(`MyScope 'FreqB' -1000 1000 100 20 15 ORANGE) debug(`MyScope TRIGGER 0 HOLDOFF 2) repeat a := qsin(1000, af++, 200) b := qsin(1000, bf++, 99) debug(`MyScope `(a,b)) waitus(200). |  |
| --- | --- |

| SCOPE Instantiation | Description | Default |
| --- | --- | --- |
| TITLE 'string' | Set the window caption to 'string'. | <none> |
| POS left top | Set the window position. | 0, 0 |
| SIZE width height | Set the display size (32..2048 x 32..2048) | 255, 256 |
| SAMPLES 16_to_2048 | Set the number of samples to track and display. | 256 |
| RATE 1_to_2048 | Set the number of samples (or triggers, if enabled) before each display update. | 1 |
| DOTSIZE 0_to_32 | Set the dot size in pixels for showing exact sample points. | 0 |
| LINESIZE 0_to_32 | Set the line size in half-pixels for connecting sample points. | 3 |
| TEXTSIZE 6_to_200 | Set the legend text size. | editor text size |
| COLOR back_color {grid_color} | Set the background and grid colors *. | BLACK, GREY 4 |
| packed_data_mode | Enable packed-data mode. See description at end of this section. | <none> |

| SCOPE Feeding | Description | Default |
| --- | --- | --- |
| 'name' {min {max {y_size {y_base {legend {color}}}}}} | Set first/next channel name, min value, max value, y size, y base, legend, and color *. Legend is %abcd, where %a to %d enable max legend, min legend, max line, min line. | full, no legend, default color |
| TRIGGER channel {arm_level {trigger_level {offset}}} | Set the trigger channel, arm level, trigger level, and right offset. If channel=-1, disabled. | -1, -1, 0, width / 2 |
| HOLDOFF 2_to_2048 | Set the minimum number of samples required from trigger to trigger. | SAMPLES |
| data | Numerical data is applied to the channels in ascending order. |  |
| CLEAR | Clear the sample buffer and display, wait for new data. |  |
| SAVE {WINDOW} 'filename' | Save a bitmap file (.bmp) of either the entire window or just the display area. |  |
| CLOSE | Close the window. |  |

* Color is rgb24 value, else BLACK / WHITE or ORANGE / BLUE / GREEN / CYAN / RED / MAGENTA / YELLOW / GREY followed by an optional 0..15 for brightness (default is 8).

## 13.10) Scope_XY Display

SCOPE_XY Display	XY oscilloscope with 1..8 channels, persistence of 1..512 samples, polar mode, log scale mode

| CON _clkfreq = 100_000_000 PUB go() ¦ i debug(`SCOPE_XY MyXY RANGE 500 POLAR 360 'G' 'R' 'B') repeat repeat i from 0 to 500 debug(`MyXY `(i, i, i, i+120, i, i+240)) waitms(5). |  |
| --- | --- |

| SCOPE_XY Instantiation | Description | Default |
| --- | --- | --- |
| TITLE 'string' | Set the window caption to 'string'. | <none> |
| POS left top | Set the window position. | 0, 0 |
| SIZE radius | Set the display radius in pixels. | 128 |
| RANGE 1_to_7FFFFFFF | Set the unit circle radius for incoming data | $7FFFFFFF |
| SAMPLES 0_to_512 | Set the number of samples to track and display with persistence. Use 0 for infinite persistence. | 256 |
| RATE 1_to_512 | Set the number of samples before each display update. | 1 |
| DOTSIZE 2_to_20 | Set the dot size in half-pixels for showing sample points. | 6 |
| TEXTSIZE 6_to_200 | Set the legend text size. | editor text size |
| COLOR back_color {grid_color} | Set the background and grid colors *. | BLACK, GREY 4 |
| POLAR {twopi {offset}} | Set polar mode, twopi value, and offset. For a twopi value of $100000000 or -$100000000, use 0 or -1. | $100000000, 0 |
| LOGSCALE | Set log-scale mode to magnify points within the unit circle. | <off> |
| 'name' {color} | Set the first/next channel name and optionally assign it a color *. | default color |
| packed_data_mode | Enable packed-data mode. See description at end of this section. | <none> |
| SCOPE_XY Feeding | Description | Default |
| x y | X-Y data pairs are applied to the channels in ascending order. In polar mode, x=length and y=angle. |  |
| CLEAR | Clear the sample buffer and display, wait for new data. |  |
| SAVE {WINDOW} 'filename' | Save a bitmap file (.bmp) of either the entire window or just the display area. |  |
| CLOSE | Close the window. |  |

* Color is rgb24 value, else BLACK / WHITE or ORANGE / BLUE / GREEN / CYAN / RED / MAGENTA / YELLOW / GREY followed by an optional 0..15 for brightness (default is 8).

## 13.11) TERM Display

Terminal for displaying text

| CON _clkfreq = 10_000_000 PUB go() ¦ i debug(`TERM MyTerm SIZE 9 1 TEXTSIZE 40) repeat repeat i from 50 to 60 debug(`MyTerm 1 'Temp = `(i)') waitms(500). |  |
| --- | --- |

| TERM Instantiation | Description | Default |
| --- | --- | --- |
| TITLE 'string' | Set the window caption to 'string'. | <none> |
| POS left top | Set the window position. | 0, 0 |
| SIZE columns rows | Set the number of terminal columns (1..256) and terminal rows (1..256). | 40, 20 |
| TEXTSIZE size | Set the terminal text size (6..200). | editor text size |
| COLOR text_color back_color ... | Set text-color and background-color combos #0..#3. * | default colors |
| BACKCOLOR color | Set the display background color. * | BLACK |
| UPDATE | Set UPDATE mode. The display will only be updated when fed an 'UPDATE' command. | automatic update |
| TERM Feeding | Description | Default |
| character | 0 = Clear terminal display and home cursor. 1 = Home cursor. 2 = Set column to next character value. 3 = Set row to next character value. 4 = Select color combo #0. 5 = Select color combo #1. 6 = Select color combo #2. 7 = Select color combo #3. 8 = Backspace. 9 = Tab to next 8th column. 13+10 or 13 or 10 = New line. 32..255 = Printable character. |  |
| 'string' | Print string. |  |
| CLEAR | Clear the display to the background color. |  |
| UPDATE | Update the window with the current text screen. Used in UPDATE mode. |  |
| SAVE {WINDOW} 'filename' | Save a bitmap file (.bmp) of either the entire window or just the display area. |  |
| CLOSE | Close the window. |  |

* Color is a modal value, else BLACK / WHITE or ORANGE / BLUE / GREEN / CYAN / RED / MAGENTA / YELLOW / GREY followed by an optional 0..15 for brightness (default is 8).

## 13.1_Example_WRD_DEBUG_zstr_lstr_udec_ubin_uhex

{{13.1_Example_WRD_DEBUG_zstr_lstr_udec_ubin_uhex}}

{{

ZSTR(hub_pointer)       Output zero-terminated string at hub_pointer            "Hello Propeller"

LSTR(hub_pointer,size)  Output 'size' characters of string at hub_pointer       "Hello"

UDEC(value)             Output unsigned decimal value                            0 .. 4_294_967_295

UHEX(value)             Output auto-size unsigned hex value                     $0 .. $FFFF_FFFF

UBIN(value)             Output auto-size unsigned binary value      %0..%11111111_11111111_11111111_11111111

}}

Con

_clkfreq = 200_000_000

Var

long varMsg

long varDec

long varBin

long varHex

Pub main()|x,y

varMsg := string("Hello Parallax")  'returns address of string

varDec := 5432

varBin := %1111_1111

varHex := $F123

debug("Output a Message Header")

debug(zstr(@MyString))     'output zero terminated string

debug(Lstr(@MyString,5))   'output the 5 characters at adress @MyString

debug(zstr(varMsg))

debug(udec(varDec))

debug(ubin(varBin))

debug(uhex(varHex))

DAT

MyString   Byte "Hello Propeller",0

# 14.0) Program Structure

## 14.0.1) Propeller Tool (IDE)

There are several programming tools other than the “Propeller Tool” but this document will only use the “Propeller Tool” with Parallax SPIN and PASM (Propeller Assembly Machine Language).

The “Propeller Tool” is Parallax IDE (integrated development environment) allows the programs to have code and document comments to organize the documentation:

‘         single line code comment (apostrophe)

‘’         single line document comment (two apostrophe not quotation)

{…}     Multi Line Code Comment

{{…}}   Multi Line document comment

Full Source            all code and comments

Condensed            code and Inline comments

Summary               Constants, Variables, Methods (no comments)

Documentation   Memeory usage comments and Mehtods

The “Propeller Tool” Compiles the user program from “Top Object” selected. Objects are compiled from “Libray” file folder and the current directory the “Top Object” is stored in (current working directory)

The default template file under “EDIT→PREFERNCES” can be used to customize User definitions for “New Projects”.

Edit →Find/Replace useful for editing files case is not used by the compiler this has advantages and disadvantages.

#### 14.0.1.1) Spin Propeller Tool (IDE) Compiler Directives

CON VAR PUB PRI DAT OBJ

#### 14.0.2) PASM Propeller Tool (IDE) Compilet Directives

ORG  Adjust Compile-time cog Address Pointer

FIT  validate that instruction/data fit in cog (511registers)

RES reserve next long(s) for symbol

$     current address      here   JMP #here  =   < Symbol>      JMP  #$

$+/- value     offset to current address        JMP    #$-4  or JMP #$+4

## 14.0.2) P2 Memory Organization

Cogs use 20-bit addresses for program counters (PC). This affords an execution space of up to 1MB.  Depending on the value of a cog's PC, an instruction will be fetched from either its register RAM, its lookup RAM, or the hub RAM.

| PC Address | Instruction Source | Memory Width | PC Increment |
| --- | --- | --- | --- |
| $00000..$001FF | cog register RAM | 32 bits | 1 |
| $00200..$003FF | cog lookup RAM | 32 bits | 1 |
| $00400..$FFFFF | hub RAM | 8 bits | 4 |

REGISTER EXECUTION

When the PC is in the range of $00000 and $001FF, the cog is fetching instructions from cog register RAM.  This is commonly referred to as "cog execution mode."  There is no special consideration when taking branches to a cog register address.

LOOKUP EXECUTION

When the PC is in the range of $00200 and $003FF, the cog is fetching instructions from cog lookup RAM.  This is commonly referred to as "lut execution mode."  There is no special consideration when taking branches to a cog lookup address,

HUB EXECUTION

When the PC is in the range of $00400 and $FFFFF, the cog is fetching instructions from hub RAM.  This is commonly referred to as "hub execution mode."  When executing from hub RAM, the cog employs the FIFO hardware to spool up instructions so that a stream of instructions will be available for continuous execution.  Branching to a hub address takes a minimum of 13 clock cycles.  If the instruction is not aligned to a slice, one additional clock cycle is required.

While in hub execution mode, the FIFO cannot be used for anything else. So, during hub execution these instructions must be avoided:

RDFAST / WRFAST / FBLOCK

RFBYTE / RFWORD / RFLONG / RFVAR / RFVARS

WFBYTE / WFWORD / WFLONG

XINIT / XZERO / XCONT - when the streamer mode engages the FIFO

It is not possible to execute code from hub addresses $00000 through $003FF, as the cog will instead read instructions from the cog register or lookup RAM as indicated above.

## 14.0.3) COG RAM

Each cog has a primary 512 x 32-bit dual-port RAM, which can be used in multiple ways:

- Direct/Register access

- As a source of program instructions

GENERAL PURPOSE REGISTERS

RAM registers $000 through $1EF are general-purpose registers for code and data usage.

DUAL-PURPOSE REGISTERS

RAM registers $1F0 through $1F7 may either be used as general-purpose registers, or may be used as special-purpose registers if their associated functions are enabled.

$1F0		RAM / IJMP3		interrupt call   address for INT3

$1F1		RAM / IRET3		interrupt return address for INT3

$1F2		RAM / IJMP2		interrupt call   address for INT2

$1F3		RAM / IRET2		interrupt return address for INT2

$1F4		RAM / IJMP1		interrupt call   address for INT1

$1F5		RAM / IRET1		interrupt return address for INT1

$1F6		RAM / PA		CALLD-imm return, CALLPA parameter, or LOC address

$1F7		RAM / PB		CALLD-imm return, CALLPB parameter, or LOC address

SPECIAL-PURPOSE REGISTERS

Each cog contains 8 special-purpose registers that are mapped into the RAM register address space from $1F8 to $1FF.  In general, when specifying an address between $1F8 and $1FF, the instruction is accessing a special-purpose register, not just the underlying RAM.

$1F8		PTRA			pointer A to hub RAM

$1F9		PTRB			pointer B to hub RAM

$1FA		DIRA			output enables for P31..P0

$1FB		DIRB			output enables for P63..P32

$1FC		OUTA			output states for P31..P0

$1FD		OUTB			output states for P63..P32

$1FE		INA *			input states for P31..P0

$1FF		INB **			input states for P63..P32

* also debug interrupt call address

** also debug interrupt return address

LOOKUP RAM

Each cog has a secondary 512 x 32-bit dual-port RAM, which can be used in multiple  ways:

- Load/Store access

- As a source or destination for the streamer hardware

- As a lookup table for bytecode execution

- As a data source for smart pins

- As a "RAM sharing" mechanism between paired cogs

- As a source of program instructions (see COGS > INSTRUCTION MODES > LOOKUP EXECUTION)

NOTE: The term "lookup" (and "lut", which is short for "look-up table") is due to historical usage in the original Propeller microcontroller.  This RAM can still be used in a "lookup" context, but can also be used for many other purposes, as indicated above.

PASM Communication Registers

Each of these cog registers can be referenced by name PR0-PR7

PR0 $1D8

PR1 $1D9

PR2 $1DA

PR3 $1DB

PR4 $1DC

PR5 $1DD

PR6 $1DE

PR7 $1DF

## 14.0.4) Program Blocks

Spin2 programs are built from one or more objects. Objects are files which contain at least one public method, along with optional constants, child objects, variables, additional methods, and data. Objects are assembled together into a top-level object with an internal hierarchy of sub-objects. Each object instance, at run-time, gets its own set of variables, as defined by the object, to maintain its unique operating state.

Different parts of an object are declared within blocks, which all begin with 3-letter block identifiers.

The compiler can actually generate PASM-only programs, as well as Spin2+PASM programs, depending upon which blocks are present in the .spin2 file.

| Block Identifier | Block Contents | Spin2+PASM Programs | PASM-only Programs |
| --- | --- | --- | --- |
| CON | Constant declarations (CON is the initial/default block type) | Permitted | Permitted |
| OBJ | Child-object instantiations | Permitted | Not Allowed |
| VAR | Variable declarations | Permitted | Not Allowed |
| PUB | Public method for use by the parent object and within this object | Required | Not Allowed |
| PRI | Private method for use within this object | Permitted | Not Allowed |
| DAT | Data declarations, including PASM code | Permitted | Required |

Here are some minimal Spin2 and PASM-only programs. If you copy and paste these into PNut.exe, you can hit F10 to run them.

| Minimal Spin2 Program | PUB MinimalSpin2Program() 'first PUB method executes REPEAT PINWRITE(63..56, GETRND()) 'write a random pattern to P63..P56 WAITMS(100) 'wait 1/10th of a second, loop |
| --- | --- |
| Minimal PASM Program | DAT ORG 'start PASM at hub $00000 for cog $000 loop DRVRND #56 ADDPINS 7 'write a random pattern to P63..P56 WAITX ##clkfreq_/10 'wait 1/10th of a second, loop JMP #loop |

Here is a Spin2 program which contains every block type.

| All-Block Spin2 Program | CON _clkfreq = 297_000_000 'set clock frequency OBJ vga : "VGA_640x480_text_80x40" 'instantiate vga object VAR time, i 'declare object-wide variables PUB go() 'this first public method executes, cog stops after vga.start(8) 'start vga on base pin 8 SEND := @vga.print 'establish SEND pointer SEND(4, $004040, 5, $00FFFF) 'set light cyan on dark cyan time := GETCT() 'capture time i := @text 'print file to vga screen REPEAT @textend-i SEND(byte[i++]) time := GETCT() - time 'capture time delta in clock cycles time := MULDIV64(time, 1_000_000, clkfreq) 'get time delta in microseconds SEND(12, "Time elapsed during printing was ", dec(time), " microseconds.") 'print time delta PRI dec(value) ¦ flag, place, digit 'private method prints decimals, three local variables flag~ 'reset digit-printed flag place := 1_000_000_000 'start at the one-billion's place and work downward REPEAT IF flag ¦¦= (digit := value / place // 10) ¦¦ place == 1 'print a digit? SEND("0" + digit) 'yes IF LOOKDOWN(place : 1_000_000_000, 1_000_000, 1_000) 'also print a comma? SEND(",") 'yes WHILE place /= 10 'next place, done? DAT text FILE "VGA_640x480_text_80x40.txt" 'include raw file data for printing textend |
| --- | --- |

## 14.1) CON Block

Symbolic constants are global to the object. If an object reference is declared in another object constants Of the child object can be referenced using  :

OBJ

Num : “Numbers”  -     Num.Constant_Symbol

Syntax 1    Symbol = Expression   (constants)

Symbol –desired name of constant

Expression –any valid integer for floating point, or constant algebraic expression

Note: Constant can be used in algebraic expression but must be previously defined.

- Symbolic constants resolve to 32-bit values.

- Symbolic constants can be assigned using '=' or by just expressing their names in an enumeration list.

- Symbolic constants can be referenced by every block within the file, including CON blocks.

- Symbolic constants can be referenced by the parent object's methods via 'objectname.constantname' syntax.

- If a decimal point is present, the value is encoded in IEEE-754 single-precision format.

CON   ‘Direct Assignment

EnableFlow = 8            'single assignments LONG data type

x = 5, y = -5, z = 1      'comma-separated assignments

HalfPi = 1.5707963268     'single-precision float values

xy = x*y

Syntax 2  #Symbol   (enumerated constants

CON   ‘Direct Assignment

#3, a,b,c       'a=3 b=4 c=5precision float values

### 14.1.1) CON  Compiler Enumeration Step option #conVar[step]

#0,a,b,c,d           'a=0, b=1, c=2, d=3    (start=0, step=1)

#1,e,f,g,h           'e=1, f=2, g=3, h=4      (start=1, step=1)

#4[2],i,j,k,l        'i=4, j=6, k=8, l=10       (start=4, step=2)

#-1[-1],m,n,p    'm=-1, n=-2, p=-3         (start=-1, step=-1)

#true,on,off                  'true = $FFFF_FFFF ,on = $FFFF_FFFF,off = %0

#true +true,on,off       'true = $FFFF_FFFE ,on = $FFFF_FFFE,off = $FFFF_FFFF

### 14.1.2) Vertical Constant Enumeration

Note: Constant[Step Increment] changes the step value for next constant if not defined the default step is 1 for example conVar[3] would increase next step value by 3

#16                'start=16, step=1   set enumeration

q                    'q=16

r[0]                 'r=17   ([0] is a step multiplier)

s                      's=17

t                     't=18

u[2]                'u=19   ([2] is a step multiplier)

v                     'v=21

w                    'w=22

#16[2]           'start=16 step=2 set enumeration

a                     'a= 16

b                     'b= 18

c                     'c= 20

d                     'd= 22

e                     'e= 24

### 14.1_Example_WRD_Constant_Enumeration

The #TRUE directive causes the enumeration constant value to be evaluated and then assigned to constants.

The case of “true or True or tRUe” all  are legal and will resolve to TRUE. Case does not matter in spin2.

## 14.2) OBJ Block

Symbol<[count]>: “Object_Name”

Symbol –name for Object

count –number of objects to be made

Object_Name –object filename without extension

- Up to 32 different child objects can be incorporated into a parent object.

- Child objects can be instantiated singularly or in arrays of up to 255.

- Up to 1024 child objects are allowed per parent object.

OBJ

NUM: “Numbers”

Term: “Terminal”

Public Methods are accessed alias dot method:  NUM.ToStr

| OBJ Child-Object Instantiations | OBJ vga : "VGA_Driver" 'instantiate "VGA_Driver.spin2" as "vga" mouse : "USB_Mouse" 'instantiate "USB_Mouse.spin2" as "mouse" v[16] : "VocalSynth" 'instantiate an array of 16 objects '..v[0] through v[15] |
| --- | --- |

### 14.2.1) Including Objects during Compiling

The example used as a child object is the “14.2_Example_WRD_FullDuplexSerial.spin2”. This object is used to communicate with PST(Parallax Serial Terminal) which is launched from the “Propeller Tool”. The bauda rate is 230400 bps and the com port is to match the loading for programs  over pins :

RX1     = 63  { I }               rogramming / debug

TX1     = 62  { O }

“14.2_Example_WRD_FullDuplesSerial_Demo.spin2” declares the Object “term” to be compiled with the program:

obj

term : "14.2_Example_WRD_FullDuplexSerial"                                  'serial IO for terminal

This is similar to an include file that a “C” program would use. The “spin2” extension is not included in the object declaration. The “Propeller Tool” checks the library folder and the folder containing the calling program for a file with the name and spin2 extension. Suggest keep all objects in the same folder as the parent program.

### 14.2.2) Accessing Constants and Pub Method from Objects

From within a parent-object method, a child-object method can be called by using the syntax:

object_name.method_name({any_parameters})

term.dec(123)               'Send character string representing decimal value 123

term.fhex($FFFF,8)       'Send string representing hex value $FFFF padded with 0 for 8 digits 0000FFFF

From within a parent-object method, a child-object constant can be referenced by using the syntax:

object_name.constant_name

term.tx(term.LF)     'Send Constant in object term for Line Feed term.LF =10

term.tx(term.CR)     'Send Constant in object term for Carriage Return term.CR =11

### 14.2_Example_WRD_FullDuplexSerial_Demo

{{14.2_Example_WRD_FullDuplexSerial_Demo}}

{{ =====================================================================================

''   File....... P2USB-format_strings_demo

''   Based on jm_formatted_strings_test.spin2

''   Objects:  term : "jm_fullduplexserial"

''   Secondary Object: nstr : "jm_nstr"

''   Purpose.... Demonstrate Propeller II Serial

''   Author..... WRD Copyright (c) 2020 Bob Drury

''               -- see below for terms of use

''   E-mail..... bob_drury@hotmail.com

''   Rev01...... 2020 Jan 5

=====================================================================================

Object term: "jm_fullduplexserial"

Public Function Call

term.null()

term.tstart(baud) : result

term.start(rxpin, txpin, mode, baud) : result

term.stop()

term.rx() : b

term.rxcheck() : b

term.rxtime(ms) : b

term.rxtix(tix) : b

term.available() : count

term.rxflush()

term.tx(b)        "send single byte b to PST"

term.txn(b, n)    "send single byte b for n times to PST"

term.str(p_str)   '

term.substr(p_str, len)

term.padstr(p_str, width, pad)

term.txflush()

term.fstr0(p_str)    "

term.fstr1(p_str, arg1)

term.fstr2(p_str, arg1, arg2)

term.fstr3(p_str, arg1, arg2, arg3)

term.fstr4(p_str, arg1, arg2, arg3, arg4)

term.fstr5(p_str, arg1, arg2, arg3, arg4, arg5)

term.fstr6(p_str, arg1, arg2, arg3, arg4, arg5, arg6)

term.format(p_str, p_args)

term.lower(c) : result

term.fmt_number(value, base, digits, width, pad)

term.dec(value)                 "Send string of characters representing decimal value to PST"

term.fdec(value, digits)    "Send characters representing decimal value padded with 0 to digits PST"

term.jdec(value, digits, width, pad) "Send  characters 8 digits with width padded to complete with pad

term.dpdec(value, dp)      "Send value as decimal with decimal point of dp size

term.jdpdec(value, dp, width, pad)

term.hex(value)

term.fhex(value, digits)

term.jhex(value, digits, width, pad)

term.oct(value)

term.foct(value, digits)

term.joct(value, digits, width, pad)

term.qrt(value)

term.fqrt(value, digits)

term.jqrt(value, digits, width, pad)

term.bin(value)

term.fbin(value, digits)

term.jbin(value, digits, width, pad)

RX1     = 63  { I }  Pin for serial input

TX1     = 62  { O }  Pin for serial output

Parallax Serial Terminal

term.HOME     =  1

term.CRSR_XY  =  2

term.CRSR_LF  =  3

term.CRSR_RT  =  4

term.CRSR_UP  =  5

term.CRSR_DN  =  6

term.BELL     =  7

term.BKSP     =  8

term.TAB      =  9

term.LF       = 10

term.CLR_EOL  = 11

term.CLR_DN   = 12

term.CR       = 13

term.CRSR_X   = 14

term.CRSR_Y   = 15

term.CLS      = 16

Formatted Arguments

%w.pf       print argument as decimal width decimal point

%[w[.p]]d   print argument as decimal

%[w[.p]]u   print argument as unsigned decimal

%[w[.p]]x   print argument as hex

%[w[.p]]o   print argument as octal

%[w[.p]]q   print argument as quarternary

%[w[.p]]b   print argument as binary

%[w]s       print argument as string

%[w]c       print argument as character (

-- w is field width

* positive w causes right alignment in field

* negative w causes left alignment in field

-- %ws aligns s in field (may truncate)

-- %wc prints w copies of c

-- p is precision characters

* number of characters to use, aligned in field

-- prefix with 0 if needed to match p

-- for %w.pf, p is number of digits after decimal point

Escaped characters

\\          backslash char

\%          percent char

\q          double quote

\b          backspace

\t          tab (horizontal)

\n          new line (vertical tab)

\r          carriage return

\nnn        arbitrary ASCII value (nnn is decimal)

}}

con { timing }

_clkfreq = 200_000_000                                       ' set system clock

BR_TERM  = 230_400                                            ' terminal baud rate

'set Parallax Serial Terminal 230_400 Baud

con { fixed io pins }

RX1     = 63  { I }                                           ' programming / debug

TX1     = 62  { O }

FS_CS   = 61  { O }                                          ' flash storage

FS_SCLK = 60  { O }

FS_MOSI = 59  { O }

FS_MISO = 58  { I }

SD_SCLK = 61  { O }                                        ' usd card storage

SD_CS   = 60  { O }

SD_MOSI = 59  { O }

SD_MISO = 58  { I }

SDA1    = 57  { IO }                                          ' i2c (optional)

SCL1    = 56  { IO }

con

BUF_SIZE = 32                                                'input character buffer size

obj

term : "14.2_Example_WRD_FullDuplexSerial"                                  ' * serial IO for terminal

var

Byte  buffer[BUF_SIZE]

Long x00,x01,x02,x03,x04,x05

Byte C01,C02,C03,C04,C05

Long xFloat

dat

Device        byte    "P2X8C4M64P\r", 0

Arg00         byte     "Arg00",0

Arg01         byte     "Arg01",0

Arg02         byte     "Arg02",0

Arg03         byte     "Arg03",0

Arg04         byte     "Arg04",0

Arg05         byte     "Arg05",0

Char01        byte     "A"

Char02        byte     "B"

Char03        byte     "C"

Char04        byte     "D"

Char05        byte     "E"

pub main() | x, y

setup()

wait_for_terminal(true)

term.fstr1(string("%s Formatted Strings Demo\r"), @Device)

term.fstr1(string("%033c\r\r"), "-")

term.fstr0(string("Enter to run Program: "))

get_str(BUF_SIZE-2)

term.fstr1(string("\r\rHello, %s, let me show you some \rformatted strings...\r\r"), @buffer)

waitms(1000)

repeat 10

term.tx("a")

term.tx(term.LF)     'Send Constant in object term for Line Feed term.LF

term.tx(term.CR)     'Send Constant in object term for Carriage Return term.CR

term.str(string("Hello Universe"))  'Send zero inline terminated string

term.tx(term.LF)

term.tx(term.CR)

waitms(1000)

term.dec(123)           'Send character string representing decimal value 123

term.tx(term.LF)

term.tx(term.CR)

term.fdec(123,8)        'Send c string representing decimal value 123 padded with 0 for 8 digits 00000123

term.tx(term.LF)

term.tx(term.CR)

term.jdec(-123,8,11,"*")    'Total length of 11 with 8 digits padded with "*"

term.tx(term.LF)

term.tx(term.CR)

term.dpdec(12300, 2)         'Send character string representing decimal with 2 decimal place

term.tx(term.LF)

term.tx(term.CR)

term.jdpdec(54312, 2, 8, "*") 'Send  string valuewith dp decimal total length width padding with pad

term.tx(term.LF)

term.tx(term.CR)

term.str(@Device)                  'Send zero terminated string ignors format escape sequence

term.tx(term.LF)

term.tx(term.CR)

term.hex(255)                          'Send character string representing hex value for decimal 255

term.tx(term.LF)

term.tx(term.CR)

term.hex($FF)                          'Send character string representing hex value $FF

term.tx(term.LF)

term.tx(term.CR)

term.fhex($FFFF,8)       'Send string representing hex value $FFFF padded with 0 for 8 digits 0000FFFF

term.tx(term.LF)

term.tx(term.CR)

term.jhex($256,8,11,"*")     'Convert Decimal to hex of 11 with 8 digits padded with "*"

term.tx(term.LF)

term.tx(term.CR)

term.oct(15)                             'Convert Decimal to oct and send character string

term.tx(term.LF)

term.tx(term.CR)

term.oct($FF)                            'Convert Hex to Oct and send character string

term.tx(term.LF)

term.tx(term.CR)

term.foct($3FF,5)                  'Convert Hex to Oct and send character string ($3FF = 1023 = 1777 octal)

term.tx(term.LF)

term.tx(term.CR)

term.joct($3FF,8,11,"*")         'Convert Hex to Oct send  string of 11 with 8 digits padded with "*"

term.tx(term.LF)

term.tx(term.CR)

term.qrt(85)                               'Convert 85 decimal to quartenary 1111 Base 4

term.tx(term.LF)

term.tx(term.CR)

term.fqrt($55,8)                        'Convert hex $55 to quartenary 1111 Base 4

term.tx(term.LF)

term.tx(term.CR)

term.jqrt($55,8,11,"*")          'Convert hex $55 to qrt send  string of 11 with 8 digits padded with "*"

term.tx(term.LF)

term.tx(term.CR)

term.bin(16)                'Convert 16 decimal to binary 10000 Base 2

term.tx(term.LF)

term.tx(term.CR)

term.fbin($10,8)         'Convert hex $10 to binary 10000 Base2 with 0 padded 8 digits

term.tx(term.LF)

term.tx(term.CR)

term.jbin($10,8,11,"*")  'Convert hex $F to binary send character string of 11 with 8 digits padded with "*"

term.tx(term.LF)

term.tx(term.CR)

waitms(1000)

C01 :=  65                'ASCII Dec 65 = A

term.tx(C01)          'Send Var C01

term.tx(term.LF)    'Send Constant in object term for Line Feed term.LF

C01 := 66                 'ASCII Dec 66 = B

term.tx(C01)          'Send Var C01

term.tx(term.LF)   'Send Constant in object term for Line Feed term.LF

term.tx(term.CR)   'Send Constant in object term for Carriage Return term.CR

term.tx("C")           'Send Character C

term.tx(10)            'Send Number 10 to PST which causes LF

term.tx(13)             'Send Number 13 to PST which causes CR

term.txn("∞",45)   'Send character A for 45 times

term.fstr0(string("\r"))       'Send CR and LF

term.tx(term.LF)                  'Send Constant in object term for Line Feed term.LF

term.tx(term.CR)                 'Send Constant in object term for Carriage Return term.CR

term.fstr0(string("\176"))  'Send ASCII degree symbol° Use character chart to print

term.tx(10)                            'Send Number 10 to PST which causes LF

term.tx(13)                            'Send Number 13 to PST which causes CR

waitms(1000)

term.fstr0(@Device)            'Send string from Dat Address @Device

term.fstr1(string("fstr01  %s\r"),@Arg01)

term.fstr2(string("fstr02  %s   %s\r"),@Arg01,@Arg02)

term.fstr3(string("fstr03  %s   %s  %s\r"),@Arg01,@Arg02,@Arg03)

term.fstr4(string("fstr04  %s   %s  %s   %s\r"),@Arg01,@Arg02,@Arg03,@Arg04)

term.fstr5(string("fstr05  %s   %s  %s   %s   %s\r"),@Arg01,@Arg02,@Arg03,@Arg04,@Arg05)

waitms(1000)

x01 := 101

x02 := 102

x03 := 103

x04 := 104

x05 := 105

xFloat := 12.345

term.fstr1(string("xFloat= %f\r"),xFloat)

term.fstr1(string("x01= %d\r"),x01)

term.fstr2(string("x01= %d x02= %d\r"),x01,x02)

term.fstr3(string("x01= %d x02= %d x03= %d\r"),x01,x02,x03)

term.fstr4(string("x01= %d x02= %d x03= %d x04= %d\r"),x01,x02,x03,x04)

term.fstr5(string("x01= %d x02= %d x04= %d x04= %d x05= %d\r"),x01,x02,x03,x04,x05)

waitms(1000)

term.fstr1(string("Char01= %c\r"),Char01)

term.fstr2(string("Char01= %c Char02= %c\r"),Char01,Char02)

term.fstr3(string("Char01= %c Char02= %c Char03= %c\r"),Char01,Char02,Char03)

term.fstr4(string("Char01= %c Char02= %c Char03= %c Char04= %c\r"),Char01,Char02,Char03,Char04)

term.fstr5(string("Char01= %c Char02= %c Char04= %c Char04= %c Char05= %c\r"),Char01,Char02,Char03,Char04,Char05)

waitms(1000)

C01 := "A"

C02 := "B"

C03 := "C"

C04 := "D"

C05 := "E"

term.fstr1(string("C01= %c\r"),C01)

term.fstr2(string("C01= %c C02= %c\r"),C01,C02)

term.fstr3(string("C01= %c C02= %c C03= %c\r"),C01,C02,C03)

term.fstr4(string("C01= %c C02= %c C03= %c C04= %c\r"),C01,C02,C03,C04)

term.fstr5(string("C01= %c C02= %c C04= %c C04= %c C05= %c\r"),C01,C02,C03,C04,C05)

waitms(1000)

term.fstr1(string("%040c\r"),"-")

term.fstr1(string("%040c\r"),"*")

term.fstr1(string("%040c\r"),176)   '176 is the decimal ascii for degree with PST

term.fstr1(string("x01 float= %13.3f\r"),x01)

term.fstr2(string("x01 in hex= %4x --> x01 in dec= %4d\r"),x01,x01)

term.fstr2(string("%d\176C --> %d\176F\r\r"),x01,x01*9/5+32)

term.fstr2(string("%-10d %13.3f\r"), x01, x01)

waitms(1000)

repeat x from 123 to 255

term.fstr1(string("%040c\r"),x)

waitms(3000)

pub get_str(maxlen) : len | k

bytefill(@buffer, 0, BUF_SIZE)                                ' clear input buffer

term.rxflush()                                                ' clear trash from terminal

repeat

k := term.rx()                                              ' wait for a character

case k

31..127 :                                                 ' if valid

if (len < maxlen)                                       '  and room

buffer[len++] := k                                    '   add to buffer

term.BKSP :

if (len > 0)                                            ' if character(s) in buffer

buffer[--len] := 0                                    '  backup and erase last

term.CR :

buffer[len] := 0                                        ' terminate string

return                                                  '  and return to caller

pub wait_for_terminal(clear)

term.rxflush()

term.rx()                                                     ' wait for keypress

if (clear)

term.tx(term.CLS)

pub setup()

term.start(RX1, TX1, %0000, BR_TERM)                          ' start terminal serial

con { license }

{{

MIT License}}

### 14.2_Example_WRD_FullDuplexSerial

{{14.2_Example_WRD_FullDuplexSerial}}

{{ =====================================================================================

''

''   File....... jm_fullduplexserial.spin2

''   Purpose.... Buffered serial communications using smart pins

''               -- mostly matches FullDuplexSerial from P1

''               -- does NOT support half-duplex communications using shared RX/TX pin

''   Authors.... Jon McPhalen

''               -- based on work by Chip Gracey

''               -- see below for terms of use

''   E-mail..... jon.mcphalen@gmail.com

''   Started....

''   Updated.... 06 SEP 2020

''

=====================================================================================

Note: Buffer size no longer has to be power-of-2 integer.

Note: The dec(), bin(), and hex() methods will no longer require the digits parameter as

in older versions of FullDuplexSerial. Use fdec(), fbin(), and fhex() for code that

requires a specific field width.

The smart pin uarts use a 16-bit value for baud timing which can limit low baud rates for

some system frequencies -- beware of these limits when connecting to older devices.

Baud     20MHz    40MHz    80MHz    100MHz    200MHz    300MHz

------    -----    -----    -----    ------    ------    ------

300       No       No       No        No        No        No

600      Yes       No       No        No        No        No

1200      Yes      Yes       No        No        No        No

2400      Yes      Yes      Yes       Yes        No        No

4800      Yes      Yes      Yes       Yes       Yes       Yes

}}

con { fixed io pins }

RX1      = 63  { I }                                          ' programming / debug

TX1      = 62  { O }

SF_CS    = 61  { O }                                          ' serial flash

SF_SCK   = 60  { O }

SF_SDO   = 59  { O }

SF_SDI   = 58  { I }

con { pst formatting }

HOME     =  1

CRSR_XY  =  2

CRSR_LF  =  3

CRSR_RT  =  4

CRSR_UP  =  5

CRSR_DN  =  6

BELL     =  7

BKSP     =  8

TAB      =  9

LF       = 10

CLR_EOL  = 11

CLR_DN   = 12

CR       = 13

CRSR_X   = 14

CRSR_Y   = 15

CLS      = 16

Con {FullDuplexSerial}

BUF_SIZE = 64

obj

nstr : "14.2_Example_WRD_Num_To_Str"                                              ' number-to-string

var

long  cog                                                     ' cog flag/id

long  rxp                                                     ' rx smart pin

long  txp                                                     ' tx smart pin

long  rxhub                                                   ' hub address of rxbuf

long  txhub                                                   ' hub address of txbuf

long  rxhead                                                  ' rx head index

long  rxtail                                                  ' rx tail index

long  txhead                                                  ' tx head index

long  txtail                                                  ' tx tail index

long  txdelay                                                 ' ticks to transmit one byte

byte  rxbuf[BUF_SIZE]                                         ' buffers

byte  txbuf[BUF_SIZE]

byte  pbuf[80]                                                ' padded strings

pub null()

'' This is not a top-level object

pub tstart(baud) : result

'' Start FDS with default pins/mode for terminal (e.g., PST)

return start(RX1, TX1, %0000, baud)

pub start(rxpin, txpin, mode, baud) : result | baudcfg, spmode

'' Start simple serial coms on rxpin and txpin at baud

'' -- rxpin... receive pin (-1 if not used)

'' -- txpin... transmit pin (-1 if not used)

'' -- mode.... %0xx1 = invert rx

''             %0x1x = invert tx

''             %01xx = open-drain/open-source tx

stop()

if (rxpin == txpin)                                           ' pin must be unique

return false

longmove(@rxp, @rxpin, 2)                                     ' save pins

rxhub := @rxbuf                                               ' point to buffers

txhub := @txbuf

txdelay := clkfreq / baud * 11                                ' tix to transmit one byte

baudcfg := muldiv64(clkfreq, $1_0000, baud) & $FFFFFC00       ' set bit timing

baudcfg |= (8-1)                                              ' set bits (8)

if (rxp >= 0)                                                 ' configure rx pin if used

spmode := P_ASYNC_RX

if (mode.[0])

spmode |= P_INVERT_IN

pinstart(rxp, spmode, baudcfg, 0)

if (txp >= 0)                                                 ' configure tx pin if used

spmode := P_ASYNC_TX | P_OE

case mode.[2..1]

%01 : spmode |= P_INVERT_OUTPUT

%10 : spmode |= P_HIGH_FLOAT                              ' requires external pull-up

%11 : spmode |= P_INVERT_OUTPUT | P_LOW_FLOAT             ' requires external pull-down

pinstart(txp, spmode, baudcfg, 0)

cog := coginit(COGEXEC_NEW, @uart_mgr, @rxp) + 1              ' start uart manager cog

return cog

pub stop()

'' Stop serial driver

'' -- frees a cog if driver was running

if (cog)                                                      ' cog active?

cogstop(cog-1)                                              '  yes, shut it down

cog := 0                                                    '  and mark stopped

longfill(@rxp,  -1, 2)                                        ' reset object globals

longfill(@rxhub, 0, 7)

pub rx() : b

'' Pulls byte from receive buffer if available

'' -- will wait if buffer is empty

repeat while (rxtail == rxhead)                               ' hold while buffer empty

b := rxbuf[rxtail]                                            ' get a byte

if (++rxtail == BUF_SIZE)                                     ' update tail pointer

rxtail := 0

pub rxcheck() : b

'' Pulls byte from receive buffer if available

'' -- returns -1 if buffer is empty

if (rxtail <> rxhead)                                         ' something in buffer?

b := rxbuf[rxtail]                                          ' get it

if (++rxtail == BUF_SIZE)                                   ' update tail pointer

rxtail := 0

else

b := -1                                                     ' mark no byte available

pub rxtime(ms) : b | mstix, t

'' Wait ms milliseconds for a byte to be received

'' -- returns -1 if no byte received, $00..$FF if byte

mstix := clkfreq / 1000

t := getct()

repeat until ((b := rxcheck()) >= 0) || (((getct()-t) / mstix) >= ms)

pub rxtix(tix) : b | t

'' Waits tix clock ticks for a byte to be received

'' -- returns -1 if no byte received

t := getct()

repeat until ((b := rxcheck()) >= 0) || ((getct()-t) >= tix)

pub available() : count

'' Returns # of bytes waiting in rx buffer

if (rxtail <> rxhead)                                         ' if byte(s) available

count := rxhead - rxtail                                    ' get count

if (count < 0)

count += BUF_SIZE                                         ' fix for wrap around

pub rxflush()

'' Flush receive buffer

repeat while (rxcheck() >= 0)

pub tx(b) | n

'' Move byte into transmit buffer if room is available

'' -- will wait if buffer is full

repeat

n := txhead - txtail                                        ' bytes in buffer

if (n < 0)                                                  ' fix for index wrap-around

n += BUF_SIZE

if (n < BUF_SIZE-1)

quit

txbuf[txhead] := b                                            ' move to buffer

if (++txhead == BUF_SIZE)                                     ' update head pointer

txhead := 0

pub txn(b, n)

'' Emit byte n times

repeat n

tx(b)

pub str(p_str)

'' Emit z-string at p_str

repeat (strsize(p_str))

tx(byte[p_str++])

pub substr(p_str, len) | b

'' Emit len characters of string at p_str

'' -- aborts if end of string detected

repeat len

b := byte[p_str++]

if (b > 0)

tx(b)

else

quit

pub padstr(p_str, width, pad)

'' Emit p_str as padded field of width characters

'' -- pad is character to use to fill out field

'' -- positive width causes right alignment

'' -- negative width causes left alignment

str(nstr.padstr(p_str, width, pad))

pub txflush()

'' Wait for transmit buffer to empty

'' -- will delay one byte period after buffer is empty

repeat until (txtail == txhead)                               ' let buffer empty

waitct(getct() + txdelay)                                     ' delay for last byte

pub fstr0(p_str)

'' Emit string with formatting characters.

format(p_str, 0)

pub fstr1(p_str, arg1)

'' Emit string with formatting characters and one argument.

format(p_str, @arg1)

pub fstr2(p_str, arg1, arg2)

'' Emit string with formatting characters and two arguments.

format(p_str, @arg1)

pub fstr3(p_str, arg1, arg2, arg3)

'' Emit string with formatting characters and three arguments.

format(p_str, @arg1)

pub fstr4(p_str, arg1, arg2, arg3, arg4)

'' Emit string with formatting characters and four arguments.

format(p_str, @arg1)

pub fstr5(p_str, arg1, arg2, arg3, arg4, arg5)

'' Emit string with formatting characters and five arguments.

format(p_str, @arg1)

pub fstr6(p_str, arg1, arg2, arg3, arg4, arg5, arg6)

'' Emit string with formatting characters and six arguments.

format(p_str, @arg1)

pub format(p_str, p_args) | idx, c, asc, field, digits

'' Emit formatted string with escape sequences and embedded values

'' -- p_str is a pointer to the format control string

'' -- p_args is pointer to array of longs that hold field values

''    * field values can be numbers, characters, or pointers to strings

idx := 0                                                     ' value index

repeat

c := byte[p_str++]

if (c == 0)

return

elseif (c == "\")

c := lower(byte[p_str++])

if (c == "\")

tx("\")

elseif (c == "%")

tx("%")

elseif (c == "q")

tx(34)

elseif (c == "b")

tx(BKSP)

elseif (c == "t")

tx(TAB)

elseif (c == "n")

tx(LF)

elseif (c == "r")

tx(CR)

elseif ((c >= "0") and (c <= "9"))

--p_str

p_str, asc, _ := get_nargs(p_str)

if ((asc >= 0) and (asc <= 255))

tx(asc)

elseif (c == "%")

p_str, field, digits := get_nargs(p_str)

c := lower(byte[p_str++])

if (c == "d")

str(nstr.fmt_number(long[p_args][idx++], "d", digits, field, " "))

elseif (c == "u")

str(nstr.fmt_number(long[p_args][idx++], "u", digits, field, " "))

elseif (c == "f")

str(nstr.fmt_number(long[p_args][idx++], "f", digits, field, " "))

elseif (c == "b")

str(nstr.fmt_number(long[p_args][idx++], "b", digits, field, " "))

elseif (c == "q")

str(nstr.fmt_number(long[p_args][idx++], "q", digits, field, " "))

elseif (c == "o")

str(nstr.fmt_number(long[p_args][idx++], "o", digits, field, " "))

elseif (c == "x")

str(nstr.fmt_number(long[p_args][idx++], "x", digits, field, " "))

elseif (c == "s")

str(nstr.padstr(long[p_args][idx++], field, " "))

elseif (c == "c")

txn(long[p_args][idx++], (abs field) #> 1)

else

tx(c)

pub lower(c) : result

if ((c >= "A") && (c <= "Z"))

c += 32

return c

pri get_nargs(p_str) : p_str1, val1, val2 | c, sign

'' Parse one or two numbers from string in n, -n, n.n, or -n.n format

'' -- dpoint separates values

'' -- only first # may be negative

'' -- returns pointer to 1st char after value(s)

c := byte[p_str]                                              ' check for negative on first value

if (c == "-")

sign := -1

++p_str

else

sign := 0

repeat                                                        ' get first value

c := byte[p_str++]

if ((c >= "0") && (c <= "9"))

val1 := (val1 * 10) + (c - "0")

else

if (sign)

val1 := -val1

quit

if (c == ".")                                                 ' if dpoint

repeat                                                      '  get second value

c := byte[p_str++]

if ((c >= "0") && (c <= "9"))

val2 := (val2 * 10) + (c - "0")

else

quit

p_str1 := p_str - 1                                           ' back up to non-digit

pub fmt_number(value, base, digits, width, pad)

'' Emit value converted to number in padded field

'' -- value is converted using base as radix

''    * 99 for decimal with digits after decimal point

'' -- digits is max number of digits to use

'' -- width is width of final field (max)

'' -- pad is character that fills out field

str(nstr.fmt_number(value, base, digits, width, pad))

pub dec(value)

'' Emit value as decimal

str(nstr.itoa(value, 10, 0))

pub fdec(value, digits)

'' Emit value as decimal using fixed # of digits

'' -- may add leading zeros

str(nstr.itoa(value, 10, digits))

pub jdec(value, digits, width, pad)

'' Emit value as decimal using fixed # of digits

'' -- aligned in padded field (negative width to left-align)

'' -- digits is max number of digits to use

'' -- width is width of final field (max)

'' -- pad is character that fills out field

str(nstr.fmt_number(value, "d", digits, width, pad))

pub dpdec(value, dp)

'' Emit value as decimal with decimal point

'' -- dp is number of digits after decimal point

str(nstr.dpdec(value, dp))

pub jdpdec(value, dp, width, pad)

'' Emit value as decimal with decimal point

'' -- aligned in padded field (negative width to left-align)

'' -- dp is number of digits after decimal point

'' -- width is width of final field (max)

'' -- pad is character that fills out field

str(nstr.fmt_number(value, "f", dp, width, pad))

pub hex(value)

'' Emit value as hexadecimal

str(nstr.itoa(value, 16, 0))

pub fhex(value, digits)

'' Emit value as hexadecimal using fixed # of digits

str(nstr.itoa(value, 16, digits))

pub jhex(value, digits, width, pad)

'' Emit value as quarternary using fixed # of digits

'' -- aligned inside field

'' -- pad fills out field

str(nstr.fmt_number(value, "x", digits, width, pad))

pub oct(value)

'' Emit value as octal

str(nstr.itoa(value, 8, 0))

pub foct(value, digits)

'' Emit value as octal using fixed # of digits

str(nstr.itoa(value, 8, digits))

pub joct(value, digits, width, pad)

'' Emit value as octal using fixed # of digits

'' -- aligned inside field

'' -- pad fills out field

str(nstr.fmt_number(value, "o", digits, width, pad))

pub qrt(value)

'' Emit value as quarternary

str(nstr.itoa(value, 4, 0))

pub fqrt(value, digits)

'' Emit value as quarternary using fixed # of digits

str(nstr.itoa(value, 4, digits))

pub jqrt(value, digits, width, pad)

'' Emit value as quarternary using fixed # of digits

'' -- aligned inside field

'' -- pad fills out field

str(nstr.fmt_number(value, "q", digits, width, pad))

pub bin(value)

'' Emit value as binary

str(nstr.itoa(value, 2, 0))

pub fbin(value, digits)

'' Emit value as binary using fixed # of digits

str(nstr.itoa(value, 2, digits))

pub jbin(value, digits, width, pad)

'' Emit value as binary using fixed # of digits

'' -- aligned inside field

'' -- pad fills out field

str(nstr.fmt_number(value, "b", digits, width, pad))

dat { smart pin uart/buffer manager }

org

uart_mgr        setq      #4-1                                  ' get 4 parameters from hub

rdlong    rxd, ptra

uart_main       testb     rxd, #31                      wc      ' rx in use?

if_nc       call      #rx_serial

testb     txd, #31                      wc      ' tx in use?

if_nc       call      #tx_serial

jmp       #uart_main

rx_serial       testp     rxd                           wc      ' anything waiting?

if_nc       ret

rdpin     t3, rxd                               ' read new byte

shr       t3, #24                               ' align lsb

mov       t1, p_rxbuf                           ' t1 := @rxbuf

rdlong    t2, ptra[4]                           ' t2 := rxhead

add       t1, t2

wrbyte    t3, t1                                ' rxbuf[rxhead] := t3

incmod    t2, #(BUF_SIZE-1)                     ' update head index

_ret_       wrlong    t2, ptra[4]                           ' write head index back to hub

tx_serial       rdpin     t1, txd                       wc      ' check busy flag

if_c        ret                                             '  abort if busy

rdlong    t1, ptra[6]                           ' t1 = txhead

rdlong    t2, ptra[7]                           ' t2 = txtail

cmp       t1, t2                        wz      ' byte(s) to tx?

if_e        ret

mov       t1, p_txbuf                           ' start of tx buffer

add       t1, t2                                ' add tail index

rdbyte    t3, t1                                ' t3 := txbuf[txtail]

wypin     t3, txd                               ' load into sp uart

incmod    t2, #(BUF_SIZE-1)                     ' update tail index

_ret_       wrlong    t2, ptra[7]                           ' write tail index back to hub

' --------------------------------------------------------------------------------------------------

rxd             res       1                                     ' receive pin

txd             res       1                                     ' transmit pin

p_rxbuf         res       1                                     ' pointer to rxbuf

p_txbuf         res       1                                     ' pointer to txbuf

t1              res       1                                     ' work vars

t2              res       1

t3              res       1

fit       472

con { license }

{{

Terms of Use: MIT License

}}

### 14.2_Example_WRD_NUM_To_STR

{{14.2_Example_WRD_NUM_To_STR}}

{{ =====================================================================================

''

''   File....... WRD_nstr.spin2

''   Purpose.... Convert numbers to strings

''   Authors.... Jon McPhalen

''               -- Copyright (c) 2020 Jon McPhalen

''               -- see below for terms of use

''   E-mail..... jon.mcphalen@gmail.com

''   Started....

''   Updated.... 29 AUG 2020

''

=====================================================================================

}}

con

NBUF_SIZE = 48

PBUF_SIZE = 128

var

byte  nbuf[NBUF_SIZE]                                         ' number conversions

byte  pbuf[PBUF_SIZE]                                         ' padded strings

pub null()

'' This is not a top level object

pub fmt_number(value, radix, digits, width, pad) : p_str        ' *** changed 19 AUG 2020 ***

'' Return pointer to string of value converted to number in padded field

'' -- value is converted using radix

'' -- radix is chararacter indicating type                      ' *** used to be number ***

'' -- digits is max number of digits to use

'' -- width is width of final fields (max)

'' -- pad is character used to pad final field (if needed)

case radix

"d", "D" : p_str := padstr(itoa(value, 10, digits), width, pad)

"u", "U" : p_str := padstr(usdec(value, digits), width, pad)

"f", "F" : p_str := padstr(dpdec(value, digits), width, pad)

"b", "B" : p_str := padstr(itoa(value,  2, digits), width, pad)

"q", "Q" : p_str := padstr(itoa(value,  4, digits), width, pad)

"o", "O" : p_str := padstr(itoa(value,  8, digits), width, pad)

"x", "X" : p_str := padstr(itoa(value, 16, digits), width, pad)

other    : p_str := string("?")

pub dec(value, digits) : p_str | sign, len

'' Convert decimal value to string

'' -- digits is 0 (auto size) to 10

p_str := itoa(value, 10, digits)

pub usdec(value, digits) : p_str | len

'' Convert unsigned decimal value to string

'' -- digits is 0 (auto size) to 10

digits := 0 #> digits <# 10                                   ' limit printable digits

bytefill(@nbuf, 0, NBUF_SIZE)                                 ' clear buffer

p_str := @nbuf + 9                                            ' point to end of udec string

len := 0

repeat

byte[--p_str] := (value +// 10) + "0"                       ' extract digit, convert to ASCII

value +/= 10                                                ' remove digit from value

if (digits)                                                 ' length limited?

if (++len == digits)                                      '  check size

quit

else

if (value == 0)                                           ' done?

quit

pub dpdec(value, dp) : p_str | len, byte scratch[12]

'' Convert value to string with decimal point

'' -- dp is digits after decimal point

'' -- returns pointer to updated fp string

'' -- modifies original string

'' -- return pointer to converted string

p_str := itoa(value, 10, 0)

if (dp <= 0)                                                  ' abort if no decimal point

return p_str

len := strsize(p_str)                                         ' digits

bytefill(@scratch, 0, 12)                                     ' clear scratch buffer

if (value < 0)                                                ' ignore "-" if present

++p_str

--len

if (len < (dp+1))                                             ' insert 0s?

bytemove(@scratch, p_str, len)                              ' move digits to scratch buffer

bytefill(p_str, "0", dp+2-len)                              ' pad string with 0s

bytemove(p_str+dp+2-len, @scratch, len+1)                   ' move digits back

byte[p_str+1] := "."                                        ' insert dpoint

else

bytemove(@scratch, p_str+len-dp, dp)                        ' move decimal part to buffer

byte[p_str+len-dp] := "."                                   ' insert dpoint

bytemove(p_str+len-dp+1, @scratch, dp+1)                    ' move decimal part back

if (value < 0)                                                ' fix pointer for negative #s

--p_str

pub itoa(value, radix, digits) : p_str | sign, len, d

'' Convert signed integer to string

'' -- supports radix 10, 2, 4, 8, and 16

'' -- digits is 0 (auto size) to limit for long using radix

bytefill(@nbuf, 0, NBUF_SIZE)                                 ' clear buffer

p_str := @nbuf                                                ' point to it

case radix                                                    ' limit printable digits

02    : digits := 0 #> digits <# 32

04    : digits := 0 #> digits <# 16

08    : digits := 0 #> digits <# 11

10    : digits := 0 #> digits <# 10

16    : digits := 0 #> digits <#  8

other :

byte[p_str] := 0

return

if ((radix == 10) && (value < 0))                             ' deal with negative decimals

if (value == negx)

sign := 2

value := posx

else

sign := 1

value := -value

else

sign := 0

len := 0

repeat

d := value +// radix                                        ' get digit (1s column)

byte[p_str++] := (d < 10) ? d + "0" : d - 10 + "A"          ' convert to ASCII

value +/= radix                                             ' remove digit

if (digits)                                                 ' length limited?

if (++len == digits)                                      '  check size

quit

else

if (value == 0)                                           ' done?

quit

if (sign)

byte[p_str++] := "-"                                        ' add sign if needed

if (sign == 2)

nbuf[0] := "8"                                            ' fix negx if needed

byte[p_str++] := 0                                            ' terminate string

return revstr(@nbuf)                                          ' fix order (reverse)

pub revstr(p_str) : result | first, len, last

'' Reverse the order of characters in a string.

result := first := p_str                                      ' start

len := strsize(p_str)                                         ' length

last := first + len - 1                                       ' end

repeat (len >> 1)                                             ' reverse them

byte[first++], byte[last--] := byte[last], byte[first]

pub padstr(p_str, width, padchar) : p_pad | len

'' Pad string with padchar character

'' -- positive width uses left pad, negative field width uses right pad

'' -- truncate if string len > width

'' -- input string is not modified

'' -- returns pointer to padded string

bytefill(@pbuf, 0, PBUF_SIZE)                                 ' clear padded buffer

len := strsize(p_str)                                         ' get length of input

width := -PBUF_SIZE+1 #> width <# PBUF_SIZE-1                 ' constrain to buffer size

if (width > 0)                                                ' right-justify in padded field

if (width > len)

bytefill(@pbuf, padchar, width-len)

bytemove(@pbuf+width-len, p_str, len)

p_pad := @pbuf

else

bytemove(@pbuf, p_str+len-width, width)                   ' truncate to right-most characters

p_pad := @pbuf

elseif (width < 0)                                            ' left-justify in padded field

width := -width

if (width > len)

bytemove(@pbuf, p_str, len)

bytefill(@pbuf+len, padchar, width-len)

p_pad := @pbuf

else

bytemove(@pbuf, p_str, width)                             ' truncate to leftmost characters

p_pad := @pbuf

else

p_pad := p_str

con { license }

{{

Terms of Use: MIT License

}}

## 14.3) VAR Block

VAR is an object Block where variables are defined of 3 data types Byte,Word and Long. The VAR block declares global variables to the object.

VAR

byte a,b,c   '8bits

word a,b,c   '16bits

long a,b,c   '32bits

- Variables can be longs (32 bits), words (16 bits), and bytes (8 bits).

- Variables can be declared as singles or arrays.

- Variables are packed in memory in the order they are declared, beginning at a long-aligned address.

- Variables are initialized to zero at run time.

- Each object's first 15 longs of variable memory are accessed via special bytecodes for improved efficiency.

- Each instance of an object will require one long, plus its declared amount of VAR space, plus 0..3 bytes to long-align for the next object's variable space.

### 143.1) VAR Compiler Directive

VAR

byte temp           ‘temp is a VAR of type byte at address @temp

byte temp[20]    ‘reserve 20 bytes at address @temp[0]

Program Usage

Some_Var ≔ Byte[@MyData]  ‘read 1 byte from address @MyData

Some_Var ≔ Byte[@MyData][Index++]  ‘read 1 byte from @MyData+Index offset

VAR

word  temp           ‘temp is a VAR of type two byte at address @temp

word  temp[20]    ‘reserve 40 bytes at address @temp[0]

### 14.3.2)  Program Usage

Some_Var ≔ word[@MyData]  ‘read 2 byte from address @MyData

Some_Var ≔ word[@MyData][Index++]  ‘read 2 byte from @MyData+Index offset

VAR

Long  temp           ‘temp is a VAR of type byte at address @temp

Long  temp[20]    ‘reserve 80 bytes at address @temp[0]

Program Usage

Some_Var ≔ Long[@MyData]  ‘read 4 byte from address @MyData

Some_Var ≔ Long[@MyData][Index++]  ‘read 4 byte from @MyData+Index offset

### 14.3.3) Variables that are pre-defined and Permanent

In Spin2, there are both user-defined and permanent variables. The user-defined variable sources are listed below and the permanent variables are shown in the table.

VAR variables (hub)

PUB/PRI parameters, return values, and local variables (hub)

DAT symbols (hub)

Cog registers

| Variables (all LONG) | Variable Name | Address or Offset | Description | Useful in Spin2 | Useful in Spin2-PASM | Useful in PASM-Only |
| --- | --- | --- | --- | --- | --- | --- |
| Hub Locations | CLKMODE CLKFREQ | $00040 $00044 | Clock mode value Clock frequency value | Yes Yes | Yes Yes | No No |
| Hub VAR | VARBASE | +0 | Object base pointer, @VARBASE is VAR base, used by method-pointer calls | Maybe | No | No |
| Cog Registers | PR0 PR1 PR2 PR3 PR4 PR5 PR6 PR7 IJMP3 IRET3 IJMP2 IRET2 IJMP1 IRET1 PA PB PTRA PTRB DIRA DIRB OUTA OUTB INA INB | $1D8 $1D9 $1DA $1DB $1DC $1DD $1DE $1DF $1F0 $1F1 $1F2 $1F3 $1F4 $1F5 $1F6 $1F7 $1F8 $1F9 $1FA $1FB $1FC $1FD $1FE $1FF | Spin2 <-> PASM communication Interrupt JMP's and RET's Pointer registers Data pointer passed from COGINIT Code pointer passed from COGINIT Output enables for P31..P0 Output enables for P63..P32 Output states for P31..P0 Output states for P63..P32 Input states from P31..P0 Input states from P63..P32 | Yes Yes Yes Yes Yes Yes Yes Yes No No No No No No No No No No Yes Yes Yes Yes Yes Yes | Yes Yes Yes Yes Yes Yes Yes Yes Yes Yes Yes Yes Yes Yes Yes Yes Yes Yes Yes Yes Yes Yes Yes Yes | No No No No No No No No Yes Yes Yes Yes Yes Yes Yes Yes Yes Yes Yes Yes Yes Yes Yes Yes |

In Spin2, all variables can be indexed and accessed as bitfields. Additionally, symbolic hub variables can have BYTE/WORD/LONG size overrides:

| Variable Usage | Example | Description |
| --- | --- | --- |
| Plain | AnyVar HubVar.WORD BYTE[address] REG[register] | Hub or permanent register variable Hub variable with BYTE/WORD/LONG size override Hub BYTE/WORD/LONG by address Register, 'register' may be symbol declared in ORG section |
| With Index | AnyVar[index] HubVar.BYTE[index] LONG[address][index] REG[register][index] | Hub or permanent register variable with index Hub variable with size override and index Hub BYTE/WORD/LONG by address with index Register with index |
| With Bitfield | AnyVar.[bitfield] HubVar.LONG.[bitfield] WORD[address].[bitfield] REG[register].[bitfield] | Hub or permanent register variable with bitfield Hub variable with size override and bitfield Hub BYTE/WORD/LONG by address with bitfield Register with bitfield |
| With Index and Bitfield | AnyVar[index].[bitfield] HubVar.BYTE[index].[bitfield] LONG[address][index].[bitfield] REG[register][index].[bitfield] | Hub or permanent register variable with index and bitfield Hub variable with size override, index, and bitfield Hub BYTE/WORD/LONG by address with index and bitfield Register with index and bitfield |

### 14.3.4) Accessing Bytes of Larger-Sized Variables

Var

Word WordVar

Long LongVar

Pub Main()

WordVar.Byte[0] ≔ 0          ‘ 0000_0000

WordVar.Byte[1] ≔ 100      ‘0110_0100   =  64 + 32 + 4 =100

‘0110_0100_0000_0000 = 16384 + 8192 + 1024 = 25600

LongVar.byte[0] ≔ 25

LongVar.byte[1]  ≔ 50

LongVar.byte[2]  ≔ 75

LongVar.byte[3]  ≔ 100

‘01100100_01001011_00110010_00011001 = 1,682,649,625

### 14.3.1_Example_WRD_VAR

{{14.3.1_Exampl_WRD_VAR}}

CON

{System Clock}

_clkfreq = 200_000_000                                        ' set system clock

VAR  var1CogNum                'The default variable size is LONG (32 bits).

var2CursorMode            'The first 15 longs have special bytecodes for faster/smaller code.

var3CursorPosx            'Declare your most common variables first, as longs.

var4CursorPosy            'for the object being defined

var5

var6

var7

var8

var9

var10

var11

var12

var13

var14

var15

BYTE StringChr                'byte variable (8 bits)

BYTE StringBuff[64]        'byte variable array (64 bytes)

BYTE a,b,c[1000],d          'comma-separated declarations

WORD CurrentCycle         'word variable (16 bits)

WORD Cycles[200]           'word variable array (200 words)

WORD e,f[5],g,h[10]         'comma-separated declarations

LONG Value                     'long variable

LONG Values[15]            'long variable array (15 longs)

LONG i[100],j,k,l             'comma-separated declarations

PUB main()|byte byteVar[10],word wordVar[10],longVar[10] 'variable will be of type long if not declared

byteVar[9] := 255

wordVar[9] := 65535

longVar[9] := 4294967295

debug(ubin(byteVar[9]),ubin(wordVar[9]),ubin(longVar[9]))

repeat

## 14.4) PUB Block

PUB methodname({parameter{,...}}) {: result{,...}} {| {ALIGNW/ALIGNL} {BYTE/WORD/LONG} localvar{[count]}{,...}}

PUB    ObJectMethod(a,b,c) : result01,result02,result03)| var01,var02,var03

a,b,c are preloaded passed parameters from method call all longs

result01,result02,result03 all longs are cleared to zero before entry and return with a result from method, you must be able to receive the same number of result parameters.

var01,var02,var03 all lonfs are local parameters and could be pre-loaded from a previous method call you must manually clear this in the method unless you are using this retentive feature.

Note: In the P1 result was automatically understood as the return method value. In the P2 if no return parameter is declared in the method no return values are generated , ie result is not automatic you must declare if you want a return value.

### 14.4.1) PUB Block Constraints

- PUB methods are available to the parent object, as well as to the object they are defined in.

- PRI methods are available only to the object they are defined in.

- The first PUB method in an object is what executes when that object is run as the top-level object.

- Methods can have from 0 to 127 input parameters, all of which are single longs.

- Methods can have from 0 to 15 output results, all of which are single longs.

- Methods can have up to 64KB of local variables, which can be bytes, words, and longs (default), in both singles and arrays.

- Local variable size overrides (BYTE/WORD) apply only to the variable being declared, not subsequent variables.

- Results are initialized to zero on method entry, while local variables are undefined.

- Parameters, then results, and then local variables are packed into stack memory in the order they are declared.

- In-line PASM code can access the first 16 longs of parameters...results...locals via registers with the same symbolic names.

### 14.4.1_Example_WRD_Multiple_Result_Method

{14.4.1_Example_WRD_Multiple_Result_Method}

{{

All return variables (results) long variables

}}

CON

_clkfreq = 200_000_000     'debug must have a clock greater than 10MHZ

PUB main():result01,result02,result03 | a,b,c,x               'method run after boot spin interperter loads

x := 0

a := b := c := 0

repeat until x == 10

a := a + x

b := b + x

c := c + x

result01,result02,result03 := MultipleReturnResult(a,b,c)

debug(udec(result01),udec(result02),udec(result03))

x += 1

PUB MultipleReturnResult(a,b,c):result01,result02,result03|x,y

result01 := 123 + a

result02 := 456 + b

result03 := 789 + c

Note: The common naming convention do not clash because they are generated local to the method, result01,result02,result03 are separate and independent. Compiler keeps track.

### 14.4.2_Example_WRD_GETRND

### 14.4.3_Example_WRD_XYPOL (Polar Co-ordinates xy to length,angle32bit)

### 14.4.4_Example_WRD_POLXY(Polar Co-Ordinates Length,angle32bit to xy)

### 14.4.5_Example_WRD_ROTXY (Polar Co-Ordinate Rotation)

### 14.4.6_Example_WRD_QSIN_QCOS

### 14.4.7_Example_WRD_QSin_QCOS_Simple_Scope

### 14.4.8_Example_WRD_MULDIV64

### 14.4.9_Example_WRD_STRING

### 14.4.10_Example_WRD_REPEAT

{{14.4.10_Example_WRD_REPEAT}}

CON

{System Clock}

_clkfreq = 200_000_000           'set system clock for debug

PUB main() |count,x,y,z

debug(zstr(@seperator))

count := 10

x := 1

repeat count                    'Repeat <count> times, if <count> is zero then <indented code> is skipped

debug(udec(x))

x += 1                       'increment x

debug(zstr(@seperator))

x,y,z := 0,9,10                  're-assign z from x and increment to y count up 0-9

repeat z from x to y             'Repeat while iterating <variable> from <first> to <last>, stepping by +1

debug(udec(z))                'After completion, <variable> = <last used>

debug(zstr(@seperator))

x,y,z := 9,0,10                  're-assign z from x and decrement to y count dn 9-0

repeat z from x to y             'Repeat while iterating <variable> from <first> to <last>, stepping by -1

debug(udec(z))                'After completion, <variable> = <last used>

debug(zstr(@seperator))

x,y,z := 0,9,10                  're-assign z from x and increment to y count up 0-9

repeat z from x to y step 2      'Repeat iterating <variable> from <first> to <last>, stepping by + <delta>

debug(udec(z))                'After completion, <variable> = <last>

debug(zstr(@seperator))

x,y,z := 9,0,10                  're-assign z from x and decrement to y count dn 9-0

repeat z from x to y step -2     'Repeat iterating <variable> from <first> to <last>, stepping by - <delta>

debug(udec(z))                'After completion, <variable> = <last used>

debug(zstr(@seperator))

x,y,z := 0,0,0

repeat while (z <= 10)            'Repeat while <condition> is true not zero

debug(udec(z))                  '<condition> is evaluated before <indented code> executes

z += 1                         'After completion, <variable> = <last used> through loop

debug(zstr(@seperator))

x,y,z := 0,0,0

repeat until (z == 10)            'Repeat until <condition> is false zero,

debug(udec(z))                  '<condition> is evaluated before <indented code> executes

z += 1                         'After completion, <variable> = <last used> through loop

debug(zstr(@seperator))

x,y,z := 0,0,0

debug(udec(z))

repeat                           'keep cog running loop forever

x += 1

DAT

Seperator byte "-----------------",0

### 14.4.11_Example_WRD CASE

The CASE construct sequentially compares a target value to a list of possible matches. When a match is found, the related code executes.

Match values/ranges must be indented past the CASE keyword. Multiple match values/ranges can be expressed with comma separators. Any additional lines of code related to the match value/range must be indented past the match value/range:

CASE target					- CASE with target value

<match> : <code>				- match value and code

<indented code>

<match..match> : <code>			- match range and code

<indented code>

<match>,<match..match> : <code>		- match value, range, and code

<indented code>

OTHER : <code>				- optional OTHER case, in case no match found

<indented code>

{{14.4.12_Example_WRD_CASE}}

{{

CASE target                                     - CASE with target value

<match> : <code>                              - match value and code

<indented code>

<match..match> : <code>                       - match range and code

<indented code>

<match>,<match..match> : <code>               - match value, range, and code

<indented code>

OTHER : <code>                                - optional OTHER case, in case no match found

<indented code>

}}

CON

{System Clock}

_clkfreq = 200_000_000                         ' set system clock

PUB main()|x,y,z,pinfield

x,y,z := 1,2,3

Case x + y

3 : debug(udec(x),udec(y))               'match value and code

1 : debug(udec(x))

2 : debug(udec(y))

x,y,z := "A","B","C"

Case "A"

"A" : debug(udec(x))

"B" : debug(udec(y))

"C" : debug(udec(z))

pinfield := (0 addpins 7)

repeat x from 0 to 8

Case x

0 : pinhigh(0)

1 : pinhigh(1)

2 : pinhigh(2)

3 : pinhigh(3)

4 : pinhigh(4)

5 : pinhigh(5)

6 : pinhigh(6)

7 : pinhigh(7)

other : pinlow(pinfield)      'optional OTHER case, in case no match found

waitms(500)

pinfloat(pinfield)

pinfield := (0 addpins 7)

repeat x from 0 to 8

Case x

0    :  pinhigh(0)

1    :  pinhigh(1)

2..3 :  pinhigh(pinfield)     'match range and code

4..5 :  pinlow(pinfield)

6..7 :  pinhigh(4)            'execute multiple lines of code

pinhigh(5)

pinhigh(6)

pinhigh(7)

other : pinlow(pinfield)

waitms(500)

pinfloat(pinfield)

pinfield := (0 addpins 7)

repeat x from 0 to 8

Case x

0      :  pinhigh(0)

1      :  pinhigh(1)

2,3..5 :  pinhigh(pinfield)     'match value, range, and code

6      :  pinlow(pinfield)

7      :  pinhigh(7)            'execute multiple lines of code

other   :  pinlow(pinfield)

waitms(500)

pinfloat(pinfield)

repeat

### 14.4.12_Example_WRD_CASE_FAST

CASE_FAST is like CASE, but rather than sequentially comparing the target to a list of possible matches, it uses an indexed jump table of up to 256 entries to immediately branch to the appropriate code, saving time at a possible cost of larger compiled code. If there are only contiguous match values and no match ranges, the resulting code will actually be smaller than a normal CASE construct with more than several match values.

For CASE_FAST to compile, the match values/ranges must be unique constants which are all within 255 of each other.

CASE flag

0: CASE_FAST chr

0:     BYTEFILL(@screen, " ", screen_size)

col := row := 0

1:     col := row := 0

2..7:  flag := chr

RETURN

8:     IF col

col--

9:     REPEAT

out(" ")

WHILE col & 7

10:    RETURN

11:    color := $00

12:    color := $80

13:    newline()

OTHER: out(chr)

2:    col := chr // cols

3:    row := chr // rows

4..7: background0_[flag-$04] := chr << 8

flag := 0

### 14.4.13_Example_WRD_IF_IFNOT_ELSEIF_ELSEIFNOT_ELSE

The IF construct begins with IF or IFNOT and optionally employs ELSEIF, ELSEIFNOT, and ELSE. To all be part of the same decision tree, these keywords must have the same level of indentation.

The indented code under IF or ELSEIF executes if <condition> is not zero. The code under IFNOT or ELSEIFNOT executes if <condition> is zero. The code under ELSE executes if no other indented code executed:

IF / IFNOT <condition>			- Initial IF or IFNOT

<indented code>

ELSEIF / ELSEIFNOT <condition>		- Optional ELSEIF or ELSEIFNOT

<indented code>

ELSE				              - Optional final ELSE

<indented code>

{{14.4.13_Example_WRD_IF_IFNOT_ELSEIF_ELSEIFNOT_ELSE}}

{{IF / IFNOT <condition>                  - Initial IF or IFNOT

<indented code>

ELSEIF / ELSEIFNOT <condition>          - Optional ELSEIF or ELSEIFNOT

<indented code>

ELSE                                    - Optional final ELSE

<indented code>}}

CON

{System Clock}

_clkfreq = 200_000_000                'set system clock

PUB main()|x,y,z,pinfield

pinfield := 0 addpins 1

debug("1",zstr(@seperator))

x,y,z := 5,10,15

if x >= 5                             'simple if statement

debug("x is greater than 5 or equal to 5")

waitms(1500)

debug("2",zstr(@seperator))

x,y,z := 4,10,15

if x >= 5

pinhigh(0)

pinlow(1)

debug("x is greater than 5 or equal to 5")

else

pinlow(0)

pinhigh(1)

debug("x is less than 5")             'simple if else statement

waitms(1500)

debug("3",zstr(@seperator))

x,y,z := 3,10,15

if x >= 5

pinhigh(0)

pinlow(1)

debug("x is greater than 5 or equal to 5")

elseif x >= 4

pinlow(0)

pinhigh(1)

debug("x is greater than 4 or equal to 4")

else

debug("x is less than 4")

pinfloat(pinfield)

repeat

DAT

seperator byte "------------------------",0

### 14.4.14_Example_WRD_SPIN2_Differences

{{14.4.14_Example_WRD_SPIN2_PUB_Differences}}

{{

PUB methodname({parameter{,...}}) {: result{,...}} {| {ALIGNW/ALIGNL} {BYTE/WORD/LONG} localvar{[count]}{,...}}

Prop1 could only have local variable type long Prop2 can define byte/word/long

}}

con

_clkfreq = 200_000_000

Pub main()|byte byteVar[10],word wordVar[10],long longVar[10]

'if no local variable type declared long is default

byteVar[0] := 255

wordVar[0] := 65535

longVar[0] := $FFFF_FFFF

debug(udec(byteVar[0]),udec(wordVar[0]),udec(longVar[0]))

repeat

## 14.5) PRI Block

PRI methodname({parameter{,...}}) {: result{,...}} {| {ALIGNW/ALIGNL} {BYTE/WORD/LONG} localvar{[count]}{,...}}

The private methods are internal to the object and can not be referenced from outside the object.

result –is the return value

PRI privateMethod()

Code

## 14.6) Dat Block

Memory Declaration:

<Symbol> Alignment <Size> <Data>  ‘ reserved memory

Symbol –optional name for the reserved space

Alignment <Size> –the byte alignment for reserved data byte,word,long (1byte,2byte,4byte)

Data –constant expression or comma separated variable or quoted strings treated as same

PASM Propeller Assembly Machine Code:

<Symbol> <Condition> Instruction <Effects> ‘Propeller Assembly Code

Symbol –optional name for the command line

Condition –flag condition C Carry or Z Zero IF_C, IF_NC, IF_Z, IF_NZ

Instruction –assembly language Instruction eg. ADD,MOV,etc

Effects –effects that cause the result to be written when executed WR,WC,WZ

### 14.6.1) Common Dat Declaration Alignment

Data is declared with alignment and size.(Byte(1),Word(2),Long(4)).

| Long | 0 |  |  |  | 1 |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Word | 0 |  | 1 |  | 2 |  | 3 |  |
| Byte | 0 | 1 | 2 | 3 | 4 | 5 | 6 | 7 |
| Data | 40 | 41 | 53 | 74 | 72 | 69 | 6E | 67 |
| Long | 2 |  |  |  | 3 |  |  |  |
| Word | 4 |  | 5 |  | 6 |  | 7 |  |
| Byte | 8 | 9 | 10 | 11 | 12 | 13 | 14 | 15 |
| Data | 00 | 00 | C2 | FF | F8 | 24 | 00 | 00 |
|  |  | 0 PAD |  |  |  |  | 0 PAD | 0 PAD |
| Long | 4 |  |  |  | 5 |  |  |  |
| Word | 8 |  | 9 |  | 10 |  | 11 |  |
| Byte | 16 | 17 | 18 | 19 | 20 | 21 | 22 | 23 |
| Data | 11 | 22 | 33 | 44 | 20 | 00 | 00 | 00 |
| Dat |  |  |  |  |  |  |  |  |
|  | Byte | 64, | "A" | ,"String" | ,0 |  |  |  |
|  | Word | $FFC2, | 75000 |  |  |  |  |  |
|  | Long | $44,332,211 |  | 32 |  |  |  |  |
|  | S | t | r | i | n | g |  |  |
| String | 53 | 74 | 72 | 69 | 6E | 67 |  |  |
| A | 41 |  |  |  |  |  |  |  |
| 75000 | 124F8 |  |  |  |  |  |  |  |
| 124F8 | Is larger than what a word can hold so upper nibbles lost |  |  |  |  |  |  |  |

Long data type will always be placed with an alignment of 4 bytes from the beginning of memory and

Will be padded to maintain convention. Word data type will always be placed with an alignment of 2 bytes from the beginning of memory and will be padded or truncated. Byte data will align to a single byte.

Dat

Byte word $FFAA ,long  $BB995511

The above example specifies byte aligned data,but a word sized value followed by a long sized value. The result that memory contains consecutive data:

| Long | 0 |  |  |  | 1 |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Word | 0 |  | 1 |  | 2 |  | 3 |  |
| Byte | 0 | 1 | 2 | 3 | 4 | 5 | 6 | 7 |
| Data | FF | C2 | 11 | 55 | 99 | BB |  |  |
| Dat |  |  |  |  |  |  |  |  |
|  | byte | word | FFC2 | long | BB995511 |  |  |  |

This looks to be away to pack data and avoid 0 Padding.

DAT

MyData      byte   64,’$AA, 55              ‘creates data table

MyString    byte  “Hello World”,0       ‘Zero 0 terminated string

Pub GetData |Temp

Temp ≔ MyData[0]                          ‘get first byte of Data Table

Pub GetData | Temp

Tem ≔ BYTE[@MyData][0]

DAT

MyData      word    40_000, $BB50

MyList         word     long $FF995544, long 1000  ‘needs clarification book may be wrong pg332

DAT

MyData      Long    640_000, $BB50

MyList         byte    long $FF995544, long 1000  ‘needs clarification book may be wrong pg237

DAT Data Pointers

DAT Str0 BYTE "Monkeys",0 'strings with symbols Str1 BYTE "Gorillas",0 Str2 BYTE "Chimpanzees",0 Str3 BYTE "Humanzees",0 StrList WORD @Str0 'in Spin2, these are offsets of strings relative to start of object WORD @Str1 'in Spin2, @@StrList[i] will return address of Str0..Str3 for i = 0..3 WORD @Str2 'in PASM-only programs, these are absolute addresses of strings WORD @Str3 '(use of WORD supposes offsets/addresses are under 64KB)

### 14.6.2)  Filling Data Tables

symbol   data_type    fill_value[array_length]

DAT

Custom  long   -1[C_CHARS]

That is the equivalent of defining C_CHARS (length)  longs filling  the value of -1 in the DAT block -- it's creating a DAT array with all values initialized to -1. You can address each long individully using Custom[idx] where idx is 0 to 7. If you need the address of the array you can get it with @Custom. If you later need 10 longs, you only have to change the definition of C_CHARS.

### 14.6.1_Example_WRD_ Data Block Address

{{14.6.1_Example_WRD_ Data Block Address}}

{{

Self Modifying Code and Pointer to Tables

ALTS D,{#}S

Alter S field of next instruction to (D + S) & $1FF. D += sign-extended S[17:9].

MOV D,{#}S {WC/WZ/WCZ}

Move S into D. D = S. C = S[31]. *

}}

CON

_clkfreq = 200_000_000  ''Debug clock must be greater than 10MHZ

P0 =0 , P1 = 1  , P2 = 2

VAR

Byte cogRunning 'cog ID started is returned or -1 if not started

PUB main()

debug("--------------------------------------")

debug("Self Modifying Code")

debug("Example Modifies MOV Destination Field")

debug("Allows Pointer to increment through Table")

debug("To Display TABLE Values In single variable Value")

debug("--------------------------------------")

cogRunning := COGINIT(COGEXEC_NEW,@S0_Dat,0)

debug(udec(cogRunning))

repeat                                    'keep cog 0 running

DAT

ORG   0

S0_Dat                DRVH    #P0                      'P0 on program running

debug("--------------------------------------")

MOV  Pointer,#valTable  'set Pointer to first address in valTable

_Next           ALTS Pointer,Index

MOV  Value,#0                  'place holder #0 value comes from ALTS D,S = Pointer +Index

debug(udec(Pointer),udec(Index),udec(Value))

debug("---------------------------------------")

ADD   Index,#1

CMP   Index,#4  WZ

IF_Z        JMP   #_Loop1

JMP   #_Next

_Loop1                NOP

JMP     #_Loop1          'remember # imediate

Index                 long      0

Pointer              long      0

Value                 long      0

valTable            long      0,1,2,3

# 15.0) Operators

## 15.1) Pre and Post Operators

## 15.2) Operator ?? PsedoRandomNumberGenerator

PRNG of XOR032 requires variable to be non zero initially "Xorshift RNGs" by George Marsaglia describes a very efficient system for generating high-quality random numbers using very little compute and storage.

- https://forums.parallax.com/discussion/168188/xoroshiro-random-number-generator/p1

Here is the xoroshiro++ pseudo code:

;{s1,s0} = state (input and output)

;prn = pseudo-random number (output)

;tmp and prn can be the same register

;xoroshiro+

xor s1,s0		;s1 = s1 ^ s0

mov tmp,s1

rol s0,a		;s0 = s0 rol a

shl tmp,b		;tmp = (s1 ^ s0) shl b

xor s0,tmp

xor s0,s1		;s0 = s0 rol a ^ (s1 ^ s0) shl b ^ s1 ^ s0

rol s1,c		;s1 = (s1 ^ s0) rol c

mov prn,s0

add prn,s1		;prn = s0 + s1

;xoroshiro++ enhancement

## 15.1_Example_WRD_ Pre_and_Post_Operators

{{15.1_Example_WRD_Pre_and_Post_Operators}}

CON

_clkfreq = 300_000_000

VAR

long  var01

long  var02

PUB main():result01,result02 | x,y

{X++ Post increment after instruction}

x := 0

repeat until x== 10

y := x++               'Post instruction increment

debug(udec(y),udec(x))

debug("x is incremented after the assignment to y")

waitms(1000)

{++X Pre increment before instruction}

x := 0

repeat until x== 10

y := ++x               'Pre instruction increment

debug(udec(y),udec(x))

debug("x is incremented before the assignment to y")

waitms(1000)

{X-- Post decrement after instruction}

x := 10

repeat until x== 0

y := x--               'Post instruction increment

debug(udec(y),udec(x))

debug("x is decremented after the assignment to y")

waitms(1000)

{--X Pre decrement before instruction}

x := 10

repeat until x== 0

y := --x               'Pre instruction increment

debug(udec(y),udec(x))

debug("x is decremented before the assignment to y")

waitms(1000)

{??X RandomNumberGenerator RNG ?? of XOR032 requires variable to be non zero initially}

x := 3

y := ??x         'Pre instruction random out

debug(udec(y),udec(x))

debug("X is randomized prior to assignment y")

waitms(1000)

{X! post negate}

x := %10101010_10101010_11111111_00000000

y :=  x!

debug(ubin(y),ubin(x))

debug("note bits are complemented individually")

waitms(1000)

{X!! post negate}

x := %10101010_10101010_11111111_00000000

y :=  x!!

debug(ubin(y),ubin(x))

debug("note long word inverted to zero from non zero")

waitms(1000)

{y\x post negate}

x := 1234

y := 4321

debug(udec(var01),udec(y),udec(x))

var01 := y\x

debug(udec(var01),udec(y),udec(x))

debug("var01= y and y= x and x= x")

waitms(1000)

{x~ set x zero}

x := 1234

y := 0

var01 := 0

debug(udec(var01),udec(x))

var01 := x~

debug(udec(var01),udec(x))

debug("var01 = xorig= 1_234 x final is 0")

waitms(1000)

{x~~ set x one}

x := 1234

y := 0

var01 := 0

debug(udec(var01),udec(x))

var01 := x~~

debug(udec(var01),udec(x),sdec(x))

debug("var01 = xorig= 1_234 x final is -1")

waitms(1000)

## 15.2_Example_ Address_Operators

{{15.2_Example_WRD_Dat_Symbols_Address}}

CON

_clkfreq = 200_000_000

VAR

long  var01

long  var02

long  var03[7]

PUB main():result01,result02 | x,y

debug(zstr(@name),zstr(@address),zstr(@country),zstr(@city))

repeat x from 0 to 6

var03[x] := x+2

debug(ubin_long_array(@var03,4))

debug(udec_long_array(@var03,7))

debug(udec_long_array(@Var03 + 0,1))

debug(udec_long_array(@Var03 + 4,1))

debug(udec_long_array(@Var03 + 8,1))

debug(udec_long_array(@Var03 + 12,1))

debug(udec_long_array(@Var03 + 16,1))

debug(udec_long_array(@Var03 + 20,1))

debug(udec_long_array(@Var03 + 24,1))

DAT

name      byte  "William Robert Drury",0

address  byte  "86 Hume Drive",0

country  byte  "Canada",0

city         byte  "Cambridge",0

## 15.3_Example_WRD_Bitwise_Decod_and_ENCOD

Note: |<  Decode 0-31  Does not exist In P2 works for P1

DECOD  create a binary bit set in accordance with bit number 0-31        BinPattern = DECOD PinNum

PinNum =31 BinPattern %10000000_00000000_00000000_00000000

ENCOD create a Decimal number of the highest bit set in a given number  PinNum = ENCOD BinPattern

BinPattern = %11111111_11111111_11111111_11111111 PinNum = 31

{{15.3_Example_WRD_Decod_Encod}}

{{

DECOD  create a binary bit set in accordance with bit number 0-31        BinPattern = DECOD PinNum

PinNum =31 BinPattern %10000000_00000000_00000000_00000000

ENCOD create a Decimal number of the highest bit set in a given number  PinNum = ENCOD BinPattern

BinPattern = %11111111_11111111_11111111_11111111 PinNum = 31

}}

Con

_clkfreq = 200_000_000

Con

'------------------------------------------------------------------------------------------

PinNum01     = 31                                   '%0001_1111

BitPattern01 = DECOD PinNum01                       '10000000_00000000_00000000_00000000

'------------------------------------------------------------------------------------------

BitPattern02 = %11111111_11111111_11111111_11111111 '$FFFF_FFFF

PinNum02     = ENCOD BitPattern02                   '31

'------------------------------------------------------------------------------------------

Var

Long VarPinNum01

Long VarBitPattern01

Long VarPinNum02

Long VarBitPattern02

Pub main()

varPinNum01 :=31

repeat 32

varBitPattern01 := DECOD varPinNum01

debug(UDEC(varPinNum01),UBIN(varBitPattern01))

varPinNum01 -= 1

waitms(250)

varBitPattern02 := %00000000_00000000_00000000_00000001

varPinNum02 := 31

repeat 32

varPinNum02 := ENCOD varBitPattern02

debug(UDEC(varPinNum02),UBIN(varBitPattern02))

varBitPattern02 <<= 1

waitms(250)

varBitPattern01 := %00000000_10101010_00000000_00000000

varPinNum01 := ENCOD varBitPattern01

debug(ubin(varBitPattern01),udec(varBitPattern01),ubin(varPinNum01),udec(varPinNum01))

## 15.4_Example_WRD_##_Operator

{{

15.4_Example_WRD_##_Operation

Pr0 $1D8 Pr1 $1D9 Pr2 $1DA Pr3 $1DB Pr4 $1DC Pr5 $1DD Pr6 $1DE Pr7 $1DF

}}

CON

_clkfreq = 200_000_000                           ' system freq as a constant

NumCon01 = $1DA

NumCon02 = $1DB

PUB main()

org

mov Pr0,##NumCon01+NumCon02       'constant NumCon01 + NumCon02 is added imediate load to Pr0

mov Pr1,##(NumCon01+NumCon02)     '#imediate stops a register reference load

mov Pr2,#$1DA

mov Pr3,##Pr2

end

debug(udec(Pr0),udec(Pr1),uhex(Pr2),uhex(Pr3))

# 16.0) Method Pointer

Method pointers are LONG values which point to a method and are then used to call that method indirectly. To establish a method pointer, you can assign a long variable using "@" before the method name.

Note that there are no parentheses after the method name:

LongVar := @SomeMethod 'a method within the current object

LongVar := @SomeObject.SomeMethod 'a method within a child object

LongVar := @SomeOject[index].SomeMethod 'a method within an indexed child object

Method pointers can be generated on-the-fly and passed as parameters:

SetUpIO(@InMethod,@OutMethod)

Method pointers are then used in the following ways to call methods:

LongVar() 'no parameters and no return values

LongVar(Par1, Par2) 'two parameters and no return values

Var := LongVar():1 'no parameters and one return value

Var1,Var2 := LongVar(Par1):2 'one parameters and two return values

Var1,Var2 := POLXY(LongVar(Par1,Par2,Par3):2) 'three parameters and two return values

There is no compile-time awareness of how many parameters the method pointed to actually has. You need to code your method pointer usage such that you supply the proper number of parameters and specify the proper number of return values after a ":" , so that there is agreement with the method pointed to.

Method pointers can be passed through object hierarchies to enable direct calling of any method from anywhere. They can also be used to dynamically point to different methods which have the same numbers of parameters and return values.

How Method Pointers Work

An @method expression generates a 32-bit value which has two bit fields:

[31..20] = Index of the method, relative to the method's object base. The index of the first method will be twice the number of objects instantiated

[19..0] = Address of the method's VAR base. The method's VAR base, in turn, contains the address of the method's object base.

By putting the method's index and VAR base address together into the 32-bit value, and having the VAR base contain the method's object base address, a complete method pointer is established in a single long, which can be treated as any other variable.

To accommodate method pointers, each object instance reserves the first long of its VAR space for the object base address. When an @method expression executes, that first long is written with the object's base address.

### 1.6.0.1_Example_WRD_METHOD_POINTER

{{16.0.1_Example_WRD_METHOD_POINTER}}

{{

LongVar := @SomeMethod                                      'a method within the current object

LongVar := @SomeObject.SomeMethod               'a method within a child object

LongVar := @SomeOject[index].SomeMethod    'a method within an indexed child object

}}

con

_clkfreq = 200_000_000

var

long testMethodPointer

pub main()| byte Pin,long debugMsg1,long debugMsg2

Pin := 0

testMethodPointer := @OutMethodBlink

debugMsg1,debugMsg2 := testMethodPointer(Pin):2     'must declare number of return variables

debug(zstr(debugMsg1),zstr(debugMsg2))                         'send messages

repeat                                                                                        'keep cog0 running

pub OutMethodBlink(Ppin):result1,result2

pinhigh(Ppin)

result1 := @msgON

result2 := @msgReturn

waitms(500)

dat

msgON          byte      "Pin is ON high ",0                                     'zero terminated string

msgReturn   byte      "OutMethodBlink Executed ",0               'zero terminated string

### 16.0.2_Example_WRD_METHOD_POINTER_Object

{{16.0.2_Example_WRD_METHOD_POINTER_Object}}

{{

LongVar := @SomeMethod 'a method within the current object

LongVar := @SomeObject.SomeMethod 'a method within a child object

LongVar := @SomeOject[index].SomeMethod 'a method within an indexed child object

}}

con

_clkfreq = 200_000_000

obj

methodPointer : "16.0.1_Example_WRD_METHOD_POINTER"         'define object to include

var

long testMethodPointerInObject

pub main()| byte Pin,long debugMsg1,long debugMsg2

Pin := 0                                                    'P0 led 1k

testMethodPointerInObject := @methodPointer.OutMethodBlink  'indirect object method pointer

debugMsg1,debugMsg2 := testMethodPointerInObject(Pin):2          'declare number of return variables

debug(zstr(debugMsg1),zstr(debugMsg2))                                             'send messages

repeat

### 16.0.3_Example_WRD_METHOD_POINTER_Object_Array

{{16.0.3_Example_WRD_METHOD_POINTER_Object_Array}}

{{

LongVar := @SomeMethod 'a method within the current object

LongVar := @SomeObject.SomeMethod 'a method within a child object

LongVar := @SomeOject[index].SomeMethod 'a method within an indexed child object

}}

con

_clkfreq = 200_000_000

obj

methodPointer[4] : "16.0.1_Example_WRD_METHOD_POINTER"         'define object to include

var

long testMethodPointerInObject

pub main()| byte Pin,long debugMsg1,long debugMsg2

Pin := 0                                                          'P0 led 1k

testMethodPointerInObject := @methodPointer[0].OutMethodBlink     'indirect object method pointer

debugMsg1,debugMsg2 := testMethodPointerInObject(Pin):2            'declare number of return variables

debug(udec(Pin),zstr(debugMsg1),zstr(debugMsg2))                             'send messages

Pin := 1                                                          'P1 led 1k

testMethodPointerInObject := @methodPointer[1].OutMethodBlink     'indirect object method pointer

debugMsg1,debugMsg2 := testMethodPointerInObject(Pin):2           'declare number of return variables

debug(udec(Pin),zstr(debugMsg1),zstr(debugMsg2))                            'send messages

Pin := 2                                                          'P2 led 1k

testMethodPointerInObject := @methodPointer[2].OutMethodBlink     'indirect object method pointer

debugMsg1,debugMsg2 := testMethodPointerInObject(Pin):2           'declare number of return variables

debug(udec(Pin),zstr(debugMsg1),zstr(debugMsg2))                            'send messages

repeat

## 16.1) SEND

SEND is a special method pointer which is inherited from the calling method and, in turn, conveyed to all called methods. It's purpose is to provide an efficient output mechanism for data.

SEND can be assigned like a method pointer, but it must point to a method which takes one parameter and has no return values:

SEND := @OutMethod

When used as a method, SEND will pass all parameters, including any return values from called methods, to the method SEND points to:

SEND("Hello! ", GetDigit()+"0", 13)

Any methods called within the SEND parameters will inherit the SEND pointer, so that they can do SEND methods, too:

PUB Go()

SEND := @SetLED

REPEAT

SEND(Flash(),$01,$02,$04,$08,$10,$20,$40,$80)

PRI Flash() : x

REPEAT 2

SEND($00,$FF,$00)

RETURN $AA

PRI SetLED(x)

PINWRITE(56 ADDPINS 7, !x)

WAITMS(125)

In the above example, the following values are output in repeating sequence: $00, $FF, $00, $00, $FF, $00, $AA, $01, $02, $04, $08, $10, $20, $40, $80 (but inverted for LEDs)

Though a called method inherits the current SEND pointer, it may change it for its own purposes. Upon return from that method, the SEND pointer will be back to what it was before the method was called. So, the SEND pointer value is propagated in method calls, but not in method returns.

SEND acts as a special type of method pointer, inherited from the calling method and, in turn, conveyed to all called methods. It provides an efficient output mechanism for data.

You may assign SEND as you would any method pointer, but it must point to a method that 1. takes one parameter and 2. has no return values:

SEND := @OutMethod   'SEND points to OutMethod

SEND will pass all parameters, including any return values from called methods, to the method SEND points to:

SEND("Hello! ", GetDigit()+"0", 13)

Any methods called from within the SEND parameters, such as GetDigit() in the example above, will inherit the SEND pointer, so that they also may use the SEND method. The following code provides an example of SEND use. It sends 8-bit patterns of 0s and 1s to LEDS at pins P56 through P63:

PUB go()

SEND := @SetLED

REPEAT

SEND($01, $02, $04, $08, $10, $20, $40, $80)

PRI SetLED(x)

PINWRITE(56 ADDPINS 7, !x)

WAITMS(125)

Note: LEDs on the P2 EVAL Board are driven by active-low signals.

Within the go() method, the statement SEND := @SetLED, gives SEND the pointer to the SetLED method. This method satifies the requirement: only one variable and no return value. Note the SetLED method above includes a short delay so the LED patterns remain visible long enough so you can see them.

Next the REPEAT loop executes the SEND($01, $02...) statement that transfers the first parameter $01 to the SetLED method. The LED at pin P56 turns on. When this method finishes, it returns control to the SEND statement, which then sends $02 to the SetLED method, which turns on the LED at P57. Each LED turns on and off in sequence again and again in the REPEAT loop.

A second example shows how other methods can inherit the SEND pointer. An added method, Flash(), will turn all LEDs on and off. This method includes a SEND statement, too.

PUB go()

SEND := @SetLED

REPEAT

SEND(Flash(), $01, $02, $04, $08, $10, $20, $40, $80)

PRI Flash() : x

REPEAT 2

SEND($00,$FF,$00)

RETURN $AA

PRI SetLED(x)

PINWRITE(56 ADDPINS 7, !x)

WAITMS(125)

The program will call the Flash() method (in the first SEND() parameter) and will eventually pass the return value from Flash() to the SetLED() method (after Flash() has fully executed).

First, the Flash() method will run and send its own values, $00, $FF, $00, to the LEDs two times. Then, if you watch the LEDs, they next display $AA next. Why?

The Flash() method returns the value $AA to the SEND statement: SEND(Flash(),$01,$02... In effect the $AA value gets inserted in place of the call to Flash() in the list of parameters, making the whole program execution behave as if the SEND(Flash(), $01, $02...) had really been:

SEND($00,$FF,$00)
SEND($00,$FF,$00)
SEND($AA, $01, $02...)

The Flash() method inherited the SetLED() address and can use it independent of other uses in this program

### 16.1_Example_WRD_Send_Led

{{16.1_Example_WRD_Send_Led}}

{{

SEND can be assigned like a method pointer, but it must point to a method which takes one parameter

and has no return values:

SEND := @OutMethod                  'method can be either PUB or PRI

SEND(anotherMethod(),parameter)     'anotherMethod single return value is sent

Any methods called within the SEND parameters will inherit the SEND pointer, so that they can do SEND methods, too:

Pub anotherMethod() :result

send(parameter)              'SEND method Pointer @OutMethod is inherited and can be called by anotherMethod

result := $FF

Pub OutMethod

"code to go here"

}}

con

_clkfreq = 200_000_000

PUB Go()

SEND := @SetLED

REPEAT

SEND(Flash(),$0F,Flash(),$33,$DD,$33,$DD)

PRI Flash() : y

y := $F0

waitms(500)

PRI SetLED(x)

PINWRITE(0 ADDPINS 7, x)

WAITMS(500)

## 16.2) RECV

RECV, like SEND, is a special method pointer which is inherited from the calling method and, in turn, conveyed to all called methods. It's purpose is to provide an efficient input mechanism for data.

RECV can be assigned like a method pointer, but it must point to a method which takes no parameters and returns a single value:

RECV := @InMethod

An example of using RECV:

VAR i

PUB Go()

RECV := @GetPattern

REPEAT

PINWRITE(56 ADDPINS 7, !RECV())

WAITMS(125)

PRI GetPattern() : Pattern

RETURN DECOD(i++ & 7)

In the above example, the following values are output in repeating sequence: $01, $02, $04, $08, $10, $20, $40, $80 (but inverted for LEDs)

Though a called method inherits the current RECV pointer, it may change it for its own purposes. Upon return from that method, the RECV pointer will be back to what it was before the method was called. So, the RECV pointer value is propagated in method calls, but not in method returns.

### 16.2_Example_WRD_Receive_Led

{{16.2_Example_WRD_Receive_Led}}

{{

RECV := @InMethod

DECOD  create a binary bit set in accordance with bit number 0-31        BinPattern = DECOD PinNum

PinNum =31 BinPattern %10000000_00000000_00000000_00000000

}}

VAR i

PUB Go()

RECV := @GetPattern

REPEAT

i := 0

REPEAT 7

PINWRITE(0 ADDPINS 7, RECV())

WAITMS(250)

i := 0

REPEAT 7

PINWRITE(0 ADDPINS 7,!RECV())

WAITMS(250)

PRI GetPattern() : Pattern

if i > 7

i := 0

RETURN DECOD(i++ & 7)

# 17.0) PASM Propeller Assembly Language

(Need more information some supposition)

Typical PASM program consists of Spin code to boot propeller and DAT section consisting of assembly code to be loaded into a Cog. The Propeller II allows also Inline assembly language not available with Propeller I. Cog 0 is launched with the spin byte interpreter.

Each Cog has PC (Program Counter) that is incremented with a Common System Clock

PC points to the next instruction to be Executed.

Each Cog Has a Instruction Result Register or Flag Register with two bits zero Z and carry C flags

Result or Flag Register can be thought as to why cogs are Risc processors (reduced instruction set computer)

## 17.0.0_Example_PASM_Template(used for testing PASM commands)

### 17.0.0.1_Example_WRD_PASM_COG_DAT_Launch_Template

The following program template launches Cog 1 to run “codePASM”:

{{17.0.0.1_Example_WRD_PASM_COG_DAT_Launch_Template}}

CON

_clkfreq = 200_000_000      'Debug must be enabled clock must be greater than 10MHZ for Debug

VAR

Byte cogRunning 'cog ID started is returned or -1 if not started

PUB main()

cogRunning := COGINIT(COGEXEC_NEW,@codePASM,$FFF_FFFF)

debug(udec(cogRunning))

repeat  'keep cog 0 running

'Start next available cog which is 1 load cog 1 memory with  @codePASM at Cog Memory $10

'PTRA register will be loaded with $FFF_FFFF

DAT     ORG   0  'COGINIT(COGEXEC_NEW,@codePASM,PTRAvalue)

'Normally start at $000 but you don't have too

codePASM

MOV     DIRA, #$FF           'Set the direction of the first 8 pins to Output

GETCT   cogCounterValue      'Get global system counter value

ADDCT1  cogCounterValue,PTRA 'set CT1 event to trigger on CT = countvalue + PTRA

_Loop   WAITCT1

ADDCT1  cogCounterValue,PTRA

XOR     OUTA, #1

NOP

debug(ubin(OUTA))        'send status to Debug Window

JMP     #_Loop           'don't forget #imediate setting or register is loaded

cogCounterValue       Long 0   'counter value CT storage

Both data and PASM may be intermixed in the DAT section . The “COGINIT”command loads 496 consecutive long values starting from the aspecified address “ORG 0” The PASM starts to run from the specified address.

Note:

1)(TBD) The :_Loop instruction symbol name is forgotten by the compiler after compiling the JMP #:_Loop thus allowing _Loop to be reused in subsequent assembly code symbol labeling. (Needs Verification)

2) PTRA register can contain an address allowing a reference for acquiring other variables in the HUB in the above example a value is passed.

3) ORG 0 directive causes compiled code to be loaded starting at 0 and initiating PC to start from 0 this could be changed for example to ORG 10 thus the first registers 0-9 long in length would be available for program data storage.

4) # stands for immediate if not included the instruction is register reference and loads from register not value.

### 17.0.0.2_Example_WRD_Inline_PASM

{{

17.0.0.2_Example_WRD_PASM_COG_DAT_Launch_Template

}}

CON

_clkfreq = 200_000_000  ''Debug must be enabled clock must be greater than 10MHZ for Debug

PUB main()

PTRA := $FFF_FFFF      'spin assignment instruction

ORG

codePASM

MOV     DIRA, #$FF                           'Set the direction of the first 8 pins to Output

GETCT   cogCounterValue                'Get global system counter value

ADDCT1  cogCounterValue,PTRA    'set CT1 event to trigger on CT == countvalue + PTRA

._Loop    WAITCT1                                               'wait till CT == cogCounterValue

ADDCT1  cogCounterValue,PTRA

XOR     OUTA, #1

NOP

debug(ubin(OUTA))    'send status to Debug Window

JMP     #._Loop            'JMP to Loop note the dot . if included name reference is forgotten by compiler

cogCounterValue    Long 0         'counter value CT storage

END

## 17.0.1) Assembly Language Syntax

Each assembly instruction has common syntax elements consisting of an optional label, optional condition, the instruction and optional effects:

<Label> <Condition> Instruction <Effects>

- Label- is an optional statement. Label canbe global (starting with and underscore “_” or letter) or can be local (starting with a colon “:”) Local labels must be separated from other same named local labels by at least one global label. Label is used by instructions like JMP,CALL and COGINIT to designate the target destination.

- Condition-is an optional execution condition (IF_C,IF_Z etc) that causes and instruction to be executed or not. There are 32 possible condition checks.

- Instruction-is a Propeller Assembly Instruction (MOV,ADD,COGINIT etc) and it’s operands. There are 409 Instructions with upto 2 operands per instruction.

- Effects- is an optional list of one to three execution effects (WZ,WC,WR and NR). They cause the Instruction to modify the Z flag,C flag and write or not write the instruction result value to the destination register.

#S-Immediate value to be used      S-register contains value to be use    S represent the source operand

#D-Immediate value to be used      D-register is the result to be written  D represents the destination operand

* Z = (result == 0).

** If #S and cogex, PC += signed(S). If #S and hubex, PC += signed(S*4). If S, PC = register S. (needs clarification)

The Instruction are listed with out Labels or Conditionas an example ROR:

<Label> <Condition> ROR D,#S  {WC,WZ,WCZ}

Rotate Right D = [31:0]  of ({D[31:0], D[31:0]}     >> S[4:0]). C = last bit shifted out if S[4:0] > 0, else D[0].

Condition        Instruction              Effects(Flags)            Destination                                   Source_________

31 30 29 28 | 27 26 25 24 23 22 21 |  20 19 18 | 17 16 15 14 13 12 11 10 09 | 08 07 06 05 04 03 02 01 00

E    E   E   E         0   0  0  0  0  0   0             C  Z  I                  D D D D D D D D  D               S  S  S  S  S   S  S  S  S

4 bit = 15               7 bit = 127                 3 bit =7         9 bit = 511  max address        9 bit = 511 max address

.

| Key | Description |
| --- | --- |
| EEEE | Conditional test (see "Instruction Prefix" list at bottom of the instruction set spreadsheet) |
| C | 0: Do not update the "C" register 1: Update the "C" register. In the instruction syntax, this is denoted by "WC" or "WCZ". |
| Z | 0: Do not update the "Z" register 1: Update the "Z" register. In the instruction syntax, this is denoted by "WZ" or "WCZ". |
| I | 0: Source field is a register address 1: Source field is a literal value. In the instruction syntax, this is denoted by the "#" character. |
| L | 0: Destination field is a register address 1: Destination field is a literal value. In the instruction syntax, this is denoted by the "#" character. |
| DDDDDDDDD | Destination field |
| SSSSSSSSS | Source field |
| N,NN,NNN | Index number. This is only used for instructions with a third index argument. |
| cccc | conditional test used to update C (%0000=clear, %1111=set, all others per EEEE) |
| zzzz | conditional test used to update Z (%0000=clear, %1111=set, all others per EEEE) |

NOTE: Some instruction not using WC or WZ  can use CZI field for different meanings such as:

CallPA  #D #S   CZI  = 1LI = 111 for #D #S (L=D I=S)

### 17.0.1.1_Example_WRD_PASM_Instruction_Syntax

This program can be run with _RET_ and debug(ubin(S1_ROR)) commented out , and can be run with uncommented to return condition code. When uncommented the _RET_    ROR  Pr0,Pr1 executes and closes the cog but debug window will return the Instruction code with condition code EEEE.

Note:

ROR rotate right shifts bit B0 back to B31. SHR shift right loses B0

Debug can be used to grab the Encoded instruction to determine condition expression

S[4:0] contains number of bits to shift maximum is 31 for 32 bit long word

{{

17.0.1.1_Example_WRD_PASM_Instruction Syntax

Pr0 = $1D8 Pr1 = $1D9

ROR D,{#}S {WC/WZ/WCZ}  Rotate right.      Note: bit B0 will rotate back to B31

D = [31:0] of ({D[31:0], D[31:0]} >> S[4:0]). C = last bit shifted out if S[4:0] > 0, else D[0]. *

SHR     D,{#}S   {WC/WZ/WCZ} Shift right.  Note: bit is lost no rotate back

D = [31:0]  of ({32'b0, D[31:0]}       >> S[4:0]). C = last bit shifted out if S[4:0] > 0, else D[0].  *

WRC     D   Write 0 or 1 to D,

according to  C. D = {31'b0,  C).

}}

CON

_clkfreq = 200_000_000  ''Debug must be enabled clock must be greater than 10MHZ for Debug

Shift = 1 'number of bits to shift

VAR

Byte cogRunning 'cog ID started is returned or -1 if not started

Byte PTRAvalue

PUB main()

PTRAvalue := %00000000_00000000_00000000_00000001

Pr0 := %0000000_00000000_00000000_00000001  'cog 0 register

Pr1 := %1000000_00000000_00000000_00000000  'cog 0 register

debug(ubin(S0_ROR))

'debug(ubin(S1_ROR))

debug(ubin(S2_ROR))

debug(ubin(S3_ROR))

debug(ubin(S4_ROR))

debug(ubin(S5_ROR))

debug(ubin(S6_ROR))

debug(ubin(S7_ROR))

debug(ubin(S8_ROR))

cogRunning := COGINIT(COGEXEC_NEW,@S0_ROR,PTRAvalue)

debug(udec(cogRunning))

repeat

DAT

ORG   0

S0_ROR                     ROR   Pr0,Pr1           'Execute and shift number of bits in Pr1 B0-> B31

S1_ROR  _RET_        ROR   Pr0,Pr1           'Execute <inst> always and return if no branch.If no branch pop stack

S2_ROR  IF_Z            ROR   Pr0,Pr1           'Execute <inst> if Z = 1.

S3_ROR  IF_C           ROR   Pr0,Pr1           'Execute <inst> if C = 1.

S4_ROR  IF_NC_AND_NZ    ROR   Pr0,Pr1           'Execute <inst> if C = 0 and Z = 0.

S5_ROR  IF_NZ_AND_NC    ROR   Pr0,Pr1           'Alias Execute <inst> if C = 0 and Z = 0.

S6_ROR                                  ROR   Pr0,Pr1      WC   'Instruction Effects change C flag

S7_ROR                                  ROR   Pr0,Pr1      WZ   'Instruction Effects change Z flag

S8_ROR  IF_C                       ROR   Pr0,#Shift  WCZ  'Instruction Effects change in C and Zflag

debug(ubin(DatVar00),ubin(DatVar01))

ROR   DatVar00,#31       'shift 31 bits

WRC   DatVar01           ' Write 0 or 1 to D based on C Flag

debug("ROR shift 31 bits", ubin(DatVar00),ubin(DatVar01))

ROR   DatVar00,#1 WC

WRC   DatVar01

debug(ubin(DatVar00),ubin(DatVar01))

debug(ubin(DatVar02),ubin(DatVar03))

SHR   DatVar02,#31

WRC   DatVar03

debug("SHR shift 31 bits",ubin(DatVar02),ubin(DatVar03))

SHR   DatVar02,#1 WC

WRC   DatVar03

debug(ubin(DatVar02),ubin(DatVar03))

_Loop                                  NOP

JMP   #_Loop    'remember imediate

DatVar00      long    %10000000_00000000_00000000_00000000

DatVar01      long    0

DatVar02      long    %10000000_00000000_00000000_00000000

DatVar03      long    0

Note:

'S1_ROR  _RET_     ROR   Pr0,Pr1   'Execute <inst> always and return if no branch. If not branching pop stack    this line and debug(ubin(S1_ROR) are commented out

Note:

S1_ROR  _RET_     ROR   Pr0,Pr1   'Execute <inst> always and return if no branch. If not branching pop stack    this line and debug(ubin(S1_ROR) is allowed to be compiled.

| PASM Syntax | Encoded Instruction 32 Bit |
| --- | --- |
| S0_ROR ROR Pr0,Pr1 | 11110000_00000011_10110001_11011001 |
| S1_ROR _RET_ ROR Pr0,Pr1 | 00000000_00000011_10110001_11011001 |
| S2_ROR IF_Z ROR Pr0,Pr1 | 10100000_00000011_10110001_11011001 |
| S3_ROR IF_C ROR Pr0,Pr1 | 11000000_00000011_10110001_11011001 |
| S4_ROR IF_NC_AND_NZ ROR Pr0,Pr1 | 00010000_00000011_10110001_11011001 |
| S5_ROR IF_NZ_AND_NC ROR Pro,Pr1 | 00010000_00000011_10110001_11011001 |
| S6_ROR ROR Pr0,Pr1 WC | 11110000_00010011_10110001_11011001 |
| S7_ROR ROR Pr0,Pr1 WZ | 11110000_00001011_10110001_11011001 |
| S8_ROR IF_C ROR Pr0,#Shift WCZ | 11000000_00011111_10110001_11011001 |

|  |  |  |  |  | CZI |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Symbol | Condition | Mnemonic | Condition | Instruction | Effects | Destination | Source |
| S0_ROR |  | ROR Pr0,Pr1 | 1111 | 0000000 | 000 | 111011000 | 111011001 |
| S1_ROR | _RET | ROR Pr0,Pr1 | 0000 | 0000000 | 000 | 111011000 | 111011001 |
| S2_ROR | IF_Z | ROR Pr0,Pr1 | 1010 | 0000000 | 000 | 111011000 | 111011001 |
| S3_ROR | IF_C | ROR Pr0,Pr1 | 1100 | 0000000 | 000 | 111011000 | 111011001 |
| S4_ROR | IF_NC_AND_NZ | ROR Pr0,Pr1 | 0001 | 0000000 | 000 | 111011000 | 111011001 |
| S5_ROR | IF_NZ_AND_NC | ROR Pr0,Pr1 | 0001 | 0000000 | 000 | 111011000 | 111011001 |
| S6_ROR |  | ROR Pr0,Pr1 WC | 1111 | 0000000 | 100 | 111011000 | 111011001 |
| S7_ROR |  | ROR Pr0,Pr1 WZ | 1111 | 0000000 | 010 | 111011000 | 111011001 |
| S8_ROR | IF_C | ROR Pr0,#Shift WCZ | 1100 | 0000000 | 111 | 111011000 | 111011001 |

### 17.0.0.3_Example_WRD_ALT_R_D_S

ALTR D,{#}S

Alter result register address (normally D field) of next instruction to (D + S) & $1FF.

D += sign-extended S[17:9].

| ALTR D,{#}S | Alter result register address (normally D field) of next instruction to (D + S) & $1FF. D += sign-extended S[17:9]. |
| --- | --- |
| ALTR D | Alter result register address (normally D field) of next instruction to D[8:0]. |
| ALTD D,{#}S | Alter D field of next instruction to (D + S) & $1FF. D += sign-extended S[17:9]. |
| ALTD D | Alter D field of next instruction to D[8:0]. |
| ALTS D,{#}S | Alter S field of next instruction to (D + S) & $1FF. D += sign-extended S[17:9]. |
| ALTS D | Alter S field of next instruction to D[8:0]. |
| ALTB D,{#}S | Alter D field of next instruction to (D[13:5] + S) & $1FF. D += sign-extended S[17:9]. |

ALTR Alter R Field result register address (normally D field) of next instruction to (D + S) & $1FF.

D += sign-extended S[17:9]. D= IndexD[8:0] + sign-Extended [17:9]
ALTD Alter D field of next instruction to (D + S) & $1FF.

D += sign-extended S[17:9]. D= IndexD[8:0] + sign-Extended [17:9]
ALTS Alter S field of next instruction to (D + S) & $1FF.

D += sign-extended S[17:9]. D= IndexD[8:0] + sign-Extended [17:9]

RDSS= Offset S[17.9] + BaseAddressS[8:0]
D = IndexD[8:0] + sign-Extended [17:9]

ALTR D,{#}S

Alter result register address (normally D field) of next instruction to (D + S) & $1FF. D += sign-extended S[17:9].

{{17.0.0.3_Example_WRD_ALTR D,R,{#}S_116}}

{{

ALTR D,{#}S

Alter result register address (normally D field) of next instruction to (D + S) & $1FF.

D += sign-extended S[17:9].

By Means of an example we want the result of XOR X,Y  but you don't want to destroy register X.

By using the ALTR instruction you can avoid a bunch of move statements.

Also some registers cannot be written too. Using the ALTR instruction you can use the assembly instructions without destroying either register and writing the instruction operation to an alternate register.

ResultAddress= BaseAdressS[8:0] + OffseD[8:0] + IndexD[8:0]

S= Offset S[17.9] + BaseAddressS[8:0]

D = IndexD[8:0] + sign-Extended [17:9]

XOR D,{#}S {WC/WZ/WCZ}

XOR S into D. D = D ^ S. C = parity of result. *

Example

Write the result of XOR Ax,Bx to 'xorResult' not affecting Ax or Bx use Offset and Index

}}

CON

_clkfreq = 200_000_000  ''Debug clock must be greater than 10MHZ

P0 =0 , P1 = 1  , P2 = 2

VAR

Byte cogRunning 'cog ID started is returned or -1 if not started

PUB main()

debug("--------------------------------------")

debug("ALTR D,{#}S")

debug("Alter result register address (normally D field) of next instruction to (D + S) & $1FF.")

debug("D += sign-extended S[17:9].")

debug(" ")

debug("XOR D,{#}S {WC/WZ/WCZ} ")

debug("XOR S into D. D = D ^ S. C = parity of result. *")

debug("Example")

debug("Write the result of XOR Ax,Bx to 'xorResult' not affecting Ax or Bx use Offset and Index")

debug("--------------------------------------")

cogRunning := COGINIT(COGEXEC_NEW,@S0_ALTR,0)

debug(udec(cogRunning))

repeat                                    'keep cog 0 running

DAT

ORG   0

S0_ALTR               DRVH    #P0                      'P0 on program running

debug("--------------------------------------")

MOV  S_ALTR,Offset

SHL  S_ALTR,#9               'Place Offset in D[17:9

OR   S_ALTR,#xorResult0      'Load BaseAddress

MOV  D_ALTR,Index          'index from Offset =0

debug("before ALTR/XOR = ",udec(D_ALTR),ubin(S_ALTR))

debug(ubin(Ax),ubin(Bx))

debug(ubin(xorResult0),ubin(xorResult1))

debug(ubin(xorResult2),ubin(xorResult3))

ALTR D_ALTR,S_ALTR

XOR  Ax,Bx

debug("after ALTR/XOR = ",udec(D_ALTR),ubin(S_ALTR))

debug(ubin(Ax),ubin(Bx))

debug(ubin(xorResult0),ubin(xorResult1))

debug(ubin(xorResult2),ubin(xorResult3))

debug("--------------------------------------")

_Loop1                NOP

JMP     #_Loop1          'remember # imediate

Offset                long      1              'Offset from BaseAddress

Index                 long      1              'Index from offset

D_ALTR                long      0

S_ALTR                long      0

Ax                    long     %10101010

Bx                    long     %01010101

xorResult0            long      0

xorResult1            long      0

xorResult2            long      0

xorResult3            long      0

## 17.1) NOP No operation

NOP instruction does not effect any flags but requires 2 cycles can be used for timming or a filler when programming.

### 17.1.1_Example_WRD_NOP_001

{{17.1_Example_WRD_NOP_001}}

{{

NOP instruction does not effect any flags

requires 2 cycles can be used for timming or a filler when programming.

}}

CON

_clkfreq = 200_000_000  ''Debug must be enabled clock must be greater than 10MHZ for Debug

P0 =0 , P1 = 1  , P2 = 2

VAR

Byte cogRunning 'cog ID started is returned or -1 if not started

PUB main()

debug("--------------------------------------")

debug("NOP no operation")

debug("--------------------------------------")

cogRunning := COGINIT(COGEXEC_NEW,@S0_NOP,0)

debug(udec(cogRunning))

repeat                                    'keep cog 0 running

DAT

ORG   0

S0_NOP                DRVH    #P0            'P0 on program running

MODCZ   _CLR, _CLR  WCZ 'set Z = 0

IF_C   MOV     Carry,#1        'true  C = 1

IF_NC  MOV     Carry,#0        'false C = 0

IF_Z   MOV     Zero,#1         'true  Z = 1

IF_NZ  MOV     Zero,#0         'false Z = 0

debug("--------------------------------------")

debug("before NOP ",ubin(Zero),ubin(Carry))

NOP

IF_C   MOV     Carry,#1         'true  C = 1

IF_NC  MOV     Carry,#0         'false C = 0

IF_Z   MOV     Zero,#1          'true  Z = 1

IF_NZ  MOV     Zero,#0          'false Z = 0

debug("after NOP  ",ubin(Zero),ubin(Carry))

debug("--------------------------------------")

_Loop1                   NOP     'place holder cost 2 cycles

JMP     #_Loop1          'remember # imediate

Carry                 long      0

Zero                  long      0

MAX                  long      $FFFF_FFFF

MIN                   long      0

## 17.2) ROR Rotate Right and ROL Rotate Left

ROL D,{#}S {WC/WZ/WCZ}  Rotate left D.     Note: B31 will rotate back to B0

D = [63:32] of ({D[31:0], D[31:0]} << S[4:0]). C = last bit shifted out if S[4:0] > 0, else D[31]. *

ROR D,{#}S {WC/WZ/WCZ}  Rotate right.      Note: bit B0 will rotate back to B31

D = [32:63] of ({D[31:0], D[31:0]} >> S[4:0]). C = last bit shifted out if S[4:0] > 0, else D[0]. *

Interpertating the above command the destination register D = [32:63] It's an ordered bitwise assignment from D on leftside of "of", as a 64-bit word, to D on rightside leftside of "of", as a 32-bit word, with S specifying the alignmentshift. This is analogy of functional description. In reality, the variability of S means the logic gates needed to achieve that, in what's called a "barrel-shifter", is a pretty large structure.

Imagine you have taken the register to be shifted and placed a copy of that beside it so now you have 64 bits, with the top half the same as the bottom half. You then use the S register value to shift the whole thing right, and you then take the top half, which gives the same effect as a circular shift of 32 bits.
It should be noted that S can hold 0, which results in no shift, but can still set the flags which would put D[0] into the C flag for Rotate Right.

While a barrel shifter is fairly large compared to a simple shifter, it gives a speed advantage that scales with register size (a 31 bit shift takes the same time as a single bit shift, giving a x31 speed up)

ROR rotate right operand and ROL rotate left operand operate in the same manner shifting right or shifting left. The following description is for ROL rotate left.

Starting with 17.2.2_Example_WRD_ROL_0000001.

valPr0                    long      %10101010_10101010_10101010_10101010    'cog 1 Dest

valPr1                    long      %00000000_00000000_00000000_00000001    'cog 1 Src

D is register Dest, the hardware latches two copies of it to give 64 bits =

10101010_It then shifts bits left by the value in S the 64 bits =

01010101_01010101_01010101_01010101__01010101_01010101_01010101_0101010x

The result is then taken as bits 63 to 32 of this (bolded above) giving D =

01

For a double shift this example value is awkward as it appears to make no change. To illustrate it better the following  shows a variety of bit shifts with different a starting value

Dest = DEADBEEF = 11011110_10101101_10111110_11101111
64 bits in the shifter (preshift) is 11011110_10101101_10111110_11101111__11011110_10101101_10111110_11101111
A single shift gives 10111101_01011011_01111101_11011111__10111101_01011011_01111101_1101111x
A double shift gives 01111010_10110110_11111011_10111111__01111010_10110110_11111011_101111xx
A 20 bit shift gives 11101110_11111101_11101010_11011011__11101110_1111xxxx_xxxxxxxx_xxxxxxxx
A 31 bit shift gives 11101111_01010110_11011111_01110111__1xxxxxxx_xxxxxxxx_xxxxxxxx_xxxxxxxx

Please note that the bits at the right hand end are x (don't care) values, because it is not known what the chip sets these to, and it doesn't matter because they never leave the shifter. In fact, bits 31:0 out of the shifter may not be implemented in the silicon to save gates, power, and heat.

All of these shifts take the same amount of time to occur, as they are all performed in parallel with only the correct one being passed to the result register.

### 17.2.1_Example_WRD_R0R_002

ROR D,{#}S {WC/WZ/WCZ}  Rotate right.      Note: bit B0 will rotate back to B31

D = [31:0] of ({D[31:0], D[31:0]} >> S[4:0]). C = last bit shifted out if S[4:0] > 0, else D[0]. *

ROR will shift bits to the right and B0 gets shifted to B31 note that leading 0’s is not shown by debug

{{17.2.1_Example_WRD_ROR_002

ROR D,{#}S {WC/WZ/WCZ}  Rotate right.Note: bit B0 will rotate back to B31

D = [31:0] of ({D[31:0], D[31:0]} >> S[4:0]). C = last bit shifted out if S[4:0] > 0, else D[0]. *

ROR will shift bits to the right and B0 gets shifted to B31 note that leading 0 is not shown by debug}}

CON

_clkfreq = 200_000_000  ''Debug must be enabled clock must be greater than 10MHZ for Debug

Shift = 1,P0 = 0 'number of bits to shift and pin P0 to Blink

VAR

Byte cogRunning 'cog ID started is returned or -1 if not started

Byte PTRAvalue

PUB main()

debug("--------------------------------------")

debug("ROR D,{#}S {WC/WZ/WCZ}  Rotate right.Note: bit B0 will rotate back to B31 ")

debug("D = [31:0] of ({D[31:0], D[31:0]} >> S[4:0]). C = last bit shifted out if S[4:0] > 0, else D[0]. *")

debug("ROR will shift bits to the right and B0 gets shifted to B31 ")

debug(" note that leading 0 is not shown by debug")

debug("--------------------------------------")

PTRAvalue := $FFF_FFFF                               'PTRA coginit transfer register not used in example

cogRunning := COGINIT(COGEXEC_NEW,@S0_ROR,PTRAvalue)

debug(udec(cogRunning))

repeat                                               'keep cog 0 running

DAT

ORG   0

S0_ROR

MOV     Dest,valDest      'load cog 1 Pro

MOV     Src,valSRrc      'load cog 1 SRr

_Loop               debug("Before ROR  ",ubin(Dest),ubin(Src),ubin(Carry))

ROR     Dest,#Shift  WC     'Execute and shift number of bits in SRrc B0-> B31  Set C

debug("After  ROR  ",ubin(Dest),ubin(Src),ubin(Carry))

debug("------------------------------------")

SimpleDRVH     IF_C   DRVH    #P0             'set direction register DIRA P0 pin P0=1 and OUTA P0 = 1

IF_NC  DRVL    #P0

IF_C   MOV     Carry,#1

IF_NC  MOV     Carry,#0

Delay                           GETCT   cog1Cnt         'the counter value is now in in cog1CountValue

ADDCT1  cog1Cnt,cog1Wait

'the cog1Cnt + cog1WaitTime = CT1 result is placed in the CT1 event register

WAITCT1                 'wait for cog1 program counter to reach CT1

JMP     #_Loop

cog1Wait              long      400_000_000   'number of clock cycles to wait

cog1Cnt              long      0

valDest               long      %10101010_10101010_10101010_10101010    'cog 1 Dest

valSRrc               long      %00000000_00000000_00000000_00000001    'cog 1 SRrc

Dest                    long      1234

Src                       long      4321

Carry                   long      0

### 17.2.2_Example_WRD_ROL_003

ROL D,{#}S {WC/WZ/WCZ}  Rotate left.

D = [63:32] (Not sure ?) of ({D[31:0], D[31:0]} << S[4:0]). C = last bit shifted out if S[4:0] > 0, else D[31]. *

ROL will shift bits to the Left and B31 gets shifted to B0 note that leading 0 is not shown by debug

{{17.2.2_Example_WRD_ROL_003

ROL D,{#}S {WC/WZ/WCZ}  Rotate left.

D = [63:32] (Not sure ?) of ({D[31:0], D[31:0]} << S[4:0]). C = last bit shifted out if S[4:0] > 0, else D[31]. *

ROL will shift bits to the Left and B31 gets shifted to B0 note that leading 0 is not shown by debug}}

CON

_clkfreq = 200_000_000  ''Debug must be enabled clock must be greater than 10MHZ for Debug

Shift = 1,P0 = 0 'number of bits to shift and pin P0 to Blink

VAR

Byte cogRunning 'cog ID started is returned or -1 if not started

Byte PTRAvalue

PUB main()

debug("--------------------------------------")

debug("ROL D,{#}S {WC/WZ/WCZ}  Rotate left.")

debug("D = [63:32] (Not sure ?) of ({D[31:0], D[31:0]} << S[4:0]")

debug("C = last bit shifted out if S[4:0] > 0, else D[31]. *")

debug("ROL will shift bits to the Left and B31 gets shifted to B0 ")

debug(" note that leading 0 is not shown by debug}}")

debug("--------------------------------------")

PTRAvalue := $FFF_FFFF                                        'PTRA coginit transfer register not used in example

cogRunning := COGINIT(COGEXEC_NEW,@S0_ROL,PTRAvalue)

debug(udec(cogRunning))

repeat                                                       'keep cog 0 running

DAT

ORG   0

S0_ROL

debug("-----------------------------")

debug("Before ROL  ",ubin(Dest),ubin(Src),ubin(Carry))

_Loop                            ROL     Dest,#Shift  WC     'Execute and shift number of bits in Src B31-> B0  Set C

debug("After ROL   ",ubin(Dest),ubin(Src),ubin(Carry))

SimpleDRVH     IF_C   DRVH    #P0             'set direction register DIRA P0 pin P0=1 and OUTA P0 = 1

IF_NC  DRVL    #P0

If_C   MOV     Carry,#1

If_NC  MOV     Carry,#0

Delay                            GETCT   cog1Cnt         'the counter value is now in in cog1CountValue

ADDCT1  cog1Cnt,cog1Wait

'the cog1Cnt + cog1WaitTime = CT1 result is placed in the CT1 event register

WAITCT1                 'wait for cog1 program counter to reach CT1

JMP     #S0_ROL

cog1Wait          long      500_000_000    'number of clock cycles to wait

cog1Cnt             long      0

valDest               long      %10101010_10101010_10101010_10101010    'cog 1 Dest

valSrc                long      %00000000_00000000_00000000_00000001    'cog 1 Src

Dest                  long      1234

Src                   long      4321

Carry                 long      0

## 17.3) SHR Shift Right and SHL Shift Left

SHR D,{#}S{WC\WZ\WCZ} Shift Right

D = [31:0] of ({32'b0, D[31:0]} >> S[4:0]). C = last bit shifted out if S[4:0] > 0, else D[0]. *

SHL D,{#}S{WC\WZ\WCZ} Shift Left

Si31:0] > Analogy of 32 bit shift register with 0 being shifted in depending if SHR/SHL

Pr0

01010101_01010101_01010101_01010101

SHR Pr0,#1 WC

00101010_10101010_10101010_10101010  C = 1

Pr0

01010101_01010101_01010101_01010101

SHR Pr0,#2 WC

00010101_01010101_01010101_01010101   C = 0

Pr0

01010101_01010101_01010101_01010101

SHL Pr0,#1 WC

C =0    10101010_10101010_10101010_10101010

Pr0

01010101_01010101_01010101_01010101

SHL Pr0,#2 WC

C = 1   01010101_01010101_01010101_01010100

### 17.3.1_Example_WRD_SHR_004

SHR D,{#}S{WC\WZ\WCZ} Shift Right

D = [32:63] of ({32'b0, D[31:0]} >> S[4:0]). C = last bit shifted out if S[4:0] > 0, else D[0]. *

SHR will shift bits to the right 0 fills Bits shifted}}

{{17.3.1_Example_WRD_SHR_0004

SHR D,{#}S{WC\WZ\WCZ} Shift Right

D = [32:63] of ({32'b0, D[31:0]} >> S[4:0]). C = last bit shifted out if S[4:0] > 0, else D[0]. *

SHR will shift bits to the right 0 fills Bits shifted}}

CON

_clkfreq = 200_000_000  ''Debug must be enabled clock must be greater than 10MHZ for Debug

Shift = 1,P0 = 0 'number of bits to shift and pin P0 to Blink

VAR

Byte cogRunning 'cog ID started is returned or -1 if not started

Byte PTRAvalue

PUB main()

debug("--------------------------------------")

debug("SHR D,{#}S{WC\WZ\WCZ} Shift Right ")

debug("D = [32:63] of ({32'b0, D[31:0]} >> S[4:0]).")

debug("C = last bit shifted out if S[4:0] > 0, else D[0]. *")

debug("SHR will shift bits to the right 0 fills Bits shifted}}")

debug("--------------------------------------")

PTRAvalue := $FFF_FFFF                                        'PTRA coginit transfer register not used in example

Dest := 1234                                                   'cog 0 Dest register

Src := 4321                                                   'cog 0 Src register

cogRunning := COGINIT(COGEXEC_NEW,@S0_SHR,PTRAvalue)

debug(udec(cogRunning))

repeat                                                       'keep cog 0 running

DAT

ORG   0

S0_SHR

Mov        Dest,valDest      'load cog 1 Pro

MOV     Src,valSrc      'load cog 1 Src

debug(ubin(Dest))

_Loop                           SHR     Dest,#Shift  WC     'Execute and shift number of bits in Src B31-> B0  Set C

debug(ubin(Dest))

SimpleDRVH     IF_C   DRVH    #P0             'set direction register DIRA P0 pin P0=1 and OUTA P0 = 1

IF_NC  DRVL    #P0

Delay                            GETCT   cog1Cnt         'the counter value is now in in cog1CountValue

ADDCT1  cog1Cnt,cog1Wait

'the cog1Cnt + cog1WaitTime = CT1 result is placed in the CT1 event register

WAITCT1                 'wait for cog1 program counter to reach CT1

JMP     #_Loop

cog1Wait        long      500_000_000    'number of clock cycles to wait

cog1Cnt           long      0

Dest                  long      0

Src                   long      0

valDest           long      %10101010_10101010_10101010_10101010    'cog 1 Dest

valSrc              long      %00000000_00000000_00000000_00000001    'cog 1 Src

### 17.3.2_Example_WRD_SHL_005

SHL D,{#}S{WC\WZ\WCZ} Shift Left

D = [32:63] of ({32'b0, D[31:0]} >> S[4:0]). C = last bit shifted out if S[4:0] > 0, else D[0]. *

SHR will shift bits to the Left 0 fills Bits shifted}}

{{17.3.2_Example_WRD_SHL_005

SHL D,{#}S{WC\WZ\WCZ} Shift Left

D = [32:63] of ({32'b0, D[31:0]} >> S[4:0]). C = last bit shifted out if S[4:0] > 0, else D[0]. *

SHR will shift bits to the Left 0 fills Bits shifted}}

CON

_clkfreq = 200_000_000  ''Debug must be enabled clock must be greater than 10MHZ for Debug

Shift = 1,P0 = 0 'number of bits to shift and pin P0 to Blink

VAR

Byte cogRunning 'cog ID started is returned or -1 if not started

Byte PTRAvalue

PUB main()

debug("--------------------------------------")

debug("SHL D,{#}S{WC\WZ\WCZ} Shift Left")

debug("D = [32:63] of ({32'b0, D[31:0]} >> S[4:0]).")

debug("C = last bit shifted out if S[4:0] > 0, else D[0]. *")

debug("SHR will shift bits to the Left 0 fills Bits shifted}}")

debug("--------------------------------------")

PTRAvalue := $FFF_FFFF                                        'PTRA coginit transfer register not used in example

Dest := 1234                                                   'cog 0 Dest register

Src := 4321                                                   'cog 0 Src register

cogRunning := COGINIT(COGEXEC_NEW,@S0_SHL,PTRAvalue)

debug(udec(cogRunning))

repeat                                                       'keep cog 0 running

DAT

ORG   0

S0_SHL

Mov     Dest,valDest      'load cog 1 Pro

MOV     Src,valSrc      'load cog 1 Src

debug(ubin(Dest))

_Loop                           SHL     Dest,#Shift  WC     'Execute and shift number of bits in Src B31-> B0  Set C

debug(ubin(Dest))

SimpleDRVH     IF_C      DRVH    #P0             'set direction register DIRA P0 pin P0=1 and OUTA P0 = 1

IF_NC  DRVL    #P0

Delay                              GETCT   cog1Cnt         'the counter value is now in in cog1CountValue

ADDCT1  cog1Cnt,cog1Wait

'the cog1Cnt + cog1WaitTime = CT1 result is placed in the CT1 event register

WAITCT1                 'wait for cog1 program counter to reach CT1

JMP     #_Loop

cog1Wait      long      500_000_000    'number of clock cycles to wait

cog1Cnt        long      0

valDest          long      %10101010_10101010_10101010_10101010    'cog 1 Dest

valSrc             long      %00000000_00000000_00000000_00000001    'cog 1 Src

Dest               long      0

Src                  long      0

## 17.4) RCR Rotate Carrry Right and RCL Rotate Carry Left

RCR D,{#}S {WC/WZ/WCZ}  Rotate Carry Right.

. D = [31:0] of ({{32{C}}, D[31:0]} >> S[4:0]). C = last bit shifted out if S[4:0] > 0, else D[0]. *

RCL D,{#}S {WC/WZ/WCZ} Rotate Carry Left

D = [63:32] of ({D[31:0], {32{C}}} << S[4:0]). C = last bit shifted out if S[4:0] > 0, else D[31]. *

RR performs a rotate right of D destination field ,bit times as {#}S using the C flags original value each of th MSB’s affected.  If WC effect is applied C will be changed to match B0 of D destination original value. D has bits specified by {#}S of C rotated right into D. R performs a rotate Left of D destination field ,bit times as {#}S using the C flags original value each of th LSB’s affected.  If WC effect is applied C will be changed to match B31 of D destination original value. D has bits specified by {#}S of C rotated right into D.

### 17.4.1_Example_WRD_RCR_006

RCR D,{#}S {WC/WZ/WCZ}  Rotate Carry Right.

''D = [31:0] of ({32{C}}, D[31:0]} >> S[4:0]). C = last bit shifted out if S[4:0] > 0, else D[0]. *

{{17.4.1_Example_WRD_RCR_006}}

''RCR D,{#}S {WC/WZ/WCZ}  Rotate Carry Right.

''D = [31:0] of ({32{C}}, D[31:0]} >> S[4:0]). C = last bit shifted out if S[4:0] > 0, else D[0]. *

CON

_clkfreq = 200_000_000  ''Debug must be enabled clock must be greater than 10MHZ for Debug

Shift = 1,P0 = 0 'number of bits to shift and pin P0 to Blink

VAR

Byte cogRunning 'cog ID started is returned or -1 if not started

PUB main()

debug("--------------------------------------")

debug("RCR D,{#}S {WC/WZ/WCZ}  Rotate Carry Right")

debug("D = [31:0] of ({32{C}}, D[31:0]} >> S[4:0]).")

debug("C = last bit shifted out if S[4:0] > 0, else D[0]. * ")

debug("--------------------------------------")

cogRunning := COGINIT(COGEXEC_NEW,@S0_RCR,0)

debug(udec(cogRunning))

repeat                                                       'keep cog 0 running

DAT

ORG   0

S0_RCR                      MOV     Dest,valDest      'load cog 1 Pro

MOV     Src,valSrc      'load cog 1 Src

_Loop                 TESTB   Dest,#0  WC      'Test Dest B0 if 1 C = 1

IF_C   MOV     Carry,MAX

IF_NC  MOV     Carry,MIN

IF_C   DRVH    #P0

If_NC  DRVL    #P0

debug(uhex(Carry))

debug("before RCR",ubin(Dest))

RCR     Dest,#Shift

debug("after RCR",ubin(Dest))

WAITX  cog1Wait          'wait cog1Wait cycles

JMP     #_Loop

cog1Wait              long      500_000_000    'number of clock cycles to wait

cog1Cnt               long      0

valDest                long      %10101010_10101010_10101010_10101010    'cog 1 Dest

valSrc                long      %00000000_00000000_00000000_00000001    'cog 1 Src

Carry                 long      0

Zero                  long      0

Dest                  long      0

Src                   long      0

MAX                   long      $FFFF_FFFF

MIN                   long      0

### 17.4.2_Example_WRD_RCL_007

RCL D,{#}S {WC/WZ/WCZ} Rotate Carry Left

'D = [63:32] of ({D[31:0], {32{C}}} << S[4:0]). C = last bit shifted out if S[4:0] > 0, else D[31]. *

{{17.4.2_Example_WRD_RCL_007}}

''RCL D,{#}S {WC/WZ/WCZ} Rotate Carry Left

''D = [63:32] of ({D[31:0], {32{C}}} << S[4:0]). C = last bit shifted out if S[4:0] > 0, else D[31]. *

CON

_clkfreq = 200_000_000  ''Debug must be enabled clock must be greater than 10MHZ for Debug

Shift = 1,P31 = 31 'number of bits to shift and pin P0 to Blink

VAR

Byte cogRunning 'cog ID started is returned or -1 if not started

PUB main()

debug("--------------------------------------")

debug("RCL D,{#}S {WC/WZ/WCZ} Rotate Carry Left")

debug("'D = [63:32] of ({D[31:0], {32{C}}} << S[4:0]).")

debug("C = last bit shifted out if S[4:0] > 0, else D[31]. *")

debug("--------------------------------------")

cogRunning := COGINIT(COGEXEC_NEW,@S0_RCR,0)

debug(udec(cogRunning))

repeat                                                       'keep cog 0 running

DAT                     ORG   0

S0_RCR        MOV     Dest,valDest      'load cog 1 Pro

MOV     Src,valSrc      'load cog 1 Src

_Loop                 MODC   _CLR  WC      'Test Dest B0 if 1 C = 1

IF_C   MOV     Carry,MAX

IF_NC  MOV     Carry,MIN

IF_C   DRVH    #P31

If_NC  DRVL    #P31

debug(uhex(Carry))

debug("before RCR",ubin(Dest))

RCL     Dest,#Shift

debug("after RCR",ubin(Dest))

WAITX  cog1Wait

NOP

JMP     #_Loop

cog1Wait         long      500_000_000    'number of clock cycles to wait

cog1Cnt           long      0

valDest            long      %10101010_10101010_10101010_10101010    'cog 1 Dest

valSrc               long      %00000000_00000000_00000000_00000001    'cog 1 Src

Carry                long      0

Zero                 long      0

Dest                 long      0

Src                   long      0

MAX                long      $FFFF_FFFF

MIN                 long      0

## 17.5) SAR Shift Arithmetic Right and Shift Arithmetic Let

Shift Arithmetic Right can thought as dividing or Shift Arithmetic Left  Multiplying the value.

It maintains the sign value.

### 17.5.1) SAR Shigt Aritmetic Right

SAR D,{#}S {WC/WZ/WCZ}

D = [31:0] of ({{32{D[31]}}, D[31:0]} >> S[4:0]). C = last bit shifted out if S[4:0] > 0, else D[0]. *

Shift Arithmetic Right is a division of a signed binary number divided by 2

the sign bit remains unchanged

-------------------------------------------------------------------------------------------------------------------------------

Positve Number

00000000_00000000_00000000_00001101 Before Right Shift  = 13

Positive Number Shifted Right 0 Loaded Into MSB to Maintain Sign

00000000_00000000_00000000_00000110 After Shift Right     = 6 Result  is 13/2 =6 divide by 2

-------------------------------------------------------------------------------------------------------------------------------

Negative Number

11111111_11111111_11111111_00011000  Before Shift Right = -232

00000000_00000000_00000000_11100111  2’s complement

00000000_00000000_00000000_00000001

00000000_00000000_00000000_ 11101000  = 232

Negative Number Shifted 1 Loaded Into MSB to Maintain Sign

11111111_11111111_11111111_10001100   =  After Shift Right = -116 result is -232/2

00000000_00000000_00000000_01110011  2’S complement

00000000_00000000_00000000_00000001

00000000_00000000_00000000_01110100  = 116

-------------------------------------------------------------------------------------------------------------------------------------

#### 17.5.1_Example_WRD_SAR_008

SAR D,{#}S {WC/WZ/WCZ}

Shift arithmetic right. D = [31:0] of ({{32{D[31]}}, D[31:0]} >> S[4:0]). C = last bit shifted out if S[4:0] > 0, else D[0]. *

{{17.5.1_Example_WRD_SAR_008}}

''SAR D,{#}S {WC/WZ/WCZ}

''Shift arithmetic right.

''D = [31:0] of ({{32{D[31]}}, D[31:0]} >> S[4:0]). C = last bit shifted out if S[4:0] > 0, else D[0]. *

''Shift Arithmetic Right is a division of a signed binary number divided by 2

''the sign bit remains unchanged

CON

_clkfreq = 200_000_000  ''Debug must be enabled clock must be greater than 10MHZ for Debug

Shift = 1,P0 = 0 'number of bits to shift and pin P0 to Blink

VAR

Byte cogRunning 'cog ID started is returned or -1 if not started

PUB main()

debug("---------------------------------")

debug("SAR D,{#}S {WC/WZ/WCZ}")

debug("Shift arithmetic right.")

debug("D = [31:0] of ({{32{D[31]}}, D[31:0]} >> S[4:0]).")

debug("C = last bit shifted out if S[4:0] > 0, else D[0]. *")

debug("SAR is a division of a signed binary number divided by 2")

debug("the sign bit remains unchanged")

debug("----------------------------------")

cogRunning := COGINIT(COGEXEC_NEW,@S0_SAR,0)

debug(udec(cogRunning))

repeat                                                       'keep cog 0 running

DAT

ORG   0

S0_SAR

MOV     Dest,valDest      'load cog 1 Pro

MOV     Src,valSrc      'load cog 1 Src

debug("before SAR",ubin(Dest))

SAR     Dest,#Shift

IF_C   MOV     Carry,MAX

IF_NC  MOV     Carry,MIN

debug(uhex(Carry))

IF_C   DRVH    #P0

If_NC  DRVL    #P0

debug("after SAR",ubin(Dest))

_Loop                 NOP

JMP     #_Loop

cog1Wait              long     500_000_000    'number of clock cycles to wait

cog1Cnt               long      0

valDest                long      %10101010_10101010_10101010_10101010    'cog 1 Dest

valSrc                long      %00000000_00000000_00000000_00000001    'cog 1 Src

Carry                 long      0

Zero                  long      0

Dest                  long      0

Src                   long      0

MAX                   long      $FFFF_FFFF

MIN                   long      0

### 17.5.2) Shift Aritmentic Left

SAL D,{#}S {WC/WZ/WCZ}

Shift arithmetic left. D = [63:32] of ({D[31:0], {32{D[0]}}} << S[4:0]). C = last bit shifted out if S[4:0] > 0, else D[31]. *

Shift Arithmetic Left is a multiplication of a signed binary number by 2

The sign bit remains unchanged

----------------------------------------------------------------------------------------------------------------------------------------

Positve Number

00000000_00000000_00000000_00001101 Before Right Left  =  13

Positive Number Shifted Left 0 Loaded into LSB

00000000_00000000_00000000_00011010 After Left Shift       =  26    Multiply by 2

------------------------------------------------------------------------------------------------------------------------------------------

Negative Number

11111111_11111111_11111111_00011000  Before Shift Left = -232

00000000_00000000_00000000_11100111  2’s complement

00000000_00000000_00000000_00000001

00000000_00000000_00000000_ 11101000  = 232

Negative Number Shifted Left 0 Loaded into LSB

11111111_11111111_11111110_00110000  = After Shift Left = -464 result is 2 x -232

00000000_00000000_00000001_11001111 2’s complement

00000000_00000000_00000000_00000001

00000000_00000000_00000001_11010000 = 464

#### 17.5.2_Example_WRD_SAL_009

SAL D,{#}S {WC/WZ/WCZ}

Shift arithmetic left. D = [63:32] of ({D[31:0], {32{D[0]}}} << S[4:0]). C = last bit shifted out if S[4:0] > 0, else D[31]. *

{{17.5.2_Example_WRD_SAL_009}}

''SAL D,{#}S {WC/WZ/WCZ}

''Shift arithmetic left.

''D = [63:32] of ({D[31:0], {32{D[0]}}} << S[4:0]). C = last bit shifted out if S[4:0] > 0, else D[31]. *

''Shift Arithmetic Left is a multiplication of a signed binary number by 2

''The sign bit remains unchanged

CON

_clkfreq = 200_000_000  ''Debug must be enabled clock must be greater than 10MHZ for Debug

Shift = 1,P31 = 31 'number of bits to shift and pin P0 to Blink

VAR

Byte cogRunning 'cog ID started is returned or -1 if not started

PUB main()

debug("--------------------------------")

debug("SAL D,{#}S {WC/WZ/WCZ}")

debug("Shift arithmetic left.")

debug("D = [63:32] of ({D[31:0], {32{D[0]}}} << S[4:0])")

debug("C = last bit shifted out if S[4:0] > 0, else D[31]. *")

debug("Shift Arithmetic Left is a multiplication of a signed binary number by 2")

debug("The sign bit remains unchanged")

debug("---------------------------------")

cogRunning := COGINIT(COGEXEC_NEW,@S0_SAL,0)

debug(udec(cogRunning))

repeat                                                       'keep cog 0 running

DAT

ORG   0

S0_SAL

MOV     Dest,valDest      'load cog 1 Pro

MOV     Src,valSrc      'load cog 1 Src

debug("before SAL",ubin(Dest))

SAL     Dest,#Shift

IF_C   MOV     Carry,MAX

IF_NC  MOV     Carry,MIN

debug(uhex(Carry))

IF_C   DRVH    #P31

If_NC  DRVL    #P31

debug("after SAL",ubin(Dest))

_Loop                 NOP

JMP     #_Loop

cog1Wait              long      500_000_000    'number of clock cycles to wait

cog1Cnt               long      0

valDest             long      %10101010_10101010_10101010_10101010    'cog 1 Dest

valSrc                long      %00000000_00000000_00000000_00000001    'cog 1 Src

Carry                 long      0

Zero                  long      0

Dest                  long      0

Src                     long      0

MAX                  long      $FFFF_FFFF

MIN                   long      0

## 17.6)ADD Addition

| ADD D,{#}S {WC/WZ/WCZ} | Add S into D. D = D + S. C = carry of (D + S). * |
| --- | --- |
| ADDX D,{#}S {WC/WZ/WCZ} | Add (S + C) into D, extended. D = D + S + C. C = carry of (D + S + C). Z = Z AND (result == 0). |
| ADDS D,{#}S {WC/WZ/WCZ} | Add S into D, signed. D = D + S. C = correct sign of (D + S). * |
| ADDSX D,{#}S {WC/WZ/WCZ} | Add (S + C) into D, signed and extended. D = D + S + C. C = correct sign of (D + S + C). Z = Z AND (result == 0). |

### 17.6.1_Example_WRD_ADD_010

ADD D,{#}S {WC/WZ/WCZ}

Add S into D. D = D + S.   C = carry of (D + S). *

Unsigned Addition

{{17.6.1_Example_WRD_ADD_010}}

''ADD D,{#}S {WC/WZ/WCZ}

''Add S into D. D = D + S. C = carry of (D + S). *

''if D + S greater than register Carry = 1

CON

_clkfreq = 200_000_000  ''Debug must be enabled clock must be greater than 10MHZ for Debug

P0 =0 , P1 = 1  , P2 = 2

VAR

Byte cogRunning 'cog ID started is returned or -1 if not started

PUB main()

cogRunning := COGINIT(COGEXEC_NEW,@S0_ADD,0)

debug(udec(cogRunning))

repeat                                                       'keep cog 0 running

DAT

ORG   0

S0_ADD                    DRVH    #P0

MOV     Pr0,valPr0      'load cog 1 Pro

MOV     Pr1,valPr1      'load cog 1 Pr1

_Loop1                     NOP

debug("before ADD ",ubin(Pr0))

ADD     Pr0,Pr1  WC      'register indirect

IF_C     MOV     Carry,MAX

IF_NC   MOV     Carry,MIN

debug(uhex(Carry))

IF_C     JMP     #_NewLoop2       'remeber # imediate

debug("after ADD ",ubin(Pr0))

WAITX  cog1Wait          'wait cog1Wait cycles

JMP     #_Loop1          'remember # imediate

_NewLoop2              DRVH    #P1

DRVL    #P0

MOV     Pr0,valPr0

_Loop2                      NOP

debug("before ADD ",ubin(Pr0))

ADD     Pr0,#2 WC           'maximum imediate value is 511

debug("after ADD ",ubin(Pr0))

WAITX   cog1Wait          'wait cog1Wait cycles

IF_C     MOV     Pr0,ValPr0        'carry generate when 2 added to #FFFF_FFFF

IF_C      DRVH    #P2

IF_C      DRVL    #P1

JMP     #_Loop2          'remember # imediate

cog1Wait          long      300_000_000    'number of clock cycles to wait

cog1Cnt            long      0

valPr0               long      $FFFF_FFF0                                       'cog 1 Pr0

valPr1               long      %00000000_00000000_00000000_00000001    'cog 1 Pr1

Carry                 long      0

Zero                  long      0

MAX                  long      $FFFF_FFFF

MIN                   long      0

### 17.6.2_Example_WRD_ADDX_011

ADDX D,{#}S {WC/WZ/WCZ}

Add (S + C) into D, extended. D = D + S + C. C = carry of (D + S + C). Z = Z AND (result == 0).

{{17.6.2_Example_WRD_ADDX_011}}

''ADDX D,{#}S {WC/WZ/WCZ}

''Add (S + C) into D, extended. D = D + S + C. C = carry of (D + S + C). Z = Z AND (result == 0).

CON

_clkfreq = 200_000_000  ''Debug must be enabled clock must be greater than 10MHZ for Debug

P0 =0 , P1 = 1  , P2 = 2

VAR

Byte cogRunning 'cog ID started is returned or -1 if not started

PUB main()

cogRunning := COGINIT(COGEXEC_NEW,@S0_ADDX,0)

debug(udec(cogRunning))

repeat                                                       'keep cog 0 running

DAT

ORG   0

S0_ADDX               DRVH    #P0

MOV     Pr0,MAX      'load cog 1 Pro

MOV     Pr1,valPr1      'load cog 1 Pr1

NOP

debug("before ADD ",ubin(Pr0))

ADD     Pr0,Pr1  WCZ    'register indirect will set C =1 Z = 1

IF_C   MOV     Carry,MAX   'MAX is true  C = 1

IF_NC  MOV     Carry,MIN   'MIN is false C = 1

IF_Z   MOV     Zero,MAX    'MAX is true  Z = 1

IF_NZ  MOV     Zero,MIN    'MIN is fals  Z =0

_Loop1                debug(uhex(Carry),uhex(Zero))

debug("Pr0 before ADDX = ",ubin(Pr0))

ADDX    Pr0,Pr1     'at start of _Loop1 Pr0 = 0 Pr1 = 1  C=1 Result Pro =2

debug("Pr0 after ADDX = ",ubin(Pr0))

WAITX  cog1Wait          'wait cog1Wait cycles

NOP

JMP     #_Loop1          'remember # imediate

cog1Wait              long      900_000_000    'number of clock cycles to wait

cog1Cnt               long      0

valPr0                long      $FFFF_FFF0                                       'cog 1 Pr0

valPr1                long      %00000000_00000000_00000000_00000001    'cog 1 Pr1

Carry                 long      0

Zero                  long      0

MAX                   long      $FFFF_FFFF

MIN                   long      0

### 17.6.3_Example_WRD_ADDS_012

ADDS D,{#}S {WC/WZ/WCZ}

Add S into D, signed. D = D + S.

C = correct sign of (D + S). * Signed Addition

{{17.6.3_Example_WRD_ADDS_012}}

''ADDS D,{#}S {WC/WZ/WCZ}

''Add S into D, signed. D = D + S. C = correct sign of (D + S). *

''Signed Addition

CON

_clkfreq = 200_000_000  ''Debug must be enabled clock must be greater than 10MHZ for Debug

P0 =0 , P1 = 1  , P2 = 2

VAR

Byte cogRunning 'cog ID started is returned or -1 if not started

PUB main()

cogRunning := COGINIT(COGEXEC_NEW,@S0_ADDS,0)

debug(udec(cogRunning))

repeat                                                       'keep cog 0 running

DAT

ORG   0

S0_ADDS               DRVH    #P0

MOV     Pr0,#1      'load cog 1 Pro = 1

MOV     Pr1,valPr1   'load cog 1 Pr1 = 1

MODC    _CLR   WC    'set C = 1

IF_C   MOV     Carry,#1   'true  C = 1

IF_NC  MOV     Carry,#0   'false C = 0

debug("--------------------------------------")

debug("before ADDS ",sdec(Pr0),sdec(Pr1),ubin(Carry))

ADDS     Pr0,Pr1  WC   'add D + S + C

IF_C   MOV     Carry,#1   'true  C = 1

IF_NC  MOV     Carry,#0   'false C = 0

debug(ubin(Pr0))

debug("after ADDS = ",sdec(Pr0),sdec(Pr1),ubin(Carry))

debug("--------------------------------------")

WAITX   cog1Wait

WAITX   cog1Wait

MOV     Pr0,MAX      'load cog 1 Pro = $FFFF_FFFF

MOV     Pr1,valPr1   'load cog 1 Pr1 = 1

MODC    _CLR   WC    'set C = 1

IF_C   MOV     Carry,#1   'true  C = 1

IF_NC  MOV     Carry,#0   'false C = 0

_Loop1                debug("before ADDS ",sdec(Pr0),sdec(Pr1),ubin(Carry))

ADDS     Pr0,Pr1  WC   'add D + S + C

IF_C   MOV     Carry,#1   'true  C = 1

IF_NC  MOV     Carry,#0   'false C = 0

debug(ubin(Pr0))

debug("after ADDS = ",sdec(Pr0),sdec(Pr1),ubin(Carry))

debug("--------------------------------------")

WAITX  cog1Wait

WAITX  cog1Wait          'wait cog1Wait cycles

NOP

JMP     #_Loop1          'remember # imediate

cog1Wait          long      900_000_000    'number of clock cycles to wait

cog1Cnt            long      0

valPr0                long      $FFFF_FFFF    ' or signed -1 cog 1 Pr0

valPr1                long      -1    'cog 1 Pr1

Carry                 long      0

Zero                   long      0

MAX                   long      $FFFF_FFFF

MIN                   long      0

### 17.6.4_Example_WRD_ADDSX_013

ADDSX D,{#}S {WC/WZ/WCZ}

Add (S + C) into D, signed and extended.

D = D + S + C. C = correct sign of (D + S + C). Z = Z AND (result == 0).

{{17.6.3_Example_WRD_ADDS_013}}

''ADDSX D,{#}S {WC/WZ/WCZ}

''Add (S + C) into D, signed and extended.

''D = D + S + C. C = correct sign of (D + S + C). Z = Z AND (result == 0).

CON

_clkfreq = 200_000_000  ''Debug must be enabled clock must be greater than 10MHZ for Debug

P0 =0 , P1 = 1  , P2 = 2

VAR

Byte cogRunning 'cog ID started is returned or -1 if not started

PUB main()

cogRunning := COGINIT(COGEXEC_NEW,@S0_ADDSX,0)

debug(udec(cogRunning))

repeat                                                       'keep cog 0 running

DAT

ORG   0

S0_ADDSX   DRVH    #P0

MOV     Pr0,#1      'load cog 1 Pro = 1

MOV     Pr1,valPr1   'load cog 1 Pr1 = 1

MODC    _SET   WC    'set C = 1

IF_C   MOV     Carry,#1   'true  C = 1

IF_NC  MOV     Carry,#0   'false C = 0

debug("--------------------------------------")

debug("before ADDSX ",sdec(Pr0),sdec(Pr1),ubin(Carry))

ADDSX   Pr0,Pr1  WC   'add D + S + C

IF_C   MOV     Carry,#1   'true  C = 1

IF_NC  MOV     Carry,#0   'false C = 0

debug(ubin(Pr0))

debug("after ADDSX = ",sdec(Pr0),sdec(Pr1),ubin(Carry))

debug("--------------------------------------")

WAITX   cog1Wait

WAITX   cog1Wait

MOV     Pr0,MAX      'load cog 1 Pro = $FFFF_FFFF

MOV     Pr1,valPr1   'load cog 1 Pr1 = 1

MODC    _SET   WC    'set C = 1

IF_C   MOV     Carry,#1   'true  C = 1

IF_NC  MOV     Carry,#0   'false C = 0

_Loop1                debug("before ADDS ",sdec(Pr0),sdec(Pr1),ubin(Carry))

ADDSX     Pr0,Pr1  WC   'add D + S + C

IF_C   MOV     Carry,#1   'true  C = 1

IF_NC  MOV     Carry,#0   'false C = 0

debug(ubin(Pr0))

debug("after ADDS = ",sdec(Pr0),sdec(Pr1),ubin(Carry))

debug("--------------------------------------")

WAITX  cog1Wait

WAITX  cog1Wait          'wait cog1Wait cycles

NOP

JMP     #_Loop1          'remember # imediate

cog1Wait          long      900_000_000    'number of clock cycles to wait

cog1Cnt            long      0

valPr0                long      $FFFF_FFFF    ' or signed -1 cog 1 Pr0

valPr1                long      -1    'cog 1 Pr1

Carry                 long      0

Zero                  long      0

MAX                  long      $FFFF_FFFF

MIN                   long      0

## 17.7) SUB Subtraction

| SUB D,{#}S {WC/WZ/WCZ} | Subtract S from D. D = D - S. C = borrow of (D - S). * |
| --- | --- |
| SUBX D,{#}S {WC/WZ/WCZ} | Subtract (S + C) from D, extended. D = D - (S + C). C = borrow of (D - (S + C)). Z = Z AND (result == 0). |
| SUBS D,{#}S {WC/WZ/WCZ} | Subtract S from D, signed. D = D - S. C = correct sign of (D - S). * |
| SUBSX D,{#}S {WC/WZ/WCZ} | Subtract (S + C) from D, signed and extended. D = D - (S + C). C = correct sign of (D - (S + C)). Z = Z AND (result == 0). |

### 17.7.1_Example_WRD_SUB_014

SUB D,{#}S {WC/WZ/WCZ}

Subtract S from D. D = D - S. C = borrow of (D - S). *

Unsigned Subtraction

{{17.7.1_Example_WRD_SUB_014}}

''SUB D,{#}S {WC/WZ/WCZ}

''Subtract S from D. D = D - S. C = borrow of (D - S). *

''unsigned subtraction

CON

_clkfreq = 200_000_000  ''Debug must be enabled clock must be greater than 10MHZ for Debug

P0 =0 , P1 = 1  , P2 = 2

VAR

Byte cogRunning 'cog ID started is returned or -1 if not started

PUB main()

debug("--------------------------------------")

debug("SUB D,{#}S {WC/WZ/WCZ}")

debug("Subtract S from D. D = D - S. C = borrow of (D - S). * ")

debug("unsigned subtraction")

debug("--------------------------------------")

cogRunning := COGINIT(COGEXEC_NEW,@S0_SUB,0)

debug(udec(cogRunning))

repeat                                                       'keep cog 0 running

DAT

ORG   0

S0_SUB                DRVH    #P0

MOV     Dest,#1      'load cog 1 Pro = 1

MOV     Src,valSrc   'load cog 1 Src = 1

MODC    _CLR   WC    'set C = 0

IF_C   MOV     Carry,#1   'true  C = 1

IF_NC  MOV     Carry,#0   'false C = 0

debug("--------------------------------------")

debug("before SUB  ",sdec(Dest),sdec(Src),ubin(Carry))

SUB   Dest,Src  WC   'add D + S + C

IF_C   MOV     Carry,#1   'true  C = 1

IF_NC  MOV     Carry,#0   'false C = 0

debug(ubin(Dest))

debug("after SUB  ",sdec(Dest),sdec(Src),ubin(Carry))

debug("--------------------------------------")

MOV     Dest,MAX      'load cog 1 Pro = $FFFF_FFFF

MOV     Src,valSrc   'load cog 1 Src = 1

MODC    _CLR   WC    'set C = 0

IF_C   MOV     Carry,#1   'true  C = 1

IF_NC  MOV     Carry,#0   'false C = 0

debug("before SUB ",sdec(Dest),sdec(Src),ubin(Carry))

SUB     Dest,Src  WC   'add D + S + C

IF_C   MOV     Carry,#1   'true  C = 1

IF_NC  MOV     Carry,#0   'false C = 0

debug(ubin(Dest))

debug("after SUB  ",sdec(Dest),sdec(Src),ubin(Carry))

debug("--------------------------------------")

WAITX  cog1Wait

WAITX  cog1Wait          'wait cog1Wait cycles

_Loop1               NOP

JMP     #_Loop1          'remember # imediate

cog1Wait              long      900_000_000    'number of clock cycles to wait

cog1Cnt               long      0

valDest                long      $FFFF_FFFF    ' or signed -1 cog 1 Dest

valSrc                long      2    'cog 1 Src

Carry                 long      0

Zero                  long      0

Dest                  long      0

Src                   long      0

MAX                   long      $FFFF_FFFF

MIN                   long      0

### 17.7.2_Example_WRD_SUBX_015

SUBSD ={{17.7.2_Example_WRD_SUBX_015}}

''SUBX    D,{#}S   {WC/WZ/WCZ}

''Subtract (S + C) from D, extended.

''D = D - (S + C).  C = borrow of (D - (S + C)). Z = Z AND (result == 0).

CON

_clkfreq = 200_000_000  ''Debug must be enabled clock must be greater than 10MHZ for Debug

P0 =0 , P1 = 1  , P2 = 2

VAR

Byte cogRunning 'cog ID started is returned or -1 if not started

PUB main()

debug("--------------------------------------")

debug("SUBX    D,{#}S   {WC/WZ/WCZ}")

debug("Subtract (S + C) from D, extended.")

debug("D = D - (S + C).  C = borrow of (D - (S + C))")

debug("Z = Z AND (result == 0)")

debug("--------------------------------------")

cogRunning := COGINIT(COGEXEC_NEW,@S0_SUBX,0)

debug(udec(cogRunning))

repeat                                                       'keep cog 0 running

DAT

ORG   0

S0_SUBX               DRVH    #P0          'program running

MOV     Dest,#1      'load cog 1 Pro = 1

MOV     Src,valSrc   'load cog 1 Src = 1

MODC    _SET   WC    'set C = 1

IF_C   MOV     Carry,#1   'true  C = 1

IF_NC  MOV     Carry,#0   'false C = 0

debug("--------------------------------------")

debug("before SUB  ",sdec(Dest),sdec(Src),ubin(Carry))

SUBX   Dest,Src  WC   'add D + S + C

IF_C   MOV     Carry,#1   'true  C = 1

IF_NC  MOV     Carry,#0   'false C = 0

debug(ubin(Dest))

debug("after SUB  ",sdec(Dest),sdec(Src),ubin(Carry))

debug("--------------------------------------")

MOV     Dest,MAX      'load cog 1 Pro = $FFFF_FFFF

MOV     Src,valSrc   'load cog 1 Src = 1

MODC    _SET   WC    'set C = 1

IF_C   MOV     Carry,#1   'true  C = 1

IF_NC  MOV     Carry,#0   'false C = 0

debug("before SUB ",sdec(Dest),sdec(Src),ubin(Carry))

SUBX     Dest,Src  WC   'add D + S + C

IF_C   MOV     Carry,#1   'true  C = 1

IF_NC  MOV     Carry,#0   'false C = 0

debug(ubin(Dest))

debug("after SUB  ",sdec(Dest),sdec(Src),ubin(Carry))

debug("--------------------------------------")

WAITX  cog1Wait

WAITX  cog1Wait          'wait cog1Wait cycles

_Loop1                NOP

JMP     #_Loop1          'remember # imediate

cog1Wait              long      900_000_000    'number of clock cycles to wait

cog1Cnt               long      0

valDest               long      $FFFF_FFFF    ' or signed -1 cog 1 Dest

valSrc                long      2    'cog 1 Src

Carry                 long      0

Zero                  long      0

Dest                  long      0

Src                   long      0

MAX                   long      $FFFF_FFFF

MIN                   long      0

### 17.7.3_Example_WRD_SUBS_016

SU SuD Signed subtraction

{{17.7.3_Example_WRD_SUBS_016}}

''SUBS    D,{#}S   {WC/WZ/WCZ}

''Subtract S from D, signed.

''D = D - S.   C = correct sign of (D - S).*

''Signed subtraction

CON

_clkfreq = 200_000_000  ''Debug must be enabled clock must be greater than 10MHZ for Debug

P0 =0 , P1 = 1  , P2 = 2

VAR

Byte cogRunning 'cog ID started is returned or -1 if not started

PUB main()

debug("--------------------------------------")

debug("SUBS    D,{#}S   {WC/WZ/WCZ}")

debug("Subtract S from D, signed.")

debug("D = D - S.   C = correct sign of (D - S).*")

debug("Signed subtraction")

debug("--------------------------------------")

cogRunning := COGINIT(COGEXEC_NEW,@S0_SUBS,0)

debug(udec(cogRunning))

repeat                                                       'keep cog 0 running

DAT

ORG   0

S0_SUBS               DRVH    #P0

MOV     Dest,#1      'load cog 1 Pro = 1

MOV     Src,valSrc   'load cog 1 Src = 1

MODC    _CLR   WC    'set C = 0

IF_C   MOV     Carry,#1   'true  C = 1

IF_NC  MOV     Carry,#0   'false C = 0

debug("--------------------------------------")

debug("before SUBS  ",sdec(Dest),sdec(Src),ubin(Carry))

SUBS   Dest,Src  WC   'add D + S + C

IF_C   MOV     Carry,#1   'true  C = 1

IF_NC  MOV     Carry,#0   'false C = 0

debug(ubin(Dest))

debug("after SUBS  ",sdec(Dest),sdec(Src),ubin(Carry))

debug("--------------------------------------")

MOV     Dest,MAX      'load cog 1 Pro = $FFFF_FFFF

MOV     Src,valSrc   'load cog 1 Src = 1

MODC    _CLR   WC    'set C = 0

IF_C   MOV     Carry,#1   'true  C = 1

IF_NC  MOV     Carry,#0   'false C = 0

debug("before SUBS ",sdec(Dest),sdec(Src),ubin(Carry))

SUBS     Dest,Src  WC   'add D + S + C

IF_C   MOV     Carry,#1   'true  C = 1

IF_NC  MOV     Carry,#0   'false C = 0

debug(ubin(Dest))

debug("after SUBS  ",sdec(Dest),sdec(Src),ubin(Carry))

debug("--------------------------------------")

WAITX  cog1Wait

WAITX  cog1Wait          'wait cog1Wait cycles

_Loop1               NOP

JMP     #_Loop1          'remember # imediate

cog1Wait              long      900_000_000    'number of clock cycles to wait

cog1Cnt               long      0

valDest                long      $FFFF_FFFF    ' or signed -1 cog 1 Dest

valSrc                long      2    'cog 1 Src

Carry                 long      0

Zero                  long      0

Dest                  long      0

Src                   long      0

MAX                   long      $FFFF_FFFF

MIN                   long      0

### 17.7.4_Example_WRD_SUBSX_017

SUBSX   D,{#}S   {WC/WZ/WCZ}

Sub C

{{17.7.4_Example_WRD_SUBSX_017}}

''SUBSX   D,{#}S   {WC/WZ/WCZ}

''Subtract (S + C) from D, signed and extended.

''D = D - (S + C).  C = correct sign of (D - (S + C)).  Z = Z AND (result == 0).

CON

_clkfreq = 200_000_000  ''Debug must be enabled clock must be greater than 10MHZ for Debug

P0 =0 , P1 = 1  , P2 = 2

VAR

Byte cogRunning 'cog ID started is returned or -1 if not started

PUB main()

debug("--------------------------------------")

debug("SUBSX   D,{#}S   {WC/WZ/WCZ}")

debug("Subtract (S + C) from D, signed and extended.")

debug("D = D - (S + C).  C = correct sign of (D - (S + C))")

debug("Z = Z AND (result == 0).   ")

debug("--------------------------------------")

cogRunning := COGINIT(COGEXEC_NEW,@S0_SUBSX,0)

debug(udec(cogRunning))

repeat                                                       'keep cog 0 running

DAT

ORG   0

S0_SUBSX              DRVH    #P0

MOV     Dest,#1      'load cog 1 Pro = 1

MOV     Src,valSrc   'load cog 1 Src = 1

MODC    _SET   WC    'set C = 1

IF_C   MOV     Carry,#1     'true  C = 1

IF_NC  MOV     Carry,#0     'false C = 0

debug("--------------------------------------")

debug("before SUBSX  ",sdec(Dest),sdec(Src),ubin(Carry))

SUBSX   Dest,Src  WC   'add D + S + C

IF_C   MOV     Carry,#1      'true  C = 1

IF_NC  MOV     Carry,#0      'false C = 0

debug(ubin(Dest))

debug("after SUBSX  ",sdec(Dest),sdec(Src),ubin(Carry))

debug("--------------------------------------")

MOV     Dest,MAX       'load cog 1 Pro = $FFFF_FFFF

MOV     Src,valSrc    'load cog 1 Src = 1

MODC    _SET   WC     'set C = 1

IF_C   MOV     Carry,#1      'true  C = 1

IF_NC  MOV     Carry,#0      'false C = 0

debug("before SUBSX ",sdec(Dest),sdec(Src),ubin(Carry))

SUBSX     Dest,Src  WC  'add D + S + C

IF_C   MOV     Carry,#1       'true  C = 1

IF_NC  MOV     Carry,#0       'false C = 0

debug(ubin(Dest))

debug("after SUBSX  ",sdec(Dest),sdec(Src),ubin(Carry))

debug("--------------------------------------")

WAITX  cog1Wait

WAITX  cog1Wait         'wait cog1Wait cycles

_Loop1               NOP

JMP     #_Loop1         'remember # imediate

cog1Wait              long      900_000_000   'number of clock cycles to wait

cog1Cnt               long      0

valDest               long      $FFFF_FFFF    'cog 1 Dest

valSrc                long      2             'cog 1 Src

Carry                 long      0

Zero                  long      0

Dest                  long      0

Src                   long      0

MAX                   long      $FFFF_FFFF

MIN                   long      0

### 17.7.5_Example_WRD_SUBR_018

SUBR    D,{#}S   {WC/WZ/WCZ}

Subtract D from S (reverse). D = S - D.

C = borrow of (S - D).

*

{{17.7.5_Example_WRD_SUBR_018}}

''SUBR    D,{#}S   {WC/WZ/WCZ}  Note: this is part of CMP seq of instruction 0000022

''Subtract D from S (reverse). D = S - D.

''C = borrow of (S - D).

CON

_clkfreq = 200_000_000  ''Debug must be enabled clock must be greater than 10MHZ for Debug

P0 =0 , P1 = 1  , P2 = 2

VAR

Byte cogRunning 'cog ID started is returned or -1 if not started

PUB main()

debug("--------------------------------------")

debug("SUBR    D,{#}S   {WC/WZ/WCZ} ")

debug("Subtract D from S (reverse). D = S - D.")

debug("C = borrow of (S - D)")

debug("--------------------------------------")

cogRunning := COGINIT(COGEXEC_NEW,@S0_SUBR,0)

debug(udec(cogRunning))

repeat                                                       'keep cog 0 running

DAT

ORG   0

S0_SUBR               DRVH    #P0

MOV     Dest,#1      'load cog 1 Pro = 1

MOV     Src,valSrc   'load cog 1 Src = 1

MODC    _SET   WC    'set C = 1

IF_C   MOV     Carry,#1     'true  C = 1

IF_NC  MOV     Carry,#0     'false C = 0

debug("--------------------------------------")

debug("before SUBSX  ",sdec(Dest),sdec(Src),ubin(Carry))

SUBR   Dest,Src  WC   'add D + S + C

IF_C   MOV     Carry,#1      'true  C = 1

IF_NC  MOV     Carry,#0      'false C = 0

debug(ubin(Dest))

debug("after SUBSX  ",sdec(Dest),sdec(Src),ubin(Carry))

debug("--------------------------------------")

MOV     Dest,MAX       'load cog 1 Pro = $FFFF_FFFF

MOV     Src,valSrc    'load cog 1 Src = 1

MODC    _SET   WC     'set C = 1

IF_C   MOV     Carry,#1      'true  C = 1

IF_NC  MOV     Carry,#0      'false C = 0

debug("before SUBSX ",sdec(Dest),sdec(Src),ubin(Carry))

SUBR     Dest,Src  WC  'add D + S + C

IF_C   MOV     Carry,#1       'true  C = 1

IF_NC  MOV     Carry,#0       'false C = 0

debug(ubin(Dest))

debug("after SUBSX  ",sdec(Dest),sdec(Src),ubin(Carry))

debug("--------------------------------------")

WAITX  cog1Wait

WAITX  cog1Wait         'wait cog1Wait cycles

_Loop1                NOP

JMP     #_Loop1         'remember # imediate

cog1Wait         long      900_000_000   'number of clock cycles to wait

cog1Cnt            long      0

valDest             long      $FFFF_FFFF    'cog 1 Dest

valSrc                long      2             'cog 1 Src

Carry                 long      0

Zero                  long      0

Dest                  long      0

Src                     long      0

MAX                  long      $FFFF_FFFF

MIN                   long      0

## 17.8) CMP Compare

| CMP D,{#}S {WC/WZ/WCZ} | Compare D to S. C = borrow of (D - S). Z = (D == S). |
| --- | --- |
| CMPX D,{#}S {WC/WZ/WCZ} | Compare D to (S + C), extended. C = borrow of (D - (S + C)). Z = Z AND (D == S + C). |
| CMPS D,{#}S {WC/WZ/WCZ} | Compare D to S, signed. C = correct sign of (D - S). Z = (D == S). |
| CMPSX D,{#}S {WC/WZ/WCZ} | Compare D to (S + C), signed and extended. C = correct sign of (D - (S + C)). Z = Z AND (D == S + C). |
| CMPR D,{#}S {WC/WZ/WCZ} | Compare S to D (reverse). C = borrow of (S - D). Z = (D == S). |
| CMPM D,{#}S {WC/WZ/WCZ} | Compare D to S, get MSB of difference into C. C = MSB of (D - S). Z = (D == S). |

Compare D register and S register with C/Z conditions entry flags and set flags accordingly.

### 17.8.1_Example_WRD_CMP_019

CMP D,{#}S {WC/WZ/WCZ}

Compare D to S. C = borrow of (D - S). Z = (D == S).

{{17.8.1_Example_WRD_CMP_019}}

''CMP D,{#}S {WC/WZ/WCZ}

''Compare D to S. C = borrow of (D - S). Z = (D == S)= 1 if equal

CON

_clkfreq = 200_000_000  ''Debug must be enabled clock must be greater than 10MHZ for Debug

P0 =0 , P1 = 1  , P2 = 2

VAR

Byte cogRunning 'cog ID started is returned or -1 if not started

PUB main()

debug("--------------------------------------")

debug("CMP D,{#}S {WC/WZ/WCZ}")

debug("Compare D to S. C = borrow of (D - S). Z = (D == S)= 1 if equal")

debug("--------------------------------------")

cogRunning := COGINIT(COGEXEC_NEW,@S0_CMP,0)

debug(udec(cogRunning))

repeat                                    'keep cog 0 running

DAT

ORG   0

S0_CMP       DRVH    #P0             'P0 on program running

MOV     Dest,valDest      'load cog 1 Pro = 1234

MOV     Src,valSrc      'load cog 1 Src = 1234

MODZ    _CLR   WZ       'set Z = 0

MODC    _CLR   WC       'set C = 0

IF_C     MOV     Carry,#1        'true  C = 1

IF_NC  MOV     Carry,#0        'false C = 0

IF_Z     MOV     Zero,#1         'true  Z = 1

IF_NZ  MOV     Zero,#0         'false Z = 0

debug("--------------------------------------")

debug("before CMP  ",sdec(Dest),sdec(Src),ubin(Zero),ubin(Carry))

CMP     Dest,Src  WCZ

IF_C     MOV     Carry,#1         'true  C = 1

IF_NC  MOV     Carry,#0         'false C = 0

IF_Z     MOV     Zero,#1          'true  Z = 1

IF_NZ  MOV     Zero,#0          'false Z = 0

debug("after CMP   ",sdec(Dest),sdec(Src),ubin(Zero),ubin(Carry))

debug("--------------------------------------")

MOV     Dest,MAX          'load cog 1 Pro = $FFFF_FFFF

MOV     Src,valSrc       'load cog 1 Src = 1

MODZ    _CLR   WZ        'set Z = 0

MODC    _CLR   WC        'set C = 0

IF_C     MOV     Carry,#1         'true  C = 1

IF_NC  MOV     Carry,#0         'false C = 0

IF_Z     MOV     Zero,#1          'true  Z = 1

IF_NZ  MOV     Zero,#0          'false Z = 0

debug("before CMP ",sdec(Dest),sdec(Src),ubin(Zero),ubin(Carry))

CMP     Dest,Src  WCZ

IF_C     MOV     Carry,#1         'true  C = 1

IF_NC  MOV     Carry,#0         'false C = 0

IF_Z     MOV     Zero,#1          'true  Z = 1

IF_NZ  MOV     Zero,#0          'false Z = 0

debug("after CMP  ",sdec(Dest),sdec(Src),ubin(Zero),ubin(Carry))

debug("--------------------------------------")

DRVH    #P0

MOV     Dest,#1        'load cog 1 Pro = 1

MOV     Src,valSrc    'load cog 1 Src = 1

MODZ    _CLR   WZ     'set Z = 0

MODC    _CLR   WC     'set C = 1

IF_C    MOV     Carry,#1      'true  C = 1

IF_NC  MOV     Carry,#0      'false C = 0

IF_Z    MOV     Zero,#1       'true  Z = 1

IF_NZ  MOV     Zero,#0       'false Z = 0

debug("before CMP  ",sdec(Dest),sdec(Src),ubin(Zero),ubin(Carry))

CMP   Dest,Src  WCZ

IF_C    MOV     Carry,#1      'true  C = 1

IF_NC  MOV     Carry,#0      'false C = 0

IF_Z    MOV     Zero,#1       'true  Z = 1

IF_NZ  MOV     Zero,#0       'false Z = 0

debug(ubin(Dest))

debug("after CMP   " ,sdec(Dest),sdec(Src),ubin(Zero),ubin(Carry))

debug("--------------------------------------")

_Loop1         NOP

JMP     #_Loop1          'remember # imediate

cog1Wait         long      900_000_000    'number of clock cycles to wait

cog1Cnt           long      0

valDest             long      1234         'cog 1 Dest

valSrc                long      1234         'cog 1 Src

Carry                 long      0

Zero                  long      0

Dest                  long      0

Src                    long      0

MAX                 long      $FFFF_FFFF

MIN                   long      0

### 17.8.2_Example_WRD_CMPX_020

CMPX D,{#}S {WC/WZ/WCZ}

Compare D to (S + C), extended. C = borrow of (D - (S + C)). Z = Z AND (D == S + C).

{{17.8.2_Example_WRD_CMP_020}}

''CMPX D,{#}S {WC/WZ/WCZ}

''Compare D to (S + C), extended. C = borrow of (D - (S + C)). Z = Z AND (D == S + C).

CON

_clkfreq = 200_000_000  ''Debug must be enabled clock must be greater than 10MHZ for Debug

P0 =0 , P1 = 1  , P2 = 2

VAR

Byte cogRunning 'cog ID started is returned or -1 if not started

PUB main()

debug("--------------------------------------")

debug("CMPX D,{#}S {WC/WZ/WCZ}")

debug("Compare D to (S + C), extended. C = borrow of (D - (S + C)). Z = Z AND (D == S + C)")

debug("--------------------------------------")

cogRunning := COGINIT(COGEXEC_NEW,@S0_CMPX,0)

debug(udec(cogRunning))

repeat                                                       'keep cog 0 running

DAT

ORG   0

S0_CMPX               DRVH    #P0

MOV     Dest,valDest    'load cog 1 Pro = 1234

MOV     Src,valSrc    'load cog 1 Src = 1234

MODZ    _CLR   WZ     'set Z = 0

MODC    _CLR   WC     'set C = 0

IF_C   MOV     Carry,#1      'true  C = 1

IF_NC  MOV     Carry,#0      'false C = 0

IF_Z   MOV     Zero,#1       'true  Z = 1

IF_NZ  MOV     Zero,#0       'false Z = 0

debug("--------------------------------------")

debug("before CMPX  ",sdec(Dest),sdec(Src),ubin(Carry),ubin(Zero))

CMPX   Dest,Src  WCZ

IF_C   MOV     Carry,#1      'true  C = 1

IF_NC  MOV     Carry,#0      'false C = 0

IF_Z   MOV     Zero,#1       'true  Z = 1

IF_NZ  MOV     Zero,#0       'false Z = 0

debug("after CMPX   ",sdec(Dest),sdec(Src),ubin(Carry),ubin(Zero))

debug("--------------------------------------")

MODZ    _CLR   WZ     'set Z = 0

MODC    _SET   WC     'set C = 1

IF_C   MOV     Carry,#1      'true  C = 1

IF_NC  MOV     Carry,#0      'false C = 0

IF_Z   MOV     Zero,#1       'true  Z = 1

IF_NZ  MOV     Zero,#0       'false Z = 0

debug("before CMPX  ",sdec(Dest),sdec(Src),ubin(Carry),ubin(Zero))

CMPX   Dest,Src  WCZ

IF_C   MOV     Carry,#1      'true  C = 1

IF_NC  MOV     Carry,#0      'false C = 0

IF_Z   MOV     Zero,#1       'true  Z = 1

IF_NZ  MOV     Zero,#0       'false Z = 0

debug("after CMPX   ",sdec(Dest),sdec(Src),ubin(Carry),ubin(Zero))

debug("--------------------------------------")

MODZ    _SET   WZ     'set Z = 0

MODC    _CLR   WC     'set C = 1

IF_C   MOV     Carry,#1      'true  C = 1

IF_NC  MOV     Carry,#0      'false C = 0

IF_Z   MOV     Zero,#1       'true  Z = 1

IF_NZ  MOV     Zero,#0       'false Z = 0

debug("before CMPX  ",sdec(Dest),sdec(Src),ubin(Carry),ubin(Zero))

CMPX   Dest,Src  WCZ

IF_C   MOV     Carry,#1      'true  C = 1

IF_NC  MOV     Carry,#0      'false C = 0

IF_Z   MOV     Zero,#1       'true  Z = 1

IF_NZ  MOV     Zero,#0       'false Z = 0

debug("after CMPX   ",sdec(Dest),sdec(Src),ubin(Carry),ubin(Zero))

debug("--------------------------------------")

MODZ    _SET   WZ     'set Z = 0

MODC    _SET   WC     'set C = 1

IF_C   MOV     Carry,#1      'true  C = 1

IF_NC  MOV     Carry,#0      'false C = 0

IF_Z   MOV     Zero,#1       'true  Z = 1

IF_NZ  MOV     Zero,#0       'false Z = 0

debug("before CMPX  ",sdec(Dest),sdec(Src),ubin(Carry),ubin(Zero))

CMPX   Dest,Src  WCZ

IF_C   MOV     Carry,#1      'true  C = 1

IF_NC  MOV     Carry,#0      'false C = 0

IF_Z   MOV     Zero,#1       'true  Z = 1

IF_NZ  MOV     Zero,#0       'false Z = 0

debug("after CMPX   ",sdec(Dest),sdec(Src),ubin(Carry),ubin(Zero))

debug("--------------------------------------")

MOV     Dest,MAX      'load cog 1 Pro = $FFFF_FFFF

MOV     Src,valSrc   'load cog 1 Src = 1

MODZ    _CLR   WZ    'set Z = 0

MODC    _CLR   WC    'set C = 0

IF_C   MOV     Carry,#1     'true  C = 1

IF_NC  MOV     Carry,#0     'false C = 0

IF_Z   MOV     Zero,#1      'true  Z = 1

IF_NZ  MOV     Zero,#0      'false Z = 0

debug("before CMPX  ",sdec(Dest),sdec(Src),ubin(carry),ubin(Zero))

CMPX    Dest,Src  WCZ

IF_C   MOV     Carry,#1      'true  Z = 1

IF_NC  MOV     Carry,#0      'false Z = 0

IF_Z   MOV     Zero,#1       'true  Z = 1

IF_NZ  MOV     Zero,#0       'false Z = 0

debug("after CMPX   ",sdec(Dest),sdec(Src),ubin(Carry),ubin(Zero))

debug("--------------------------------------")

MOV     Dest,#1       'load cog 1 Pro = 1

MOV     Src,valSrc    'load cog 1 Src = 1

MODZ    _CLR   WZ     'set Z = 0

MODC    _CLR   WC     'set C = 0

IF_C   MOV     Carry,#1      'true  C = 1

IF_NC  MOV     Carry,#0      'false C = 0

IF_Z   MOV     Zero,#1       'true  Z = 1

IF_NZ  MOV     Zero,#0       'false Z = 0

debug("before CMPX  ",sdec(Dest),sdec(Src),ubin(Carry),ubin(Zero))

CMPX   Dest,Src  WCZ

IF_C   MOV     Carry,#1      'true  C = 1

IF_NC  MOV     Carry,#0      'false C = 0

IF_Z   MOV     Zero,#1       'true  Z = 1

IF_NZ  MOV     Zero,#0       'false Z = 0

debug("after CMPX   ",sdec(Dest),sdec(Src),ubin(Carry),ubin(Zero))

debug("--------------------------------------")

_Loop1                NOP

JMP     #_Loop1          'remember # imediate  Keep Cog 1 Running

cog1Wait              long      900_000_000    'number of clock cycles to wait

cog1Cnt               long      0

valDest                long      1234         'cog 1 Dest

valSrc                long      1234         'cog 1 Src

Carry                 long      0

Zero                  long      0

Dest                  long      0

Src                   long      0

MAX                   long      $FFFF_FFFF

MIN                   long      0

### 17.8.3_Example_WRD_CMPS_021

CMPS    D,{#}S   {WC/WZ/WCZ}

Compare D to S, signed.C = correct sign of (D - S). Z = (D == S)=1 if equal.

{{17.8.3_Example_WRD_CMPS_021}}

''CMPS    D,{#}S   {WC/WZ/WCZ}

''Compare D to S, signed.C = correct sign of (D - S). Z = (D == S)=1 if equal.

CON

_clkfreq = 200_000_000  ''Debug must be enabled clock must be greater than 10MHZ for Debug

P0 =0 , P1 = 1  , P2 = 2

VAR

Byte cogRunning 'cog ID started is returned or -1 if not started

PUB main()

debug("--------------------------------------")

debug("CMPS    D,{#}S   {WC/WZ/WCZ}")

debug("Compare D to S, signed.C = correct sign of (D - S). Z = (D == S)=1 if equal")

cogRunning := COGINIT(COGEXEC_NEW,@S0_CMPS,0)

debug(udec(cogRunning))

repeat                                                       'keep cog 0 running

DAT

ORG   0

S0_CMPS               DRVH    #P0

MOV     Dest,valDest    'load cog 1 Pro = 1234

MOV     Src,valSrc    'load cog 1 Src = 1234

MODZ    _CLR   WZ     'set Z = 0

MODC    _CLR   WC     'set C = 0

IF_C   MOV     Carry,#1      'true  C = 1

IF_NC  MOV     Carry,#0      'false C = 0

IF_Z   MOV     Zero,#1       'true  Z = 1

IF_NZ  MOV     Zero,#0       'false Z = 0

debug("--------------------------------------")

debug("before CMPS  ",sdec(Dest),sdec(Src),ubin(Carry),ubin(Zero))

CMPS   Dest,Src  WCZ

IF_C   MOV     Carry,#1      'true  C = 1

IF_NC  MOV     Carry,#0      'false C = 0

IF_Z   MOV     Zero,#1       'true  Z = 1

IF_NZ  MOV     Zero,#0       'false Z = 0

debug("after CMPS   ",sdec(Dest),sdec(Src),ubin(Carry),ubin(Zero))

debug("--------------------------------------")

MODZ    _CLR   WZ     'set Z = 0

MODC    _SET   WC     'set C = 1

IF_C   MOV     Carry,#1      'true  C = 1

IF_NC  MOV     Carry,#0      'false C = 0

IF_Z   MOV     Zero,#1       'true  Z = 1

IF_NZ  MOV     Zero,#0       'false Z = 0

debug("before CMPS  ",sdec(Dest),sdec(Src),ubin(Carry),ubin(Zero))

CMPS   Dest,Src  WCZ

IF_C   MOV     Carry,#1      'true  C = 1

IF_NC  MOV     Carry,#0      'false C = 0

IF_Z   MOV     Zero,#1       'true  Z = 1

IF_NZ  MOV     Zero,#0       'false Z = 0

debug("after CMPS   ",sdec(Dest),sdec(Src),ubin(Carry),ubin(Zero))

debug("--------------------------------------")

MODZ    _SET   WZ     'set Z = 0

MODC    _CLR   WC     'set C = 1

IF_C   MOV     Carry,#1      'true  C = 1

IF_NC  MOV     Carry,#0      'false C = 0

IF_Z   MOV     Zero,#1       'true  Z = 1

IF_NZ  MOV     Zero,#0       'false Z = 0

debug("before CMPS  ",sdec(Dest),sdec(Src),ubin(Carry),ubin(Zero))

CMPS   Dest,Src  WCZ

IF_C   MOV     Carry,#1      'true  C = 1

IF_NC  MOV     Carry,#0      'false C = 0

IF_Z   MOV     Zero,#1       'true  Z = 1

IF_NZ  MOV     Zero,#0       'false Z = 0

debug("after CMPS   ",sdec(Dest),sdec(Src),ubin(Carry),ubin(Zero))

debug("--------------------------------------")

MODZ    _SET   WZ     'set Z = 0

MODC    _SET   WC     'set C = 1

IF_C   MOV     Carry,#1      'true  C = 1

IF_NC  MOV     Carry,#0      'false C = 0

IF_Z   MOV     Zero,#1       'true  Z = 1

IF_NZ  MOV     Zero,#0       'false Z = 0

debug("before CMPS  ",sdec(Dest),sdec(Src),ubin(Carry),ubin(Zero))

CMPS   Dest,Src  WCZ

IF_C   MOV     Carry,#1      'true  C = 1

IF_NC  MOV     Carry,#0      'false C = 0

IF_Z   MOV     Zero,#1       'true  Z = 1

IF_NZ  MOV     Zero,#0       'false Z = 0

debug("after CMPS   ",sdec(Dest),sdec(Src),ubin(Carry),ubin(Zero))

debug("--------------------------------------")

MOV     Dest,MAX      'load cog 1 Pro = $FFFF_FFFF

MOV     Src,valSrc   'load cog 1 Src = 1

MODZ    _CLR   WZ    'set Z = 0

MODC    _CLR   WC    'set C = 0

IF_C   MOV     Carry,#1     'true  C = 1

IF_NC  MOV     Carry,#0     'false C = 0

IF_Z   MOV     Zero,#1      'true  Z = 1

IF_NZ  MOV     Zero,#0      'false Z = 0

debug("before CMPS  ",sdec(Dest),sdec(Src),ubin(carry),ubin(Zero))

CMPS    Dest,Src  WCZ

IF_C   MOV     Carry,#1      'true  Z = 1

IF_NC  MOV     Carry,#0      'false Z = 0

IF_Z   MOV     Zero,#1       'true  Z = 1

IF_NZ  MOV     Zero,#0       'false Z = 0

debug("after CMPS   ",sdec(Dest),sdec(Src),ubin(Carry),ubin(Zero))

debug("--------------------------------------")

MOV     Dest,#1        'load cog 1 Pro = 1

MOV     Src,valSrc    'load cog 1 Src = 1

MODZ    _CLR   WZ     'set Z = 0

MODC    _CLR   WC     'set C = 0

IF_C   MOV     Carry,#1      'true  C = 1

IF_NC  MOV     Carry,#0      'false C = 0

IF_Z   MOV     Zero,#1       'true  Z = 1

IF_NZ  MOV     Zero,#0       'false Z = 0

debug("before CMPS  ",sdec(Dest),sdec(Src),ubin(Carry),ubin(Zero))

CMPS   Dest,Src  WCZ

IF_C   MOV     Carry,#1      'true  C = 1

IF_NC  MOV     Carry,#0      'false C = 0

IF_Z   MOV     Zero,#1       'true  Z = 1

IF_NZ  MOV     Zero,#0       'false Z = 0

debug("after CMPS   ",sdec(Dest),sdec(Src),ubin(Carry),ubin(Zero))

debug("--------------------------------------")

_Loop1                NOP

JMP     #_Loop1          'remember # imediate  Keep Cog 1 Running

cog1Wait              long      900_000_000    'number of clock cycles to wait

cog1Cnt               long      0

valDest                long      1234         'cog 1 Dest

valSrc                long      1234         'cog 1 Src

Carry                 long      0

Dest                  long      0

Src                   long      0

Zero                  long      0

MAX                   long      $FFFF_FFFF

MIN                   long      0

### 17.8.4_Example_WRD_CMPSX_022

CMPC S

{{17.8.4_Example_WRD_CMPSX_022}}

''CMPSX   D,{#}S   {WC/WZ/WCZ}

''Compare D to (S + C), signed and extended. C = correct sign of (D - (S + C)).  Z = Z AND (D == S + C).

CON

_clkfreq = 200_000_000  ''Debug must be enabled clock must be greater than 10MHZ for Debug

P0 =0 , P1 = 1  , P2 = 2

VAR

Byte cogRunning 'cog ID started is returned or -1 if not started

PUB main()

debug("--------------------------------------")

debug("CMPSX   D,{#}S   {WC/WZ/WCZ}")

debug("Compare D to (S + C), signed and extended.")

debug("C = correct sign of (D - (S + C)).  Z = Z AND (D == S + C)")

debug("Dest = D  Src = S")

debug("--------------------------------------")

waitms(100)

cogRunning := COGINIT(COGEXEC_NEW,@S0_CMPSX,0)

debug(udec(cogRunning))

repeat                                                       'keep cog 0 running

DAT

ORG   0

S0_CMPSX               DRVH    #P0

MOV     Dest,valDest  'load cog 1 Pro = 1234

MOV     Src,valSrc    'load cog 1 Src = 1234

MODZ    _CLR   WZ     'set Z = 0

MODC    _CLR   WC     'set C = 0

IF_C   MOV     Carry,#1      'true  C = 1

IF_NC  MOV     Carry,#0      'false C = 0

IF_Z   MOV     Zero,#1       'true  Z = 1

IF_NZ  MOV     Zero,#0       'false Z = 0

debug("--------------------------------------")

debug("before CMPS  ",sdec(Dest),sdec(Src),ubin(Carry),ubin(Zero))

CMPSX   Dest,Src  WCZ

IF_C   MOV     Carry,#1      'true  C = 1

IF_NC  MOV     Carry,#0      'false C = 0

IF_Z   MOV     Zero,#1       'true  Z = 1

IF_NZ  MOV     Zero,#0       'false Z = 0

debug("after CMPS   ",sdec(Dest),sdec(Src),ubin(Carry),ubin(Zero))

debug("--------------------------------------")

MODZ    _CLR   WZ     'set Z = 0

MODC    _SET   WC     'set C = 1

IF_C   MOV     Carry,#1      'true  C = 1

IF_NC  MOV     Carry,#0      'false C = 0

IF_Z   MOV     Zero,#1       'true  Z = 1

IF_NZ  MOV     Zero,#0       'false Z = 0

debug("before CMPS  ",sdec(Dest),sdec(Src),ubin(Carry),ubin(Zero))

CMPSX   Dest,Src  WCZ

IF_C   MOV     Carry,#1      'true  C = 1

IF_NC  MOV     Carry,#0      'false C = 0

IF_Z   MOV     Zero,#1       'true  Z = 1

IF_NZ  MOV     Zero,#0       'false Z = 0

debug("after CMPS   ",sdec(Dest),sdec(Src),ubin(Carry),ubin(Zero))

debug("--------------------------------------")

MODZ    _SET   WZ     'set Z = 0

MODC    _CLR   WC     'set C = 1

IF_C   MOV     Carry,#1      'true  C = 1

IF_NC  MOV     Carry,#0      'false C = 0

IF_Z   MOV     Zero,#1       'true  Z = 1

IF_NZ  MOV     Zero,#0       'false Z = 0

debug("before CMPS  ",sdec(Dest),sdec(Src),ubin(Carry),ubin(Zero))

CMPSX   Dest,Src  WCZ

IF_C   MOV     Carry,#1      'true  C = 1

IF_NC  MOV     Carry,#0      'false C = 0

IF_Z   MOV     Zero,#1       'true  Z = 1

IF_NZ  MOV     Zero,#0       'false Z = 0

debug("after CMPS   ",sdec(Dest),sdec(Src),ubin(Carry),ubin(Zero))

debug("--------------------------------------")

MODZ    _SET   WZ     'set Z = 0

MODC    _SET   WC     'set C = 1

IF_C   MOV     Carry,#1      'true  C = 1

IF_NC  MOV     Carry,#0      'false C = 0

IF_Z   MOV     Zero,#1       'true  Z = 1

IF_NZ  MOV     Zero,#0       'false Z = 0

debug("before CMPS  ",sdec(Dest),sdec(Src),ubin(Carry),ubin(Zero))

CMPSX   Dest,Src  WCZ

IF_C   MOV     Carry,#1      'true  C = 1

IF_NC  MOV     Carry,#0      'false C = 0

IF_Z   MOV     Zero,#1       'true  Z = 1

IF_NZ  MOV     Zero,#0       'false Z = 0

debug("after CMPS   ",sdec(Dest),sdec(Src),ubin(Carry),ubin(Zero))

debug("--------------------------------------")

MOV     Dest,MAX      'load cog 1 Pro = $FFFF_FFFF

MOV     Src,Dest     'load cog 1 Src = 1

MODZ    _CLR   WZ    'set Z = 0

MODC    _CLR   WC    'set C = 0

IF_C   MOV     Carry,#1     'true  C = 1

IF_NC  MOV     Carry,#0     'false C = 0

IF_Z   MOV     Zero,#1      'true  Z = 1

IF_NZ  MOV     Zero,#0      'false Z = 0

debug("before CMPS  ",sdec(Dest),sdec(Src),ubin(carry),ubin(Zero))

CMPSX    Dest,Src  WCZ

IF_C   MOV     Carry,#1      'true  Z = 1

IF_NC  MOV     Carry,#0      'false Z = 0

IF_Z   MOV     Zero,#1       'true  Z = 1

IF_NZ  MOV     Zero,#0       'false Z = 0

debug("after CMPS   ",sdec(Dest),sdec(Src),ubin(Carry),ubin(Zero))

debug("--------------------------------------")

MOV     Dest,#1        'load cog 1 Dest = 1

MOV     Src,valSrc    'load cog 1 Src = 1

MODZ    _CLR   WZ     'set Z = 0

MODC    _CLR   WC     'set C = 0

IF_C   MOV     Carry,#1      'true  C = 1

IF_NC  MOV     Carry,#0      'false C = 0

IF_Z   MOV     Zero,#1       'true  Z = 1

IF_NZ  MOV     Zero,#0       'false Z = 0

debug("before CMPS  ",sdec(Dest),sdec(Src),ubin(Carry),ubin(Zero))

CMPSX   Dest,Src  WCZ

IF_C   MOV     Carry,#1      'true  C = 1

IF_NC  MOV     Carry,#0      'false C = 0

IF_Z   MOV     Zero,#1       'true  Z = 1

IF_NZ  MOV     Zero,#0       'false Z = 0

debug("after CMPS   ",sdec(Dest),sdec(Src),ubin(Carry),ubin(Zero))

debug("--------------------------------------")

_Loop1                NOP

JMP     #_Loop1          'remember # imediate  Keep Cog 1 Running

cog1Wait              long      900_000_000    'number of clock cycles to wait

cog1Cnt               long      0

valDest               long      1234         'cog 1 Dest

valSrc                long      1234          'cog 1 Src

Dest                  long      0

Src                   long      0

Carry                 long      0

Zero                  long      0

MAX                   long      $FFFF_FFFF

MIN                   long      0

### 17.8.5_Example_WRD_CMPR_023

CMPCompa{{17.8.5_Example_WRD_CMP_023}}

''CMPR    D,{#}S   {WC/WZ/WCZ}

''Compare S to D (reverse).C = borrow of (S - D). Z = (D == S).

CON

_clkfreq = 200_000_000  ''Debug must be enabled clock must be greater than 10MHZ for Debug

P0 =0 , P1 = 1  , P2 = 2

VAR

Byte cogRunning 'cog ID started is returned or -1 if not started

PUB main()

debug("--------------------------------------")

debug("CMPR D,{#}S {WC/WZ/WCZ}")

debug("Compare S to D. C = borrow of (S - D). Z = (D == S)= 1 if equal")

debug("--------------------------------------")

cogRunning := COGINIT(COGEXEC_NEW,@S0_CMPR,0)

debug(udec(cogRunning))

repeat                                    'keep cog 0 running

DAT

ORG   0

S0_CMPR          DRVH    #P0             'P0 on program running

MOV     Dest,valDest    'load cog 1 Pro = 1234

MOV     Src,valSrc      'load cog 1 Src = 1234

MODZ    _CLR   WZ       'set Z = 0

MODC    _CLR   WC       'set C = 0

IF_C     MOV     Carry,#1        'true  C = 1

IF_NC  MOV     Carry,#0        'false C = 0

IF_Z     MOV     Zero,#1         'true  Z = 1

IF_NZ  MOV     Zero,#0         'false Z = 0

debug("--------------------------------------")

debug("before CMPR  ",sdec(Dest),sdec(Src),ubin(Zero),ubin(Carry))

CMPR    Dest,Src  WCZ

IF_C    MOV     Carry,#1         'true  C = 1

IF_NC   MOV     Carry,#0         'false C = 0

IF_Z     MOV     Zero,#1          'true  Z = 1

IF_NZ  MOV     Zero,#0          'false Z = 0

debug("after CMPR   ",sdec(Dest),sdec(Src),ubin(Zero),ubin(Carry))

debug("--------------------------------------")

MOV     Dest,MAX          'load cog 1 Pro = $FFFF_FFFF

MOV     Src,valSrc       'load cog 1 Src = 1

MODZ    _CLR   WZ        'set Z = 0

MODC    _CLR   WC        'set C = 0

IF_C    MOV     Carry,#1         'true  C = 1

IF_NC  MOV     Carry,#0         'false C = 0

IF_Z     MOV     Zero,#1          'true  Z = 1

IF_NZ   MOV     Zero,#0          'false Z = 0

debug("before CMPR ",sdec(Dest),sdec(Src),ubin(Zero),ubin(Carry))

CMPR     Dest,Src  WCZ

IF_C     MOV     Carry,#1         'true  C = 1

IF_NC  MOV     Carry,#0         'false C = 0

IF_Z     MOV     Zero,#1          'true  Z = 1

IF_NZ  MOV     Zero,#0          'false Z = 0

debug("after CMPR  ",sdec(Dest),sdec(Src),ubin(Zero),ubin(Carry))

debug("--------------------------------------")

DRVH    #P0

MOV     Dest,#1       'load cog 1 Pro = 1

MOV     Src,valSrc    'load cog 1 Src = 1

MODZ    _CLR   WZ     'set Z = 0

MODC    _CLR   WC     'set C = 1

IF_C      MOV     Carry,#1      'true  C = 1

IF_NC    MOV     Carry,#0      'false C = 0

IF_Z      MOV     Zero,#1       'true  Z = 1

IF_NZ   MOV     Zero,#0       'false Z = 0

debug("before CMPR  ",sdec(Dest),sdec(Src),ubin(Zero),ubin(Carry))

CMPR    Dest,Src  WCZ

IF_C      MOV     Carry,#1      'true  C = 1

IF_NC    MOV     Carry,#0      'false C = 0

IF_Z       MOV     Zero,#1       'true  Z = 1

IF_NZ    MOV     Zero,#0       'false Z = 0

debug(ubin(Dest))

debug("after CMP   " ,sdec(Dest),sdec(Src),ubin(Zero),ubin(Carry))

debug("--------------------------------------")

_Loop1                NOP

JMP     #_Loop1          'remember # imediate

cog1Wait         long      900_000_000    'number of clock cycles to wait

cog1Cnt            long      0

valDest             long      1234         'cog 1 Dest

valSrc                long      1234         'cog 1 Src

Carry                 long      0

Zero                  long      0

Dest                  long      0

Src                     long      0

MAX                  long      $FFFF_FFFF

MIN                   long      0

### 17.8.6_Example_WRD_CMPM_024

CMPC

{{17.8.6_Example_WRD_CMPM_024}}

''CMPM    D,{#}S   {WC/WZ/WCZ}

''Compare D to S, get MSB of difference into C.C = MSB of (D - S).

''Z = (D == S).

CON

_clkfreq = 200_000_000  ''Debug must be enabled clock must be greater than 10MHZ for Debug

P0 =0 , P1 = 1  , P2 = 2

VAR

Byte cogRunning 'cog ID started is returned or -1 if not started

PUB main()

debug("--------------------------------------")

debug("CMPM D,{#}S {WC/WZ/WCZ}")

debug("Compare D to S.get MSB of difference into C.C = MSB of (D - S)")

debug("Z = (D == S)")

debug("--------------------------------------")

cogRunning := COGINIT(COGEXEC_NEW,@S0_CMPM,0)

debug(udec(cogRunning))

repeat                                    'keep cog 0 running

DAT

ORG   0

S0_CMPM               DRVH    #P0             'P0 on program running

MOV     Dest,valDest    'load cog 1 Pro = 1234

MOV     Src,valSrc      'load cog 1 Src = 1234

MODZ    _CLR   WZ       'set Z = 0

MODC    _CLR   WC       'set C = 0

IF_C   MOV     Carry,#1        'true  C = 1

IF_NC  MOV     Carry,#0        'false C = 0

IF_Z   MOV     Zero,#1         'true  Z = 1

IF_NZ  MOV     Zero,#0         'false Z = 0

debug("--------------------------------------")

debug("before CMPM  ",sdec(Dest),sdec(Src),ubin(Zero),ubin(Carry))

CMPM    Dest,Src  WCZ

IF_C   MOV     Carry,#1         'true  C = 1

IF_NC  MOV     Carry,#0         'false C = 0

IF_Z   MOV     Zero,#1          'true  Z = 1

IF_NZ  MOV     Zero,#0          'false Z = 0

debug("after CMPM   ",sdec(Dest),sdec(Src),ubin(Zero),ubin(Carry))

debug("--------------------------------------")

MOV     Dest,MAX          'load cog 1 Pro = $FFFF_FFFF

MOV     Src,valSrc       'load cog 1 Src = 1

MODZ    _CLR   WZ        'set Z = 0

MODC    _CLR   WC        'set C = 0

IF_C   MOV     Carry,#1         'true  C = 1

IF_NC  MOV     Carry,#0         'false C = 0

IF_Z   MOV     Zero,#1          'true  Z = 1

IF_NZ  MOV     Zero,#0          'false Z = 0

debug("before CMPM ",sdec(Dest),sdec(Src),ubin(Zero),ubin(Carry))

CMPM     Dest,Src  WCZ

IF_C   MOV     Carry,#1         'true  C = 1

IF_NC  MOV     Carry,#0         'false C = 0

IF_Z   MOV     Zero,#1          'true  Z = 1

IF_NZ  MOV     Zero,#0          'false Z = 0

debug("after CMPM  ",sdec(Dest),sdec(Src),ubin(Zero),ubin(Carry))

debug("--------------------------------------")

DRVH    #P0

MOV     Dest,#1       'load cog 1 Pro = 1

MOV     Src,valSrc    'load cog 1 Src = 1

MODZ    _CLR   WZ     'set Z = 0

MODC    _CLR   WC     'set C = 1

IF_C   MOV     Carry,#1      'true  C = 1

IF_NC  MOV     Carry,#0      'false C = 0

IF_Z   MOV     Zero,#1       'true  Z = 1

IF_NZ  MOV     Zero,#0       'false Z = 0

debug("before CMPM  ",sdec(Dest),sdec(Src),ubin(Zero),ubin(Carry))

CMPM    Dest,Src  WCZ

IF_C   MOV     Carry,#1      'true  C = 1

IF_NC  MOV     Carry,#0      'false C = 0

IF_Z   MOV     Zero,#1       'true  Z = 1

IF_NZ  MOV     Zero,#0       'false Z = 0

debug(ubin(Dest))

debug("after CMPM   " ,sdec(Dest),sdec(Src),ubin(Zero),ubin(Carry))

debug("--------------------------------------")

_Loop1                NOP

JMP     #_Loop1          'remember # imediate

cog1Wait              long      900_000_000    'number of clock cycles to wait

cog1Cnt               long      0

valDest                long      1234         'cog 1 Dest

valSrc                long      1234         'cog 1 Src

Carry                 long      0

Zero                  long      0

Dest                  long      0

Src                   long      0

MAX                   long      $FFFF_FFFF

MIN                   long      0

### 17.8.7_Example_WRD_CMPSUB_025

CMPComIf

{{17.8.7_Example_WRD_CMPSUB_025}}

''CMPSUB  D,{#}S   {WC/WZ/WCZ}

''Compare and subtract S from D if D >= S.

''If D => S then D = D - S and C = 1, else D same and C = 0.  *

CON

_clkfreq = 200_000_000  ''Debug must be enabled clock must be greater than 10MHZ for Debug

P0 =0 , P1 = 1  , P2 = 2

VAR

Byte cogRunning 'cog ID started is returned or -1 if not started

PUB main()

debug("--------------------------------------")

debug("CMPSUB  D,{#}S   {WC/WZ/WCZ}")

debug("Compare and subtract S from D if D >= S.")

debug("If D => S then D = D - S and C = 1, else D same and C = 0.  *")

debug("--------------------------------------")

cogRunning := COGINIT(COGEXEC_NEW,@S0_CMPSUB,0)

debug(udec(cogRunning))

repeat                                    'keep cog 0 running

DAT

ORG   0

S0_CMPSUB   DRVH    #P0             'P0 on program running

MOV     Dest,valDest    'load cog 1 Pro = 1234

MOV     Src,valSrc      'load cog 1 Src = 1234

MODZ    _CLR   WZ       'set Z = 0

MODC    _CLR   WC       'set C = 0

IF_C   MOV     Carry,#1        'true  C = 1

IF_NC  MOV     Carry,#0        'false C = 0

IF_Z    MOV     Zero,#1         'true  Z = 1

IF_NZ  MOV     Zero,#0         'false Z = 0

debug("--------------------------------------")

debug("before CMPSUB  ",sdec(Dest),sdec(Src),ubin(Zero),ubin(Carry))

CMPSUB  Dest,Src  WCZ

IF_C   MOV     Carry,#1         'true  C = 1

IF_NC  MOV     Carry,#0         'false C = 0

IF_Z   MOV     Zero,#1          'true  Z = 1

IF_NZ  MOV     Zero,#0          'false Z = 0

debug("after CMPSUB   ",sdec(Dest),sdec(Src),ubin(Zero),ubin(Carry))

debug("--------------------------------------")

MOV     Dest,MAX          'load cog 1 Pro = $FFFF_FFFF

MOV     Src,valSrc       'load cog 1 Src = 1

MODZ    _CLR   WZ        'set Z = 0

MODC    _CLR   WC        'set C = 0

IF_C   MOV     Carry,#1         'true  C = 1

IF_NC  MOV     Carry,#0         'false C = 0

IF_Z   MOV     Zero,#1          'true  Z = 1

IF_NZ  MOV     Zero,#0          'false Z = 0

debug("before CMPSUB ",sdec(Dest),sdec(Src),ubin(Zero),ubin(Carry))

CMPSUB  Dest,Src  WCZ

IF_C   MOV     Carry,#1         'true  C = 1

IF_NC  MOV     Carry,#0         'false C = 0

IF_Z   MOV     Zero,#1          'true  Z = 1

IF_NZ  MOV     Zero,#0          'false Z = 0

debug("after CMPSUB  ",sdec(Dest),sdec(Src),ubin(Zero),ubin(Carry))

debug("--------------------------------------")

DRVH    #P0

MOV     Dest,#1       'load cog 1 Pro = 1

MOV     Src,valSrc    'load cog 1 Src = 1

MODZ    _CLR   WZ     'set Z = 0

MODC    _CLR   WC     'set C = 1

IF_C   MOV     Carry,#1      'true  C = 1

IF_NC  MOV     Carry,#0      'false C = 0

IF_Z   MOV     Zero,#1       'true  Z = 1

IF_NZ  MOV     Zero,#0       'false Z = 0

debug("before CMPSUB  ",sdec(Dest),sdec(Src),ubin(Zero),ubin(Carry))

CMPSUB  Dest,Src  WCZ

IF_C   MOV     Carry,#1      'true  C = 1

IF_NC  MOV     Carry,#0      'false C = 0

IF_Z   MOV     Zero,#1       'true  Z = 1

IF_NZ  MOV     Zero,#0       'false Z = 0

debug(ubin(Dest))

debug("after CMPSUB   " ,sdec(Dest),sdec(Src),ubin(Zero),ubin(Carry))

debug("--------------------------------------")

_Loop1          NOP

JMP     #_Loop1          'remember # imediate

cog1Wait          long      900_000_000    'number of clock cycles to wait

cog1Cnt            long      0

valDest              long      1234         'cog 1 Dest

valSrc                long      1234         'cog 1 Src

Carry                 long      0

Zero                  long      0

Dest                  long      0

Src                     long      0

MAX                   long      $FFFF_FFFF

MIN                   long      0

## 17.9 F Force

| FGE D,{#}S {WC/WZ/WCZ} | Force D >= S. If D < S then D = S and C = 1, else D same and C = 0. * |
| --- | --- |
| FLE D,{#}S {WC/WZ/WCZ} | Force D <= S. If D > S then D = S and C = 1, else D same and C = 0. * |
| FGES D,{#}S {WC/WZ/WCZ} | Force D >= S, signed. If D < S then D = S and C = 1, else D same and C = 0. * |
| FLES D,{#}S {WC/WZ/WCZ} | Force D <= S, signed. If D > S then D = S and C = 1, else D same and C = 0. * |

### For o 17.9.1_Example_WRD_FGE_026

FGE     D,{#}S   {WC/WZ/WCZ}

Force D >= S. If D < S then D = S and C = 1,

else D same and C = 0. *

{{17.9.1_Example_WRD_FGE_026}}

''FGE     D,{#}S   {WC/WZ/WCZ}

''Force D >= S. If D < S then D = S and C = 1,

''else D same and C = 0. *

CON

_clkfreq = 200_000_000  ''Debug must be enabled clock must be greater than 10MHZ for Debug

P0 =0 , P1 = 1  , P2 = 2

VAR

Byte cogRunning 'cog ID started is returned or -1 if not started

PUB main()

debug("--------------------------------------")

debug("FGE     D,{#}S   {WC/WZ/WCZ} ")

debug("Force D >= S. If D < S then D = S and C = 1")

debug("else D same and C = 0. *")

debug("--------------------------------------")

cogRunning := COGINIT(COGEXEC_NEW,@S0_FGE,0)

debug(udec(cogRunning))

repeat                                    'keep cog 0 running

DAT

ORG   0

S0_FGE              DRVH    #P0             'P0 on program running

MOV     Dest,#150    'load cog 1 Pro = 1234

MOV     Src,#100      'load cog 1 Src = 1234

MODZ    _CLR   WZ       'set Z = 0

MODC    _CLR   WC       'set C = 0

IF_C     MOV     Carry,#1        'true  C = 1

IF_NC  MOV     Carry,#0        'false C = 0

IF_Z     MOV     Zero,#1         'true  Z = 1

IF_NZ  MOV     Zero,#0         'false Z = 0

debug("--------------------------------------")

debug("before FGE  ",sdec(Dest),sdec(Src),ubin(Zero),ubin(Carry))

FGE  Dest,Src  WCZ

IF_C    MOV     Carry,#1         'true  C = 1

IF_NC  MOV     Carry,#0         'false C = 0

IF_Z     MOV     Zero,#1          'true  Z = 1

IF_NZ  MOV     Zero,#0          'false Z = 0

debug("after FGE   ",sdec(Dest),sdec(Src),ubin(Zero),ubin(Carry))

debug("--------------------------------------")

MOV     Dest,MAX         'load cog 1 Pro = $FFFF_FFFF

MOV     Src,valSrc       'load cog 1 Src = 1

MODZ    _CLR   WZ        'set Z = 0

MODC    _CLR   WC        'set C = 0

IF_C       MOV     Carry,#1         'true  C = 1

IF_NC    MOV     Carry,#0         'false C = 0

IF_Z      MOV     Zero,#1          'true  Z = 1

IF_NZ    MOV     Zero,#0          'false Z = 0

debug("before FGE ",sdec(Dest),sdec(Src),ubin(Zero),ubin(Carry))

FGE  Dest,Src  WCZ

IF_C      MOV     Carry,#1         'true  C = 1

IF_NC    MOV     Carry,#0         'false C = 0

IF_Z      MOV     Zero,#1          'true  Z = 1

IF_NZ    MOV     Zero,#0          'false Z = 0

debug("after FGE  ",sdec(Dest),sdec(Src),ubin(Zero),ubin(Carry))

debug("--------------------------------------")

MOV     Dest,#1       'load cog 1 Pro = 1

MOV     Src,valSrc    'load cog 1 Src = 1

MODZ    _CLR   WZ     'set Z = 0

MODC    _CLR   WC     'set C = 1

IF_C       MOV     Carry,#1      'true  C = 1

IF_NC    MOV     Carry,#0      'false C = 0

IF_Z      MOV     Zero,#1       'true  Z = 1

IF_NZ   MOV     Zero,#0       'false Z = 0

debug("before FGE  ",sdec(Dest),sdec(Src),ubin(Zero),ubin(Carry))

FGE  Dest,Src  WCZ

IF_C     MOV     Carry,#1      'true  C = 1

IF_NC   MOV     Carry,#0      'false C = 0

IF_Z      MOV     Zero,#1       'true  Z = 1

IF_NZ   MOV     Zero,#0       'false Z = 0

debug(ubin(Dest))

debug("after FGE   " ,sdec(Dest),sdec(Src),ubin(Zero),ubin(Carry))

debug("--------------------------------------")

_Loop1                NOP

JMP     #_Loop1          'remember # imediate

cog1Wait             long      900_000_000    'number of clock cycles to wait

cog1Cnt               long      0

valDest                long      1234         'cog 1 Dest

valSrc                   long      1234         'cog 1 Src

Carry                   long      0

Zero                  long      0

Dest                  long      0

Src                    long      0

MAX                 long      $FFFF_FFFF

MIN                   long      0

### 17.9.2_Example_WRD_FLE_027

FLEFor

{{17.9.2_Example_WRD_FLE_027}}

''FLE     D,{#}S   {WC/WZ/WCZ}   Less than or Equal

''Force D <= S. If D > S then D = S and C = 1,

''else D same and C = 0. *

CON

_clkfreq = 200_000_000  ''Debug must be enabled clock must be greater than 10MHZ for Debug

P0 =0 , P1 = 1  , P2 = 2

VAR

Byte cogRunning 'cog ID started is returned or -1 if not started

PUB main()

debug("--------------------------------------")

debug("FLE     D,{#}S   {WC/WZ/WCZ}   Less than or Equal")

debug("Force D <= S. If D > S then D = S and C = 1, ")

debug("else D same and C = 0. *  ")

debug("--------------------------------------")

cogRunning := COGINIT(COGEXEC_NEW,@S0_FLE,0)

debug(udec(cogRunning))

repeat                                    'keep cog 0 running

DAT

ORG   0

S0_FLE                DRVH    #P0             'P0 on program running

MOV     Dest,#150    'load cog 1 Pro = 1234

MOV     Src,#100      'load cog 1 Src = 1234

MODZ    _CLR   WZ       'set Z = 0

MODC    _CLR   WC       'set C = 0

IF_C   MOV     Carry,#1        'true  C = 1

IF_NC  MOV     Carry,#0        'false C = 0

IF_Z   MOV     Zero,#1         'true  Z = 1

IF_NZ  MOV     Zero,#0         'false Z = 0

debug("--------------------------------------")

debug("before FLE  ",sdec(Dest),sdec(Src),ubin(Zero),ubin(Carry))

FLE  Dest,Src  WCZ

IF_C   MOV     Carry,#1         'true  C = 1

IF_NC  MOV     Carry,#0         'false C = 0

IF_Z   MOV     Zero,#1          'true  Z = 1

IF_NZ  MOV     Zero,#0          'false Z = 0

debug("after FLE   ",sdec(Dest),sdec(Src),ubin(Zero),ubin(Carry))

debug("--------------------------------------")

MOV     Dest,MAX         'load cog 1 Pro = $FFFF_FFFF

MOV     Src,valSrc       'load cog 1 Src = 1

MODZ    _CLR   WZ        'set Z = 0

MODC    _CLR   WC        'set C = 0

IF_C   MOV     Carry,#1         'true  C = 1

IF_NC  MOV     Carry,#0         'false C = 0

IF_Z   MOV     Zero,#1          'true  Z = 1

IF_NZ  MOV     Zero,#0          'false Z = 0

debug("before FLE ",sdec(Dest),sdec(Src),ubin(Zero),ubin(Carry))

FLE  Dest,Src  WCZ

IF_C   MOV     Carry,#1         'true  C = 1

IF_NC  MOV     Carry,#0         'false C = 0

IF_Z   MOV     Zero,#1          'true  Z = 1

IF_NZ  MOV     Zero,#0          'false Z = 0

debug("after FLE  ",sdec(Dest),sdec(Src),ubin(Zero),ubin(Carry))

debug("--------------------------------------")

MOV     Dest,#1       'load cog 1 Pro = 1

MOV     Src,valSrc    'load cog 1 Src = 1

MODZ    _CLR   WZ     'set Z = 0

MODC    _CLR   WC     'set C = 1

IF_C   MOV     Carry,#1      'true  C = 1

IF_NC  MOV     Carry,#0      'false C = 0

IF_Z   MOV     Zero,#1       'true  Z = 1

IF_NZ  MOV     Zero,#0       'false Z = 0

debug("before FLE  ",sdec(Dest),sdec(Src),ubin(Zero),ubin(Carry))

FLE  Dest,Src  WCZ

IF_C   MOV     Carry,#1      'true  C = 1

IF_NC  MOV     Carry,#0      'false C = 0

IF_Z   MOV     Zero,#1       'true  Z = 1

IF_NZ  MOV     Zero,#0       'false Z = 0

debug(ubin(Dest))

debug("after FLE   " ,sdec(Dest),sdec(Src),ubin(Zero),ubin(Carry))

debug("--------------------------------------")

_Loop1               NOP

JMP     #_Loop1          'remember # imediate

cog1Wait         long      900_000_000    'number of clock cycles to wait

cog1Cnt           long      0

valDest             long      1234         'cog 1 Dest

valSrc                long      1234         'cog 1 Src

Carry                 long      0

Zero                  long      0

Dest                  long      0

Src                     long      0

MAX                  long      $FFFF_FFFF

MIN                   long      0

### 17.9.3_Example_WRD_FGES_028

FGES    D,{#}S   {WC/WZ/WCZ}

Force D >= S, signed. If D < S then D = S and C = 1,

else D same and C = 0. *

{{17.9.3_Example_WRD_FGES_028}}

''FGES    D,{#}S   {WC/WZ/WCZ}

''Force D >= S, signed. If D < S then D = S and C = 1,

''else D same and C = 0. *

CON

_clkfreq = 200_000_000  ''Debug must be enabled clock must be greater than 10MHZ for Debug

P0 =0 , P1 = 1  , P2 = 2

VAR

Byte cogRunning 'cog ID started is returned or -1 if not started

PUB main()

debug("--------------------------------------")

debug("FGES    D,{#}S   {WC/WZ/WCZ} ")

debug("Force D >= S, signed. If D < S then D = S and C = 1,")

debug("else D same and C = 0. * ")

debug("--------------------------------------")

cogRunning := COGINIT(COGEXEC_NEW,@S0_FGES,0)

debug(udec(cogRunning))

repeat                                    'keep cog 0 running

DAT

ORG   0

S0_FGES               DRVH    #P0             'P0 on program running

MOV     Dest,#150    'load cog 1 Pro = 1234

MOV     Src,#100      'load cog 1 Src = 1234

MODZ    _CLR   WZ       'set Z = 0

MODC    _CLR   WC       'set C = 0

IF_C   MOV     Carry,#1        'true  C = 1

IF_NC  MOV     Carry,#0        'false C = 0

IF_Z   MOV     Zero,#1         'true  Z = 1

IF_NZ  MOV     Zero,#0         'false Z = 0

debug("--------------------------------------")

debug("before FGES ",sdec(Dest),sdec(Src),ubin(Zero),ubin(Carry))

FGES  Dest,Src  WCZ

IF_C   MOV     Carry,#1         'true  C = 1

IF_NC  MOV     Carry,#0         'false C = 0

IF_Z   MOV     Zero,#1          'true  Z = 1

IF_NZ  MOV     Zero,#0          'false Z = 0

debug("after FGES  ",sdec(Dest),sdec(Src),ubin(Zero),ubin(Carry))

debug("--------------------------------------")

MOV     Dest,MAX         'load cog 1 Pro = $FFFF_FFFF

MOV     Src,valSrc       'load cog 1 Src = 1

MODZ    _CLR   WZ        'set Z = 0

MODC    _CLR   WC        'set C = 0

IF_C   MOV     Carry,#1         'true  C = 1

IF_NC  MOV     Carry,#0         'false C = 0

IF_Z   MOV     Zero,#1          'true  Z = 1

IF_NZ  MOV     Zero,#0          'false Z = 0

debug("before FGES ",sdec(Dest),sdec(Src),ubin(Zero),ubin(Carry))

FGES    Dest,Src  WCZ

IF_C   MOV     Carry,#1         'true  C = 1

IF_NC  MOV     Carry,#0         'false C = 0

IF_Z   MOV     Zero,#1          'true  Z = 1

IF_NZ  MOV     Zero,#0          'false Z = 0

debug("after FGES  ",sdec(Dest),sdec(Src),ubin(Zero),ubin(Carry))

debug("--------------------------------------")

MOV     Dest,Max      'load cog 1 Pro = 1

MOV     Src,valNeg2   'load cog 1 Src = 1

MODZ    _CLR   WZ     'set Z = 0

MODC    _CLR   WC     'set C = 1

IF_C   MOV     Carry,#1      'true  C = 1

IF_NC  MOV     Carry,#0      'false C = 0

IF_Z   MOV     Zero,#1       'true  Z = 1

IF_NZ  MOV     Zero,#0       'false Z = 0

debug("before FGES  ",sdec(Dest),sdec(Src),ubin(Zero),ubin(Carry))

FGES  Dest,Src  WCZ

IF_C   MOV     Carry,#1      'true  C = 1

IF_NC  MOV     Carry,#0      'false C = 0

IF_Z   MOV     Zero,#1       'true  Z = 1

IF_NZ  MOV     Zero,#0       'false Z = 0

debug(ubin(Dest),ubin(Src))

debug("after FGES   " ,sdec(Dest),sdec(Src),ubin(Zero),ubin(Carry))

debug("--------------------------------------")

_Loop1          NOP

JMP     #_Loop1          'remember # imediate

cog1Wait         long      900_000_000    'number of clock cycles to wait

cog1Cnt           long      0

valDest             long      1234         'cog 1 Dest

valSrc                long      1234         'cog 1 Src

valNeg2            long      -2

Carry                 long      0

Zero                  long      0

Dest                  long      0

Src                    long      0

MAX                  long      $FFFF_FFFF

MIN                   long      0

### 17.9.4_Example_WRD_FLES_029

FLES    D,{#}S   {WC/WZ/WCZ}

Force D <= S, signed. If D > S then D = S and C = 1,

else D same and C = 0. *

{{17.9.4_Example_WRD_FLES_029}}

''FLES    D,{#}S   {WC/WZ/WCZ}

''Force D <= S, signed. If D > S then D = S and C = 1,

''else D same and C = 0. *

CON

_clkfreq = 200_000_000  ''Debug must be enabled clock must be greater than 10MHZ for Debug

P0 =0 , P1 = 1  , P2 = 2

VAR

Byte cogRunning 'cog ID started is returned or -1 if not started

PUB main()

debug("--------------------------------------")

debug("FLES    D,{#}S   {WC/WZ/WCZ}  ")

debug("Force D <= S, signed. If D > S then D = S and C = 1,")

debug("else D same and C = 0. *")

debug("--------------------------------------")

cogRunning := COGINIT(COGEXEC_NEW,@S0_FLES,0)

debug(udec(cogRunning))

repeat                                    'keep cog 0 running

DAT

ORG   0

S0_FLES               DRVH    #P0             'P0 on program running

MOV     Dest,#150    'load cog 1 Pro = 1234

MOV     Src,#100      'load cog 1 Src = 1234

MODZ    _CLR   WZ       'set Z = 0

MODC    _CLR   WC       'set C = 0

IF_C   MOV     Carry,#1        'true  C = 1

IF_NC  MOV     Carry,#0        'false C = 0

IF_Z   MOV     Zero,#1         'true  Z = 1

IF_NZ  MOV     Zero,#0         'false Z = 0

debug("--------------------------------------")

debug("before FLES ",sdec(Dest),sdec(Src),ubin(Zero),ubin(Carry))

FLES  Dest,Src  WCZ

IF_C   MOV     Carry,#1         'true  C = 1

IF_NC  MOV     Carry,#0         'false C = 0

IF_Z   MOV     Zero,#1          'true  Z = 1

IF_NZ  MOV     Zero,#0          'false Z = 0

debug("after FLES  ",sdec(Dest),sdec(Src),ubin(Zero),ubin(Carry))

debug("--------------------------------------")

MOV     Dest,MAX         'load cog 1 Pro = $FFFF_FFFF

MOV     Src,valSrc       'load cog 1 Src = 1

MODZ    _CLR   WZ        'set Z = 0

MODC    _CLR   WC        'set C = 0

IF_C   MOV     Carry,#1         'true  C = 1

IF_NC  MOV     Carry,#0         'false C = 0

IF_Z   MOV     Zero,#1          'true  Z = 1

IF_NZ  MOV     Zero,#0          'false Z = 0

debug("before FLES ",sdec(Dest),sdec(Src),ubin(Zero),ubin(Carry))

FLES    Dest,Src  WCZ

IF_C   MOV     Carry,#1         'true  C = 1

IF_NC  MOV     Carry,#0         'false C = 0

IF_Z   MOV     Zero,#1          'true  Z = 1

IF_NZ  MOV     Zero,#0          'false Z = 0

debug("after FLES  ",sdec(Dest),sdec(Src),ubin(Zero),ubin(Carry))

debug("--------------------------------------")

MOV     Dest,Max      'load cog 1 Pro = 1

MOV     Src,valNeg2   'load cog 1 Src = 1

MODZ    _CLR   WZ     'set Z = 0

MODC    _CLR   WC     'set C = 1

IF_C   MOV     Carry,#1      'true  C = 1

IF_NC  MOV     Carry,#0      'false C = 0

IF_Z   MOV     Zero,#1       'true  Z = 1

IF_NZ  MOV     Zero,#0       'false Z = 0

debug("before FLES  ",sdec(Dest),sdec(Src),ubin(Zero),ubin(Carry))

FLES    Dest,Src  WCZ

IF_C   MOV     Carry,#1      'true  C = 1

IF_NC  MOV     Carry,#0      'false C = 0

IF_Z   MOV     Zero,#1       'true  Z = 1

IF_NZ  MOV     Zero,#0       'false Z = 0

debug(ubin(Dest))

debug("after FLES   " ,sdec(Dest),sdec(Src),ubin(Zero),ubin(Carry))

debug("--------------------------------------")

_Loop1                NOP

JMP     #_Loop1          'remember # imediate

cog1Wait              long      900_000_000    'number of clock cycles to wait

cog1Cnt               long      0

valDest               long      1234         'cog 1 Dest

valSrc                long      1234         'cog 1 Src

valNeg2               long      -2

Carry                 long      0

Zero                  long      0

Dest                  long      0

Src                   long      0

MAX                   long      $FFFF_FFFF

MIN                   long      0

## 17.10) SUM  ADD/SUB Based on C/Z

| SUMC D,{#}S {WC/WZ/WCZ} | Sum +/-S into D by C. If C = 1 then D = D - S, else D = D + S. C = correct sign of (D +/- S). * |
| --- | --- |
| SUMNC D,{#}S {WC/WZ/WCZ} | Sum +/-S into D by !C. If C = 0 then D = D - S, else D = D + S. C = correct sign of (D +/- S). * |
| SUMZ D,{#}S {WC/WZ/WCZ} | Sum +/-S into D by Z. If Z = 1 then D = D - S, else D = D + S. C = correct sign of (D +/- S). * |
| SUMNZ D,{#}S {WC/WZ/WCZ} | Sum +/-S into D by !Z. If Z = 0 then D = D - S, else D = D + S. C = correct sign of (D +/- S). * |

### 17.10.1_Example_WRD_SUMC_030

SUMC D,{#}S {WC/WZ/WCZ}

Sum +/-S into D by C. If C = 1 then D = D - S, else D = D + S. C = correct sign of (D +/- S). *

{{17.10.1_Example_WRD_SUMC_030}}

''SUMC D,{#}S {WC/WZ/WCZ}

''Sum +/-S into D by C. If C = 1 then D = D - S, else D = D + S. C = correct sign of (D +/- S). *

CON

_clkfreq = 200_000_000  ''Debug must be enabled clock must be greater than 10MHZ for Debug

P0 =0 , P1 = 1  , P2 = 2

VAR

Byte cogRunning 'cog ID started is returned or -1 if not started

PUB main()

debug("--------------------------------------")

debug("SUMC D,{#}S {WC/WZ/WCZ}")

debug("Sum +/-S into D by C. If C = 1 then D = D - S,")

debug("else D = D + S. C = correct sign of (D +/- S). *")

debug("--------------------------------------")

cogRunning := COGINIT(COGEXEC_NEW,@S0_SUMC,0)

debug(udec(cogRunning))

repeat                                    'keep cog 0 running

DAT

ORG   0

S0_SUMC               DRVH    #P0             'P0 on program running

MOV     Dest,#150    'load cog 1 Pro = 1234

MOV     Src,#100      'load cog 1 Src = 1234

MODZ    _CLR   WZ       'set Z = 0

MODC    _CLR   WC       'set C = 0

IF_C   MOV     Carry,#1        'true  C = 1

IF_NC  MOV     Carry,#0        'false C = 0

IF_Z   MOV     Zero,#1         'true  Z = 1

IF_NZ  MOV     Zero,#0         'false Z = 0

debug("--------------------------------------")

debug("before SUMC ",sdec(Dest),sdec(Src),ubin(Zero),ubin(Carry))

SUMC    Dest,Src  WCZ

IF_C   MOV     Carry,#1         'true  C = 1

IF_NC  MOV     Carry,#0         'false C = 0

IF_Z   MOV     Zero,#1          'true  Z = 1

IF_NZ  MOV     Zero,#0          'false Z = 0

debug("after SUMC  ",sdec(Dest),sdec(Src),ubin(Zero),ubin(Carry))

debug("--------------------------------------")

MOV     Dest,MAX         'load cog 1 Pro = $FFFF_FFFF

MOV     Src,valSrc       'load cog 1 Src = 1

MODZ    _CLR   WZ        'set Z = 0

MODC    _SET   WC        'set C = 1

IF_C   MOV     Carry,#1         'true  C = 1

IF_NC  MOV     Carry,#0         'false C = 0

IF_Z   MOV     Zero,#1          'true  Z = 1

IF_NZ  MOV     Zero,#0          'false Z = 0

debug("before SUMC ",sdec(Dest),sdec(Src),ubin(Zero),ubin(Carry))

SUMC    Dest,Src  WCZ

IF_C   MOV     Carry,#1         'true  C = 1

IF_NC  MOV     Carry,#0         'false C = 0

IF_Z   MOV     Zero,#1          'true  Z = 1

IF_NZ  MOV     Zero,#0          'false Z = 0

debug("after SUMC  ",sdec(Dest),sdec(Src),ubin(Zero),ubin(Carry))

debug("--------------------------------------")

MOV     Dest,Max      'load cog 1 Pro = 1

MOV     Src,valNeg2   'load cog 1 Src = 1

MODZ    _CLR   WZ     'set Z = 0

MODC    _CLR   WC     'set C = 1

IF_C   MOV     Carry,#1      'true  C = 1

IF_NC  MOV     Carry,#0      'false C = 0

IF_Z   MOV     Zero,#1       'true  Z = 1

IF_NZ  MOV     Zero,#0       'false Z = 0

debug("before SUMC  ",sdec(Dest),sdec(Src),ubin(Zero),ubin(Carry))

SUMC    Dest,Src  WCZ

IF_C   MOV     Carry,#1      'true  C = 1

IF_NC  MOV     Carry,#0      'false C = 0

IF_Z   MOV     Zero,#1       'true  Z = 1

IF_NZ  MOV     Zero,#0       'false Z = 0

debug(ubin(Dest))

debug("after SUMC   " ,sdec(Dest),sdec(Src),ubin(Zero),ubin(Carry))

debug("--------------------------------------")

_Loop1                NOP

JMP     #_Loop1          'remember # imediate

cog1Wait              long      900_000_000    'number of clock cycles to wait

cog1Cnt               long      0

valDest               long      1234         'cog 1 Dest

valSrc                long      1234         'cog 1 Src

valNeg2               long      -2

Carry                 long      0

Zero                  long      0

Dest                  long      0

Src                   long      0

MAX                   long      $FFFF_FFFF

MIN                   long      0

### 17.10.2_Example_WRD_SUMC_031

SUMNC D,{#}S {WC/WZ/WCZ}

Sum +/-S into D by !C. If C = 0 then D = D - S, else D = D + S. C = correct sign of (D +/- S). *

{{17.10.2_Example_WRD_SUMNC_031}}

''SUMNC D,{#}S {WC/WZ/WCZ}

''Sum +/-S into D by !C. If C = 0 then D = D - S, else D = D + S. C = correct sign of (D +/- S). *

CON

_clkfreq = 200_000_000  ''Debug must be enabled clock must be greater than 10MHZ for Debug

P0 =0 , P1 = 1  , P2 = 2

VAR

Byte cogRunning 'cog ID started is returned or -1 if not started

PUB main()

debug("--------------------------------------")

debug("SUMNC D,{#}S {WC/WZ/WCZ}")

debug("Sum +/-S into D by !C. If C = 0 then D = D - S,")

debug("else D = D + S. C = correct sign of (D +/- S). * ")

debug("--------------------------------------")

cogRunning := COGINIT(COGEXEC_NEW,@S0_SUMNC,0)

debug(udec(cogRunning))

repeat                                    'keep cog 0 running

DAT

ORG   0

S0_SUMNC              DRVH    #P0             'P0 on program running

MOV     Dest,#150    'load cog 1 Pro = 1234

MOV     Src,#100      'load cog 1 Src = 1234

MODZ    _CLR   WZ       'set Z = 0

MODC    _CLR   WC       'set C = 0

IF_C   MOV     Carry,#1        'true  C = 1

IF_NC  MOV     Carry,#0        'false C = 0

IF_Z   MOV     Zero,#1         'true  Z = 1

IF_NZ  MOV     Zero,#0         'false Z = 0

debug("--------------------------------------")

debug("before SUMNC ",sdec(Dest),sdec(Src),ubin(Zero),ubin(Carry))

SUMNC    Dest,Src  WCZ

IF_C   MOV     Carry,#1         'true  C = 1

IF_NC  MOV     Carry,#0         'false C = 0

IF_Z   MOV     Zero,#1          'true  Z = 1

IF_NZ  MOV     Zero,#0          'false Z = 0

debug("after SUMNC  ",sdec(Dest),sdec(Src),ubin(Zero),ubin(Carry))

debug("--------------------------------------")

MOV     Dest,MAX         'load cog 1 Pro = $FFFF_FFFF

MOV     Src,valSrc       'load cog 1 Src = 1

MODZ    _CLR   WZ        'set Z = 0

MODC    _SET   WC        'set C = 1

IF_C   MOV     Carry,#1         'true  C = 1

IF_NC  MOV     Carry,#0         'false C = 0

IF_Z   MOV     Zero,#1          'true  Z = 1

IF_NZ  MOV     Zero,#0          'false Z = 0

debug("before SUMNC ",sdec(Dest),sdec(Src),ubin(Zero),ubin(Carry))

SUMNC    Dest,Src  WCZ

IF_C   MOV     Carry,#1         'true  C = 1

IF_NC  MOV     Carry,#0         'false C = 0

IF_Z   MOV     Zero,#1          'true  Z = 1

IF_NZ  MOV     Zero,#0          'false Z = 0

debug("after SUMNC  ",sdec(Dest),sdec(Src),ubin(Zero),ubin(Carry))

debug("--------------------------------------")

MOV     Dest,Max      'load cog 1 Pro = 1

MOV     Src,valNeg2   'load cog 1 Src = 1

MODZ    _CLR   WZ     'set Z = 0

MODC    _CLR   WC     'set C = 1

IF_C   MOV     Carry,#1      'true  C = 1

IF_NC  MOV     Carry,#0      'false C = 0

IF_Z   MOV     Zero,#1       'true  Z = 1

IF_NZ  MOV     Zero,#0       'false Z = 0

debug("before SUMNC  ",sdec(Dest),sdec(Src),ubin(Zero),ubin(Carry))

SUMNC    Dest,Src  WCZ

IF_C   MOV     Carry,#1      'true  C = 1

IF_NC  MOV     Carry,#0      'false C = 0

IF_Z   MOV     Zero,#1       'true  Z = 1

IF_NZ  MOV     Zero,#0       'false Z = 0

debug(ubin(Dest))

debug("after SUMNC   " ,sdec(Dest),sdec(Src),ubin(Zero),ubin(Carry))

debug("--------------------------------------")

_Loop1                NOP

JMP     #_Loop1          'remember # imediate

cog1Wait              long      900_000_000    'number of clock cycles to wait

cog1Cnt               long      0

valDest               long      1234         'cog 1 Dest

valSrc                long      1234         'cog 1 Src

valNeg2               long      -2

Carry                 long      0

Zero                  long      0

Dest                  long      0

Src                   long      0

MAX                   long      $FFFF_FFFF

MIN                   long      0

### 17.10.3_Example_WRD_SUMZ_032

SUMZ D,{#}S {WC/WZ/WCZ}

Sum +/-S into D by Z. If Z = 1 then D = D - S, else D = D + S. C = correct sign of (D +/- S). *

{{17.10.3_Example_WRD_SUMZ_032}}

''SUMZ D,{#}S {WC/WZ/WCZ}

''Sum +/-S into D by Z. If Z = 1 then D = D - S, else D = D + S. C = correct sign of (D +/- S). *

CON

_clkfreq = 200_000_000  ''Debug must be enabled clock must be greater than 10MHZ for Debug

P0 =0 , P1 = 1  , P2 = 2

VAR

Byte cogRunning 'cog ID started is returned or -1 if not started

PUB main()

debug("--------------------------------------")

debug("SUMZ D,{#}S {WC/WZ/WCZ} ")

debug("Sum +/-S into D by Z. If Z = 1 then D = D - S,")

debug("else D = D + S. C = correct sign of (D +/- S). * ")

debug("--------------------------------------")

cogRunning := COGINIT(COGEXEC_NEW,@S0_SUMZ,0)

debug(udec(cogRunning))

repeat                                    'keep cog 0 running

DAT

ORG   0

S0_SUMZ               DRVH    #P0             'P0 on program running

MOV     Dest,#150    'load cog 1 Pro = 1234

MOV     Src,#100      'load cog 1 Src = 1234

MODZ    _CLR   WZ       'set Z = 0

MODC    _CLR   WC       'set C = 0

IF_C   MOV     Carry,#1        'true  C = 1

IF_NC  MOV     Carry,#0        'false C = 0

IF_Z   MOV     Zero,#1         'true  Z = 1

IF_NZ  MOV     Zero,#0         'false Z = 0

debug("--------------------------------------")

debug("before SUMZ ",sdec(Dest),sdec(Src),ubin(Zero),ubin(Carry))

SUMZ    Dest,Src  WCZ

IF_C   MOV     Carry,#1         'true  C = 1

IF_NC  MOV     Carry,#0         'false C = 0

IF_Z   MOV     Zero,#1          'true  Z = 1

IF_NZ  MOV     Zero,#0          'false Z = 0

debug("after SUMZ  ",sdec(Dest),sdec(Src),ubin(Zero),ubin(Carry))

debug("--------------------------------------")

MOV     Dest,MAX         'load cog 1 Pro = $FFFF_FFFF

MOV     Src,valSrc       'load cog 1 Src = 1

MODZ    _CLR   WZ        'set Z = 0

MODC    _SET   WC        'set C = 1

IF_C   MOV     Carry,#1         'true  C = 1

IF_NC  MOV     Carry,#0         'false C = 0

IF_Z   MOV     Zero,#1          'true  Z = 1

IF_NZ  MOV     Zero,#0          'false Z = 0

debug("before SUMZ ",sdec(Dest),sdec(Src),ubin(Zero),ubin(Carry))

SUMZ    Dest,Src  WCZ

IF_C   MOV     Carry,#1         'true  C = 1

IF_NC  MOV     Carry,#0         'false C = 0

IF_Z   MOV     Zero,#1          'true  Z = 1

IF_NZ  MOV     Zero,#0          'false Z = 0

debug("after SUMZ  ",sdec(Dest),sdec(Src),ubin(Zero),ubin(Carry))

debug("--------------------------------------")

MOV     Dest,Max      'load cog 1 Pro = 1

MOV     Src,valNeg2   'load cog 1 Src = 1

MODZ    _CLR   WZ     'set Z = 0

MODC    _CLR   WC     'set C = 1

IF_C   MOV     Carry,#1      'true  C = 1

IF_NC  MOV     Carry,#0      'false C = 0

IF_Z   MOV     Zero,#1       'true  Z = 1

IF_NZ  MOV     Zero,#0       'false Z = 0

debug("before SUMZ  ",sdec(Dest),sdec(Src),ubin(Zero),ubin(Carry))

SUMZ    Dest,Src  WCZ

IF_C   MOV     Carry,#1      'true  C = 1

IF_NC  MOV     Carry,#0      'false C = 0

IF_Z   MOV     Zero,#1       'true  Z = 1

IF_NZ  MOV     Zero,#0       'false Z = 0

debug(ubin(Dest))

debug("after SUMZ   " ,sdec(Dest),sdec(Src),ubin(Zero),ubin(Carry))

debug("--------------------------------------")

_Loop1                NOP

JMP     #_Loop1          'remember # imediate

cog1Wait              long      900_000_000    'number of clock cycles to wait

cog1Cnt               long      0

valDest               long      1234         'cog 1 Dest

valSrc                long      1234         'cog 1 Src

valNeg2               long      -2

Carry                 long      0

Zero                  long      0

Dest                  long      0

Src                   long      0

MAX                   long      $FFFF_FFFF

MIN                   long      0

### 17.10..4_Example_WRD_SUMNZ_033

SUMNZ D,{#}S {WC/WZ/WCZ}

Sum +/-S into D by !Z. If Z = 0 then D = D - S, else D = D + S. C = correct sign of (D +/- S). *

{{17.10.4_Example_WRD_SUMNZ_033}}

''SUMNZ D,{#}S {WC/WZ/WCZ}

''Sum +/-S into D by !Z. If Z = 0 then D = D - S, else D = D + S. C = correct sign of (D +/- S). *

CON

_clkfreq = 200_000_000  ''Debug must be enabled clock must be greater than 10MHZ for Debug

P0 =0 , P1 = 1  , P2 = 2

VAR

Byte cogRunning 'cog ID started is returned or -1 if not started

PUB main()

debug("--------------------------------------")

debug("SUMNZ D,{#}S {WC/WZ/WCZ}    ")

debug("Sum +/-S into D by !Z. If Z = 0 then D = D - S,")

debug("else D = D + S. C = correct sign of (D +/- S). *")

debug("--------------------------------------")

cogRunning := COGINIT(COGEXEC_NEW,@S0_SUMNZ,0)

debug(udec(cogRunning))

repeat                                    'keep cog 0 running

DAT

ORG   0

S0_SUMNZ              DRVH    #P0             'P0 on program running

MOV     Dest,#150    'load cog 1 Pro = 1234

MOV     Src,#100      'load cog 1 Src = 1234

MODZ    _CLR   WZ       'set Z = 0

MODC    _CLR   WC       'set C = 0

IF_C   MOV     Carry,#1        'true  C = 1

IF_NC  MOV     Carry,#0        'false C = 0

IF_Z   MOV     Zero,#1         'true  Z = 1

IF_NZ  MOV     Zero,#0         'false Z = 0

debug("--------------------------------------")

debug("before SUMNZ ",sdec(Dest),sdec(Src),ubin(Zero),ubin(Carry))

SUMNZ    Dest,Src  WCZ

IF_C   MOV     Carry,#1         'true  C = 1

IF_NC  MOV     Carry,#0         'false C = 0

IF_Z   MOV     Zero,#1          'true  Z = 1

IF_NZ  MOV     Zero,#0          'false Z = 0

debug("after SUMNZ  ",sdec(Dest),sdec(Src),ubin(Zero),ubin(Carry))

debug("--------------------------------------")

MOV     Dest,MAX         'load cog 1 Pro = $FFFF_FFFF

MOV     Src,valSrc       'load cog 1 Src = 1

MODZ    _CLR   WZ        'set Z = 0

MODC    _SET   WC        'set C = 1

IF_C   MOV     Carry,#1         'true  C = 1

IF_NC  MOV     Carry,#0         'false C = 0

IF_Z   MOV     Zero,#1          'true  Z = 1

IF_NZ  MOV     Zero,#0          'false Z = 0

debug("before SUMNZ ",sdec(Dest),sdec(Src),ubin(Zero),ubin(Carry))

SUMNZ   Dest,Src  WCZ

IF_C   MOV     Carry,#1         'true  C = 1

IF_NC  MOV     Carry,#0         'false C = 0

IF_Z   MOV     Zero,#1          'true  Z = 1

IF_NZ  MOV     Zero,#0          'false Z = 0

debug("after SUMNZ  ",sdec(Dest),sdec(Src),ubin(Zero),ubin(Carry))

debug("--------------------------------------")

MOV     Dest,Max      'load cog 1 Pro = 1

MOV     Src,valNeg2   'load cog 1 Src = 1

MODZ    _SET   WZ     'set Z = 0

MODC    _CLR   WC     'set C = 1

IF_C   MOV     Carry,#1      'true  C = 1

IF_NC  MOV     Carry,#0      'false C = 0

IF_Z   MOV     Zero,#1       'true  Z = 1

IF_NZ  MOV     Zero,#0       'false Z = 0

debug("before SUMNZ  ",sdec(Dest),sdec(Src),ubin(Zero),ubin(Carry))

SUMNZ    Dest,Src  WCZ

IF_C   MOV     Carry,#1      'true  C = 1

IF_NC  MOV     Carry,#0      'false C = 0

IF_Z   MOV     Zero,#1       'true  Z = 1

IF_NZ  MOV     Zero,#0       'false Z = 0

debug(ubin(Dest))

debug("after SUMNZ   " ,sdec(Dest),sdec(Src),ubin(Zero),ubin(Carry))

debug("--------------------------------------")

_Loop1                NOP

JMP     #_Loop1          'remember # imediate

cog1Wait              long      900_000_000    'number of clock cycles to wait

cog1Cnt               long      0

valDest               long      1234         'cog 1 Dest

valSrc                long      1234         'cog 1 Src

valNeg2               long      -2

Carry                 long      0

Zero                  long      0

Dest                  long      0

Src                   long      0

MAX                   long      $FFFF_FFFF

MIN                   long      0

## 17.11) TEST Register Bit Set Flags

| TESTB D,{#}S WC/WZ | Test bit S[4:0] of D, write to C/Z. C/Z = D[S[4:0]]. |
| --- | --- |
| TESTBN D,{#}S WC/WZ | Test bit S[4:0] of !D, write to C/Z. C/Z = !D[S[4:0]]. |
| TESTB D,{#}S ANDC/ANDZ | Test bit S[4:0] of D, AND into C/Z. C/Z = C/Z AND D[S[4:0]]. |
| TESTBN D,{#}S ANDC/ANDZ | Test bit S[4:0] of !D, AND into C/Z. C/Z = C/Z AND !D[S[4:0]]. |
| TESTB D,{#}S ORC/ORZ | Test bit S[4:0] of D, OR into C/Z. C/Z = C/Z OR D[S[4:0]]. |
| TESTBN D,{#}S ORC/ORZ | Test bit S[4:0] of !D, OR into C/Z. C/Z = C/Z OR !D[S[4:0]]. |
| TESTB D,{#}S XORC/XORZ | Test bit S[4:0] of D, XOR into C/Z. C/Z = C/Z XOR D[S[4:0]]. |
| TESTBN D,{#}S XORC/XORZ | Test bit S[4:0] of !D, XOR into C/Z. C/Z = C/Z XOR !D[S[4:0]]. |

Test with conditions status of D register bits as requested by S register  if set place result in C/Z.

### 17.11.1_Example_WRD_TESTB_034

TESTB D,{#}S WC/WZ

Test bit S[4:0] of D, write to C/Z. C/Z = D[S[4:0]].

{{17.11.1_Example_WRD_TESTB_034}}

{{

TESTB D,{#}S WC/WZ

Test bit S[4:0] of D, write to C/Z. C/Z = D[S[4:0]].

Example

TESTB with WC Set Bits in 'Dest' and check Carry then TESTB with WZ Set Bits in 'Dest' and Check Zero

}}

CON

_clkfreq = 200_000_000  ''Debug must be enabled clock must be greater than 10MHZ for Debug

P0 =0 , P1 = 1  , P2 = 2

VAR

Byte cogRunning 'cog ID started is returned or -1 if not started

PUB main()

debug("--------------------------------------")

debug("TESTB D,{#}S WC/WZ")

debug("Test bit S[4:0] of D, write to C/Z. C/Z = D[S[4:0]].")

debug(" ")

debug("Example")

debug("TESTB with WC Set Bits in 'Dest' and check Carry")

debug("then TESTB with WZ Set Bits in 'Dest' and Check Zero")

debug("--------------------------------------")

cogRunning := COGINIT(COGEXEC_NEW,@S0_TESTB,0)

debug(udec(cogRunning))

repeat                                    'keep cog 0 running

DAT

ORG   0

S0_TESTB              DRVH    #P0             'P0 on program running

MOV     Dest,#1+256         'set In Dest set B0 = 1

MOV     Src,#0          'point to B0 S[4:0] = 0

MODZ    _CLR   WZ       'set Z = 0

MODC    _CLR   WC       'set C = 0

IF_C   MOV     Carry,#1        'true  C = 1

IF_NC  MOV     Carry,#0        'false C = 0

IF_Z   MOV     Zero,#1         'true  Z = 1

IF_NZ  MOV     Zero,#0         'false Z = 0

debug("--------------------------------------")

debug("TESTB with WC")

debug("--------------------------------------")

debug("before TESTB WC = ",ubin(Dest),ubin(Src),udec(Zero),udec(Carry))

TESTB   Dest,Src  WC

IF_C   MOV     Carry,#1         'true  C = 1

IF_NC  MOV     Carry,#0         'false C = 0

IF_Z   MOV     Zero,#1          'true  Z = 1

IF_NZ  MOV     Zero,#0          'false Z = 0

debug("after TESTB WC =  ",ubin(Dest),ubin(Src),udec(Zero),udec(Carry))

debug("--------------------------------------")

MOV     Dest,#2+256      'set In Dest set B2 = 1

MOV     Src,#1           'point to B1 S[4:0] = 1

MODZ    _CLR   WZ        'set Z = 0

MODC    _CLR   WC        'set C = 1

IF_C   MOV     Carry,#1         'true  C = 1

IF_NC  MOV     Carry,#0         'false C = 0

IF_Z   MOV     Zero,#1          'true  Z = 1

IF_NZ  MOV     Zero,#0          'false Z = 0

debug("before TESTB WC = ",ubin(Dest),ubin(Src),udec(Zero),udec(Carry))

TESTB   Dest,Src  WC

IF_C   MOV     Carry,#1         'true  C = 1

IF_NC  MOV     Carry,#0         'false C = 0

IF_Z   MOV     Zero,#1          'true  Z = 1

IF_NZ  MOV     Zero,#0          'false Z = 0

debug("after TESTB WC = ",ubin(Dest),ubin(Src),udec(Zero),udec(Carry))

debug("--------------------------------------")

MOV     Dest,#4+256   'set In Dest set B2 = 1

MOV     Src,#2        'point to B2 S[4:0] = 2

MODZ    _CLR   WZ     'set Z = 0

MODC    _CLR   WC     'set C = 1

IF_C   MOV     Carry,#1      'true  C = 1

IF_NC  MOV     Carry,#0      'false C = 0

IF_Z   MOV     Zero,#1       'true  Z = 1

IF_NZ  MOV     Zero,#0       'false Z = 0

debug("before TESTB WC = ",ubin(Dest),ubin(Src),udec(Zero),udec(Carry))

TESTB   Dest,Src  WC

IF_C   MOV     Carry,#1      'true  C = 1

IF_NC  MOV     Carry,#0      'false C = 0

IF_Z   MOV     Zero,#1       'true  Z = 1

IF_NZ  MOV     Zero,#0       'false Z = 0

debug("after TESTB   " ,ubin(Dest),ubin(Src),udec(Zero),udec(Carry))

debug("--------------------------------------")

debug("TestB with WZ")

MOV     Dest,#1+256     'set In Dest set B0 = 1

MOV     Src,#0          'point to B0 S[4:0] = 0

MODZ    _CLR   WZ       'set Z = 0

MODC    _CLR   WC       'set C = 0

IF_C   MOV     Carry,#1        'true  C = 1

IF_NC  MOV     Carry,#0        'false C = 0

IF_Z   MOV     Zero,#1         'true  Z = 1

IF_NZ  MOV     Zero,#0         'false Z = 0

debug("--------------------------------------")

debug("before TESTB WZ = ",ubin(Dest),ubin(Src),udec(Zero),udec(Carry))

TESTB   Dest,Src  WZ

IF_C   MOV     Carry,#1         'true  C = 1

IF_NC  MOV     Carry,#0         'false C = 0

IF_Z   MOV     Zero,#1          'true  Z = 1

IF_NZ  MOV     Zero,#0          'false Z = 0

debug("after TESTB WZ ",ubin(Dest),ubin(Src),udec(Zero),udec(Carry))

debug("--------------------------------------")

MOV     Dest,#2+256      'set In Dest set B1 = 1

MOV     Src,#1           'point to B2 S[4:0] = 1

MODZ    _CLR   WZ        'set Z = 0

MODC    _CLR   WC        'set C = 1

IF_C   MOV     Carry,#1         'true  C = 1

IF_NC  MOV     Carry,#0         'false C = 0

IF_Z   MOV     Zero,#1          'true  Z = 1

IF_NZ  MOV     Zero,#0          'false Z = 0

debug("before TESTB WZ = ",ubin(Dest),ubin(Src),udec(Zero),udec(Carry))

TESTB   Dest,Src  WZ

IF_C   MOV     Carry,#1         'true  C = 1

IF_NC  MOV     Carry,#0         'false C = 0

IF_Z   MOV     Zero,#1          'true  Z = 1

IF_NZ  MOV     Zero,#0          'false Z = 0

debug("after TESTB WZ =  ",ubin(Dest),ubin(Src),udec(Zero),udec(Carry))

debug("--------------------------------------")

MOV     Dest,#4+256   'set In Dest set B2 = 1

MOV     Src,#2        'point to B2 S[4:0] = 2

MODZ    _CLR   WZ     'set Z = 0

MODC    _CLR   WC     'set C = 1

IF_C   MOV     Carry,#1      'true  C = 1

IF_NC  MOV     Carry,#0      'false C = 0

IF_Z   MOV     Zero,#1       'true  Z = 1

IF_NZ  MOV     Zero,#0       'false Z = 0

debug("before TESTB WZ = ",ubin(Dest),ubin(Src),udec(Zero),udec(Carry))

TESTB   Dest,Src  WZ

IF_C   MOV     Carry,#1      'true  C = 1

IF_NC  MOV     Carry,#0      'false C = 0

IF_Z   MOV     Zero,#1       'true  Z = 1

IF_NZ  MOV     Zero,#0       'false Z = 0

debug("after TESTB WZ =  " ,ubin(Dest),ubin(Src),udec(Zero),udec(Carry))

debug("--------------------------------------")

_Loop1                NOP

JMP     #_Loop1          'remember # imediate

Carry                 long      0

Zero                  long      0

Dest                  long      0

Src                   long      0

### 17.11.2_Example_WRD_TESTBN_035

TESTBN D,{#}S WC/WZ

Test bit S[4:0] of !D, write to C/Z. C/Z = !D[S[4:0]].

{{17.11.2_Example_WRD_TESTB_035}}

''TESTBN D,{#}S WC/WZ

''Test bit S[4:0] of !D, write to C/Z. C/Z = !D[S[4:0]].

''Negate D and test looking for zero

CON

_clkfreq = 200_000_000  ''Debug must be enabled clock must be greater than 10MHZ for Debug

P0 =0 , P1 = 1  , P2 = 2

VAR

Byte cogRunning 'cog ID started is returned or -1 if not started

PUB main()

debug("--------------------------------------")

debug("TESTBN D,{#}S WC/WZ")

debug("Test bit S[4:0] of !D, write to C/Z. C/Z = D[S[4:0]].")

debug("--------------------------------------")

cogRunning := COGINIT(COGEXEC_NEW,@S0_TESTBN,0)

debug(udec(cogRunning))

repeat                                    'keep cog 0 running

DAT

ORG   0

S0_TESTBN             DRVH    #P0             'P0 on program running

MOV     Dest,#%1111_1110 'load cog 1 Pro = 1234

MOV     Src,#0           'load cog 1 Src = 1234

MODZ    _CLR   WZ       'set Z = 0

MODC    _CLR   WC       'set C = 0

IF_C   MOV     Carry,#1        'true  C = 1

IF_NC  MOV     Carry,#0        'false C = 0

IF_Z   MOV     Zero,#1         'true  Z = 1

IF_NZ  MOV     Zero,#0         'false Z = 0

debug("--------------------------------------")

debug("before TESTBN ",sbin(Dest),sdec(Src),ubin(Zero),ubin(Carry))

TESTBN  Dest,Src  WC

IF_C   MOV     Carry,#1         'true  C = 1

IF_NC  MOV     Carry,#0         'false C = 0

IF_Z   MOV     Zero,#1          'true  Z = 1

IF_NZ  MOV     Zero,#0          'false Z = 0

debug("after TESTBN  ",sbin(Dest),sdec(Src),ubin(Zero),ubin(Carry))

debug("--------------------------------------")

MOV     Dest,#%1111_1101         'load cog 1 Pro = $FFFF_FFFF

MOV     Src,#1                   'load cog 1 Src = 1

MODZ    _CLR   WZ        'set Z = 0

MODC    _CLR   WC        'set C = 1

IF_C   MOV     Carry,#1         'true  C = 1

IF_NC  MOV     Carry,#0         'false C = 0

IF_Z   MOV     Zero,#1          'true  Z = 1

IF_NZ  MOV     Zero,#0          'false Z = 0

debug("before TESTBN ",sbin(Dest),sdec(Src),ubin(Zero),ubin(Carry))

TESTBN   Dest,Src  WC

IF_C   MOV     Carry,#1         'true  C = 1

IF_NC  MOV     Carry,#0         'false C = 0

IF_Z   MOV     Zero,#1          'true  Z = 1

IF_NZ  MOV     Zero,#0          'false Z = 0

debug("after TESTBN  ",sbin(Dest),sdec(Src),ubin(Zero),ubin(Carry))

debug("--------------------------------------")

MOV     Dest,#%1111_1011      'load cog 1 Pro = 1

MOV     Src,#2       'load cog 1 Src = 1

MODZ    _CLR   WZ     'set Z = 0

MODC    _CLR   WC     'set C = 1

IF_C   MOV     Carry,#1      'true  C = 1

IF_NC  MOV     Carry,#0      'false C = 0

IF_Z   MOV     Zero,#1       'true  Z = 1

IF_NZ  MOV     Zero,#0       'false Z = 0

debug("before TESTBN  ",sbin(Dest),sdec(Src),ubin(Zero),ubin(Carry))

TESTBN   Dest,Src  WC

IF_C   MOV     Carry,#1      'true  C = 1

IF_NC  MOV     Carry,#0      'false C = 0

IF_Z   MOV     Zero,#1       'true  Z = 1

IF_NZ  MOV     Zero,#0       'false Z = 0

debug("after TESTBN   " ,sbin(Dest),sdec(Src),ubin(Zero),ubin(Carry))

debug("--------------------------------------")

MOV     Dest,#%1111_1110    'load cog 1 Pro = 1234

MOV     Src,#0      'load cog 1 Src = 1234

MODZ    _CLR   WZ       'set Z = 0

MODC    _CLR   WC       'set C = 0

IF_C   MOV     Carry,#1        'true  C = 1

IF_NC  MOV     Carry,#0        'false C = 0

IF_Z   MOV     Zero,#1         'true  Z = 1

IF_NZ  MOV     Zero,#0         'false Z = 0

debug("--------------------------------------")

debug("before TESTBN ",sbin(Dest),sdec(Src),ubin(Zero),ubin(Carry))

TESTBN   Dest,Src  WZ

IF_C   MOV     Carry,#1         'true  C = 1

IF_NC  MOV     Carry,#0         'false C = 0

IF_Z   MOV     Zero,#1          'true  Z = 1

IF_NZ  MOV     Zero,#0          'false Z = 0

debug("after TESTBN  ",sbin(Dest),sdec(Src),ubin(Zero),ubin(Carry))

debug("--------------------------------------")

MOV     Dest,#%1111_1101         'load cog 1 Pro = $FFFF_FFFF

MOV     Src,#1       'load cog 1 Src = 1

MODZ    _CLR   WZ        'set Z = 0

MODC    _CLR   WC        'set C = 1

IF_C   MOV     Carry,#1         'true  C = 1

IF_NC  MOV     Carry,#0         'false C = 0

IF_Z   MOV     Zero,#1          'true  Z = 1

IF_NZ  MOV     Zero,#0          'false Z = 0

debug("before TESTBN ",sbin(Dest),sdec(Src),ubin(Zero),ubin(Carry))

TESTBN   Dest,Src  WZ

IF_C   MOV     Carry,#1         'true  C = 1

IF_NC  MOV     Carry,#0         'false C = 0

IF_Z   MOV     Zero,#1          'true  Z = 1

IF_NZ  MOV     Zero,#0          'false Z = 0

debug("after TESTBN  ",sbin(Dest),sdec(Src),ubin(Zero),ubin(Carry))

debug("--------------------------------------")

MOV     Dest,#%1111_1011      'load cog 1 Pro = 1

MOV     Src,#2       'load cog 1 Src = 1

MODZ    _CLR   WZ     'set Z = 0

MODC    _CLR   WC     'set C = 1

IF_C   MOV     Carry,#1      'true  C = 1

IF_NC  MOV     Carry,#0      'false C = 0

IF_Z   MOV     Zero,#1       'true  Z = 1

IF_NZ  MOV     Zero,#0       'false Z = 0

debug("before TESTBN  ",sbin(Dest),sdec(Src),ubin(Zero),ubin(Carry))

TESTBN  Dest,Src  WZ

IF_C   MOV     Carry,#1      'true  C = 1

IF_NC  MOV     Carry,#0      'false C = 0

IF_Z   MOV     Zero,#1       'true  Z = 1

IF_NZ  MOV     Zero,#0       'false Z = 0

debug("after TESTBN   " ,sbin(Dest),sdec(Src),ubin(Zero),ubin(Carry))

debug("--------------------------------------")

_Loop1                NOP

JMP     #_Loop1          'remember # imediate

cog1Wait              long      900_000_000    'number of clock cycles to wait

cog1Cnt               long      0

valDest               long      1234         'cog 1 Dest

valSrc                long      1234         'cog 1 Src

valNeg2               long      -2

Carry                 long      0

Zero                  long      0

Dest                  long      0

Src                   long      0

MAX                   long      $FFFF_FFFF

MIN                   long      0

### 17.11.3_Example_WRD_TESTB_ANDC/ANDZ_036

TESTB D,{#}S ANDC/ANDZ

Test bit S[4:0] of D, AND into C/Z. C/Z = C/Z AND D[S[4:0]].

{{17.11.3_Example_WRD_TESTB_ANDC_ANDZ_036}}

''TESTB D,{#}S ANDC/ANDZ

''Test bit S[4:0] of D, AND into C/Z. C/Z = C/Z AND D[S[4:0]].

CON

_clkfreq = 200_000_000  ''Debug must be enabled clock must be greater than 10MHZ for Debug

P0 =0 , P1 = 1  , P2 = 2

VAR

Byte cogRunning 'cog ID started is returned or -1 if not started

PUB main()

debug("--------------------------------------")

debug("TESTB D,{#}S ANDC/ANDZ")

debug("Test bit S[4:0] of D, AND into C/Z. C/Z = C/Z AND D[S[4:0]].")

debug("--------------------------------------")

cogRunning := COGINIT(COGEXEC_NEW,@S0_TESTBN_ANDC_ANDZ,0)

debug(udec(cogRunning))

repeat                                    'keep cog 0 running

DAT

ORG   0

S0_TESTB_ANDC_ANDZ             DRVH    #P0            'P0 on program running

MOV     Dest,#%1       'load cog 1 Pro = 1234

MOV     Src,#0          'load cog 1 Src = 1234

MODZ    _CLR   WZ       'set Z = 0

MODC    _CLR   WC       'set C = 0

IF_C   MOV     Carry,#1        'true  C = 1

IF_NC  MOV     Carry,#0        'false C = 0

IF_Z   MOV     Zero,#1         'true  Z = 1

IF_NZ  MOV     Zero,#0         'false Z = 0

debug("--------------------------------------")

debug("before TESTB_ANDC/ANDZ ",sbin(Dest),sdec(Src),ubin(Zero),ubin(Carry))

TESTB   Dest,Src  ANDC

IF_C   MOV     Carry,#1         'true  C = 1

IF_NC  MOV     Carry,#0         'false C = 0

IF_Z   MOV     Zero,#1          'true  Z = 1

IF_NZ  MOV     Zero,#0          'false Z = 0

debug("after TESTB_ANDC/ANDZ  ",sbin(Dest),sdec(Src),ubin(Zero),ubin(Carry))

debug("--------------------------------------")

MOV     Dest,#%1         'load cog 1 Pro = $FFFF_FFFF

MOV     Src,#0           'load cog 1 Src = 1

MODZ    _CLR   WZ        'set Z = 0

MODC    _SET   WC        'set C = 1

IF_C   MOV     Carry,#1         'true  C = 1

IF_NC  MOV     Carry,#0         'false C = 0

IF_Z   MOV     Zero,#1          'true  Z = 1

IF_NZ  MOV     Zero,#0          'false Z = 0

debug("before TESTBN_ANDC/ANDZ ",sbin(Dest),sdec(Src),ubin(Zero),ubin(Carry))

TESTB   Dest,Src  ANDC

IF_C   MOV     Carry,#1         'true  C = 1

IF_NC  MOV     Carry,#0         'false C = 0

IF_Z   MOV     Zero,#1          'true  Z = 1

IF_NZ  MOV     Zero,#0          'false Z = 0

debug("after TESTB_ANDC/ANDZ  ",sbin(Dest),sdec(Src),ubin(Zero),ubin(Carry))

debug("--------------------------------------")

MOV     Dest,#%1      'load cog 1 Pro = 1

MOV     Src,#0       'load cog 1 Src = 1

MODZ    _CLR   WZ     'set Z = 0

MODC    _CLR   WC     'set C = 1

IF_C   MOV     Carry,#1      'true  C = 1

IF_NC  MOV     Carry,#0      'false C = 0

IF_Z   MOV     Zero,#1       'true  Z = 1

IF_NZ  MOV     Zero,#0       'false Z = 0

debug("before TESTB_ANDC/ANDZ  ",sbin(Dest),sdec(Src),ubin(Zero),ubin(Carry))

TESTB   Dest,Src  ANDZ

IF_C   MOV     Carry,#1      'true  C = 1

IF_NC  MOV     Carry,#0      'false C = 0

IF_Z   MOV     Zero,#1       'true  Z = 1

IF_NZ  MOV     Zero,#0       'false Z = 0

debug("after TESTB_ANDC/ANDZ   " ,sbin(Dest),sdec(Src),ubin(Zero),ubin(Carry))

debug("--------------------------------------")

MOV     Dest,#%1      'load cog 1 Pro = 1

MOV     Src,#0       'load cog 1 Src = 1

MODZ    _SET   WZ     'set Z = 0

MODC    _CLR   WC     'set C = 1

IF_C   MOV     Carry,#1      'true  C = 1

IF_NC  MOV     Carry,#0      'false C = 0

IF_Z   MOV     Zero,#1       'true  Z = 1

IF_NZ  MOV     Zero,#0       'false Z = 0

debug("before TESTB_ANDC/ANDZ  ",sbin(Dest),sdec(Src),ubin(Zero),ubin(Carry))

TESTB   Dest,Src  ANDZ

IF_C   MOV     Carry,#1      'true  C = 1

IF_NC  MOV     Carry,#0      'false C = 0

IF_Z   MOV     Zero,#1       'true  Z = 1

IF_NZ  MOV     Zero,#0       'false Z = 0

debug("after TESTB_ANDC/ANDZ   " ,sbin(Dest),sdec(Src),ubin(Zero),ubin(Carry))

debug("--------------------------------------")

_Loop1                NOP

JMP     #_Loop1          'remember # imediate

cog1Wait              long      900_000_000    'number of clock cycles to wait

cog1Cnt               long      0

valDest             long      1234         'cog 1 Dest

valSrc                long      1234         'cog 1 Src

valNeg2            long      -2

Carry                 long      0

Zero                   long      0

Dest                   long      0

Src                      long      0

MAX                   long      $FFFF_FFFF

MIN                    long      0

### 17.11.4_Example_WRD_TESTBN_ANDC/ANDZ_037

TESTBN  D,{#}S     ANDC/ANDZ

Test bit S[4:0] of !D, AND into C/Z. C/Z = C/Z AND !D[S[4:0]].

{{17.11.4_Example_WRD_TESTBN_ANDC/ANDZ_037}}

''TESTBN  D,{#}S     ANDC/ANDZ

''Test bit S[4:0] of !D, AND into C/Z. C/Z = C/Z AND !D[S[4:0]].

CON

_clkfreq = 200_000_000  ''Debug must be enabled clock must be greater than 10MHZ for Debug

P0 =0 , P1 = 1  , P2 = 2

VAR

Byte cogRunning 'cog ID started is returned or -1 if not started

PUB main()

debug("--------------------------------------")

debug("TESTBN  D,{#}S     ANDC/ANDZ ")

debug("Test bit S[4:0] of !D, AND into C/Z. C/Z = C/Z AND !D[S[4:0]].")

debug("--------------------------------------")

cogRunning := COGINIT(COGEXEC_NEW,@S0_TESTBN,0)

debug(udec(cogRunning))

repeat                                    'keep cog 0 running

DAT

ORG   0

S0_TESTBN_ANDC_ANDZ             DRVH    #P0            'P0 on program running

MOV     Dest,#%0       'load cog 1 Pro = 1234

MOV     Src,#0          'load cog 1 Src = 1234

MODZ    _CLR   WZ       'set Z = 0

MODC    _CLR   WC       'set C = 0

IF_C   MOV     Carry,#1        'true  C = 1

IF_NC  MOV     Carry,#0        'false C = 0

IF_Z   MOV     Zero,#1         'true  Z = 1

IF_NZ  MOV     Zero,#0         'false Z = 0

debug("--------------------------------------")

debug("before TESTBN_ANDC ",sbin(Dest),sdec(Src),ubin(Zero),ubin(Carry))

TESTBN  Dest,Src  ANDC

IF_C   MOV     Carry,#1         'true  C = 1

IF_NC  MOV     Carry,#0         'false C = 0

IF_Z   MOV     Zero,#1          'true  Z = 1

IF_NZ  MOV     Zero,#0          'false Z = 0

debug("after TESTBN_ANDC  ",sbin(Dest),sdec(Src),ubin(Zero),ubin(Carry))

debug("--------------------------------------")

MOV     Dest,#%0         'load cog 1 Pro = $FFFF_FFFF

MOV     Src,#0           'load cog 1 Src = 1

MODZ    _CLR   WZ        'set Z = 0

MODC    _SET   WC        'set C = 1

IF_C   MOV     Carry,#1         'true  C = 1

IF_NC  MOV     Carry,#0         'false C = 0

IF_Z   MOV     Zero,#1          'true  Z = 1

IF_NZ  MOV     Zero,#0          'false Z = 0

debug("before TESTBN_ANDC ",sbin(Dest),sdec(Src),ubin(Zero),ubin(Carry))

TESTBN   Dest,Src  ANDC

IF_C   MOV     Carry,#1         'true  C = 1

IF_NC  MOV     Carry,#0         'false C = 0

IF_Z   MOV     Zero,#1          'true  Z = 1

IF_NZ  MOV     Zero,#0          'false Z = 0

debug("after TESTBN_ANDC  ",sbin(Dest),sdec(Src),ubin(Zero),ubin(Carry))

debug("--------------------------------------")

MOV     Dest,#%0      'load cog 1 Pro = 1

MOV     Src,#0       'load cog 1 Src = 1

MODZ    _CLR   WZ     'set Z = 0

MODC    _CLR   WC     'set C = 1

IF_C   MOV     Carry,#1      'true  C = 1

IF_NC  MOV     Carry,#0      'false C = 0

IF_Z   MOV     Zero,#1       'true  Z = 1

IF_NZ  MOV     Zero,#0       'false Z = 0

debug("before TESTBN_ANDZ  ",sbin(Dest),sdec(Src),ubin(Zero),ubin(Carry))

TESTBN   Dest,Src  ANDZ

IF_C   MOV     Carry,#1      'true  C = 1

IF_NC  MOV     Carry,#0      'false C = 0

IF_Z   MOV     Zero,#1       'true  Z = 1

IF_NZ  MOV     Zero,#0       'false Z = 0

debug("after TESTBN_ANDZ   " ,sbin(Dest),sdec(Src),ubin(Zero),ubin(Carry))

debug("--------------------------------------")

MOV     Dest,#%0      'load cog 1 Pro = 1

MOV     Src,#0       'load cog 1 Src = 1

MODZ    _SET   WZ     'set Z = 0

MODC    _CLR   WC     'set C = 1

IF_C   MOV     Carry,#1      'true  C = 1

IF_NC  MOV     Carry,#0      'false C = 0

IF_Z   MOV     Zero,#1       'true  Z = 1

IF_NZ  MOV     Zero,#0       'false Z = 0

debug("before TESTBN_ANDZ  ",sbin(Dest),sdec(Src),ubin(Zero),ubin(Carry))

TESTBN  Dest,Src  ANDZ

IF_C   MOV     Carry,#1      'true  C = 1

IF_NC  MOV     Carry,#0      'false C = 0

IF_Z   MOV     Zero,#1       'true  Z = 1

IF_NZ  MOV     Zero,#0       'false Z = 0

debug("after TESTBN_ANDZ   " ,sbin(Dest),sdec(Src),ubin(Zero),ubin(Carry))

debug("--------------------------------------")

_Loop1                NOP

JMP     #_Loop1          'remember # imediate

cog1Wait              long      900_000_000    'number of clock cycles to wait

cog1Cnt               long      0

valDest               long      1234         'cog 1 Dest

valSrc                long      1234         'cog 1 Src

valNeg2               long      -2

Carry                 long      0

Zero                  long      0

Dest                  long      0

Src                   long      0

MAX                   long      $FFFF_FFFF

MIN                   long      0

### 17.11.5_Example_WRD_TESTB_ORC_ORZ_038

TESTB   D,{#}S       ORC/ORZ

Test bit S[4:0] of  D, OR  into C/Z. C/Z = C/Z OR   D[S[4:0]].

{{17.11.5_Example_WRD_TESTB_ORC_ORZ_038}}

''TESTB   D,{#}S       ORC/ORZ

''Test bit S[4:0] of  D, OR  into C/Z. C/Z = C/Z OR   D[S[4:0]].

CON

_clkfreq = 200_000_000  ''Debug must be enabled clock must be greater than 10MHZ for Debug

P0 =0 , P1 = 1  , P2 = 2

VAR

Byte cogRunning 'cog ID started is returned or -1 if not started

PUB main()

debug("--------------------------------------")

debug("TESTB   D,{#}S       ORC/ORZ ")

debug("Test bit S[4:0] of  D, OR  into C/Z. C/Z = C/Z OR   D[S[4:0]].")

debug("--------------------------------------")

cogRunning := COGINIT(COGEXEC_NEW,@S0_TESTB_ORC_ORZ,0)

debug(udec(cogRunning))

repeat                                    'keep cog 0 running

DAT

ORG   0

S0_TESTB_ORC_ORZ      DRVH    #P0            'P0 on program running

MOV     Dest,#%1       'load cog 1 Pro = 1234

MOV     Src,#0          'load cog 1 Src = 1234

MODZ    _CLR   WZ       'set Z = 0

MODC    _CLR   WC       'set C = 0

IF_C   MOV     Carry,#1        'true  C = 1

IF_NC  MOV     Carry,#0        'false C = 0

IF_Z   MOV     Zero,#1         'true  Z = 1

IF_NZ  MOV     Zero,#0         'false Z = 0

debug("--------------------------------------")

debug("before TESTB_ORC ",sbin(Dest),sdec(Src),ubin(Zero),ubin(Carry))

TESTB   Dest,Src  ORC

IF_C   MOV     Carry,#1         'true  C = 1

IF_NC  MOV     Carry,#0         'false C = 0

IF_Z   MOV     Zero,#1          'true  Z = 1

IF_NZ  MOV     Zero,#0          'false Z = 0

debug("after TESTB_ORC  ",sbin(Dest),sdec(Src),ubin(Zero),ubin(Carry))

debug("--------------------------------------")

MOV     Dest,#%0         'load cog 1 Pro = $FFFF_FFFF

MOV     Src,#0           'load cog 1 Src = 1

MODZ    _CLR   WZ        'set Z = 0

MODC    _SET   WC        'set C = 1

IF_C   MOV     Carry,#1         'true  C = 1

IF_NC  MOV     Carry,#0         'false C = 0

IF_Z   MOV     Zero,#1          'true  Z = 1

IF_NZ  MOV     Zero,#0          'false Z = 0

debug("before TESTB_ORC ",sbin(Dest),sdec(Src),ubin(Zero),ubin(Carry))

TESTB   Dest,Src  ORC

IF_C   MOV     Carry,#1         'true  C = 1

IF_NC  MOV     Carry,#0         'false C = 0

IF_Z   MOV     Zero,#1          'true  Z = 1

IF_NZ  MOV     Zero,#0          'false Z = 0

debug("after TESTB_ORC  ",sbin(Dest),sdec(Src),ubin(Zero),ubin(Carry))

debug("--------------------------------------")

MOV     Dest,#%1      'load cog 1 Pro = 1

MOV     Src,#0       'load cog 1 Src = 1

MODZ    _CLR   WZ     'set Z = 0

MODC    _CLR   WC     'set C = 1

IF_C   MOV     Carry,#1      'true  C = 1

IF_NC  MOV     Carry,#0      'false C = 0

IF_Z   MOV     Zero,#1       'true  Z = 1

IF_NZ  MOV     Zero,#0       'false Z = 0

debug("before TESTBN_ORZ  ",sbin(Dest),sdec(Src),ubin(Zero),ubin(Carry))

TESTB   Dest,Src  ORZ

IF_C   MOV     Carry,#1      'true  C = 1

IF_NC  MOV     Carry,#0      'false C = 0

IF_Z   MOV     Zero,#1       'true  Z = 1

IF_NZ  MOV     Zero,#0       'false Z = 0

debug("after TESTB_ORZ   " ,sbin(Dest),sdec(Src),ubin(Zero),ubin(Carry))

debug("--------------------------------------")

MOV     Dest,#%0      'load cog 1 Pro = 1

MOV     Src,#0       'load cog 1 Src = 1

MODZ    _SET   WZ     'set Z = 0

MODC    _CLR   WC     'set C = 1

IF_C   MOV     Carry,#1      'true  C = 1

IF_NC  MOV     Carry,#0      'false C = 0

IF_Z   MOV     Zero,#1       'true  Z = 1

IF_NZ  MOV     Zero,#0       'false Z = 0

debug("before TESTB_ORZ  ",sbin(Dest),sdec(Src),ubin(Zero),ubin(Carry))

TESTB  Dest,Src  ORZ

IF_C   MOV     Carry,#1      'true  C = 1

IF_NC  MOV     Carry,#0      'false C = 0

IF_Z   MOV     Zero,#1       'true  Z = 1

IF_NZ  MOV     Zero,#0       'false Z = 0

debug("after TESTB_ORZ   " ,sbin(Dest),sdec(Src),ubin(Zero),ubin(Carry))

debug("--------------------------------------")

_Loop1                NOP

JMP     #_Loop1          'remember # imediate

cog1Wait              long      900_000_000    'number of clock cycles to wait

cog1Cnt               long      0

valDest               long      1234         'cog 1 Dest

valSrc                long      1234         'cog 1 Src

valNeg2               long      -2

Carry                 long      0

Zero                  long      0

Dest                  long      0

Src                   long      0

MAX                   long      $FFFF_FFFF

MIN                   long      0

### 17.11.6_Example_WRD_TESTBN_ORC_ORZ_039

TESTBN  D,{#}S       ORC/ORZ

Test bit S[4:0] of !D, OR  into C/Z. C/Z = C/Z OR  !D[S[4:0]].

{{17.11.6_Example_WRD_TESTBN_ORC_ORZ_039}}

''TESTBN  D,{#}S       ORC/ORZ

''Test bit S[4:0] of !D, OR  into C/Z. C/Z = C/Z OR  !D[S[4:0]].

CON

_clkfreq = 200_000_000  ''Debug must be enabled clock must be greater than 10MHZ for Debug

P0 =0 , P1 = 1  , P2 = 2

VAR

Byte cogRunning 'cog ID started is returned or -1 if not started

PUB main()

debug("--------------------------------------")

debug("TESTBN   D,{#}S       ORC/ORZ ")

debug("Test bit S[4:0] of !D, OR  into C/Z. C/Z = C/Z OR D[S[4:0]].")

debug("--------------------------------------")

cogRunning := COGINIT(COGEXEC_NEW,@S0_TESTBN_ORC_ORZ,0)

debug(udec(cogRunning))

repeat                                    'keep cog 0 running

DAT

ORG   0

S0_TESTBN_ORC_ORZ     DRVH    #P0            'P0 on program running

MOV     Dest,#%0       'load cog 1 Pro = 1234

MOV     Src,#0          'load cog 1 Src = 1234

MODZ    _CLR   WZ       'set Z = 0

MODC    _CLR   WC       'set C = 0

IF_C   MOV     Carry,#1        'true  C = 1

IF_NC  MOV     Carry,#0        'false C = 0

IF_Z   MOV     Zero,#1         'true  Z = 1

IF_NZ  MOV     Zero,#0         'false Z = 0

debug("--------------------------------------")

debug("before TESTBN_ORC ",sbin(Dest),sdec(Src),ubin(Zero),ubin(Carry))

TESTBN  Dest,Src  ORC

IF_C   MOV     Carry,#1         'true  C = 1

IF_NC  MOV     Carry,#0         'false C = 0

IF_Z   MOV     Zero,#1          'true  Z = 1

IF_NZ  MOV     Zero,#0          'false Z = 0

debug("after TESTBN_ORC  ",sbin(Dest),sdec(Src),ubin(Zero),ubin(Carry))

debug("--------------------------------------")

MOV     Dest,#%1         'load cog 1 Pro = $FFFF_FFFF

MOV     Src,#0           'load cog 1 Src = 1

MODZ    _CLR   WZ        'set Z = 0

MODC    _SET   WC        'set C = 1

IF_C   MOV     Carry,#1         'true  C = 1

IF_NC  MOV     Carry,#0         'false C = 0

IF_Z   MOV     Zero,#1          'true  Z = 1

IF_NZ  MOV     Zero,#0          'false Z = 0

debug("before TESTBN_ORC ",sbin(Dest),sdec(Src),ubin(Zero),ubin(Carry))

TESTBN  Dest,Src  ORC

IF_C   MOV     Carry,#1         'true  C = 1

IF_NC  MOV     Carry,#0         'false C = 0

IF_Z   MOV     Zero,#1          'true  Z = 1

IF_NZ  MOV     Zero,#0          'false Z = 0

debug("after TESTBN_ORC  ",sbin(Dest),sdec(Src),ubin(Zero),ubin(Carry))

debug("--------------------------------------")

MOV     Dest,#%0     'load cog 1 Pro = 1

MOV     Src,#0       'load cog 1 Src = 1

MODZ    _CLR   WZ     'set Z = 0

MODC    _CLR   WC     'set C = 1

IF_C   MOV     Carry,#1      'true  C = 1

IF_NC  MOV     Carry,#0      'false C = 0

IF_Z   MOV     Zero,#1       'true  Z = 1

IF_NZ  MOV     Zero,#0       'false Z = 0

debug("before TESTBN_ORZ  ",sbin(Dest),sdec(Src),ubin(Zero),ubin(Carry))

TESTBN  Dest,Src  ORZ

IF_C   MOV     Carry,#1      'true  C = 1

IF_NC  MOV     Carry,#0      'false C = 0

IF_Z   MOV     Zero,#1       'true  Z = 1

IF_NZ  MOV     Zero,#0       'false Z = 0

debug("after TESTBN_ORZ   " ,sbin(Dest),sdec(Src),ubin(Zero),ubin(Carry))

debug("--------------------------------------")

MOV     Dest,#%1      'load cog 1 Pro = 1

MOV     Src,#0       'load cog 1 Src = 1

MODZ    _SET   WZ     'set Z = 0

MODC    _CLR   WC     'set C = 1

IF_C   MOV     Carry,#1      'true  C = 1

IF_NC  MOV     Carry,#0      'false C = 0

IF_Z   MOV     Zero,#1       'true  Z = 1

IF_NZ  MOV     Zero,#0       'false Z = 0

debug("before TESTBN_ORZ  ",sbin(Dest),sdec(Src),ubin(Zero),ubin(Carry))

TESTBN Dest,Src  ORZ

IF_C   MOV     Carry,#1      'true  C = 1

IF_NC  MOV     Carry,#0      'false C = 0

IF_Z   MOV     Zero,#1       'true  Z = 1

IF_NZ  MOV     Zero,#0       'false Z = 0

debug("after TESTBN_ORZ   " ,sbin(Dest),sdec(Src),ubin(Zero),ubin(Carry))

debug("--------------------------------------")

_Loop1                NOP

JMP     #_Loop1          'remember # imediate

cog1Wait              long      900_000_000    'number of clock cycles to wait

cog1Cnt               long      0

valDest               long      1234         'cog 1 Dest

valSrc                long      1234         'cog 1 Src

valNeg2               long      -2

Carry                 long      0

Zero                  long      0

Dest                  long      0

Src                   long      0

MAX                   long      $FFFF_FFFF

MIN                   long      0

### 17.11.7_Example_WRD_TESTB_XORC_XORZ_040

TESTB D,{#}S XORC/XORZ

Test bit S[4:0] of D, XOR into C/Z. C/Z = C/Z XOR D[S[4:0]].

{{17.11.7_Example_WRD_TESTB_XORC_XORZ_040}}

''TESTB D,{#}S XORC/XORZ

''Test bit S[4:0] of D, XOR into C/Z. C/Z = C/Z XOR D[S[4:0]].

CON

_clkfreq = 200_000_000  ''Debug must be enabled clock must be greater than 10MHZ for Debug

P0 =0 , P1 = 1  , P2 = 2

VAR

Byte cogRunning 'cog ID started is returned or -1 if not started

PUB main()

debug("--------------------------------------")

debug("TESTB   D,{#}S      XORC/XORZ ")

debug("Test bit S[4:0] of D, XOR into C/Z. C/Z = C/Z XOR D[S[4:0]].")

debug("--------------------------------------")

cogRunning := COGINIT(COGEXEC_NEW,@S0_TESTB_XORC_XORZ,0)

debug(udec(cogRunning))

repeat                                    'keep cog 0 running

DAT

ORG   0

S0_TESTB_XORC_XORZ    DRVH    #P0            'P0 on program running

MOV     Dest,#%1       'load cog 1 Pro = 1234

MOV     Src,#0          'load cog 1 Src = 1234

MODZ    _CLR   WZ       'set Z = 0

MODC    _CLR   WC       'set C = 0

IF_C   MOV     Carry,#1        'true  C = 1

IF_NC  MOV     Carry,#0        'false C = 0

IF_Z   MOV     Zero,#1         'true  Z = 1

IF_NZ  MOV     Zero,#0         'false Z = 0

debug("--------------------------------------")

debug("before TESTB_XORC ",sbin(Dest),sdec(Src),ubin(Zero),ubin(Carry))

TESTB  Dest,Src  XORC

IF_C   MOV     Carry,#1         'true  C = 1

IF_NC  MOV     Carry,#0         'false C = 0

IF_Z   MOV     Zero,#1          'true  Z = 1

IF_NZ  MOV     Zero,#0          'false Z = 0

debug("after TESTB_XORC  ",sbin(Dest),sdec(Src),ubin(Zero),ubin(Carry))

debug("--------------------------------------")

MOV     Dest,#%1         'load cog 1 Pro = $FFFF_FFFF

MOV     Src,#0           'load cog 1 Src = 1

MODZ    _CLR   WZ        'set Z = 0

MODC    _SET   WC        'set C = 1

IF_C   MOV     Carry,#1         'true  C = 1

IF_NC  MOV     Carry,#0         'false C = 0

IF_Z   MOV     Zero,#1          'true  Z = 1

IF_NZ  MOV     Zero,#0          'false Z = 0

debug("before TESTBN_XORC ",sbin(Dest),sdec(Src),ubin(Zero),ubin(Carry))

TESTB  Dest,Src  XORC

IF_C   MOV     Carry,#1         'true  C = 1

IF_NC  MOV     Carry,#0         'false C = 0

IF_Z   MOV     Zero,#1          'true  Z = 1

IF_NZ  MOV     Zero,#0          'false Z = 0

debug("after TESTB_XORC  ",sbin(Dest),sdec(Src),ubin(Zero),ubin(Carry))

debug("--------------------------------------")

MOV     Dest,#%0     'load cog 1 Pro = 1

MOV     Src,#0       'load cog 1 Src = 1

MODZ    _CLR   WZ     'set Z = 0

MODC    _CLR   WC     'set C = 1

IF_C   MOV     Carry,#1      'true  C = 1

IF_NC  MOV     Carry,#0      'false C = 0

IF_Z   MOV     Zero,#1       'true  Z = 1

IF_NZ  MOV     Zero,#0       'false Z = 0

debug("before TESTB_XORZ  ",sbin(Dest),sdec(Src),ubin(Zero),ubin(Carry))

TESTBN  Dest,Src  ORZ

IF_C   MOV     Carry,#1      'true  C = 1

IF_NC  MOV     Carry,#0      'false C = 0

IF_Z   MOV     Zero,#1       'true  Z = 1

IF_NZ  MOV     Zero,#0       'false Z = 0

debug("after TESTB_XORZ   " ,sbin(Dest),sdec(Src),ubin(Zero),ubin(Carry))

debug("--------------------------------------")

MOV     Dest,#%1      'load cog 1 Pro = 1

MOV     Src,#0       'load cog 1 Src = 1

MODZ    _SET   WZ     'set Z = 0

MODC    _CLR   WC     'set C = 1

IF_C   MOV     Carry,#1      'true  C = 1

IF_NC  MOV     Carry,#0      'false C = 0

IF_Z   MOV     Zero,#1       'true  Z = 1

IF_NZ  MOV     Zero,#0       'false Z = 0

debug("before TESTB_XORZ  ",sbin(Dest),sdec(Src),ubin(Zero),ubin(Carry))

TESTB Dest,Src  XORZ

IF_C   MOV     Carry,#1      'true  C = 1

IF_NC  MOV     Carry,#0      'false C = 0

IF_Z   MOV     Zero,#1       'true  Z = 1

IF_NZ  MOV     Zero,#0       'false Z = 0

debug("after TESTB_XORZ   " ,sbin(Dest),sdec(Src),ubin(Zero),ubin(Carry))

debug("--------------------------------------")

_Loop1                NOP

JMP     #_Loop1          'remember # imediate

cog1Wait              long      900_000_000    'number of clock cycles to wait

cog1Cnt               long      0

valDest               long      1234         'cog 1 Dest

valSrc                long      1234         'cog 1 Src

valNeg2               long      -2

Carry                 long      0

Zero                  long      0

Dest                  long      0

Src                   long      0

MAX                   long      $FFFF_FFFF

MIN                   long      0

### 17.11.8_Example_WRD_TESTBN_XORC_XORZ_041

TESTBN D,{#}S XORC/XORZ

Test bit S[4:0] of !D, XOR into C/Z. C/Z = C/Z XOR !D[S[4:0]].

{{17.11.8_Example_WRD_TESTBN_XORC_XORZ_041}}

''TESTBN D,{#}S XORC/XORZ

''Test bit S[4:0] of !D, XOR into C/Z. C/Z = C/Z XOR !D[S[4:0]].

CON

_clkfreq = 200_000_000  ''Debug must be enabled clock must be greater than 10MHZ for Debug

P0 =0 , P1 = 1  , P2 = 2

VAR

Byte cogRunning 'cog ID started is returned or -1 if not started

PUB main()

debug("--------------------------------------")

debug("TESTBN   D,{#}S      XORC/XORZ ")

debug("Test bit S[4:0] of !D, XOR into C/Z. C/Z = C/Z XOR !D[S[4:0]].")

debug("--------------------------------------")

cogRunning := COGINIT(COGEXEC_NEW,@S0_TESTBN_XORC_XORZ,0)

debug(udec(cogRunning))

repeat                                    'keep cog 0 running

DAT

ORG   0

S0_TESTBN_XORC_XORZ   DRVH    #P0            'P0 on program running

MOV     Dest,#%0       'load cog 1 Pro = 1234

MOV     Src,#0          'load cog 1 Src = 1234

MODZ    _CLR   WZ       'set Z = 0

MODC    _CLR   WC       'set C = 0

IF_C   MOV     Carry,#1        'true  C = 1

IF_NC  MOV     Carry,#0        'false C = 0

IF_Z   MOV     Zero,#1         'true  Z = 1

IF_NZ  MOV     Zero,#0         'false Z = 0

debug("--------------------------------------")

debug("before TESTBN_XORC ",sbin(Dest),sdec(Src),ubin(Zero),ubin(Carry))

TESTBN  Dest,Src  XORC

IF_C   MOV     Carry,#1         'true  C = 1

IF_NC  MOV     Carry,#0         'false C = 0

IF_Z   MOV     Zero,#1          'true  Z = 1

IF_NZ  MOV     Zero,#0          'false Z = 0

debug("after TESTBN_XORC  ",sbin(Dest),sdec(Src),ubin(Zero),ubin(Carry))

debug("--------------------------------------")

MOV     Dest,#%0         'load cog 1 Pro = $FFFF_FFFF

MOV     Src,#0           'load cog 1 Src = 1

MODZ    _CLR   WZ        'set Z = 0

MODC    _SET   WC        'set C = 1

IF_C   MOV     Carry,#1         'true  C = 1

IF_NC  MOV     Carry,#0         'false C = 0

IF_Z   MOV     Zero,#1          'true  Z = 1

IF_NZ  MOV     Zero,#0          'false Z = 0

debug("before TESTBN_XORC ",sbin(Dest),sdec(Src),ubin(Zero),ubin(Carry))

TESTBN  Dest,Src  XORC

IF_C   MOV     Carry,#1         'true  C = 1

IF_NC  MOV     Carry,#0         'false C = 0

IF_Z   MOV     Zero,#1          'true  Z = 1

IF_NZ  MOV     Zero,#0          'false Z = 0

debug("after TESTBN_XORC  ",sbin(Dest),sdec(Src),ubin(Zero),ubin(Carry))

debug("--------------------------------------")

MOV     Dest,#%1     'load cog 1 Pro = 1

MOV     Src,#0       'load cog 1 Src = 1

MODZ    _CLR   WZ     'set Z = 0

MODC    _CLR   WC     'set C = 1

IF_C   MOV     Carry,#1      'true  C = 1

IF_NC  MOV     Carry,#0      'false C = 0

IF_Z   MOV     Zero,#1       'true  Z = 1

IF_NZ  MOV     Zero,#0       'false Z = 0

debug("before TESTBN_XORZ  ",sbin(Dest),sdec(Src),ubin(Zero),ubin(Carry))

TESTBN  Dest,Src  ORZ

IF_C   MOV     Carry,#1      'true  C = 1

IF_NC  MOV     Carry,#0      'false C = 0

IF_Z   MOV     Zero,#1       'true  Z = 1

IF_NZ  MOV     Zero,#0       'false Z = 0

debug("after TESTBN_XORZ   " ,sbin(Dest),sdec(Src),ubin(Zero),ubin(Carry))

debug("--------------------------------------")

MOV     Dest,#%0      'load cog 1 Pro = 1

MOV     Src,#0       'load cog 1 Src = 1

MODZ    _SET   WZ     'set Z = 0

MODC    _CLR   WC     'set C = 1

IF_C   MOV     Carry,#1      'true  C = 1

IF_NC  MOV     Carry,#0      'false C = 0

IF_Z   MOV     Zero,#1       'true  Z = 1

IF_NZ  MOV     Zero,#0       'false Z = 0

debug("before TESTBN_XORZ  ",sbin(Dest),sdec(Src),ubin(Zero),ubin(Carry))

TESTBN  Dest,Src  XORZ

IF_C   MOV     Carry,#1      'true  C = 1

IF_NC  MOV     Carry,#0      'false C = 0

IF_Z   MOV     Zero,#1       'true  Z = 1

IF_NZ  MOV     Zero,#0       'false Z = 0

debug("after TESTBN_XORZ   " ,sbin(Dest),sdec(Src),ubin(Zero),ubin(Carry))

debug("--------------------------------------")

_Loop1                NOP

JMP     #_Loop1          'remember # imediate

cog1Wait              long      900_000_000    'number of clock cycles to wait

cog1Cnt               long      0

valDest               long      1234         'cog 1 Dest

valSrc                long      1234         'cog 1 Src

valNeg2               long      -2

Carry                 long      0

Zero                  long      0

Dest                  long      0

Src                   long      0

MAX                   long      $FFFF_FFFF

MIN                   long      0

## 17.12) BIT Set Bits

| BITL D,{#}S {WCZ} | Bits D[S[9:5]+S[4:0]:S[4:0]] = 0. Other bits unaffected. Prior SETQ overrides S[9:5]. C,Z = original D[S[4:0]]. |
| --- | --- |
| BITH D,{#}S {WCZ} | Bits D[S[9:5]+S[4:0]:S[4:0]] = 1. Other bits unaffected. Prior SETQ overrides S[9:5]. C,Z = original D[S[4:0]]. |
| BITC D,{#}S {WCZ} | Bits D[S[9:5]+S[4:0]:S[4:0]] = C. Other bits unaffected. Prior SETQ overrides S[9:5]. C,Z = original D[S[4:0]]. |
| BITNC D,{#}S {WCZ} | Bits D[S[9:5]+S[4:0]:S[4:0]] = !C. Other bits unaffected. Prior SETQ overrides S[9:5]. C,Z = original D[S[4:0]]. |
| BITZ D,{#}S {WCZ} | Bits D[S[9:5]+S[4:0]:S[4:0]] = Z. Other bits unaffected. Prior SETQ overrides S[9:5]. C,Z = original D[S[4:0]]. |
| BITNZ D,{#}S {WCZ} | Bits D[S[9:5]+S[4:0]:S[4:0]] = !Z. Other bits unaffected. Prior SETQ overrides S[9:5]. C,Z = original D[S[4:0]]. |
| BITRND D,{#}S {WCZ} | Bits D[S[9:5]+S[4:0]:S[4:0]] = RNDs. Other bits unaffected. Prior SETQ overrides S[9:5]. C,Z = original D[S[4:0]]. |
| BITNOT D,{#}S {WCZ} | Toggle bits D[S[9:5]+S[4:0]:S[4:0]]. Other bits unaffected. Prior SETQ overrides S[9:5]. C,Z = original D[S[4:0]]. |

### 17.12.1_Example_WRD_BITL_042

BITL D,{#}S {WCZ}

Bits D[S[9:5]+S[4:0]:S[4:0]] = 0. Other bits unaffected. Prior SETQ overrides S[9:5].

C,Z = original D[S[4:0]].

This instruction can be used to set a bit to 0 or a group of bits to 0

D = 11111111_11111111_11111111_11111111

To set B0 =0   BitL D,#1

To set B31 =0 BITL D,#31

D[BH:BL] = D[S[9:5] + S[4:0] : S[4:0]] = 0

S=%00000000_00000000_000000_B9B8B7B6B5_B4B3B2B1B0

Where BH is high bit to be zero and BL the low bit to be zero of the group of zero bits

Example D[23:16] =0 Let D[31:0] =$FFFF_FFFFF
We want the following Pattern:
D[31:0] = 11111111_00000000_11111111_11111111 BH = 23 BL =16
then S[9:5] + S[4:0] = 23 S[9:5] = 23 - S[4:0] = 23-16 =7 Then S[31:10] = 0 S[9:5] =7 S[4:0] = 16 S= S S = %00000000_00000000_000000_00111_10000

BITL D,#240

{{17.12.1_Example_WRD_BITL_042}}

{{

BITL D,{#}S {WCZ}

D[BH:BL] = D[S[9:5]+S[4:0]:S[4:0]] = 0. Other bits unaffected.

Prior SETQ overrides S[9:5]. C,Z = original D[S[4:0]].

Example D[23:16] =0 Let D[31:0] =$FFFF_FFFFF

We want the following Pattern:

D[31:0] = 11111111_00000000_11111111_11111111 BH = 23 BL =16

then S[9:5] + S[4:0] = 23 S[9:5] = 23 - S[4:0] = 23-16 =7

Then S[31:10] = 0 S[9:5] =7 S[4:0] = 16 S=%00000000_00000000_000000_00111_10000 = 240

BITL D,#240

}}

CON

_clkfreq = 200_000_000  ''Debug must be enabled clock must be greater than 10MHZ for Debug

P0 =0 , P1 = 1  , P2 = 2

VAR

Byte cogRunning 'cog ID started is returned or -1 if not started

PUB main()

debug("--------------------------------------")

debug("BITL D,{#}S {WCZ}")

debug("D[BH:BL] = D[S[9:5]+S[4:0]:S[4:0]] = 0. Other bits unaffected.")

debug("Prior SETQ overrides S[9:5]. C,Z = original D[S[4:0]].")

debug("--------------------------------------")

cogRunning := COGINIT(COGEXEC_NEW,@S0_BITL,0)

debug(udec(cogRunning))

repeat                                    'keep cog 0 running

DAT

ORG   0

S0_BITL               DRVH    #P0            'P0 on program running

MOV     Dest,valDest   '$FFFF_FFFF

MOV     Src,#0

MODCZ   _CLR, _CLR  WCZ 'set Z = 0

IF_C   MOV     Carry,#1        'true  C = 1

IF_NC  MOV     Carry,#0        'false C = 0

IF_Z   MOV     Zero,#1         'true  Z = 1

IF_NZ  MOV     Zero,#0         'false Z = 0

debug("--------------------------------------")

debug("before BITL ",ubin(Dest),udec(Src),ubin(Zero),ubin(Carry))

BITL    Dest,Src  WCZ

IF_C   MOV     Carry,#1         'true  C = 1

IF_NC  MOV     Carry,#0         'false C = 0

IF_Z   MOV     Zero,#1          'true  Z = 1

IF_NZ  MOV     Zero,#0          'false Z = 0

debug("after BITL  ",ubin(Dest),udec(Src),ubin(Zero),ubin(Carry))

debug("--------------------------------------")

MOV     Dest,valDest   '$FFFF_FFFF

MOV     Src,#31

MODCZ   _CLR, _CLR  WCZ 'set Z = 0

IF_C   MOV     Carry,#1        'true  C = 1

IF_NC  MOV     Carry,#0        'false C = 0

IF_Z   MOV     Zero,#1         'true  Z = 1

IF_NZ  MOV     Zero,#0         'false Z = 0

debug("before BITL ",ubin(Dest),udec(Src),ubin(Zero),ubin(Carry))

BITL    Dest,Src  WCZ

IF_C   MOV     Carry,#1         'true  C = 1

IF_NC  MOV     Carry,#0         'false C = 0

IF_Z   MOV     Zero,#1          'true  Z = 1

IF_NZ  MOV     Zero,#0          'false Z = 0

debug("after BITL  ",ubin(Dest),udec(Src),ubin(Zero),ubin(Carry))

debug("--------------------------------------")

MOV     Dest,valDest   '$FFFF_FFFF

MOV     Src,#32

MODCZ   _CLR, _CLR  WCZ 'set Z = 0

IF_C   MOV     Carry,#1        'true  C = 1

IF_NC  MOV     Carry,#0        'false C = 0

IF_Z   MOV     Zero,#1         'true  Z = 1

IF_NZ  MOV     Zero,#0         'false Z = 0

debug("before BITL ",ubin(Dest),udec(Src),ubin(Zero),ubin(Carry))

BITL    Dest,Src  WCZ

IF_C   MOV     Carry,#1         'true  C = 1

IF_NC  MOV     Carry,#0         'false C = 0

IF_Z   MOV     Zero,#1          'true  Z = 1

IF_NZ  MOV     Zero,#0          'false Z = 0

debug("after BITL  ",ubin(Dest),udec(Src),ubin(Zero),ubin(Carry))

debug("--------------------------------------")

MOV     Dest,valDest   '$FFFF_FFFF

MOV     Src,#33

MODCZ   _CLR, _CLR  WCZ 'set Z = 0

IF_C   MOV     Carry,#1        'true  C = 1

IF_NC  MOV     Carry,#0        'false C = 0

IF_Z   MOV     Zero,#1         'true  Z = 1

IF_NZ  MOV     Zero,#0         'false Z = 0

debug("before BITL ",ubin(Dest),udec(Src),ubin(Zero),ubin(Carry))

BITL    Dest,Src  WCZ

IF_C   MOV     Carry,#1         'true  C = 1

IF_NC  MOV     Carry,#0         'false C = 0

IF_Z   MOV     Zero,#1          'true  Z = 1

IF_NZ  MOV     Zero,#0          'false Z = 0

debug("after BITL  ",ubin(Dest),udec(Src),ubin(Zero),ubin(Carry))

debug("--------------------------------------")

MOV     Dest,valDest   '$FFFF_FFFF

MOV     Src,#240 'S = %00000000_00000000_111_10000

MODCZ   _CLR, _CLR  WCZ 'set Z = 0

IF_C   MOV     Carry,#1        'true  C = 1

IF_NC  MOV     Carry,#0        'false C = 0

IF_Z   MOV     Zero,#1         'true  Z = 1

IF_NZ  MOV     Zero,#0         'false Z = 0

debug("before BITL ",ubin(Dest),udec(Src),ubin(Zero),ubin(Carry))

BITL    Dest,Src  WCZ

IF_C   MOV     Carry,#1         'true  C = 1

IF_NC  MOV     Carry,#0         'false C = 0

IF_Z   MOV     Zero,#1          'true  Z = 1

IF_NZ  MOV     Zero,#0          'false Z = 0

debug("after BITL  ",ubin(Dest),udec(Src),ubin(Zero),ubin(Carry))

debug("--------------------------------------")

_Loop1                NOP

JMP     #_Loop1          'remember # imediate

cog1Wait              long      900_000_000    'number of clock cycles to wait

cog1Cnt               long      0

valDest               long      %1111_1111_1111_1111_1111_1111_1111_1111

valSrc                long      0

valNeg2               long      -2

val848                long      848

Carry                 long      0

Zero                  long      0

Dest                  long      0

Src                   long      0

MAX                   long      $FFFF_FFFF

MIN                   long      0

### 17.12.2_Example_WRD_BITH_043

BITH    D,{#}S         {WCZ}

Bits D[S[9:5]+S[4:0]:S[4:0]] = 1.    Other bits unaffected. Prior SETQ overrides S[9:5].

C,Z = original D[S[4:0]].

This instruction can be used to set a bit to 1 or a group of bits to 1

D = %00000000_00000000_00000000_00000000

To set B0 =1   BitH D,#1

To set B31 =1 BITH D,#31

D[BH:BL] = D[S[9:5] + S[4:0] : S[4:0]] = 1

S=%00000000_00000000_000000_B9B8B7B6B5_B4B3B2B1B0

Where BH is high bit to be one and BL the low bit to be onezero of the group of one bits

Example D[23:16] =0 Let D[31:0] =$ 0000_0000
We want the following Pattern:
D[31:0] = 00000000_11111111_00000000_00000000 BH = 23 BL =16
then S[9:5] + S[4:0] = 23 S[9:5] = 23 - S[4:0] = 23-16 =7 Then S[31:10] = 0 S[9:5] =7 S[4:0] = 16 S=%00000000_00000000_00000000_111_10000 = 240

BITH D,#240

{{17.12.1_Example_WRD_BITH_043}}

{{

BITL D,{#}S {WCZ}

D[23:16] = D[S[9:5]+S[4:0]:S[4:0]] = 0. Other bits unaffected.

Prior SETQ overrides S[9:5]. C,Z = original D[S[4:0]].

Example D[23:16] =1 Let D[31:0] =$0000_0000

We want the following Pattern:

D[31:0] = = 500000000_11111111_00000000_00000000 BH = 23 BL =16

then S[9:5] + S[4:0] = 23 S[9:5] = 23 - S[4:0] = 23-16 =7

Then S[31:10] = 0 S[9:5] =7 S[4:0] = 16 S=%00000000_00000000_000000_00111_10000 = 240

BITH D,#240

}}

CON

_clkfreq = 200_000_000  ''Debug must be enabled clock must be greater than 10MHZ for Debug

P0 =0 , P1 = 1  , P2 = 2

VAR

Byte cogRunning 'cog ID started is returned or -1 if not started

PUB main()

debug("--------------------------------------")

debug("BITL D,{#}S {WCZ}")

debug("D[BH:BL] = D[S[9:5]+S[4:0]:S[4:0]] = 0. Other bits unaffected.")

debug("Prior SETQ overrides S[9:5]. C,Z = original D[S[4:0]].")

debug("--------------------------------------")

cogRunning := COGINIT(COGEXEC_NEW,@S0_BITH,0)

debug(udec(cogRunning))

repeat                                    'keep cog 0 running

DAT

ORG   0

S0_BITH               DRVH    #P0            'P0 on program running

MOV     Dest,valDest   '$1000_0000

MOV     Src,#1

MODCZ   _CLR, _CLR  WCZ 'set C = Z = 0

IF_C   MOV     Carry,#1        'true  C = 1

IF_NC  MOV     Carry,#0        'false C = 0

IF_Z   MOV     Zero,#1         'true  Z = 1

IF_NZ  MOV     Zero,#0         'false Z = 0

debug("--------------------------------------")

debug("before BITH ",ubin(Dest),udec(Src),ubin(Zero),ubin(Carry))

BITH    Dest,Src  WCZ

IF_C   MOV     Carry,#1         'true  C = 1

IF_NC  MOV     Carry,#0         'false C = 0

IF_Z   MOV     Zero,#1          'true  Z = 1

IF_NZ  MOV     Zero,#0          'false Z = 0

debug("after BITH  ",ubin(Dest),udec(Src),ubin(Zero),ubin(Carry))

debug("--------------------------------------")

MOV     Dest,valDest   '$1000_0000FF

MOV     Src,#240 'S = %00000000_00000000_111_10000

MODCZ   _CLR, _CLR  WCZ 'clr C = Z = 0

IF_C   MOV     Carry,#1        'true  C = 1

IF_NC  MOV     Carry,#0        'false C = 0

IF_Z   MOV     Zero,#1         'true  Z = 1

IF_NZ  MOV     Zero,#0         'false Z = 0

debug("--------------------------------------")

debug("before BITH ",ubin(Dest),udec(Src),ubin(Zero),ubin(Carry))

BITH    Dest,Src  WCZ

IF_C   MOV     Carry,#1         'true  C = 1

IF_NC  MOV     Carry,#0         'false C = 0

IF_Z   MOV     Zero,#1          'true  Z = 1

IF_NZ  MOV     Zero,#0          'false Z = 0

debug("after BITH  ",ubin(Dest),udec(Src),ubin(Zero),ubin(Carry))

debug("--------------------------------------")

_Loop1                NOP

JMP     #_Loop1          'remember # imediate

cog1Wait              long      900_000_000    'number of clock cycles to wait

cog1Cnt               long      0

valDest               long      %0000_0000_0000_0000_0000_0000_0000_0000

valSrc                long      0

valNeg2               long      -2

Carry                 long      0

Zero                  long      0

Dest                  long      0

Src                   long      0

MAX                   long      $FFFF_FFFF

MIN                   long      0

### 17.12.3_Example_WRD_BITC_044

BITC D,{#}S {WCZ}

Bits D[S[9:5]+S[4:0]:S[4:0]] = C. Other bits unaffected. Prior SETQ overrides S[9:5]. C,Z = original D[S[4:0]].

This instruction can be used to set a bit to C carry or a group of bits to C carry

D = 11111111_11111111_11111111_11111111

To set B0 =C   BitC D,#1

To set B31 =C BITC D,#31

D[BH:BL] = D[S[9:5] + S[4:0] : S[4:0]] = 0

S=%00000000_00000000_000000_B9B8B7B6B5_B4B3B2B1B0

Where BH is high bit to be zero and BL the low bit to be zero of the group of zero bits

Example D[23:16] =0 Let D[31:0] =$FFFF_FFFFF
We want the following Pattern:
D[31:0] = 11111111_CCCCCCCC_11111111_11111111 BH = 23 BL =16
then S[9:5] + S[4:0] = 23 S[9:5] = 23 - S[4:0] = 23-16 =7 Then S[31:10] = 0 S[9:5] =7 S[4:0] = 16 S=%00000000_00000000_00000000_111_10000 = 240

BITC D,#240

{{17.12.3_Example_WRD_BITC_044}}

{{

BITC D,{#}S {WCZ}

D[BH:BL] = D[S[9:5]+S[4:0]:S[4:0]] = C. Other bits unaffected.

Prior SETQ overrides S[9:5]. C,Z = original D[S[4:0]].

Example D[23:16] =0 Let D[31:0] =$FFFF_FFFFF

We want the following Pattern:

D[31:0] = 11111111_CCCCCCCC_11111111_11111111 BH = 23 BL =16

then S[9:5] + S[4:0] = 23 S[9:5] = 23 - S[4:0] = 23-16 =7

Then S[31:10] = 0 S[9:5] =7 S[4:0] = 16 S=%00000000_00000000_000000_00111_10000 = 240

BITC D,#240

}}

CON

_clkfreq = 200_000_000  ''Debug must be enabled clock must be greater than 10MHZ for Debug

P0 =0 , P1 = 1  , P2 = 2

VAR

Byte cogRunning 'cog ID started is returned or -1 if not started

PUB main()

debug("--------------------------------------")

debug("BITC D,{#}S {WCZ}")

debug("D[BH:BL] = D[S[9:5]+S[4:0]:S[4:0]] = C. Other bits unaffected.")

debug("Prior SETQ overrides S[9:5]. C,Z = original D[S[4:0]].")

debug("--------------------------------------")

cogRunning := COGINIT(COGEXEC_NEW,@S0_BITC,0)

debug(udec(cogRunning))

repeat                                    'keep cog 0 running

DAT

ORG   0

S0_BITC               DRVH    #P0            'P0 on program running

MOV     Dest,valDest   '$FFFF_FFFF

MOV     Src,#0

MODCZ   _CLR, _CLR  WCZ 'set Z = 0

IF_C   MOV     Carry,#1        'true  C = 1

IF_NC  MOV     Carry,#0        'false C = 0

IF_Z   MOV     Zero,#1         'true  Z = 1

IF_NZ  MOV     Zero,#0         'false Z = 0

debug("--------------------------------------")

debug("before BITC ",ubin(Dest),udec(Src),ubin(Zero),ubin(Carry))

BITC    Dest,Src  WCZ

IF_C   MOV     Carry,#1         'true  C = 1

IF_NC  MOV     Carry,#0         'false C = 0

IF_Z   MOV     Zero,#1          'true  Z = 1

IF_NZ  MOV     Zero,#0          'false Z = 0

debug("after BITC  ",ubin(Dest),udec(Src),ubin(Zero),ubin(Carry))

debug("--------------------------------------")

MOV     Dest,valDest   '$FFFF_FFFF

MOV     Src,#0

MODCZ   _SET, _CLR  WCZ 'set Z = 0

IF_C   MOV     Carry,#1        'true  C = 1

IF_NC  MOV     Carry,#0        'false C = 0

IF_Z   MOV     Zero,#1         'true  Z = 1

IF_NZ  MOV     Zero,#0         'false Z = 0

debug("before BITC ",ubin(Dest),udec(Src),ubin(Zero),ubin(Carry))

BITC    Dest,Src  WCZ

IF_C   MOV     Carry,#1         'true  C = 1

IF_NC  MOV     Carry,#0         'false C = 0

IF_Z   MOV     Zero,#1          'true  Z = 1

IF_NZ  MOV     Zero,#0          'false Z = 0

debug("after BITC  ",ubin(Dest),udec(Src),ubin(Zero),ubin(Carry))

debug("--------------------------------------")

MOV     Dest,valDest   '$FFFF_FFFF

MOV     Src,#240 'S = %00000000_00000000_111_10000

MODCZ   _CLR, _CLR  WCZ 'set Z = 0

IF_C   MOV     Carry,#1        'true  C = 1

IF_NC  MOV     Carry,#0        'false C = 0

IF_Z   MOV     Zero,#1         'true  Z = 1

IF_NZ  MOV     Zero,#0         'false Z = 0

debug("before BITC ",ubin(Dest),udec(Src),ubin(Zero),ubin(Carry))

BITC    Dest,Src  WCZ

IF_C   MOV     Carry,#1         'true  C = 1

IF_NC  MOV     Carry,#0         'false C = 0

IF_Z   MOV     Zero,#1          'true  Z = 1

IF_NZ  MOV     Zero,#0          'false Z = 0

debug("after BITC  ",ubin(Dest),udec(Src),ubin(Zero),ubin(Carry))

debug("--------------------------------------")

_Loop1                NOP

JMP     #_Loop1          'remember # imediate

cog1Wait              long      900_000_000    'number of clock cycles to wait

cog1Cnt               long      0

valDest               long      %1111_1111_1111_1111_1111_1111_1111_1111

valSrc                long      0

valNeg2               long      -2

val848                long      848

Carry                 long      0

Zero                  long      0

Dest                  long      0

Src                   long      0

MAX                   long      $FFFF_FFFF

MIN                   long      0

### 17.12.4_Example_WRD_BITNC_045

BITNC D,{#}S {WCZ}

Bits D[S[9:5]+S[4:0]:S[4:0]] = !C. Other bits unaffected. Prior SETQ overrides S[9:5]. C,Z = original D[S[4:0]].

This instruction can be used to set a bit to !C carry or a group of bits to !C carry

{{17.12.4_Example_WRD_BITNC_045}}

{{

BITNC D,{#}S {WCZ}

Bits D[S[9:5]+S[4:0]:S[4:0]] = !C. Other bits unaffected. Prior SETQ overrides S[9:5]. C,Z = original D[S[4:0]].

This instruction can be used to set a bit to !C carry or a group of bits to !C carry

Example D[23:16] =0 Let D[31:0] =$FFFF_FFFFF

We want the following Pattern:

D[31:0] = 11111111_cccccccc_11111111_11111111 BH = 23 BL =16 let !C = c

then S[9:5] + S[4:0] = 23 S[9:5] = 23 - S[4:0] = 23-16 = 7

Then S[31:10] = 0 S[9:5] =7 S[4:0] = 16 S=%00000000_00000000_000000_00111_10000 = 240

BITC D,#240

}}

CON

_clkfreq = 200_000_000  ''Debug must be enabled clock must be greater than 10MHZ for Debug

P0 =0 , P1 = 1  , P2 = 2

VAR

Byte cogRunning 'cog ID started is returned or -1 if not started

PUB main()

debug("--------------------------------------")

debug("BITNC D,{#}S {WCZ}")

debug("D[BH:BL] = D[S[9:5]+S[4:0]:S[4:0]] = !C. Other bits unaffected.")

debug("Prior SETQ overrides S[9:5]. C,Z = original D[S[4:0]].")

debug("--------------------------------------")

cogRunning := COGINIT(COGEXEC_NEW,@S0_BITNC,0)

debug(udec(cogRunning))

repeat                                    'keep cog 0 running

DAT

ORG   0

S0_BITNC              DRVH    #P0            'P0 on program running

MOV     Dest,valDest   '$FFFF_FFFE

MOV     Src,#0

MODCZ   _CLR, _CLR  WCZ 'set Z = 0

IF_C   MOV     Carry,#1        'true  C = 1

IF_NC  MOV     Carry,#0        'false C = 0

IF_Z   MOV     Zero,#1         'true  Z = 1

IF_NZ  MOV     Zero,#0         'false Z = 0

debug("--------------------------------------")

debug("before BITNC ",ubin(Dest),udec(Src),ubin(Zero),ubin(Carry))

BITNC   Dest,Src  WCZ

IF_C   MOV     Carry,#1         'true  C = 1

IF_NC  MOV     Carry,#0         'false C = 0

IF_Z   MOV     Zero,#1          'true  Z = 1

IF_NZ  MOV     Zero,#0          'false Z = 0

debug("after BITNC  ",ubin(Dest),udec(Src),ubin(Zero),ubin(Carry))

debug("--------------------------------------")

MOV     Dest,MAX   '$FFFF_FFFF

MOV     Src,#0

MODCZ   _SET, _CLR  WCZ 'set Z = 0

IF_C   MOV     Carry,#1        'true  C = 1

IF_NC  MOV     Carry,#0        'false C = 0

IF_Z   MOV     Zero,#1         'true  Z = 1

IF_NZ  MOV     Zero,#0         'false Z = 0

debug("before BITNC ",ubin(Dest),udec(Src),ubin(Zero),ubin(Carry))

BITNC    Dest,Src  WCZ

IF_C   MOV     Carry,#1         'true  C = 1

IF_NC  MOV     Carry,#0         'false C = 0

IF_Z   MOV     Zero,#1          'true  Z = 1

IF_NZ  MOV     Zero,#0          'false Z = 0

debug("after BITNC  ",ubin(Dest),udec(Src),ubin(Zero),ubin(Carry))

debug("--------------------------------------")

MOV     Dest,MIN        '$FFFF_FFFF

MOV     Src,#240        'S = %00000000_00000000_111_10000

MODCZ   _CLR, _CLR  WCZ 'set Z = 0

IF_C   MOV     Carry,#1        'true  C = 1

IF_NC  MOV     Carry,#0        'false C = 0

IF_Z   MOV     Zero,#1         'true  Z = 1

IF_NZ  MOV     Zero,#0         'false Z = 0

debug("before BITNC ",ubin(Dest),udec(Src),ubin(Zero),ubin(Carry))

BITNC   Dest,Src  WCZ

IF_C   MOV     Carry,#1         'true  C = 1

IF_NC  MOV     Carry,#0         'false C = 0

IF_Z   MOV     Zero,#1          'true  Z = 1

IF_NZ  MOV     Zero,#0          'false Z = 0

debug("after BITNC  ",ubin(Dest),udec(Src),ubin(Zero),ubin(Carry))

debug("--------------------------------------")

_Loop1                NOP

JMP     #_Loop1          'remember # imediate

cog1Wait         long      900_000_000    'number of clock cycles to wait

cog1Cnt            long      0

valDest             long      %1111_1111_1111_1111_1111_1111_1111_1110

valSrc                long      0

valNeg2            long      -2

val848               long      848

Carry                 long      0

Zero                  long      0

Dest                  long      0

Src                   long      0

MAX                   long      $FFFF_FFFF

MIN                   long      0

### 17.12.5_Examp1le_WRD_BITZ_046

BITZ D,{#}S {WCZ}

Bits D[S[9:5]+S[4:0]:S[4:0]] = Z. Other bits unaffected. Prior SETQ overrides S[9:5].

C,Z = original D[S[4:0]].

This instruction can be used to set a bit to Z flag or a group of bits to Z flag

{{17.12.5_Example_WRD_BITZ_046}}

{{

BITZ D,{#}S {WCZ}

Bits D[S[9:5]+S[4:0]:S[4:0]] = Z. Other bits unaffected. Prior SETQ overrides S[9:5].

C,Z = original D[S[4:0]].

This instruction can be used to set a bit to Z flag or a group of bits to Z flag

Example D[23:16] =0 Let D[31:0] =$FFFF_FFFFF

We want the following Pattern:

D[31:0] = 11111111_ZZZZZZZZ_11111111_11111111 BH = 23 BL =16

then S[9:5] + S[4:0] = 23 S[9:5] = 23 - S[4:0] = 23-16 =7

Then S[31:10] = 0 S[9:5] =7 S[4:0] = 16 S=%00000000_00000000_000000_00111_10000 = 240

BITC D,#240

}}

CON

_clkfreq = 200_000_000  ''Debug must be enabled clock must be greater than 10MHZ for Debug

P0 =0 , P1 = 1  , P2 = 2

VAR

Byte cogRunning 'cog ID started is returned or -1 if not started

PUB main()

debug("--------------------------------------")

debug("BITNC D,{#}S {WCZ}")

debug("D[BH:BL] = D[S[9:5]+S[4:0]:S[4:0]] = !C. Other bits unaffected.")

debug("Prior SETQ overrides S[9:5]. C,Z = original D[S[4:0]].")

debug("--------------------------------------")

cogRunning := COGINIT(COGEXEC_NEW,@S0_BITZ,0)

debug(udec(cogRunning))

repeat                                    'keep cog 0 running

DAT

ORG   0

S0_BITZ             DRVH    #P0            'P0 on program running

MOV     Dest,valDest   '$FFFF_FFFE

MOV     Src,#0

MODCZ   _CLR, _CLR  WCZ 'set Z = 0

IF_C    MOV     Carry,#1        'true  C = 1

IF_NC  MOV     Carry,#0        'false C = 0

IF_Z    MOV     Zero,#1         'true  Z = 1

IF_NZ  MOV     Zero,#0         'false Z = 0

debug("--------------------------------------")

debug("before BITZ ",ubin(Dest),udec(Src),ubin(Zero),ubin(Carry))

BITZ    Dest,Src  WCZ

IF_C     MOV     Carry,#1         'true  C = 1

IF_NC  MOV     Carry,#0         'false C = 0

IF_Z   MOV     Zero,#1          'true  Z = 1

IF_NZ  MOV     Zero,#0          'false Z = 0

debug("after BITZ  ",ubin(Dest),udec(Src),ubin(Zero),ubin(Carry))

debug("--------------------------------------")

MOV     Dest,MAX   '$FFFF_FFFF

MOV     Src,#0

MODCZ   _SET, _CLR  WCZ 'set Z = 0

IF_C   MOV     Carry,#1        'true  C = 1

IF_NC  MOV     Carry,#0        'false C = 0

IF_Z   MOV     Zero,#1         'true  Z = 1

IF_NZ  MOV     Zero,#0         'false Z = 0

debug("before BITZ ",ubin(Dest),udec(Src),ubin(Zero),ubin(Carry))

BITZ    Dest,Src  WCZ

IF_C    MOV     Carry,#1         'true  C = 1

IF_NC  MOV     Carry,#0         'false C = 0

IF_Z     MOV     Zero,#1          'true  Z = 1

IF_NZ  MOV     Zero,#0          'false Z = 0

debug("after BITZ  ",ubin(Dest),udec(Src),ubin(Zero),ubin(Carry))

debug("--------------------------------------")

MOV     Dest,MAX        '$FFFF_FFFF

MOV     Src,#240        'S = %00000000_00000000_111_10000

MODCZ   _CLR, _CLR  WCZ 'set Z = 0

IF_C       MOV     Carry,#1        'true  C = 1

IF_NC    MOV     Carry,#0        'false C = 0

IF_Z       MOV     Zero,#1         'true  Z = 1

IF_NZ   MOV     Zero,#0         'false Z = 0

debug("before BITZ ",ubin(Dest),udec(Src),ubin(Zero),ubin(Carry))

BITZ   Dest,Src  WCZ

IF_C      MOV     Carry,#1         'true  C = 1

IF_NC    MOV     Carry,#0         'false C = 0

IF_Z       MOV     Zero,#1          'true  Z = 1

IF_NZ    MOV     Zero,#0          'false Z = 0

debug("after BITZ  ",ubin(Dest),udec(Src),ubin(Zero),ubin(Carry))

debug("--------------------------------------")

_Loop1                NOP

JMP     #_Loop1          'remember # imediate

cog1Wait         long      900_000_000    'number of clock cycles to wait

cog1Cnt            long      0

valDest             long      %1111_1111_1111_1111_1111_1111_1111_1110

valSrc                long      0

valNeg2            long      -2

Carry                 long      0

Zero                  long      0

Dest                  long      0

Src                     long      0

MAX                  long      $FFFF_FFFF

MIN                   long      0

### 17.12.6_Examp1le_WRD_BITNZ_047

BITNZ    D,{#}S         {WCZ}

Bits D[S[9:5]+S[4:0]:S[4:0]] = !Z.   Other bits unaffected.

Prior SETQ overrides S[9:5]. C,Z = original D[S[4:0]].

{{17.12.6_Example_WRD_BITNZ_047}}

{{

BITNZ    D,{#}S         {WCZ}

Bits D[S[9:5]+S[4:0]:S[4:0]] = !Z.   Other bits unaffected.

Prior SETQ overrides S[9:5]. C,Z = original D[S[4:0]].

Example D[23:16] =0 Let D[31:0] =$FFFF_FFFFF

We want the following Pattern:

D[31:0] = 11111111_zzzzzzzz_11111111_11111111 BH = 23 BL =16    let  !Z = z

then S[9:5] + S[4:0] = 23 S[9:5] = 23 - S[4:0] = 23-16 =7

Then S[31:10] = 0 S[9:5] =7 S[4:0] = 16 S=%00000000_00000000_000000_00111_10000 = 240

BITC D,#240

}}

CON

_clkfreq = 200_000_000  ''Debug must be enabled clock must be greater than 10MHZ for Debug

P0 =0 , P1 = 1  , P2 = 2

VAR

Byte cogRunning 'cog ID started is returned or -1 if not started

PUB main()

debug("--------------------------------------")

debug("BITNZ D,{#}S {WCZ}")

debug("D[BH:BL] = D[S[9:5]+S[4:0]:S[4:0]] = !Z. Other bits unaffected.")

debug("Prior SETQ overrides S[9:5]. C,Z = original D[S[4:0]].")

debug("--------------------------------------")

cogRunning := COGINIT(COGEXEC_NEW,@S0_BITNZ,0)

debug(udec(cogRunning))

repeat                                    'keep cog 0 running

DAT

ORG   0

S0_BITNZ              DRVH    #P0            'P0 on program running

MOV     Dest,valDest   '$FFFF_FFFE

MOV     Src,#0

MODCZ   _CLR, _CLR  WCZ 'set Z = 0

IF_C   MOV     Carry,#1        'true  C = 1

IF_NC  MOV     Carry,#0        'false C = 0

IF_Z   MOV     Zero,#1         'true  Z = 1

IF_NZ  MOV     Zero,#0         'false Z = 0

debug("--------------------------------------")

debug("before BITNZ ",ubin(Dest),udec(Src),ubin(Zero),ubin(Carry))

BITNZ    Dest,Src  WCZ

IF_C   MOV     Carry,#1         'true  C = 1

IF_NC  MOV     Carry,#0         'false C = 0

IF_Z   MOV     Zero,#1          'true  Z = 1

IF_NZ  MOV     Zero,#0          'false Z = 0

debug("after BITNZ  ",ubin(Dest),udec(Src),ubin(Zero),ubin(Carry))

debug("--------------------------------------")

MOV     Dest,MAX   '$FFFF_FFFF

MOV     Src,#0

MODCZ   _CLR, _SET  WCZ 'set Z = 0

IF_C   MOV     Carry,#1        'true  C = 1

IF_NC  MOV     Carry,#0        'false C = 0

IF_Z   MOV     Zero,#1         'true  Z = 1

IF_NZ  MOV     Zero,#0         'false Z = 0

debug("before BITNZ ",ubin(Dest),udec(Src),ubin(Zero),ubin(Carry))

BITNZ   Dest,Src  WCZ

IF_C   MOV     Carry,#1         'true  C = 1

IF_NC  MOV     Carry,#0         'false C = 0

IF_Z   MOV     Zero,#1          'true  Z = 1

IF_NZ  MOV     Zero,#0          'false Z = 0

debug("after BITNZ  ",ubin(Dest),udec(Src),ubin(Zero),ubin(Carry))

debug("--------------------------------------")

MOV     Dest,MIN        '$0

MOV     Src,#240        'S = %00000000_00000000_111_10000

MODCZ   _CLR, _CLR  WCZ 'set Z = 0

IF_C   MOV     Carry,#1        'true  C = 1

IF_NC  MOV     Carry,#0        'false C = 0

IF_Z   MOV     Zero,#1         'true  Z = 1

IF_NZ  MOV     Zero,#0         'false Z = 0

debug("before BITZ ",ubin(Dest),udec(Src),ubin(Zero),ubin(Carry))

BITNZ   Dest,Src  WCZ

IF_C   MOV     Carry,#1         'true  C = 1

IF_NC  MOV     Carry,#0         'false C = 0

IF_Z   MOV     Zero,#1          'true  Z = 1

IF_NZ  MOV     Zero,#0          'false Z = 0

debug("after BITNZ  ",ubin(Dest),udec(Src),ubin(Zero),ubin(Carry))

debug("--------------------------------------")

_Loop1                NOP

JMP     #_Loop1          'remember # imediate

cog1Wait        long      900_000_000    'number of clock cycles to wait

cog1Cnt           long      0

valDest            long      %1111_1111_1111_1111_1111_1111_1111_1110

valSrc               long      0

Carry                 long      0

Zero                  long      0

Dest                  long      0

Src                     long      0

MAX                  long      $FFFF_FFFF

MIN                   long      0

### 17.12.7_Example_WRD_BITRND_048

BITRND D,{#}S {WCZ}

Bits D[S[9:5]+S[4:0]:S[4:0]] = RNDs. Other bits unaffected. Prior SETQ overrides S[9:5]. C,Z = original D[S[4:0]].

{{17.12.7_Example_WRD_BITRND_048}}

{{

BITRND D,{#}S {WCZ}

Bits D[S[9:5]+S[4:0]:S[4:0]] = RNDs. Other bits unaffected.

Prior SETQ overrides S[9:5]. C,Z = original D[S[4:0]].

Value Returned is Randomized

Example D[23:16] =0 Let D[31:0] =$FFFF_FFFFF

We want the following Pattern:

D[31:0] = 11111111_RRANDOMM_11111111_11111111 BH = 23 BL =16

then S[9:5] + S[4:0] = 23 S[9:5] = 23 - S[4:0] = 23-16 =7

Then S[31:10] = 0 S[9:5] =7 S[4:0] = 16 S=%00000000_00000000_000000_00111_10000 = 240

BITC D,#240

}}

CON

_clkfreq = 200_000_000  ''Debug must be enabled clock must be greater than 10MHZ for Debug

P0 =0 , P1 = 1  , P2 = 2

VAR

Byte cogRunning 'cog ID started is returned or -1 if not started

PUB main()

debug("--------------------------------------")

debug("BITRND D,{#}S {WCZ}")

debug("D[BH:BL] = D[S[9:5]+S[4:0]:S[4:0]] = RANDOM. Other bits unaffected.")

debug("Prior SETQ overrides S[9:5]. C,Z = original D[S[4:0]].")

debug("--------------------------------------")

cogRunning := COGINIT(COGEXEC_NEW,@S0_BITRND,0)

debug(udec(cogRunning))

repeat                                    'keep cog 0 running

DAT

ORG   0

S0_BITRND             DRVH    #P0            'P0 on program running

MOV     Dest,valDest   '$FFFF_FFFE

MOV     Src,#0

MODCZ   _CLR, _CLR  WCZ 'set Z = 0

IF_C   MOV     Carry,#1        'true  C = 1

IF_NC  MOV     Carry,#0        'false C = 0

IF_Z   MOV     Zero,#1         'true  Z = 1

IF_NZ  MOV     Zero,#0         'false Z = 0

debug("--------------------------------------")

debug("before BITRND ",ubin(Dest),udec(Src),ubin(Zero),ubin(Carry))

BITRND    Dest,Src  WCZ

IF_C   MOV     Carry,#1         'true  C = 1

IF_NC  MOV     Carry,#0         'false C = 0

IF_Z   MOV     Zero,#1          'true  Z = 1

IF_NZ  MOV     Zero,#0          'false Z = 0

debug("after BITRND  ",ubin(Dest),udec(Src),ubin(Zero),ubin(Carry))

debug("--------------------------------------")

MOV     Dest,MAX   '$FFFF_FFFF

MOV     Src,#0

MODCZ   _CLR, _SET  WCZ 'set Z = 0

IF_C   MOV     Carry,#1        'true  C = 1

IF_NC  MOV     Carry,#0        'false C = 0

IF_Z   MOV     Zero,#1         'true  Z = 1

IF_NZ  MOV     Zero,#0         'false Z = 0

debug("before BITRND ",ubin(Dest),udec(Src),ubin(Zero),ubin(Carry))

BITRND   Dest,Src  WCZ

IF_C   MOV     Carry,#1         'true  C = 1

IF_NC  MOV     Carry,#0         'false C = 0

IF_Z   MOV     Zero,#1          'true  Z = 1

IF_NZ  MOV     Zero,#0          'false Z = 0

debug("after BITRND  ",ubin(Dest),udec(Src),ubin(Zero),ubin(Carry))

debug("--------------------------------------")

MOV     Dest,MIN        '$0

MOV     Src,#240        'S = %00000000_00000000_111_10000

MODCZ   _CLR, _CLR  WCZ 'set Z = 0

IF_C   MOV     Carry,#1        'true  C = 1

IF_NC  MOV     Carry,#0        'false C = 0

IF_Z   MOV     Zero,#1         'true  Z = 1

IF_NZ  MOV     Zero,#0         'false Z = 0

debug("before BITRND  ",ubin(Dest),udec(Src),ubin(Zero),ubin(Carry))

BITRND   Dest,Src  WCZ

IF_C   MOV     Carry,#1         'true  C = 1

IF_NC  MOV     Carry,#0         'false C = 0

IF_Z   MOV     Zero,#1          'true  Z = 1

IF_NZ  MOV     Zero,#0          'false Z = 0

debug("after BITRND  ",ubin(Dest),udec(Src),ubin(Zero),ubin(Carry))

debug("--------------------------------------")

_Loop1                NOP

JMP     #_Loop1          'remember # imediate

cog1Wait              long      900_000_000    'number of clock cycles to wait

cog1Cnt               long      0

valDest               long      %1111_1111_1111_1111_1111_1111_1111_1110

valSrc                long      0

Carry                 long      0

Zero                  long      0

Dest                  long      0

Src                   long      0

MAX                   long      $FFFF_FFFF

MIN                   long      0

### 17.12.8_Example_WRD_BITNOT_049

BITNOT D,{#}S {WCZ}

Toggle bits D[S[9:5]+S[4:0]:S[4:0]]. Other bits unaffected. Prior SETQ overrides S[9:5].

C,Z = original D[S[4:0]].

{{17.12.8_Example_WRD_BITNOT_049}}

{{

BITNOT D,{#}S {WCZ}

Toggle bits D[S[9:5]+S[4:0]:S[4:0]]. Other bits unaffected. Prior SETQ overrides S[9:5].

C,Z = original D[S[4:0]].

Example D[23:16] =0 Let D[31:0] =$FF_%10101010_$FFFF

We want the following Pattern:

D[31:0] = 11111111_01010101_11111111_11111111 BH = 23 BL =16

then S[9:5] + S[4:0] = 23 S[9:5] = 23 - S[4:0] = 23-16 =7

Then S[31:10] = 0 S[9:5] =7 S[4:0] = 16 S=%00000000_00000000_000000_00111_10000 = 240

BITC D,#240

}}

CON

_clkfreq = 200_000_000  ''Debug must be enabled clock must be greater than 10MHZ for Debug

P0 =0 , P1 = 1  , P2 = 2

VAR

Byte cogRunning 'cog ID started is returned or -1 if not started

PUB main()

debug("--------------------------------------")

debug("BITNOT D,{#}S {WCZ}")

debug("D[BH:BL] = D[S[9:5]+S[4:0]:S[4:0]] = RANDOM. Other bits unaffected.")

debug("Prior SETQ overrides S[9:5]. C,Z = original D[S[4:0]].")

debug("--------------------------------------")

cogRunning := COGINIT(COGEXEC_NEW,@S0_BITNOT,0)

debug(udec(cogRunning))

repeat                                    'keep cog 0 running

DAT

ORG   0

S0_BITNOT             DRVH    #P0            'P0 on program running

MOV     Dest,valDest   '$FFFF_FFFE

MOV     Src,#0

MODCZ   _CLR, _CLR  WCZ 'set Z = 0

IF_C   MOV     Carry,#1        'true  C = 1

IF_NC  MOV     Carry,#0        'false C = 0

IF_Z   MOV     Zero,#1         'true  Z = 1

IF_NZ  MOV     Zero,#0         'false Z = 0

debug("--------------------------------------")

debug("before BITNOT ",ubin(Dest),udec(Src),ubin(Zero),ubin(Carry))

BITNOT  Dest,Src  WCZ

IF_C   MOV     Carry,#1         'true  C = 1

IF_NC  MOV     Carry,#0         'false C = 0

IF_Z   MOV     Zero,#1          'true  Z = 1

IF_NZ  MOV     Zero,#0          'false Z = 0

debug("after BITNOT  ",ubin(Dest),udec(Src),ubin(Zero),ubin(Carry))

debug("--------------------------------------")

MOV     Dest,MAX   '$FFFF_FFFF

MOV     Src,#0

MODCZ   _CLR, _SET  WCZ 'set Z = 0

IF_C   MOV     Carry,#1        'true  C = 1

IF_NC  MOV     Carry,#0        'false C = 0

IF_Z   MOV     Zero,#1         'true  Z = 1

IF_NZ  MOV     Zero,#0         'false Z = 0

debug("before BITNOT ",ubin(Dest),udec(Src),ubin(Zero),ubin(Carry))

BITNOT  Dest,Src  WCZ

IF_C   MOV     Carry,#1         'true  C = 1

IF_NC  MOV     Carry,#0         'false C = 0

IF_Z   MOV     Zero,#1          'true  Z = 1

IF_NZ  MOV     Zero,#0          'false Z = 0

debug("after BITNOT  ",ubin(Dest),udec(Src),ubin(Zero),ubin(Carry))

debug("--------------------------------------")

MOV     Dest,valExample '$FF_01010101_$FF_$FFFF

MOV     Src,#240        'S = %00000000_00000000_111_10000

MODCZ   _CLR, _CLR  WCZ 'set Z = 0

IF_C   MOV     Carry,#1        'true  C = 1

IF_NC  MOV     Carry,#0        'false C = 0

IF_Z   MOV     Zero,#1         'true  Z = 1

IF_NZ  MOV     Zero,#0         'false Z = 0

debug("before BITNOT  ",ubin(Dest),udec(Src),ubin(Zero),ubin(Carry))

BITNOT   Dest,Src  WCZ

IF_C   MOV     Carry,#1         'true  C = 1

IF_NC  MOV     Carry,#0         'false C = 0

IF_Z   MOV     Zero,#1          'true  Z = 1

IF_NZ  MOV     Zero,#0          'false Z = 0

debug("after BITNOT  ",ubin(Dest),udec(Src),ubin(Zero),ubin(Carry))

debug("--------------------------------------")

_Loop1                NOP

JMP     #_Loop1          'remember # imediate

cog1Wait          long      900_000_000    'number of clock cycles to wait

cog1Cnt            long      0

valDest              long      %1111_1111_1111_1111_1111_1111_1111_1110

valSrc                long      0

valExample      long      %11111111_01010101_11111111_11111111

Carry                 long      0

Zero                  long      0

Dest                  long      0

Src                     long      0

MAX                 long      $FFFF_FFFF

MIN                  long      0

## 17.13) AND Boolean AND

| AND D,{#}S {WC/WZ/WCZ} | AND S into D. D = D & S. C = parity of result. * |
| --- | --- |
| ANDN D,{#}S {WC/WZ/WCZ} | AND !S into D. D = D & !S. C = parity of result. * |

### 17.13.1_Example_WRD_AND_050

AND D,{#}S {WC/WZ/WCZ}

AND S into D. D = D & S. C = parity of result. *

D__S__ D_

0     0  |  0    AND Truth Table

0     1  |  0

1     0  |  0

1     1  |  1

Symbol for AND = “&”

{{17.13.1_Example_WRD_AND_050}}

{{

AND D,{#}S {WC/WZ/WCZ}

AND S into D. D = D & S. C = parity of result. *

Parity even if Dest has even number of 1 bits C =  0

Parity odd if Dest odd number bits C =  1

D_____S__ D___

0     0  |  0    AND Truth Table

0     1  |  0

1     0  |  0

1     1  |  1

}}

CON

_clkfreq = 200_000_000  ''Debug must be enabled clock must be greater than 10MHZ for Debug

P0 =0 , P1 = 1  , P2 = 2

VAR

Byte cogRunning 'cog ID started is returned or -1 if not started

PUB main()

debug("--------------------------------------")

debug("AND D,{#}S {WC/WZ/WCZ} ")

debug("AND S into D. D = D & S. C = parity of result. * ")

debug("--------------------------------------")

cogRunning := COGINIT(COGEXEC_NEW,@S0_AND,0)

debug(udec(cogRunning))

repeat                                    'keep cog 0 running

DAT

ORG   0

S0_AND             DRVH    #P0            'P0 on program running

MOV     Dest,valDest   'valDest=%11001100_11001100_11001100_11001100

MOV     Src,valSrc     'valSrc =%10101010_10101010_10101010_10101010

MODCZ   _CLR, _CLR  WCZ 'set Z = 0

IF_C     MOV     Carry,#1        'true  C = 1

IF_NC  MOV     Carry,#0        'false C = 0

IF_Z     MOV     Zero,#1         'true  Z = 1

IF_NZ  MOV     Zero,#0         'false Z = 0

debug("--------------------------------------")

debug("before AND ")

debug(ubin(Dest))

debug(" ",ubin(Src))

AND     Dest,Src  WCZ

IF_C     MOV     Carry,#1         'true  C = 1

IF_NC  MOV     Carry,#0         'false C = 0

IF_Z     MOV     Zero,#1          'true  Z = 1

IF_NZ  MOV     Zero,#0          'false Z = 0

debug("after AND  ")

debug(ubin(Dest),ubin(Src))

debug("even parity C = 0 odd parity C =1 ",ubin(Carry))

debug("--------------------------------------")

_Loop1                NOP

JMP     #_Loop1          'remember # imediate

cog1Wait        long      900_000_000    'number of clock cycles to wait

cog1Cnt          long      0

valDest            long      %11001100_11001100_11001100_11001100

valSrc                long      %10101010_10101010_10101010_10101010

Carry                 long      0

Zero                  long      0

Dest                  long      0

Src                     long      0

MAX                  long      $FFFF_FFFF

MIN                   long      0

### 17.13.2 Example_WRD_ANDN_051

ANDN D,{#}S {WC/WZ/WCZ}

AND  !S into D. D = D & !S.  C = parity of result. *

Parity even if Dest has even number of 1 bits C =  0

Parity odd if Dest odd number bits C =  1

D_  S__!S |_ D_

0     0     1 |   0    ANDN Truth Table

0     1     1 |   0

1     0     1 |   1

1     1     0 |   0

{{17.13.2_Example_WRD_ANDN_051}}

{{

ANDN D,{#}S {WC/WZ/WCZ}

AND  !S into D. D = D & !S.  C = parity of result. *

D__S_!S_|_ D_

0    0    1 |  0    ANDN Truth Table

0    1    1 |  0

1    0    1 |  1

1    1    0 |  0

}}

CON

_clkfreq = 200_000_000  ''Debug must be enabled clock must be greater than 10MHZ for Debug

P0 =0 , P1 = 1  , P2 = 2

VAR

Byte cogRunning 'cog ID started is returned or -1 if not started

PUB main()

debug("--------------------------------------")

debug("ANDN D,{#}S {WC/WZ/WCZ} ")

debug("AND !S into D. D = D & !S. C = parity of result. * ")

debug("--------------------------------------")

cogRunning := COGINIT(COGEXEC_NEW,@S0_ANDN,0)

debug(udec(cogRunning))

repeat                                    'keep cog 0 running

DAT

ORG   0

S0_ANDN          DRVH    #P0            'P0 on program running

MOV     Dest,valDest   'valDest=%11110011_00110011_00110011_00110011

MOV     Src,valSrc     'valSrc =%00000101_01010101_01010101_01010101

MODCZ   _CLR, _CLR  WCZ 'set Z = 0

IF_C     MOV     Carry,#1        'true  C = 1

IF_NC  MOV     Carry,#0        'false C = 0

IF_Z     MOV     Zero,#1         'true  Z = 1

IF_NZ  MOV     Zero,#0         'false Z = 0

debug("--------------------------------------")

debug("before ANDN ")

debug(ubin(Dest))

debug("      ",ubin(Src))

ANDN    Dest,Src  WCZ

IF_C       MOV     Carry,#1         'true  C = 1

IF_NC    MOV     Carry,#0         'false C = 0

IF_Z      MOV     Zero,#1          'true  Z = 1

IF_NZ    MOV     Zero,#0          'false Z = 0

debug("after ANDN  ")

debug(ubin(Dest),ubin(Src))

debug("even parity C = 0 odd parity C =1 ",ubin(Carry))

debug("--------------------------------------")

_Loop1                NOP

JMP     #_Loop1          'remember # imediate

cog1Wait         long      900_000_000    'number of clock cycles to wait

cog1Cnt            long      0

valDest             long      %11110011_00110011_00110011_00110011

valSrc                long      %00000101_01010101_01010101_01010101

Carry                 long      0

Zero                  long      0

Dest                  long      0

Src                     long      0

MAX                  long      $FFFF_FFFF

MIN                   long      0

## 17.14) OR Boolean OR XOR

| OR D,{#}S {WC/WZ/WCZ} | OR S into D. D = D ¦ S. C = parity of result. * |
| --- | --- |
| XOR D,{#}S {WC/WZ/WCZ} | XOR S into D. D = D ^ S. C = parity of result. * |

D___S__ D__                                             D___S__ D__

0       0  |  0   OR Truth Table                    0       0  |  0      XOR Truth Table

0       1  |  0                                                  0       1  |  1

1       0  |  0                                                  1       0  |  1

1       1  |  1                                                  1       1  |  0

Symbol for OR = “|”                                  Symbol for XOR  = “^”

### 17.14.1_Example_WRD_OR_052

OR D,{#}S {WC/WZ/WCZ}

OR S into D. D = D | S. C = parity of result. *

{{17.14.1_Example_WRD_OR_052}}

{{

OR D,{#}S {WC/WZ/WCZ}

OR S into D. D = D | S. C = parity of result. *

Parity even if Dest has even number of 1 bits C =  0

Parity odd if Dest odd number bits C =  1

D_____S__ D___

0     0  |  0   OR Truth Table

0     1  |  0

1     0  |  0

1     1  |  1

}}

CON

_clkfreq = 200_000_000  ''Debug must be enabled clock must be greater than 10MHZ for Debug

P0 =0 , P1 = 1  , P2 = 2

VAR

Byte cogRunning 'cog ID started is returned or -1 if not started

PUB main()

debug("--------------------------------------")

debug("OR D,{#}S {WC/WZ/WCZ} ")

debug("OR S into D. D = D | S. C = parity of result. *")

debug("--------------------------------------")

cogRunning := COGINIT(COGEXEC_NEW,@S0_OR,0)

debug(udec(cogRunning))

repeat                                    'keep cog 0 running

DAT

ORG   0

S0_OR               DRVH    #P0            'P0 on program running

MOV     Dest,valDest   '%11110011_00110011_00110011_00110011

MOV     Src,valSrc     '%11110101_01010101_01010101_01010101

MODCZ   _CLR, _CLR  WCZ 'set Z = 0

IF_C   MOV     Carry,#1        'true  C = 1

IF_NC  MOV     Carry,#0        'false C = 0

IF_Z   MOV     Zero,#1         'true  Z = 1

IF_NZ  MOV     Zero,#0         'false Z = 0

debug("--------------------------------------")

debug("before OR ")

debug(ubin(Dest))

debug(" ",ubin(Src))

OR    Dest,Src  WCZ

IF_C   MOV     Carry,#1         'true  C = 1

IF_NC  MOV     Carry,#0         'false C = 0

IF_Z   MOV     Zero,#1          'true  Z = 1

IF_NZ  MOV     Zero,#0          'false Z = 0

debug("after OR  ")

debug(ubin(Dest),ubin(Src))

debug("even parity C = 0 odd parity C =1 ",ubin(Carry))

debug("--------------------------------------")

_Loop1         NOP

JMP     #_Loop1          'remember # imediate

cog1Wait         long      900_000_000    'number of clock cycles to wait

cog1Cnt           long      0

valDest             long      %11110011_00110011_00110011_00110001

valSrc                long      %11110101_01010101_01010101_01010101

Carry                 long      0

Zero                  long      0

Dest                  long      0

Src                     long      0

MAX                  long      $FFFF_FFFF

MIN                   long      0

### 17.14.2_Example_WRD_XOR_053

XOR D,{#}S {WC/WZ/WCZ}

XOR S into D. D = D ^ S. C = parity of result. *

{{17.14.2_Example_WRD_XOR_053}}

{{

XOR D,{#}S {WC/WZ/WCZ}

XOR S into D. D = D ^ S. C = parity of result. *

Parity even if Dest has even number of 1 bits C =  0

Parity odd if Dest odd number bits C =  1

D_____S__ D___

0     0  |  0   XOR Truth Table Symbol for exlusive or XOR = "^"

0     1  |  1

1     0  |  1

1     1  |  0

}}

CON

_clkfreq = 200_000_000  ''Debug must be enabled clock must be greater than 10MHZ for Debug

P0 =0 , P1 = 1  , P2 = 2

VAR

Byte cogRunning 'cog ID started is returned or -1 if not started

PUB main()

debug("--------------------------------------")

debug("XOR D,{#}S {WC/WZ/WCZ} ")

debug("XOR S into D. D = D ^ S. C = parity of result. *")

debug("--------------------------------------")

cogRunning := COGINIT(COGEXEC_NEW,@S0_XOR,0)

debug(udec(cogRunning))

repeat                                    'keep cog 0 running

DAT

ORG   0

S0_XOR               DRVH    #P0            'P0 on program running

MOV     Dest,valDest   '%11110011_00110011_00110011_00110011

MOV     Src,valSrc     '%11110101_01010101_01010101_01010101

MODCZ   _CLR, _CLR  WCZ 'set Z = 0

IF_C   MOV     Carry,#1        'true  C = 1

IF_NC  MOV     Carry,#0        'false C = 0

IF_Z   MOV     Zero,#1         'true  Z = 1

IF_NZ  MOV     Zero,#0         'false Z = 0

debug("--------------------------------------")

debug("before XOR ")

debug(ubin(Dest))

debug(" ",ubin(Src))

XOR     Dest,Src  WCZ

IF_C   MOV     Carry,#1         'true  C = 1

IF_NC  MOV     Carry,#0         'false C = 0

IF_Z   MOV     Zero,#1          'true  Z = 1

IF_NZ  MOV     Zero,#0          'false Z = 0

debug("after XOR  ")

debug(ubin(Dest),ubin(Src))

debug("even parity C = 0 odd parity C =1 ",ubin(Carry))

debug("--------------------------------------")

_Loop1                NOP

JMP     #_Loop1          'remember # imediate

cog1Wait         long      900_000_000    'number of clock cycles to wait

cog1Cnt           long      0

valDest             long      %11110011_00110011_00110011_00110001

valSrc                long      %11110101_01010101_01010101_01010101

Carry                 long      0

Zero                  long      0

Dest                  long      0

Src                    long      0

MAX                 long      $FFFF_FFFF

MIN                   long      0

## 17.15) MUX Mask Destination Register

| MUXC D,{#}S {WC/WZ/WCZ} | Mux C into each D bit that is '1' in S. D = (!S & D ) ¦ (S & {32{ C}}). C = parity of result. * |
| --- | --- |
| MUXNC D,{#}S {WC/WZ/WCZ} | Mux !C into each D bit that is '1' in S. D = (!S & D ) ¦ (S & {32{!C}}). C = parity of result. * |
| MUXZ D,{#}S {WC/WZ/WCZ} | Mux Z into each D bit that is '1' in S. D = (!S & D ) ¦ (S & {32{ Z}}). C = parity of result. * |
| MUXNZ D,{#}S {WC/WZ/WCZ} | Mux !Z into each D bit that is '1' in S. D = (!S & D ) ¦ (S & {32{!Z}}). C = parity of result. * |

17.15.1) MUXC Set discrete bits of a Value to the state of C flag

17.15.2) MUXNC Set discrete bits of a Value to the stare of !C flag

17.15.3) MUXZ Set discrete bits of a Value to the state of Z flag

17.15.4) MUXNZ Set discrete bits of a Value to state of !Z flag

### 17.15.1_Example_WRD_MUXC_054

MUXC D,{#}S {WC/WZ/WCZ}

Mux C into each D bit that is '1' in S. D = (!S & D ) | (S & {32{ C}}). C = parity of result. *

WC -Parity even if Dest has even number of 1 bits C == 0  Parity odd if Dest odd number bits C == 1

WZ-IF Destination result == 0 then Z =1 else Z = 0

{{17.15.1_Example_WRD_MUXC_054}}

''MUXC D,{#}S {WC/WZ/WCZ}

''Mux C into each D bit that is '1' in S.

''D = (!S & D ) | (S & {32{ C}}). C = parity of result. *

''WC -Parity even if Dest has even number of 1 bits C = 0

''    Parity odd if Dest odd number bits C = 1

''WZ-IF Destination result == 0 then Z =1

''   IF Destination result != 0 then Z = 0

CON

_clkfreq = 200_000_000  ''Debug must be enabled clock must be greater than 10MHZ for Debug

P0 =0 , P1 = 1  , P2 = 2

VAR

Byte cogRunning 'cog ID started is returned or -1 if not started

PUB main()

debug("--------------------------------------")

debug("MUXC D,{#}S {WC/WZ/WCZ}")

debug("Mux C into each D bit that is '1' in S.")

debug("D = (!S & D ) | (S & {32{ C}}). C = parity of result. *")

debug("--------------------------------------")

cogRunning := COGINIT(COGEXEC_NEW,@S0_MUXC,0)

debug(udec(cogRunning))

repeat                                    'keep cog 0 running

DAT

ORG   0

S0_MUXC               DRVH    #P0            'P0 on program running

MOV     Dest,valDest    '%1111_1111

MOV     Src,valSrc      '%1111_1110

MODCZ   _CLR, _CLR  WCZ 'set Z = 0

IF_C   MOV     Carry,#1        'true  C = 1

IF_NC  MOV     Carry,#0        'false C = 0

IF_Z   MOV     Zero,#1         'true  Z = 1

IF_NZ  MOV     Zero,#0         'false Z = 0

debug("--------------------------------------")

debug("before MUXC ",ubin(Dest),ubin(Src),ubin(Carry),ubin(Zero))

MUXC    Dest,Src  WCZ

IF_C   MOV     Carry,#1         'true  C = 1

IF_NC  MOV     Carry,#0         'false C = 0

IF_Z   MOV     Zero,#1          'true  Z = 1

IF_NZ  MOV     Zero,#0          'false Z = 0

debug("after MUXC ",ubin(Dest),ubin(Src),ubin(Carry),ubin(Zero))

MOV     Dest,MAX        '$FFFF_FFFF

MOV     Src,valSrc      '%1111_1110

MODCZ   _CLR, _CLR  WCZ 'set Z = 0

IF_C   MOV     Carry,#1        'true  C = 1

IF_NC  MOV     Carry,#0        'false C = 0

IF_Z   MOV     Zero,#1         'true  Z = 1

IF_NZ  MOV     Zero,#0         'false Z = 0

debug("--------------------------------------")

debug("before MUXC ",ubin(Dest),ubin(Src),ubin(Carry),ubin(Zero))

MUXC    Dest,Src  WCZ

IF_C   MOV     Carry,#1         'true  C = 1

IF_NC  MOV     Carry,#0         'false C = 0

IF_Z   MOV     Zero,#1          'true  Z = 1

IF_NZ  MOV     Zero,#0          'false Z = 0

debug("after MUXC ",ubin(Dest),ubin(Src),ubin(Carry),ubin(Zero))

debug("--------------------------------------")

_Loop1                NOP

JMP     #_Loop1          'remember # imediate

cog1Cnt          long      0

valDest            long      %1111_1111

valSrc              long      %1111_1110

Carry               long      0

Zero                long      0

Dest                long      0

Src                   long      0

MAX                long      $FFFF_FFFF

MIN                 long      0

17.15.2_Example_WRD_MUXNC_055

MUXNC D,{#}S {WC/WZ/WCZ}

Mux !C into each D bit that is '1' in S. D = (!S & D ) | (S & {32{!C}}). C = parity of result. *

WC -Parity even if Dest has even number of 1 bits C == 1  Parity odd if Dest odd number bits C == 0

WZ-IF Destination result == 0 then Z =1 else Z = 0

{{17.15.2_Example_WRD_MUXNC_055}}

''MUXNC D,{#}S {WC/WZ/WCZ}

''Mux !C into each D bit that is '1' in S. D = (!S & D ) | (S & {32{!C}}). C = parity of result. *

''WC -Parity even if Dest has even number of 1 bits C == 1  Parity odd if Dest odd number bits C == 0

''WZ-IF Destination result == 0 then Z =1 else Z = 0

CON

_clkfreq = 200_000_000  ''Debug must be enabled clock must be greater than 10MHZ for Debug

P0 =0 , P1 = 1  , P2 = 2

VAR

Byte cogRunning 'cog ID started is returned or -1 if not started

PUB main()

debug("--------------------------------------")

debug("MUXC D,{#}S {WC/WZ/WCZ}")

debug("Mux C into each D bit that is '1' in S.")

debug("D = (!S & D ) | (S & {32{ C}}). C = parity of result. *")

debug("--------------------------------------")

cogRunning := COGINIT(COGEXEC_NEW,@S0_MUXC,0)

debug(udec(cogRunning))

repeat                                    'keep cog 0 running

DAT

ORG   0

S0_MUXC                DRVH    #P0            'P0 on program running

MOV     Dest,valDest    '%1111_1111

MOV     Src,valSrc      '%1111_1111

MODCZ   _CLR, _CLR  WCZ 'set Z = 0

IF_C   MOV     Carry,#1        'true  C = 1

IF_NC  MOV     Carry,#0        'false C = 0

IF_Z   MOV     Zero,#1         'true  Z = 1

IF_NZ  MOV     Zero,#0         'false Z = 0

debug("--------------------------------------")

debug("before MUXNC ",ubin(Dest),ubin(Src),ubin(Carry),ubin(Zero))

MUXNC    Dest,Src  WCZ

IF_C   MOV     Carry,#1         'true  C = 1

IF_NC  MOV     Carry,#0         'false C = 0

IF_Z   MOV     Zero,#1          'true  Z = 1

IF_NZ  MOV     Zero,#0          'false Z = 0

debug("after MUXC ",ubin(Dest),ubin(Src),ubin(Carry),ubin(Zero))

debug("--------------------------------------")

_Loop1                NOP

JMP     #_Loop1          'remember # imediate

valDest              long      %1111_1111

valSrc                long      %1111_1111

Carry                  long      0

Zero                   long      0

Dest                   long      0

Src                     long      0

MAX                  long      $FFFF_FFFF

MIN                   long      0

### 17.15.2-Example_MUXNC_055

MUXNC D,{#}S {WC/WZ/WCZ}

Mux !C into each D bit that is '1' in S. D = (!S & D ) | (S & {32{!C}}). C = parity of result. *

WC -Parity even if Dest has even number of 1 bits C == 0  Parity odd if Dest odd number bits C == 1

WZ-IF Destination result == 0 then Z =1 else Z = 0

{{17.15.2_Example_WRD_MUXNC_055}}

''MUXNC D,{#}S {WC/WZ/WCZ}

''Mux !C into each D bit that is '1' in S.

''D = (!S & D ) | (S & {32{!C}}).

''C = parity of result. *

''WC -Parity even if Dest has even number of 1 bits C = 0

''    Parity odd if Dest odd number bits C = 1

''WZ-IF Destination result == 0 then Z = 1

''   IF Destination result != 0 then Z = 0

CON

_clkfreq = 200_000_000  ''Debug must be enabled clock must be greater than 10MHZ for Debug

P0 =0 , P1 = 1  , P2 = 2

VAR

Byte cogRunning 'cog ID started is returned or -1 if not started

PUB main()

debug("--------------------------------------")

debug("MUXNC D,{#}S {WC/WZ/WCZ}")

debug("Mux !C into each D bit that is '1' in S.")

debug("D = (!S & D ) | (S & {32{ C}}). C = parity of result. *")

debug("--------------------------------------")

cogRunning := COGINIT(COGEXEC_NEW,@S0_MUXNC,0)

debug(udec(cogRunning))

repeat                                    'keep cog 0 running

DAT

ORG   0

S0_MUXNC              DRVH    #P0            'P0 on program running

MOV     Dest,valDest    '%1111_1111

MOV     Src,valSrc      '%1111_1110

MODCZ   _Set, _CLR  WCZ 'set Z = 0

IF_C   MOV     Carry,#1        'true  C = 1

IF_NC  MOV     Carry,#0        'false C = 0

IF_Z   MOV     Zero,#1         'true  Z = 1

IF_NZ  MOV     Zero,#0         'false Z = 0

debug("--------------------------------------")

debug("before MUXNC ",ubin(Dest),ubin(Src),ubin(Carry),ubin(Zero))

MUXNC    Dest,Src  WCZ

IF_C   MOV     Carry,#1         'true  C = 1

IF_NC  MOV     Carry,#0         'false C = 0

IF_Z   MOV     Zero,#1          'true  Z = 1

IF_NZ  MOV     Zero,#0          'false Z = 0

debug("after MUXC ",ubin(Dest),ubin(Src),ubin(Carry),ubin(Zero))

debug("--------------------------------------")

MOV     Dest,MIN       '0

MOV     Src,valSrc      '%1111_1110

MODCZ   _Clr, _CLR  WCZ 'set Z = 0

IF_C   MOV     Carry,#1        'true  C = 1

IF_NC  MOV     Carry,#0        'false C = 0

IF_Z   MOV     Zero,#1         'true  Z = 1

IF_NZ  MOV     Zero,#0         'false Z = 0

debug("--------------------------------------")

debug("before MUXNC ",ubin(Dest),ubin(Src),ubin(Carry),ubin(Zero))

MUXNC    Dest,Src  WCZ

IF_C    MOV     Carry,#1         'true  C = 1

IF_NC  MOV     Carry,#0         'false C = 0

IF_Z    MOV     Zero,#1          'true  Z = 1

IF_NZ  MOV     Zero,#0          'false Z = 0

debug("after MUXC ",ubin(Dest),ubin(Src),ubin(Carry),ubin(Zero))

debug("--------------------------------------")

_Loop1                NOP

JMP     #_Loop1          'remember # imediate

cog1Cnt         long      0

valDest           long      %1111_1111

valSrc              long      %1111_1110

Carry               long      0

Zero                long      0

Dest                long      0

Src                   long      0

MAX                long      $FFFF_FFFF

MIN                 long      0

### 17.15.3_Example_WRD_MUXZ_056

MUXZ D,{#}S {WC/WZ/WCZ}

Mux Z into each D bit that is '1' in S. D = (!S & D ) | (S & {32{ Z}}). C = parity of result. *

WC -Parity even if Dest has even number of 1 bits C == 0  Parity odd if Dest odd number bits C == 1

WZ-IF Destination result == 0 then Z =1 else Z = 0

{{17.15.3_Example_WRD_MUXZ_056}}

''MUXZ D,{#}S {WC/WZ/WCZ}

''Mux Z into each D bit that is '1' in S.

''D = (!S & D ) | (S & {32{ Z}}). C = parity of result. *

''WC -Parity even if Dest has even number of 1 bits C == 0

''    Parity odd if Dest odd number bits C == 1

''WZ-IF Destination result == 0 then Z = 1

''   IF Destination result != 0 then Z = 0

CON

_clkfreq = 200_000_000  ''Debug must be enabled clock must be greater than 10MHZ for Debug

P0 =0 , P1 = 1  , P2 = 2

VAR

Byte cogRunning 'cog ID started is returned or -1 if not started

PUB main()

debug("--------------------------------------")

debug("MUXZ D,{#}S {WC/WZ/WCZ}")

debug("Mux Z into each D bit that is '1' in S.")

debug("D = (!S & D ) | (S & {32{Z}}). C = parity of result. *")

debug("--------------------------------------")

cogRunning := COGINIT(COGEXEC_NEW,@S0_MUXZ,0)

debug(udec(cogRunning))

repeat                                    'keep cog 0 running

DAT

ORG   0

S0_MUXZ               DRVH    #P0            'P0 on program running

MOV     Dest,valDest    '%0000_00001

MOV     Src,valSrc      '%0000_1110

MODCZ   _Clr, _Set  WCZ 'set Z = 0

IF_C   MOV     Carry,#1        'true  C = 1

IF_NC  MOV     Carry,#0        'false C = 0

IF_Z   MOV     Zero,#1         'true  Z = 1

IF_NZ  MOV     Zero,#0         'false Z = 0

debug("--------------------------------------")

debug("before MUXZ ",ubin(Dest),ubin(Src),ubin(Carry),ubin(Zero))

MUXZ    Dest,Src  WCZ

IF_C   MOV     Carry,#1         'true  C = 1

IF_NC  MOV     Carry,#0         'false C = 0

IF_Z   MOV     Zero,#1          'true  Z = 1

IF_NZ  MOV     Zero,#0          'false Z = 0

debug("after MUXZ ",ubin(Dest),ubin(Src),ubin(Carry),ubin(Zero))

debug("--------------------------------------")

MOV     Dest,valDest    '%0000_0001

MOV     Src,valSrc      '%0000_1110

MODCZ   _Clr, _Clr  WCZ 'set Z = 0

IF_C   MOV     Carry,#1        'true  C = 1

IF_NC  MOV     Carry,#0        'false C = 0

IF_Z   MOV     Zero,#1         'true  Z = 1

IF_NZ  MOV     Zero,#0         'false Z = 0

debug("before MUXZ ",ubin(Dest),ubin(Src),ubin(Carry),ubin(Zero))

MUXZ    Dest,Src  WCZ

IF_C   MOV     Carry,#1         'true  C = 1

IF_NC  MOV     Carry,#0         'false C = 0

IF_Z   MOV     Zero,#1          'true  Z = 1

IF_NZ  MOV     Zero,#0          'false Z = 0

debug("after MUXZ ",ubin(Dest),ubin(Src),ubin(Carry),ubin(Zero))

debug("--------------------------------------")

_Loop1                NOP

JMP     #_Loop1          'remember # imediate

cog1Cnt               long      0

valDest                 long      %0000_0000

valSrc                    long      %0000_1110

Carry                     long      0

Zero                      long      0

Dest                      long      0

Src                        long      0

MAX                     long      $FFFF_FFFF

MIN                     long      0

### 17.15.4_Example_WRD_MUXNZ_057

MUXNZ D,{#}S {WC/WZ/WCZ}

Mux !Z into each D bit that is '1' in S. D = (!S & D ) | (S & {32{!Z}}). C = parity of result. *

{{17.15.4_Example_WRD_MUXNZ_056}}

''MUXNZ D,{#}S {WC/WZ/WCZ}

''Mux !Z into each D bit that is '1' in S.

''D = (!S & D ) | (S & {32{!Z}}). C = parity of result. *

''WC -Parity even if Dest has even number of 1 bits C == 0

''    Parity odd if Dest odd number bits C == 1

''WZ-IF Destination result == 0 then Z = 1

''   IF Destination result != 0 then Z = 0

CON

_clkfreq = 200_000_000  ''Debug must be enabled clock must be greater than 10MHZ for Debug

P0 =0 , P1 = 1  , P2 = 2

VAR

Byte cogRunning 'cog ID started is returned or -1 if not started

PUB main()

debug("--------------------------------------")

debug("MUXNZ D,{#}S {WC/WZ/WCZ}")

debug("Mux !Z into each D bit that is '1' in S.")

debug("D = (!S & D ) | (S & {32{!Z}}). C = parity of result. *")

debug("--------------------------------------")

cogRunning := COGINIT(COGEXEC_NEW,@S0_MUXNZ,0)

debug(udec(cogRunning))

repeat                                    'keep cog 0 running

DAT

ORG   0

S0_MUXNZ       DRVH    #P0            'P0 on program running

MOV     Dest,valDest    '%0000_00001

MOV     Src,valSrc      '%0000_1110

MODCZ   _Clr, _Set  WCZ 'set Z = 0

IF_C     MOV     Carry,#1        'true  C = 1

IF_NC  MOV     Carry,#0        'false C = 0

IF_Z     MOV     Zero,#1         'true  Z = 1

IF_NZ  MOV     Zero,#0         'false Z = 0

debug("--------------------------------------")

debug("before MUXNZ ",ubin(Dest),ubin(Src),ubin(Carry),ubin(Zero))

MUXNZ    Dest,Src  WCZ

IF_C     MOV     Carry,#1         'true  C = 1

IF_NC  MOV     Carry,#0         'false C = 0

IF_Z     MOV     Zero,#1          'true  Z = 1

IF_NZ  MOV     Zero,#0          'false Z = 0

debug("after MUXNZ ",ubin(Dest),ubin(Src),ubin(Carry),ubin(Zero))

debug("--------------------------------------")

MOV     Dest,valDest    '%0000_0001

MOV     Src,valSrc      '%0000_1110

MODCZ   _Clr, _Clr  WCZ 'set Z = 0

IF_C      MOV     Carry,#1        'true  C = 1

IF_NC    MOV     Carry,#0        'false C = 0

IF_Z      MOV     Zero,#1         'true  Z = 1

IF_NZ    MOV     Zero,#0         'false Z = 0

debug("before MUXNZ ",ubin(Dest),ubin(Src),ubin(Carry),ubin(Zero))

MUXNZ    Dest,Src  WCZ

IF_C      MOV     Carry,#1         'true  C = 1

IF_NC   MOV     Carry,#0         'false C = 0

IF_Z      MOV     Zero,#1          'true  Z = 1

IF_NZ   MOV     Zero,#0          'false Z = 0

debug("after MUXNZ ",ubin(Dest),ubin(Src),ubin(Carry),ubin(Zero))

debug("--------------------------------------")

_Loop1                NOP

JMP     #_Loop1          'remember # imediate

cog1Cnt            long      0

valDest             long      %0000_0000

valSrc                long      %0000_1110

Carry                 long      0

Zero                  long      0

Dest                  long      0

Src                     long      0

MAX                  long      $FFFF_FFFF

MIN                   long      0

## 17.16) MOV Move From Source to Destination Register

| MOV D,{#}S {WC/WZ/WCZ} | Move S into D. D = S. C = S[31]. * |
| --- | --- |

### 17.16.1_Example_WRD_MOV_058

MOV D,{#}S {WC/WZ/WCZ}

Move S into D. D = S. C = S[31]. *

{{17.16.1_Example_WRD_MOV_058}}

''MOV D,{#}S {WC/WZ/WCZ}

''Move S into D. D = S. C = S[31]. *

CON

_clkfreq = 200_000_000  ''Debug must be enabled clock must be greater than 10MHZ for Debug

P0 =0 , P1 = 1  , P2 = 2

VAR

Byte cogRunning 'cog ID started is returned or -1 if not started

PUB main()

debug("--------------------------------------")

debug("MOV D,{#}S {WC/WZ/WCZ} ")

debug("Move S into D. D = S. C = S[31]. * ")

debug("--------------------------------------")

cogRunning := COGINIT(COGEXEC_NEW,@S0_MOV,0)

debug(udec(cogRunning))

repeat                                    'keep cog 0 running

DAT

ORG   0

S0_MOV                DRVH    #P0             'P0 on program running

MOV     Dest,MAX        '$FFFF_FFFF

MOV     Src,MIN         '0

MODCZ   _Clr, _Clr  WCZ 'C = Z = 0

IF_C   MOV     Carry,#1        'true  C = 1

IF_NC  MOV     Carry,#0        'false C = 0

IF_Z   MOV     Zero,#1         'true  Z = 1

IF_NZ  MOV     Zero,#0         'false Z = 0

debug("--------------------------------------")

debug("before MOV ",ubin(Dest),ubin(Src),ubin(Carry),ubin(Zero))

MOV     Dest,Src  WCZ

IF_C   MOV     Carry,#1         'true  C = 1

IF_NC  MOV     Carry,#0         'false C = 0

IF_Z   MOV     Zero,#1          'true  Z = 1

IF_NZ  MOV     Zero,#0          'false Z = 0

debug("after MOV ",ubin(Dest),ubin(Src),ubin(Carry),ubin(Zero))

debug("--------------------------------------")

MOV     Dest,MIN        '0

MOV     Src,MAX         '$FFFF_FFFF

MODCZ   _Clr, _Clr  WCZ 'set Z = 0

IF_C   MOV     Carry,#1        'true  C = 1

IF_NC  MOV     Carry,#0        'false C = 0

IF_Z   MOV     Zero,#1         'true  Z = 1

IF_NZ  MOV     Zero,#0         'false Z = 0

debug("before MOV ",ubin(Dest),ubin(Src),ubin(Carry),ubin(Zero))

MOV    Dest,Src  WCZ

IF_C    MOV     Carry,#1         'true  C = 1

IF_NC  MOV     Carry,#0         'false C = 0

IF_Z    MOV     Zero,#1          'true  Z = 1

IF_NZ  MOV     Zero,#0          'false Z = 0

debug("after MOV ",uhex(Dest),uhex(Src),ubin(Carry),ubin(Zero))

debug("--------------------------------------")

_Loop1              NOP

JMP     #_Loop1          'remember # imediate

valDest               long      %0000_0000

valSrc                 long      %0000_1110

Carry                  long      0

Zero                   long      0

Dest                   long      0

Src                      long      0

MAX                   long      $FFFF_FFFF

MIN                    long      0

## 17.17) NOT Negate Destination Register

| NOT D,{#}S {WC/WZ/WCZ} | Get !S into D. D = !S. C = !S[31]. * |
| --- | --- |
| NOT D {WC/WZ/WCZ} | Get !D into D. D = !D. C = !D[31]. * |

17.16.1) NOT D,{#}S  Negate {#}S and Move into D

17.6.2) NOTD Negate register D

### 17.17.1_Example_WRD_NOTDS_059

NOT D,{#}S {WC/WZ/WCZ}

Get !S into D. D = !S. C = !S[31]. *

{{17.17.1_Example_WRD_NOTDS_059}}

''NOT D,{#}S {WC/WZ/WCZ}

''Get !S into D. D = !S. C = !S[31]. *

''WC- C = !S[31]

''WZ- IF Destination result == 0 then Z = 1

''    IF Destination result != 0 then Z = 0

CON

_clkfreq = 200_000_000  ''Debug must be enabled clock must be greater than 10MHZ for Debug

P0 =0 , P1 = 1  , P2 = 2

VAR

Byte cogRunning 'cog ID started is returned or -1 if not started

PUB main()

debug("--------------------------------------")

debug("MOV D,{#}S {WC/WZ/WCZ} ")

debug("Move S into D. D = S. C = S[31]. * ")

debug("--------------------------------------")

cogRunning := COGINIT(COGEXEC_NEW,@S0_NOTDS,0)

debug(udec(cogRunning))

repeat                                    'keep cog 0 running

DAT

ORG   0

S0_NOTDS              DRVH    #P0             'P0 on program running

MOV     Dest,MAX        '$FFFF_FFFF

MOV     Src,MAX         '$FFFF_FFFF

MODCZ   _Clr, _Clr  WCZ 'C = Z = 0

IF_C   MOV     Carry,#1        'true  C = 1

IF_NC  MOV     Carry,#0        'false C = 0

IF_Z   MOV     Zero,#1         'true  Z = 1

IF_NZ  MOV     Zero,#0         'false Z = 0

debug("--------------------------------------")

debug("before NOT ",ubin(Dest),ubin(Src),ubin(Carry),ubin(Zero))

NOT     Dest,Src  WCZ

IF_C   MOV     Carry,#1         'true  C = 1

IF_NC  MOV     Carry,#0         'false C = 0

IF_Z   MOV     Zero,#1          'true  Z = 1

IF_NZ  MOV     Zero,#0          'false Z = 0

debug("after NOT ",ubin(Dest),ubin(Src),ubin(Carry),ubin(Zero))

debug("--------------------------------------")

MOV     Dest,MIN        '0

MOV     Src,MAX         '$FFFF_FFFF

MODCZ   _Clr, _Clr  WCZ 'set Z = 0

IF_C   MOV     Carry,#1        'true  C = 1

IF_NC  MOV     Carry,#0        'false C = 0

IF_Z   MOV     Zero,#1         'true  Z = 1

IF_NZ  MOV     Zero,#0         'false Z = 0

debug("before NOT ",ubin(Dest),ubin(Src),ubin(Carry),ubin(Zero))

NOT    Dest,Src  WCZ

IF_C   MOV     Carry,#1         'true  C = 1

IF_NC  MOV     Carry,#0         'false C = 0

IF_Z   MOV     Zero,#1          'true  Z = 1

IF_NZ  MOV     Zero,#0          'false Z = 0

debug("after NOT ",uhex(Dest),uhex(Src),ubin(Carry),ubin(Zero))

debug("--------------------------------------")

_Loop1                NOP

JMP     #_Loop1          'remember # imediate

valDest               long      %0000_0000

valSrc                long      %0000_1110

Carry                 long      0

Zero                  long      0

Dest                  long      0

Src                   long      0

MAX                   long      $FFFF_FFFF

MIN                   long      0

### 17.17.2_Example_WRD_NOTD_060

NOT D {WC/WZ/WCZ}

Get !D into D. D = !D. C = !D[31]. *

{{17.17.2_Example_WRD_NOTD_060}}

''NOT D {WC/WZ/WCZ}

''Get !D into D. D = !D. C = !D[31]. *

''WC- C = !D[31]

''WZ- IF Destination result == 0 then Z = 1

''    IF Destination result != 0 then Z = 0

CON

_clkfreq = 200_000_000  ''Debug must be enabled clock must be greater than 10MHZ for Debug

P0 =0 , P1 = 1  , P2 = 2

VAR

Byte cogRunning 'cog ID started is returned or -1 if not started

PUB main()

debug("--------------------------------------")

debug("MOV D {WC/WZ/WCZ} ")

debug("Get !D into D. D = !D. C = !D. * ")

debug("--------------------------------------")

cogRunning := COGINIT(COGEXEC_NEW,@S0_NOTD,0)

debug(udec(cogRunning))

repeat                                    'keep cog 0 running

DAT

ORG   0

S0_NOTD               DRVH    #P0             'P0 on program running

MOV     Dest,MAX        '$FFFF_FFFF

MOV     Src,MAX         '$FFFF_FFFF

MODCZ   _Clr, _Clr  WCZ 'C = Z = 0

IF_C   MOV     Carry,#1        'true  C = 1

IF_NC  MOV     Carry,#0        'false C = 0

IF_Z   MOV     Zero,#1         'true  Z = 1

IF_NZ  MOV     Zero,#0         'false Z = 0

debug("--------------------------------------")

debug("before NOT ",ubin(Dest),ubin(Src),ubin(Carry),ubin(Zero))

NOT     Dest        WCZ

IF_C   MOV     Carry,#1         'true  C = 1

IF_NC  MOV     Carry,#0         'false C = 0

IF_Z   MOV     Zero,#1          'true  Z = 1

IF_NZ  MOV     Zero,#0          'false Z = 0

debug("after NOT ",ubin(Dest),ubin(Src),ubin(Carry),ubin(Zero))

debug("--------------------------------------")

MOV     Dest,MIN        '0

MODCZ   _Clr, _Clr  WCZ 'set Z = 0

IF_C   MOV     Carry,#1        'true  C = 1

IF_NC  MOV     Carry,#0        'false C = 0

IF_Z   MOV     Zero,#1         'true  Z = 1

IF_NZ  MOV     Zero,#0         'false Z = 0

debug("before NOT ",ubin(Dest),ubin(Carry),ubin(Zero))

NOT    Dest         WCZ

IF_C     MOV     Carry,#1         'true  C = 1

IF_NC   MOV     Carry,#0         'false C = 0

IF_Z      MOV     Zero,#1          'true  Z = 1

IF_NZ   MOV     Zero,#0          'false Z = 0

debug("after NOT ",uhex(Dest),ubin(Carry),ubin(Zero))

debug("--------------------------------------")

_Loop1               NOP

JMP     #_Loop1          'remember # imediate

valDest              long      %0000_0000

valSrc                long      %0000_1110

Carry                 long      0

Zero                  long      0

Dest                  long      0

Src                     long      0

MAX                  long      $FFFF_FFFF

MIN                   long      0

## 17.18) ABS Absolute Value

| ABS D,{#}S {WC/WZ/WCZ} | Get absolute value of S into D. D = ABS(S). C = S[31]. * |
| --- | --- |
| ABS D {WC/WZ/WCZ} | Get absolute value of D into D. D = ABS(D). C = D[31]. * |

### 17.18.1_Example_WRD_ABSDS_061

ABS D,{#}S {WC/WZ/WCZ}

Get absolute value of S into D. D = ABS(S). C = S[31]. *

{{17.18.1_Example_WRD_ABSDS_061}}

''ABS D,{#}S {WC/WZ/WCZ}

''Get absolute value of S into D. D = ABS(S). C = S[31]. *

''WC- C = S[31]

''WZ- IF Destination result == 0 then Z = 1

''    IF Destination result != 0 then Z = 0

CON

_clkfreq = 200_000_000  ''Debug must be enabled clock must be greater than 10MHZ for Debug

P0 =0 , P1 = 1  , P2 = 2

VAR

Byte cogRunning 'cog ID started is returned or -1 if not started

PUB main()

debug("--------------------------------------")

debug("ABS D,{#}S {WC/WZ/WCZ}")

debug("Get absolute value of S into D. D = ABS(S). C = S[31]. *")

debug("--------------------------------------")

cogRunning := COGINIT(COGEXEC_NEW,@S0_ABSDS,0)

debug(udec(cogRunning))

repeat                                    'keep cog 0 running

DAT

ORG   0

S0_ABSDS              DRVH    #P0             'P0 on program running

MOV     Dest,MAX        '$FFFF_FFFF

MOV     Src,MAX         '$FFFF_FFFF

MODCZ   _Clr, _Clr  WCZ 'C = Z = 0

IF_C   MOV     Carry,#1        'true  C = 1

IF_NC  MOV     Carry,#0        'false C = 0

IF_Z   MOV     Zero,#1         'true  Z = 1

IF_NZ  MOV     Zero,#0         'false Z = 0

debug("--------------------------------------")

debug("before ABS ",ubin(Dest),ubin(Src),ubin(Carry),ubin(Zero))

ABS     Dest,Src    WCZ

IF_C   MOV     Carry,#1         'true  C = 1

IF_NC  MOV     Carry,#0         'false C = 0

IF_Z   MOV     Zero,#1          'true  Z = 1

IF_NZ  MOV     Zero,#0          'false Z = 0

debug("after ABS ",ubin(Dest),ubin(Src),ubin(Carry),ubin(Zero))

debug("--------------------------------------")

MOV     Dest,MIN        '0

MOV     Src,valSrc      '-2

MODCZ   _Clr, _Clr  WCZ 'set Z = 0

IF_C   MOV     Carry,#1        'true  C = 1

IF_NC  MOV     Carry,#0        'false C = 0

IF_Z   MOV     Zero,#1         'true  Z = 1

IF_NZ  MOV     Zero,#0         'false Z = 0

debug("before ABS ",ubin(Dest),ubin(Src),ubin(Carry),ubin(Zero))

ABS     Dest,Src    WCZ

IF_C   MOV     Carry,#1         'true  C = 1

IF_NC  MOV     Carry,#0         'false C = 0

IF_Z   MOV     Zero,#1          'true  Z = 1

IF_NZ  MOV     Zero,#0          'false Z = 0

debug("after ABS ",uhex(Dest),ubin(Src),ubin(Carry),ubin(Zero))

debug("--------------------------------------")

_Loop1                NOP

JMP     #_Loop1          'remember # imediate

valDest             long      %0000_0000

valSrc                long      -2

Carry                 long      0

Zero                  long      0

Dest                  long      0

Src                     long      0

MAX                  long      $FFFF_FFFF

MIN                   long      0

### 17.18.2_Example_WRD_ABSD_062

ABS D {WC/WZ/WCZ}

Get absolute value of D into D. D = ABS(D). C = D[31]. *

{{17.18.2_Example_WRD_ABSD_062}}

''ABS D {WC/WZ/WCZ}

''Get absolute value of D into D. D = ABS(D). C = D[31]. *

''WC- C = S[31]

''WZ- IF Destination result == 0 then Z = 1

''    IF Destination result != 0 then Z = 0

CON

_clkfreq = 200_000_000  ''Debug must be enabled clock must be greater than 10MHZ for Debug

P0 =0 , P1 = 1  , P2 = 2

VAR

Byte cogRunning 'cog ID started is returned or -1 if not started

PUB main()

debug("--------------------------------------")

debug("ABS D,{#}S {WC/WZ/WCZ}")

debug("Get absolute value of S into D. D = ABS(S). C = S[31]. *")

debug("--------------------------------------")

cogRunning := COGINIT(COGEXEC_NEW,@S0_ABSD,0)

debug(udec(cogRunning))

repeat                                    'keep cog 0 running

DAT

ORG   0

S0_ABSD               DRVH    #P0             'P0 on program running

MOV     Dest,MIN        '0

MOV     Src,MAX         '$FFFF_FFFF

MODCZ   _Clr, _Clr  WCZ 'C = Z = 0

IF_C   MOV     Carry,#1        'true  C = 1

IF_NC  MOV     Carry,#0        'false C = 0

IF_Z   MOV     Zero,#1         'true  Z = 1

IF_NZ  MOV     Zero,#0         'false Z = 0

debug("--------------------------------------")

debug("before ABS ",ubin(Dest),sdec(Src),ubin(Carry),ubin(Zero))

ABS     Dest,Src    WCZ

IF_C   MOV     Carry,#1         'true  C = 1

IF_NC  MOV     Carry,#0         'false C = 0

IF_Z   MOV     Zero,#1          'true  Z = 1

IF_NZ  MOV     Zero,#0          'false Z = 0

debug("after ABS ",ubin(Dest),ubin(Src),ubin(Carry),ubin(Zero))

debug("--------------------------------------")

MOV     Dest,valNeg2    '-2

MODCZ   _Clr, _Clr  WCZ 'set Z = 0

IF_C   MOV     Carry,#1        'true  C = 1

IF_NC  MOV     Carry,#0        'false C = 0

IF_Z   MOV     Zero,#1         'true  Z = 1

IF_NZ  MOV     Zero,#0         'false Z = 0

debug("before ABS ",sdec(Dest),ubin(Carry),ubin(Zero))

ABS     Dest        WCZ

IF_C    MOV     Carry,#1         'true  C = 1

IF_NC  MOV     Carry,#0         'false C = 0

IF_Z    MOV     Zero,#1          'true  Z = 1

IF_NZ  MOV     Zero,#0          'false Z = 0

debug("after ABS ",sdec(Dest),ubin(Carry),ubin(Zero))

debug("--------------------------------------")

_Loop1              NOP

JMP     #_Loop1          'remember # imediate

valDest              long      %0000_0000

valSrc                long      0

valNeg2            long      -2

Carry                 long      0

Zero                  long      0

Dest                  long      0

Src                     long      0

MAX                   long      $FFFF_FFFF

MIN                   long      0

## 17.19) NEG Negate Negative

| NEG D,{#}S {WC/WZ/WCZ} | Negate S into D. D = -S. C = MSB of result. * |
| --- | --- |
| NEG D {WC/WZ/WCZ} | Negate D. D = -D. C = MSB of result. * |
| NEGC D,{#}S {WC/WZ/WCZ} | Negate S by C into D. If C = 1 then D = -S, else D = S. C = MSB of result. * |
| NEGC D {WC/WZ/WCZ} | Negate D by C. If C = 1 then D = -D, else D = D. C = MSB of result. * |
| NEGNC D,{#}S {WC/WZ/WCZ} | Negate S by !C into D. If C = 0 then D = -S, else D = S. C = MSB of result. * |
| NEGNC D {WC/WZ/WCZ} | Negate D by !C. If C = 0 then D = -D, else D = D. C = MSB of result. * |
| NEGZ D,{#}S {WC/WZ/WCZ} | Negate S by Z into D. If Z = 1 then D = -S, else D = S. C = MSB of result. * |
| NEGZ D {WC/WZ/WCZ} | Negate D by Z. If Z = 1 then D = -D, else D = D. C = MSB of result. * |
| NEGNZ D,{#}S {WC/WZ/WCZ} | Negate S by !Z into D. If Z = 0 then D = -S, else D = S. C = MSB of result. * |
| NEGNZ D {WC/WZ/WCZ} | Negate D by !Z. If Z = 0 then D = -D, else D = D. C = MSB of result. * |

### 17.19.1_Example_WRD_NEGDS_063

NEG D,{#}S {WC/WZ/WCZ}

Negate S into D. D = -S. C = MSB of result. *

{{17.19.1_Example_WRD_NEGDS_063}}

''NEG D,{#}S {WC/WZ/WCZ}

''Negate S into D. D = -S. C = MSB of result. *

''WC- C = MSB

''WZ- IF Destination result == 0 then Z = 1

''    IF Destination result != 0 then Z = 0

CON

_clkfreq = 200_000_000  ''Debug must be enabled clock must be greater than 10MHZ for Debug

P0 =0 , P1 = 1  , P2 = 2

VAR

Byte cogRunning 'cog ID started is returned or -1 if not started

PUB main()

debug("--------------------------------------")

debug("NEG D,{#}S {WC/WZ/WCZ}")

debug("Negate S into D. D = -S. C = MSB of result. *")

debug("--------------------------------------")

cogRunning := COGINIT(COGEXEC_NEW,@S0_NEGDS,0)

debug(udec(cogRunning))

repeat                                    'keep cog 0 running

DAT

ORG   0

S0_NEGDS              DRVH    #P0             'P0 on program running

MOV     Dest,MIN        '0

MOV     Src,MIN         '0

MODCZ   _Clr, _Clr  WCZ 'C = Z = 0

IF_C      MOV     Carry,#1        'true  C = 1

IF_NC    MOV     Carry,#0        'false C = 0

IF_Z      MOV     Zero,#1         'true  Z = 1

IF_NZ   MOV     Zero,#0         'false Z = 0

debug("--------------------------------------")

debug("before NEG ",ubin(Dest),sdec(Src),ubin(Carry),ubin(Zero))

NEG     Dest,Src    WCZ

IF_C      MOV     Carry,#1         'true  C = 1

IF_NC   MOV     Carry,#0         'false C = 0

IF_Z      MOV     Zero,#1          'true  Z = 1

IF_NZ   MOV     Zero,#0          'false Z = 0

debug("after NEG ",ubin(Dest),ubin(Src),ubin(Carry),ubin(Zero))

debug("--------------------------------------")

MOV     Dest,MIN        '0

MOV     Src,valNeg2     '-2

MODCZ   _Clr, _Clr  WCZ 'set Z = 0

IF_C     MOV     Carry,#1        'true  C = 1

IF_NC  MOV     Carry,#0        'false C = 0

IF_Z      MOV     Zero,#1         'true  Z = 1

IF_NZ   MOV     Zero,#0         'false Z = 0

debug("before NEG ",sdec(Dest),sdec(Src),ubin(Carry),ubin(Zero))

NEG     Dest,Src    WCZ

IF_C       MOV     Carry,#1         'true  C = 1

IF_NC    MOV     Carry,#0         'false C = 0

IF_Z       MOV     Zero,#1          'true  Z = 1

IF_NZ    MOV     Zero,#0          'false Z = 0

debug("after NEG ",sdec(Dest),sdec(Src),ubin(Carry),ubin(Zero))

debug("--------------------------------------")

_Loop1                NOP

JMP     #_Loop1          'remember # imediate

valDest            long      %0000_0000

valSrc               long      0

valNeg2           long      -2

Carry                 long      0

Zero                  long      0

Dest                  long      0

Src                    long      0

MAX                  long      $FFFF_FFFF

MIN                   long      0

### 17.19.2_Example_WRD_NEGD_064

NEG D {WC/WZ/WCZ}

Negate D. D = -D. C = MSB of result. *

{{17.19.2_Example_WRD_NEGD_064}}

''NEG D {WC/WZ/WCZ}

''Negate D. D = -D. C = MSB of result. *

''WC- C = MSB

''WZ- IF Destination result == 0 then Z = 1

''    IF Destination result != 0 then Z = 0

CON

_clkfreq = 200_000_000  ''Debug must be enabled clock must be greater than 10MHZ for Debug

P0 =0 , P1 = 1  , P2 = 2

VAR

Byte cogRunning 'cog ID started is returned or -1 if not started

PUB main()

debug("--------------------------------------")

debug("NEG D {WC/WZ/WCZ}")

debug("Negate D. D = -D. C = MSB of result. *")

debug("--------------------------------------")

cogRunning := COGINIT(COGEXEC_NEW,@S0_NEGD,0)

debug(udec(cogRunning))

repeat                                    'keep cog 0 running

DAT

ORG   0

S0_NEGD             DRVH    #P0             'P0 on program running

MOV     Dest,#1         '1

MODCZ   _Clr, _Clr  WCZ 'C = Z = 0

IF_C       MOV     Carry,#1        'true  C = 1

IF_NC    MOV     Carry,#0        'false C = 0

IF_Z      MOV     Zero,#1         'true  Z = 1

IF_NZ    MOV     Zero,#0         'false Z = 0

debug("--------------------------------------")

debug("before NEG ",ubin(Dest),ubin(Carry),ubin(Zero))

NEG     Dest        WCZ

IF_C        MOV     Carry,#1         'true  C = 1

IF_NC     MOV     Carry,#0         'false C = 0

IF_Z       MOV     Zero,#1          'true  Z = 1

IF_NZ    MOV     Zero,#0          'false Z = 0

debug("after NEG ",sdec(Dest),ubin(Carry),ubin(Zero))

debug("--------------------------------------")

MOV     Dest,MAX        '-1

MODCZ   _Clr, _Clr  WCZ 'set Z = 0

IF_C     MOV     Carry,#1        'true  C = 1

IF_NC  MOV     Carry,#0        'false C = 0

IF_Z       MOV     Zero,#1         'true  Z = 1

IF_NZ     MOV     Zero,#0         'false Z = 0

debug("before NEG ",sdec(Dest),ubin(Carry),ubin(Zero))

NEG     Dest        WCZ

IF_C      MOV     Carry,#1         'true  C = 1

IF_NC   MOV     Carry,#0         'false C = 0

IF_Z      MOV     Zero,#1          'true  Z = 1

IF_NZ   MOV     Zero,#0          'false Z = 0

debug("after NEG ",sdec(Dest),ubin(Carry),ubin(Zero))

debug("--------------------------------------")

_Loop1                NOP

JMP     #_Loop1          'remember # imediate

valDest             long      %0000_0000

valSrc                long      0

valNeg2            long      -2

Carry                 long      0

Zero                  long      0

Dest                  long      0

Src                     long      0

MAX                  long      $FFFF_FFFF

MIN                   long      0

### 17.19.3_Example_WRD_NEGCDS_065

NEGC D,{#}S {WC/WZ/WCZ}

Negate S by C into D. If C = 1 then D = -S, else D = S. C = MSB of result. *

{{17.19.3_Example_WRD_NEGCDS_065}}

''NEGC D,{#}S {WC/WZ/WCZ}

''Negate S by C into D. If C = 1 then D = -S, else D = S. C = MSB of result. *

''WC- C = MSB

''WZ- IF Destination result == 0 then Z = 1

''    IF Destination result != 0 then Z = 0

CON

_clkfreq = 200_000_000  ''Debug must be enabled clock must be greater than 10MHZ for Debug

P0 =0 , P1 = 1  , P2 = 2

VAR

Byte cogRunning 'cog ID started is returned or -1 if not started

PUB main()

debug("--------------------------------------")

debug("NEGC D,{#}S {WC/WZ/WCZ}")

debug("Negate S by C into D. If C = 1 then D = -S, else D = S. C = MSB of result. *")

debug("--------------------------------------")

cogRunning := COGINIT(COGEXEC_NEW,@S0_NEGCDS,0)

debug(udec(cogRunning))

repeat                                    'keep cog 0 running

DAT

ORG   0

S0_NEGCDS    DRVH    #P0             'P0 on program running

MOV     Dest,valNeg2

MOV     Src,valNeg1

MODCZ   _Clr, _Clr  WCZ 'C = Z = 0

IF_C   MOV     Carry,#1        'true  C = 1

IF_NC  MOV     Carry,#0        'false C = 0

IF_Z   MOV     Zero,#1         'true  Z = 1

IF_NZ  MOV     Zero,#0         'false Z = 0

debug("--------------------------------------")

debug("before NEGC ",sdec(Dest),sdec(Src),ubin(Carry),ubin(Zero))

NEGC    Dest,Src     WCZ

IF_C   MOV     Carry,#1         'true  C = 1

IF_NC  MOV     Carry,#0         'false C = 0

IF_Z   MOV     Zero,#1          'true  Z = 1

IF_NZ  MOV     Zero,#0          'false Z = 0

debug("after NEGC ",sdec(Dest),sdec(Src),ubin(Carry),ubin(Zero))

debug("--------------------------------------")

MOV     Dest,valNeg2

MOV     Src,valNeg1

MODCZ   _Set, _Clr  WCZ 'set C = 1 Z = 0

IF_C   MOV     Carry,#1        'true  C = 1

IF_NC  MOV     Carry,#0        'false C = 0

IF_Z   MOV     Zero,#1         'true  Z = 1

IF_NZ  MOV     Zero,#0         'false Z = 0

debug("before NEGC ",sdec(Dest),sdec(Src),ubin(Carry),ubin(Zero))

NEGC    Dest,Src    WCZ

IF_C   MOV     Carry,#1         'true  C = 1

IF_NC  MOV     Carry,#0         'false C = 0

IF_Z   MOV     Zero,#1          'true  Z = 1

IF_NZ  MOV     Zero,#0          'false Z = 0

debug("after NEGC ",udec(Dest),sdec(Src),ubin(Carry),ubin(Zero))

debug("--------------------------------------")

MOV     Dest,valNeg1

MOV     Src,Min

MODCZ   _Set, _Clr  WCZ 'set C = 1 Z = 0

IF_C   MOV     Carry,#1        'true  C = 1

IF_NC  MOV     Carry,#0        'false C = 0

IF_Z   MOV     Zero,#1         'true  Z = 1

IF_NZ  MOV     Zero,#0         'false Z = 0

debug("before NEGC ",sdec(Dest),sdec(Src),ubin(Carry),ubin(Zero))

NEGC    Dest,Src    WCZ

IF_C   MOV     Carry,#1         'true  C = 1

IF_NC  MOV     Carry,#0         'false C = 0

IF_Z   MOV     Zero,#1          'true  Z = 1

IF_NZ  MOV     Zero,#0          'false Z = 0

debug("after NEGC ",udec(Dest),sdec(Src),ubin(Carry),ubin(Zero))

debug("--------------------------------------")

_Loop1             NOP

JMP     #_Loop1          'remember # imediate

valDest               long      %0000_0000

valSrc                  long      0

valNeg1              long      -1

valNeg2              long      -2

Carry                   long      0

Zero                    long      0

Dest                    long      0

Src                      long      0

MAX                   long      $FFFF_FFFF

MIN                   long      0

### 17.19.4_Example_WRD_NEGCD_066

NEGC D {WC/WZ/WCZ}

Negate D by C. If C = 1 then D = -D, else D = D. C = MSB of result. *

{{17.19.4_Example_WRD_NEGCD_066}}

''NEGC D {WC/WZ/WCZ}

''Negate D by C. If C = 1 then D = -D, else D = D. C = MSB of result. *

''WC- C = MSB

''WZ- IF Destination result == 0 then Z = 1

''    IF Destination result != 0 then Z = 0

CON

_clkfreq = 200_000_000  ''Debug must be enabled clock must be greater than 10MHZ for Debug

P0 =0 , P1 = 1  , P2 = 2

VAR

Byte cogRunning 'cog ID started is returned or -1 if not started

PUB main()

debug("--------------------------------------")

debug("NEGC D {WC/WZ/WCZ}")

debug("Negate D by C.If C = 1 then D = -D, else D = S. C = MSB of result. *")

debug("--------------------------------------")

cogRunning := COGINIT(COGEXEC_NEW,@S0_NEGCD,0)

debug(udec(cogRunning))

repeat                                    'keep cog 0 running

DAT

ORG   0

S0_NEGCD              DRVH    #P0             'P0 on program running

MOV     Dest,valNeg2

MODCZ   _Clr, _Clr  WCZ 'C = Z = 0

IF_C   MOV     Carry,#1        'true  C = 1

IF_NC  MOV     Carry,#0        'false C = 0

IF_Z   MOV     Zero,#1         'true  Z = 1

IF_NZ  MOV     Zero,#0         'false Z = 0

debug("--------------------------------------")

debug("before NEGC ",sdec(Dest),ubin(Carry),ubin(Zero))

NEGC    Dest         WCZ

IF_C   MOV     Carry,#1         'true  C = 1

IF_NC  MOV     Carry,#0         'false C = 0

IF_Z   MOV     Zero,#1          'true  Z = 1

IF_NZ  MOV     Zero,#0          'false Z = 0

debug("after NEGC  ",sdec(Dest),ubin(Carry),ubin(Zero))

debug("--------------------------------------")

MOV     Dest,valNeg2

MODCZ   _Set, _Clr  WCZ 'set C = 1 Z = 0

IF_C   MOV     Carry,#1        'true  C = 1

IF_NC  MOV     Carry,#0        'false C = 0

IF_Z   MOV     Zero,#1         'true  Z = 1

IF_NZ  MOV     Zero,#0         'false Z = 0

debug("before NEGC ",sdec(Dest),ubin(Carry),ubin(Zero))

NEGC    Dest         WCZ

IF_C   MOV     Carry,#1         'true  C = 1

IF_NC  MOV     Carry,#0         'false C = 0

IF_Z   MOV     Zero,#1          'true  Z = 1

IF_NZ  MOV     Zero,#0          'false Z = 0

debug("after NEGC  ",udec(Dest),ubin(Carry),ubin(Zero))

debug("--------------------------------------")

MOV     Dest,MIN

MODCZ   _Set, _Clr  WCZ 'set C = 1 Z = 0

IF_C   MOV     Carry,#1        'true  C = 1

IF_NC  MOV     Carry,#0        'false C = 0

IF_Z   MOV     Zero,#1         'true  Z = 1

IF_NZ  MOV     Zero,#0         'false Z = 0

debug("before NEGC ",sdec(Dest),ubin(Carry),ubin(Zero))

NEGC    Dest         WCZ

IF_C   MOV     Carry,#1         'true  C = 1

IF_NC  MOV     Carry,#0         'false C = 0

IF_Z   MOV     Zero,#1          'true  Z = 1

IF_NZ  MOV     Zero,#0          'false Z = 0

debug("after NEGC  ",udec(Dest),ubin(Carry),ubin(Zero))

debug("--------------------------------------")

_Loop1               NOP

JMP     #_Loop1          'remember # imediate

valDest             long      %0000_0000

valSrc                long      0

valNeg1            long      -1

valNeg2            long      -2

Carry                 long      0

Zero                  long      0

Dest                  long      0

Src                     long      0

MAX                  long      $FFFF_FFFF

MIN                   long      0

### 17.19.5_Example_WRD_NEGNCDS_067

NEGNC D,{#}S {WC/WZ/WCZ}

Negate S by !C into D. If C = 0 then D = -S, else D = S. C = MSB of result. *

{{17.19.5_Example_WRD_NEGNCDS_067}}

''NEGNC D,{#}S {WC/WZ/WCZ}

''Negate S by !C into D. If C = 0 then D = -S, else D = S. C = MSB of result. *

''WC- C = MSB

''WZ- IF Destination result == 0 then Z = 1

''    IF Destination result != 0 then Z = 0

CON

_clkfreq = 200_000_000  ''Debug must be enabled clock must be greater than 10MHZ for Debug

P0 =0 , P1 = 1  , P2 = 2

VAR

Byte cogRunning 'cog ID started is returned or -1 if not started

PUB main()

debug("--------------------------------------")

debug("NEGNC D,{#}S {WC/WZ/WCZ}")

debug("Negate S by !C into D. If C = 0 then D = -S, else D = S. C = MSB of result. *")

debug("--------------------------------------")

cogRunning := COGINIT(COGEXEC_NEW,@S0_NEGNCDS,0)

debug(udec(cogRunning))

repeat                                    'keep cog 0 running

DAT

ORG   0

S0_NEGNCDS            DRVH    #P0             'P0 on program running

MOV     Dest,valNeg2

MOV     Src,valNeg1

MODCZ   _Clr, _Clr  WCZ 'C = Z = 0

IF_C   MOV     Carry,#1        'true  C = 1

IF_NC  MOV     Carry,#0        'false C = 0

IF_Z   MOV     Zero,#1         'true  Z = 1

IF_NZ  MOV     Zero,#0         'false Z = 0

debug("--------------------------------------")

debug("before NEGNC ",sdec(Dest),sdec(Src),ubin(Carry),ubin(Zero))

NEGNC   Dest,Src         WCZ

IF_C   MOV     Carry,#1         'true  C = 1

IF_NC  MOV     Carry,#0         'false C = 0

IF_Z   MOV     Zero,#1          'true  Z = 1

IF_NZ  MOV     Zero,#0          'false Z = 0

debug("after NEGNC  ",sdec(Dest),sdec(Src),ubin(Carry),ubin(Zero))

debug("--------------------------------------")

MOV     Dest,valNeg2

MOV     Src,valNeg1

MODCZ   _Set, _Clr  WCZ 'set C = 1 Z = 0

IF_C   MOV     Carry,#1        'true  C = 1

IF_NC  MOV     Carry,#0        'false C = 0

IF_Z   MOV     Zero,#1         'true  Z = 1

IF_NZ  MOV     Zero,#0         'false Z = 0

debug("before NEGNC ",sdec(Dest),sdec(Src),ubin(Carry),ubin(Zero))

NEGNC    Dest,Src    WCZ

IF_C   MOV     Carry,#1         'true  C = 1

IF_NC  MOV     Carry,#0         'false C = 0

IF_Z   MOV     Zero,#1          'true  Z = 1

IF_NZ  MOV     Zero,#0          'false Z = 0

debug("after NEGNC  ",sdec(Dest),sdec(Src),ubin(Carry),ubin(Zero))

debug("--------------------------------------")

MOV     Dest,valNeg2

MOV     Src,MIN

MODCZ   _Clr, _Clr  WCZ 'set C = 1 Z = 0

IF_C   MOV     Carry,#1        'true  C = 1

IF_NC  MOV     Carry,#0        'false C = 0

IF_Z   MOV     Zero,#1         'true  Z = 1

IF_NZ  MOV     Zero,#0         'false Z = 0

debug("before NEGNC ",sdec(Dest),sdec(Src),ubin(Carry),ubin(Zero))

NEGNC    Dest,Src     WCZ

IF_C   MOV     Carry,#1         'true  C = 1

IF_NC  MOV     Carry,#0         'false C = 0

IF_Z   MOV     Zero,#1          'true  Z = 1

IF_NZ  MOV     Zero,#0          'false Z = 0

debug("after NEGNC  ",sdec(Dest),sdec(Src),ubin(Carry),ubin(Zero))

debug("--------------------------------------")

_Loop1                NOP

JMP     #_Loop1          'remember # imediate

valDest               long      %0000_0000

valSrc                  long      0

valNeg1              long      -1

valNeg2              long      -2

Carry                   long      0

Zero                    long      0

Dest                    long      0

Src                      long      0

MAX                   long      $FFFF_FFFF

MIN                   long      0

### 17.19.6_Example_WRD_NEGNCD_068

NEGNC D {WC/WZ/WCZ}

Negate D by !C. If C = 0 then D = -D, else D = D. C = MSB of result. *

{{17.19.5_Example_WRD_NEGNCDS_067}}

''NEGNC D,{#}S {WC/WZ/WCZ}

''Negate S by !C into D. If C = 0 then D = -S, else D = S. C = MSB of result. *

''WC- C = MSB

''WZ- IF Destination result == 0 then Z = 1

''    IF Destination result != 0 then Z = 0

CON

_clkfreq = 200_000_000  ''Debug must be enabled clock must be greater than 10MHZ for Debug

P0 =0 , P1 = 1  , P2 = 2

VAR

Byte cogRunning 'cog ID started is returned or -1 if not started

PUB main()

debug("--------------------------------------")

debug("NEGNC D,{#}S {WC/WZ/WCZ}")

debug("Negate S by !C into D. If C = 0 then D = -S, else D = S. C = MSB of result. *")

debug("--------------------------------------")

cogRunning := COGINIT(COGEXEC_NEW,@S0_NEGNCDS,0)

debug(udec(cogRunning))

repeat                                    'keep cog 0 running

DAT

ORG   0

S0_NEGNCDS            DRVH    #P0             'P0 on program running

MOV     Dest,valNeg2

MOV     Src,valNeg1

MODCZ   _Clr, _Clr  WCZ 'C = Z = 0

IF_C   MOV     Carry,#1        'true  C = 1

IF_NC  MOV     Carry,#0        'false C = 0

IF_Z   MOV     Zero,#1         'true  Z = 1

IF_NZ  MOV     Zero,#0         'false Z = 0

debug("--------------------------------------")

debug("before NEGNC ",sdec(Dest),sdec(Src),ubin(Carry),ubin(Zero))

NEGNC   Dest,Src         WCZ

IF_C   MOV     Carry,#1         'true  C = 1

IF_NC  MOV     Carry,#0         'false C = 0

IF_Z   MOV     Zero,#1          'true  Z = 1

IF_NZ  MOV     Zero,#0          'false Z = 0

debug("after NEGNC  ",sdec(Dest),sdec(Src),ubin(Carry),ubin(Zero))

debug("--------------------------------------")

MOV     Dest,valNeg2

MOV     Src,valNeg1

MODCZ   _Set, _Clr  WCZ 'set C = 1 Z = 0

IF_C     MOV     Carry,#1        'true  C = 1

IF_NC   MOV     Carry,#0        'false C = 0

IF_Z     MOV     Zero,#1         'true  Z = 1

IF_NZ   MOV     Zero,#0         'false Z = 0

debug("before NEGNC ",sdec(Dest),sdec(Src),ubin(Carry),ubin(Zero))

NEGNC    Dest,Src    WCZ

IF_C      MOV     Carry,#1         'true  C = 1

IF_NC   MOV     Carry,#0         'false C = 0

IF_Z      MOV     Zero,#1          'true  Z = 1

IF_NZ   MOV     Zero,#0          'false Z = 0

debug("after NEGNC  ",sdec(Dest),sdec(Src),ubin(Carry),ubin(Zero))

debug("--------------------------------------")

MOV     Dest,valNeg2

MOV     Src,MIN

MODCZ   _Clr, _Clr  WCZ 'set C = 1 Z = 0

IF_C      MOV     Carry,#1        'true  C = 1

IF_NC    MOV     Carry,#0        'false C = 0

IF_Z       MOV     Zero,#1         'true  Z = 1

IF_NZ    MOV     Zero,#0         'false Z = 0

debug("before NEGNC ",sdec(Dest),sdec(Src),ubin(Carry),ubin(Zero))

NEGNC    Dest,Src     WCZ

IF_C       MOV     Carry,#1         'true  C = 1

IF_NC    MOV     Carry,#0         'false C = 0

IF_Z       MOV     Zero,#1          'true  Z = 1

IF_NZ    MOV     Zero,#0          'false Z = 0

debug("after NEGNC  ",sdec(Dest),sdec(Src),ubin(Carry),ubin(Zero))

debug("--------------------------------------")

_Loop1                NOP

JMP     #_Loop1          'remember # imediate

valDest                long      %0000_0000

valSrc                   long      0

valNeg1               long      -1

valNeg2               long      -2

Carry                    long      0

Zero                     long      0

Dest                     long      0

Src                        long      0

MAX                    long      $FFFF_FFFF

MIN                     long      0

### 17.19.7_Example_WRD_NEGZDS_069

NEGZ D,{#}S {WC/WZ/WCZ}

Negate S by Z into D. If Z = 1 then D = -S, else D = S. C = MSB of result. *

{{17.19.7_Example_WRD_NEGZDS_069}}

''NEGZ D,{#}S {WC/WZ/WCZ}

''Negate S by Z into D. If Z = 1 then D = -S, else D = S. C = MSB of result. *

''WC- C = MSB

''WZ- IF Destination result == 0 then Z = 1

''    IF Destination result != 0 then Z = 0

CON

_clkfreq = 200_000_000  ''Debug must be enabled clock must be greater than 10MHZ for Debug

P0 =0 , P1 = 1  , P2 = 2

VAR

Byte cogRunning 'cog ID started is returned or -1 if not started

PUB main()

debug("--------------------------------------")

debug("NEGZ D,{#}S {WC/WZ/WCZ}")

debug("Negate S by Z into D. If Z = 1 then D = -S, else D = S. C = MSB of result. *")

debug("--------------------------------------")

cogRunning := COGINIT(COGEXEC_NEW,@S0_NEGZDS,0)

debug(udec(cogRunning))

repeat                                    'keep cog 0 running

DAT

ORG   0

S0_NEGZDS             DRVH    #P0             'P0 on program running

MOV     Dest,valNeg2

MOV     Src,valNeg1

MODCZ   _Clr, _Clr  WCZ 'C = Z = 0

IF_C   MOV     Carry,#1        'true  C = 1

IF_NC  MOV     Carry,#0        'false C = 0

IF_Z   MOV     Zero,#1         'true  Z = 1

IF_NZ  MOV     Zero,#0         'false Z = 0

debug("--------------------------------------")

debug("before NEGZ ",sdec(Dest),sdec(Src),ubin(Carry),ubin(Zero))

NEGZ    Dest,Src         WCZ

IF_C   MOV     Carry,#1         'true  C = 1

IF_NC  MOV     Carry,#0         'false C = 0

IF_Z   MOV     Zero,#1          'true  Z = 1

IF_NZ  MOV     Zero,#0          'false Z = 0

debug("after NEGZ  ",sdec(Dest),sdec(Src),ubin(Carry),ubin(Zero))

debug("--------------------------------------")

MOV     Dest,valNeg2

MOV     Src,valNeg1

MODCZ   _Clr, _Set  WCZ 'set C = 1 Z = 0

IF_C   MOV     Carry,#1        'true  C = 1

IF_NC  MOV     Carry,#0        'false C = 0

IF_Z   MOV     Zero,#1         'true  Z = 1

IF_NZ  MOV     Zero,#0         'false Z = 0

debug("before NEGZ ",sdec(Dest),sdec(Src),ubin(Carry),ubin(Zero))

NEGZ    Dest,Src    WCZ

IF_C   MOV     Carry,#1         'true  C = 1

IF_NC  MOV     Carry,#0         'false C = 0

IF_Z   MOV     Zero,#1          'true  Z = 1

IF_NZ  MOV     Zero,#0          'false Z = 0

debug("after NEGZ  ",sdec(Dest),sdec(Src),ubin(Carry),ubin(Zero))

debug("--------------------------------------")

MOV     Dest,valNeg2

MOV     Src,MIN

MODCZ   _Clr, _Clr  WCZ 'set C = 1 Z = 0

IF_C    MOV     Carry,#1        'true  C = 1

IF_NC  MOV     Carry,#0        'false C = 0

IF_Z     MOV     Zero,#1         'true  Z = 1

IF_NZ  MOV     Zero,#0         'false Z = 0

debug("before NEGNC ",sdec(Dest),sdec(Src),ubin(Carry),ubin(Zero))

NEGZ    Dest,Src     WCZ

IF_C      MOV     Carry,#1         'true  C = 1

IF_NC    MOV     Carry,#0         'false C = 0

IF_Z      MOV     Zero,#1          'true  Z = 1

IF_NZ    MOV     Zero,#0          'false Z = 0

debug("after NEGNC  ",sdec(Dest),sdec(Src),ubin(Carry),ubin(Zero))

debug("--------------------------------------")

_Loop1                NOP

JMP     #_Loop1          'remember # imediate

valDest              long      %0000_0000

valSrc                long      0

valNeg1            long      -1

valNeg2            long      -2

Carry                 long      0

Zero                  long      0

Dest                  long      0

Src                     long      0

MAX                  long      $FFFF_FFFF

MIN                   long      0

### 17.19.8_Example_WRD_NEGZD_070

NEGZ D {WC/WZ/WCZ}

Negate D by Z. If Z = 1 then D = -D, else D = D. C = MSB of result. *

{{17.19.8_Example_WRD_NEGZD_070}}

''NEGZ D {WC/WZ/WCZ}

''Negate D by Z. If Z = 1 then D = -D, else D = D. C = MSB of result. *

''WC- C = MSB

''WZ- IF Destination result == 0 then Z = 1

''    IF Destination result != 0 then Z = 0

CON

_clkfreq = 200_000_000  ''Debug must be enabled clock must be greater than 10MHZ for Debug

P0 =0 , P1 = 1  , P2 = 2

VAR

Byte cogRunning 'cog ID started is returned or -1 if not started

PUB main()

debug("--------------------------------------")

debug("NEGZ D {WC/WZ/WCZ}")

debug("Negate D by Z. If Z = 1 then D = -D, else D = D. C = MSB of result. * ")

debug("--------------------------------------")

cogRunning := COGINIT(COGEXEC_NEW,@S0_NEGZD,0)

debug(udec(cogRunning))

repeat                                    'keep cog 0 running

DAT

ORG   0

S0_NEGZD              DRVH    #P0             'P0 on program running

MOV     Dest,valNeg2

MODCZ   _Clr, _Clr  WCZ 'C = Z = 0

IF_C     MOV     Carry,#1        'true  C = 1

IF_NC   MOV     Carry,#0        'false C = 0

IF_Z      MOV     Zero,#1         'true  Z = 1

IF_NZ    MOV     Zero,#0         'false Z = 0

debug("--------------------------------------")

debug("before NEGNC ",sdec(Dest),ubin(Carry),ubin(Zero))

NEGZ    Dest         WCZ

IF_C      MOV     Carry,#1         'true  C = 1

IF_NC    MOV     Carry,#0         'false C = 0

IF_Z       MOV     Zero,#1          'true  Z = 1

IF_NZ     MOV     Zero,#0          'false Z = 0

debug("after NEGNC  ",sdec(Dest),ubin(Carry),ubin(Zero))

debug("--------------------------------------")

MOV     Dest,valNeg2

MODCZ   _Clr, _Set  WCZ 'set C = 1 Z = 0

IF_C       MOV     Carry,#1        'true  C = 1

IF_NC     MOV     Carry,#0        'false C = 0

IF_Z       MOV     Zero,#1         'true  Z = 1

IF_NZ     MOV     Zero,#0         'false Z = 0

debug("before NEGZ ",sdec(Dest),ubin(Carry),ubin(Zero))

NEGZ    Dest   WCZ

IF_C       MOV     Carry,#1         'true  C = 1

IF_NC     MOV     Carry,#0         'false C = 0

IF_Z       MOV     Zero,#1          'true  Z = 1

IF_NZ     MOV     Zero,#0          'false Z = 0

debug("after NEGZ  ",sdec(Dest),ubin(Carry),ubin(Zero))

debug("--------------------------------------")

MOV     Dest,valNeg2

MODCZ   _Clr, _Set  WCZ 'set C = 1 Z = 0

IF_C       MOV     Carry,#1        'true  C = 1

IF_NC    MOV     Carry,#0        'false C = 0

IF_Z       MOV     Zero,#1         'true  Z = 1

IF_NZ     MOV     Zero,#0         'false Z = 0

debug("before NEGZ ",sdec(Dest),ubin(Carry),ubin(Zero))

NEGZ   Dest     WCZ

IF_C       MOV     Carry,#1         'true  C = 1

IF_NC    MOV     Carry,#0         'false C = 0

IF_Z       MOV     Zero,#1          'true  Z = 1

IF_NZ    MOV     Zero,#0          'false Z = 0

debug("after NEGZ  ",sdec(Dest),sdec(Src),ubin(Carry),ubin(Zero))

debug("--------------------------------------")

_Loop1                 NOP

JMP     #_Loop1          'remember # imediate

valDest               long      %0000_0000

valSrc                long      0

valNeg1               long      -1

valNeg2               long      -2

Carry                 long      0

Zero                  long      0

Dest                  long      0

Src                   long      0

MAX                   long      $FFFF_FFFF

MIN                   long      0

### 17.19.9_Example_WRD_NEGNZDS_071

NEGNZ D,{#}S {WC/WZ/WCZ}

Negate S by !Z into D. If Z = 0 then D = -S, else D = S. C = MSB of result. *

{{17.19.9_Example_WRD_NEGNZDS_071}}

''NEGNZ D,{#}S {WC/WZ/WCZ}

''Negate S by !Z into D. If Z = 0 then D = -S, else D = S. C = MSB of result. *

''WC- C = MSB

''WZ- IF Destination result == 0 then Z = 1

''    IF Destination result != 0 then Z = 0

CON

_clkfreq = 200_000_000  ''Debug must be enabled clock must be greater than 10MHZ for Debug

P0 =0 , P1 = 1  , P2 = 2

VAR

Byte cogRunning 'cog ID started is returned or -1 if not started

PUB main()

debug("--------------------------------------")

debug("NEGNZ D,{#}S {WC/WZ/WCZ} ")

debug("Negate S by !Z into D. If Z = 0 then D = -S, else D = S. C = MSB of result. * ")

debug("--------------------------------------")

cogRunning := COGINIT(COGEXEC_NEW,@S0_NEGNZDS,0)

debug(udec(cogRunning))

repeat                                    'keep cog 0 running

DAT

ORG   0

S0_NEGNZDS             DRVH    #P0             'P0 on program running

MOV     Dest,valNeg2

MOV     Src,valNeg1

MODCZ   _Clr, _Clr  WCZ 'C = Z = 0

IF_C   MOV     Carry,#1        'true  C = 1

IF_NC  MOV     Carry,#0        'false C = 0

IF_Z   MOV     Zero,#1         'true  Z = 1

IF_NZ  MOV     Zero,#0         'false Z = 0

debug("--------------------------------------")

debug("before NEGNZ ",sdec(Dest),sdec(Src),ubin(Carry),ubin(Zero))

NEGNZ    Dest,Src         WCZ

IF_C   MOV     Carry,#1         'true  C = 1

IF_NC  MOV     Carry,#0         'false C = 0

IF_Z   MOV     Zero,#1          'true  Z = 1

IF_NZ  MOV     Zero,#0          'false Z = 0

debug("after NEGNZ  ",sdec(Dest),sdec(Src),ubin(Carry),ubin(Zero))

debug("--------------------------------------")

MOV     Dest,valNeg2

MOV     Src,valNeg1

MODCZ   _Clr, _Set  WCZ 'set C = 1 Z = 0

IF_C   MOV     Carry,#1        'true  C = 1

IF_NC  MOV     Carry,#0        'false C = 0

IF_Z   MOV     Zero,#1         'true  Z = 1

IF_NZ  MOV     Zero,#0         'false Z = 0

debug("before NEGNZ ",sdec(Dest),sdec(Src),ubin(Carry),ubin(Zero))

NEGNZ    Dest,Src    WCZ

IF_C   MOV     Carry,#1         'true  C = 1

IF_NC  MOV     Carry,#0         'false C = 0

IF_Z   MOV     Zero,#1          'true  Z = 1

IF_NZ  MOV     Zero,#0          'false Z = 0

debug("after NEGNZ  ",sdec(Dest),sdec(Src),ubin(Carry),ubin(Zero))

debug("--------------------------------------")

MOV     Dest,valNeg2

MOV     Src,MIN

MODCZ   _Clr, _Clr  WCZ 'set C = 1 Z = 0

IF_C      MOV     Carry,#1        'true  C = 1

IF_NC   MOV     Carry,#0        'false C = 0

IF_Z      MOV     Zero,#1         'true  Z = 1

IF_NZ    MOV     Zero,#0         'false Z = 0

debug("before NEGNZ ",sdec(Dest),sdec(Src),ubin(Carry),ubin(Zero))

NEGZ    Dest,Src     WCZ

IF_C      MOV     Carry,#1         'true  C = 1

IF_NC    MOV     Carry,#0         'false C = 0

IF_Z       MOV     Zero,#1          'true  Z = 1

IF_NZ    MOV     Zero,#0          'false Z = 0

debug("after NEGNZ  ",sdec(Dest),sdec(Src),ubin(Carry),ubin(Zero))

debug("--------------------------------------")

_Loop1                NOP

JMP     #_Loop1          'remember # imediate

valDest              long      %0000_0000

valSrc                long      0

valNeg1            long      -1

valNeg2            long      -2

Carry                 long      0

Zero                  long      0

Dest                  long      0

Src                     long      0

MAX                  long      $FFFF_FFFF

MIN                   long      0

### 17.19.10_Example_WRD_NEGNZ_072

NEGNZ D {WC/WZ/WCZ}

Negate D by !Z. If Z = 0 then D = -D, else D = D. C = MSB of result. *

{{17.19.10_Example_WRD_NEGNZD_072}}

''NEGNZ D {WC/WZ/WCZ}

''Negate D by !Z. If Z = 0 then D = -D, else D = D. C = MSB of result. *

''WC- C = MSB

''WZ- IF Destination result == 0 then Z = 1

''    IF Destination result != 0 then Z = 0

CON

_clkfreq = 200_000_000  ''Debug must be enabled clock must be greater than 10MHZ for Debug

P0 =0 , P1 = 1  , P2 = 2

VAR

Byte cogRunning 'cog ID started is returned or -1 if not started

PUB main()

debug("--------------------------------------")

debug("NEGNZ D {WC/WZ/WCZ}")

debug("Negate D by !Z. If Z = 0 then D = -D, else D = D. C = MSB of result. *")

debug("--------------------------------------")

cogRunning := COGINIT(COGEXEC_NEW,@S0_NEGD,0)

debug(udec(cogRunning))

repeat                                    'keep cog 0 running

DAT

ORG   0

S0_NEGD          DRVH    #P0             'P0 on program running

MOV     Dest,#1         '1

MODCZ   _Clr, _Clr  WCZ 'C = Z = 0

IF_C    MOV     Carry,#1        'true  C = 1

IF_NC  MOV     Carry,#0        'false C = 0

IF_Z     MOV     Zero,#1         'true  Z = 1

IF_NZ   MOV     Zero,#0         'false Z = 0

debug("--------------------------------------")

debug("before NEG ",ubin(Dest),ubin(Carry),ubin(Zero))

NEGNZ   Dest        WCZ

IF_C    MOV     Carry,#1         'true  C = 1

IF_NC  MOV     Carry,#0         'false C = 0

IF_Z     MOV     Zero,#1          'true  Z = 1

IF_NZ   MOV     Zero,#0          'false Z = 0

debug("after NEG ",sdec(Dest),ubin(Carry),ubin(Zero))

debug("--------------------------------------")

MOV     Dest,MIN        '0

MOV     Src,valNeg2     '-2

MODCZ   _Clr, _Clr  WCZ 'set C = Z = 0

IF_C    MOV     Carry,#1        'true  C = 1

IF_NC     MOV     Carry,#0        'false C = 0

IF_Z       MOV     Zero,#1         'true  Z = 1

IF_NZ     MOV     Zero,#0         'false Z = 0

debug("before NEG ",sdec(Dest),ubin(Carry),ubin(Zero))

NEGNZ   Dest        WCZ

IF_C       MOV     Carry,#1         'true  C = 1

IF_NC    MOV     Carry,#0         'false C = 0

IF_Z       MOV     Zero,#1          'true  Z = 1

IF_NZ    MOV     Zero,#0          'false Z = 0

debug("after NEG ",sdec(Dest),ubin(Carry),ubin(Zero))

debug("--------------------------------------")

_Loop1                NOP

JMP     #_Loop1          'remember # imediate

valDest             long      %0000_0000

valSrc                long      0

valNeg2            long      -2

Carry                 long      0

Zero                  long      0

Dest                  long      0

Src                     long      0

MAX                  long      $FFFF_FFFF

MIN                   long      0

## 17.20) INCMOD/DECMOD  Increment Modulus

| 73 | INCMOD D,{#}S {WC/WZ/WCZ} | Increment with modulus. If D = S then D = 0 and C = 1, else D = D + 1 and C = 0. * |
| --- | --- | --- |
| 74 | DECMOD D,{#}S {WC/WZ/WCZ} | Decrement with modulus. If D = 0 then D = S and C = 1, else D = D - 1 and C = 0. * |

S can be thought as the modulus. See section E.3 Modular Arithmetic. As an example modulus 12 used for time clock count goes from 0-12 when count at 12 reset 0 to allow increment . IF D = 0 wanting to go back in time by decrementing D reset 12  to decrement to 11.

### 17.20.1_Example_WRD_INCMOD_073

INCMOD D,{#}S {WC/WZ/WCZ}

Increment with modulus. If D = S then D = 0 and C = 1, else D = D + 1 and C = 0. *

{{17.20.1_Example_WRD_INCMOD_073}}

''INCMOD D,{#}S {WC/WZ/WCZ}

''Increment with modulus.

''If D = S then D = 0 and C = 1, else D = D + 1 and C = 0. *

''WC- IF D = 0 then C = 1 else C =0

''WZ- IF Destination result == 0 then Z = 1

''    IF Destination result != 0 then Z = 0

CON

_clkfreq = 200_000_000  ''Debug must be enabled clock must be greater than 10MHZ for Debug

P0 =0 , P1 = 1  , P2 = 2

VAR

Byte cogRunning 'cog ID started is returned or -1 if not started

PUB main()

debug("--------------------------------------")

debug("INCMOD D,{#}S {WC/WZ/WCZ}")

debug("Increment with modulus.")

debug("If D = S then D = 0 and C = 1, else D = D + 1 and C = 0.*")

debug("--------------------------------------")

cogRunning := COGINIT(COGEXEC_NEW,@S0_INCMOD,0)

debug(udec(cogRunning))

repeat                                    'keep cog 0 running

DAT

ORG   0

S0_INCMOD             DRVH    #P0             'P0 on program running

MOV     Dest,#12         '1

MOV     SRC,#12

MODCZ   _Clr, _Clr  WCZ 'C = Z = 0

IF_C   MOV     Carry,#1        'true  C = 1

IF_NC  MOV     Carry,#0        'false C = 0

IF_Z   MOV     Zero,#1         'true  Z = 1

IF_NZ  MOV     Zero,#0         'false Z = 0

debug("--------------------------------------")

debug("before INCMOD ",sdec(Dest),sdec(Src),ubin(Carry),ubin(Zero))

INCMOD  Dest,Src    WCZ

IF_C   MOV     Carry,#1         'true  C = 1

IF_NC  MOV     Carry,#0         'false C = 0

IF_Z   MOV     Zero,#1          'true  Z = 1

IF_NZ  MOV     Zero,#0          'false Z = 0

debug("after INCMOD ",sdec(Dest),sdec(Src),ubin(Carry),ubin(Zero))

debug("--------------------------------------")

MOV     Dest,#0

MOV     Src,#12

MODCZ   _Clr, _Clr  WCZ 'set C = Z = 0

IF_C       MOV     Carry,#1        'true  C = 1

IF_NC    MOV     Carry,#0        'false C = 0

IF_Z       MOV     Zero,#1         'true  Z = 1

IF_NZ     MOV     Zero,#0         'false Z = 0

debug("before INCMOD ",sdec(Dest),sdec(Src),ubin(Carry),ubin(Zero))

INCMOD  Dest,SRC        WCZ

IF_C       MOV     Carry,#1         'true  C = 1

IF_NC     MOV     Carry,#0         'false C = 0

IF_Z       MOV     Zero,#1          'true  Z = 1

IF_NZ     MOV     Zero,#0          'false Z = 0

debug("after INCMOD  ",sdec(Dest),sdec(Src),ubin(Carry),ubin(Zero))

debug("--------------------------------------")

_Loop1                  NOP

JMP     #_Loop1          'remember # imediate

valDest             long      %0000_0000

valSrc                long      0

valNeg2           long      -2

Carry                 long      0

Zero                  long      0

Dest                  long      0

Src                     long      0

MAX                  long      $FFFF_FFFF

MIN                   long      0

### 17.20.2_Example_WRD_DECMOD_074

DECMOD D,{#}S {WC/WZ/WCZ}

Decrement with modulus. If D = 0 then D = S and C = 1, else D = D - 1 and C = 0. *

{{17.20.2_Example_WRD_DECMOD_074}}

''DECMOD D,{#}S {WC/WZ/WCZ}

''Decrement with modulus.

''If D = 0 then D = S and C = 1, else D = D - 1 and C = 0. *

''WC- IF D = S then C = 1 else C =0

''WZ- IF Destination result == 0 then Z = 1

''    IF Destination result != 0 then Z = 0

CON

_clkfreq = 200_000_000  ''Debug must be enabled clock must be greater than 10MHZ for Debug

P0 =0 , P1 = 1  , P2 = 2

VAR

Byte cogRunning 'cog ID started is returned or -1 if not started

PUB main()

debug("--------------------------------------")

debug("DECMOD D,{#}S {WC/WZ/WCZ}")

debug("Decrement with modulus.")

debug("If D = 0 then D = S and C = 1, else D = D - 1 and C = 0. *")

debug("--------------------------------------")

cogRunning := COGINIT(COGEXEC_NEW,@S0_DECMOD,0)

debug(udec(cogRunning))

repeat                                    'keep cog 0 running

DAT

ORG   0

S0_DECMOD             DRVH    #P0             'P0 on program running

MOV     Dest,#0         '1

MOV     SRC,#12

MODCZ   _Clr, _Clr  WCZ 'C = Z = 0

IF_C   MOV     Carry,#1        'true  C = 1

IF_NC  MOV     Carry,#0        'false C = 0

IF_Z   MOV     Zero,#1         'true  Z = 1

IF_NZ  MOV     Zero,#0         'false Z = 0

debug("--------------------------------------")

debug("before DECMOD ",sdec(Dest),sdec(Src),ubin(Carry),ubin(Zero))

DECMOD  Dest,Src    WCZ

IF_C   MOV     Carry,#1         'true  C = 1

IF_NC  MOV     Carry,#0         'false C = 0

IF_Z   MOV     Zero,#1          'true  Z = 1

IF_NZ  MOV     Zero,#0          'false Z = 0

debug("after DECMOD ",sdec(Dest),sdec(Src),ubin(Carry),ubin(Zero))

debug("--------------------------------------")

MOV     Dest,#11

MOV     Src,#12

MODCZ   _Clr, _Clr  WCZ 'set C = Z = 0

IF_C   MOV     Carry,#1        'true  C = 1

IF_NC  MOV     Carry,#0        'false C = 0

IF_Z   MOV     Zero,#1         'true  Z = 1

IF_NZ  MOV     Zero,#0         'false Z = 0

debug("before DECMOD ",sdec(Dest),sdec(Src),ubin(Carry),ubin(Zero))

DECMOD  Dest,SRC        WCZ

IF_C   MOV     Carry,#1         'true  C = 1

IF_NC  MOV     Carry,#0         'false C = 0

IF_Z   MOV     Zero,#1          'true  Z = 1

IF_NZ  MOV     Zero,#0          'false Z = 0

debug("after DECMOD  ",sdec(Dest),sdec(Src),ubin(Carry),ubin(Zero))

debug("--------------------------------------")

_Loop1                NOP

JMP     #_Loop1          'remember # imediate

valDest               long      %0000_0000

valSrc                long      0

valNeg2               long      -2

Carry                 long      0

Zero                  long      0

Dest                  long      0

Src                   long      0

MAX                   long      $FFFF_FFFF

MIN                   long      0

## 17.21 Zeror/Sign Extend

| ZEROX D,{#}S {WC/WZ/WCZ} | Zero-extend D above bit S[4:0]. C = MSB of result. * |
| --- | --- |
| SIGNX D,{#}S {WC/WZ/WCZ} | Sign-extend D from bit S[4:0]. C = MSB of result. * |

L[31:0]   = B31B30B29B28B27B26B25B24_ B23B22B21B20B19B18B17B16_ B15B14B13B12B11B10B9B8_B7B6B5B4B3B2B1B0

S[4:0]   =  upto this value bits will remain the same

ZEROX = 0 will fill in all bits beyond S[4:0] value

SIGNX = sign value 1 will be filled in all bits beyond S[4:0] value

### 17.21.1_Example_WRD_ZEROX_075

ZEROX D,{#}S {WC/WZ/WCZ}

Zero-extend D above bit S[4:0]. C = MSB of result. *

ZEROX = 0 will fill in all bits beyond S[4:0] value

{{17.21.1_Example_WRD_ZEROX_075}}

''ZEROX D,{#}S {WC/WZ/WCZ}

''Zero-extend D above bit S[4:0]. C = MSB of result. *

''WC- C = MSB of result.

''WZ- IF Destination result == 0 then Z = 1

''    IF Destination result != 0 then Z = 0

''B31B30B29B28B27B26B25B24_ B23B22B21B20B19B18B17B16_ B15B14B13B12B11B10B9B8_B7B6B5B4B3B2B1B0

CON

_clkfreq = 200_000_000  ''Debug must be enabled clock must be greater than 10MHZ for Debug

P0 =0 , P1 = 1  , P2 = 2

VAR

Byte cogRunning 'cog ID started is returned or -1 if not started

PUB main()

debug("--------------------------------------")

debug("ZEROX D,{#}S {WC/WZ/WCZ}")

debug("Zero-extend D above bit S[4:0]. C = MSB of result. *")

debug("--------------------------------------")

cogRunning := COGINIT(COGEXEC_NEW,@S0_ZEROX,0)

debug(udec(cogRunning))

repeat                                    'keep cog 0 running

DAT

ORG   0

S0_ZEROX            DRVH    #P0             'P0 on program running

MOV     Dest,MAX         '1

MOV     SRC,#3

MODCZ   _Clr, _Clr  WCZ 'C = Z = 0

IF_C        MOV     Carry,#1        'true  C = 1

IF_NC      MOV     Carry,#0        'false C = 0

IF_Z         MOV     Zero,#1         'true  Z = 1

IF_NZ      MOV     Zero,#0         'false Z = 0

debug("--------------------------------------")

debug("before ZEROX ",ubin(Dest),sdec(Src),ubin(Carry),ubin(Zero))

ZEROX   Dest,Src    WCZ

IF_C       MOV     Carry,#1         'true  C = 1

IF_NC    MOV     Carry,#0         'false C = 0

IF_Z       MOV     Zero,#1          'true  Z = 1

IF_NZ    MOV     Zero,#0          'false Z = 0

debug("after ZEROX ",ubin(Dest),sdec(Src),ubin(Carry),ubin(Zero))

debug("--------------------------------------")

MOV     Dest,valFFFF_FFFA

MOV     Src,#3

MODCZ   _Clr, _Clr  WCZ 'set C = Z = 0

IF_C        MOV     Carry,#1        'true  C = 1

IF_NC      MOV     Carry,#0        'false C = 0

IF_Z         MOV     Zero,#1         'true  Z = 1

IF_NZ      MOV     Zero,#0         'false Z = 0

debug("before ZEROX ",ubin(Dest),sdec(Src),ubin(Carry),ubin(Zero))

ZEROX   Dest,SRC    WCZ

IF_C         MOV     Carry,#1         'true  C = 1

IF_NC      MOV     Carry,#0         'false C = 0

IF_Z         MOV     Zero,#1          'true  Z = 1

IF_NZ      MOV     Zero,#0          'false Z = 0

debug("after ZEROX  ",ubin(Dest),sdec(Src),ubin(Carry),ubin(Zero))

debug("--------------------------------------")

_Loop1                   NOP

JMP     #_Loop1          'remember # imediate

valDest                long      %0000_0000

valSrc                   long      0

valNeg2               long      -2

valFFFF_FFFA     long      $FFFF_FFFA

Carry                   long      0

Zero                    long      0

Dest                    long      0

Src                       long      0

MAX                   long      $FFFF_FFFF

MIN                    long      0

### 17.21.2_Example_WRD_SIGNX_076

SIGNX D,{#}S {WC/WZ/WCZ}

Sign-extend D from bit S[4:0]. C = MSB of result. *

SIGNX = sign value 1 will be filled in all bits beyond S[4:0] value

{{17.21.2_Example_WRD_SIGNX_076}}

''SIGNX D,{#}S {WC/WZ/WCZ}

''Sign-extend D from bit S[4:0]. C = MSB of result. *

''WC- C = MSB of result.

''WZ- IF Destination result == 0 then Z = 1

''    IF Destination result != 0 then Z = 0

''B31B30B29B28B27B26B25B24_ B23B22B21B20B19B18B17B16_ B15B14B13B12B11B10B9B8_B7B6B5B4B3B2B1B0

CON

_clkfreq = 200_000_000  ''Debug must be enabled clock must be greater than 10MHZ for Debug

P0 =0 , P1 = 1  , P2 = 2

VAR

Byte cogRunning 'cog ID started is returned or -1 if not started

PUB main()

debug("--------------------------------------")

debug("SIGNX D,{#}S {WC/WZ/WCZ}")

debug("Sign-extend D from bit S[4:0]. C = MSB of result. *")

debug("--------------------------------------")

cogRunning := COGINIT(COGEXEC_NEW,@S0_SIGNX,0)

debug(udec(cogRunning))

repeat                                    'keep cog 0 running

DAT

ORG   0

S0_SIGNX              DRVH    #P0             'P0 on program running

MOV     Dest,valNeg1000

MOV     SRC,#3

MODCZ   _Clr, _Clr  WCZ 'C = Z = 0

IF_C   MOV     Carry,#1        'true  C = 1

IF_NC  MOV     Carry,#0        'false C = 0

IF_Z   MOV     Zero,#1         'true  Z = 1

IF_NZ  MOV     Zero,#0         'false Z = 0

debug("--------------------------------------")

debug("before SIGNX ",ubin(Dest),sdec(Src),ubin(Carry),ubin(Zero))

SIGNX   Dest,Src    WCZ

IF_C   MOV     Carry,#1         'true  C = 1

IF_NC  MOV     Carry,#0         'false C = 0

IF_Z   MOV     Zero,#1          'true  Z = 1

IF_NZ  MOV     Zero,#0          'false Z = 0

debug("after SIGNX ",ubin(Dest),sdec(Src),ubin(Carry),ubin(Zero))

debug("--------------------------------------")

MOV     Dest,val0000_00FA

MOV     Src,#3       's[4:0] = 3 B4 B3 B2 B1 B0 =  0 1 0 0 0

MODCZ   _Clr, _Clr  WCZ 'set C = Z = 0

IF_C   MOV     Carry,#1        'true  C = 1

IF_NC  MOV     Carry,#0        'false C = 0

IF_Z   MOV     Zero,#1         'true  Z = 1

IF_NZ  MOV     Zero,#0         'false Z = 0

debug("before SIGNX ",ubin(Dest),sdec(Src),ubin(Carry),ubin(Zero))

SIGNX   Dest,SRC    WCZ

IF_C   MOV     Carry,#1         'true  C = 1

IF_NC  MOV     Carry,#0         'false C = 0

IF_Z   MOV     Zero,#1          'true  Z = 1

IF_NZ  MOV     Zero,#0          'false Z = 0

debug("after SIGNX  ",ubin(Dest),sdec(Src),ubin(Carry),ubin(Zero))

debug("--------------------------------------")

_Loop1                NOP

JMP     #_Loop1          'remember # imediate

valDest               long      %0000_0000

valSrc                long      0

valNeg1000            long      -1000

valNeg2               long      -2

valFFFF_FFFA          long      $FFFF_FFFA

val0000_00FA          long      $0000_00FA

Carry                 long      0

Zero                  long      0

Dest                  long      0

Src                     long      0

MAX                  long      $FFFF_FFFF

MIN                  long      0

## 17.22) ENCOD Get top Bit Position

| ENCOD D,{#}S {WC/WZ/WCZ} | Get bit position of top-most '1' in S into D. D = position of top '1' in S (0..31). C = (S != 0). * |
| --- | --- |
| ENCOD D {WC/WZ/WCZ} | Get bit position of top-most '1' in D into D. D = position of top '1' in S (0..31). C = (S != 0). * |

### 17.22.1_Example_WRD_ENCODDS_077

ENCOD D,{#}S {WC/WZ/WCZ}

Get bit position of top-most '1' in S into D. D = position of top '1' in S (0..31). C = (S != 0). *

{{17.22.1_Example_WRD_ENCODDS_077}}

''ENCOD D,{#}S {WC/WZ/WCZ}

''Get bit position of top-most '1' in S into D.

''D = position of top '1' in S (0..31). C = (S != 0). *

''WZ- IF Destination result == 0 then Z = 1

''    IF Destination result != 0 then Z = 0

''B31B30B29B28B27B26B25B24_ B23B22B21B20B19B18B17B16_ B15B14B13B12B11B10B9B8_B7B6B5B4B3B2B1B0

CON

_clkfreq = 200_000_000  ''Debug must be enabled clock must be greater than 10MHZ for Debug

P0 =0 , P1 = 1  , P2 = 2

VAR

Byte cogRunning 'cog ID started is returned or -1 if not started

PUB main()

debug("--------------------------------------")

debug("ENCOD D,{#}S {WC/WZ/WCZ}")

debug("D = position of top '1' in S (0..31). C = (S != 0). *")

debug("--------------------------------------")

cogRunning := COGINIT(COGEXEC_NEW,@S0_ENCODDS,0)

debug(udec(cogRunning))

repeat                                    'keep cog 0 running

DAT

ORG   0

S0_ENCODDS    DRVH    #P0             'P0 on program running

MOV     Dest,#0

MOV     SRC,valSrc

MODCZ   _Clr, _Clr  WCZ 'C = Z = 0

IF_C      MOV     Carry,#1        'true  C = 1

IF_NC   MOV     Carry,#0        'false C = 0

IF_Z     MOV     Zero,#1         'true  Z = 1

IF_NZ  MOV     Zero,#0         'false Z = 0

debug("--------------------------------------")

debug("before ENCOD ",sdec(Dest),ubin(Src),ubin(Carry),ubin(Zero))

ENCOD   Dest,Src    WCZ

IF_C     MOV     Carry,#1         'true  C = 1

IF_NC     MOV     Carry,#0         'false C = 0

IF_Z         MOV     Zero,#1          'true  Z = 1

IF_NZ      MOV     Zero,#0          'false Z = 0

debug("after ENCOD ",sdec(Dest),ubin(Src),ubin(Carry),ubin(Zero))

debug("--------------------------------------")

MOV     Dest,val0000_00FA

MOV     Src,#7       's[4:0] = 3 B4 B3 B2 B1 B0 =  0 1 0 0 0

MODCZ   _Clr, _Clr  WCZ 'set C = Z = 0

IF_C        MOV     Carry,#1        'true  C = 1

IF_NC      MOV     Carry,#0        'false C = 0

IF_Z         MOV     Zero,#1         'true  Z = 1

IF_NZ      MOV     Zero,#0         'false Z = 0

debug("before ENCOD ",sdec(Dest),ubin(Src),ubin(Carry),ubin(Zero))

ENCOD   Dest,SRC    WCZ

IF_C        MOV     Carry,#1         'true  C = 1

IF_NC      MOV     Carry,#0         'false C = 0

IF_Z        MOV     Zero,#1          'true  Z = 1

IF_NZ      MOV     Zero,#0          'false Z = 0

debug("after ENCOD  ",sdec(Dest),ubin(Src),ubin(Carry),ubin(Zero))

debug("--------------------------------------")

_Loop1                   NOP

JMP     #_Loop1          'remember # imediate

valDest                    long      %0000_0000

valSrc                       long      %00000000_101010101_111111111_11111111

valNeg1000            long      -1000

valNeg2                   long      -2

valFFFF_FFFA          long      $FFFF_FFFA

val0000_00FA          long      $0000_00FA

Carry                          long      0

Zero                            long      0

Dest                            long      0

Src                               long      0

MAX                            long      $FFFF_FFFF

MIN                             long      0

### 17.22.1_Example_WRD_ENCODD_078

ENCOD D {WC/WZ/WCZ}

Get bit position of top-most '1' in D into D. D = position of top '1' in S (0..31). C = (S != 0). *

{{17.22.2_Example_WRD_ENCODD_078}}

''ENCOD D {WC/WZ/WCZ}

''Get bit position of top-most '1' in D into D.

''D = position of top '1' in S (0..31). C = (S != 0). *

''WZ- IF Destination result == 0 then Z = 1

''    IF Destination result != 0 then Z = 0

''B31B30B29B28B27B26B25B24_ B23B22B21B20B19B18B17B16_ B15B14B13B12B11B10B9B8_B7B6B5B4B3B2B1B0

CON

_clkfreq = 200_000_000  ''Debug must be enabled clock must be greater than 10MHZ for Debug

P0 =0 , P1 = 1  , P2 = 2

VAR

Byte cogRunning 'cog ID started is returned or -1 if not started

PUB main()

debug("--------------------------------------")

debug("ENCOD D {WC/WZ/WCZ}")

debug("Get bit position of top-most '1' in D into D.")

debug("D = position of top '1' in S (0..31). C = (S != 0). * ")

debug("--------------------------------------")

cogRunning := COGINIT(COGEXEC_NEW,@S0_ENCODD,0)

debug(udec(cogRunning))

repeat                                    'keep cog 0 running

DAT

ORG   0

S0_ENCODD     DRVH    #P0             'P0 on program running

MOV     Dest,valDest

MODCZ   _Clr, _Clr  WCZ 'C = Z = 0

IF_C    MOV     Carry,#1        'true  C = 1

IF_NC  MOV     Carry,#0        'false C = 0

IF_Z     MOV     Zero,#1         'true  Z = 1

IF_NZ  MOV     Zero,#0         'false Z = 0

debug("--------------------------------------")

debug("before ENCOD ",ubin(Dest),ubin(Carry),ubin(Zero))

ENCOD   Dest        WCZ

IF_C     MOV     Carry,#1         'true  C = 1

IF_NC  MOV     Carry,#0         'false C = 0

IF_Z     MOV     Zero,#1          'true  Z = 1

IF_NZ  MOV     Zero,#0          'false Z = 0

debug("after ENCOD ",ubin(Dest),ubin(Carry),ubin(Zero))

debug("--------------------------------------")

MOV     Dest,val0000_00FA

MODCZ   _Clr, _Clr  WCZ 'set C = Z = 0

IF_C        MOV     Carry,#1        'true  C = 1

IF_NC      MOV     Carry,#0        'false C = 0

IF_Z         MOV     Zero,#1         'true  Z = 1

IF_NZ       MOV     Zero,#0         'false Z = 0

debug("before ENCOD ",ubin(Dest),ubin(Carry),ubin(Zero))

ENCOD   Dest        WCZ

IF_C         MOV     Carry,#1         'true  C = 1

IF_NC      MOV     Carry,#0         'false C = 0

IF_Z         MOV     Zero,#1          'true  Z = 1

IF_NZ       MOV     Zero,#0          'false Z = 0

debug("after ENCOD  ",ubin(Dest),ubin(Carry),ubin(Zero))

debug("--------------------------------------")

_Loop1                    NOP

JMP     #_Loop1          'remember # imediate

valDest                    long      %00000000_000010101_111111111_11111111

valSrc                        long      %00000000_101010101_111111111_11111111

valNeg1000             long      -1000

valNeg2                    long      -2

valFFFF_FFFA          long      $FFFF_FFFA

val0000_00FA          long      $0000_00FA

Carry                         long      0

Zero                          long      0

Dest                          long      0

Src                            long      0

MAX                          long      $FFFF_FFFF

MIN                           long      0

## 17.23)Ones Count number of 1’s in S Put value in D

| ONES D,{#}S {WC/WZ/WCZ} | Get number of '1's in S into D. D = number of '1's in S (0..32). C = LSB of result. * |
| --- | --- |
| ONES D {WC/WZ/WCZ} | Get number of '1's in D into D. D = number of '1's in S (0..32). C = LSB of result. * |

### 17.23.1_Example_WRD_ONESDS_079

ONES D,{#}S {WC/WZ/WCZ}

Get number of '1's in S into D. D = number of '1's in S (0..32). C = LSB of result. *

Number of ones in S are counted and this value is put in D

{{17.23.1_Example_WRD_ONESDS_079}}

''ONES D,{#}S {WC/WZ/WCZ}

''Get number of '1's in S into D.

''D = number of '1's in S (0..32). C = LSB of result. *

''C = LSB of result. *

''WZ- IF Destination result == 0 then Z = 1

''    IF Destination result != 0 then Z = 0

''B31B30B29B28B27B26B25B24_ B23B22B21B20B19B18B17B16_ B15B14B13B12B11B10B9B8_B7B6B5B4B3B2B1B0

CON

_clkfreq = 200_000_000  ''Debug must be enabled clock must be greater than 10MHZ for Debug

P0 =0 , P1 = 1  , P2 = 2

VAR

Byte cogRunning 'cog ID started is returned or -1 if not started

PUB main()

debug("--------------------------------------")

debug("ONES D,{#}S {WC/WZ/WCZ}")

debug("Get number of '1's in S into D.")

debug("D = number of '1's in S (0..32). C = LSB of result. *")

debug("--------------------------------------")

cogRunning := COGINIT(COGEXEC_NEW,@S0_ONESDS,0)

debug(udec(cogRunning))

repeat                                    'keep cog 0 running

DAT

ORG   0

S0_ONESDS             DRVH    #P0             'P0 on program running

MOV     Dest,#0

MOV     Src,#14         '3 1's in S

MODCZ   _Clr, _Clr  WCZ 'C = Z = 0

IF_C          MOV     Carry,#1        'true  C = 1

IF_NC        MOV     Carry,#0        'false C = 0

IF_Z           MOV     Zero,#1         'true  Z = 1

IF_NZ        MOV     Zero,#0         'false Z = 0

debug("--------------------------------------")

debug("before ONES ",ubin(Dest),ubin(Src),ubin(Carry),ubin(Zero))

ONES    Dest,Src     WCZ

IF_C    MOV     Carry,#1         'true  C = 1

IF_NC  MOV     Carry,#0         'false C = 0

IF_Z     MOV     Zero,#1          'true  Z = 1

IF_NZ  MOV     Zero,#0          'false Z = 0

debug("after ONES  ",ubin(Dest),ubin(Src),ubin(Carry),ubin(Zero))

debug("--------------------------------------")

MOV     Dest,#0

MOV     Src,4

MODCZ   _Clr, _Clr  WCZ 'set C = Z = 0

IF_C      MOV     Carry,#1        'true  C = 1

IF_NC   MOV     Carry,#0        'false C = 0

IF_Z      MOV     Zero,#1         'true  Z = 1

IF_NZ     MOV     Zero,#0         'false Z = 0

debug("before ONES  ",ubin(Dest),ubin(Src),ubin(Carry),ubin(Zero))

ONES    Dest,Src    WCZ

IF_C        MOV     Carry,#1         'true  C = 1

IF_NC      MOV     Carry,#0         'false C = 0

IF_Z        MOV     Zero,#1          'true  Z = 1

IF_NZ     MOV     Zero,#0          'false Z = 0

debug("after ONES  ",ubin(Dest),ubin(Src),ubin(Carry),ubin(Zero))

debug("--------------------------------------")

_Loop1                NOP

JMP     #_Loop1          'remember # imediate

valDest                      long      %00000000_000010101_111111111_11111111

valSrc                         long      %00000000_101010101_111111111_11111111

valNeg1000               long      -1000

valNeg2                     long      -2

valFFFF_FFFA           long      $FFFF_FFFA

val0000_00FA          long      $0000_00FA

Carry                         long      0

Zero                           long      0

Dest                           long      0

Src                              long      0

MAX                           long      $FFFF_FFFF

MIN                            long      0

### 17.23.2_Example_WRD_ONESD_080

ONES D {WC/WZ/WCZ}

Get number of '1's in D into D. D = number of '1's in S (0..32). C = LSB of result. *

{{17.23.2_Example_WRD_ONESD_080}}

''ONES D {WC/WZ/WCZ}

''Get number of '1's in D into D.

''WC- C = LSB of result. *

''WZ- IF Destination result == 0 then Z = 1

''    IF Destination result != 0 then Z = 0

''B31B30B29B28B27B26B25B24_ B23B22B21B20B19B18B17B16_ B15B14B13B12B11B10B9B8_B7B6B5B4B3B2B1B0

CON

_clkfreq = 200_000_000  ''Debug must be enabled clock must be greater than 10MHZ for Debug

P0 =0 , P1 = 1  , P2 = 2

VAR

Byte cogRunning 'cog ID started is returned or -1 if not started

PUB main()

debug("--------------------------------------")

debug("ONES D {WC/WZ/WCZ}")

debug("Get number of '1's in D into D.")

debug("--------------------------------------")

cogRunning := COGINIT(COGEXEC_NEW,@S0_ONESD,0)

debug(udec(cogRunning))

repeat                                    'keep cog 0 running

DAT

ORG   0

S0_ONESD              DRVH    #P0             'P0 on program running

MOV     Dest,#%10101010

MODCZ   _Clr, _Clr  WCZ 'C = Z = 0

IF_C   MOV     Carry,#1        'true  C = 1

IF_NC  MOV     Carry,#0        'false C = 0

IF_Z   MOV     Zero,#1         'true  Z = 1

IF_NZ  MOV     Zero,#0         'false Z = 0

debug("--------------------------------------")

debug("before ONES ",ubin(Dest),ubin(Carry),ubin(Zero))

ONES    Dest         WCZ

IF_C   MOV     Carry,#1         'true  C = 1

IF_NC  MOV     Carry,#0         'false C = 0

IF_Z   MOV     Zero,#1          'true  Z = 1

IF_NZ  MOV     Zero,#0          'false Z = 0

debug("after ONES  ",ubin(Dest),sdec(Dest),ubin(Carry),ubin(Zero))

debug("--------------------------------------")

MOV     Dest,ValDest

MODCZ   _Clr, _Clr  WCZ 'set C = Z = 0

IF_C    MOV     Carry,#1        'true  C = 1

IF_NC  MOV     Carry,#0        'false C = 0

IF_Z    MOV     Zero,#1         'true  Z = 1

IF_NZ  MOV     Zero,#0         'false Z = 0

debug("before ONES  ",ubin(Dest),ubin(Carry),ubin(Zero))

ONES    Dest         WCZ

IF_C    MOV     Carry,#1         'true  C = 1

IF_NC  MOV     Carry,#0         'false C = 0

IF_Z    MOV     Zero,#1          'true  Z = 1

IF_NZ  MOV     Zero,#0          'false Z = 0

debug("after ONES  ",ubin(Dest),sdec(Dest),ubin(Carry),ubin(Zero))

debug("--------------------------------------")

_Loop1                NOP

JMP     #_Loop1          'remember # imediate

valDest                   long      %00000000_000010101_111111111_11111111

valSrc                      long      %00000000_101010101_111111111_11111111

valNeg1000            long      -1000

valNeg2                  long      -2

valFFFF_FFFA        long      $FFFF_FFFA

val0000_00FA       long      $0000_00FA

Carry                       long      0

Zero                         long      0

Dest                        long      0

Src                           long      0

MAX                        long      $FFFF_FFFF

MIN                         long      0

## 17.24)TEST set carry base on test D&S (no register changes)

| TEST D,{#}S {WC/WZ/WCZ} | Test D with S. C = parity of (D & S). Z = ((D & S) == 0). |
| --- | --- |
| TEST D {WC/WZ/WCZ} | Test D. C = parity of D. Z = (D == 0). |
| TESTN D,{#}S {WC/WZ/WCZ} | Test D with !S. C = parity of (D & !S). Z = ((D & !S) == 0). |

### 17.24.1_Example_WRD_TESTDS_081

TEST D,{#}S {WC/WZ/WCZ}

Test D with S. C = parity of (D & S). Z = ((D & S) == 0).

{{17.24.1_Example_WRD_TESTDS_081}}

''TEST D,{#}S {WC/WZ/WCZ}

''Test D with S. C = parity of (D & S). Z = ((D & S) == 0).

''WC- C = parity of (D & S) odd parity = 1 even parity =0

''WZ- IF Destination result == 0 then Z = 1

''    IF Destination result != 0 then Z = 0

CON

_clkfreq = 200_000_000  ''Debug must be enabled clock must be greater than 10MHZ for Debug

P0 =0 , P1 = 1  , P2 = 2

VAR

Byte cogRunning 'cog ID started is returned or -1 if not started

PUB main()

debug("--------------------------------------")

debug("TEST D,{#}S {WC/WZ/WCZ}")

debug("Test D with S. C = parity of (D & S). Z = ((D & S) == 0).")

debug("--------------------------------------")

cogRunning := COGINIT(COGEXEC_NEW,@S0_TESTDS,0)

debug(udec(cogRunning))

repeat                                    'keep cog 0 running

DAT

ORG   0

S0_TESTDS             DRVH    #P0             'P0 on program running

MOV     Dest,#%10101010

MOV     Src, #%10101010

MODCZ   _Clr, _Clr  WCZ 'C = Z = 0

IF_C   MOV     Carry,#1        'true  C = 1

IF_NC  MOV     Carry,#0        'false C = 0

IF_Z   MOV     Zero,#1         'true  Z = 1

IF_NZ  MOV     Zero,#0         'false Z = 0

debug("--------------------------------------")

debug("before TEST ",ubin(Dest),ubin(Src),ubin(Carry),ubin(Zero))

TEST    Dest,Src     WCZ

IF_C   MOV     Carry,#1         'true  C = 1

IF_NC  MOV     Carry,#0         'false C = 0

IF_Z     MOV     Zero,#1          'true  Z = 1

IF_NZ  MOV     Zero,#0          'false Z = 0

debug("after TEST  ",ubin(Dest),ubin(Src),ubin(Carry),ubin(Zero))

debug("--------------------------------------")

MOV     Dest,valDest

MOV     Src,valSrc

MODCZ   _Clr, _Clr  WCZ 'set C = Z = 0

IF_C     MOV     Carry,#1        'true  C = 1

IF_NC   MOV     Carry,#0        'false C = 0

IF_Z      MOV     Zero,#1         'true  Z = 1

IF_NZ   MOV     Zero,#0         'false Z = 0

debug("before TEST  ",ubin(Dest),ubin(Src),ubin(Carry),ubin(Zero))

TEST    Dest,Src    WCZ

IF_C        MOV     Carry,#1         'true  C = 1

IF_NC     MOV     Carry,#0         'false C = 0

IF_Z        MOV     Zero,#1          'true  Z = 1

IF_NZ     MOV     Zero,#0          'false Z = 0

debug("after TEST  ",ubin(Dest),ubin(Src),ubin(Carry),ubin(Zero))

debug("--------------------------------------")

_Loop1                 NOP

JMP     #_Loop1          'remember # imediate

valDest                  long      %00000000_000010101_111111111_11111110

valSrc                     long      %00000000_101010101_111111111_11111111

valNeg1000          long      -1000

valNeg2                long      -2

valFFFF_FFFA      long      $FFFF_FFFA

val0000_00FA      long      $0000_00FA

Carry                      long      0

Zero                        long      0

Dest                       long      0

Src                          long      0

MAX                      long      $FFFF_FFFF

MIN                       long      0

### 17.24.2_Example_WRD_TESTD_082

TEST D {WC/WZ/WCZ}

Test D. C = parity of D. Z = (D == 0).

{{17.24.2_Example_WRD_TESTD_082}}

''TEST D {WC/WZ/WCZ}

''Test D. C = parity of D. Z = (D == 0).

''WC- C = parity of (D) odd parity = 1 even parity =0

''WZ- IF Destination result == 0 then Z = 1

''    IF Destination result != 0 then Z = 0

CON

_clkfreq = 200_000_000  ''Debug must be enabled clock must be greater than 10MHZ for Debug

P0 =0 , P1 = 1  , P2 = 2

VAR

Byte cogRunning 'cog ID started is returned or -1 if not started

PUB main()

debug("--------------------------------------")

debug("TEST D {WC/WZ/WCZ}")

debug("Test D. C = parity of (D). Z = (D == 0).")

debug("--------------------------------------")

cogRunning := COGINIT(COGEXEC_NEW,@S0_TESTD,0)

debug(udec(cogRunning))

repeat                                    'keep cog 0 running

DAT

ORG   0

S0_TESTD            DRVH    #P0             'P0 on program running

MOV     Dest,#%10101011

MODCZ   _Clr, _Clr  WCZ 'C = Z = 0

IF_C       MOV     Carry,#1        'true  C = 1

IF_NC     MOV     Carry,#0        'false C = 0

IF_Z       MOV     Zero,#1         'true  Z = 1

IF_NZ    MOV     Zero,#0         'false Z = 0

debug("--------------------------------------")

debug("before TEST ",ubin(Dest),ubin(Carry),ubin(Zero))

TEST    Dest        WCZ

IF_C       MOV     Carry,#1         'true  C = 1

IF_NC    MOV     Carry,#0         'false C = 0

IF_Z       MOV     Zero,#1          'true  Z = 1

IF_NZ    MOV     Zero,#0          'false Z = 0

debug("after TEST  ",ubin(Dest),ubin(Carry),ubin(Zero))

debug("--------------------------------------")

MOV     Dest,valDest

MODCZ   _Clr, _Clr  WCZ 'set C = Z = 0

IF_C     MOV     Carry,#1        'true  C = 1

IF_NC  MOV     Carry,#0        'false C = 0

IF_Z        MOV     Zero,#1         'true  Z = 1

IF_NZ     MOV     Zero,#0         'false Z = 0

debug("before TEST  ",ubin(Dest),ubin(Carry),ubin(Zero))

TEST    Dest         WCZ

IF_C       MOV     Carry,#1         'true  C = 1

IF_NC     MOV     Carry,#0         'false C = 0

IF_Z       MOV     Zero,#1          'true  Z = 1

IF_NZ     MOV     Zero,#0          'false Z = 0

debug("after TEST  ",ubin(Dest),ubin(Carry),ubin(Zero))

debug("--------------------------------------")

_Loop1                  NOP

JMP     #_Loop1          'remember # imediate

valDest                  long      %00000000_000010101_111111111_11111111

valSrc                   long      %00000000_101010101_111111111_11111111

valNeg1000         long      -1000

valNeg2               long      -2

valFFFF_FFFA     long      $FFFF_FFFA

val0000_00FA    long      $0000_00FA

Carry                   long      0

Zero                    long      0

Dest                     long      0

Src                       long      0

MAX                    long      $FFFF_FFFF

MIN                     long      0

### 17.24.3_Example_WRD_TESTND_83

TESTN D,{#}S {WC/WZ/WCZ}

Test D with !S. C = parity of (D & !S). Z = ((D & !S) == 0).

{{17.24.3_Example_WRD_TESTNDS_083}}

''TESTN D,{#}S {WC/WZ/WCZ}

''Test D with !S. C = parity of (D & !S). Z = ((D & !S) == 0).

''WC- C = parity of (D) odd parity = 1 even parity =0

''WZ- Z = ((D & !S) == 0)

CON

_clkfreq = 200_000_000  ''Debug must be enabled clock must be greater than 10MHZ for Debug

P0 =0 , P1 = 1  , P2 = 2

VAR

Byte cogRunning 'cog ID started is returned or -1 if not started

PUB main()

debug("--------------------------------------")

debug("TESTN D,{#}S {WC/WZ/WCZ}")

debug("Test D with !S. C = parity of (D & !S). Z = ((D & !S) == 0).")

debug("--------------------------------------")

cogRunning := COGINIT(COGEXEC_NEW,@S0_TESTNDS,0)

debug(udec(cogRunning))

repeat                                    'keep cog 0 running

DAT

ORG   0

S0_TESTNDS            DRVH    #P0             'P0 on program running

MOV     Dest,#%10101011

MOV     Src,MIN

MODCZ   _Clr, _Clr  WCZ 'C = Z = 0

IF_C   MOV     Carry,#1        'true  C = 1

IF_NC  MOV     Carry,#0        'false C = 0

IF_Z   MOV     Zero,#1         'true  Z = 1

IF_NZ  MOV     Zero,#0         'false Z = 0

debug("--------------------------------------")

debug("before TESTN ",ubin(Dest),ubin(Src),ubin(Carry),ubin(Zero))

TESTN   Dest,Src     WCZ

IF_C   MOV     Carry,#1         'true  C = 1

IF_NC  MOV     Carry,#0         'false C = 0

IF_Z   MOV     Zero,#1          'true  Z = 1

IF_NZ  MOV     Zero,#0          'false Z = 0

debug("after TESTN  ",ubin(Dest),ubin(Src),ubin(Carry),ubin(Zero))

debug("--------------------------------------")

MOV     Dest,valDest

MOV     Src,valSrc

MODCZ   _Clr, _Clr  WCZ 'set C = Z = 0

IF_C   MOV     Carry,#1        'true  C = 1

IF_NC  MOV     Carry,#0        'false C = 0

IF_Z   MOV     Zero,#1         'true  Z = 1

IF_NZ  MOV     Zero,#0         'false Z = 0

debug("before TESTN  ",ubin(Dest),ubin(Src),ubin(Carry),ubin(Zero))

TESTN   Dest,Src    WCZ

IF_C   MOV     Carry,#1         'true  C = 1

IF_NC  MOV     Carry,#0         'false C = 0

IF_Z   MOV     Zero,#1          'true  Z = 1

IF_NZ  MOV     Zero,#0          'false Z = 0

debug("after TESTN  ",ubin(Dest),ubin(Src),ubin(Carry),ubin(Zero))

debug("--------------------------------------")

_Loop1                NOP

JMP     #_Loop1          'remember # imediate

valDest                      long      %00000000_00000000_00000000_00001111

valSrc                         long      %11111111_11111111_11111111_11110100

valNeg1000              long      -1000

valNeg2                     long      -2

valFFFF_FFFA           long      $FFFF_FFFA

val0000_00FA          long      $0000_00FA

Carry                          long      0

Zero                            long      0

Dest                            long      0

Src                               long      0

MAX                            long      $FFFF_FFFF

MIN                              long      0

## 17.25) SETNIB  Set Nibble

| SETNIB D,{#}S,#N | Set S[3:0] into nibble N in D, keeping rest of D same. |
| --- | --- |
| SETNIB {#}S | Set S[3:0] into nibble established by prior ALTSN instruction. |

S[31:0] = N7N6N5N4N3N2N1N0  N7 =  n73n72n71n70  N6 =  n63n62n61n60   N5 = n53n52n51n50

N7 =  n73n72n71n70  N6 =  n63n62n61n60   N5 =  n53n52n51n50   N4 = n43n42n41n40

N3 =  n33n32n31n30  N2 =  n23n22n21n20   N5 =  n13n12n11n10   N0 = n03n02n01n00

The idea is that D/ can serve as a register base address and D can be used as an index.

ALTSN (offset + N field),BaseAddress

Next Instruction :

D Field = (D[11:3} + S)&1FF    N Field = D[2:0]

ALTSN D,{#}S

Alter subsequent SETNIB instruction. Next D field = (D[11:3] + S) & $1FF, N field = D[2:0].

D += sign-extended S[17:9].

1) D field = (D[11:3] + S) & $1FF register to have SETNIB D,{#}S,#N point too

2) S is the BaseAddress and  D[11:3] is the offset from the Base

3) N field =  D[2:0] nibble to write too in SETNIB D,{#}S,#N

### 17.25.1_Example_WRD_SETNIBDS_084

SETNIB D,{#}S,#N

Set S[3:0] into nibble N in D, keeping rest of D same.

The nibble Number N is a fixed value and cannot be indexed

{{17.25.1_Example_WRD_SETNIBDS_084}}

''SETNIB D,{#}S,#N

''Set value for niblle using S[3:0] for nibble N in D, keeping rest of D same.

''S[31:0] = N7N6_N5N4_N3N2_N1N0

''N7 =  n73n72n71n70  N6 =  n63n62n61n60   N5 =  n53n52n51n50   N4 = n43n42n41n40

''N3 =  n33n32n31n30  N2 =  n23n22n21n20   N1 =  n13n12n11n10   N0 = n03n02n01n00

CON

_clkfreq = 200_000_000  ''Debug must be enabled clock must be greater than 10MHZ for Debug

P0 =0 , P1 = 1  , P2 = 2

VAR

Byte cogRunning 'cog ID started is returned or -1 if not started

PUB main()

debug("--------------------------------------")

debug("SETNIB D,{#}S,#N")

debug("Set S[3:0] into nibble N in D, keeping rest of D same.")

debug("--------------------------------------")

cogRunning := COGINIT(COGEXEC_NEW,@S0_SETNIBDSN,0)

debug(udec(cogRunning))

repeat                                    'keep cog 0 running

DAT

ORG   0

S0_SETNIBDSN          DRVH    #P0             'P0 on program running

MOV     Dest,MIN

MOV     Src,#%1111

debug("--------------------------------------")

debug("before SETNIB ",ubin(Dest),ubin(Src))

SETNIB   Dest,Src,#0

debug("after SETNIBN  ",ubin(Dest),ubin(Src))

debug("--------------------------------------")

MOV     Dest,MIN

MOV     Src,#%1111

debug("before TESTN  ",ubin(Dest),ubin(Src))

SETNIB  Dest,Src,#7

debug("after TESTN   ",ubin(Dest),ubin(Src))

debug("--------------------------------------")

_Loop1          NOP

JMP     #_Loop1          'remember # imediate

Dest                long      0

Src                   long      0

MAX                long      $FFFF_FFFF

MIN                 long      0

### 17.25.2_Example_WRD_ALTSN D_SETNIB {#}S_085

SETNIB {#}S

Set S[3:0] into nibble established by prior ALTSN instruction.

This allows the Indexing of the nibble Number N

Example

Set up the SETNIB {#}S Instruction to write to register PR0 nibble N7-N0

Solution  D[11:3] = PR0 = $1D8 = 472 = %111011000 D[2:0] = $7 = 7 =%111

Dest     = %00000000_00000000_0000_111011000_111

= %00000000_00000000_00001110_11000111

= $EC7

= 3783

ALTSN Dest

{{17.25.2_Example_WRD_SETNIBS_085}}

''SETNIB {#}S

''Set S[3:0] into nibble established by prior ALTSN instruction.

''ALTSN   D

''Alter subsequent SETNIB instruction. Next D field = D[11:3], N field = D[2:0].

''S[31:0] = N7N6_N5N4_N3N2_N1N0

''N7 =  n73n72n71n70  N6 =  n63n62n61n60   N5 =  n53n52n51n50   N4 = n43n42n41n40

''N3 =  n33n32n31n30  N2 =  n23n22n21n20   N1 =  n13n12n11n10   N0 = n03n02n01n00

''PR0=$1D8 PR1=$1D9 PR2=$1DA PR3=$1DB PR4=$1DC PR5=$1DD PR6=$1DE PR7=$1DF

{{

Example

Set up the SETNIB {#}S Instruction to write to register PR0 nibble N7-N0

Solution  D[11:3] = PR0 = $1D8 = 472 = %111011000 D[2:0] = $7 = 7 =%111

Dest     = %00000000_00000000_0000_111011000_111

= %00000000_00000000_00001110_11000111

= $EC7

= 3783

ALTSN Dest

}}

CON

_clkfreq = 200_000_000  ''Debug must be enabled clock must be greater than 10MHZ for Debug

P0 =0 , P1 = 1  , P2 = 2

VAR

Byte cogRunning 'cog ID started is returned or -1 if not started

PUB main()

debug("--------------------------------------")

debug("ALTSN   D")

debug("Alter subsequent SETNIB instruction.")

debug("Next D field = D[11:3], N field = D[2:0].")

debug("--------------------------------------")

debug("--------------------------------------")

debug("SETNIB {#}S")

debug("Set S[3:0] into nibble established by prior ALTSN instruction.")

debug("--------------------------------------")

cogRunning := COGINIT(COGEXEC_NEW,@S0_SETNIBS,0)

debug(udec(cogRunning))

repeat                                    'keep cog 0 running

DAT

ORG   0

S0_SETNIBS    DRVH    #P0             'P0 on program running

MOV  PR0,valPR0         'load test nibble words

_nibbleLoop   NOP

debug("--------------------------------------")

debug("before ALTSN/SETNIB ",ubin(Src),ubin(Dest),uhex(PR0),ubin(PR0))

MOV Dest,addressPR0

SHL Dest,#3

OR  Dest,nibbleNum

ALTSN Dest

SETNIB Src

debug("after ALTSN/SETNIB  ",ubin(Src),ubin(Dest),uhex(PR0),ubin(PR0))

debug("--------------------------------------")

debug(udec(nibbleNum))

SUB    nibbleNum,#1

MODC _clr WC

CMP  nibbleNum,valNeg1 WZ

IF_NZ   JMP  #_nibbleLoop    'remember # imediate

IF_Z    JMP  #_LoopEnd       'remember # imediate

_LoopEnd        NOP

JMP     #_LoopEnd    'remember # imediate

Dest                  long      0

Src                     long      %1010

valPR0               long      %11110000_11110000_11110000_11110000

addressPR0      long      %111011000 'Register Address of PR0 = $1D8

nibbleNum       long      7

valNeg1            long     -1

### 17.25.3_Example_WRD_ALTSN D,{#},#N_SETNIB D,{#}S#N_084

SETNIB D,{#}S,#N

Set S[3:0] into nibble N in D, keeping rest of D same.

The nibble Number N is a fixed value and cannot be indexed

1) D is set by ALTSN D,{#}S where D is an indexed possible register to be written too.

2) S[3:0] is the nibble value to be written. It is not from ALTSN D,{#}S

3) N is nibble to be written set by ALTSN D,{#}S

ALTSN D,{#}S

Alter subsequent SETNIB instruction. Next D field = (D[11:3] + S) & $1FF, N field = D[2:0].

D += sign-extended S[17:9].

1) D field = (D[11:3] + S) & $1FF register to have SETNIB D,{#}S,#N point too

2) S is the BaseAddress and  D[11:3] is the offset from the Base

3) N field =  D[2:0] nibble to write too in SETNIB D,{#}S,#N

{{17.25.3_Example_WRD_SETNIBDS_084}}

{{

SETNIB D,{#}S,#N

Set S[3:0] into nibble N in D, keeping rest of D same.

The nibble Number N fixed value and cannot be indexed unless ALTSN is used

1) D is set by ALTSN D,{#}S where D is an indexed possible register to be written too.

2) S[3:0] is the nibble value to be written. It is not from ALTSN D,{#}S

3) N is nibble to be written set by ALTSN D,{#}S

ALTSN D,{#}S

Alter subsequent SETNIB instruction. Next D field = (D[11:3] + S) & $1FF, N field = D[2:0].

D += sign-extended S[17:9].

1) D field = (D[11:3] + S) & $1FF register to have SETNIB D,{#}S,#N point too

2) D[11:3] is offset from Base register S

3) N field =  D[2:0] nibble to write too in SETNIB D,{#}S,#N

S[31:0] = N7N6_N5N4_N3N2_N1N0

N7 =  n73n72n71n70  N6 =  n63n62n61n60   N5 =  n53n52n51n50   N4 = n43n42n41n40

N3 =  n33n32n31n30  N2 =  n23n22n21n20   N1 =  n13n12n11n10   N0 = n03n02n01n00

PR0=$1D8 PR1=$1D9 PR2=$1DA PR3=$1DB PR4=$1DC PR5=$1DD PR6=$1DE PR7=$1DF

Example

Write to PR7 the Value %1010 at niblle N7 using S baseAddress of  PR0 and offset of 7

BaseAddressS = $1D8   Offset to PR0 D[3:11] = 7  Nibble to write to D[2:0] = 7

}}

CON

_clkfreq = 200_000_000  ''Debug must be enabled clock must be greater than 10MHZ for Debug

P0 =0 , P1 = 1  , P2 = 2

VAR

Byte cogRunning 'cog ID started is returned or -1 if not started

PUB main()

debug("--------------------------------------")

debug("ALTSN D,{#}S ")

debug("Alter subsequent SETNIB instruction.")

debug("Next D field = (D[11:3] + S) & $1FF, N field = D[2:0].")

debug("D += sign-extended S[17:9].")

debug("--------------------------------------")

debug("--------------------------------------")

debug("SETNIB D,{#}S,#N ")

debug("Set S[3:0] into nibble N in D, keeping rest of D same.")

debug("The nibble Number N fixed value and cannot be indexed unless ALTSN is used")

debug("--------------------------------------")

cogRunning := COGINIT(COGEXEC_NEW,@S0_SETNIBDS,0)

debug(udec(cogRunning))

repeat                                    'keep cog 0 running

DAT

ORG   0

S0_SETNIBDS            DRVH    #P0             'P0 on program running

MOV  PR7,valPreset       'write nibble to PR7

_nibbleLoop           NOP

debug("--------------------------------------")

debug("before D_ALTSN Loade ",ubin(Src),ubin(D_ALTNS),uhex(PR7),ubin(PR7))

MOV D_ALTNS,addressOffset    'offset from PR0($1D8) to PR7($1DF) value 7

SHL D_ALTNS,#3               'shift offset to D field D[11:3]

OR  D_ALTNS,nibbleNum        'set the nibble N7 to be loaded to PR7

debug("after D_ALTSN Loaded")

debug("and before ALTSN",ubin(Src),ubin(D_ALTNS),uhex(PR7),ubin(PR7))

ALTSN D_ALTNS,S_ALTNS

SETNIB Src

debug("before ALTSN/SETNIB ",ubin(Src),ubin(D_ALTNS),uhex(PR7),ubin(PR7))

_LoopEnd              NOP

JMP     #_LoopEnd    'remember # imediate

valPreset        long      %11110000_11110000_11110000_11110000

addressBase      long      $1D8  'Register Address of PR0 = $1D8 = %111011000

addressOffset    long      7

nibbleNum        long      7

S_ALTNS          long      $1D8     'PR0

D_ALTNS          long      0

Src              long      %1010

## 17.26)GETNIB Getnibble from register

| GETNIB D,{#}S,#N | Get nibble N of S into D. D = {28'b0, S.NIBBLE[N]). |
| --- | --- |
| GETNIB D | Get nibble established by prior ALTGN instruction into D. |

S[31:0] = N7N6N5N4N3N2N1N0

N7 =  n73n72n71n70  N6 =  n63n62n61n60   N5 =  n53n52n51n50   N4 = n43n42n41n40

N3 =  n33n32n31n30  N2 =  n23n22n21n20   N5 =  n13n12n11n10   N0 = n03n02n01n00

GETNIB D,{#}S,#N

Get nibble N of S into D. D = D= {28'b0, S[N4+3:N4]}

1) N is nibble number 0-7 in S that is to be moved to D N0

2) 28’b0 stands for 28 bits of type 0 the {,} stands for concatenate (join)

3)  S.NIBBLE[N] Is the nibble poition with right to left level of signifigance .(B3B2B1B0)

Note: D = D= {28'b0, S[N4+3:N4]} this is Verilog notation see section E.4

GETNIB D

Get nibble established by prior ALTGN instruction into D.

ALTGN D,{#}S (104)

Alter subsequent GETNIB/ROLNIB instruction. Next S field = (D[11:3] + S) & $1FF, N field = D[2:0]. D += sign-extended S[17:9].

1) D field = (D[11:3] + S) & $1FF register to have SETNIB D,{#}S,#N point too

2) S is the BaseAddress and  D[11:3] is the offset from the Base

3) N field =  D[2:0] nibble to write too in GETNIB D,{#}S,#N

ALTGN D (105)

Alter subsequent GETNIB/ROLNIB instruction. Next S field = D[11:3], N field = D[2:0].

### 17.26.1_EXAMPLE_WRD_GETNIB D,{#}S,#N_086

Get nibble N of S into D. D = D= {28'b0, S[N4+3:N4]}

{{17.26.1_Example_WRD_GETNIB D,{#}S,#N_086}}

{{

GETNIB D,{#}S,#N

Get nibble N of S into D. D = {28'b0, S[N*4+3:N*4]}

1) N nibble of N7N6N5N4N3N2N1N0

2) S register with nibbles required

3) D is target register of the nibble

N7 =  n73n72n71n70  N6 =  n63n62n61n60   N5 =  n53n52n51n50   N4 = n43n42n41n40

N3 =  n33n32n31n30  N2 =  n23n22n21n20   N1 =  n13n12n11n10   N0 = n03n02n01n00

Example

Get Nibble N7 in register Src and move to Dest register

}}

CON

_clkfreq = 200_000_000  ''Debug requires clock greater than 10MHZ

P0 =0 , P1 = 1  , P2 = 2

VAR

Byte cogRunning 'cog ID started is returned or -1 if not started

PUB main()

debug("--------------------------------------")

debug("GETNIB D,{#}S,#N")

debug("Get nibble N of S into D. D = {28'b0, S[N*4+3:N*4]}")

debug(" ")

debug("Example")

debug("Get Nibble N7 in register Src and move to Dest register Nibble N0")

debug("--------------------------------------")

cogRunning := COGINIT(COGEXEC_NEW,@S0_GETNIBDSN,0)

debug(udec(cogRunning))

repeat                                    'keep cog 0 running

DAT

ORG   0

S0_GETNIBDSN          DRVH    #P0                 'P0 on program running

MOV     Dest,#0             'Result for nibble transfer

MOV     Src,valNibbleWord   'register holding nibble to get

debug("--------------------------------------")

debug("before GETNIB D,S,#N ",ubin(Dest),ubin(Src))

GETNIB Dest,Src,#7

debug("after GETNIB D,S,#N  ",ubin(Dest),ubin(Src))

debug("--------------------------------------")

_Loop1                NOP

JMP     #_Loop1          'remember # imediate

Dest                  long      0

Src                   long      0

valNibbleWord         long      %01110110_01010100_00110010_00010000 '$76_54_32_10

nibbleNum             long      7

### 17.26.2_EXAMPLE_GETNIB D_ 087

GETNIB D

Get nibble established by prior ALTGN instruction into D.

ALTGN D,{#}S

Alter subsequent GETNIB/ROLNIB instruction.

Next S field = (D[11:3] + S) & $1FF, N field = D[2:0]. D += sign-extended S[17:9].

{{17.26.2_Example_WRD_GETNIB 087}}

{{

GETNIB D

Get nibble established by prior ALTGN instruction into D.

1) D is target register of the nibble

ALTGN D,{#}S

Alter subsequent GETNIB/ROLNIB instruction.

Next S field = (D[11:3] + S) & $1FF, N field = D[2:0]. D += sign-extended S[17:9].

1) S is the addressBase D[11:3]

2) S Field = (D[11:3] + S) & $1FF, D[11:3] is the offset index from addressBase

3) N Field = D[2:0]

N7 =  n73n72n71n70  N6 =  n63n62n61n60   N5 =  n53n52n51n50   N4 = n43n42n41n40

N3 =  n33n32n31n30  N2 =  n23n22n21n20   N1 =  n13n12n11n10   N0 = n03n02n01n00

Example

Get Nibble N7 in register Src and move to Dest register N0

}}

CON

_clkfreq = 200_000_000  ''Debug clock must be greater than 10MHZ

P0 =0 , P1 = 1  , P2 = 2

VAR

Byte cogRunning 'cog ID started is returned or -1 if not started

PUB main()

debug("--------------------------------------")

debug("ALTGN D,{#}S ")

debug("Alter subsequent GETNIB/ROLNIB instruction.")

debug("Next S field = (D[11:3] + S) & $1FF")

debug("N field = D[2:0]. D += sign-extended S[17:9].")

debug(" ")

debug("GETNIB D ")

debug("Get nibble established by prior ALTGN instruction into D.")

debug(" ")

debug("Example")

debug("Get Nibble in N7 register valNibbleWord and move to Nibble N0 D_GETNIB register")

debug("--------------------------------------")

cogRunning := COGINIT(COGEXEC_NEW,@S0_GETNIBD,0)

debug(udec(cogRunning))

repeat                                    'keep cog 0 running

DAT

ORG   0

S0_GETNIBD            DRVH    #P0                      'P0 on program running

NibbleLoop            MOV     S_ALTGN,#valNibbleWord   'register holding nibble to get

MOV     D_GETNIB,#0

debug("--------------------------------------")

MOV   D_ALTGN,#0          'offset

SHL   D_ALTGN,#3          'shift 3 bits for D[11:3]

OR    D_ALTGN,nibbleNum   'Offset = 0 Nibble = N7

debug(ubin(D_GETNIB),ubin(D_ALTGN),ubin(S_ALTGN))

ALTGN D_ALTGN,S_ALTGN

GETNIB D_GETNIB

debug("after  ALTGN D,S/GETNIB D  ",ubin(D_GETNIB),ubin(D_ALTGN),ubin(S_ALTGN))

debug("--------------------------------------")

debug(udec(nibbleNum))

SUB     nibbleNum,#1

SUB     valStep,#1 WZ

IF_Z     JMP     #_Loop1

JMP     #NibbleLoop

_Loop1         NOP

JMP     #_Loop1          'remember # imediate

D_ALTGN                 long      0              'offset =0 to index

S_ALTGN                  long      0

D_GETNIB                long      0

nibbleNum               long      7

valNibbleWord        long      %01110110_01010100_00110010_00010000 '$76_54_32_10

valStep                      long      8

### 17.26.3_Example_WRD_GETNIB D_087

GETNIB D

Get nibble established by prior ALTGN instruction into D.

ALTGN D

Alter subsequent GETNIB/ROLNIB instruction. Next S field = D[11:3], N field = D[2:0].

{{17.26.3_Example_WRD_GETNIB D_087}}

{{

GETNIB D

Get nibble established by prior ALTGN instruction into D.

1) D is target register of the nibble

ALTGN D

Alter subsequent GETNIB/ROLNIB instruction. Next S field = D[11:3], N field = D[2:0].

1) D[11:3] is the addressBase

2) N Field = D[2:0]

N7 =  n73n72n71n70  N6 =  n63n62n61n60   N5 =  n53n52n51n50   N4 = n43n42n41n40

N3 =  n33n32n31n30  N2 =  n23n22n21n20   N1 =  n13n12n11n10   N0 = n03n02n01n00

Example

Get Nibble N7 in register Src and move to Dest register N0

}}

CON

_clkfreq = 200_000_000  ''Debug clock must be greater than 10MHZ

P0 =0 , P1 = 1  , P2 = 2

VAR

Byte cogRunning 'cog ID started is returned or -1 if not started

PUB main()

debug("--------------------------------------")

debug("ALTGN D")

debug("Alter subsequent GETNIB/ROLNIB instruction.")

debug("Next S field = D[11:3], N field = D[2:0]")

debug("N field = D[2:0]. D += sign-extended S[17:9].")

debug(" ")

debug("GETNIB D ")

debug("Get nibble established by prior ALTGN instruction into D.")

debug(" ")

debug("Example")

debug("Get Nibble in N7 register valNibbleWord and move to Nibble N0 D_GETNIB register")

debug("--------------------------------------")

cogRunning := COGINIT(COGEXEC_NEW,@S0_GETNIBD,0)

debug(udec(cogRunning))

repeat                                    'keep cog 0 running

DAT

ORG   0

S0_GETNIBD            DRVH    #P0                      'P0 on program running

NibbleLoop            MOV     S_ALTGN,#valNibbleWord   'register address holding nibble to get

MOV     D_ALTGN,#0

MOV     D_GETNIB,#0

debug("--------------------------------------")

MOV   D_ALTGN,#S_ALTGN    'move base address into D_ALGN

debug(ubin(D_GETNIB),ubin(D_ALTGN),ubin(S_ALTGN))

SHL   D_ALTGN,#3          'shift 3 bits for D[11:3]

OR    D_ALTGN,nibbleNum   'base address of valNibbleWord + Nibble = N7

debug(ubin(D_GETNIB),ubin(D_ALTGN),ubin(S_ALTGN))

ALTGN   D_ALTGN

GETNIB  D_GETNIB

debug("after  ALTGN D/GETNIB D  ",ubin(D_GETNIB),ubin(D_ALTGN),ubin(S_ALTGN),udec(nibbleNum))

SUB     nibbleNum,#1

SUB     valStep,#1 WZ

IF_Z     JMP     #_Loop1

JMP     #NibbleLoop

_Loop1         NOP

JMP     #_Loop1          'remember # imediate

D_ALTGN                  long      0              'offset =0 to index

S_ALTGN                   long      0

D_GETNIB                 long      0

nibbleNum               long      7

valNibbleWord         long      %01110110_01010100_00110010_00010000     '$76_54_32_10

valStep                       long      8

## 17.27) ROLNIB Rotate Nibble

| ROLNIB D,{#}S,#N | Rotate-left nibble N of S into D. D = {D[27:0], S.NIBBLE[N]). |
| --- | --- |
| ROLNIB D | Rotate-left nibble established by prior ALTGN instruction into D. |

S[31:0] = N7N6N5N4N3N2N1N0

N7 =  n73n72n71n70  N6 =  n63n62n61n60   N5 =  n53n52n51n50   N4 = n43n42n41n40

N3 =  n33n32n31n30  N2 =  n23n22n21n20   N5 =  n13n12n11n10   N0 = n03n02n01n00

ROLNIB D,{#}S,#N

Rotate-left nibble N of S into D. D = {D[27:0], S.NIBBLE[N]).

1) N is nibble number 7 in S that is to be moved to D to N0

2) 28’b0 stands for 28 bits of type 0 the {,} stands for concatenate (join)

3)  S.NIBBLE[N] Is the nibble poition with right to left level of signifigance .(B3B2B1B0)

Note: D = D= {28'b0, S[N4+3:N4]} this is Verilog notation see section E.4

ROLNIB D

Rotate-left nibble established by prior ALTGN instruction into D.

ALTGN D,{#}S (104)

Alter subsequent GETNIB/ROLNIB instruction. Next S field = (D[11:3] + S) & $1FF, N field = D[2:0]. D += sign-extended S[17:9].

1) D field = (D[11:3] + S) & $1FF register to have SETNIB D,{#}S,#N point too

2) S is the BaseAddress and  D[11:3] is the offset from the Base

3) N field =  D[2:0] nibble to write too in GETNIB D,{#}S,#N

ALTGN D (105)

Alter subsequent GETNIB/ROLNIB instruction. Next S field = D[11:3], N field = D[2:0].

### 17.27.1_Example_WRD_ROLNIBDSN_088

ROLNIB D,{#}S,#N

Rotate-left nibble N of S into D. D = {D[27:0], S.NIBBLE[N]).

S contains the nibble values the N7 nibble is placed into D nibble N0.

{{17.27.1_Example_WRD_ROLNIBDSN_088}}

{{

17.27.1_Example_WRD_ROLNIBDSN_088

ROLNIB D,{#}S,#N

Rotate-left nibble N of S into D. D = {D[27:0], S.NIBBLE[N]).

N7 =  n73n72n71n70  N6 =  n63n62n61n60   N5 =  n53n52n51n50   N4 = n43n42n41n40

N3 =  n33n32n31n30  N2 =  n23n22n21n20   N1 =  n13n12n11n10   N0 = n03n02n01n00

Example

Get Nibble N7 in register Src and move to Dest register

}}

CON

_clkfreq = 200_000_000  ''Debug clock must be greater than 10MHZ

P0 =0 , P1 = 1  , P2 = 2

VAR

Byte cogRunning 'cog ID started is returned or -1 if not started

PUB main()

debug("--------------------------------------")

debug("ROLNIB D,{#}S,#N ")

debug("Rotate-left nibble N of S into D.")

debug("D = {D[27:0], S.NIBBLE[N]).")

debug("Example")

debug("Get Nibble N7 in register Src and move to Dest register")

debug("--------------------------------------")

cogRunning := COGINIT(COGEXEC_NEW,@S0_ROLNIBDSN,0)

debug(udec(cogRunning))

repeat                                    'keep cog 0 running

DAT

ORG   0

S0_ROLNIBDSN          DRVH    #P0                 'P0 on program running

MOV     Dest,#0             'Result for nibble transfer

MOV     Src,valNibbleWord   'register holding nibble to get

debug("--------------------------------------")

debug("before ROLNIB D,{#}S,#N  ",ubin(Dest),ubin(Src))

GETNIB Dest,Src,#7           '7 is the nibble number

debug("after  ROLNIB D,{#}S,#N  ",ubin(Dest),ubin(Src))

debug("--------------------------------------")

_Loop1         NOP

JMP     #_Loop1          'remember # imediate

Dest                           long      0

Src                              long      0

valNibbleWord         long      %01110110_01010100_00110010_00010000   '$76_54_32_10

nibbleNum                long      7

### 17.27.2_Example_WRD_ROLNIBD_089

ROLNIB D (089)

Rotate-left nibble established by prior ALTGN instruction into D.

1) D is the register that will contain nibble pointed from ALTGN statement

ALTGN D,{#}S (104)

Alter subsequent GETNIB/ROLNIB instruction. Next S field = (D[11:3] + S) & $1FF, N field = D[2:0]. D += sign-extended S[17:9].

1) D field = (D[11:3] + S) & $1FF register to have SETNIB D,{#}S,#N point too

2) S is the BaseAddress and  D[11:3] is the offset from the Base

3) N field =  D[2:0] nibble to write too in GETNIB D,{#}S,#N

Note:

S with an offset can point to different words holding nibbles and the N field can point different nibbles in the different words. The Nibble is always in the ROLNIB D register.

{{17.27.2_Example_WRD_ROLNIB_D_089}}

{{

ROLNIB D (089)

Rotate-left nibble established by prior ALTGN instruction into D.

1) D is the register that will contain nibble pointed from ALTGN statement

ALTGN D,{#}S

Alter subsequent GETNIB/ROLNIB instruction.

Next S field = (D[11:3] + S) & $1FF, N field = D[2:0]. D += sign-extended S[17:9].

1) S is the addressBase D[11:3]

2) S Field = (D[11:3] + S) & $1FF, D[11:3] is the offset index from addressBase

3) N Field = D[2:0]

N7 =  n73n72n71n70  N6 =  n63n62n61n60   N5 =  n53n52n51n50   N4 = n43n42n41n40

N3 =  n33n32n31n30  N2 =  n23n22n21n20   N1 =  n13n12n11n10   N0 = n03n02n01n00

Example

Get Nibble N7 in register Src and move to Dest register N0

}}

CON

_clkfreq = 200_000_000  ''Debug clock must be greater than 10MHZ

P0 =0 , P1 = 1  , P2 = 2

VAR

Byte cogRunning 'cog ID started is returned or -1 if not started

PUB main()

debug("--------------------------------------")

debug("ALTGN D,{#}S ")

debug("Alter subsequent GETNIB/ROLNIB instruction.")

debug("Next S field = (D[11:3] + S) & $1FF")

debug("N field = D[2:0]. D += sign-extended S[17:9].")

debug(" ")

debug("ROLNIB D ")

debug("Rotate-left nibble established by prior ALTGN instruction into D.")

debug("---------------------------------------")

debug("Example")

debug("Get Nibble in N7 register Src and move to N0 Dest register")

debug("--------------------------------------")

cogRunning := COGINIT(COGEXEC_NEW,@S0_ROLNIBD,0)

debug(udec(cogRunning))

repeat                                    'keep cog 0 running

DAT

ORG   0

S0_ROLNIBD            DRVH    #P0                      'P0 on program running

NibbleLoop            debug("--------------------------------------")

MOV   S_ALTGN,#valNibbleWord0   'register holding nibble to get

MOV   D_ROLNIB,#0               'clear D_ROLNIB

MOV   D_ALTGN,#0                'no offset use #valNibbleWord0 base address

SHL   D_ALTGN,#3                'shift 3 bits for D[11:3]

OR    D_ALTGN,#7         'base address of valNibbleWord + Nibble = N7

debug("before ALTGN D,S/ROLNIB D   ",ubin(D_ALTGN),ubin(S_ALTGN))

ALTGN D_ALTGN,S_ALTGN

ROLNIB D_ROLNIB

debug("after ALTGN/ROLNIB offset 0  ",ubin(D_ALTGN),ubin(S_ALTGN),ubin(D_ROLNIB))

debug("--------------------------------------")

debug("--------------------------------------")

MOV   S_ALTGN,#valNibbleWord0   'register holding nibble to get

MOV   D_ROLNIB,#0                'clear D_ROLNIB

MOV   D_ALTGN,#6                 'base address + offset of 6

SHL   D_ALTGN,#3                 'shift 3 bits for D[11:3]

OR    D_ALTGN,#6                 'offset of 7 address valNibbleWord + Nibble = N7

debug("before ALTGN D,S/ROLNIB D   ",ubin(D_ALTGN),ubin(S_ALTGN))

ALTGN D_ALTGN,S_ALTGN

ROLNIB D_ROLNIB

debug("after ALTGN/ROLNIB offset 6  ",ubin(D_ALTGN),ubin(S_ALTGN),ubin(D_ROLNIB))

debug("-------------------------------------")

_Loop1                NOP

JMP     #_Loop1          'remember # imediate

D_ALTGN               long      0

S_ALTGN               long      0

D_ROLNIB              long      0

valNibbleWord0        long      $76_54_32_10  '%01110110_01010100_00110010_00010000

valNibbleWord1        long      $65_43_21_07

valNibbleWord2        long      $54_32_10_67

valNibbleWord3        long      $43_21_05_67

valNibbleWord4        long      $32_10_45_67

valNibbleWord5        long      $21_03_45_67

valNibbleWord6        long      $10_23_45_67

valNibbleWord7        long      $01_23_45_67

### 17.27.3_Example_WRD_ROLNIBDSN_ ALTGNDS_088

ROLNIB D,{#}S,#N

Rotate-left nibble N of S into D. D = {D[27:0], S.NIBBLE[N]).

ALTGN D,{#}S (104)

Alter subsequent GETNIB/ROLNIB instruction. Next S field = (D[11:3] + S) & $1FF, N field = D[2:0]. D += sign-extended S[17:9].

1) D field = (D[11:3] + S) & $1FF register to have SETNIB D,{#}S,#N point too

2) S is the BaseAddress and  D[11:3] is the offset from the Base

3) N field =  D[2:0] nibble to write too in GETNIB D,{#}S,#N

Note:

S with an offset can point to different words holding nibbles and the N field can point different nibbles in the different words. The Nibble is always in the ROLNIB D register.

{{17.27.3_Example_WRD_ROLNIBDSN_ALTGNDS_088}}

{{

ROLNIB D,{#}S,#N

Rotate-left nibble N of S into D. D = {D[27:0], S.NIBBLE[N]).

ALTGN D,{#}S

Alter subsequent GETNIB/ROLNIB instruction.

Next S field = (D[11:3] + S) & $1FF, N field = D[2:0]. D += sign-extended S[17:9].

1) S is the addressBase D[11:3]

2) S Field = (D[11:3] + S) & $1FF, D[11:3] is the offset index from addressBase

3) N Field = D[2:0]

N7 =  n73n72n71n70  N6 =  n63n62n61n60   N5 =  n53n52n51n50   N4 = n43n42n41n40

N3 =  n33n32n31n30  N2 =  n23n22n21n20   N1 =  n13n12n11n10   N0 = n03n02n01n00

Note:

S with an offset can point to different words holding nibbles and

the N field can point different nibbles in the different words. The Nibble is always in the ROLNIB D register.

Example

Get Nibble N7 in register Src and move to Dest register

}}

CON

_clkfreq = 200_000_000  ''Debug clock must be greater than 10MHZ

P0 =0 , P1 = 1  , P2 = 2

VAR

Byte cogRunning 'cog ID started is returned or -1 if not started

PUB main()

debug("--------------------------------------")

debug("ALTGN D,{#}S  ")

debug("Alter subsequent GETNIB/ROLNIB instruction. ")

debug("Next S field = (D[11:3] + S) & $1FF, N field = D[2:0]. D += sign-extended S[17:9].")

debug(" ")

debug("ROLNIB D,{#}S,#N ")

debug("Rotate-left nibble N of S into D. D = {D[27:0], S.NIBBLE[N]).")

debug(" ")

debug("Example")

debug("Get Nibble N7 in register valNibbelWord0 and move to D_ROLNIB register")

debug("Get Nibble N7 in register valNibbelWord0 +  and move to D_ROLNIB register")

debug("--------------------------------------")

cogRunning := COGINIT(COGEXEC_NEW,@S0_ROLNIBDSN,0)

debug(udec(cogRunning))

repeat                                    'keep cog 0 running

DAT

ORG   0

S0_ROLNIBDSN          DRVH    #P0                 'P0 on program running

debug("--------------------------------------")

MOV   S_ALTGN,#valNibbleWord0   'register holding Base Address to get

MOV   D_ROLNIB,#0               'clear D_ROLNIB

Offset_0              MOV   D_ALTGN,#0                'no offset use #valNibbleWord0 base address

SHL   D_ALTGN,#3                'shift 3 bits for D[11:3]

OR    D_ALTGN,#7                'base address of valNibbleWord + Nibble = N7

debug("before ALTGN D,{#}S/ROLNIB D,{#}S,#N  ",ubin(D_ALTGN),ubin(S_ALTGN))

debug(ubin(D_ROLNIB),ubin(S_ROLNIB))

ALTGN  D_ALTGN,S_ALTGN

ROLNIB D_ROLNIB,S_ROLNIB,#7

debug("after  ALTGN D,{#}S/ROLNIB D,{#}S,#N  ",ubin(D_ALTGN),ubin(S_ALTGN))

debug(ubin(D_ROLNIB),ubin(S_ROLNIB))

debug("--------------------------------------")

debug("--------------------------------------")

MOV   S_ALTGN,#valNibbleWord0   'register holding Base Address to get

MOV   D_ROLNIB,#0               'clear D_ROLNIB

Offset_6              MOV   D_ALTGN,#6                'offset of 6 use #valNibbleWord6 base address

SHL   D_ALTGN,#3                'shift 3 bits for D[11:3]

OR    D_ALTGN,#7                'base address of valNibbleWord + Nibble = N7

debug("before ALTGN D,{#}S/ROLNIB D,{#}S,#N  ",ubin(D_ALTGN),ubin(S_ALTGN))

debug(ubin(D_ROLNIB),ubin(S_ROLNIB))

ALTGN  D_ALTGN,S_ALTGN

ROLNIB D_ROLNIB,S_ROLNIB,#7

debug("after  ALTGN D,{#}S/ROLNIB D,{#}S,#N  ",ubin(D_ALTGN),ubin(S_ALTGN))

debug(ubin(D_ROLNIB),ubin(S_ROLNIB))

debug("--------------------------------------")

_Loop1                NOP

JMP     #_Loop1          'remember # imediate

D_ALTGN               long      0

S_ALTGN                long      0

D_ROLNIB              long      0

S_ROLNIB                   long      0

valNibbleWord0        long      $76_54_32_10  '%01110110_01010100_00110010_00010000

valNibbleWord1        long      $65_43_21_07

valNibbleWord2        long      $54_32_10_67

valNibbleWord3        long      $43_21_05_67

valNibbleWord4        long      $32_10_45_67

valNibbleWord5        long      $21_03_45_67

valNibbleWord6        long      $10_23_45_67

valNibbleWord7        long      $01_23_45_67

## 17.28) SETBYTE  Set Byte N into Register

| SETBYTE D,{#}S,#N | Set S[7:0] into byte N in D, keeping rest of D same. |
| --- | --- |
| SETBYTE {#}S | Set S[7:0] into byte established by prior ALTSB instruction. |

Byte Addressing

D[31:0] = D3D2D1D0

Bit Addressing

D[31:0] =d31d30d29d28d27d26d25d24_d23d22d21d20d19d18d17d16_d15d14d13d12d11d10d09d08_d07d06d05d04d03d02d01d00

Byte Addressing

S[31:0] = S3S2S1S0

Byte Bit Addressing

S[31:0] = S37S36S35S34S33S32S31S30_ S27S26S25S24S23S22S21S20_ S17S16S15S14S13S12S11S10_ S07S06S05S04S03S02S01S00

Next D Field D[10:2] =  d10d09d08d07d06d05d04d03d02  9 bit address range $000-$1FF  (0 – 511)

Word Field   D[1:0]   =   d01d00                                                              2 bit address range 0-3

### 17.28.1_Example_WRD_SETBYTE D,{#}S,#N_090

SETBYTE D,{#}S,#N

Set S[7:0] into byte N in D, keeping rest of D same.

{{17.28.1_Example_WRD_SETBYTE D,{#}S,#N_090}}

{{

SETBYTE D,{#}S,#N

Set S[7:0] into byte N in D, keeping rest of D same.

Byte Addressing

D[31:0] = D3D2D1D0

= d31d30d29d28d27d26d25d24_d23d2 2d21d20d19d18d17d16_d15d14d13d12d11d10d09d08_d07d06d05d04d03d02d01d00

Byte Addressing

S[31:0] = S3S2S1S0

= S37S36S35S34S33S32S31S30_ S27S26S25S24S23S22S21S20_ S17S16S15S14S13S12S11S10_ S07S06S05S04S03S02S01S00

Example

Set S[7:0] = %10101010 and move this byte into B2 in D

}}

CON

_clkfreq = 200_000_000  ''Debug clock must be greater than 10MHZ

P0 =0 , P1 = 1  , P2 = 2

VAR

Byte cogRunning 'cog ID started is returned or -1 if not started

PUB main()

debug("--------------------------------------")

debug("SETBYTE D,{#}S,#N ")

debug("Set S[7:0] into byte N in D, keeping rest of D same.")

debug(" ")

debug("Example")

debug("Set S[7:0] = %10101010 and move this byte into B3 in D")

debug("--------------------------------------")

cogRunning := COGINIT(COGEXEC_NEW,@S0_SETBYTE,0)

debug(udec(cogRunning))

repeat                                    'keep cog 0 running

DAT

ORG   0

S0_SETBYTE            DRVH    #P0                      'P0 on program running

debug("--------------------------------------")

MOV   S_SETBYTE,valByte

debug("before SETBYTE D,S,#N ",ubin(D_SETBYTE),ubin(S_SETBYTE))

SETBYTE D_SETBYTE,S_SETBYTE,#2    'B2 to be loaded

debug("after SETBYTE D,S,#N ",ubin(D_SETBYTE),ubin(S_SETBYTE))

debug("---------------------------------------")

_Loop1         NOP

JMP     #_Loop1          'remember # imediate

D_SETBYTE            long      %11111111_00000000_11111111_00000000

S_SETBYTE             long      0

valByte                   long      %10101010

### 17.28.2_Example_WRD_SETBYTE {#}S_091

SETBYTE {#}S

Set S[7:0] into byte established by prior ALTSB instruction.

ALTSB D

Alter subsequent SETBYTE instruction. Next D field = D[10:2], N field = D[1:0].

D[31:0] = D3D2D1D0

= d31 d30 d29d28d27d26d2 5d24_d23d2 2d21d20d19d18d17d16_d15d14d13d12d11d10d09d08_d07d06d05d04d03d02d01d00

Next D Field D[10:2] =  d10d09d08d07d06d05d04d03d02  9 bit address range $000-$1FF  (0 – 511)

Word Field   D[1:0]   =   d01d00                                                              2 bit address range 0-3

{{17.28.2_Example_WRD_SETBYTE {#}S_091}}

{{

SETBYTE {#}S

Set S[7:0] into byte established by prior ALTSB instruction.

ALTSB D

Alter subsequent SETBYTE instruction. Next D field = D[10:2], N field = D[1:0].

D contains where S[7:0] byte value is to be written

D[31:0] = D3D2D1D0

= d31d30d29d28d27d26d25d24_d23d2 2d21d20d19d18d17d16_d15d14d13d12d11_d10d09d08d07d06d05d04d03d02_d01d00

Next D Field D[10:2] =  d10d09d08d07d06d05d04d03d02  9 bit address range $000-$1FF  (0 - 511)

Word Field   D[1:0]   =   d01d00                     2 bit address range 0-3

Example

Set S[7:0] = %10101010 and move this byte into D2 in D

}}

CON

_clkfreq = 200_000_000  ''Debug clock must be greater than 10MHZ

P0 =0 , P1 = 1  , P2 = 2

VAR

Byte cogRunning 'cog ID started is returned or -1 if not started

PUB main()

debug("--------------------------------------")

debug("ALTSB D  ")

debug("Alter subsequent SETBYTE instruction. Next D field = D[10:2], N field = D[1:0]")

debug(" ")

debug("SETBYTE {#}S")

debug("Set S[7:0] into byte established by prior ALTSB instruction.")

debug(" ")

debug("Example")

debug("Set S[7:0] = %10101010 and move this byte into D2 in D")

debug("--------------------------------------")

cogRunning := COGINIT(COGEXEC_NEW,@S0_SETBYTE,0)

debug(udec(cogRunning))

repeat                                    'keep cog 0 running

DAT

ORG   0

S0_SETBYTE            DRVH    #P0                      'P0 on program running

debug("--------------------------------------")

MOV   S_SETBYTE,valByte

MOV   D_ALTSB,#destRegister

SHL   D_ALTSB,#2                   'shift for N byte

OR    D_ALTSB,ByteNum              'ByteNum = 0,1,2,3

debug("before SETBYTE {#}S ",ubin(D_ALTSB),ubin(S_SETBYTE),ubin(destRegister))

ALTSB   D_ALTSB

SETBYTE S_SETBYTE    'B2 to be loaded

debug("before SETBYTE {#}S ",ubin(D_ALTSB),ubin(S_SETBYTE),ubin(destRegister))

debug("---------------------------------------")

_Loop1                NOP

JMP     #_Loop1          'remember # imediate

D_ALTSB               long      %11111111_00000000_11111111_00000000

S_SETBYTE             long      0

valByte               long      %10101010

destRegister          long      0

ByteNum               long      2

### 17.28.3_Example_WRD_SETBYTE D,{#}S,#N_ALTSB D,{#}S_090

SETBYTE D,{#}S,#N

Set S[7:0] into byte N in D, keeping rest of D same.

ALTSB D,{#}S

Alter subsequent SETBYTE instruction. Next D field = (D[10:2] + S) & $1FF, N field = D[1:0]. D += sign-extended S[17:9].

{{17.28.3_Example_WRD_SETBYTE D,{#}S,#N_ALTSB D,{#}S_090}}

{{

SETBYTE D,{#}S,#N

Set S[7:0] into byte N in D, keeping rest of D same.

ALTSB D,{#}S

Alter subsequent SETBYTE instruction. Next D field = (D[10:2] + S) & $1FF, N field = D[1:0]. D += sign-extended S[17:9].

Byte Addressing

D[31:0] = D3D2D1D0

= d31d30d29d28d27d26d25d24_d23d22d21d20d19d18d17d16_d15d14d13d12d11d10d09d08_d07d06d05d04d03d02d01d00

Byte Addressing

S[31:0] = S3S2S1S0

= S37S36S35S34S33S32S31S30_ S27S26S25S24S23S22S21S20_ S17S16S15S14S13S12S11S10_ S07S06S05S04S03S02S01S00

Example

Set S[7:0] = %10101010 and move this byte into B2 in D

To test example modify Dest_Offset and Byte_Offset

}}

CON

_clkfreq = 200_000_000  ''Debug clock must be greater than 10MHZ

P0 =0 , P1 = 1  , P2 = 2

VAR

Byte cogRunning 'cog ID started is returned or -1 if not started

PUB main()

debug("--------------------------------------")

debug("ALTSB D,{#}S")

debug("Alter subsequent SETBYTE instruction. Next D field = (D[10:2] + S) & $1FF,")

debug("N field = D[1:0]. D += sign-extended S[17:9]")

debug(" ")

debug("SETBYTE D,{#}S,#N ")

debug("Set S[7:0] into byte N in D, keeping rest of D same.")

debug(" ")

debug("Example")

debug("Set S[7:0] = %10101010 and move this byte into B3 in D")

debug("--------------------------------------")

cogRunning := COGINIT(COGEXEC_NEW,@S0_SETBYTE,0)

debug(udec(cogRunning))

repeat                                    'keep cog 0 running

DAT

ORG   0

S0_SETBYTE            DRVH    #P0                            'P0 on program running

debug("--------------------------------------")

MOV S_ALTSB,#D_ALTSB0                  'BaseAddress

MOV D_ALTSB,Dest_Offset                'Offset from BaseAddress

SHL D_ALTSB,#2                         'shift address for N load

OR  D_ALTSB,Byte_Offset                'write to N = 2

debug("before ALTSB/SETBYTE D,S,#N ",ubin(D_ALTSB),ubin(S_ALTSB))

debug(ubin(D_ALTSB0),ubin(D_ALTSB1),ubin(D_ALTSB2))

debug(ubin(D_ALTSB3),ubin(D_ALTSB4),ubin(D_ALTSB5))

debug(" ")

ALTSB D_ALTSB,S_ALTSB

SETBYTE D_SETBYTE,S_SETBYTE,#0

debug("after  ALTSB/SETBYTE D,S,#N ",ubin(D_ALTSB),ubin(S_ALTSB))

debug(ubin(D_ALTSB0),ubin(D_ALTSB1),ubin(D_ALTSB2))

debug(ubin(D_ALTSB3),ubin(D_ALTSB4),ubin(D_ALTSB5))

debug("---------------------------------------")

_Loop1                NOP

JMP     #_Loop1

Dest_Offset           long      0               'Destination Register D_AlTSB 0-5 = 0-5

Byte_Offset           long      0               'Destination Byter B0 B1 B2 B3 = Byte_Offset = 0 1 2 3

S_SETBYTE             long      %10101010       'Byte value to write to register

D_SETBYTE            long      0

S_ALTSB                 long      0

D_ALTSB                long      0

D_ALTSB0              long      %1000_0000__1111_0000__1111_0000__1111_0000

D_ALTSB1              long      %1001_0000__1111_0000__1111_0000__1111_0000

D_ALTSB2              long      %1010_0000__1111_0000__1111_0000__1111_0000

D_ALTSB3              long      %1011_0000__1111_0000__1111_0000__1111_0000

D_ALTSB4              long      %1100_0000__1111_0000__1111_0000__1111_0000

D_ALTSB5              long      %1101_0000__1111_0000__1111_0000__1111_0000

## 17.29) GETBYTE Get Byte N of S  into D

| GETBYTE D,{#}S,#N | Get byte N of S into D. D = {24'b0, S.BYTE[N]). |
| --- | --- |
| GETBYTE D | Get byte established by prior ALTGB instruction into D. |

### 17.29.1_Example_WRD_GETBYTE D,{#}S,#N_092

GETBYTE D,{#}S,#N

Get byte N of S into D. D = {24'b0, S.BYTE[N]).

{{17.29.1_Example_WRD_GETBYTE D,{#}S,#N_092}}

{{

GETBYTE D,{#}S,#N

Get byte N of S into D. D = {24'b0, S.BYTE[N]).

Byte Addressing

D[31:0] = D3D2D1D0

= d31d30d29d28d27d26d25d24_d23d2 2d21d20d19d18d17d16_d15d14d13d12d11d10d09d08_d07d06d05d04d03d02d01d00

Byte Addressing

S[31:0] = S3S2S1S0

= S37S36S35S34S33S32S31S30_ S27S26S25S24S23S22S21S20_ S17S16S15S14S13S12S11S10_ S07S06S05S04S03S02S01S00

Example

Load S = 1000_0000___1000_0001___1000_0010___

}}

CON

_clkfreq = 200_000_000  ''Debug clock must be greater than 10MHZ

P0 =0 , P1 = 1  , P2 = 2

VAR

Byte cogRunning 'cog ID started is returned or -1 if not started

PUB main()

debug("--------------------------------------")

debug("GETBYTE D,{#}S,#N ")

debug("Get byte N of S into D. D = {24'b0, S.BYTE[N]).")

debug(" ")

debug("Example")

debug("Set S[7:0] = %10101010 and move this byte into B3 in D")

debug("--------------------------------------")

cogRunning := COGINIT(COGEXEC_NEW,@S0_GETBYTE,0)

debug(udec(cogRunning))

repeat                                    'keep cog 0 running

DAT

ORG   0

S0_GETBYTE            DRVH    #P0                      'P0 on program running

debug("--------------------------------------")

MOV   S_GETBYTE,valByte           'load long valByte into S

debug("before GETBYTE D,S,#N ",ubin(D_GETBYTE),ubin(S_GETBYTE))

GETBYTE D_GETBYTE,S_GETBYTE,#3    'N = 0,1,2,3

debug("before GETBYTE D,S,#N ",ubin(D_GETBYTE),ubin(S_GETBYTE))

debug("---------------------------------------")

_Loop1        NOP

JMP     #_Loop1          'remember # imediate

D_GETBYTE             long      0

S_GETBYTE             long      0

valByte               long      %10101010_11111111_11110000_10010010

### 17.29.2_Example_WRD_ALTGB_GETBYTE D_093

GETBYTE D

Get byte established by prior ALTGB instruction into D.

{{17.29.2_Example_WRD_ALTGB_GETBYTE {#}S_093}}

{{

GETBYTE D

Get byte established by prior ALTGB instruction into D.

ALTGB D

Alter subsequent GETBYTE/ROLBYTE instruction. Next S field = D[10:2], N field = D[1:0].

D[31:0] = D3D2D1D0

= d31d30d29d28d27d26d25d24_d23d2 2d21d20d19d18d17d16_d15d14d13d12d11_d10d09d08d07d06d05d04d03d02_d01d00

Next D Field D[10:2] =  d10d09d08d07d06d05d04d03d02  9 bit address range $000-$1FF  (0 - 511)

Word Field   D[1:0]   =   d01d00                     2 bit address range 0-3

Example

Set S[7:0] = %10101010 and move this byte into D2 in D

}}

CON

_clkfreq = 200_000_000  ''Debug clock must be greater than 10MHZ

P0 =0 , P1 = 1  , P2 = 2

VAR

Byte cogRunning 'cog ID started is returned or -1 if not started

PUB main()

debug("--------------------------------------")

debug("ALTGB D  ")

debug("Alter subsequent GETBYTE instruction. Next S field = D[10:2], N field = D[1:0]")

debug(" ")

debug("GETBYTE D")

debug("Get byte established by prior ALTGB instruction into D.")

debug(" ")

debug("Example")

debug("Get ByteNum in Src_Register and move to  D_GETBYTE Register")

debug("--------------------------------------")

cogRunning := COGINIT(COGEXEC_NEW,@S0_GETBYTE,0)

debug(udec(cogRunning))

repeat                                    'keep cog 0 running

DAT

ORG   0

S0_GETBYTE            DRVH    #P0                      'P0 on program running

debug("--------------------------------------")

MOV   D_ALTGB,#Src_Register

SHL   D_ALTGB,#2                   'shift for N byte

OR    D_ALTGB,ByteNum              'set for ByteNum

debug("before ALTGB/GETBYTE   ",ubin(D_ALTGB),ubin(Src_Register),ubin(D_GETBYTE),udec(ByteNum))

ALTGB    D_ALTGB

GETBYTE  D_GETBYTE    'B2 to be loaded

debug("after ALTGB/GETBYTE    ",ubin(D_ALTGB),ubin(Src_Register),ubin(D_GETBYTE),udec(ByteNum))

debug("---------------------------------------")

_Loop1                NOP

JMP     #_Loop1          'remember # imediate

Src_Register        long      %10101010_11001100_10011001_11101110  'Source Register

D_GETBYTE           long      0                                     'Destination of ByteNum ie) Byte Value

D_ALTGB             long      0                                     'S Field for GETBYTE (Src_Register and ByteNum)

ByteNum             long      1                                     'Byte Number to Move N = 0,1,2,3

### 17.29.3_Example_WRD_ALTGB D,S_GETBYTE D,{#}S,#N

GETBYTE D,{#}S,#N

Get byte N of S into D. D = {24'b0, S.BYTE[N]).

ALTGB D,{#}S

Alter subsequent GETBYTE/ROLBYTE instruction. Next S field = (D[10:2] + S) & $1FF, N field = D[1:0]. D += sign-extended S[17:9].

{{17.29.2_Example_WRD_ALTGB D/GETBYTE D_093}}

{{

GETBYTE D,{#}S,#N

Get byte N of S into D. D = {24'b0, S.BYTE[N]).

ALTGB D,{#}S

Alter subsequent GETBYTE/ROLBYTE instruction. Next S field = (D[10:2] + S) & $1FF, N field = D[1:0]. D += sign-extended S[17:9].

Byte Addressing

D[31:0] = D3D2D1D0

= d31d30d29d28d27d26d25d24_d23d2 2d21d20d19d18d17d16_d15d14d13d12d11d10d09d08_d07d06d05d04d03d02d01d00

Byte Addressing

S[31:0] = S3S2S1S0

= S37S36S35S34S33S32S31S30_ S27S26S25S24S23S22S21S20_ S17S16S15S14S13S12S11S10_ S07S06S05S04S03S02S01S00

Example

BaseAddress = D_GETBYTE0     Offset = 0-3  S Field = BaseAddress + Offset

ByteNum = N = 0,1,2,3  Byte to be taken from long (BaseAdress + Offset)

}}

CON

_clkfreq = 200_000_000  ''Debug clock must be greater than 10MHZ

P0 =0 , P1 = 1  , P2 = 2

VAR

Byte cogRunning 'cog ID started is returned or -1 if not started

PUB main()

debug("--------------------------------------")

debug("ALTGB D,{#}S ")

debug("Alter subsequent GETBYTE/ROLBYTE instruction.")

debug("Next S field = (D[10:2] + S) & $1FF, N field = D[1:0]. D += sign-extended S[17:9].")

debug(" ")

debug("GETBYTE D,{#}S,#N ")

debug("Get byte N of S into D. D = {24'b0, S.BYTE[N]).")

debug(" ")

debug("Example")

debug("D_GETBYTE0 = BaseAddress 'S' Source Byte = BaseAddress + Offset stored in D[10:2]")

debug("S_ALTGB = BaseAddress    D_ALTGB = Offset from Base and contains ByteNUM")

debug("--------------------------------------")

cogRunning := COGINIT(COGEXEC_NEW,@S0_GETBYTE,0)

debug(udec(cogRunning))

repeat                                    'keep cog 0 running

DAT

ORG   0

S0_GETBYTE            DRVH    #P0                      'P0 on program running

debug("--------------------------------------")

MOV   S_ALTGB,#D_GETBYTE0

MOV   D_ALTGB,Offset

SHL   D_ALTGB,#2

OR    D_ALTGB,ByteNUM             'load ByteNum to access

debug("before ALTgb/GETBYTE ",udec(Offset),udec(ByteNum))

debug(ubin(D_ALTGB),ubin(D_GETBYTE))

debug(ubin(D_GETBYTE0),ubin(D_GETBYTE1))

DEBUG(ubin(D_GETBYTE2),ubin(D_GETBYTE3))

ALTGB   D_ALTGB,S_ALTGB

GETBYTE D_GETBYTE,S_GETBYTE,#0

debug("after  ALTGB/GETBYTE ",udec(Offset),udec(ByteNum))

debug(ubin(D_ALTGB),ubin(D_GETBYTE))

debug("---------------------------------------")

_Loop1                NOP

JMP     #_Loop1          'remember # imediate

Offset                long      3              'Offset from BaseAddress =D_GETBYTE0

ByteNum               long      1              'Byte to get N = 0,1,2,3,4

valByte               long      %10101010_11111111_11110000_10010010

D_ALTGB               long      0

S_ALTGB               long      0

D_GETBYTE             long      0                                 '

S_GETBYTE             long      0

D_GETBYTE0            long      %10000000_11110000_10010000_11001000

D_GETBYTE1            long      %10000001_11110001_10010001_11001001

D_GETBYTE2            long      %10000010_11110010_10010010_11001010

D_GETBYTE3            long      %10000011_11110011_10010011_11001011

## 17.30) ROLBYTE Rotate Left Byte N of S Into D

| ROLBYTE D,{#}S,#N | Rotate-left byte N of S into D. D = {D[23:0], S.BYTE[N]). |
| --- | --- |
| ROLBYTE D | Rotate-left byte established by prior ALTGB instruction into D. |

### 17.30.1_Example_WRD_ROLBYTE D,{#}S,#N_094

ROLBYTE D,{#}S,#N

Rotate-left byte N of S into D. D = {D[23:0], S.BYTE[N]).

{{17.30.1_Example_WRD_ROLBYTE D,{#}S,#N_094}}

{{

ROLBYTE D,{#}S,#N

Rotate-left byte N of S into D. D = {D[23:0], S.BYTE[N]).

Byte Addressing

D[31:0] = D3D2D1D0

= d31d30d29d28d27d26d25d24_d23d2 2d21d20d19d18d17d16_d15d14d13d12d11d10d09d08_d07d06d05d04d03d02d01d00

Byte Addressing

S[31:0] = S3S2S1S0

= S37S36S35S34S33S32S31S30_ S27S26S25S24S23S22S21S20_ S17S16S15S14S13S12S11S10_ S07S06S05S04S03S02S01S00

Example

SElect Byte in selByteLong and rotate left into valLong do this for all 4 byte 0-3

}}

CON

_clkfreq = 200_000_000  ''Debug clock must be greater than 10MHZ

P0 =0 , P1 = 1  , P2 = 2

VAR

Byte cogRunning 'cog ID started is returned or -1 if not started

PUB main()

debug("--------------------------------------")

debug("ROLBYTE D,{#}S,#N ")

debug("Rotate-left byte N of S into D. D = {D[23:0], S.BYTE[N]).")

debug(" ")

debug("Example")

debug(" ")

debug("--------------------------------------")

cogRunning := COGINIT(COGEXEC_NEW,@S0_ROLBYTE,0)

debug(udec(cogRunning))

repeat                                    'keep cog 0 running

DAT

ORG   0

S0_ROLBYTE            DRVH    #P0                      'P0 on program running

debug("--------------------------------------")

MOV   S_ROLBYTE,selByteLong

MOV   D_ROLBYTE,valLong

debug("before ROLBYTE D,S,#N ",ubin(D_ROLBYTE),ubin(S_ROLBYTE))

debug("N = 0 ",ubin(selByteLong))

ROLBYTE D_ROLBYTE,S_ROLBYTE,#0    'N to be loaded

debug("after  ROLBYTE D,S,#N ",ubin(D_ROLBYTE),ubin(S_ROLBYTE))

debug("---------------------------------------")

MOV   S_ROLBYTE,selByteLong

MOV   D_ROLBYTE,valLong

debug("before ROLBYTE D,S,#N ",ubin(D_ROLBYTE),ubin(S_ROLBYTE))

debug("N = 1 ",ubin(selByteLong))

ROLBYTE D_ROLBYTE,S_ROLBYTE,#1    'N to be loaded

debug("after  ROLBYTE D,S,#N ",ubin(D_ROLBYTE),ubin(S_ROLBYTE))

debug("---------------------------------------")

MOV   S_ROLBYTE,selByteLong

MOV   D_ROLBYTE,valLong

debug("before ROLBYTE D,S,#N ",ubin(D_ROLBYTE),ubin(S_ROLBYTE))

debug("N = 2 ",ubin(selByteLong))

ROLBYTE D_ROLBYTE,S_ROLBYTE,#2    'N to be loaded

debug("after  ROLBYTE D,S,#N ",ubin(D_ROLBYTE),ubin(S_ROLBYTE))

debug("---------------------------------------")

MOV   S_ROLBYTE,selByteLong

MOV   D_ROLBYTE,valLong

debug("before ROLBYTE D,S,#N ",ubin(D_ROLBYTE),ubin(S_ROLBYTE))

debug("N = 3 ",ubin(selByteLong))

ROLBYTE D_ROLBYTE,S_ROLBYTE,#2    'N to be loaded

debug("after  ROLBYTE D,S,#N ",ubin(D_ROLBYTE),ubin(S_ROLBYTE))

debug("---------------------------------------")

_Loop1         NOP

JMP     #_Loop1          'remember # imediate

valLong                  long      %10010001_11001100_11111111_00000000

selByteLong           long      %10101010_10010010_11100011_11110000

S_ROLBYTE            long      0

D_ROLBYTE            long      0

### 17.30.2_Example_WRD_ALTGB D_ROLBYTE D_095

ROLBYTE D

Rotate-left byte established by prior ALTGB instruction into D.

ALTGB D

Alter subsequent GETBYTE/ROLBYTE instruction. Next S field = D[10:2], N field = D[1:0].

Note: Rotate-left byte N of S into D. D = {D[23:0], S.BYTE[N]).

### 17.30.3_Example_WRD_ALTGB D,{#}S_ROLBYTE D,{#}S,#N_094

ROLBYTE D,{#}S,#N

Rotate-left byte N of S into D. D = {D[23:0], S.BYTE[N]).

ALTGB D,{#}S

Alter subsequent GETBYTE/ROLBYTE instruction. Next S field = (D[10:2] + S) & $1FF, N field = D[1:0]. D += sign-extended S[17:9].

{{17.30.3_Example_WRD_ALTGB D,{#}S_ROLBYTE D,{#}S,#N_094}}

{{

ROLBYTE D,{#}S,#N

Rotate-left byte N of S into D. D = {D[23:0], S.BYTE[N]).

ALTGB D,{#}S

Alter subsequent GETBYTE/ROLBYTE instruction. Next S field = (D[10:2] + S) & $1FF, N field = D[1:0].

D += sign-extended S[17:9].

Byte Addressing

D[31:0] = D3D2D1D0

= d31d30d29d28d27d26d25d24_d23d2 2d21d20d19d18d17d16_d15d14d13d12d11d10d09d08_d07d06d05d04d03d02d01d00

Byte Addressing

S[31:0] = S3S2S1S0

= S37S36S35S34S33S32S31S30_ S27S26S25S24S23S22S21S20_ S17S16S15S14S13S12S11S10_ S07S06S05S04S03S02S01S00

Example

Select Byte from Src_Register and Shift to D_ROLBYTE reverse order of Src_Register is the result

}}

CON

_clkfreq = 200_000_000  ''Debug clock must be greater than 10MHZ

P0 =0 , P1 = 1  , P2 = 2

VAR

Byte cogRunning 'cog ID started is returned or -1 if not started

PUB main()

debug("--------------------------------------")

debug("ALTGB D,{#}S")

debug("Alter subsequent ROLBYTE instruction. Next S field = D[10:2], N field = D[1:0]")

debug(" ")

debug("ROLBYTE D,{#}S,#N")

debug("Rotate-left byte N of S into D. D = {D[23:0], S.BYTE[N]).")

debug(" ")

debug("Example")

debug(" ")

debug("--------------------------------------")

cogRunning := COGINIT(COGEXEC_NEW,@S0_ROLBYTE,0)

debug(udec(cogRunning))

repeat                                    'keep cog 0 running

DAT

ORG   0

S0_ROLBYTE            DRVH    #P0                        'P0 on program running

debug("--------------------------------------")

MOV   D_ALTGB,#Src_Register        'place address of Src_Register for S Field D[10:2]

NextByte              SHL   D_ALTGB,#2                   'shift for N Field

OR    D_ALTGB,ByteNum              'set for N  ByteNum N Field = D[1:0]

debug("before ALTGB/ROLBYTE   ",ubin(D_ALTGB),ubin(Src_Register))

debug(ubin(D_ROLBYTE),udec(ByteNum))

debug(" ")

ALTGB    D_ALTGB

ROLBYTE  D_ROLBYTE                 '

debug("after ALTGB/ROLBYTE    ",ubin(D_ALTGB),ubin(Src_Register),ubin(D_ROLBYTE),udec(ByteNum))

debug("---------------------------------------")

ADD     Index,#1

ADD     ByteNum,#1

CMP     Index,#4  WZ

IF_Z        JMP     #_Loop1

JMP     NextByte

_Loop1                NOP

JMP     #_Loop1          'remember # imediate

Index                   long      0

ByteNum             long      0                                     'Byte Number to Move N = 0,1,2,3

Src_Register        long      %10101010_11001100_10011001_11101110  'Source Register

D_ROLBYTE         long      0                                     'Destination of ByteNum ie) Byte Value

D_ALTGB             long      0                                     'S Field for GETBYTE (Src_Register and ByteNum)

## 17.31) SETWORD Set S[15:0] into Word N in D

| SETWORD D,{#}S,#N | Set S[15:0] into word N in D, keeping rest of D same. |
| --- | --- |
| SETWORD {#}S | Set S[15:0] into word established by prior ALTSW instruction. |

SETWORD D,{#}S,#N  (096)

Set S[15:0] into word N in D, keeping rest of D same.

SETWORD {#}S  (097)

Set S[15:0] into word established by prior ALTSW instruction.

ALTSW D,{#}S  (110)

Alter subsequent SETWORD instruction. Next D field = (D[9:1] + S) & $1FF, N field = D[0]. D += sign-extended S[17:9].

ALTSW D (111)

Alter subsequent SETWORD instruction. Next D field = D[9:1], N field = D[0].

Word Addressing

W[31:0] = W1W0 = w31w30w29w28w27w26w25w24_w23w22w21w20w19w18w17w16_w15w14w13w12w11w10w9w8_w7w6w5w4w3w2w1w0

W0 = w015w014w013w012w01w010w009w008w007w006w005w004w003w002w001w000

W1 = w115w114w113w112w11w110w109w108w107w106w105w104w103w102w101w100

### 17.31.1_Example_WRD_SETWORD D,{#}S,#N_096

SETWORD D,{#}S,#N

Set S[15:0] into word N in D, keeping rest of D same.

{{17.31.1_Example_WRD_SETWORD D,{#}S,#N_096}}

{{

SETWORD D,{#}S,#N

Set S[15:0] into word N in D, keeping rest of D same.

Word Addressing

W[31:0] = W1W0 = w31w30w29w28w27w26w25w24_w23w22w21w20w19w18w17w16_w15w14w13w12w11w10w9w8_w7w6w5w4w3w2w1w0

W0 = w015w014w013w012w01w010w009w008w007w006w005w004w003w002w001w000

W1 = w115w114w113w112w11w110w109w108w107w106w105w104w103w102w101w100

Example

D_SETWORD is to have selWord written to word 0 an word 1

}}

CON

_clkfreq = 200_000_000  ''Debug clock must be greater than 10MHZ

P0 =0 , P1 = 1  , P2 = 2

VAR

Byte cogRunning 'cog ID started is returned or -1 if not started

PUB main()

debug("--------------------------------------")

debug("SETWORD D,{#}S,#N ")

debug("Set S[15:0] into word N of W1W0 in D, keeping rest of D same.")

debug(" ")

debug("Example")

debug("D_SETWORD is to have selWord written to word 0 an word 1  ")

debug("--------------------------------------")

cogRunning := COGINIT(COGEXEC_NEW,@S0_SETWORD,0)

debug(udec(cogRunning))

repeat                                    'keep cog 0 running

DAT

ORG   0

S0_SETWORD            DRVH    #P0                      'P0 on program running

debug("--------------------------------------")

MOV   S_SETWORD,selWord

MOV   D_SETWORD,selLong

debug("before SETWORD D,S,#N ",ubin(D_SETWORD),ubin(S_SETWORD))

debug("Word W0 N = 0 to be written too ")

SETWORD D_SETWORD,S_SETWORD,#0    'N to be loaded

debug("after  SETWORD D,S,#N ",ubin(D_SETWORD),ubin(S_SETWORD))

debug("---------------------------------------")

debug("---------------------------------------")

MOV   S_SETWORD,selWord

MOV   D_SETWORD,selLong

debug("before SETWORD D,S,#N ",ubin(D_SETWORD),ubin(S_SETWORD))

debug("Word W1 N = 1 to be written too ")

SETWORD D_SETWORD,S_SETWORD,#1    'N to be loaded

debug("after  SETWORD D,S,#N ",ubin(D_SETWORD),ubin(S_SETWORD))

debug("---------------------------------------")

_Loop1                NOP

JMP     #_Loop1          'remember # imediate

selLong               long      %11111111_11111111_11111111_11111111

selWord               long      %10000000_00000001

S_SETWORD             long      0

D_SETWORD             long      0

### 17.31.2_Example_WRD_SETWORD {#}S_097

SETWORD {#}S (097)

Set S[15:0] into word established by prior ALTSW instruction.

ALTSW D (111)

Alter subsequent SETWORD instruction. Next D field = D[9:1], N field = D[0].

{{17.31.2_Example_WRD_ALTSW D_SETWORD {#}S_097}}

{{

SETWORD {#}S

Set S[15:0] into word established by prior ALTSW instruction.

ALTSW D

Alter subsequent SETWORD instruction. Next D field = D[9:1], N field = D[0].

Word Addressing

W[31:0] = W1W0 = w31w30w29w28w27w26w25w24_w23w22w21w20w19w18w17w16_w15w14w13w12w11w10w9w8_w7w6w5w4w3w2w1w0

W0 = w015w014w013w012w01w010w009w008w007w006w005w004w003w002w001w000

W1 = w115w114w113w112w11w110w109w108w107w106w105w104w103w102w101w100

Example

D_SETWORD is to have selWord written to word 0 an word 1

}}

CON

_clkfreq = 200_000_000  ''Debug clock must be greater than 10MHZ

P0 =0 , P1 = 1  , P2 = 2

VAR

Byte cogRunning 'cog ID started is returned or -1 if not started

PUB main()

debug("--------------------------------------")

debug("ALTSW D")

debug("Alter subsequent SETWORD instruction.")

debug("Next D field = D[9:1], N field = D[0].")

debug(" ")

debug("SETWORD {#}S")

debug("Set S[15:0] into word established by prior ALTSW instruction.")

debug(" ")

debug("Example")

debug("D_SETWORD is to have selWord written to word 0 an word 1  ")

debug("--------------------------------------")

cogRunning := COGINIT(COGEXEC_NEW,@S0_SETWORD,0)

debug(udec(cogRunning))

repeat                                    'keep cog 0 running

DAT

ORG   0

S0_SETWORD            DRVH    #P0                     'P0 on program running

debug("--------------------------------------")

MOV   S_SETWORD,selWord

MOV   D_ALTSW,#D_SETWORD       'address to write too

SHL   D_ALTSW,#1               'shift left 1

OR    D_ALTSW,#0               'load W0 N to be target

debug("before ALTSW/SETWORD  ",ubin(D_SETWORD),ubin(S_SETWORD))

debug("Word W0 N = 0 to be written too ")

ALTSW   D_ALTSW                'Load D Field  D[9:1]  N Field D[0]

SETWORD S_SETWORD              'ALTSW contains D  and N to be loaded

debug("after  ALTSW/SETWORD  ",ubin(D_SETWORD),ubin(S_SETWORD))

debug("---------------------------------------")

MOV   S_SETWORD,selWord        'word value to be written

MOV   D_ALTSW,#D_SETWORD       'address to write too

SHL   D_ALTSW,#1               'shift left 1

OR    D_ALTSW,#1               'load W1 N to be target

debug("before ALTSW/SETWORD  ",ubin(D_SETWORD),ubin(S_SETWORD))

debug("Word W1 N = 1 to be written too ")

ALTSW   D_ALTSW                'Load D Field  D[9:1]  N Field D[0]

SETWORD S_SETWORD              'ALTSW contains D  and N to be loaded

debug("after  ALTSW/SETWORD  ",ubin(D_SETWORD),ubin(S_SETWORD))

debug("---------------------------------------")

_Loop1         NOP

JMP     #_Loop1          'remember # imediate

selLong                   long      %11111111_11111111_11111111_11111111

selWord                  long      %10000000_00000001

D_ALTSW               long      0

S_SETWORD          long      0

D_SETWORD         long      %11111111_11111111_11111111_11111111

### 17.31.3_Example_WRD_ALTSW D,{#}S_SETWORD D,{S},#N

SETWORD D,{#}S,#N   (096)

Set S[15:0] into word N in D, keeping rest of D same.

ALTSW D,{#}S    (110)

Alter subsequent SETWORD instruction. Next D field = (D[9:1] + S) & $1FF, N field = D[0]. D += sign-extended S[17:9].

{{17.31.3_Example_WRD_ALTSW D,{#}S_SETWORD D,{#}S,#N_096}}

{{

SETWORD D,{#}S,#N   (096)

Set S[15:0] into word N in D, keeping rest of D same.

ALTSW D,{#}S    (110)

Alter subsequent SETWORD instruction. Next D field = (D[9:1] + S) & $1FF, N field = D[0]. D += sign-extended S[17:9].

W[31:0] = W1W0 = w31w30w29w28w27w26w25w24_w23w22w21w20w19w18w17w16_w15w14w13w12w11w10w9w8_w7w6w5w4w3w2w1w0

W0 = w015w014w013w012w01w010w009w008w007w006w005w004w003w002w001w000

W1 = w115w114w113w112w11w110w109w108w107w106w105w104w103w102w101w100

Example

Write 'valWord' too W0 of 'Src_Register0' then write 'valWord too W1 of 'Src_Register1'

}}

CON

_clkfreq = 200_000_000  ''Debug clock must be greater than 10MHZ

P0 =0 , P1 = 1  , P2 = 2

VAR

Byte cogRunning 'cog ID started is returned or -1 if not started

PUB main()

debug("--------------------------------------")

debug("ALTSW D,{#}S ")

debug("Alter subsequent SETWORD instruction. Next D field = (D[9:1] + S) & $1FF")

debug("N field = D[0]. D += sign-extended S[17:9].")

debug(" ")

debug("SETWORD D,{#}S,#N")

debug("Set S[15:0] into word N in D, keeping rest of D same.")

debug(" ")

debug("Example")

debug("Write 'valWord' too W0 of 'Src_Register0' then write 'valWord' too W1 of 'Src_Register1'")

debug("--------------------------------------")

cogRunning := COGINIT(COGEXEC_NEW,@S0_SETWORD,0)

debug(udec(cogRunning))

repeat                                    'keep cog 0 running

DAT

ORG   0

S0_SETWORD            DRVH    #P0                        'P0 on program running

debug("--------------------------------------")

MOV   S_SETWORD,valWord

MOV   S_ALTSW,#Src_Register0        'BaseAddress

MOV   D_ALTSW,#0                    'no Offset address for Src_Register0 for S Field D[10:2]

SHL   D_ALTSW,#1                    'shift for N Field

OR    D_ALTSW,#0                    'set for N =0 W0 N Field = D[0]

debug("before ALTSW/SETWORD   ",ubin(D_ALTSW),ubin(D_SETWORD))

debug(ubin(Src_Register0),ubin(Src_Register1))

debug(" ")

ALTSW    D_ALTSW,S_ALTSW           'Next D Field BaseAddress + Offset Next N Field W0

SETWORD  D_SETWORD,valWord,#0                 '

debug("after ALTSW/SETWORD    ",ubin(D_ALTSW),ubin(D_SETWORD))

debug(ubin(Src_Register0),ubin(Src_Register1))

debug("---------------------------------------")

MOV   S_SETWORD,valWord

MOV   S_ALTSW,#Src_Register0        'BaseAddress

MOV   D_ALTSW,#1                    'Offset address of Src_Register1 for S Field D[10:2]

SHL   D_ALTSW,#1                    'shift for N Field

OR    D_ALTSW,#1                    'set for N = 1 W1 N Field = D[0]

debug("before ALTSW/SETWORD   ",ubin(D_ALTSW),ubin(D_SETWORD))

debug(ubin(Src_Register0),ubin(Src_Register1))

debug(" ")

ALTSW    D_ALTSW,S_ALTSW           'Next D Field BaseAdres + Offset Next N Field W1

SETWORD  D_SETWORD,valWord,#0                 '

debug("after ALTSW/SETWORD    ",ubin(D_ALTSW),ubin(D_SETWORD))

debug(ubin(Src_Register0),ubin(Src_Register1))

debug("---------------------------------------")

_Loop1                NOP

JMP     #_Loop1          'remember # imediate

D_SETWORD        long      0                                     'Destination of ByteNum ie) Byte Value

S_SETWORD         long      0

D_ALTSW              long      0                                     'set for N  ByteNum N Field = D[1:0]

S_ALTSW               long      0                                     'S Field for GETBYTE (Src_Register and ByteNum)

valWord                long      %10000001_10000001                    'word to write 16 bits

Src_Register0       long      %10101010_11001100_10011001_11101110  'Source Register BaseAddress +0

Src_Register1       long      %10101010_11001100_10011001_11101110  'Source Register BaseAddress +1

## 17.32) GETWORD Get Word N of S into D

| GETWORD D,{#}S,#N | Get word N of S into D. D = {16'b0, S.WORD[N]). |
| --- | --- |
| GETWORD D | Get word established by prior ALTGW instruction into D. |

GETWORD D,{#}S,#N  (098)

Get word N of S into D. D = {16'b0, S.WORD[N]).

GETWORD D (099)

Get word established by prior ALTGW instruction into D.

ALTGW D,{#},S (112)

Alter subsequent GETWORD/ROLWORD instruction. Next S field = ((D[9:1] + S) & $1FF), N field = D[0]. D += sign-extended S[17:9].

ALTGW D (113)

Alter subsequent GETWORD/ROLWORD instruction. Next S field = D[9:1], N field = D[0].

Word Addressing

W[31:0] = W1W0 = w31w30w29w28w27w26w25w24_w23w22w21w20w19w18w17w16_w15w14w13w12w11w10w9w8_w7w6w5w4w3w2w1w0

W0 = w015w014w013w012w01w010w009w008w007w006w005w004w003w002w001w000

W1 = w115w114w113w112w11w110w109w108w107w106w105w104w103w102w101w100

### 17.32.1_Example_WRD_GETWORD D,{#}S,#N _098

GETWORD D,{#}S,#N  (098)

Get word N of S into D. D = {16'b0, S.WORD[N]).

{{17.32.1_Example_WRD_GETWORD D,{#}S,#N_098}}

{{

GETWORD D,{#}S,#N  (098)

Get word N of S into D. D = {16'b0, S.WORD[N]).

Word Addressing

W[31:0] = W1W0 = w31w30w29w28w27w26w25w24_w23w22w21w20w19w18w17w16_w15w14w13w12w11w10w9w8_w7w6w5w4w3w2w1w0

W0 = w015w014w013w012w01w010w009w008w007w006w005w004w003w002w001w000

W1 = w115w114w113w112w11w110w109w108w107w106w105w104w103w102w101w100

Example

write W0 of 'selLong' to 'D_GETWORD' then write W1 to 'D_GETWORD'

}}

CON

_clkfreq = 200_000_000  ''Debug clock must be greater than 10MHZ

P0 =0 , P1 = 1  , P2 = 2

VAR

Byte cogRunning 'cog ID started is returned or -1 if not started

PUB main()

debug("--------------------------------------")

debug("GETWORD D,{#}S,#N ")

debug(" ")

debug(" ")

debug("Example")

debug("write W0 of 'selLong' to 'D_GETWORD' then write W1 to 'D_GETWORD'  ")

debug("--------------------------------------")

cogRunning := COGINIT(COGEXEC_NEW,@S0_GETWORD,0)

debug(udec(cogRunning))

repeat                                    'keep cog 0 running

DAT

ORG   0

S0_GETWORD            DRVH    #P0                      'P0 on program running

debug("--------------------------------------")

MOV   D_GETWORD,#0

MOV   S_GETWORD,selLong

debug("before GETWORD D,S,#N ",ubin(D_GETWORD),ubin(S_GETWORD))

debug("Word W0 N = 0 to be written too ")

GETWORD D_GETWORD,S_GETWORD,#0    'N to be loaded

debug("after  GETWORD D,S,#N ",ubin(D_GETWORD),ubin(S_GETWORD))

debug("---------------------------------------")

debug("---------------------------------------")

MOV   D_GETWORD,#0

MOV   S_GETWORD,selLong

debug("before GETWORD D,S,#N ",ubin(D_GETWORD),ubin(S_GETWORD))

debug("Word W1 N = 1 to be written too ")

GETWORD D_GETWORD,S_GETWORD,#1    'N to be loaded

debug("after  GETWORD D,S,#N ",ubin(D_GETWORD),ubin(D_GETWORD))

debug("---------------------------------------")

_Loop1                NOP

JMP     #_Loop1          'remember # imediate

selLong               long      %10000001_10010011_10001000_11111111

S_GETWORD             long      0

D_GETWORD             long      0

### 17.32.2_Example_WRD_ALTGW D_GETWORD D_099

GETWORD D (099)

Get word established by prior ALTGW instruction into D.

{{17.32.2_Example_WRD_ALTGW D_GETWORD {#}S_099}}

{{

GETWORD D (099)

Get word established by prior ALTGW instruction into D.

ALTGW D (113)

Alter subsequent GETWORD/ROLWORD instruction. Next S field = D[9:1], N field = D[0].

Word Addressing

W[31:0] = W1W0 = w31w30w29w28w27w26w25w24_w23w22w21w20w19w18w17w16_w15w14w13w12w11w10w9w8_w7w6w5w4w3w2w1w0

W0 = w015w014w013w012w01w010w009w008w007w006w005w004w003w002w001w000

W1 = w115w114w113w112w11w110w109w108w107w106w105w104w103w102w101w100

Example

Write 'selWord' W0 too 'D_GETWORD' then write W1 to 'D_GETWORD' note upper bits cleared in 'D_GETWORD'

}}

CON

_clkfreq = 200_000_000  ''Debug clock must be greater than 10MHZ

P0 =0 , P1 = 1  , P2 = 2

VAR

Byte cogRunning 'cog ID started is returned or -1 if not started

PUB main()

debug("--------------------------------------")

debug("ALTGW D")

debug("Alter subsequent GETWORD instruction.")

debug("Next S field = D[9:1], N field = D[0].")

debug(" ")

debug("GETWOR D")

debug("Get word established by prior ALTGW instruction into D.")

debug(" ")

debug("Example")

debug("Write 'selWord' W0 too 'D_GETWORD' then write W1 to 'D_GETWORD' note upper bits cleared in 'D_GETWORD'")

debug("--------------------------------------")

cogRunning := COGINIT(COGEXEC_NEW,@S0_GETWORD,0)

debug(udec(cogRunning))

repeat                                    'keep cog 0 running

DAT

ORG   0

S0_GETWORD            DRVH    #P0                     'P0 on program running

debug("--------------------------------------")

MOV   S_GETWORD,selWord

MOV   D_ALTGW,#S_GETWORD       'address to write too

SHL   D_ALTGW,#1               'shift left 1

OR    D_ALTGW,#0               'load W0 N to be target

debug("before ALTGW/GETWORD  ",ubin(D_GETWORD),ubin(S_GETWORD))

debug("Word W0 N = 0 to be written too ")

ALTGW   D_ALTGW                'Load D Field  D[9:1]  N Field D[0]

GETWORD D_GETWORD              'ALTGW contains D  and N to be loaded

debug("after  ALTGW/GETWORD  ",ubin(D_GETWORD),ubin(S_GETWORD))

debug("---------------------------------------")

MOV   D_ALTGW,#S_GETWORD       'address to write too

SHL   D_ALTGW,#1               'shift left 1

OR    D_ALTGW,#1               'load W1 N to be target

debug("before ALTGW/GETWORD  ",ubin(D_GETWORD),ubin(S_GETWORD))

debug("Word W1 N = 1 to be written too ")

ALTGW   D_ALTGW                'Load D Field  D[9:1]  N Field D[0]

GETWORD D_GETWORD              'ALTGW contains D  and N to be loaded

debug("after  ALTGW/GETWORD  ",ubin(D_GETWORD),ubin(S_GETWORD))

debug("---------------------------------------")

_Loop1         NOP

JMP     #_Loop1          'remember # imediate

selWord                    long      %10000000_00000001_11000011_11000011

D_ALTGW                 long      0

S_GETWORD            long      %10101010_11001100_11100011_11110000

D_GETWORD            long      %11111111_11111111_11111111_11111111

### 17.32.3_Example_WRD_ ALTGW D,{#},S_ GETWORD D,{#}S,#N _098

GETWORD D,{#}S,#N  (098)

Get word N of S into D. D = {16'b0, S.WORD[N]).

ALTGW D,{#},S (112)

Alter subsequent GETWORD/ROLWORD instruction. Next S field = ((D[9:1] + S) & $1FF), N field = D[0]. D += sign-extended S[17:9].

{{17.32.3_Example_WRD_ALTGW D,{#}S_GETWORD D,{#}S,#N_098}}

{{

GETWORD D,{#}S,#N  (098)

Get word N of S into D. D = {16'b0, S.WORD[N]).

ALTGW D,{#},S (112)

Alter subsequent GETWORD instruction. Next S field = ((D[9:1] + S) & $1FF), N field = D[0]. D += sign-extended S[17:9].

W[31:0] = W1W0 = w31w30w29w28w27w26w25w24_w23w22w21w20w19w18w17w16_w15w14w13w12w11w10w9w8_w7w6w5w4w3w2w1w0

W0 = w015w014w013w012w01w010w009w008w007w006w005w004w003w002w001w000

W1 = w115w114w113w112w11w110w109w108w107w106w105w104w103w102w101w100

Example

Write W0 of 'Src_Register0' too 'D_GETWORD' then write W1 of 'Src_Register1'too 'D_GETWORD'

}}

CON

_clkfreq = 200_000_000  ''Debug clock must be greater than 10MHZ

P0 =0 , P1 = 1  , P2 = 2

VAR

Byte cogRunning 'cog ID started is returned or -1 if not started

PUB main()

debug("--------------------------------------")

debug("ALTGW D,{#}S ")

debug("Alter subsequent GETWORD instruction.")

debug("Next S field = ((D[9:1] + S) & $1FF), N field = D[0].D += sign-extended S[17:9].")

debug(" ")

debug("GETWORD D,{#}S,#N")

debug("Get word N of S into D. D = {16'b0, S.WORD[N])")

debug(" ")

debug("Example")

debug("Write W0 of 'Src_Register0' too 'D_GETWORD'")

debug("then Write W1 of 'Src_Register1'too 'D_GETWORD'")

debug("--------------------------------------")

cogRunning := COGINIT(COGEXEC_NEW,@S0_GETWORD,0)

debug(udec(cogRunning))

repeat                                    'keep cog 0 running

DAT

ORG   0

S0_GETWORD            DRVH    #P0                        'P0 on program running

debug("--------------------------------------")

MOV   S_ALTGW,#Src_Register0        'BaseAddress

MOV   D_ALTGW,#0                    'no Offset address for Src_Register0 for S Field D[10:2]

SHL   D_ALTGW,#1                    'shift for N Field

OR    D_ALTGW,#0                    'set for N =0 W0 N Field = D[0]

debug("before ALTGW/GETWORD   ",ubin(D_ALTGW),ubin(D_GETWORD))

debug(ubin(Src_Register0),ubin(Src_Register1))

debug(" ")

'D_ALTGW=Offset BaseAddress N Field W0,S_ALTGW=BaseAddress

ALTGW    D_ALTGW,S_ALTGW

'D_GETWORD = Destination of W0 ALTGW replaces S_GETWORD,(#N=#0)

GETWORD  D_GETWORD,S_GETWORD,#0             '

debug("after ALTGW/GETWORD W0 N=0 S = Src_Register0+(Offset = 0)=Src_Register0")

debug(ubin(Src_Register0),ubin(Src_Register1))

debug("Write W0 of 'Src_Register0' too 'D_GETWORD'")

debug(ubin(D_ALTGW),ubin(D_GETWORD))

debug("---------------------------------------")

MOV   S_ALTGW,#Src_Register0        'BaseAddress

MOV   D_ALTGW,#1                    'Offset address of Src_Register1 for S Field D[10:2]

SHL   D_ALTGW,#1                    'shift for N Field

OR    D_ALTGW,#1                    'set for N = 1 W1 N Field = D[0]

debug("before ALTGW/GETWORD   ",ubin(D_ALTGW),ubin(D_GETWORD))

debug(ubin(Src_Register0),ubin(Src_Register1))

debug(" ")

'D_ALTGW=Offset BaseAddress N Field W1,S_ALTGW=BaseAddress

ALTGW    D_ALTGW,S_ALTGW

'D_GETWORD = Destination of W1 ALTGW replaces S_GETWORD,(#N=#1)

GETWORD  D_GETWORD,S_GETWORD,#0                '

debug("after ALTGW/GETWORD W1 N=1 S = Src_Register0+(Offset =1)=Src_Register1")

debug(ubin(Src_Register0),ubin(Src_Register1))

debug("Write W1 of 'Src_Register1'too 'D_GETWORD'")

debug(ubin(D_ALTGW),ubin(D_GETWORD))

debug("---------------------------------------")

_Loop1                NOP

JMP     #_Loop1          'remember # imediate

D_GETWORD           long      0                                     'Destination of Word ie) WORD Value

S_GETWORD           long      0                                     'not required replaced by ALTGW

D_ALTGW             long      0                                     'D[9:1] offset of S  N Field = D[1:0]

S_ALTGW             long      0                                     'S BaseAddress Sorce of Word

Src_Register0       long      %10101010_11001100_10011001_11101110  'Source Register BaseAddress +0

Src_Register1       long      %10101010_11001100_10011011_11101111  'Source Register BaseAddress +1

## 17.33) ROLWORD Rotate Left Word N of S Into D

| ROLWORD D,{#}S,#N | Rotate-left word N of S into D. D = {D[15:0], S.WORD[N]). |
| --- | --- |
| ROLWORD D | Rotate-left word established by prior ALTGW instruction into D. |

ROLWORD D,{#}S,#N (100)

Rotate-left word N of S into D. D = {D[15:0], S.WORD[N]).

ROLWORD D (101)

Rotate-left word established by prior ALTGW instruction into D.

ALTGW D,{#},S (112)

Alter subsequent GETWORD/ROLWORD instruction. Next S field = ((D[9:1] + S) & $1FF), N field = D[0]. D += sign-extended S[17:9].

ALTGW D (113)

Alter subsequent GETWORD/ROLWORD instruction. Next S field = D[9:1], N field = D[0].

Word Addressing

W[31:0] = W1W0 = w31w30w29w28w27w26w25w24_w23w22w21w20w19w18w17w16_w15w14w13w12w11w10w9w8_w7w6w5w4w3w2w1w0

W0 = w015w014w013w012w01w010w009w008w007w006w005w004w003w002w001w000

W1 = w115w114w113w112w11w110w109w108w107w106w105w104w103w102w101w100

### 17.33.1_Example_WRD_ROLWORD D,{#}S_100

ROLWORD D,{#}S,#N (100)

Rotate-left word N of S into D. D = {D[15:0], S.WORD[N]).

{{17.33.1_Example_WRD_ROLWORD D,{#}S,#N_100}}

{{

ROLWORD D,{#}S,#N (100)

Rotate-left word N of S into D. D = {D[15:0], S.WORD[N]).

Word Addressing

W[31:0] = W1W0 = w31w30w29w28w27w26w25w24_w23w22w21w20w19w18w17w16_w15w14w13w12w11w10w9w8_w7w6w5w4w3w2w1w0

W0 = w015w014w013w012w01w010w009w008w007w006w005w004w003w002w001w000

W1 = w115w114w113w112w11w110w109w108w107w106w105w104w103w102w101w100

Example

'selLong' rotates N 0,1,2,3, of 'S_ROLWORD' into 'D_ROLWORD' keep the same byte order

}}

CON

_clkfreq = 200_000_000  ''Debug clock must be greater than 10MHZ

P0 =0 , P1 = 1  , P2 = 2

VAR

Byte cogRunning 'cog ID started is returned or -1 if not started

PUB main()

debug("--------------------------------------")

debug("ROLWORD D,{#}S,#N ")

debug("Rotate-left word N of S into D. D = {D[15:0], S.WORD[N]).")

debug(" ")

debug("Example")

debug("'selLong' rotates N 0,1,2,3, of 'S_ROLWORD' into 'D_ROLWORD' keep the same byte order")

debug("--------------------------------------")

cogRunning := COGINIT(COGEXEC_NEW,@S0_ROLWORD,0)

debug(udec(cogRunning))

repeat                                    'keep cog 0 running

DAT

ORG   0

S0_ROLWORD            DRVH    #P0                      'P0 on program running

debug("--------------------------------------")

MOV   S_ROLWORD,selLong

debug("before ROLWORD D,S,#N ",ubin(D_ROLWORD),ubin(S_ROLWORD))

debug("Word W1 N = 1 to be written ")

ROLWORD D_ROLWORD,S_ROLWORD,#1    'N to be loaded

debug("after  ROLWORD D,S,#N ",ubin(D_ROLWORD),ubin(S_ROLWORD))

debug("---------------------------------------")

debug("---------------------------------------")

debug("before ROLWORD D,S,#N ",ubin(D_ROLWORD),ubin(S_ROLWORD))

debug("Word W0 N = 0 to be written ")

ROLWORD D_ROLWORD,S_ROLWORD,#0    'N to be loaded

debug("after  ROLWORD D,S,#N ",ubin(D_ROLWORD),ubin(S_ROLWORD))

debug("---------------------------------------")

_Loop1                NOP

JMP     #_Loop1          'remember # imediate

selLong               long      %10000001_10010011_10001000_11111111

S_ROLWORD             long      0

D_ROLWORD             long      %11111111_11111111_11111111_11111111

### 17.33.2_Example_WRD_ALTGW D_ROLWORD D_101

ROLWORD D  (101)

Rotate-left word established by prior ALTGW instruction into D.

{{17.33.2_Example_WRD_ALTGW D_ROLWORD {#}S_101}}

{{

ROLWORD D  (101)

Rotate-left word established by prior ALTGW instruction into D.

ALTGW D (113)

Alter subsequent GETWORD/ROLWORD instruction. Next S field = D[9:1], N field = D[0].

Word Addressing

W[31:0] = W1W0 = w31w30w29w28w27w26w25w24_w23w22w21w20w19w18w17w16_w15w14w13w12w11w10w9w8_w7w6w5w4w3w2w1w0

W0 = w015w014w013w012w01w010w009w008w007w006w005w004w003w002w001w000

W1 = w115w114w113w112w11w110w109w108w107w106w105w104w103w102w101w100

Example

Rotate left Word W0 from 'Src_Long0' to 'D_ROLWORD'

then Rotate left Word W1 from Src_Long1 to 'D_ROLWORD'

}}

CON

_clkfreq = 200_000_000  ''Debug clock must be greater than 10MHZ

P0 =0 , P1 = 1  , P2 = 2

VAR

Byte cogRunning 'cog ID started is returned or -1 if not started

PUB main()

debug("--------------------------------------")

debug("ALTGW D")

debug("Alter subsequent ROLWORD instruction.")

debug("Next S field = D[9:1], N field = D[0].")

debug(" ")

debug("ROLWORD D")

debug("Rotate-left word established by prior ALTGW instruction into D.")

debug(" ")

debug("Example")

debug("Rotate left Word W0 from 'Src_Long0' to 'D_ROLWORD'")

debug("then Rotate left Word W1 from Src_Long1 to 'D_ROLWORD'")

debug("--------------------------------------")

cogRunning := COGINIT(COGEXEC_NEW,@S0_ROLWORD,0)

debug(udec(cogRunning))

repeat                                    'keep cog 0 running

DAT

ORG   0

S0_ROLWORD            DRVH    #P0                     'P0 on program running

debug("--------------------------------------")

MOV   D_ALTGW,#Src_Long0       'address to write too

SHL   D_ALTGW,#1               'shift left 1

OR    D_ALTGW,#0               'load W0 N to be target

debug("before ALTGW/ROLTWORD W0 N=0 ",ubin(D_ALTGW))

debug(ubin(Src_Long0),ubin(Src_Long1))

debug(ubin(D_ROLWORD))

debug(" ")

ALTGW   D_ALTGW                'Load S Field  D[9:1]  N Field D[0]

ROLWORD D_ROLWORD              'ALTGW contains S  and N to be loaded

debug("after ALTGW/ROLTWORD  W0 N=0 ",ubin(D_ALTGW))

debug(ubin(Src_Long0),ubin(Src_Long1))

debug(ubin(D_ROLWORD))

debug("---------------------------------------")

MOV   D_ALTGW,#Src_Long0       'address to write too

SHL   D_ALTGW,#1               'shift left 1

OR    D_ALTGW,#1               'load W1 N to be target

debug("before ALTGW/ROLTWORD W1 N=1 ",ubin(D_ALTGW))

debug(ubin(Src_Long0),ubin(Src_Long1))

debug(ubin(D_ROLWORD))

debug(" ")

ALTGW   D_ALTGW                'Load S Field  D[9:1]  N Field D[0]

ROLWORD D_ROLWORD              'ALTGW contains S  and N to be loaded

debug("after ALTGW/ROLTWORD  W1 N=1 ",ubin(D_ALTGW))

debug(ubin(Src_Long0),ubin(Src_Long1))

debug(ubin(D_ROLWORD))

debug("---------------------------------------")

_Loop1                NOP

JMP     #_Loop1          'remember # imediate

S_ALTGW               long      0              'not regquired no destination used by subsequent instruction

D_ALTGW               long      0              'Next S field = D[9:1], N field = D[0].

S_ROLWORD             long      0              'not required D_ALTGW is loaded into S (S_ROLWORD)

D_ROLWORD             long      0              'Destination of Word written W0 or W1

Src_Long0             long      %10000001_00000001_11000000_00000011

Src_Long1             long      %11100000_00000111_11110000_00001111

ALTGW D (113)

Alter subsequent GETWORD/ROLWORD instruction. Next S field = D[9:1], N field = D[0].

### 17.33.3_Example_WRD_ ALTGW D,{#},S_ ROLWORD D,{#}S,#N_100

ROLWORD D,{#}S,#N (100)

Rotate-left word N of S into D. D = {D[15:0], S.WORD[N]).

ALTGW D,{#},S (112)

Alter subsequent GETWORD/ROLWORD instruction. Next S field = ((D[9:1] + S) & $1FF), N field = D[0]. D += sign-extended S[17:9].

{{17.33.3_Example_WRD_ALTGW D,{#}S_ROLWORD D,{#}S,#N_100}}

{{

ROLWORD D,{#}S,#N (100)

Rotate-left word N of S into D. D = {D[15:0], S.WORD[N]).

ALTGW D,{#},S (112)

Alter subsequent GETWORD/ROLWORD instruction. Next S field = ((D[9:1] + S) & $1FF), N field = D[0]. D += sign-extended S[17:9].

W[31:0] = W1W0 = w31w30w29w28w27w26w25w24_w23w22w21w20w19w18w17w16_w15w14w13w12w11w10w9w8_w7w6w5w4w3w2w1w0

W0 = w015w014w013w012w01w010w009w008w007w006w005w004w003w002w001w000

W1 = w115w114w113w112w11w110w109w108w107w106w105w104w103w102w101w100

Example

ROTATE left W0 of 'Src_Register0' too 'D_ROLWORD' then ROTATE left W1 of 'Src_Register1'too 'D_ROLWORD'

}}

CON

_clkfreq = 200_000_000  ''Debug clock must be greater than 10MHZ

P0 =0 , P1 = 1  , P2 = 2

VAR

Byte cogRunning 'cog ID started is returned or -1 if not started

PUB main()

debug("--------------------------------------")

debug("ALTGW D,{#},S (112)")

debug("Alter subsequent GETWORD/ROLWORD instruction.")

debug(" Next S field = ((D[9:1] + S) & $1FF), N field = D[0]. D += sign-extended S[17:9].")

debug(" ")

debug("ROLWORD D,{#}S,#N (100)")

debug("Rotate-left word N of S into D. D = {D[15:0], S.WORD[N]).")

debug(" ")

debug("Example")

debug("ROTATE left W0 of 'Src_Register0' too 'D_ROLWORD'")

debug("then ROTATE left W1 of 'Src_Register1'too 'D_ROLWORD'")

debug("--------------------------------------")

cogRunning := COGINIT(COGEXEC_NEW,@S0_ROLWORD,0)

debug(udec(cogRunning))

repeat                                    'keep cog 0 running

DAT

ORG   0

S0_ROLWORD            DRVH    #P0                        'P0 on program running

debug("--------------------------------------")

MOV   S_ALTGW,#Src_Register0        'BaseAddress

MOV   D_ALTGW,#0                    'no Offset address for Src_Register0 for S Field D[10:2]

SHL   D_ALTGW,#1                    'shift for N Field

OR    D_ALTGW,#0                    'set for N =0 W0 N Field = D[0]

debug("before ALTGW/ROLWORD   ")

debug(ubin(Src_Register0),ubin(Src_Register1))

debug(ubin(D_ALTGW),ubin(D_ROLWORD))

debug(" ")

'D_ALTGW=Offset BaseAddress N Field W0,S_ALTGW=BaseAddress

ALTGW    D_ALTGW,S_ALTGW

'D_ROLWORD = Destination of W0 ALTGW replaces S_GETWORD,(#N=#0)

ROLWORD  D_ROLWORD,S_ROLWORD,#0                 '

debug("after ALTGW/GETWORD    ")

debug(ubin(Src_Register0),ubin(Src_Register1))

debug("ROTATE W0 of 'Src_Register0' too 'D_ROLWORD'")

debug(ubin(D_ALTGW),ubin(D_ROLWORD))

debug("---------------------------------------")

MOV   S_ALTGW,#Src_Register0        'BaseAddress

MOV   D_ALTGW,#1                    'Offset address of Src_Register1 for S Field D[10:2]

SHL   D_ALTGW,#1                    'shift for N Field

OR    D_ALTGW,#1                    'set for N = 1 W1 N Field = D[0]

debug("before ALTGW/ROLWORD   ")

debug(ubin(Src_Register0),ubin(Src_Register1))

debug(ubin(D_ALTGW),ubin(D_ROLWORD))

debug(" ")

'D_ALTGW=Offset BaseAddress N Field W1,S_ALTGW=BaseAddress

ALTGW    D_ALTGW,S_ALTGW

'D_ROLWORD = Destination of W1 ALTGW replaces S_GETWORD,(#N=#1)

ROLWORD  D_ROLWORD,S_ROLWORD,#0                 '

debug("after ALTGW/ROLWORD    ")

debug(ubin(Src_Register0),ubin(Src_Register1))

debug("Write W1 of 'Src_Register1' too 'D_ROLWORD'")

debug(ubin(D_ALTGW),ubin(D_ROLWORD))

debug("---------------------------------------")

_Loop1                NOP

JMP     #_Loop1          'remember # imediate

D_ROLWORD           long      0                                     'Destination of WORD ie) WORD Value

S_ROLWORD           long      0                                     'not required replaced by ALTGW

D_ALTGW             long      0                                     'D[9:1] offset of S  N Field = D[1:0]

S_ALTGW             long      0                                     'S BaseAddress Sorce of Word

Src_Register0       long      %10101010_11001100_10011001_11101110  'Source Register BaseAddress +0

Src_Register1       long      %10101010_11001100_10011001_11101110  'Source Register BaseAddress +1

## 17.34) ALTSN Alter Susequent SETNIB Instruction

| ALTSN D,{#}S | Alter subsequent SETNIB instruction. Next D field = (D[11:3] + S) & $1FF, N field = D[2:0]. D += sign-extended S[17:9]. |
| --- | --- |
| ALTSN D | Alter subsequent SETNIB instruction. Next D field = D[11:3], N field = D[2:0]. |

See 17.25) SETNIB  Set Nibble for examples

The idea is that S/# can serve as a register base address and D can be used as an index.

ALTSN (offset + N field),BaseAddress

Next Instruction :

D Field = (D[10:2} + S)&1FF    N Field = D[1:0]

ALTGN D,{#}S

Alter subsequent SETNIB instruction. Next D field = (D[11:3] + S) & $1FF, N field = D[2:0].

D += sign-extended S[17:9].

1) D field = (D[10:2] + S) & $1FF register to have SETBYTE {#}S,#N point too

2) S is the BaseAddress and  D[11:3] is the offset from the Base

3) N field =  D[2:0] byte to write too in SETBYTE D,{#}S,#N

### 17.34.1_Example_WRD_ALTSN D,{#}S_102

ALTSN D,{#}S

Alter subsequent SETNIB instruction. Next D field = (D[11:3] + S) & $1FF, N field = D[2:0]. D += sign-extended S[17:9].

### 17.34.2.103_Example_WRD_ALTSN D_103

ALTSN D

Alter subsequent SETNIB instruction. Next D field = D[11:3], N field = D[2:0].

## 17.35) ALTGN  Alter Subsequent GETNIB/ROLNIB Instruction

| ALTGN D,{#}S | Alter subsequent GETNIB/ROLNIB instruction. Next S field = (D[11:3] + S) & $1FF, N field = D[2:0]. D += sign-extended S[17:9]. |
| --- | --- |
| ALTGN D | Alter subsequent GETNIB/ROLNIB instruction. Next S field = D[11:3], N field = D[2:0]. |

See 17.26) GETNIB Getnibble from register

See 17.27) ROLNIB Rotate Nibble

The idea is that S/# can serve as a register base address and D can be used as an index.

ALTGN (offset + N field),BaseAddress

Next Instruction :

S Field = (D[11:3} + S)&1FF    N Field = D[2:0]

ALTGN D,{#}S

Alter subsequent SETNIB instruction. Next D field = (D[11:3] + S) & $1FF, N field = D[2:0].

D += sign-extended S[17:9].

1) S Field = (D[11:3] + S) & $1FF register to have  SETNIB  {#}S,#N point too

2) S is the BaseAddress and  D[11:3] is the offset from the Base

3) N field =  D[2:0] nibble to write too in SETNIB D,{#}S,#N

### 17.35.1_Example_WRD_ALTGN D,{#}S_104

ALTGN D,{#}S

Alter subsequent GETNIB/ROLNIB instruction. Next S field = (D[11:3] + S) & $1FF, N field = D[2:0]. D += sign-extended S[17:9].

### 17.35.2_Example_WRD_ALTGN D_105

ALTGN D

Alter subsequent GETNIB/ROLNIB instruction. Next S field = D[11:3], N field = D[2:0].

## 17.36) ALTSB Alter Subsequent SETBYTE Instruction

| 106 | ALTSB D,{#}S | Alter subsequent SETBYTE instruction. Next D field = (D[10:2] + S) & $1FF, N field = D[1:0]. D += sign-extended S[17:9]. |
| --- | --- | --- |
| 107 | ALTSB D | Alter subsequent SETBYTE instruction. Next D field = D[10:2], N field = D[1:0]. |

See 17.28) SETBYTE  Set Byte N into Register

The idea is that S/# can serve as a register base address and D can be used as an index.

ALTSB (offset + N field),BaseAddress

Next Instruction :

D Field = (D[10:2} + S)&1FF    N Field = D[1:0]

ALTGN D,{#}S

Alter subsequent SETNIB instruction. Next D field = (D[10:2] + S) & $1FF, N field = D[1:0].

D += sign-extended S[17:9].

1) D field = (D[10:2] + S) & $1FF register to have SETBYTE D,#N point too

2) S is the BaseAddress and  D[10:2] is the offset from the Base

3) N field =  D[1:0] byte to write too in SETBYTE D,{#}S,#N

### 17.36.1_Example_WRD_ALTSB D,{#}S_106

ALTSB D,{#}S

Alter subsequent SETBYTE instruction. Next D field = (D[10:2] + S) & $1FF, N field = D[1:0]. D += sign-extended S[17:9].

### 17.36.2_Example_WRD_ALTSB D_107

ALTSB D

Alter subsequent SETBYTE instruction. Next D field = D[10:2], N field = D[1:0].

## 17.37) ALTGB Alter Subsequent GETBYTE/ROLBYTE

| ALTGB D,{#}S | Alter subsequent GETBYTE/ROLBYTE instruction. Next S field = (D[10:2] + S) & $1FF, N field = D[1:0]. D += sign-extended S[17:9]. |
| --- | --- |
| ALTGB D | Alter subsequent GETBYTE/ROLBYTE instruction. Next S field = D[10:2], N field = D[1:0]. |

See 17.29) GETBYTE Get Byte N of S  into D

See 17.30) ROLBYTE Rotate Left Byte N of S Into D

The idea is that S/# can serve as a register base address and D can be used as an index.

ALTGB (offset + N field),BaseAddress

Next Instruction :

S Field = (D[10:2} + S)&1FF    N Field = D[1:0]

ALTGB D,{#}S

Alter subsequent GETBYTE/ROLBYTE instruction. Next S field = (D[10:2] + S) & $1FF, N field = D[1:0].

D += sign-extended S[17:9].

1) S field = (D[10:2] + S) & $1FF register to have GETBYTE/ROLBYTE D,{#}S,#N point too

2) S is the BaseAddress and  D[10:2] is the offset from the Base

3) N field =  D[1:0] byte to write too in SETBYTE D,{#}S,#N

### 17.37.1_Example_WRD_ALTGB D,{#}S_108

ALTGB D,{#}S

Alter subsequent GETBYTE/ROLBYTE instruction. Next S field = (D[10:2] + S) & $1FF, N field = D[1:0]. D += sign-extended S[17:9].

### 17.37.2_Example_WRD_ALTGB_109

ALTGB D

Alter subsequent GETBYTE/ROLBYTE instruction. Next S field = D[10:2], N field = D[1:0].

17.xx.110

| ALTSW D,{#}S | Alter subsequent SETWORD instruction. Next D field = (D[9:1] + S) & $1FF, N field = D[0]. D += sign-extended S[17:9]. |
| --- | --- |
| ALTSW D | Alter subsequent SETWORD instruction. Next D field = D[9:1], N field = D[0]. |

## 17.38) ALTSW Alter Subsequent SETWORD

| ALTSW D,{#}S | Alter subsequent SETWORD instruction. Next D field = (D[9:1] + S) & $1FF, N field = D[0]. D += sign-extended S[17:9]. |
| --- | --- |
| ALTSW D | Alter subsequent SETWORD instruction. Next D field = D[9:1], N field = D[0]. |

See 17.31) SETWORD Set S[15:0] into Word N in D

The idea is that S/# can serve as a register base address and D can be used as an index.

ALTSW (offset + N field),BaseAddress

Next Instruction :

D Field = (D[10:2} + S)&1FF    N Field = D[1:0]

ALTGN D,{#}S

Alter subsequent SETNIB instruction. Next D field = (D[10:2] + S) & $1FF, N field = D[1:0].

D += sign-extended S[17:9].

1) D field = (D[10:2] + S) & $1FF register to have SETBYTE D,{#}S,#N point too

2) S is the BaseAddress and  D[10:2] is the offset from the Base

3) N field =  D[1:0] byte to write too in SETBYTE D,{#}S,#N

### 17.38.1_Example_WRD_ALTSW D,{#}S_110

ALTSW D,{#}S

Alter subsequent SETWORD instruction. Next D field = (D[9:1] + S) & $1FF, N field = D[0]. D += sign-extended S[17:9].

### 17.38.2_Example_WRD_ALTSW D_111

ALTSW D

Alter subsequent SETWORD instruction. Next D field = D[9:1], N field = D[0].

## 17.39 ALTGW  Alter Subsequent GETWORD

| ALTGW D,{#}S | Alter subsequent GETWORD/ROLWORD instruction. Next S field = ((D[9:1] + S) & $1FF), N field = D[0]. D += sign-extended S[17:9]. |
| --- | --- |
| ALTGW D | Alter subsequent GETWORD/ROLWORD instruction. Next S field = D[9:1], N field = D[0]. |

See 17.32) GETWORD Get Word N of S into D

ALTGW (offset + N field),BaseAddress

Next Instruction :

S Field = (D[9:1} + S)&1FF    N Field = D[0]

ALTGW D,{#}S

Alter subsequent GETWORD instruction. Next S field = (D[9:1] + S) & $1FF, N field = D[0].

D += sign-extended S[17:9].

1) S field = (D[9:1] + S) & $1FF register to have GETWORD {#}S,#N point too

2) S is the BaseAddress and  D[9:1] is the offset from the Base

3) N field =  D[0] byte to write too in GETWORD {#}S,#N

### 17.39.1_Example_WRD ALTGW D,{#}S_112

ALTGW D,{#},S (112)

Alter subsequent GETWORD/ROLWORD instruction. Next S field = ((D[9:1] + S) & $1FF), N field = D[0]. D += sign-extended S[17:9].

### 17.39.1_Example_WRD ALTGW D_113

ALTGW D (113)

Alter subsequent GETWORD/ROLWORD instruction. Next S field = D[9:1], N field = D[0].

## 17.40) ALTR D Alter result Register D of next insturction

| ALTR D,{#}S | Alter result register address (normally D field) of next instruction to (D + S) & $1FF. D += sign-extended S[17:9]. |
| --- | --- |
| ALTR D | Alter result register address (normally D field) of next instruction to D[8:0]. |

RDSaddress= BaseAdressS[8:0] + OffsetS[17:9] + IndexD[8:0]

S= Offset S[17.9] + BaseAddressS[8:0]
D = IndexD[8:0] + sign-Extended [17:9]

Hardware Registers

DIRA  $1FA      Output enables for P31..P0

DIRB  $1FB      Output enables for P63..P32

OUTA  $1FC     Output states for P31..P0

OUTB $1FD     Output states for P63..P32

INA     $1FE     Input states from P31..P0

INB     $1FF     Input states from P63..P32

XOR D,{#}S {WC/WZ/WCZ}

XOR S into D. D = D ^ S. C = parity of result. *

For some reason (quite a reach for XOR INA,INB) we want the result of XOR X,Y  but you don’t want to destroy register X.  By using the ALTR instruction you can avoid a bunch of move statements. Also some registers cannot be written too. Using the ALTR instruction you can use the instructions without destroying either register. Typicaly in PASM the D register is where the instruction result is stored.

ALTR    index,#table	'set next write to table+index

XOR     INA,INB		'write INA^INB to register[table+index]

ALTR D,{#}S

AlternateRegister = (D + S) & $1FF

D = Offset(Index)

S = BaseAddress(Table

XOR D,{#}S {WC/WZ/WCZ}

XOR S into D. D = D ^ S. C = parity of result. *

### 17.40.1_Example_WRD_ALTR D,{#}S_114

ALTR D,{#}S

Alter result register address (normally D field) of next instruction to (D + S) & $1FF. D += sign-extended S[17:9].

{{17.40.1_Example_WRD_ALTR D,{#}S_114}}

{{

ALTR D,{#}S

Alter result register address (normally D field) of next instruction to (D + S) & $1FF. D += sign-extended S[17:9].

By Means of an example we want the result of XOR X,Y  but you don't want to destroy register X.

By using the ALTR instruction you can avoid a bunch of move statements.

Also some registers cannot be written too. Using the ALTR instruction you can use the assembly instructions without destroying either register and writing the instruction operation to an alternate register.

ALTR    index,#table    'set next write to table+index

XOR     INA,INB         'write INA^INB to register[table+index]

Raddress= BaseAdressS[8:0] + OffsetS[17:9] + IndexD[8:0]

S= Offset S[17.9] + BaseAddressS[8:0]

D = IndexD[8:0] + sign-Extended [17:9]

XOR D,{#}S {WC/WZ/WCZ}

XOR S into D. D = D ^ S. C = parity of result. *

Example

Write the result of XOR Ax,Bx to 'xorResult' not affecting Ax or Bx

}}

CON

_clkfreq = 200_000_000  ''Debug clock must be greater than 10MHZ

P0 =0 , P1 = 1  , P2 = 2

VAR

Byte cogRunning 'cog ID started is returned or -1 if not started

PUB main()

debug("--------------------------------------")

debug("ALTR D,{#}S ")

debug("D += sign-extended S[17:9]. D= S[17:9] + D[8:0]")

debug("Alter result register address (normally D field) of next instruction to (D + S) & $1FF")

debug(" ")

debug("XOR D,{#}S {WC/WZ/WCZ} ")

debug("XOR S into D. D = D ^ S. C = parity of result. *")

debug("Example")

debug("Write the result of XOR Ax,Bx to 'xorResult' not affecting Ax or Bx")

debug("--------------------------------------")

cogRunning := COGINIT(COGEXEC_NEW,@S0_ALTR,0)

debug(udec(cogRunning))

repeat                                    'keep cog 0 running

DAT

ORG   0

S0_ALTR               DRVH    #P0                      'P0 on program running

debug("--------------------------------------")

MOV  xorResult0,#0

MOV  xorResult1,#0

MOV  xorResult2,#0

MOV  D_ALTR,#0           'index =0

MOV  S_ALTR,#xorResult0  'set xorResult0 register BaseAddress

debug("before ALTR/XOR = ",udec(D_ALTR),udec(S_ALTR))

debug(ubin(xorResult0),ubin(xorResult1),ubin(xorResult2))

ALTR D_ALTR,S_ALTR       'preset xorResult0 register

XOR  Ax,Bx

debug("after ALTR/XOR = ",udec(D_ALTR),udec(S_ALTR))

debug(ubin(Ax),ubin(Bx))

debug(ubin(xorResult0),ubin(xorResult1),ubin(xorResult2))

debug("--------------------------------------")

MOV  xorResult0,#0

MOV  xorResult1,#0

MOV  xorResult2,#0

MOV  D_ALTR,#1           'index =1

MOV  S_ALTR,#xorResult0  'set xorResult0 register BaseAddress

debug("before ALTR/XOR = ",udec(D_ALTR),udec(S_ALTR))

debug(ubin(Ax),ubin(Bx))

debug(ubin(xorResult0),ubin(xorResult1),ubin(xorResult2))

ALTR D_ALTR,S_ALTR       'preset xorResult0 register

XOR  Ax,Bx

debug("after ALTR/XOR = ",udec(D_ALTR),udec(S_ALTR))

debug(ubin(Ax),ubin(Bx))

debug(ubin(xorResult0),ubin(xorResult1),ubin(xorResult2))

debug("---------------------------------------")

_Next0                MOV  xorResult0,#0

MOV  xorResult1,#0

MOV  xorResult2,#0

MOV  D_ALTR,Index0           'index

MOV  S_ALTR,#xorResult0  'set xorResult0 register BaseAddress

debug("before ALTR/XOR = ",udec(D_ALTR),udec(S_ALTR))

debug(ubin(Ax),ubin(Bx))

debug(ubin(xorResult0),ubin(xorResult1),ubin(xorResult2))

ALTR D_ALTR,S_ALTR       'preset xorResult0 register

XOR  Ax,Bx

debug("after ALTR/XOR = ",udec(D_ALTR),udec(S_ALTR))

debug(ubin(Ax),ubin(Bx))

debug(ubin(xorResult0),ubin(xorResult1),ubin(xorResult2))

debug("---------------------------------------")

ADD  Index0,#1

CMP  Index0,#3  WZ

IF_Z       JMP  #_Loop1

JMP  #_Next0

_Loop1                NOP

JMP     #_Loop1          'remember # imediate

Index0                long      0

Index1                long      0

Offset                long      10

D_ALTR                long      0              'Index from BaseAddress

S_ALTR                long      0              'BaseAddress of valTable0 + Offset

D_XOR                 long      0              'Operand Ax and ResultRegister

S_XOR                 long      0              'Operand Bx

Ax                    long     %10101010

Bx                    long     %01010101

xorResult0            long      0

xorResult1            long      0

xorResult2            long      0

### 17.40.2_Example_WRD_ALTR D_115

ALTR D

Alter result register address (normally D field) of next instruction to D[8:0].

{{17.40.2_Example_WRD_ALTR D_115}}

{{

ALTR D

Alter result register address (normally D field) of next instruction to D[8:0].

By Means of an example we want the result of XOR X,Y  but you don't want to destroy register X.

By using the ALTR instruction you can avoid a bunch of move statements.

Also some registers cannot be written too. Using the ALTR instruction you can use the assembly instructions without destroying either register and writing the instruction operation to an alternate register.

XOR D,{#}S {WC/WZ/WCZ}

XOR S into D. D = D ^ S. C = parity of result. *

Example

Write the result of XOR Ax,Bx to 'xorResult' not affecting Ax or Bx

}}

CON

_clkfreq = 200_000_000  ''Debug clock must be greater than 10MHZ

P0 =0 , P1 = 1  , P2 = 2

VAR

Byte cogRunning 'cog ID started is returned or -1 if not started

PUB main()

debug("--------------------------------------")

debug("ALTR D ")

debug("Alter result register address (normally D field) of next instruction to D[8:0].")

debug(" ")

debug("XOR D,{#}S {WC/WZ/WCZ} ")

debug("XOR S into D. D = D ^ S. C = parity of result. *")

debug("Example")

debug("Write the result of XOR Ax,Bx to 'xorResult' not affecting Ax or Bx")

debug("--------------------------------------")

cogRunning := COGINIT(COGEXEC_NEW,@S0_ALTR,0)

debug(udec(cogRunning))

repeat                                    'keep cog 0 running

DAT

ORG   0

S0_ALTR               DRVH    #P0                      'P0 on program running

debug("--------------------------------------")

MOV  D_ALTR,#xorResult           'index =0

debug("before ALTR/XOR = ",udec(D_ALTR),ubin(xorResult))

debug(ubin(Ax),ubin(Bx))

ALTR D_ALTR

XOR  Ax,Bx

debug("before ALTR/XOR = ",udec(D_ALTR),ubin(xorResult))

debug(ubin(Ax),ubin(Bx))

debug("--------------------------------------")

_Loop1         NOP

JMP     #_Loop1          'remember # imediate

D_ALTR        long      0              'Index from BaseAddress

Ax                    long     %10101010

Bx                    long     %01010101

xorResult       long      0

## 17.41) ALTD D Alter D Field of next Instruction

| ALTD D,{#}S | Alter D field of next instruction to (D + S) & $1FF. D += sign-extended S[17:9]. |
| --- | --- |
| ALTD D | Alter D field of next instruction to D[8:0]. |

RDSaddress= BaseAdressS[8:0] + OffseS[17:9] + IndexD[8:0]

S= Offset S[17.9] + BaseAddressS[8:0]
D = IndexD[8:0] + sign-Extended [17:9]

### 17.41.1_Example_WRD_ALTD D,{#}S_116

ALTD D,{#}S

Alter D field of next instruction to (D + S) & $1FF. D += sign-extended S[17:9].

{{17.41.1_Example_WRD_ALTD D,{#}S_116}}

{{

ALTD D,{#}S

Alter D field of next instruction to (D + S) & $1FF. D += sign-extended S[17:9].

MOV D,{#}S {WC/WZ/WCZ}

Move S into D. D = S. C = S[31]. *

The idea is that S/# can serve as a register base address and D can be used as an index.

ALTS Offset,BaseAddress

Next Instruction :

S Field = (D + S) & 1FF

D[31:0] = D3D2D1D0

D[31:0] = d31d30d29d28d27d26d25d24d23d22d21d20d19d18d17d16d15d14d13d12d11d10d09d08d07d06d05d04d03d02d01d00

S[31:0] = S3S2S1S0

S[31:0] = s31s30s29s28s27s26s25s24s23s22s21s20s19s18s17s16s15s14s13s12s11s10s09s08s07s06s05s04s03s02s01s00

D Field = (D + S) & $1FF =00000000_00000000_0000000_d08d07d06d05d04d03d02d01d00   typical 9 bit address

Example

Self Modifying Code Alter D Field allows 'Pointer' to be indexed through a table

}}

CON

_clkfreq = 200_000_000  ''Debug clock must be greater than 10MHZ

P0 =0 , P1 = 1  , P2 = 2

VAR

Byte cogRunning 'cog ID started is returned or -1 if not started

PUB main()

debug("--------------------------------------")

debug("ALTD D,{#}S ")

debug("Alter D field of next instruction to (D + S) & $1FF. D += sign-extended S[17:9]. ")

debug(" ")

debug("MOV D,{#}S {WC/WZ/WCZ}")

debug("Move S into D. D = S. C = S[31]. *")

debug(" ")

debug("Example")

debug("Self Modifying Code Alter D Field allows 'Pointer' to be indexed through a table")

debug("--------------------------------------")

cogRunning := COGINIT(COGEXEC_NEW,@S0_ALTD,0)

debug(udec(cogRunning))

repeat                                    'keep cog 0 running

DAT

ORG   0

S0_ALTD                DRVH    #P0                      'P0 on program running

debug("--------------------------------------")

MOV  S_ALTD,#valTable0             'S_ALTD BaseAddress

_Next             MOV  D_ALTD,Index                  'INDEX for D

ALTD D_ALTD,S_ALTD                 'Next Instruction D Field = (D+S)&1FF

'The following MOV instruction source value 0 is just a register place holder

'do not plac #imediate if #0 is used the address in  #addressPointer will be used

MOV  D_MOV,S_MOV          'The source value #0 is just a place holder

debug(udec(S_ALTD),udec(D_ALTD),udec(Index),udec(S_MOV),udec(D_MOV))

debug(udec(valTable0),udec(valTable1),udec(valTable2),udec(valTable3))

debug("---------------------------------------")

ADD   Index,#1

CMP   Index,#4  WZ

IF_Z        JMP   #_Loop1

JMP   #_Next

_Loop1         NOP

JMP     #_Loop1          'remember # imediate

Index                    long      0

D_ALTD                long      0              'MOV instruction Destination

S_ALTD                 long      0              'not required or used

D_MOV                long      0

S_MOV                 long      1234

valTable0             long      0

valTable1             long      0

valTable2             long      0

valTable3             long      0

### 17.41.2_Example_WRD_ALTD D_117

ALTD D

Alter D field of next instruction to D[8:0].

{{17.41.2_Example_WRD_ALTD D_117}}

{{

ALTD D

Alter D field of next instruction to D[8:0].

MOV D,{#}S {WC/WZ/WCZ} (058)      EEEE 0110000 CZI DDDDDDDDD SSSSSSSSS

Move S into D. D = S. C = S[31]. *

D Field = D  & $1FF =00000000_00000000_0000000_d08d07d06d05d04d03d02d01d00   typical 9 bit address

Example

D[31:0] = D3D2D1D0

D[31:0] =d31d30d29d28d27d26d25d24_d23d22d21d20d19d18d17d16_d15d14d13d12d11d10d09d08_d07d06d05d04d03d02d01d00

S[31:0] = S3S2S1S0

S[31:0] = S37S36S35S34S33S32S31S30_ S27S26S25S24S23S22S21S20_ S17S16S15S14S13S12S11S10_ S07S06S05S04S03S02S01S00

Example

'D_ALTD' Destination Register  write value 'S_MOV'  too 'valTable0'

}}

CON

_clkfreq = 200_000_000  ''Debug clock must be greater than 10MHZ

P0 =0 , P1 = 1  , P2 = 2

VAR

Byte cogRunning 'cog ID started is returned or -1 if not started

PUB main()

debug("--------------------------------------")

debug("ALTD D")

debug("Alter D field of next instruction to D[8:0].")

debug(" ")

debug("MOV D,{#}S {WC/WZ/WCZ}")

debug("Move S into D. D = S. C = S[31]. *")

debug("Example")

debug("D_ALTD Destination Register  write value S_MOV  too 'valTable0")

debug("--------------------------------------")

cogRunning := COGINIT(COGEXEC_NEW,@S0_ALTD,0)

debug(udec(cogRunning))

repeat                                    'keep cog 0 running

DAT

ORG   0

S0_ALTD               DRVH    #P0                      'P0 on program running

debug("--------------------------------------")

MOV  D_ALTD,#valTable0  'set address Pointer to first value address

_Next                 ALTD D_ALTD

'The following MOV instruction source value 0 is just a register place holder

'do not plac #imediate if #0 is used the address in  #addressPointer will be used

MOV  D_MOV,S_MOV

debug(udec(D_ALTD),udec(S_MOV),udec(Index),udec(D_MOV))

debug(udec(valTable0),udec(valTable1),udec(valTable2),udec(valTable3))

debug("---------------------------------------")

ADD   D_ALTD,#1

ADD   Index,#1

CMP   Index,#4  WZ

_Z        JMP   #_Loop1

JMP   #_Next

_Loop1         NOP

JMP     #_Loop1          'remember # imediate

Index                 long      0              'D Field for next instruction = D[8:0]

D_ALTD                long      0              'address Pointer

S_ALTD                long      0              'not required or used

D_MOV                long      0              'not required could be a placeholder 0

S_MOV                 long      1234           'value to be written to valTable0

valTable0             long      0

valTable1             long      0

valTable2             long      0

valTable3             long      0

## 17.42) ALTS Alter S field of next Instruction

| ALTS D,{#}S | Alter S field of next instruction to (D + S) & $1FF. D += sign-extended S[17:9]. |
| --- | --- |
| ALTS D | Alter S field of next instruction to D[8:0]. |

RDSaddress= BaseAdressS[8:0] + OffsetD[17:9] + IndexD[8:0]

S= Offset S[17.9] + BaseAddressS[8:0]

D = IndexD[8:0] + sign-Extended [17:9]

Byte Addressing

D[31:0] = D3D2D1D0

S[31:0] = S3S2S1S0

Bit Addressing

D[31:0] =d31d30d29d28d27d26d25d24d23d22d21d20d19d18d17d1d15d14d13d12d11d10d09d08d07d06d05d04d03d02d01d00

S[31:0] = S37S36S35S34S33S32S31S30_ S27S26S25S24S23S22S21S20_ S17S16S15S14S13S12S11S10_ S07S06S05S04S03S02S01S00

The idea is that S/# can serve as a register base address and D can be used as an index.

ALTS Offset,BaseAddress

Next Instruction :

S Field = (D + S) & 1FF

ALTS D,{#}S

Alter subsequent instruction. Next S field = (D + S) & $1FF

D += sign-extended S[17:9].

ALTS D

Alter S field of next instruction to D[8:0]. Next S Field = D[8:0]

### 17.42.1_Example_WRD_ALTS D,{#}S_118

ALTS D,{#}S

Alter S field of next instruction to (D + S) & $1FF. D += sign-extended S[17:9].

{{17.42.1_Example_WRD_ALTS D,{#}S_118}}

{{

ALTS D,{#}S

Alter S field of next instruction to (D + S) & $1FF. D += sign-extended S[17:9].

MOV D,{#}S {WC/WZ/WCZ}

Move S into D. D = S. C = S[31]. *

The idea is that S/# can serve as a register base address and D can be used as an index.

ALTS Offset,BaseAddress

Next Instruction :

S Field = (D + S) & 1FF

D[31:0] = D3D2D1D0

D[31:0] =d31d30d29d28d27d26d25d24_d23d22d21d20d19d18d17d16_d15d14d13d12d11d10d09d08_d07d06d05d04d03d02d01d00

S[31:0] = S3S2S1S0

S[31:0] = S37S36S35S34S33S32S31S30_ S27S26S25S24S23S22S21S20_ S17S16S15S14S13S12S11S10_ S07S06S05S04S03S02S01S00

S Field = (D + S) & $1FF =00000000_00000000_0000000_d08d07d06d05d04d03d02d01d00   typical 9 bit address

Example

Self Modifying Code Alter S Field allows 'S_Pointer + Index' to be indexed through a table

}}

CON

_clkfreq = 200_000_000  ''Debug clock must be greater than 10MHZ

P0 =0 , P1 = 1  , P2 = 2

VAR

Byte cogRunning 'cog ID started is returned or -1 if not started

PUB main()

debug("--------------------------------------")

debug("ALTS D,{#}S ")

debug("Alter S field of next instruction to (D + S) & $1FF. D += sign-extended S[17:9]. ")

debug(" ")

debug("MOV D,{#}S {WC/WZ/WCZ}")

debug("Move S into D. D = S. C = S[31]. *")

debug("Example")

debug("Self Modifying Code Alter S Field allows 'S_Pointer + Index' to be indexed through a table ")

debug("--------------------------------------")

cogRunning := COGINIT(COGEXEC_NEW,@S0_ALTS,0)

debug(udec(cogRunning))

repeat                                    'keep cog 0 running

DAT

ORG   0

S0_ALTS               DRVH    #P0                      'P0 on program running

debug("--------------------------------------")

MOV  S_ALTS,#valTable0  'set Pointer to first address in valTable

_Next                 ALTS S_ALTS,Index

'The following MOV instruction source value 0 is just a register place holder

'do not plac #imediate if #0 is used the address in  #addressPointer will be used

MOV  D_MOV,S_MOV           'The source value #0 is just a place holder

debug("Result D_MOV    =        ",udec(S_ALTS),udec(Index),udec(D_MOV))

debug(udec(valTable0),udec(valTable1),udec(valTable2),udec(valTable3)

debug("---------------------------------------")

ADD   Index,#1

CMP   Index,#4  WZ

IF_Z        JMP   #_Loop1

JMP   #_Next

_Loop1           NOP

JMP     #_Loop1          'remember # imediate

Index                    long      0

D_ALTS                long      0              'Offset from BaseAddress

S_ALTS                 long      0              'BaseAddress of valTable0 + Offset

D_MOV                long      0              'destination of valTable

S_MOV                 long      0              'not required 0 placeholder could be used

valTable0             long      0

valTable1             long      1

valTable2             long      2

valTable3             long      3

### 17.42.2_Example_WRD_ALTS D_119

ALTS D

Alter S field of next instruction to D[8:0].

{{17.42.2_Example_WRD_ALTS D_119}}

{{

ALTS D   (119)

Alter S field of next instruction to D[8:0].

MOV D,{#}S {WC/WZ/WCZ} (058)      EEEE 0110000 CZI DDDDDDDDD SSSSSSSSS

Move S into D. D = S. C = S[31]. *

S Field = D  & $1FF =00000000_00000000_0000000_d08d07d06d05d04d03d02d01d00   typical 9 bit address

Example

D[31:0] = D3D2D1D0

D[31:0] =d31d30d29d28d27d26d25d24_d23d22d21d20d19d18d17d16_d15d14d13d12d11d10d09d08_d07d06d05d04d03d02d01d00

S[31:0] = S3S2S1S0

S[31:0] = S37S36S35S34S33S32S31S30_ S27S26S25S24S23S22S21S20_ S17S16S15S14S13S12S11S10_ S07S06S05S04S03S02S01S00

Example

D_ALTS to obtain values stored in 'valTable0

}}

CON

_clkfreq = 200_000_000  ''Debug clock must be greater than 10MHZ

P0 =0 , P1 = 1  , P2 = 2

VAR

Byte cogRunning 'cog ID started is returned or -1 if not started

PUB main()

debug("--------------------------------------")

debug("ALTS D")

debug("Alter S field of next instruction to D[8:0].")

debug(" ")

debug("MOV D,{#}S {WC/WZ/WCZ}")

debug("Move S into D. D = S. C = S[31]. *")

debug("Example")

debug("D_ALTS to obtain values stored in 'valTable0'")

debug("--------------------------------------")

cogRunning := COGINIT(COGEXEC_NEW,@S0_ALTS,0)

debug(udec(cogRunning))

repeat                                    'keep cog 0 running

DAT

ORG   0

S0_ALTS               DRVH    #P0                      'P0 on program running

debug("--------------------------------------")

MOV  D_ALTS,#valTable0  'set address Pointer to first value address

_Next                 ALTS D_ALTS

'The following MOV instruction source value 0 is just a register place holder

'do not plac #imediate if #0 is used the address in  #addressPointer will be used

MOV  D_MOV,S_MOV

debug(udec(D_ALTS),udec(Index),udec(D_MOV))

debug("---------------------------------------")

ADD   D_ALTS,#1

ADD   Index,#1

CMP   Index,#4  WZ

IF_Z        JMP   #_Loop1

JMP   #_Next

_Loop1         NOP

JMP     #_Loop1          'remember # imediate

Index                    long      0

D_ALTS                long      0              'address Pointer

S_ALTS                 long      0              'not required or used

D_MOV                long      0

S_MOV                 long      0              'not required 0 place holder not #0

valTable0             long      0

valTable1             long      1

valTable2             long      2

valTable3             long      3

## 17.43)  ALTB D Alter D field of next instruction usually associated with Bits

| ALTB D,{#}S | Alter D field of next instruction to (D[13:5] + S) & $1FF. D += sign-extended S[17:9]. |
| --- | --- |
| ALTB D | Alter D field of next instruction to D[13:5]. |

For accessing bit fields that span multiple registers, there is the ALTB instruction which sums D[13:5]

and S/#[8:0] values to compute an address which is substituted into the next instruction's D field. It can be used with and without S/#:

ALTB    bitindex,#base	'set next D field to base+bitindex[13:5]

BITC    0,bitindex	'write C to bit[bitindex[4:0]]

ALTB    bitindex		'set next D field to bitindex[13:5]

TESTB   0,bitindex	WC	'read bit[bitindex[4:0]] into C

TESTB D,{#}S WC/WZ (034)

Test bit S[4:0] of D, write to C/Z. C/Z = D[S[4:0]].

BITC D,{#}S {WCZ}   (044)

Bits D[S[9:5]+S[4:0]:S[4:0]] = C. Other bits unaffected. Prior SETQ overrides S[9:5].

C,Z = original D[S[4:0]].

This instruction can be used to set a bit to C carry or a group of bits to C carry

### 17.43.1_Example_WRD_ALTB D,{#}S_120

ALTB D,{#}S

Alter D field of next instruction to (D[13:5] + S) & $1FF. D += sign-extended S[17:9].

Next Instruction D Field = (D[13:5] +S[17:9] + S) & $1FF

{{17.43.1_Example_WRD_ALTB D,{#}S_120}}

{{

ALTB D,{#}S (120)

Alter D field of next instruction to (D[13:5] + S) & $1FF. D += sign-extended S[17:9].

S = OffsetBitIndexS[17.9] | BaseAddressS[8:0]

D = IndexWordD[13:5] | BitIndexD[4:0]

Next Instruction D Field = (D[13:5] +S[17:9] + S) & $1FF

Note: S[17:9] is a bit offset in addition to the BitIndex

TESTB D,{#}S WC/WZ (034)

Test bit S[4:0] of D, write to C/Z. C/Z = D[S[4:0]].

Example

Use ALTB BaseAddress with Index and Offset to test bit

B10 is test bit in valBit2 IndexWord = 2 IndexBit D[4:0]= %1001] = 9 OffsetBitIndex = 1

Test by setting value 0/1 to valBit2

}}

CON

_clkfreq = 200_000_000  ''Debug must be enabled clock must be greater than 10MHZ for Debug

P0 =0 , P1 = 1  , P2 = 2

VAR

Byte cogRunning 'cog ID started is returned or -1 if not started

PUB main()

debug("--------------------------------------")

debug("ALTB D,{#}S  (120)")

debug("Alter D field of next instruction to (D[13:5] + S) & $1FF.")

debug("D += sign-extended S[17:9].")

debug(" ")

debug("TESTB D,{#}S WC/WZ (034) TESTB 0-0,D_ALTB")

debug("Test bit S[4:0] of D, write to C/Z. C/Z = D[S[4:0]].")

debug(" ")

debug("Example")

debug("Use ALTB BaseAddress with Index and Offset to test bit")

debug("B10 is test bit in valBit2 IndexWord = 2 IndexBit D[4:0]= %1100] = 9 OffsetBitIndex = 1")

debug("Test by setting value 0/1 to valBit2")

debug("--------------------------------------")

cogRunning := COGINIT(COGEXEC_NEW,@S0_TESTB,0)

debug(udec(cogRunning))

repeat                                    'keep cog 0 running

DAT

ORG   0

S0_TESTB              DRVH    #P0                       'P0 on program running

MOV     D_ALTB,IndexWord          'Load Index from BaseAddress , word = BaseAddress + IndexWord

SHL     D_ALTB,#5                 'Shift Index to D[13:5]

OR      D_ALTB,IndexBit  '         Set BitIndex D[4:0] = 9

'D_ALTB = IndexWordD[13:5] | BitIndexD[4:0]

MOV     valBit0Address,#valBit0

'D += sign-extended S[17:9].

'D Field = (D[13:5]  +  S[17:9]        + S ) & $1FF

'D Field = (IndexWord + OffsetBitIndex + BaseAddress) & $1FF

MOV     S_ALTB,OffsetBitIndex      'Load Bit Offset from BitIndex

SHL     S_ALTB,#9                  'move OffsetBitIndex to S[17:9]

OR      S_ALTB,#valBit0  '         'BaseAddress of Table valBit0

'S_ALTB =  OffsetBitIndexS[17:9] | BaseAddressS[8:0]

MODZ    _CLR   WZ        'set Z = 0

MODC    _CLR   WC        'set C = 0

IF_C   MOV     Carry,#1         'true  C = 1

IF_NC  MOV     Carry,#0         'false C = 0

IF_Z   MOV     Zero,#1          'true  Z = 1

IF_NZ  MOV     Zero,#0          'false Z = 0

debug("--------------------------------------")

debug("before ALTB/TESTB check valBit2 word bit B10")

debug(udec(Zero),udec(Carry))

debug(ubin(D_ALTB),ubin(S_ALTB),ubin(valBit0Address))

debug(ubin(valBit0),ubin(valBit1))

debug(ubin(valBit2),ubin(valBit3))

debug(" ")

MOV     D_TESTB,#0

ALTB    D_ALTB,S_ALTB

TESTB   D_TESTB,D_ALTB  WC

'D_TESTB Place Holder could be 0-0 ALTB will load D Field = (D[13:5] + S) & $1FF

IF_C   MOV     Carry,#1         'true  C = 1

IF_NC  MOV     Carry,#0         'false C = 0

IF_Z   MOV     Zero,#1          'true  Z = 1

IF_NZ  MOV     Zero,#0          'false Z = 0

debug("after ALTB/TESTB check valBit2 word bit B10  ")

debug(udec(Zero),udec(Carry))

debug(ubin(D_ALTB),ubin(S_ALTB))

debug(ubin(valBit0),ubin(valBit1))

debug(ubin(valBit2),ubin(valBit3))

debug("--------------------------------------")

_Loop1         NOP

JMP     #_Loop1            'remember # imediate

Carry                 long      0

Zero                  long      0

OffsetBitIndex      long      1               'To be Loaded to OffsetS[17:9]

IndexWord            long      2               'To be Loaded to IndexD[13:5]

IndexBit                 long      9               'To be Loaded to BitIndexD[4:0]

valBit0Address     long      0               'BaseAdress of Table valBit = valBit0

D_ALTB                  long      0               'ALTB uses destination value D_ALTB

S_ALTB                  long      0               'S= OffsetS[13:5] or  BaseAddressS[8:0]

D_TESTB                long      0               'D_TESTB not required 0-0 could be used ALTB writes to D[31:0]

S_TESTB                 long      0               'S_TESTB = D_ALTB not used

zeroBits                 long      %00000000_00000000_00000000_00000000

oneBits                   long      %11111111_11111111_11111111_11111111

valBit0                    long      %00000000_00000000_00000000_00000000     'BaseAddress

valBit1                    long      %00000000_00000000_00000000_00000000     'B9 sets Carry

valBit2                    long      %00000000_00000000_00000100_00000000

valBit3                     long      %00000000_00000000_00000000_00000000

### 17.43.2_Example_WRD_ALTB D_121

ALTB D

Alter D field of next instruction to D[13:5].

{{17.43.1_Example_WRD_ALTB D,{#}S_120}}

{{

ALTB D  (121)

Alter D field of next instruction to D[13:5].

TESTB D,{#}S WC/WZ (034)

Test bit S[4:0] of D, write to C/Z. C/Z = D[S[4:0]].

Example

Use ALTB BaseAddress

Defined in valBit0 bit postition B9 S[4:0= %1001} = 9

Check by changing word valBit0 B9 from 1 to zero

}}

CON

_clkfreq = 200_000_000  ''Debug must be enabled clock must be greater than 10MHZ for Debug

P0 =0 , P1 = 1  , P2 = 2

VAR

Byte cogRunning 'cog ID started is returned or -1 if not started

PUB main()

debug("--------------------------------------")

debug("ALTB D (121)")

debug("Alter D field of next instruction to D[13:5]..")

debug(" ")

debug("TESTB D,{#}S WC/WZ (034)")

debug("Test bit S[4:0] of D, write to C/Z. C/Z = D[S[4:0]].")

debug(" ")

debug("Example")

debug("Defined in valBit0 bit postition B9 S[4:0]= %1001} = 9")

debug("--------------------------------------")

cogRunning := COGINIT(COGEXEC_NEW,@S0_TESTB,0)

debug(udec(cogRunning))

repeat                                    'keep cog 0 running

DAT

ORG   0

S0_TESTB              DRVH    #P0                 'P0 on program running

MOV     valBit0Address,#valBit0

MOV     D_ALTB,#valBit0    'Load Offset from BaseAddress

SHL     D_ALTB,#5          'move address to D{13:5}

debug("**********************")

debug(ubin(D_ALTB),ubin(valBit0Address))

debug("**********************")

MODZ    _CLR   WZ       'set Z = 0

MODC    _CLR   WC       'set C = 0

IF_C   MOV     Carry,#1        'true  C = 1

IF_NC  MOV     Carry,#0        'false C = 0

IF_Z   MOV     Zero,#1         'true  Z = 1

IF_NZ  MOV     Zero,#0         'false Z = 0

debug("--------------------------------------")

debug("before ALTB/TESTB check valBit0 word bit B9")

debug(udec(Zero),udec(Carry))

debug(ubin(D_ALTB))

debug(ubin(valBit0),ubin(valBit1))

debug(ubin(valBit2),ubin(valBit3))

debug(" ")

'NEXT Instruction D Field = (D[13:5])

ALTB    D_ALTB

'D_TESTB Place Holder could be 0 ALTB will load D Field = (D[13:5])

TESTB   D_TESTB,S_TESTB  WC    'WC use Carry for testing

IF_C   MOV     Carry,#1         'true  C = 1

IF_NC  MOV     Carry,#0         'false C = 0

IF_Z   MOV     Zero,#1          'true  Z = 1

IF_NZ  MOV     Zero,#0          'false Z = 0

debug("after ALTB/TESTB check valBit0 word bit B9  ")

debug(udec(Zero),udec(Carry))

debug(ubin(D_ALTB))

debug(ubin(valBit0),ubin(valBit1))

debug(ubin(valBit2),ubin(valBit3))

debug("--------------------------------------")

_Loop1                NOP

JMP     #_Loop1            'remember # imediate

Carry                 long      0

Zero                  long      0

valBit0Address        long      0                'valBit0 address

D_ALTB                long      0                'select word for bit check

D_TESTB               long      0                'Place Holder could be 0 ALTB will load D Field = (D[13:5])

S_TESTB               long      9                'S_TESTB = S[4:0] = 9 B9 to be Tested

valBit0               long      %11111111_00000000_1111111_00000000  'BaseAddress

valBit1               long      %10101010_11001100_11100011_11100011

valBit2               long      %10000000_10000011_10000010_10000011

valBit3               long      %10000000_10000001_10000001_10000001

valBit4               long      %10000000_10000001_10000001_10000001

valBit5               long      %10000000_10000001_10000001_10000001

## 17.44) ALTI Substitute next instructions I/R/D/S Fields

| ALTI D,{#}S | Substitute next instruction's I/R/D/S fields with fields from D, per S. Modify D per S. |
| --- | --- |
| ALTI D | Execute D in place of next instruction. D stays same. |

I = Instruction Field  [27:21]

R = Result Field [27:19]

D = Destination Field [17:9]

S = Source Field [8:0]

Condition        Instruction              Effects(Flags)            Destination                                   Source_________

31 30 29 28 | 27 26 25 24 23 22 21 |  20 19 18 | 17 16 15 14 13 12 11 10 09 | 08 07 06 05 04 03 02 01 00

E    E   E   E         0   0  0  0  0  0   0             C  Z  I                  D D D D D D D D  D               S  S  S  S  S   S  S  S  S

4 bit = 15               7 bit = 127                 3 bit =7         9 bit = 511  max address        9 bit = 511 max address

First, ALTI For more complex S field, D field, and result register substitutions, there is the ALTI instruction. ALTI actually does a few different things can be used to individually increment or decrement three different nine-bit fields within a register.

Second, ALTI can substitute each of those fields (before incrementing or decrementing) into the next instruction's S field, D field, or result register address, in the same way ALTS, ALTD, and ALTR do. Lastly, ALTI can substitute D[31..18] into the next instruction's upper bits [31..18] to enable full instruction substitution with a register's contents.

ALTI    D,S/#			'modify D and/or next instruction's fields according to S/#

S/# = %rrr_ddd_sss_RRR_DDD_SSS

%rrr		Result register field D[27..19] increment/decrement masking

%ddd		D register field D[17..9] increment/decrement masking

%sss		S register field D[8..0] increment/decrement masking

%rrr/%ddd/%sss:

000 = 9 bits increment/decrement (default, full span)

001 = 8 LSBs increment/decrement (256-register looped buffer)

010 = 7 LSBs increment/decrement (128-register looped buffer)

011 = 6 LSBs increment/decrement (64-register looped buffer)

100 = 5 LSBs increment/decrement (32-register looped buffer)

101 = 4 LSBs increment/decrement (16-register looped buffer)

110 = 3 LSBs increment/decrement (8-register looped buffer)

111 = 2 LSBs increment/decrement (4-register looped buffer)

%RRR		result register / instruction modification:

000 = D[27..19] stays same, no result register substitution

001 = D[27..19] stays same, but result register writing is canceled

010 = D[27..19] decrements per %rrr, no result register substitution

011 = D[27..19] increments per %rrr, no result register substitution

100 = D[27..19] sets next instruction's result register, stays same

101 = D[31..18] substitutes into next instruction's [31..18] (execute D)

110 = D[27..19] sets next instruction's result register, decrements per %rrr

111 = D[27..19] sets next instruction's result register, increments per %rrr

%DDD		D field modification:

x0x = D[17..9] stays same

x10 = D[17..9] decrements per %ddd

x11 = D[17..9] increments per %ddd

0xx = no D field substitution

1xx = D[17..9] substitutes into next instruction's D field [17..9]

%SSS		S field modification:

x0x = D[8..0] stays same

x10 = D[8..0] decrements per %sss

x11 = D[8..0] increments per %sss

0xx = no S field substitution

1xx = D[8..0] substitutes into next instruction's S field [8..0]

Here are some examples of ALTI usage:

'set next D and S fields, increment ptrs[17:9] and ptrs[8:0]

ALTI    ptrs,#%111_111

ADD     0,0			'add registers

ALTI    inst,#%101_100_100	'execute inst (same as 'ALTI inst')

NOP				'NOP becomes inst

Note: Not going to do examples at this time modifying Instructions not of high priority for learning existing instructions. If someone wants to submit an example please do so.

### 17.44.1_Example_WRD_ALTI D,{#}S_122

ALTI D,{#}S

Substitute next instruction's I/R/D/S fields with fields from D, per S. Modify D per S.

### 17.44.2_Exampl_WRD_ALTI D_123

ALTI D

Execute D in place of next instruction. D stays same.

## 17.45) SETR SETD SETS  Set Instruction Field of Register

| SETR D,{#}S | Set R field of D to S[8:0]. D = {D[31:28], S[8:0], D[18:0]}. |
| --- | --- |
| SETD D,{#}S | Set D field of D to S[8:0]. D = {D[31:18], S[8:0], D[8:0]}. |
| SETS D,{#}S | Set S field of D to S[8:0]. D = {D[31:9], S[8:0]}. |

I = Instruction Field  [27:21]

R = Result Field [27:19]

D = Destination Field [17:9]

S = Source Field [8:0]

Condition        Instruction              Effects(Flags)            Destination                                   Source_________

31 30 29 28 | 27 26 25 24 23 22 21 |  20 19 18 | 17 16 15 14 13 12 11 10 09 | 08 07 06 05 04 03 02 01 00

E    E   E   E         0   0  0  0  0  0   0             C  Z  I                  D D D D D D D D  D               S  S  S  S  S   S  S  S  S

4 bit = 15               7 bit = 127                 3 bit =7         9 bit = 511  max address        9 bit = 511 max address

Ea  IF_Z   MOV PRO,PR1    WZMOV     D,{#}S   {WC/WZ/WCZ} EEEE 0110000 CZI DDDDDDDDD SSSSSSSSS

IF_Z    EEEE = 1010

MOV     Instruction = 0110000

C       Effects C = 0  (no WC)

Z       Effects Z = 1  (WZ)

I       Effects I =0   (# not present0

PR0     DDDDDDDDD = $1D8 = %111011000

PR1     SSSSSSSSSS     = $1D9 =%111011001

Condition        Instruction              Effects(Flags)            Destination                                   Source_________

31 30 29 28 | 27 26 25 24 23 22 21 |  20 19 18 | 17 16 15 14 13 12 11 10 09 | 08 07 06 05 04 03 02 01 00

1    0   1    0        0   1  1  0  0  0   0             0  1   0                  1 1 1 0 1 1 0 0  0                1 1 1 0 1 1 0  0  1

4 bit = 15               7 bit = 127                 3 bit =7         9 bit = 511  max address        9 bit = 511 max address

The SETS/SETD/SETR instructions allow you to write the S field, D field and instruction field of a register without affecting other bits. They copy the lower 9 bits of S/# into their respective 9-bit field within D. These instructions are useful for establishing the fields that will be used by ALTI:

SETS    D,S/#			'set D[8:0] to S/#[8:0]

SETD    D,S/#			'set D[17:9] to S/#[8:0]

SETR    D,S/#			'set D[27:19] to S/#[8:0]

S/#  = operate on bit field contained in S[31:0] or #value bit field #[31:0]

SETS/SETD/SETR can also be used in self-modifying cog-register code. After modifying a cog register, It is necessary to elapse two instructions before executing the modified register, due to pipelining:

SETR    inst,op		'set reg[27:19] to op[8:0]

NOP				'first spacer instruction, could be anything

NOP				'second spacer instruction, could be anything

inst	MOV     x,y			'operate on x using y, MOV can become AND/OR/XOR/etc.

### Nt Not going to do examples at this time modifying Instructions not of high priority for learning existing instructions. If someone wants to submit an example please do so.
17.45.1_Example_WRD_Get_MOV_Instruction_Code

IF_Z   MOV PRO,PR1    WZ

{{17.45.1_Example_Get_MOV_Istruction}}

{{

IF_Z   MOV PRO,PR1    WZ

}}

CON

_clkfreq = 200_000_000  ''Debug clock must be greater than 10MHZ

P0 =0 , P1 = 1  , P2 = 2

VAR

Byte cogRunning 'cog ID started is returned or -1 if not started

PUB main()

debug("--------------------------------------")

debug("Example")

debug("Get  IF_Z   MOV PRO,PR1    WZ")

debug("--------------------------------------")

cogRunning := COGINIT(COGEXEC_NEW,@S0_GetMOV,0)

debug(udec(cogRunning))

repeat                                    'keep cog 0 running

DAT

ORG   0

S0_GetMOV             DRVH    #P0                      'P0 on program running

debug("--------------------------------------")

MOV PR3,MOV_Instr

debug("IF_Z MOV PR0,PR1  WZ =  ",ubin(MOV_Instr))

debug("--------------------------------------")

_Loop1                NOP

JMP #_Loop1         'don't forget imediat

MOV_Instr     IF_Z    MOV PR0,PR1   WZ

## 17.46) DECOD value 0-31 into Long with Coresponding bit set High

| DECOD D,{#}S | Decode S[4:0] into D. D = 1 << S[4:0]. |
| --- | --- |
| DECOD D | Decode D[4:0] into D. D = 1 << D[4:0]. |

DECOD PASM instruction performs the same way as Bitwise Decode  spin operator

|<  (Pin ≔ |< PinNum Bitwise Decode decodes  a value (0-31) into a 32 bit long value with a single bit set high corresponding to the bit position of the original value

### 17.46.1_Example_WRD_DECOD D,{S}#_127

DECOD D,{#}S

Decode S[4:0] into D. D = 1 << S[4:0].

{{17.46.1_Example_WRD_DECOD D,{#}S_127}}

{{

DECOD D,{#}S

Decode S[4:0] into D. D = 1 << S[4:0].

Example

DECOD S_DECOD 0-31 into D_DECOD

}}

CON

_clkfreq = 200_000_000  ''Debug must be enabled clock must be greater than 10MHZ for Debug

P0 =0 , P1 = 1  , P2 = 2

VAR

Byte cogRunning 'cog ID started is returned or -1 if not started

PUB main()

debug("--------------------------------------")

debug("DECOD D,{#}S")

debug("Decode S[4:0] into D. D = 1 << S[4:0].")

debug(" ")

debug("Example")

debug("DECOD S_DECOD 0-31 into D_DECOD")

debug("--------------------------------------")

cogRunning := COGINIT(COGEXEC_NEW,@S0_DECOD,0)

debug(udec(cogRunning))

repeat                                    'keep cog 0 running

DAT

ORG   0

S0_DECOD              DRVH    #P0                 'P0 on program running

_Loop0         MOV   S_DECOD,Index

DECOD D_DECOD,S_DECOD

debug(ubin(D_DECOD),udec(S_DECOD))

ADD  Index,#1

CMP  Index,#32 WZ

IF_Z       JMP  #_Loop1     '# remember Imediate

JMP  #_Loop0

_Loop1         NOP

JMP     #_Loop1            'remember # imediate

Index               long     0

D_DECOD       long     0

S_DECOD        long     0

### 17.46.2_Example_WRD_DECOD D_128

DECOD D

Decode D[4:0] into D. D = 1 << D[4:0].

{{17.46.2_Example_WRD_DECOD D,{#}S_128}}

{{

DECOD D

Decode D[4:0] into D. D = 1 << D[4:0].

Example

DECOD  0-31 into D_DECOD

}}

CON

_clkfreq = 200_000_000  ''Debug must be enabled clock must be greater than 10MHZ for Debug

P0 =0 , P1 = 1  , P2 = 2

VAR

Byte cogRunning 'cog ID started is returned or -1 if not started

PUB main()

debug("--------------------------------------")

debug("DECOD D")

debug("Decode D[4:0] into D. D = 1 << D[4:0].")

debug(" ")

debug("Example")

debug("DECOD S_DECOD 0-31 into D_DECOD")

debug("--------------------------------------")

cogRunning := COGINIT(COGEXEC_NEW,@S0_DECOD,0)

debug(udec(cogRunning))

repeat                                    'keep cog 0 running

DAT

ORG   0

S0_DECOD              DRVH    #P0                 'P0 on program running

_Loop0         MOV   D_DECOD,Index

DECOD D_DECOD

debug(udec(Index),ubin(D_DECOD))

ADD  Index,#1

CMP  Index,#32 WZ

IF_Z       JMP  #_Loop1     '# remember Imediate

JMP  #_Loop0

_Loop1         NOP

JMP     #_Loop1            'remember # imediate

Index                long     0

D_DECOD        long     0

## 17.47)BMASKD Get LSB Justifie bit mask

| BMASK D,{#}S | Get LSB-justified bit mask of size (S[4:0] + 1) into D. D = ($0000_0002 << S[4:0]) - 1. |
| --- | --- |
| BMASK D | Get LSB-justified bit mask of size (D[4:0] + 1) into D. D = ($0000_0002 << D[4:0]) - 1. |

### 17.47.1_Example_WRD_BMASK D,{#}S_129

BMASK D,{#}S

Get LSB-justified bit mask of size (S[4:0] + 1) into D. D = ($0000_0002 << S[4:0]) - 1.

{{17.47.1_Example_WRD_BMASK D,{#}S_129}}

{{

BMASK D,{#}S  (129)

Get LSB-justified bit mask of size (S[4:0] + 1) into D. D = ($0000_0002 << S[4:0]) - 1.

Example

Get Mask from 0-31 and then Mask out TestMask value

}}

CON

_clkfreq = 200_000_000  ''Debug must be enabled clock must be greater than 10MHZ for Debug

P0 =0 , P1 = 1  , P2 = 2

VAR

Byte cogRunning 'cog ID started is returned or -1 if not started

PUB main()

debug("--------------------------------------")

debug("BMASK D,{#}S  ")

debug(" ")

debug("Example")

debug("Get Mask from 0-31 and then Mask out TestMask value")

debug("--------------------------------------")

cogRunning := COGINIT(COGEXEC_NEW,@S0_BMASK,0)

debug(udec(cogRunning))

repeat                                    'keep cog 0 running

DAT

ORG   0

S0_BMASK              DRVH    #P0                 'P0 on program running

_Loop0                MOV   S_BMASK,Index

MOV   TestMask,valMask

BMASK D_BMASK,S_BMASK

AND   TestMask,D_BMASK

debug(udec(Index),ubin(valMask),ubin(D_BMASK),ubin(S_BMASK),ubin(TestMask))

ADD  Index,#1

CMP  Index,#32 WZ

IF_Z          JMP  #_Loop1     '# remember Imediate

JMP  #_Loop0

_Loop1                NOP

JMP     #_Loop1            'remember # imediate

Index          long     0

D_BMASK        long     0

S_BMASK        long     0

TestMask       long     0

valMask        long     %10101010_10101010_10101010_10101010

### 17.47.2_Example_WRD_BMASK D_130

BMASK D

Get LSB-justified bit mask of size (D[4:0] + 1) into D. D = ($0000_0002 << D[4:0]) - 1.

{{17.47.2_Example_WRD_BMASK D_130}}

{{

BMASK D

Get LSB-justified bit mask of size (D[4:0] + 1) into D. D = ($0000_0002 << D[4:0]) - 1.

Example

Get Mask from 0-31 and then Mask out TestMask value

}}

CON

_clkfreq = 200_000_000  ''Debug must be enabled clock must be greater than 10MHZ for Debug

P0 =0 , P1 = 1  , P2 = 2

VAR

Byte cogRunning 'cog ID started is returned or -1 if not started

PUB main()

debug("--------------------------------------")

debug("BMASK D,{#}S  ")

debug(" ")

debug("Example")

debug("Get Mask from 0-31 and then Mask out TestMask value")

debug("--------------------------------------")

cogRunning := COGINIT(COGEXEC_NEW,@S0_BMASK,0)

debug(udec(cogRunning))

repeat                                    'keep cog 0 running

DAT

ORG   0

S0_BMASK              DRVH    #P0                 'P0 on program running

_Loop0                MOV   D_BMASK,Index

MOV   TestMask,valMask

BMASK D_BMASK

AND   TestMask,D_BMASK

debug(udec(Index),ubin(valMask),ubin(D_BMASK),ubin(TestMask))

ADD  Index,#1

CMP  Index,#32 WZ

IF_Z          JMP  #_Loop1     '# remember Imediate

JMP  #_Loop0

_Loop1                NOP

JMP     #_Loop1            'remember # imediate

Index          long     0

D_BMASK        long     0

TestMask       long     0

valMask        long     %10101010_10101010_10101010_10101010

## 17.48) CRCBIT Cyclic Reduncy Check of Byte

| CRCBIT D,{#}S | Iterate CRC value in D using C and polynomial in S. If (C XOR D[0]) then D = (D >> 1) XOR S, else D = (D >> 1). |
| --- | --- |
| CRCNIB D,{#}S | Iterate CRC value in D using Q[31:28] and polynomial in S. Like CRCBIT x 4. Q = Q << 4. Use 'REP #n,#1'+SETQ+CRCNIB+CRCNIB+CRCNIB... |

pub crc8x_pasm2(p_src, n) : crc | b, t1

'' Returns CRC8 of n bytes at p_src

org

.loop                   rdbyte  b, p_src

add     p_src, #1

rep     @.done, #8

shr     b, #1                 wc

crcbit  crc, #$8C

.done

djnz    n, #.loop

end

### 17.48.1_Example_WRD_CRCBIT D,{#}S_131

CRCBIT D,{#}S

Iterate CRC value in D using C and polynomial in S. If (C XOR D[0])

then D = (D >> 1) XOR S, else D = (D >> 1).

{{17.48.1_Example_WRD_CRCBIT D,{#}S_131}}

{{

CRCBIT D,{#}S

Iterate CRC value in D using C and polynomial in S. If (C XOR D[0])

then D = (D >> 1) XOR S, else D = (D >> 1).

Example

}}

CON

_clkfreq = 200_000_000  ''Debug must be enabled clock must be greater than 10MHZ for Debug

P0 =0 , P1 = 1  , P2 = 2

VAR

Byte src[4]

long byteNum

long address_src

long answerCRC8

PUB main()|x

debug("--------------------------------------")

debug("CRCBIT D,{#}S")

debug("Iterate CRC value in D using C and polynomial in S. If (C XOR D[0])")

debug("then D = (D >> 1) XOR S, else D = (D >> 1).")

debug("Example")

debug(" ")

debug("--------------------------------------")

byteNum := 4

src[0] := $77

src[1] := $78

src[2] := $79

src[3] := $7A

debug("before CRC8 Calculation",ubin(answerCRC8))

repeat x from 0 to 3

address_src[x] := @src[x]

debug(udec(address_src[x]),ubin(src[x]),uhex(src[x]))

answerCRC8 := CR8(@src,byteNum)

debug("after CRC8 Calculation",ubin(answerCRC8))

debug("--------------------------------------")

repeat                                    'keep cog 0 running

PUB CR8(p_src,n): crc | b

org

S0_CRCBIT             drvh     #P0

.loop                 rdbyte     b, p_src

debug(ubin(b),udec(p_src))

add     p_src, #1

rep     @.done, #8

shr     b, #1                 wc

crcbit  crc, #$8C

.done                 djnz    n, #.loop

end

### 17.48.2_Example_WRD_CRCNIB D,{#}S_132

CRCNIB D,{#}S

Iterate CRC value in D using Q[31:28] and polynomial in S.

Like CRCBIT x 4. Q = Q << 4. Use 'REP #n,#1'+SETQ+CRCNIB+CRCNIB+CRCNIB...

{{17.48.2_Example_WRD_CRCNIB D,{#}S_132}}

{{

CRCNIB D,{#}S  (132)

Iterate CRC value in D using Q[31:28] and polynomial in S.

Like CRCBIT x 4. Q = Q << 4. Use 'REP #n,#1'+SETQ+CRCNIB+CRCNIB+CRCNIB...

Example

Generate CRC8 for 4 byte data stream

}}

CON {Processor Timing}

_clkfreq  = 200_000_000  'processor clock speed

P0 =0 , P1 = 1  , P2 = 2

var

byte src[4]

long address_src[4]

long num_long

long answerCRC8

PUB main()|temp,x

debug("--------------------------------------")

debug("CRCNIB D,{#}S  (132)")

debug("Iterate CRC value in D using Q[31:28] and polynomial in S.")

debug("Like CRCBIT x 4. Q = Q << 4. Use 'REP #n,#1'+SETQ+CRCNIB+CRCNIB+CRCNIB..")

debug(" ")

debug("Example")

debug("Generate CRC8 for 4 byte data stream ")

debug("--------------------------------------")

answerCRC8 := 0

debug("before CRC8 calculation ",ubin(answerCRC8))

num_long := 4

src[0] := $77

src[1] := $78

src[2] := $79

src[3] := $7A

repeat x from 0 to 3

address_src[x] := @src[x]

debug(udec(address_src[x]),ubin(src[x]),uhex(src[x]))

answerCRC8 := crc8(@src,num_long)

debug("after CRC8 calculation ",ubin(answerCRC8))

debug("--------------------------------------")

repeat

pub crc8(p_src, n) : crc | b

'' Returns CRC8 of n bytes from data at p_src

org

S0_CRCBIT       drvh     #P0                   'program running

.loop           rdbyte    b, p_src

debug(ubin(b),udec(p_src))

add       p_src, #1

rev       b

setq      b

crcnib    crc, #$8C

crcnib    crc, #$8C

djnz      n, #.loop

end

### 17.48.3_Example_WRD_CRCBIT_Function

This is included with Progam examples but has not been tested

## 17.49) MUXNITS/MUXNIB D,{#}S Set  bit in D from S

| 133 | MUXNITS D,{#}S | For each non-zero bit pair in S, copy that bit pair into the corresponding D bits, else leave that D bit pair the same. |
| --- | --- | --- |
| 134 | MUXNIBS D,{#}S | For each non-zero nibble in S, copy that nibble into the corresponding D nibble, else leave that D nibble the same. |

### 17.49.1_Example_WRD_MUXNITS D,{#}S _133

MUXNITS D,{#}S

For each non-zero bit pair in S, copy that bit pair into the corresponding D bits, else leave that D bit pair the same.

{{17.49.1_Example_WRD_MUXNITS D,{#}S_133}}

{{

MUXNITS D,{#}S    (133)

For each non-zero bit pair in S, copy that bit pair into the corresponding D bits, else leave that D bit pair the same.

Example

"Copy bit pattern ins S to D"

}}

CON

_clkfreq = 200_000_000  ''Debug must be enabled clock must be greater than 10MHZ for Debug

P0 =0 , P1 = 1  , P2 = 2

VAR

byte cogRunning 'cog ID started is returned or -1 if not started

PUB main()|x

debug("--------------------------------------")

debug("MUXNITS D,{#}S    (133) ")

debug("For each non-zero bit pair in S, copy that bit pair into the corresponding D bits,")

debug("else leave that D bit pair the same.")

debug("Example")

debug("Copy bit pattern ins S to D")

debug("--------------------------------------")

cogRunning := COGINIT(COGEXEC_NEW,@S0_MUXNITS,0)

debug(udec(cogRunning))

repeat                                    'keep cog 0 running

DAT

org     0

S0_MUXNITS            drvh     #P0

debug("before MUXNITS")

debug(ubin(D_MUXNITS))

debug(ubin(S_MUXNITS))

debug(" ")

MUXNITS D_MUXNITS,S_MUXNITS

debug("after MUXNITS ")

debug(ubin(D_MUXNITS))

debug(ubin(S_MUXNITS))

debug("--------------------------------------")

_Loop1                NOP

JMP     #_Loop1          'remember # imediate

D_MUXNITS     long    %10000000_00000000_00000000_00000000

S_MUXNITS     long    %00110011_00110011_00110011_01100000

### 17.49.2_Example_WRD_MUXNIBS D,{#}S_134

MUXNIBS D,{#}S

For each non-zero nibble in S, copy that nibble into the corresponding D nibble, else leave that D nibble the same.

{{17.49.2_Example_WRD_MUXNIBS D,{#}S_134}}

{{

MUXNIBS D,{#}S   (134)

For each non-zero nibble in S, copy that nibble into the corresponding D nibble, else leave that D nibble the same.

Example

"Copy bit pattern ins S to D"

}}

CON

_clkfreq = 200_000_000  ''Debug must be enabled clock must be greater than 10MHZ for Debug

P0 =0 , P1 = 1  , P2 = 2

VAR

byte cogRunning 'cog ID started is returned or -1 if not started

PUB main()|x

debug("--------------------------------------")

debug("MUXNIbS D,{#}S    (133) ")

debug("For each non-zero nibble in S, copy that nibble into the corresponding D nibble,")

debug("else leave that D bit pair the same.")

debug("Example")

debug("Copy bit pattern ins S to D")

debug("--------------------------------------")

cogRunning := COGINIT(COGEXEC_NEW,@S0_MUXNIBS,0)

debug(udec(cogRunning))

repeat                                    'keep cog 0 running

DAT

org     0

S0_MUXNIBS            drvh     #P0

debug("before MUXNIBS")

debug(ubin(D_MUXNIBS))

debug(ubin(S_MUXNIBS))

debug(" ")

MUXNITS D_MUXNIBS,S_MUXNIBS

debug("after MUXNITS ")

debug(ubin(D_MUXNIBS))

debug(ubin(S_MUXNIBS))

debug("--------------------------------------")

_Loop1                NOP

JMP     #_Loop1          'remember # imediate

D_MUXNIBS     long    %10000000_00000000_00000000_00000000

S_MUXNIBS     long    %00000011_00001011_00110000_01100000

## 17.50) MUXQ D,{#}S

| MUXQ D,{#}S | Used after SETQ. For each '1' bit in Q, copy the corresponding bit in S into D. D = (D & !Q) ¦ (S & Q). |
| --- | --- |

### 17.50.1_Example_WRD_MUXQ D,{#}S_135

MUXQ D,{#}S

Used after SETQ. For each '1' bit in Q, copy the corresponding bit in S into D. D = (D & !Q) | (S & Q).

{{17.50.1_Example_WRD_MUXQ D,{#}S_135}}

{{

MUXQ D,{#}S      (135)

Used after SETQ. For each '1' bit in Q, copy the corresponding bit in S into D. D = (D & !Q) | (S & Q).

SETQ {#}D

Set Q to D. Use before RDLONG/WRLONG/WMLONG to set block transfer.

Also used before MUXQ/COGINIT/QDIV/QFRAC/QROTATE/WAITxxx.

Example

"Copy bit pattern in S to D masked by Q"

}}

CON

_clkfreq = 200_000_000  ''Debug must be enabled clock must be greater than 10MHZ for Debug

P0 =0 , P1 = 1  , P2 = 2

VAR

byte cogRunning 'cog ID started is returned or -1 if not started

PUB main()|x

debug("--------------------------------------")

debug("SETQ {#}D ")

debug("Set Q to D. Use before RDLONG/WRLONG/WMLONG to set block transfe")

debug("Also used before MUXQ/COGINIT/QDIV/QFRAC/QROTATE/WAITxxx.")

debug(" ")

debug("MUXQ D,{#}S    (135) ")

debug("Used after SETQ.")

debug("For each '1' bit in Q, copy the corresponding bit in S into D. D = (D & !Q) | (S & Q).")

debug(" ")

debug("Example")

debug("Copy bit pattern ins S to D masked by Q")

debug("--------------------------------------")

cogRunning := COGINIT(COGEXEC_NEW,@S0_MUXNIBS,0)

debug(udec(cogRunning))

repeat                                    'keep cog 0 running

DAT

org     0

S0_MUXNIBS            drvh     #P0

debug("before SETQ/MUXQ ")

debug(ubin(D_SETQ))

debug(ubin(D_MUXQ))

debug(ubin(S_MUXQ))

debug(" ")

SETQ D_SETQ

MUXQ D_MUXQ,S_MUXQ

debug("after SETQ/MUXQ ")

debug(ubin(D_SETQ))

debug(ubin(D_MUXQ))

debug(ubin(S_MUXQ))

debug("--------------------------------------")

_Loop1                NOP

JMP     #_Loop1          'remember # imediate

D_SETQ       long    %11110000_00001111_11110000_11110000

D_MUXQ     long    %10000000_00000000_00000000_00000000

S_MUXQ     long    %00000011_00001011_00110000_01100000

## 17.51) MOVBYT D,{#}S move bytes within a register

| MOVBYTS D,{#}S | Move bytes within D, per S. D = {D.BYTE[S[7:6]], D.BYTE[S[5:4]], D.BYTE[S[3:2]], D.BYTE[S[1:0]]}. |
| --- | --- |

### 17.51_Example_WRD_MOVBYTS D,{#}S_136

MOVBYTS D,{#}S

Move bytes within D, per S. D = {D.BYTE[S[7:6]], D.BYTE[S[5:4]], D.BYTE[S[3:2]], D.BYTE[S[1:0]]}.

{{17.51.1_Example_WRD_MOVBYTS D,{#}S_136}}

{{

MOVBYTS D,{#}S   (136)

Move bytes within D, per S. D = {D.BYTE[S[7:6]], D.BYTE[S[5:4]], D.BYTE[S[3:2]], D.BYTE[S[1:0]]}.

Example

Reverse bytes in a register

}}

CON

_clkfreq = 200_000_000  ''Debug must be enabled clock must be greater than 10MHZ for Debug

P0 =0 , P1 = 1  , P2 = 2

VAR

byte cogRunning 'cog ID started is returned or -1 if not started

PUB main()|x

debug("--------------------------------------")

debug("MOVBYTS D,{#}S   (136)")

debug("Move bytes within D, per S.")

debug("D = {D.BYTE[S[7:6]], D.BYTE[S[5:4]], D.BYTE[S[3:2]], D.BYTE[S[1:0]]}.")

debug(" ")

debug("Example")

debug("Reverse bytes in a register")

debug("--------------------------------------")

cogRunning := COGINIT(COGEXEC_NEW,@S0_MOVBYT,0)

debug(udec(cogRunning))

repeat                                    'keep cog 0 running

DAT

org     0

S0_MOVBYT            drvh     #P0

debug("before MOVBYT ")

debug(ubin(D_MOVBYT))

debug(ubin(S_MOVBYT))

debug(" ")

MOVBYTS D_MOVBYT,S_MOVBYT  'S(7:6)=0 S(5:4)=1 S(3:2)=2 S(1:0)=3

debug("after MOVBYT ")

debug(ubin(D_MOVBYT))

debug(ubin(S_MOVBYT))

debug("--------------------------------------")

_Loop1         NOP

JMP     #_Loop1          'remember # imediate

D_MOVBYT   long    %10000011_10000010_10000001_10000000

S_MOVBYT   long    %00000000_00000000_00000000_00011011  'S(7:6)=0 S(5:4)=1 S(3:2)=2 S(1:0)=3

## 17.52)MUL D,{#}S Multiply D x S

| MUL D,{#}S {WZ} | D = unsigned (D[15:0] * S[15:0]). Z = (S == 0) ¦ (D == 0). |
| --- | --- |
| MULS D,{#}S {WZ} | D = signed (D[15:0] * S[15:0]). Z = (S == 0) ¦ (D == 0). |

### 17.52.1_Example_WRD_MUL D,{#}S {WZ}_137

MUL D,{#}S {WZ}

D = unsigned (D[15:0] * S[15:0]). Z = (S == 0) | (D == 0).

{{17.52.1_Example_WRD_MUL D,{#}S {WZ}_137}}

{{

MUL D,{#}S {WZ}  (137)

D = unsigned (D[15:0] * S[15:0]). Z = (S == 0) | (D == 0).

Example

Multiply two unsigned numbers

}}

CON

_clkfreq = 200_000_000  ''Debug must be enabled clock must be greater than 10MHZ for Debug

P0 =0 , P1 = 1  , P2 = 2

VAR

byte cogRunning 'cog ID started is returned or -1 if not started

PUB main()|x

debug("--------------------------------------")

debug("MUL D,{#}S {WZ}  (137)")

debug("D = unsigned (D[15:0] * S[15:0]). Z = (S == 0) | (D == 0).")

debug(" ")

debug("Example")

debug("Multiply two unsigned Numbers")

debug("--------------------------------------")

cogRunning := COGINIT(COGEXEC_NEW,@S0_MUL,0)

debug(udec(cogRunning))

repeat                                    'keep cog 0 running

DAT                      org     0

S0_MUL            drvh     #P0

debug("before MUL ")

debug(udec(D_MUL),uhex(D_MUL))

debug(udec(S_MUL))

debug(" ")

MUL D_MUL,S_MUL        'D = unsigned (D[15:0] * S[15:0])

debug("after MUL ")

debug(udec(D_MUL),uhex(D_MUL))

debug(udec(S_MUL))

debug("--------------------------------------")

_Loop1                NOP

JMP     #_Loop1          'remember # imediate

D_MUL      long   65535

S_MUL       long   4

MAX           long   $FFFFFFFF   '4294967295    = %11111111_11111111_11111111_11111111

'2147483647.5  = %1111111_11111111_11111111_11111111.1  = $7FFFFFFF.8

HalfMax    long   2147483647  '2147483647.5  = %1111111_11111111_11111111_11111111.1

Hex00FF    long   %00000000_000000000_11111111_11111111  '$00FF = 65535

### 17.52.2_Example_WRD_MULS D,{#}S {WZ}_138

MULS D,{#}S {WZ}

D = signed (D[15:0] * S[15:0]). Z = (S == 0) | (D == 0).

{{17.52.2_Example_WRD_MULS D,{#}S {WZ}_138}}

{{

MULS D,{#}S {WZ}  (138)

D = signed (D[15:0] * S[15:0]). Z = (S == 0) | (D == 0).

Example

Multiply two signed numbers

}}

CON

_clkfreq = 200_000_000  ''Debug must be enabled clock must be greater than 10MHZ for Debug

P0 =0 , P1 = 1  , P2 = 2

VAR

byte cogRunning 'cog ID started is returned or -1 if not started

PUB main()|x

debug("--------------------------------------")

debug("MULS D,{#}S {WZ}  (138)")

debug("D = signed (D[15:0] * S[15:0]). Z = (S == 0) | (D == 0)")

debug(" ")

debug("Example")

debug("Multiply two signed Numbers")

debug("--------------------------------------")

cogRunning := COGINIT(COGEXEC_NEW,@S0_MULS,0)

debug(udec(cogRunning))

repeat                                    'keep cog 0 running

DAT

org     0

S0_MULS           drvh     #P0

debug("before MULS")

debug(sdec(D_MULS))

debug(sdec(S_MULS))

debug(" ")

MULS  D_MULS,S_MULS       'D = unsigned (D[15:0] * S[15:0])

debug("after MULS")

debug(sdec(D_MULS))

debug(sdec(S_MULS))

debug("--------------------------------------")

_Loop1         NOP

JMP     #_Loop1          'remember # imediate

D_MULS     long   -12

S_MULS     long    45

## 17.53) SCA D,{#}S Multiply and shift result

| SCA D,{#}S {WZ} | Next instruction's S value = unsigned (D[15:0] * S[15:0]) >> 16. * |
| --- | --- |
| SCAS D,{#}S {WZ} | Next instruction's S value = signed (D[15:0] * S[15:0]) >> 14. In this scheme, $4000 = 1.0 and $C000 = -1.0. * |

### 17.53.1_Example_WRD_SCA D,{#}S {WZ}_139

SCA D,{#}S {WZ}

Next instruction's S value = unsigned (D[15:0] * S[15:0]) >> 16. *

Note: Need to find an example how this could be beneficially used

{{17.53.1_Example_WRD_SCA D,{#}S {WZ}_139}}

{{

SCA D,{#}S {WZ}      (139)

Next instruction's S value = unsigned (D[15:0] * S[15:0]) >> 16. *

Example

Multiply two unsigned numbers

}}

CON

_clkfreq = 200_000_000  ''Debug must be enabled clock must be greater than 10MHZ for Debug

P0 =0 , P1 = 1  , P2 = 2

VAR

byte cogRunning 'cog ID started is returned or -1 if not started

PUB main()|x

debug("--------------------------------------")

debug("SCA D,{#}S {WZ}      (139)")

debug("Next instruction's S value = unsigned (D[15:0] * S[15:0]) >> 16. *")

debug(" ")

debug("Example")

debug("Multiply two unsigned Numbers")

debug("--------------------------------------")

cogRunning := COGINIT(COGEXEC_NEW,@S0_SCA,0)

debug(udec(cogRunning))

repeat                                    'keep cog 0 running

DAT                      org     0

S0_SCA           drvh     #P0

debug("before SCA")

debug(udec(D_SCA),udec(S_SCA))

debug(ubin(D_SCA),ubin(S_SCA))

debug(udec(D_MOV),udec(S_MOV))

debug(ubin(D_MOV),ubin(S_MOV))

debug(" ")

SCA   D_SCA,S_SCA       'D = unsigned (D[15:0] * S[15:0])

MOV   D_MOV,S_MOV

debug("after SCA")

debug(udec(D_SCA),udec(S_SCA))

debug(ubin(D_SCA),ubin(S_SCA))

debug(udec(D_MOV),udec(S_MOV))

debug(ubin(D_MOV),ubin(S_MOV))

debug("--------------------------------------")

_Loop1                NOP

JMP     #_Loop1          'remember # imediate

D_SCA          long   %0000_0000_0000_0000_1000_0000_0000_0000

S_SCA           long   %10

D_MOV         long    0

S_MOV          long    0

MAX               long   $FFFFFFFF   '4294967295    = %11111111_11111111_11111111_11111111

HalfMax         long   2147483647

'2147483647.5  = %1111111_11111111_11111111_11111111.1  = $7FFFFFFF.8

Hex00FF        long   65535       '$00FF         = %00000000_000000000_11111111_11111111

Hex7FFF0000    long  '%111_1111_1111_1111_0000_0000_0000_0000_0000 = $7FFF_0000

### 17.53.2_Example_WRD_ SCAS D,{#}S {WZ}_140

SCAS D,{#}S {WZ}

Next instruction's S value = signed (D[15:0] * S[15:0]) >> 14. In this scheme, $4000 = 1.0 and $C000 = -1.0. *

Noneed to find an example how this is used
17.54)ADDPIX D,{#}S for setting pix intensity

| ADDPIX D,{#}S | Add bytes of S into bytes of D, with $FF saturation. |
| --- | --- |

### 17.54.1_Example-WRD_ADDPIX D,{#}S_141

ADDPIX D,{#}S

Add bytes of S into bytes of D, with $FF saturation.

{{17.54.1_Example_WRD_ADDPIX D,{#}S_141}}

{{

ADDPIX D,{#}S  (141)

Add bytes of S into bytes of D, with $FF saturation.

Example

add D3+S3 ,D2+S2,D1+S1,D0+S0 max value in DX = FF

}}

CON

_clkfreq = 200_000_000  ''Debug must be enabled clock must be greater than 10MHZ for Debug

P0 =0 , P1 = 1  , P2 = 2

VAR

byte cogRunning 'cog ID started is returned or -1 if not started

PUB main()|x

debug("--------------------------------------")

debug("ADDPIX D,{#}S (141)")

debug("Add bytes of S into bytes of D, with $FF saturation. ")

debug(" ")

debug("Example")

debug("add D3+S3 ,D2+S2,D1+S1,D0+S0 max value in DX = FF ")

debug("--------------------------------------")

cogRunning := COGINIT(COGEXEC_NEW,@S0_ADDPIX,0)

debug(udec(cogRunning))

repeat                                    'keep cog 0 running

DAT                      org     0

S0_ADDPIX           drvh     #P0

debug("before ADDPIX")

debug(udec(D_ADDPIX),udec(S_ADDPIX))

debug(ubin(D_ADDPIX),ubin(S_ADDPIX))

debug(" ")

ADDPIX   D_ADDPIX,S_ADDPIX

debug("after ADDPIX")

debug(udec(D_ADDPIX),udec(S_ADDPIX))

debug(ubin(D_ADDPIX),ubin(S_ADDPIX))

debug("--------------------------------------")

_Loop1                NOP

JMP     #_Loop1          'remember # imediate

D_ADDPIX          long   %10000000_10101010_00001000_11111111

S_ADDPIX          long   %00101010_10101010_00001000_11111111

## 17.55) MULPIX D,{#}S Multipy Bytes Dx*Sx (info from AJL)

| MULPIX D,{#}S | Multiply bytes of S into bytes of D, where $FF = 1.0 and $00 = 0.0. |
| --- | --- |

From the instruction description, with $FF = 1.0 and $00 = 0.0 then with D of $0080FFFF and S of $008080FF I would expect a result of something like $004080FF.

That’s:
$00 x $00 = 0.0 x 0.0 = 0.0 = $00
$80 x $80 = 0.5 x 0.5 = 0.25 = $40
$80 x $FF = 0,5 x 1.0 = 0.5 = $80
$FF x $FF = 1.0 x 1.0 = 1.0 = $FF

### 17.55.1_Example_WRD_MULPIX D,{#}S_142

MULPIX D,{#}S

Multiply bytes of S into bytes of D, where $FF = 1.0 and $00 = 0.0.

{{17.55.1_Example_WRD_MULPIX D,{#}S_142}}

{{

MULPIX D,{#}S    (142)

Multiply bytes of S into bytes of D, where $FF = 1.0 and $00 = 0.0.

D0=D0*S0 D1=D1*S1 D2=D2*S2 D3=D3*S3

Example

From the instruction description, with $FF = 1.0 and $00 = 0.0

then with D of $FFFF8000 and S of $FF808000 D-> $FF804000

That is:

$00 x $00 = 0.0 x 0.0 = 0.0 = $00

$80 x $80 = 0.5 x 0.5 = 0.25 = $40

$80 x $FF = 0,5 x 1.0 = 0.5 = $80

$FF x $FF = 1.0 x 1.0 = 1.0 = $FF

}}

CON

_clkfreq = 200_000_000  ''Debug must be enabled clock must be greater than 10MHZ for Debug

P0 =0 , P1 = 1  , P2 = 2

VAR

byte cogRunning 'cog ID started is returned or -1 if not started

PUB main()|x

debug("--------------------------------------")

debug("MULPIX D,{#}S (142)")

debug("Multiply bytes of S into bytes of D, where $FF = 1.0 and $00 = 0.0.")

debug("D0=D0*S0 D1=D1*S1 D2=D2*S2 D3=D3*S3")

debug(" ")

debug("Example")

debug("From the instruction description, with $FF = 1.0 and $00 = 0.0")

debug("then with D of $FFFF8000   and S $FF808000  of  D-> $FF804000.")

debug("--------------------------------------")

cogRunning := COGINIT(COGEXEC_NEW,@S0_MULPIX,0)

debug(udec(cogRunning))

repeat                                    'keep cog 0 running

DAT                      org     0

S0_MULPIX             drvh     #P0

debug("before MULPIX")

debug(uhex(D_MULPIX),uhex(S_MULPIX))

debug(" ")

MULPIX   D_MULPIX,S_MULPIX

debug("after MULPIX")

debug(uhex(D_MULPIX),uhex(S_MULPIX))

debug("--------------------------------------")

_Loop1                NOP

JMP     #_Loop1          'remember # imediate

D_MULPIX          long   $FFFF8000

S_MULPIX          long   $FF808000

## 17.xx.149 RD Read Pin,LUT(LookUpTable)Byte,Word,Long

| RQPIN D,{#}S {WC} | Read smart pin S[5:0] result "Z" into D, don't acknowledge smart pin ("Q" in RQPIN means "quiet"). C = modal result. |
| --- | --- |
| RDPIN D,{#}S {WC} | Read smart pin S[5:0] result "Z" into D, acknowledge smart pin. C = modal result. |
| RDLUT D,{#}S/P {WC/WZ/WCZ} | Read data from LUT address {#}S/PTRx into D. C = MSB of data. * |
| RDBYTE D,{#}S/P {WC/WZ/WCZ} | Read zero-extended byte from hub address {#}S/PTRx into D. C = MSB of byte. * |
| RDWORD D,{#}S/P {WC/WZ/WCZ} | Read zero-extended word from hub address {#}S/PTRx into D. C = MSB of word. * |
| RDLONG D,{#}S/P {WC/WZ/WCZ} | Read long from hub address {#}S/PTRx into D. C = MSB of long. * Prior SETQ/SETQ2 invokes cog/LUT block transfer. |

### 17.xx.149_Example_WRD_RQPIN_149

RQPIN D,{#}S {WC}

Read smart pin S[5:0] result "Z" into D, don't acknowledge smart pin ("Q" in RQPIN means "quiet"). C = modal result.

### 17.xx.150_Example_WRD_RDPIN_150

RDPIN D,{#}S {WC}

Read smart pin S[5:0] result "Z" into D, acknowledge smart pin. C = modal result.

### 17.xx.151_Example_WRD_RDLUT_151

RDLUT D,{#}S/P {WC/WZ/WCZ}

Read data from LUT address {#}S/PTRx into D. C = MSB of data. *

### 17.xx.152_Example_WRD_RDBYTE_152

RDBYTE D,{#}S/P {WC/WZ/WCZ}

Read zero-extended byte from hub address {#}S/PTRx into D. C = MSB of byte. *

### 17.xx.153_Example_WRD_RDWORD_153

RDWORD D,{#}S/P {WC/WZ/WCZ}

Read zero-extended word from hub address {#}S/PTRx into D. C = MSB of word. *

### 17.xx.154_Example_WRD_RDLONG_154

RDLONG D,{#}S/P {WC/WZ/WCZ}

Read long from hub address {#}S/PTRx into D. C = MSB of long. * Prior SETQ/SETQ2 invokes cog/LUT block transfer.

## 17.xx.233) REP {#}D,{#}S Repeat Instruction S Times

| REP {#}D,{#}S | Execute next D[8:0] instructions S times. If S = 0, repeat instructions infinitely. If D[8:0] = 0, nothing repeats. |
| --- | --- |

_Symbol      REP @Done,S

First Pasm Instruction to be repeated

.

.

.

Last Pasm Instruction to be repeated

_Done         PASM instruction to be run after REp

The REP instruction needs (the number of ins to repeat)-1 so the calculation is done by the compiler and the result is stored in the rep instruction generated.

Since instruction counting and adjusting is tedious, the @ syntax was  to look a bit alike the P1 syntax, consider the pair of labels:
xxx    and
xxx   _ret
which also create a instruction when you write CALL xxx and the compiler writes a JMPRET instruction for you.

So you can use rep without any label, but it is more tedious.(Note: Not sure how this is done guessing that you can manipulate the count inside D register of  REP D,S)

rstart  rep #rend-#rstart-1,S_REP

...

...

rend    ...

That it is @ again its just another character to distinguish the operation REP.

The @ ins and the friends @@ and @@@ are usually a Spin-syntax but often used in DAT sections thus also valid for assembler. So in the case of REP the @ has a complete different meaning. @ instructs the compiler to See how many instructions follow the REP instruction to the symbol pointed to with @Symbol.

REP puts a hold on interrupts, and debug is highest level IRQ in the prop2 . So debug won't respond until the REP is completed. The REP instruction is built this way to prevent unexpected branching.

Debug will possibly create a bug because a branch instruction cancels the REP for good. Branching out of a REP is legal, but you need to account for it terminating the REP.

### 17.xx.233_Example_WRD_REP {#}D,{#}S_233

REP {#}D,{#}S

Execute next D[8:0] instructions S times. If S = 0, repeat instructions infinitely. If D[8:0] = 0, nothing repeats.

{{17.xx.233.1_Example_WRD_REP {#}D,{#}S_233}}

{{

REP {#}D,{#}S

Execute next D[8:0] instructions S times. If S = 0, repeat instructions infinitely. If D[8:0] = 0, nothing repeats.

_Start         REP @Symbol,S

First Pasm Instruction to be repeated

.

.

Last PASM Instruction to be repeated

_Symbol        PASM instruction to be run after REP

@Symbolsyntax instructs the compiler to See how many instructions follow the REP

instruction to the symbol pointed to with @Symbol this value is then stored in D register.

Example

Demonstrate REP using ADD Instruction and @Symbol Compiler directive

}}

CON

_clkfreq = 200_000_000  ''Debug must be enabled clock must be greater than 10MHZ for Debug

P0 =0 , P1 = 1  , P2 = 2

VAR

Byte cogRunning 'cog ID started is returned or -1 if not started

PUB main()

debug("--------------------------------------")

debug("REP {#}D,{#}S")

debug("Execute next D[8:0] instructions S times.")

debug("If S = 0, repeat instructions infinitely. If D[8:0] = 0, nothing repeats.")

debug("Example")

debug("demonstrate REP using ADD Instruction")

debug("--------------------------------------")

cogRunning := COGINIT(COGEXEC_NEW,@S0_REP,0)

debug(udec(cogRunning))

repeat                                    'keep cog 0 running

DAT

ORG   0

S0_REP         DRVH    #P0                       'P0 on program running

MOV    AddressDone,#_Done

debug("--------------------------------------")

debug("before REP ",udec(D_REP),udec(S_REP))

debug(udec(ResultAddOne),udec(ResultAddTwo),udec(AddressDone))

StartREP       REP   @_Done,S_REP

ADD   ResultAddOne,#1

EndREP         ADD   ResultAddTwo,#2

_Done          debug("after REP  ",udec(D_REP),udec(S_Rep))

debug(udec(ResultAddOne),udec(ResultAddTwo),udec(AddressDone))

debug("--------------------------------------")

_Loop1         NOP

JMP     #_Loop1             'remember # imediate

ResultAddOne         long     0

ResultAddTwo         long     0

AddressDone           long     0

D_REP                       long     0

S_REP                        long     9

## 17.xx.314 SETQ SetQ register prior to Instruction

SET

et Q to D. Use before RDLONG/WRLONG/WMLONG to set block transfer. Also used before    MUXQ/COGINIT/QDIV/QFRAC/QROTATE/WAITxxx.

### 17.xx.314 SETQ CONSIDERATIONS

Q is a hidden special purpose register inside the cog's processor core (ALU). The Program Counter (PC) is another one of these. Q also must have a couple of associated flags to tell subsequent instructions that Q has just been refreshed. At least two flags are needed for RDLONG/WRLONG to know if they should burst read/write to cogRAM or lutRAM.  SETQ sets first flag and SETQ2 sets the second flag.

- TeluXORO32 executes - Q is set to the XORO32 result.

- RDLUT executes - Q is set to the data read from the lookup RAM.

- GETXACC executes - Q is set to the Goertzel sine accumulator value.

- CRCNIB executes - Q gets shifted left by four bits.

- COGINIT/QDIV/QFRAC/QROTATE executes without a preceding SETQ instruction - Q is set to zero.

CRC ied

I is possible tMOV			MUX		STQ/SETQ2 shields the next instruction from interruption to prevent an interrupt service routine fromA-SETQ works differently to the ALTx instructions. SETQ fills the Q register and sets a flag as future notification. ALTx instructions modify the already fetched next instruction inside the pipeline.B will have logic to detect the notification flag and change its data source for S[9:5] to Q[??:??]. Best guess is Q[4:0].

## 17.xx.390 REV D Reverse D Bits

| REV D | Reverse D bits. D = D[0:31]. |
| --- | --- |

### 17.xx.390_Example_WRD_REV D_390

REV D Reverse D Bits. D = D[0:31]

{{17.xx.390_Example_WRD_REV D_390}}

{{

REV D

Reverse D Bits. D = D[0:31]

Example

Reverse Bit Order of D_REV

}}

CON

_clkfreq = 200_000_000  ''Debug must be enabled clock must be greater than 10MHZ for Debug

P0 =0 , P1 = 1  , P2 = 2

VAR

Byte cogRunning 'cog ID started is returned or -1 if not started

PUB main()

debug("--------------------------------------")

debug("REV D  ")

debug("Reverse D Bits. D = D[0:31]")

debug(" ")

debug("Example")

debug(" ")

debug("--------------------------------------")

cogRunning := COGINIT(COGEXEC_NEW,@S0_REV,0)

debug(udec(cogRunning))

repeat                                    'keep cog 0 running

DAT

ORG   0                           'Returns CRC8 of n bytes at p_src

S0_REV                DRVH    #P0                       'P0 on program running

debug("--------------------------------------")

debug("before D_REV ",ubin(D_REV))

REV D_REV

debug("after D_REV  ",ubin(D_REV))

debug("--------------------------------------")

_Loop1                NOP

JMP     #_Loop1            'remember # imediate

D_REV                 long   %11111111_10011000_00001111_10000001

## 17.xx.397 MODCZ Modify C or Z Flag

| MODCZ c,z {WC/WZ/WCZ} | Math and Logic | EEEE 1101011 CZ1 0cccczzzz 001101111 |
| --- | --- | --- |
| MODC c {WC} | Math and Logic | EEEE 1101011 C01 0cccc0000 001101111 |
| MODZ z {WZ} | Math and Logic | EEEE 1101011 0Z1 00000zzzz 001101111 |

MODCZ   c,z   {WC/WZ/WCZ}   Modify C and Z according to cccc and zzzz. C = cccc[{C,Z}], Z = zzzz[{C,Z}].

MODC    c       {WC}                     Modify C according to cccc. C = cccc[{C,Z}].

MODZ    z           {WZ}                 Modify Z according to zzzz. Z = zzzz[{C,Z}].

Operand MODCZ/MODC/MODZ

cccc and zzzz are the 4 bits that the constants define. They will be put into the D and S field.

MODCZ/MODC/MODZ is a "D-only" instruction, where D[7:4] = cccc and D[3:0] = zzzz.
S field is fixed and part of the opcode. D_xxxxxxxx_xxxxxxxx_xxxxxxxx_CCCCC_ZZZZZ  
Note: x indicates uknown

The block diagram is always the same for MODCZ/MODC/MODZ. You can replace D[3..0] by zzzz and D[7..4] by cccc, to match the bit names in the instruction encoding. You can see this LUTs as a 1bit wide ROM with 2 address inputs = 4 bits total. This address inputs are connected to the current state of C and Z. Depending on the state of C and Z one of 4 bits in the ROM is read and defines the new state of the C or Z flag, if the WC and/or WZ effect is set.

The instruction is very universal, but therefore also a bit complex. Like in the LUTs of an FPGA, you define in a truth table the resulting bit for any combination of C and Z.
Say you want to set Z to the state of C, then the truth table looks like that:

C    Z  | zzzz

--------|-----

0    0 | 0

0    1 | 0

1    0 | 1

1    1 | 1

'-> %1100 = zzzz

when C is 1 the result is 1, if C is 0 the result is 0
so you can write the MODx instruction like that: MODZ %1100 WZ 'sets Z to C
but it is easier to understand with the constant: MODZ _C WZ 'the same with the named constant

MODCZ lets you affect the C and the Z flag in one instruction, you can for example swap the two flags:

MODCZ _Z, _C  WCZ       'swap c and Z

|   |

v   v

C   Z

From the instruction encoding,  that all 3 instructions are the same. Unused bits are just set to zero for MODC and MODZ. You can also affect only C or Z with MODCZ, but then the assembly syntax requires a dummy argument for the not used flag.

M.1) MODCZ constants      From the instructions_v32.txt:

_CLR                    =       %0000

_NC_AND_NZ              =       %0001

_NZ_AND_NC              =       %0001

_GT                     =       %0001

_NC_AND_Z               =       %0010

_Z_AND_NC               =       %0010

_NC                     =       %0011

_GE                     =       %0011

_C_AND_NZ               =       %0100

_NZ_AND_C               =       %0100

_NZ                     =       %0101

_NE                     =       %0101

_C_NE_Z                 =       %0110

_Z_NE_C                 =       %0110

_NC_OR_NZ               =       %0111

_NZ_OR_NC               =       %0111

_C_AND_Z                =       %1000

_Z_AND_C                =       %1000

_C_EQ_Z                 =       %1001

_Z_EQ_C                 =       %1001

_Z                      =       %1010

_E                      =       %1010

_NC_OR_Z                =       %1011

_Z_OR_NC                =       %1011

_C                      =       %1100

_LT                     =       %1100

_C_OR_NZ                =       %1101

_NZ_OR_C                =       %1101

_C_OR_Z                 =       %1110

_Z_OR_C                 =       %1110

_LE                     =       %1110

_SET                    =       %1111

Examples:

MODCZ   _CLR, _Z_OR_C   WCZ     'C = 0, Z |= C

MODCZ   _NZ,0           WC      'C = !Z

MODCZ   0,_SET          WZ      'Z = 1

MODC    _NZ_AND_C       WC      'C = !Z & C

MODZ    _Z_NE_C         WZ      'Z = Z ^ C

### 17.xx.397_Example_WRD_MODCZ_Operand

{{M.1_Example_WRD_MODCZ_Operand}}

{{

cccc and zzzz are the 4 bits that the constants define. They will be put into the D and S field.

MODCZ/MODC/MODZ is a "D-only" instruction, where D[7:4] = cccc and D[3:0] = zzzz.

S field is fixed and part of the opcode. D_xxxxxxxx_xxxxxxxx_xxxxxxxx_CCCCC_ZZZZZ

Note: x indicates uknown MODCZ useful for setting FLag Bits CZ

_CLR                    =       %0000

_NC_AND_NZ              =       %0001

_NZ_AND_NC              =       %0001

_GT                     =       %0001

_NC_AND_Z               =       %0010

_Z_AND_NC               =       %0010

_NC                     =       %0011

_GE                     =       %0011

_C_AND_NZ               =       %0100

_NZ_AND_C               =       %0100

_NZ                     =       %0101

_NE                     =       %0101

_C_NE_Z                 =       %0110

_Z_NE_C                 =       %0110

_NC_OR_NZ               =       %0111

_NZ_OR_NC               =       %0111

_C_AND_Z                =       %1000

_Z_AND_C                =       %1000

_C_EQ_Z                 =       %1001

_Z_EQ_C                 =       %1001

_Z                      =       %1010

_E                      =       %1010

_NC_OR_Z                =       %1011

_Z_OR_NC                =       %1011

_C                      =       %1100

_LT                     =       %1100

_C_OR_NZ                =       %1101

_NZ_OR_C                =       %1101

_C_OR_Z                 =       %1110

_Z_OR_C                 =       %1110

_LE                     =       %1110

_SET                    =       %1111

Examples:

MODCZ   _CLR, _Z_OR_C   WCZ     'C = 0, Z |= C

MODCZ   _NZ,0           WC      'C = !Z

MODCZ   0,_SET          WZ      'Z = 1

MODC    _NZ_AND_C       WC      'C = !Z & C

MODZ    _Z_NE_C         WZ      'Z = Z ^ C

}}

CON

_clkfreq = 200_000_000  ''Debug must be enabled clock must be greater than 10MHZ for Debug

VAR

Byte cogRunning 'cog ID started is returned or -1 if not started

PUB main()

cogRunning := COGINIT(COGEXEC_NEW,@S0_MODCZ,0)

debug(udec(cogRunning))

repeat

DAT

ORG   0

S0_MODCZ              NOP

MOV     Pr0,Max     'load cog 1 Pro  FFFF_FFFF

debug("-----------------------")

debug("before  ADD overflow",udec(Pr0),uhex(Carry),uhex(Zero))

ADD     Pr0,#1       wcz    ' after insctruction Pr0 = 0 C = 1 Z = 1

IF_C   MOV     Carry,#1   'is true  C = 1

IF_NC  MOV     Carry,#0   'is false C = 0

IF_Z   MOV     Zero,#1    'is true  Z = 1

IF_NZ  MOV     Zero,#0    'is false Z = 0

debug("after ADD overflow   ",udec(Pr0),uhex(Carry),uhex(Zero))

{Pr0 = 0 C =1  Z=1}

debug("-----------------------")

debug("before MODC _CLR WC ",udec(Pr0),uhex(Carry),uhex(Zero))

MODC    _CLR   WC

IF_C   MOV     Carry,#1   'is true  C = 1

IF_NC  MOV     Carry,#0   'is false C = 0

IF_Z   MOV     Zero,#1    'is true  Z = 1

IF_NZ  MOV     Zero,#0    'is false Z = 0

debug("after MODC _CLR WC ",udec(Pr0),uhex(Carry),uhex(Zero))

debug("-----------------------")

debug("before MODC _SET WC ",udec(Pr0),uhex(Carry),uhex(Zero))

MODC    _SET   WC

IF_C   MOV     Carry,#1   'is true  C = 1

IF_NC  MOV     Carry,#0   'is false C = 0

IF_Z   MOV     Zero,#1    'is true  Z = 1

IF_NZ  MOV     Zero,#0    'is false Z = 0

debug("after MODC _SET WC ",udec(Pr0),uhex(Carry),uhex(Zero))

debug("-----------------------")

debug("before MODZ _CLR WZ ",udec(Pr0),uhex(Carry),uhex(Zero))

MODZ _CLR  WZ

IF_C   MOV     Carry,#1   'is true  C = 1

IF_NC  MOV     Carry,#0   'is false C = 0

IF_Z   MOV     Zero,#1    'is true  Z = 1

IF_NZ  MOV     Zero,#0    'is false Z = 0

debug("after MODZ _CLR WZ ",udec(Pr0),uhex(Carry),uhex(Zero))

debug("-----------------------")

debug("before MODZ _SET WZ ",udec(Pr0),uhex(Carry),uhex(Zero))

MODZ _SET  WZ

IF_C   MOV     Carry,#1   'is true  C = 1

IF_NC  MOV     Carry,#0   'is false C = 0

IF_Z   MOV     Zero,#1    'is true  Z = 1

IF_NZ  MOV     Zero,#0    'is false Z = 0

debug("after MODZ _SET WZ ",udec(Pr0),uhex(Carry),uhex(Zero))

debug("-----------------------")

MODZ _CLR WZ    'Z = 0

MODC _CLR WC    'C = 0

IF_C   MOV     Carry,#1   'is true  C = 1

IF_NC  MOV     Carry,#0   'is false C = 0

IF_Z   MOV     Zero,#1    'is true  Z = 1

IF_NZ  MOV     Zero,#0    'is false Z = 0

debug("before MODC _NC_AND_NZ WC ",udec(Pr0),uhex(Carry),uhex(Zero))

MODC _NC_AND_NZ  WC

IF_C   MOV     Carry,#1   'is true  C = 1

IF_NC  MOV     Carry,#0   'is false C = 0

IF_Z   MOV     Zero,#1    'is true  Z = 1

IF_NZ  MOV     Zero,#0    'is fals  Z = 0

debug("after MODC _NC_AND_NZ WC ",udec(Pr0),uhex(Carry),uhex(Zero))

debug("-----------------------")

MODZ _SET WZ   'Z = 1

MODC _SET WC   'C = 1

IF_C   MOV     Carry,#1   'is true  C = 1

IF_NC  MOV     Carry,#0   'is false C = 0

IF_Z   MOV     Zero,#1    'is true  Z = 1

IF_NZ  MOV     Zero,#0    'is fals  Z = 0

debug("before MODC _NC_ADND_NZ WC ",udec(Pr0),uhex(Carry),uhex(Zero))

MODC _NC_AND_NZ  WC

IF_C   MOV     Carry,#1   'is true  C = 1

IF_NC  MOV     Carry,#0   'is false C = 0

IF_Z   MOV     Zero,#1    'is true  Z = 1

IF_NZ  MOV     Zero,#0    'is fals  Z = 0

debug("after MODC _NC_AND_NZ WC ",udec(Pr0),uhex(Carry),uhex(Zero))

debug("-----------------------")

MODZ _CLR WZ    'Z = 0

MODC _CLR WC    'C = 1

IF_C   MOV     Carry,#1   'is true  C = 1

IF_NC  MOV     Carry,#0   'is false C = 0

IF_Z   MOV     Zero,#1    'is true  Z = 1

IF_NZ  MOV     Zero,#0    'is fals  Z = 0

debug("before MODZ _NC_ADND_NZ WZ ",udec(Pr0),uhex(Carry),uhex(Zero))

MODZ _NC_AND_NZ WZ

IF_C   MOV     Carry,#1  'is true  C = 1

IF_NC  MOV     Carry,#0  'is false C = 0

IF_Z   MOV     Zero,#1   'is true  Z = 1

IF_NZ  MOV     Zero,#0   'is fals  Z = 0

debug("after MODZ _NC_AND_NZ WZ ",udec(Pr0),uhex(Carry),uhex(Zero))

debug("-----------------------")

MODZ _CLR WZ    'Z = 0

MODC _CLR WC    'C = 0

IF_C   MOV     Carry,#1   'is true  C = 1

IF_NC  MOV     Carry,#0   'is false C = 0

IF_Z   MOV     Zero,#1    'is true  Z = 1

IF_NZ  MOV     Zero,#0    'is fals  Z = 0

debug("before MODCZ _SET,SET WCZ ",udec(Pr0),uhex(Carry),uhex(Zero))

MODCZ  _SET, _SET  WCZ  'C = 1 Z = 1

IF_C   MOV     Carry,#1    'is true  C = 1

IF_NC  MOV     Carry,#0    'is false C = 0

IF_Z   MOV     Zero,#1     'is true  Z = 1

IF_NZ  MOV     Zero,#0     'is fals  Z = 0

debug("after MODCZ _SET,SET WCZ  ",udec(Pr0),uhex(Carry),uhex(Zero))

debug("-----------------------")

debug("before MODCZ _CLR,_CLR WCZ ",udec(Pr0),uhex(Carry),uhex(Zero))

MODCZ  _CLR, _CLR  WCZ   'C = 1 Z = 1

IF_C   MOV     Carry,#1     'is true  C = 1

IF_NC  MOV     Carry,#0     'is false C = 0

IF_Z   MOV     Zero,#1      'is true  Z = 1

IF_NZ  MOV     Zero,#0      'is fals  Z = 0

debug("after MODCZ _CLR,CLR WCZ  ",udec(Pr0),uhex(Carry),uhex(Zero))

debug("-----------------------")

_Loop                 NOP

JMP     #_Loop

Max                 long      $FFFF_FFFF

Min                 long      $0

Carry               long      $0

Zero                long      $0

UnSignedMax         long      $FFFF_FFFF

UnSignedMIN         long      0

SignedMax           long      %01111111_11111111_11111111_11111111

SignedMin           long      %10000000_00000000_00000000_00000000

UnSignedBinMax      long      %11111111_11111111_11111111_11111111

UnSignedBinMin      long      %00000000_00000000_00000000_00000000

UnSignedHexMax      long      $FFFF_FFFF

UnSignedHexMin      long      $0000_0000

UnSignedDecMax      long      4_294_967_295

UnSignedDecMin      long      0

SignedBinMax        long      %01111111_11111111_11111111_11111111

SignedBinMin        long      %10000000_00000000_00000000_00000000

SignedHexMax        long      $7FF_FFFF

SignedHexMin        long     -$800_0000

SignedDecMax        long      2_147_483_647

SignedDecMin        long     -2_147_483_648

# Appendix “A” Programming Style Convention

## A.1) Default Program Template

The default program template is stored in the “Propeller Tool” under Edit→ preferences. One file

Template preference can be used for each of the two propelle types. In particular the propeller ii template used for these examples is store in:

c:\users\wrd\documents\Propeller Tool\templates\WRD_P2_Template_Debug.spin2”

{{WRD_P2_Template_Debug}}

''Debug must be enabled in propeller tool

{===================================================================}

CON {Processor Timing}

_clkfreq  = 200_000_000                                      'processor clock speed

CON

{Processor I/O Hardware}

Rx1     = 63  { I }                                          ' programming / debug

Tx1     = 62  { O }

FsCs    = 61  { O }                                          ' flash storage

FsClk   = 60  { O }

FsMosi  = 59  { O }

FsMiso  = 58  { I }

SdClk   = 61  { O }                                          ' usd card storage

SdCs    = 60  { O }

SdMosi  = 59  { O }

SdMiso  = 58  { I }

Sda1    = 57  { IO }                                         ' i2c (optional)

Scl1    = 56  { IO }

#0,  Cog0,Cog1,Cog2,Cog3,Cog4,Cog5,Cog6,Cog7  'CogNum

#0,  P0, P1, P2, P3, P4, P5, P6, P7, P8 ,P9, P10,P11,P12,P13,P14,P15  'PinNum

#16, P16,P17,P18,P19,P20,P21,P22,P23,P24,P25,P26,P27,P28,P29,P30,P31  'PinNum

#32, P32,P33,P34,P35,P36,P37,P38,P39,P40,P41,P42,P43,P44,P45,P46,P47  'PinNum

#48, P48,P49,P50,P51,P52,P53,P54,P55                                  'PinNum

VAR {Processor Variables}

'CogNumStatus ≔ COGCHK(CogNum)

'  Running Check if cog CogNum is running, returns -1 if running or 0 if not

byte Cog0Status,Cog1Status,Cog2Status,Cog3Status,Cog4Status,Cog5Status,Cog6Status,Cog7Status

{Program Test Variables}

long TestVar01,TestVar02,TestVar03

OBJ  {Processor Objects}

'mon    "VideoDriver"

'term   "KeyboardDriver"

'mouse  "MouseDriver"

'mem    "MemoryStorageDriver"

DAT  {Processor Data}

strSalute       byte    "Hello World",0            'debug notice application running

PUB bootEntry():result01,result02 | x,y      'method run after boot spin interperter loads

debug(zstr(@strSalute))                             'debug used sen string

mainApp()                                                     'turn control over mainApp() method

{==================================================================================}

CON  {Application Constants}                       'user defined constants

VAR  {Application Variables}                       'user defined variables

OBJ  {Application Objects}                            'user defined objects

DAT  {Application Data}                              'user defined data

PUB   mainApp()                                          'user Spin Program

repeat                                                      'keep cog 0 running

DAT  {Application PASM}                            'user PASM Program

# Appendix “B” DEBUG INTERRUPT

In addition to the three visible interrupts, there is a fourth "hidden" interrupt that has priority over all the others. It is the debug interrupt, and it is inaccessible to normal cog programs.

Debug interrupts are enabled on a per-cog basis via HUBSET. Each debug-enabled cog will generate a debug interrupt on (re)start from each COGINIT exercised upon it. Within that initial debug ISR and within each subsequent debug ISR, multiple trigger conditions may be set for the next debug interrupt. If no trigger conditions are set before the debug ISR ends, no more debug interrupts will occur until the cog is restarted from another COGINIT.

The last 16KB of hub RAM, which is also mapped to $FC000..$FFFFF, gets partially used as a buffer area for saving and restoring cog registers during debug ISR's. The initial debug ISR routines are also stored in this upper RAM. Once initialized with debug ISR code, this upper hub RAM can be write-protected, in which case it is mapped only to $FC000..$FFFFF and it is only writable from within debug ISR's.

Each cog has an execute-only ROM in cog registers $1F8..$1FF which contains special debug-ISR-entry and -exit routines. These tiny routines perform seamless register-load and register-restore operations for your debugger program, which must be realized entirely within debug ISR's.

Execute-only ROM in cog registers $1F8..$1FF (%cccc = !CogNumber)

Debug ISR Entry - IJMP0 is initialized to $1F8 on cog start

$1F8 - SETQ #$0F 'save registers $000..$00F $1F9 - WRLONG 0,* '* = %1111_1111_1ccc_c000_0000 $1FA - SETQ #$0F 'load program into $000..$00F $1FB - RDLONG 0,* '* = %1111_1111_1ccc_c100_0000 $1FC - JMP #0 'jump to loaded program

Debug ISR Exit - Jump here to exit your debug ISR

$1FD - SETQ #$0F 'restore registers $000..$00F $1FE - RDLONG 0,* '* = %1111_1111_1ccc_c000_0000 $1FF - RETI0 'CALLD IRET0,IRET0 WCZ

During a debug ISR, INA and INB, normally read-only input-pin registers, become readable/writable RAM registers named IJMP0 and IRET0, and are used by the debug interrupt as jump and return addresses. On COGINIT, IJMP0 is initialized to $1F8 which is the debug-ISR-entry routine's address.

When a debug interrupt occurs with IJMP0 pointing to $1F8, the following sequence happens:

Cog registers $000 to $00F are saved to hub RAM starting at ($FF800 + !CogNumber << 7), or %1111_1111_1ccc_c000_0000, where %cccc = !CogNumber.

Cog registers $000 to $00F are loaded from hub RAM starting at ($FF840 + !CogNumber << 7), or %1111_1111_1ccc_c100_0000, where %cccc = !CogNumber.

A "JMP #$000" executes to run the 16-instruction debugger program that was just loaded into registers $000 to $00F.

Your 16-instruction debugger program will likely want to determine if this debug interrupt was due to a COGINIT, in which case the debugger will probably want to note that a new program is now running in this cog. Depending on what the debugger must do next, it is likely that it will need to save more registers to the upper hub RAM and then load in more code from the upper hub RAM to facilitate more complex operations than the initial 16-instruction ISR can achieve. The ISR may then need to perform some communication between itself and a host system which may be serving as the debugger's user interface. It may be necessary to employ a LOCK to time-share P2-to-host communication channels among cogs, likely on P63 (serial Rx) and P62 (serial Tx). This scenario is somewhat hypothetical, but illustrates the design intent behind the debug interrupt mechanism.

When your debug ISR is complete, you can do a 'JMP #$1FD' to execute the debug-ISR-exit routine which does the following:

Original cog registers $000 to $00F are restored from hub RAM starting at ($FF800 + !CogNumber << 7), or %1111_1111_1ccc_c000_0000, where %cccc = !CogNumber.

A "RETI0" executes to return to the interrupted cog program.

Here is a table of the hub RAM locations used by each cog for register save/restore and ISR images during the debug interrupt when the register ROM routines are used for ISR entry and exit:

| Cog | Save/Restore in Hub RAM for Registers $000..$00F | ISR image in Hub RAM for Registers $000..$00F |
| --- | --- | --- |
| 7 | $FFC00..$FFC3F | $FFC40..$FFC7F |
| 6 | $FFC80..$FFCBF | $FFCC0..$FFCFF |
| 5 | $FFD00..$FFD3F | $FFD40..$FFD7F |
| 4 | $FFD80..$FFDBF | $FFDC0..$FFDFF |
| 3 | $FFE00..$FFE3F | $FFE40..$FFE7F |
| 2 | $FFE80..$FFEBF | $FFEC0..$FFEFF |
| 1 | $FFF00..$FFF3F | $FFF40..$FFF7F |
| 0 | $FFF80..$FFFBF | $FFFC0..$FFFFF |

Though the first debug interrupt upon cog (re)start will always use the debug-ISR-entry routine at $1F8, you may redirect IJMP0 during any debug ISR to point elsewhere for use by subsequent debug interrupts. This would mean that you would lose the initial register-saving function provided by the small ROM starting at $1F8, so you would have to use some cog registers for debugger-state storage that don't interfere with the cog program that is being debugged. If no register saving/restoring or host communications are required, your debug ISR may execute very quickly.

What terminates a debug interrupt is not only RETI0 (CALLD INB,INB WCZ), but any D-register variant (CALLD anyreg,INB WCZ). For example RESI0 (CALLD INA,INB WCZ) may be used to resume next time from where this debug ISR left off, but this would imply that you are not using the debug-ISR-entry and -exit routines in the cog-register ROM and have, instead, permanently located debugger code into some cog registers, so that your debugger program is already present at the start of the debug interrupt.

This debug interrupt scheme was designed to operate stealthily, without any cooperation from the cog program being debugged. All control has been placed within the debug ISR. This isolation from normal programming is intended to prevent, or at least discourage, programmers from making any aspect of the debug interrupt system part of their application, thereby rendering the debug interrupt compromised as a standard debugging mechanism. Also, by executing the ISR strictly in cog register space, this scheme does not interfere with the hub FIFO state, which would be impossible to reconstruct if disturbed by hub execution within the debug ISR.

Below are the instructions which are used in the debugging mechanism:

BRK D/#

During normal program execution, the BRK instruction is used to generate a debug interrupt with an 8-bit code which can be read within the debug ISR. The BRK instruction interrupt must be enabled from within a prior debug ISR for this to work. Regardless of the execution condition, the BRK instruction will trigger a debug interrupt, if enabled. The execution condition only gates the writing of the 8-bit code:

D/# = %BBBBBBBB: 8-bit BRK code

During a debug ISR, the BRK instruction operates differently and is used to establish the next debug interrupt condition(s). It is also used to select INA/INB, instead of the IJMP0/IRET0 registers exposed during the ISR, so that the pins' inputs states may be read:

D/# = %aaaaaaaaaaaaaaaaeeee_LKJIHGFEDCBA

%aaaaaaaaaaaaaaaaeeee: 20-bit breakpoint address or 4-bit event code (%eeee)

%L: 1 = map INA/INB normally, 0 = map IJMP0/IRET0 at INA/INB (default during ISR) *

%K: 1 = enable interrupt on breakpoint address match

%J: 1 = enable interrupt on event %eeee

%I: 1 = enable interrupt on asynchronous breakpoint (via COGBRK on another cog)

%H: 1 = enable interrupt on INT3 ISR entry

%G: 1 = enable interrupt on INT2 ISR entry

%F: 1 = enable interrupt on INT1 ISR entry

%E: 1 = enable interrupt on BRK instruction

%D: 1 = enable interrupt on INT3 ISR code (single step)

%C: 1 = enable interrupt on INT2 ISR code (single step)

%B: 1 = enable interrupt on INT1 ISR code (single step)

%A: 1 = enable interrupt on non-ISR code  (single step)

* If set to 1 by the debug ISR, %L must be reset to 0 before exiting the debug ISR, so

that the RETI0 instruction is able to see IJMP0 and IRET0.

On debug ISR entry, bits A to L, are cleared to '0'. If a subsequent debug interrupt is desired, a BRK instruction must be executed before exiting the debug ISR, in order to establish the next breakpoint condition(s).

COGBRK D/#

The COGBRK instruction can trigger an asynchronous breakpoint in another cog. For this to work, the cog executing the COGBRK instruction must be in its own debug ISR and the other cog must have its asynchronous breakpoint interrupt enabled:

D/# = %CCCC: the cog in which to trigger an asynchronous breakpoint

GETBRK D WCZ

During normal program execution, GETBRK with WCZ returns various data about the cog's internal status:

C = 1 if STALLI mode or 0 if ALLOWI mode (established by STALLI/ALLOWI)

Z = 1 if cog started in hubexec or 0 if cog started in cogexec

D[31:23] = 0

D[22] = 1 if colorspace converter is active

D[21] = 1 if streamer is active

D[20] = 1 if WRFAST mode or 0 if RDFAST mode

D[19:16] = INT3 selector, established by SETINT3

D[15:12] = INT2 selector, established by SETINT2

D[11:08] = INT1 selector, established by SETINT1

D[07:06] = INT3 state: %0x = idle, %10 = interrupt pending, %11 = ISR executing

D[05:04] = INT2 state: %0x = idle, %10 = interrupt pending, %11 = ISR executing

D[03:02] = INT1 state: %0x = idle, %10 = interrupt pending, %11 = ISR executing

D[01] = 1 if STALLI mode or 0 if ALLOWI mode (established by STALLI/ALLOWI)

D[00] = 1 if cog started in hubexec or 0 if cog started in cogexec

During a debug ISR, GETBRK with WCZ returns additional data that is useful to a debugger:

C = 1 if debug interrupt was from a COGINIT, indicating that the cog was (re)started

D[31:24] = 8-bit break code from the last 'BRK #/D' during normal execution

D[23] = 1 if debug interrupt was from a COGINIT, indicating that the cog was (re)started

GETBRK D WC

GETBRK with WC always returns the following:

C = LSB of SKIP/SKIPF/EXECF/XBYTE pattern

D[31:28] = 4-bit CALL depth since SKIP/SKIPF/EXECF/XBYTE (skipping suspended if not %0000)

D[27] = 1 if SKIP mode or 0 if SKIPF/EXECF/XBYTE mode

D[26] = 1 if LUT sharing enabled (established by SETLUTS)

D[25] = 1 if top of stack = $001FF, indicating XBYTE will execute on next _RET_/RET

D[24:16] = 9-bit XBYTE mode, established by '_RET_ SETQ/SETQ2' when top of stack = $001FF

D[15:00] = 16 event-trap flags

GETBRK D WZ

GETBRK with WZ always returns the following:

Z = 1 if no SKIP/SKIPF/EXECF/XBYTE pattern queued (D = 0) or 1 if pattern queued (D <> 0)

D = 32-bit SKIP/SKIPF/EXECF/XBYTE pattern, used LSB-first to skip instructions in main code

# Appendix “C” EVENTS

Cogs monitor and track 16 different background events:

- An interrupt occurred

- CT passed CT1 (CT is the 32-bit free-running global counter)

- CT passed CT2

- CT passed CT3

- Selectable event 1 occurred

- Selectable event 2 occurred

- Selectable event 3 occurred

- Selectable event 4 occurred

- A pattern match or mismatch occurred on either INA or INB

- Hub FIFO block-wrap occurred - a new start address and block count were loaded

- Streamer command buffer is empty - it's ready to accept a new command

- Streamer finished - it ran out of commands, now idle

- Streamer NCO rollover occurred

- Streamer read lookup RAM location $1FF

- Attention was requested by another cog or other cogs

- GETQX/GETQY executed without any CORDIC results available

## C.1) Polled Events

Events are tracked and can be polled, waited for, and used as interrupt sources.

Before explaining the details, consider the event-related instructions.

First are the POLLxxx instructions which simultaneously return their event-occurred flag into C and clear their event-occurred flag (unless it's being set again by the event sensor):

Interrupt source (0=off):

POLLINT	Poll the interrupt-occurred event flag			-

POLLCT1	Poll the CT-passed-CT1 event flag				1

POLLCT2	Poll the CT-passed-CT2 event flag				2

POLLCT3	Poll the CT-passed-CT3 event flag				3

POLLSE1	Poll the selectable-event-1 event flag			4

POLLSE2	Poll the selectable-event-2 event flag			5

POLLSE3	Poll the selectable-event-3 event flag			6

POLLSE4	Poll the selectable-event-4 event flag			7

POLLPAT	Poll the pin-pattern-detected event flag			8

POLLFBW	Poll the hub-FIFO-interface-block-wrap event flag		9

POLLXMT	Poll the streamer-empty event flag				10

POLLXFI	Poll the streamer-finished event flag				11

POLLXRO	Poll the streamer-NCO-rollover event flag			12

POLLXRL	Poll the streamer-lookup-RAM-$1FF-read event flag		13

POLLATN	poll the attention-requested event flag			14

POLLQMT	Poll the CORDIC-read-but-no-results event flag		15

## C.2) Wait Instructions

Next are the WAITxxx instructions, which will wait for their event-occurred flag to be set (in case it's not, already) and then clear their event-occurred flag (unless it's being set again by the event sensor), before resuming.

By doing a SETQ right before one of these instructions, you can supply a future CT target value which will be used to end the wait prematurely, in case the event-occurred flag never went high before the CT target was reached. When using SETQ with 'WAITxxx WC', C will be set if the timeout occurred before the event; otherwise, C will be cleared.

WAITINT	Wait for an interrupt to occur, stalls the cog to save power

WAITCT1	Wait for the CT-passed-CT1 event flag

WAITCT2	Wait for the CT-passed-CT2 event flag

WAITCT3	Wait for the CT-passed-CT3 event flag

WAITSE1	Wait for the selectable-event-1 event flag

WAITSE2	Wait for the selectable-event-2 event flag

WAITSE3	Wait for the selectable-event-3 event flag

WAITSE4	Wait for the selectable-event-4 event flag

WAITPAT	Wait for the pin-pattern-detected event flag

WAITFBW	Wait for the hub-FIFO-interface-block-wrap event flag

WAITXMT	Wait for the streamer-empty event flag

WAITXFI	Wait for the streamer-finished event flag

WAITXRO	Wait for the streamer-NCO-rollover event flag

WAITXRL	Wait for the streamer-lookup-RAM-$1FF-read event flag

WAITATN	Wait for the attention-requested event flag

There's no 'WAITQMT' because the event could not happen while waiting.

## C.3) Interupt Jump Instructions

Last are the 'Jxxx/JNxxx S/#' instructions, which each jump to S/# if their event-occurred flag is set (Jxxx) or clear (JNxxx). Whether or not a branch occurs, the event-occurred flag will be cleared, unless it's being set again by the event sensor.

JINT/JNINT	Jump to S/# if the interrupt-occurred event flag is set/clear

JCT1/JNCT1	Jump to S/# if the CT-passed-CT1 event flag is set/clear

JCT2/JNCT2	Jump to S/# if the CT-passed-CT2 event flag is set/clear

JCT3/JNCT3 	Jump to S/# if the CT-passed-CT3 event flag is set/clear

JSE1/JNSE1	Jump to S/# if the selectable-event-1 event flag is set/clear

JSE2/JNSE2	Jump to S/# if the selectable-event-2 event flag is set/clear

JSE3/JNSE3	Jump to S/# if the selectable-event-3 event flag is set/clear

JSE4/JNSE4	Jump to S/# if the selectable-event-4 event flag is set/clear

JPAT/JNPAT	Jump to S/# if the pin-pattern-detected event flag is set/clear

JFBW/JNFBW	Jump to S/# if the hub-FIFO-interface-block-wrap event flag is set/clear

JXMT/JNXMT	Jump to S/# if the streamer-empty event flag is set/clear

JXFI/JNXFI	Jump to S/# if the streamer-finished event flag is set/clear

JXRO/JNXRO	Jump to S/# if the streamer-NCO-rollover event flag is set/clear

JXRL/JNXRL	Jump to S/# if the streamer-lookup-RAM-$1FF-read event flag is set/clear

JATN/JNATN	Jump to S/# if the attention-requested event flag is set/clear

JQMT/JNQMT	Jump to S/# if the CORDIC-read-but-no-results event flag is set/clear

Here are detailed descriptions of each event flag. Understand that the 'set' events can also be used as interrupt sources (except in the case of the first flag which is set when an interrupt occurs):

## C.4) Details on Polled/Wait/Interupt Instructions

POLLINT/WAITINT event flag

Cleared on cog start.

Set whenever interrupt 1, 2, or 3 occurs (debug interrupts are ignored).

Also cleared on POLLINT/WAITINT/JINT/JNINT.

POLLCT1/WAITCT1 event flag

Cleared on ADDCT1.

Set whenever CT passes the result of the ADDCT1 (MSB of CT minus CT1 is 0).

Also cleared on POLLCT1/WAITCT1/JCT1/JNCT1.

POLLCT2/WAITCT2 event flag

Cleared on ADDCT2.

Set whenever CT passes the result of the ADDCT2 (MSB of CT minus CT2 is 0).

Also cleared on POLLCT2/WAITCT2/JCT2/JNCT2.

POLLCT3/WAITCT3 event flag

Cleared on ADDCT3.

Set whenever CT passes the result of the ADDCT3 (MSB of CT minus CT3 is 0).

Also cleared on POLLCT3/WAITCT3/JCT3/JNCT3.

POLLPAT/WAITPAT event flag

Cleared on SETPAT

Set whenever (INA & D) != S after 'SETPAT D/#,S/#' with C=0 and Z=0.

Set whenever (INA & D) == S after 'SETPAT D/#,S/#' with C=0 and Z=1.

Set whenever (INB & D) != S after 'SETPAT D/#,S/#' with C=1 and Z=0.

Set whenever (INB & D) == S after 'SETPAT D/#,S/#' with C=1 and Z=1.

Also cleared on POLLPAT/WAITPAT/JPAT/JNPAT.

POLLFBW/WAITFBW event flag

Cleared on RDFAST/WRFAST/FBLOCK.

Set whenever the hub RAM FIFO interface exhausts its block count and reloads its 'block count' and 'start address'.

Also cleared on POLLFBW/WAITFBW/JFBW/JNFBW.

POLLXMT/WAITXMT event flag

Cleared on XINIT/XZERO/XCONT.

Set whenever the streamer is ready for a new command.

Also cleared on POLLXMT/WAITXMT/JXMT/JNXMT.

POLLXFI/WAITXFI event flag

Cleared on XINIT/XZERO/XCONT.

Set whenever the streamer runs out of commands.

Also cleared on POLLXFI/WAITXFI/JXFI/JNXFI.

POLLXRO/WAITXRO event flag

Cleared on XINIT/XZERO/XCONT.

Set whenever the the streamer NCO rolls over.

Also cleared on POLLXRO/WAITXRO/JXRO/JNXRO.

POLLXRL/WAIXTRL event flag

Cleared on cog start.

Set whenever location $1FF of the lookup RAM is read by the streamer.

Also cleared on POLLXRL/WAITXRL/JXRL/JNXRL.

POLLATN/WAITATN event flag

Cleared on cog start.

Set whenever any cogs request attention.

Also cleared on POLLATN/WAITATN/JATN/JNATN.

POLLQMT event flag

Cleared on cog start.

Set whenever GETQX/GETQY executes without any CORDIC results available or in progress.

Also cleared on POLLQMT/WAITQMT/JQMT/JNQMT.

Example:	ADDCT1/WAITCT1

'ADDCT1 D,S/#' must be used to establish a CT target. This is done by first using 'GETCT D' to get the current CT value into a register, and then using ADDCT1 to add into that register, thereby making a future CT target, which, when passed, will trigger the CT-passed-CT1 event and set the related event flag.

GETCT   x               'get initial CT

ADDCT1  x,#500          'make initial CT1 target

.loop  WAITCT1                 'wait for CT to pass CT1 target

ADDCT1  x,#500          'update CT1 target

DRVNOT  #0              'toggle P0

JMP     #.loop          'loop to the WAITCT1

It doesn't matter what register is used to keep track of the CT1 target. Whenever ADDCT1 executes, S/# is added into D, and the result gets copied into a dedicated CT1 target register that is compared to CT on every clock. When the CT1 target passes CT, the event flag is set. ADDCT1 clears the CT-passed-CT1 event flag to help with initialization and cycling.

Note: the .loop operator is cleared after compiling instruction and loop can be re-used for writing jump code. Remember to include #(immediate) directive

# Appendix “D” P2 Edge

## D.1) Edge Specifications

Features

● Compact module with Propeller 2 P2X8C4M64P multicore microcontroller

● 6-layer, low noise, system-on-board module

● Integrated thermal planes for low temperature rise characteristics at high speed operation ● Double-sided 80 way 0.05” (1.27mm) edge connector

● Orientation / module locking hole

● Two mounting holes connected to the module ground planes

● 20 MHz crystal ● Adjustable operating frequency; recommended maximum 180 MHz clock

● Overclocking possible beyond 300 MHz

● 16 MB SPI Flash memory

● 64 Smart I/O pins brought out to the Edge Connector

● Buffered LEDs on I/O pins P56 and P57, visible from both sides of the module PCB

● Onboard LED feature enable/disable switch

● Onboard 1.8 V 2-Amp switching regulator with short-circuit, over-current fault and brownout detection protection for the P2 core (VDD)

● Onboard low-noise LDO 3.3 V regulators for the P2 smart-pins (VIO), with short-circuit and over-current fault protection

● Dual power inputs via the edge connector or optional header pads on the back of the module, with reverse polarity protection

● Compatible with the Parallax Prop-Plug #32201 for system programming Key Specifications ● Voltage input requirements: 5 VDC; absolute maximum 5.5 VDC

Input Current requirements:

● Recommended minimum 100 mA

● Typical experimentation 500–1000 mA

● Maximum according to customer application

● Voltage input protection: reverse voltage

● Propeller 2 chip: P2X8C4M64P (8 cogs, 512 KB shared hub RAM, 64 smart pins)

● Non-volatile Memory: 16 MB (128 Mb) SPI Flash

● Crystal: 20 MHz SMT

● Smart I/O pins: 64 accessible, 56 fully free, grouped in 8 sets of 8 I/Os

● Smart I/O pin logic voltage: 3.3 V

● Internal VDD Power Supply: 1.8 V up to 2 A, 1 MHz nominal switching frequency

● VIO Power Supplies: 3.3V up to 300 mA per 8 I/O pins

● Edge Connector: Double sided 80 way 0.05” (1.27mm) pitch edge slot

● Programming: Serial up to 2 MBaud

● Operating temperature: -40 to +185 °F (-40 to +85 °C)

● PCB Dimensions: 1.45 in x 2.04 in (37mm x 52mm)

# Appendix “E” Miscelanous Variable Type Definition

## E.0) Byte/Word/Long Declaration

Propeller has only 3 types of varaibles Byte\Word\Long.

Propeller/Spin endianness is very simple:

The Propeller is a little-endian processor. The less significants bytes of a word or long are stored in the lower memory locations. VARiables and DATa are stored little-endian once compiled. However, since we (i.e. humans who are used to LTR languages) are used to writing numbers in big-endian order, the Spin compiler, for convenience, lets us write "byte $76543210" when we want "byte $10, $32, $54, $76".

The variables are arranged long, word, byte at compile time. There aren't empty bytes between each byte variable. Individual bytes are addressable in hub RAM. Cog RAM is addressable only by longs.

### E.0.1) Byte Declaration 8 bits

Syntax 1  Var (variable declaration)

VAR

byte Temp     ‘Temp is a byte variable

byte Str[25]    ‘Str is a byte array Str[0]-Str[24]

Syntax 2 Dat (data declaration)

DAT

MyData   byte 41,” A”, $2A

MyString  byte “Hello”, 0                   ‘zero terminated string

Hub Memory DAT Block Access

Pub GetData|Temp

Temp ≔ MyData    ‘reads first byte of MyData

<more code>

Pub GetData |Index,Temp

Index ≔ 0

Repeat

Temp ≔ MyString[Index++]

<more code>

While temp > 0                         ‘check after loop

Syntax 3 Byte Hub Memory DAT Block Access

Pub GetData|Temp

Temp ≔Byte[@MyData]

<more code>

Pub GetData|Index,Temp

Index ≔ 0

Repeat

Temp ≔ Byte[@MyString][Index++]

<more code>

While temp > 0

#### E.0.1.1_Example_WRD_Byte_Access from Memory

{{E.0.1.1_Example_WRD_Byte_Access from Memory}}

{{

Temp := Byte[@MyString][Index++]

Temp := MyData

Example

Demonstrate Byte data access

}}

CON

_clkfreq = 200_000_000  ''Debug must be enabled clock must be greater than 10MHZ for Debug

P0 =0 , P1 = 1  , P2 = 2

VAR

byte Temp     'Temp is a byte variable

byte Str[25]  'Str is a byte array Str[0]-Str[24]

PUB main()

debug("--------------------------------------")

debug("Example")

debug("Demonstrate Byte data access")

debug("--------------------------------------")

GetData1()

GetData2()

GetData3()

GetData4()

Repeat

Pub GetData1()|Temp1

Temp1 := MyData                         'reads first byte of MyData

debug("MyData ",udec_byte(Temp1))

Pub GetData2() |Index2,Temp2

Index2 := 0

Repeat

Temp2 := MyString[Index2++]

debug("MyString ",udec_byte(Temp2))

While Temp2 > 0                          'check after loop entry

Pub GetData3()|Index3,Temp3

Index3 :=0

Repeat

Temp3 :=Byte[@MyData][Index3++]

debug("MyData ",udec_byte(Temp3))

while Temp3 > 0                         'check after loop entry

Pub GetData4()|Index4,Temp4

Index4 := 0

Repeat

Temp4 := Byte[@MyString][Index4++]

debug("MyString ",udec_byte(Temp4))

While Temp4 > 0                          'check after loop entry

DAT

MyData    byte  41,"A","B","C", $2A,0

MyString  byte  "Hello", 0              'zero terminated string

### E.0.2) Word  Declaration 16 bits

Syntax 1  Var (variable declaration)

VAR

word var01     ‘var01 Temp is a word variable

word  List[25]    ‘List  is a word array Str[0]-Str[24]

Syntax 2 Dat (data declaration)

DAT

MyList word  $FFFF,  41,” A”, $2A

### E.0.3) Long Declaration 32 bits

Syntax 1  Var (variable declaration)

VAR

Long Temp        ‘ Temp is a Long variable

Long List[25]      ‘List is a long array

Syntax 2 Dat (data declaration)

DAT

MyData       Long     640_000, $BB50                                  ‘Long-aligned/sized data

MyList         Byte      Long       $FF995544 , Long 1_000    ‘ Byte-aligned/long sized

Syntax 3 PUB/PRI

Pub method| Index, var01      ‘declares local method variable of type Long

#### E.0.3.1_Example_WRD_Long_Aligned_Memory

### E.0.4) Address Convention

B[7:0]    = B7B6B5B4B3B2B1B0

W[15:0] = B15B14B13B12B11B10B9B8B7B6B5B4B3B2B1B0

L[31:0]   = B31B30B29B28B27B26B25B24 B23B22B21B20B19B18B17B16 B15B14B13B12B11B10B9B8B7B6B5B4B3B2B1B0

Byte Addressing

DB[3:0]

=D3D2D1D0

Bit Addressing

D[31:0]

= d31d30d29d28d27d26d25d24d23d22d21d20d19d18d17d16d15d14d13d12d11d10d09d08d07d06d05d04d03d02d01d00

Byte Bit Addressing

DBB[37:00]

=d37d36d35d34d33d32d31d30_d27d26d25d24d23d22d21d20_d17d16d15d14d13d12d11d10_d07d06d05d04d03d02d01d00

Nibble Addressing

DN[7:0] = D7D6D5D4D3D2D1D0

D7 =  d73d72d71d70  D6 =  d63d62d61d60   D5 =  d53d52d51d50   D4 = s43s42s41s40

D3 =  d33d32d31d30  D2 =  d23d22d21d20   D5 =  d13d12d11d10   D0 = d03d02d01d00

Byte Addressing

SB[3:0]

= S3S2S1S0

Bit Addressing

S[31:0]

= s31s30s29s28s27s26s25s24s23s22s21s20s19s18s17s16s15s14s13s12s11s10s09s08s07s06s05s04s03s02s01s00

Byte Bit Addressing

SBB[31:0]

= S37S36S35S34S33S32S31S30_ S27S26S25S24S23S22S21S20_ S17S16S15S14S13S12S11S10_ S07S06S05S04S03S02S01S00

Nibble Addressing

SN[7:0] = S7S6S5S4S3S2S1S0

S7 =  s73s72s71s70  S6 =  s63s62s61s60   S5 =  s53s52s51s50   S4 = s43s42s41s40

S3 =  s33s32s31s30  S2 =  s23s22s21s20   S5 =  s13s12s11s10   S0 = s03s02s01s00

B[31:0]

= B31B30B29B28B27B26B25B24B23B22B21B20B19B18B17B16B15B14B13B12B11B10 B9B8B7B6B5 B4B3B2B1B0

S[31:0]

= s31s30s29s28s27s26s25s24s23s22s21s20s19s18s17s16s15s14s13s12s11s10s09s08s07s06s05s04s03s02s01s00

D[31:0]

= d31d30d29d28d27d26d25d24d23d22d21d20d19d18d17d16d15d14d13d12d11d10d09d08d07d06d05d04d03d02d01d00

N[7:0] = N7N6N5N4N3N2N1N0

N7 =  n73n72n71n70  N6 =  n63n62n61n60   N5 =  n53n52n51n50   N4 = n43n42n41n40

N3 =  n33n32n31n30  N2 =  n23n22n21n20   N5 =  n13n12n11n10   N0 = n03n02n01n00

SN[7:0] = S7S6S5S4S3S2S1S0

S7 =  s73s72s71s70  S6 =  s63s62s61s60   S5 =  s53s52s51s50   S4 = s43s42s41s40

S3 =  s33s32s31s30  S2 =  s23s22s21s20   S5 =  s13s12s11s10   S0 = s03s02s01s00

DN[7:0] = D7D6D5D4D3D2D1D0

D7 =  d73d72d71d70  D6 =  d63d62d61d60   D5 =  d53d52d51d50   D4 = s43s42s41s40

D3 =  d33d32d31d30  D2 =  d23d22d21d20   D5 =  d13d12d11d10   D0 = d03d02d01d00

W[31:0] = W1W0

= w31w30w29w28w27w26w25w24w23w22w21w20w19w18w17w16_w15w14w13w12w11w10w9w8_w7w6w5w4w3w2w1w0

W1 = w015w014w013w012w01w010w009w008w007w006w005w004w003w002w001w000

W2 =  w115w114w113w112w11w110w109w108w107w106w105w104w103w102w101w100

Example

D[BH:BL] = D[S[9:5] + S[4:0] : S[4:0]]  =  0 Defines a range of bits

S[9:5] =  %10001 = 17  S[4:0]=%00001 = 1

D[BH:BL] = D[%1001 + %00001 : %00001 = D[17 + 1: 1]  = D[18:1] = 0

D[BH:BL] = D[S[9:5] + S[4:0] : S[4:0]]  =  0 Defines a range of bits

S[31:0] = S[31:10] + S[9:5] + S[4:0] Defines Special function range

## E.1) Number Types Signed,Unsigned,Float, Modular

Unsigned 32 bit number has a range from 0   ----  4_294_967_295

Signed 32 bit number has a range from   -2_147_483_648  ----  2_147_483_647

Note: When possible integer operations should be used they are faster and simpler Float values can be scaled up:  5.6/7.8 = 56/78

### E.1.1_Example_WRD_Signed_Unsigned_Numbers

{{E.1_Example_WRD_Signed_Unsigned_Numbers}}

''unsigned,signed

CON

_clkfreq = 200_000_000  ''Debug must be enabled clock must be greater than 10MHZ for Debug

VAR

Byte cogRunning 'cog ID started is returned or -1 if not started

PUB main()

cogRunning := COGINIT(COGEXEC_NEW,@NumTypes,0)

debug(udec(cogRunning))

debug(ubin(UnSignedMax),uhex(UnSignedMax),udec(UnSignedMax))

debug(ubin(UnSignedMin),uhex(UnsignedMin),udec(UnSignedMin))

debug(sbin(SignedMax),shex(SignedMax),sdec(SignedMax))

debug(sbin(SignedMin),shex(SignedMin),sdec(SignedMin))

repeat

DAT

ORG   0

NumTypes

_Loop                     NOP

JMP     #_Loop

UnSignedMax        long      $FFFF_FFFF

UnSignedMIN        long      0

SignedMax             long      %01111111_11111111_11111111_11111111

SignedMin              long      %10000000_00000000_00000000_00000000

UnSignedBinMax      long      %11111111_11111111_11111111_11111111

UnSignedBinMin       long      %00000000_00000000_00000000_00000000

UnSignedHexMax      long      $FFFF_FFFF

UnSignedHexMin       long      $0000_0000

UnSignedDecMax      long      4_294_967_295

UnSignedDecMin       long      0

SignedBinMax         long      %01111111_11111111_11111111_11111111

SignedBinMin          long      %10000000_00000000_00000000_00000000

SignedHexMax        long      $7FF_FFFF

SignedHexMin         long     -$800_0000

SignedDecMax         long      2_147_483_647

SignedDecMin         long     -2_147_483_648

### E.1.2) Floating Point Numbers

Float  Base 10     12.120    =   1 X 101 + 2 X 100  +  1 X 10-1 + 2 X 10-2

Float  Base 2     101.101    =   1 X 22   +  0 X 21     +  1 X 20    + 1 X 2-1  + 0 X 2-2  +  1 X 2-3

Convert Base 2 Float 101.101 to Base 10 Float

(101.101)2

(101)2  =  4 + 0 + 1 =  5

(.101)2  = 1X1/21 + 0X1/22 + 1X1/23

(.101)2 =  1 X .5   + 0 X .25  + 1 X .125  = (.625)10

(101.101)2 = (5.625)10

Convert Base 10 Float 5.625 to Base 2 Float

5.625

(5)10  =    (4 + 0 + 1)10 = 1 X 22 + 0 X 21 + 1 X 20 =  (101)2

.625  X 2  = 1.25    1

.25    X 2  =  .5        0

.5      X 2  = 1.0       1

(.625)10  = (.101)2

( 5.625)10 = (101.101)2

### E.1.3) Modular Arithmetic

An Introduction to Modular Math

When we divide two integers we will have an equation that looks like the following:

A is the dividend
B is the divisor
Q is the quotient
R is the remainder

Sometimes, we are only interested in what the remainder is when we divide A by B.
For these cases there is an operator called the modulo operator (abbreviated as mod).

Using the same A, B, Q, and R as above, we would have:  A  mod  B = R

We would say this as A modulo B is equal to R. Where B is referred to as the modulus.

Fo

13

#### E.3.1) Time as Modular Arithmetic

Clock Arithmetic or a Circle as a Number Line One way to turn a circle into a number line is to divide it into twelve equal parts. In this case, one step is usually called one hour.

The hour hand moves from 0 to 1, from 1 to 2, ... from 11 to 12 just as it would have on the straight number line. However, 12 equals 0 on this circle, so there it goes 2 Notice that 0 coincides with 12, and as the hour hand moves to the right, 1 coincides with 13, 2 with 14, and so on. The hour hand rotates clockwise which corresponds with numbers increasing when moving to the right on a number line. However, 12 is equivalent to 0 on this circle, which can be written as follows:

12 ≡ 0 (mod 12)

This can be read as 12 is congruent to 0 modulo 12. The usual ”=” sign is reserved for the straight number line; we use ” ≡ ” on the circle instead. The symbol “mod 12” tells us that the circle is divided into 12 equal parts, so that 12 coincides with 0, 13 with 1, etc. In the new notation we have:

12 ≡ 0 (mod 12),     13 ≡ 1 (mod 12), ... 23 ≡ 11 (mod 12)

The 24-Hour Clock There are 24 hours in a day, so one more standard way to turn a circle into a number line is to divide it into 24 equal parts. The US military uses the 24 hour clock. Since 60 is not a multiple of 24, we can’t use the same marks on the face of a 24 hour clock for minutes and hours (look at the minute marks on the face of the 24 hour clock).

Modular Arithmetic

In addition to clock analogy, one can view modular arithmetic as arithmetic of remainders. For example, in mod 12 arithmetic, all the multiples of 12 (i.e., all the numbers that give remainder 0 when divided by 12) are equivalent to 0. In the modular arithmetic notation, this can be written as:

12 x n ≡ 0 (mod 12) for any whole number n.

Similarly, all numbers that give remainder 1 when divided by 12 are equivalent to 1. In other words:

12 x n + 1 ≡ 1 (mod 12) for any whole number n.

Recall that any whole number a can be uniquely written in the form:

a = 12 X n + r

where r is one of the numbers 0, 1, ..., 11. Notice that r is the remainder of the division of a by 12. Therefore, a ≡ r (mod 12). For example:

50 = 5 x 12 + 10, which implies 50 ≡ 10 (mod 12),

40 = 3 x 12 + 4, which means 40 ≡  4 (mod 12).

#### E3.2) Addition /Subtraction property of modular arithmetic:

(A + B) mod C = (A mod C + B mod C) mod C

(A - B) mod C = (A mod C - B mod C) mod C

Exa

Let A=Let (A
LH =
RH =LHS
14) 
 31 m
LHRHS
14) 
4 ) 
RHLS = RHS = 1

#### E3.3) Multiplication property of modular arithmetic:

(A ExaLet A=Let (A moA  * B mo) LHS= RHS= tiLHS (A moLHS (4 moLHS 28 moLHS 4RHSA mo * B mo) RHS4  * 7 ) RHS4 ) RHS 4 RHS 4LS = RHS = 4

### E.1.4) Binary Operations

E.2.3) CRC8 Dallas/Maxim  Algorithum

Binary Multiplication

101          5           111001 + 1 = 26     26 mod 5 = 1

X101        x5

101        25

0000

10100

11001  = 25

Binary Division

__ 101________

101 |11010

101

10

00

100

101

1  Remainder

26 mod 5 = 1

Modulo 2 Division XOR

Modulo-2 division is performed similarly to “normal” arithmetic division. The only difference is that we use modulo-2 subtraction (XOR)) instead of arithmetic subtraction for calculating the remainders in each step.  The quotient  is not of interest.

__111________

101 |11010

101

111

101

100

101

26 mod 5 = 1                                    1  Remainder

## E.2) CRC8 Cycle Redundancy Check

CRC stands for Cyclic Redundancy Check. It is an error-detecting code used to determine if a block of data has been corrupted. The idea is given a block of N bits, let’s compute a checksum of a sort to see if the N bits were damaged in some way, for instance by transit over a network. The extra data we transmit with this checksum is the “Redundancy” part of CRC, and the second C just means this is a “Check” to see if the data are corrupted (as opposed to an ECC code, which both detects and corrects errors).

Simple Parity  is another method for error checking for example the number of 1 ‘s and zero’s are even or odd parity  for examp 10101010 to be even parity  a  1 would be required to append 10101010_1 to obtain 4 one bits for even parity if s 0 odd parity used a 0 bit would be added. If two bits are switched or lost the error checking will not detect it. Only single bit errors can be detected .

Ce is another methof for error detection. Thck Unfortunately sometimes a CRC value is termed a CheckSum.CRC , treats the message as a big number, we choose a special number to divide the message by (referred to as the “CRC generation polynomial” divisor in the literature), and the remainder of the division is the CRC. Intuitively, it should be obvious that we can detect more than single bit errors with this scheme. Additionally, I think it is obvious that some divisors are better than others at detecting errors. Most implementation do not use division in the normal sense but use Module 2 arithmetic which eliminates the need for the borrowing operation. Modulus 2 arithmetic is XOR exclusive OR operation. For CRC calculations, no normal subtraction is used, but all calculations are done modulo 2. In that situation you ignore carry bits and in effect the subtraction will be equal to an exclusive or operation. This looks strange, the resulting remainder has a different value, but from an algebraic point of view the functionality is equal. A discussion of this would need university level knowledge of algebraic field theory.

The CRC is a predetermined number of bits to be used for the error detection. 8,16,32 or 64 bits are commonly used. This set of notes will concentrate on C 1 Wire usage errors. The number of bits in the error code is n and with CRC8 Dallas/Maxim n = 8.

Wyusrator polynomial?  Farca.SufveheBinary Numbers can be represented as a Polynomial:

BinaryNumber = B[7:0]    = B7X7 +  B6X6 +  B5X5 +  B4X4 + B3 X3 +  B2X2 +  B1 X1 +   B0X0

= B727 +  B626 +  B525 +  B424 + B3 23 +  B222 +  B1 21 +   B020

B020  =   B0*1

B1 21 =   B1 *2

B222  =   B2*4

B3 23  =  B3*8

B424  =   B4*16

B525  =   B5*32

B626  =   B6*64

B727  =   B7*128

G(X)  = B[8:0]  =  G8X8 + G7X7 +  G6X6 +  G5X5 + G4X4 + G3 X3 + G2X2 + G1 X1 +  G0X0

G(2) = x8 + x5 + x4 + x0 = 28 + 25 + 24 + 20 =  G7G6G5G4G3G2G1G0  =  100110001

CRC8Maxim Divisor = %1_0011_0001 = 13116 = 30510

Polynomial Generator  bits 0-8 ( 9 actual bits B8B7B6B5B4B3B2B1B0 )

CRC8Dallas\Maxim =  X8 +X5+X4+X0

X8 +X5+X4+X0 = 1*X8+0*X7+0*X6+1*X5+1*X4+0*X3+0*X2+0*X1+1*X0

=  %1_0011_0001  this is the CRC8  9 bit divisor(Coefficeints)

Note: Polynomial is a shift left multiplier of base 2 = %10

Endianness

The endianness is the order of bytes with which data words are stored. We distinguish the following to types:

Little-endian: The least significant byte is stored at the smallest memory address. In terms of data transmission, the least significant byte is transmitted first.

Big-endian: The most significant byte is stored at the smallest memory address. In terms of data transmission, the most significant byte is transmitted first.

Note: Parallax propeller is Little Endian processor LSBytes stored in lowest memory address to MSBytes in increasing memory value.

The same conventions can be used in the ordering of the Polynomials . Typically Big Endian convention is mostly used for CRC calculations but little Endian convention can be used.

End_1X8+2+1+:8B7B6B5B4B3B2B1B0

Big Endian

G(X) = B8X8 +  B7X7 +   B6X6 +  B5X5 +   B4X4 +   B3X3 +   B2X2 +  B1X1 + B0X0

100000111 -   B[8:0] = 100000111

Little Endian

G(X) = B0X8 +  B1X7 +   B2X6 +  B3X5 +   B4X4 +   B5X3 +   B6X2 +  B7X1 + B8X0

100000111 -   B[8:0] = 111000001

Endian Example_2

X8 + X5 + X4 + 1      B[8:0] = B8B7B6B5B4B3B2B1B0

Big Endian

G(X) = B8X8 +  B7X7 +   B6X6 +  B5X5 +   B4X4 +   B3X3 +   B2X2 +  B1X1 + B0X0

100110001 -   B[8:0] = 100110001

Little Endian

G(X) = B0X8 +  B1X7 +   B2X6 +  B3X5 +   B4X4 +   B5X3 +   B6X2 +  B7X1 + B8X0

100110001 -   B[8:0] = 100011001

Note: That most polynomial specifications either drop the MSB or LSB, since they are always 1.

CRC8Dallas/Maxim = $8C = %10001100 or %100011001 adding 1 to LSB

The most commonly used generation polynomials are as follows:

Most Hobbyist usage of CRC values use an 8 bit CRC value and for the remainder of this discussion

CRC-8Maxim will be used. The manufacturer “MAXIM Integrated” now part of Analog devices originally used this for their 1 wire devices which has the CRC8Maxim registers built into their devices.

### E.2.0) CRC Transmission Process

1) Create DataStream = Data + Checksum(CRC)

Divisor is CRC8Dallas\Maxim =  X8 +X5+X4+X0 = 30510 = 13116 =  1001100012

Data to be Transferred Let $7778797A = DATA  the bytes to be transferred

Dividend = DataStream + 8 zeroes (k = number bits in Divisor = n+1 = 9bits)

ADD “n” zero’s (CRC8 is “8+1 = n+1”) so add 8 zero’s to stream to be transferred:

Dividend  $7778797A00 = 111_0111_0111_1000_0111_1001_0111_1010_0000_0000

Calculate the CRC (See CRC8 Dalla/Maxim Algorithum)

CRC  = 10210 = 0x6616 = 1001100012   from CRC calculator n = number bits in CRC = 8

Data Stream = $7778797A66

2) Transmit Data From Sender to Receiver Device

Both Transmitter and receiver must be aware of “Generation Polynomial” in this case

CRC8Dallas\Maxim =  X8 +X5+X4+X0 = 30510 = 13116 =  1001100012

3) Receive data and create a receiving end CRC of Data Stream

When Checking CRC from the receiving end the , the generated CRC is appended to the Data since CRC is a linear function with a property that  CRC( xꚚyꚚz)=CRC(x)ꚚCRC(y)ꚚCRC(z).

DataStream XOR CRC = 0 in above example 0x66 Ꚛ 0x66 = 0. Doing a CRC on the DataStream if data is good will have a CRC of 0.

### E.2.1) Maxim 1-Wire CRC

The error detection scheme most effective at locating errors in a serial-data stream with a minimal amount of hardware is the CRC. The operation and properties of the CRC function used in Maxim products is presented without going into the mathematical details of proving the statements and descriptions. The mathematical concepts behind the properties of the CRC are described in detail in the references. The CRC can be most easily understood by considering the function as it would actually be built in hardware, usually represented as a shift register arrangement with feedback as shown in Figure 2. Alternatively, the CRC is sometimes referred to as a polynomial expression in a dummy variable X, with binary coefficients for each of the terms. The coefficients correspond directly to the feedback paths shown in the shift register implementation. The number of stages in the shift register for the hardware description, or the highest order coefficient present in the polynomial expression, indicate the magnitude of the CRC value that is computed. CRC codes that are commonly used in digital data communications include the CRC-16 and the CRC-CCITT, each of which computes a 16-bit CRC value. The Maxim 1-Wire CRC magnitude is 8 bits, which is used for checking the 64-bit ROM code written into each 1-Wire product. This ROM code consists of an 8-bit family code written into the least significant byte, a unique 48-bit serial number written into the next 6 bytes, and a CRC value that is computed based on the preceding 56 bits of ROM and then written into the most significant byte. The location of the feedback paths represented by the exclusive-OR gates in Figure 2, or the presence of coefficients in the polynomial expression, determine the properties of the CRC and the ability of the algorithm to locate certain types of errors in the data. For the 1-Wire CRC, the types of errors that are detectable are:

- Any odd number of errors anywhere within the 64-bit number.

- All double-bit errors anywhere within the 64-bit number.

- Any cluster of errors that can be contained within an 8-bit "window" (1-8 bits incorrect).

- Most larger clusters of errors.

The input data is exclusive-OR'ed with the output of the eighth stage of the shift register in Figure 2. The shift register can be considered mathematically as a dividing circuit. The input data is the dividend, and the shift register with feedback acts as a divisor. The resulting quotient is discarded, and the remainder is the CRC value for that particular stream of input data, which resides in the shift register after the last data bit has been shifted in. From the shift register implementation it is obvious that the final result (CRC value) is dependent, in a very complex way, on the past history of the bits presented. Therefore, it would take an extremely rare combination of errors to escape detection by this method.

Figure 2. Maxim 1-Wire 8-bit CRC.

### E.2.1) Modul 2 Binary Division Vs Traditional Division

The basic idea of CRC algorithm is to treat the transmitted data as a very long number of digits.Divide this number by another number.The resulting remainder is appended to the original data as check data.Also take the data from the above example:

6, 23, 4 can be seen as a binary number: 0000011000010111 00000010

If 9 is chosen by dividing, the binary representation is: 1001

Then the division operation can be expressed as:

As you can see, the last remaining number is 1.If we use this remainder as a checksum, the data transferred is: 6, 23, 4, 1.

The CRC algorithm is a bit similar to this process, but it does not use the usual division in the example above.In the CRC algorithm, binary data streams are used as coefficients of the polynomial, followed by the multiplication and division of the polynomial.Let's give an example.

For example, we have two binary numbers: 1101 and 1011.

1101 is associated with the following polynomial: 1x3+1x2+0x1+1x0=x3+x2+x0

1011 is associated with the following polynomial: 1x3+0x2+1x1+1x0=x3+x1+x0

Multiplication of two polynomials: (x3+x2+x0) (x3+x1+x0)=x6+x5+x4+x3+x3+x2+x1+x0

When the result is obtained, the modulo 2 operation is used to merge the same items.That is, multiplication and division use normal polynomial multiplication and division, while addition and subtraction use modulo 2 operations.The so-called modulo 2 operation is to divide the result by 2 and take the remainder.For example, 3 mod 2 = 1.Therefore, the resulting polynomial above is: x6+x5+x4+x3+x2+x1+x0, corresponding to the binary number: 111111

Addition and subtraction with modulo 2 is actually an operation, which is what we usually call XOR:

As mentioned above, half-day polynomials, in fact, even without introducing the concept of polynomial multiplication and division, can explain the particularity of these operations.Only polynomials are mentioned in almost all the literature explaining the CRC algorithm, so a few basic concepts are simply written here.However, it is very tedious to always use this polynomial representation, and the following instructions will try to use a more concise way of writing.

The division operation is similar to the multiplication concept given above, or the addition and subtraction are replaced by XOR.Here is an example:

The data to be transferred is: 1101011011

The divisor is set to 10011

Before calculating, four 0:11010110000 are added to the back of the original data, so the reason for adding 0 is explained later.

From this example, it can be seen that after the addition and subtraction of module 2, the problem of borrowing does not need to be considered, so division becomes simpler.The final remainder is the CRC checkword.In order to perform the CRC operation, that is, this special division operation, a dividend must be specified. In the CRC algorithm, this divider has a special name called "Generate Polynomial".Selection of the resulting polynomial is a very difficult problem. If not, the probability of detecting errors will be much lower.Fortunately, this problem has been studied by experts for a long time. For those of us users, we just need to use the ready-made results.

### E.2.2) CRC8Maxim x^8+x^5+x^4 + 1 Lookup Table

CRC linear f w  CRC( xꚚyꚚz)=CRC(x)ꚚCRC(y)ꚚCRC(z)

{\displaystyle \operatorname {crc} (x\oplus y\oplus z)=\operatorname {crc} (x)\oplus \operatorname {crc} (y)\oplus \operatorname {crc} (z);}

E.2.3) CRC8 Dallas/Maxim  Algorithum

Binary Multiplication

101          5           111001 + 1 = 26     26 mod 5 = 1

X101        x5

101        25

0000

10100

11001  = 25

Binary Division

__ 101________

101 |11010

101

10

00

100

101

1  Remainder

26 mod 5 = 1

Modulo 2 Division XOR

Modulo-2 division is performed similarly to “normal” arithmetic division. The only difference is that we use modulo-2 subtraction (XOR)) instead of arithmetic subtraction for calculating the remainders in each step.  The quotient  is not of interest.

__111________

101 |11010

101

111

101

100

101

26 mod 5 = 1                                    1  Remainder

### E.2.4) Types of CRCs

There are different types of CRCs. They are categorized by the degree of the polynomial they use. As the first exponent of a polynomial of degree n is always present by definition (otherwise it would have a lower degree), its binary representation always begins with a 1.

In other words, the first bit of a binary polynomial representation doesn’t carry any information about the polynomial when we agree on a fixed degree.

For that reason, the first bit of a binary polynomial representation is always dropped when computing a CRC in software. So the bit size of the resulting binary is always n for a polynomial of degree n.

It is apparent there is a myriad of CRC implementations and the sending and receiving devices must be using the sam methodology. It is because of this non standardations complexity (obfuscation) rules.

Example:

| Polynomial | Binary Representation | Binary (1st bit dropped) | Bit Size |
| --- | --- | --- | --- |
| x4 + x2 + x + 1 | 10111 | 0111 | 4 |
| x4 + x3 + x2 + 1 | 11101 | 1101 | 4 |
| x8 + x4 + x2 + 1 | 100010101 | 00010101 | 8 |

CRCs types are named by their bit size. Here are the most common ones:

CRC-8

CRC-16

CRC-32

CRC-64

CRC-1 (parity bit) is a special case

Generally, we can refer to a CRC as CRC-n, where n is the number of CRC bits and the number of bits of the polynomial’s binary representation with a dropped first bit. Obviously, different CRCs are possible for the same n as multiple polynomials exist for the same degree.

#### E.2.4.1) Error Detection

How do we choose a suitable CRC and a respective polynomial? There are three things we need to consider:

Random Error Detection Accuracy

Burst Error Detection Accuracy

The Redundancy Factor

#### E.2.4.2) Random Error Detection Accuracy

Random errors are errors that can occur randomly in any data. For example, a single bit is flipped when transmitting data or a few bits are lost during the transmission.

Depending on the bit size of the CRC we use, we can detect most of these random errors. However, for a CRC-n, 1/2n of these errors cannot be detected. The following table shows the percentage of the possible random errors that remain undetected for each CRC type:

| CRC Type | Undetected Errors | % Undetected |
| --- | --- | --- |
| CRC-8 | 1/28 | 0.39 |
| CRC-16 | 1/216 | 0.0015 |
| CRC-32 | 1/232 | 0.00000002 |
| CRC-64 | 1/264 | 5.4 x 10-20 |

#### E.2.4.3) Burst Error Detection Accuracy

Errors in data transmission are oftentimes not random but produced over a consecutive sequence of bits. Such errors are called burst errors. They are the most common errors in data communication.

It’s one of the CRC’s strongest properties to detecting burst errors reliably.

A CRC-n can detect single burst errors with a maximum length of n bits. However, this depends a lot on the polynomial used for computing the CRC. Some polynomials are able to detect multiple bursts of errors in the transmitted data.

| CRC Type | Burst Error Detection |
| --- | --- |
| CRC-8 | at least a single burst of <= 8 bits |
| CRC-16 | at least a single burst of <= 16 bits |
| CRC-32 | at least a single burst of <= 32 bits |
| CRC-64 | at least a single burst of <= 64 bits |

#### E.2.4.4) The Redundancy Factor

Using a CRC for error detection comes at the cost of extra (non-meaningful) data. When we use a CRC-32 (4 bytes), we need to transmit two more bytes of “unnecessary” data as compared to a CRC-16. CRCs with a lower bit size are obviously cheaper with respect to storage space.

Based on these three factors, we can decide which CRC type to choose for our application. However, the polynomial you choose for your CRC also affects the efficiency and quality of your error detection. But that’s a topic for itself and we won’t cover it in this article. Fortunately, there are a couple of standard polynomials used for a particular CRC type and in most cases it makes sense to just use one of these.

### E.2.5) CRC8Dallas\Maxim Algorithum (Rayman thanks for info)

1) Divisor is 10011001 n = 9 bit divisor (CRC bits = n-1)

2) Data bits are to be revesed ordered and add n-1 zeroes (8)

3) Perform Modul 2 division

4) Reverse order of remainder is th CRC8Dallas\Maxim

#### E.2.5.1_Example 1 $C2

Data = $C2  = %11000010  ReverseC2 = > %01000011 = $43   Divisor = 100110001

_________________

100110001 |0100001100000000

100110001

111101000

100110001

110110010

100110001

100000110

100110001

01101110  =>  0111011 reverse order 8 bit

CRCMaxim($C2) =%01110110 = $76 = 118

#### E.2.5.2_Example $BC

Data = $BC  = %10111100  ReverseBC = > 00111101 = $3E   Divisor = 100110001

_________________

100110001 |0011110100000000

100110001

110110010

100110001

100000110

100110001

110111000

100110001

10001001  =>  10010001 reverse order 8 bit

CRCMaxim($BC) =%10010001 = $91 = 145

#### E.2.5.3 Example 2 $C2BC

CRC8Maxim(Byte1) = CRC8Maxim(C2) = $76

Byte2Data = $BC

CRC8Maxim(C2) Ꚛ Data(BC)  = $76 Ꚛ $BC = 01110110

10111100

11001010  =$CA

Data = $CA  = %11001010  ReverseC2 = > %01010011 = $53   Divisor = 100110001

_________________

100110001 |0101001100000000

100110001

111110100

100110001

110001010

100110001

101110110

100110001

100011100

100110001

000101101    =>  10101000 reverse order 8 bit

CRCMaxim($CA)      =  %10101000 = $B4 = 180

CRCMaxim($C2BC) =  %10101000 = $B4 = 180

#### E.2.5.4 Example 4 $7778797A

a) CRC($7778) = CRC(CRC($77) Ꚛ $78)

CRC($77) Ꚛ $78  = $7B Ꚛ $78 = 01111011

01111000

00000011  =$03

CRC($7778) = CRC($03) = $E2 =226

b) CRC($777879) = CRC(CRC($7778) Ꚛ $79)

CRC($7778) Ꚛ $79  = $E2 Ꚛ $79 =   11100010

01111001

10011011  =$9B

CRC($777879) = CRC($9B) = $31 = 49

c) CRC($7778797A) = CRC(CRC($777879) Ꚛ $7A)

CRC($777879) Ꚛ $7A  = $31 Ꚛ $7A =   00110001

01111010

01001011 =$4B

CRC($7778797A) = CRC($4B) = $66 = 102

### E.2.5) Websites for CRC:

https://rndtool.info/CRC-step-by-step-calculator/   dividend/divisor steps

https://crccalc.com    CRC Calculator different values

https://www.youtube.com/watch?v=izG7qT0EpBw    provides overview of how CRC derived

https://quickbirdstudios.com/blog/validate-data-with-crc/  CRC8 polynomial generation droping bit

# Appendix “F” Hardware and Constants

## F.1) Cog CPU

Each Cog has a PC program Counter, C Carry Flag, Z Zero Flag , ALU Arithmetic Logic Unit. Q is a hidden special purpose register inside the cog's processor core (ALU). Q also must have a couple of associated flags to tell subsequent instructions that Q has just been refreshed. At least two flags are needed for RDLONG/WRLONG to know if they should burst read/write to cogRAM or lutRAM. SETQ sets first flag and SETQ2 sets the second flag of the Q register.

## F.2) Hardware Register Constants

| Variables (all LONG) | Variable Name | Address or Offset | Description | Useful in Spin2 | Useful in Spin2-PASM | Useful in PASM-Only |
| --- | --- | --- | --- | --- | --- | --- |
| Hub Locations | CLKMODE CLKFREQ | $00040 $00044 | Clock mode value Clock frequency value | Yes Yes | Yes Yes | No No |
| Hub VAR | VARBASE | +0 | Object base pointer, @VARBASE is VAR base, used by method-pointer calls | Maybe | No | No |
| Cog Registers | PR0 PR1 PR2 PR3 PR4 PR5 PR6 PR7 IJMP3 IRET3 IJMP2 IRET2 IJMP1 IRET1 PA PB PTRA PTRB DIRA DIRB OUTA OUTB INA INB | $1D8 $1D9 $1DA $1DB $1DC $1DD $1DE $1DF $1F0 $1F1 $1F2 $1F3 $1F4 $1F5 $1F6 $1F7 $1F8 $1F9 $1FA $1FB $1FC $1FD $1FE $1FF | Spin2 <-> PASM communication Interrupt JMP's and RET's Pointer registers Data pointer passed from COGINIT Code pointer passed from COGINIT Output enables for P31..P0 Output enables for P63..P32 Output states for P31..P0 Output states for P63..P32 Input states from P31..P0 Input states from P63..P32 | Yes Yes Yes Yes Yes Yes Yes Yes No No No No No No No No No No Yes Yes Yes Yes Yes Yes | Yes Yes Yes Yes Yes Yes Yes Yes Yes Yes Yes Yes Yes Yes Yes Yes Yes Yes Yes Yes Yes Yes Yes Yes | No No No No No No No No Yes Yes Yes Yes Yes Yes Yes Yes Yes Yes Yes Yes Yes Yes Yes Yes |

## F.3) HUB Memory

Hub Memory is located in (and managed by) the Hub and is accessible to each cog, in a time-shared, round-robin fashion. It consists of Hub RAM and Hub ROM.

Hub RAM is 512 KB, accessible as bytes, words, and longs. It holds your program, data, global variables, and stack space, which collectively make up your Propeller Application. Hub RAM is also used to share information between cogs or process larger blocks of data than will fit into Cog RAM.

The Hub ROM is 16 KB and holds read-only system resources such as the Boot Loader. It is loaded into the last 16 KB of Hub RAM upon boot-up.

## F.4) HUB Memory Spin2 Stack.

The Spin Interpreter implements a call stack to facilitate Spin method calling, parameter passing, expression evaluation, and returning method results. 

The Propeller Application (if Spin2-based) has an automatically allocated stack located in Hub RAM immediately following the application's global variable memory. It expands and collapses as needed; growing towards higher addresses and shrinking towards lower addresses. 

Spin methods that are manually launched into other cogs store their stack starting at the StkAddr address given by the COGSPIN command that launched them (usually inside a long array in variable space). Their stacks expand and contract in the same manner as with the Propeller Application stack. In both cases, the capacity of the stack (method nesting-depth, parameter list length, expression complexity, and return result length) is limited only by the amount of free memory available (for the application) or memory provided (by the developer).

## F.5) DAT Blocks

DAT block symbols exist in Hub RAM, but if they are part of PASM2 code that is launched, they are also in Cog RAM where they are manipulated independently.

The DAT block itself is stored in the application image in Hub RAM. Spin2-based references to DAT symbols access the corresponding location and data in Hub RAM.

When a cog is launched with assembly code, any DAT symbols within 504 longs of the launch point are copied into Register RAM. Unlike with Spin2 code, PASM2 code that references those symbols accesses the corresponding Register RAM* locations (its local copy) instead of Hub RAM. In addition, those symbolic references are addressed as longs of Register RAM memory, regardless of how the symbol was actually declared.

* Or Lookup RAM, if the code launched into Register RAM manually loads PASM2+symbol code into, and executes code from, Lookup RAM.

The DAT block's purpose is to hold fixed data and Propeller 2 Assembly code for the application. Symbols may be included to reference this data and code.

DAT blocks are stored in the application image in Hub RAM. Just like with code in PUB and PRI blocks, there is only one instance of each DAT block in the running application, regardless of how many instances of the containing object there are. This means that Spin-based references to DAT symbols each access the same corresponding location and data in Hub RAM, regardless of which instance of that object is making the reference. This is handy to share memory between multiple instances of a Spin2 object.

When a cog is launched with assembly code, any DAT symbols within 504 longs of the launch point are copied into Register RAM. Unlike with Spin2 code, PASM2 code that references those symbols accesses the corresponding Register RAM* locations (its local copy) instead of Hub RAM. In addition, those symbolic references are addressed as longs of Register RAM memory, regardless of how the symbol was actually declared. In PASM2, no Hub RAM references can be made by simply using the declared symbolic name; instead, the absolute address of that symbol must be passed from the Spin2 object and used along with instructions like RDLONG and WRLONG.

There's nothing preventing the contents of DAT from being modified at runtime. This naturally leads to a special use–; "special values" may be defined in a DAT block that are easily referenced by every Spin2 object instance (and every new launch of PASM2 code) and can be modified at runtime to instantly change what each Spin2 instance (and future new PASM2 launched cogs) sees.

* Or Lookup RAM locations, if symbolic data were initially loaded in by the code running in Register RAM.

## F.6) Propeller Electrical Specifications

Absolute Maximum Electrical Ratings Stresses in excess of the absolute maximum ratings can cause permanent damage to the device. These are absolute stress ratings only. Functional operation of the device is not implied at these or any other conditions in excess of those given. Exposure to absolute maximum ratings for extended periods can adversely affect device reliability.

## F.7) Built In Numeric Constants

| Symbol Value | Symbol Name | Details |
| --- | --- | --- |
| $0000_0000 | FALSE | Same as 0 |
| $FFFF_FFFF | TRUE | Same as -1 |
| $8000_0000 | NEGX | Negative-extreme integer, -2_147_483_648 ($8000_0000) |
| $7FFF_FFFF | POSX | Positive-extreme integer, +2_147_483_647 ($7FFF_FFFF) |
| $4049_0FDB | PI | Single-precision floating-point value of Pi, 3.14159265 |

# Appendix “G” Table of Operators

Operators

Below is a table of all the operators available for use in Spin2 methods. Compile-time expressions can use the unary, binary, ternary and float operators.

# Appendix “H” Table of Built In Methods

# Appendix “I” Hub Operation

## I.1) Hub RAM

The globally-accessible Hub RAM can be read and written as bytes, words, and longs, in little-endian format. Specifically, little-endian is when the least significant bytes are stored before the more significant bytes, and big-endian is when the most significant bytes are stored before the less significant bytes.Hub addresses are always byte-oriented.

There are no special alignment rules for words and longs in Hub RAM. Cogs can read and write bytes, words, and longs starting at any hub address, as well as execute PASM2 instructions (longs) from any hub address starting at $400. The last 16 KB of Hub RAM is normally addressable at both its normal address range, as well as at $FC000..$FFFFF. This provides a stable address space (regardless of future Propeller 2 variations) for the 16 KB of internal ROM which gets cached into the last 16 KB of Hub RAM on startup.

This upper 16 KB mapping is also used by the cog debugging scheme. The last 16 KB of RAM can be hidden from its normal address range and made read-only at $FC000..$FFFFF. This is useful for making the last 16 KB of RAM persistent, like ROM. It is also how debugging is realized, as the RAM mapped to $FC000..$FFFFF can still be written to while executing code from within debug interrupt service routines, permitting the otherwise-protected RAM to be used as debugger-application space and cog-register swap buffers for debug interrupts.

Cog-to-Hub RAM Interface Hub RAM consists of 32-bit-wide single-port RAMs with byte-level write controls. This RAM is split into slices (one per cog) that are multiplexed among all cogs. On the Propeller 2 (P2X8C4M64P), each RAM slice holds every 8th long in the composite Hub RAM. Upon every clock cycle, each cog can access the "next" RAM slice, allowing for continuous bidirectional streaming of sequential Hub RAM longs.

The Hub RAM Interface diagram illustrates this process conceptually as the collective of RAM slices rotates around, each facing a new cog every clock cycle. When a cog wants to read or write the Hub RAM, it must wait up to #cogs-1 clocks to access the initial RAM slice of interest. Once that occurs, subsequent locations (slices) can be accessed on every clock, thereafter, for continuous reading or writing of 32-bit longs. Normally, if the cog chooses not to access the next available location upon the next clock, it must once again wait up to 7 clocks to re-align with the desired slice.

However, each cog has an optional hub FIFO interface that smooths out data flow for less than 32-bits-per-clock access. This hub FIFO interface can be set for hub-RAM-read or hub-RAM-write operation to allow Hub RAM to be either sequentially read or sequentially written in any combination of bytes, words, or longs, at any rate, up to one long per clock. Regardless of the transfer frequency or the word size, the FIFO will ensure that the cog's reads or writes are all properly conducted from/to the composite Hub RAM.

## 1.2) COG HUB Access

Cogs can access Hub RAM either via the sequential FIFO interface, or by waiting for RAM slices of interest, while yielding to the FIFO. If the FIFO is not busy (which is soon the case if data is not being read from or written to it) random accesses will have full opportunity to access the composite Hub RAM. There are three ways the hub FIFO interface can be used, and it can only be used for one of these at a time:

● Hub execution (when the PC is $00400..$FFFFF)

● Streamer usage (background transfers from Hub RAM → pins/DACs, or from pins/ADCs → Hub RAM)

● Software usage (fast sequential-reading or sequential-writing instructions)

For streamer or software usage, FIFO operation must be established by a RDFAST or WRFAST instruction executed from Cog RAM (Register/Lookup, $00000..$003FF). After that, and while remaining in Cog RAM, the streamer can be enabled to begin moving data in the background, or the two-clock RFxxxx/WFxxxx instructions can be used to manually read and write sequential data.

The FIFO contains (#cogs+11) stages. When in read mode, the FIFO loads continuously whenever less than (#cogs+7) stages are filled, after which point, up to 5 more longs may stream in, potentially filling all stages. These metrics ensure that the FIFO never underflows, under all potential reading scenarios.

# Appendix “J” System Clock

The system clock is the time base for all internal components and can be configured in several ways.

● Direct from internal slow clock (RCSLOW); a ~20 kHz oscillator is intended for low-power operation

● Direct from internal fast clock (RCFAST); a 20 MHz+ oscillator designed for minimum 20 MHz operation

● Direct from XI pin; driven externally via a clock oscillator or a crystal oscillator

● PLL-modified XI pin; driven externally via a clock oscillator or a crystal oscillator and the signal internally modified by the PLL (phase-locked loop), usually to multiple to a much higher frequency

The system clock is configured by the running Propeller 2 application using the HUBSET instruction in this format:

HUBSET ##%0000_000E_DDDD_DDMM_MMMM_MMMM_PPPP_CCSS 'set clock mode

The bit fields (E, D, M, P, C, and S) are described in the following tables.

WARNING: Incorrectly switching away from the PLL setting (%SS = %11) can cause a glitch which will hang the clock circuit. In order to safely switch, always start by switching to an internal oscillator using either HUBSET #$F0 (for RCFAST) or HUBSET #$F1 (for RCSLOW).

PLL Example The PLL divides the XI pin frequency from 1 to 64, then multiplies the resulting frequency from 1 to 1024 in the VCO. The VCO frequency can be used directly, or divided by 2, 4, 6, ...30, to get the final PLL clock frequency which can be used as the system clock.

The PLL's VCO is designed to run between 100 MHz and 200 MHz and should be kept within that range.

Let's say you have a 20 MHz crystal attached to XI and XO and you want to run the Prop2 at 148.5 MHz. You could divide the crystal by 40 (%DDDDDD = 39) to get a 500 kHz reference, then multiply that by 297 (%MMMMMMMMMM = 296) in the VCO to get 148.5 MHz. You would set %PPPP to %1111 to use the VCO output directly. The configuration value would be %1_100111_0100101000_1111_10_11. The last two 2-bit fields select 15 pf crystal mode and the PLL. In order to realize this clock setting, though, it must be done over a few steps:

The clock selector controlled by the %SS bits has a deglitching circuit which waits for a positive edge on the old clock source before disengaging, holding its output high, and then waiting for a positive edge on the new clock source before switching over to it. It is necessary to select mode %00 or %01 while waiting for the crystal and/or PLL to settle into operation, before switching over to either.

# Appendix “K” Locks (HUB Cog Memory Access)

Locks For application-defined cog coordination, the hub provides a pool of 16 semaphore bits, called locks. Cogs may use locks, for example, to manage exclusive access of a resource or to represent an exclusive state, shared among multiple cogs. What a lock represents is completely up to the application using it; they are a means allowing one cog at a time the exclusive status of 'owner' of a particular lock ID. In order to be useful, all participant cogs must agree on a lock's ID and what purpose it serves. The LOCK instructions are:

LOCKNEW D {WC}

LOCKRET {#}D

LOCKTRY {#}D {WC}

LOCKREL {#}D {WC}

Lock Usage In order to use a lock, one cog must first allocate a lock with LOCKNEW and communicate that lock's ID with other cooperative cogs. Cooperative cogs then use LOCKTRY and LOCKREL to respectively take or release ownership of the state which that lock represents. If the lock is no longer needed by the application, it may be returned to the unallocated lock pool by executing LOCKRET. A cog may allocate more than one lock.

At any time, a cog may attempt to own a lock (ie: the state that lock represents) by using LOCKTRY. The Hub grants or denies ownership in response, ensuring that, at most, one cog owns the lock at any time. If a cog is granted ownership, it can perform the task defined for that lock and then use LOCKRET to release ownership, allowing any other cog to attempt ownership. Only the cog that has taken ownership of the lock can release it; however, a lock will also be implicitly released if the owner cog is stopped (COGSTOP) or restarted (COGINIT).

# Appendix “L” Cordic Solver(HUB contains “Co0rdinate RotationDigital Computer”)

The Hub contains a 54-stage pipelined CORDIC solver (Coordinate Rotation Digital Computer) that can compute the following functions for all cogs:

● 32 x 32 unsigned multiply with 64-bit product

● 64 / 32 unsigned divide with 32-bit quotient and 32-bit remainder

● Square root of 64-bit unsigned value with 32-bit result

● 32-bit signed (X, Y) rotation around (0, 0) by a 32-bit angle with 32-bit signed (X, Y) results

● 32-bit signed (X, Y) to 32-bit (length, angle) cartesian to polar operation

● 32-bit (length, angle) to 32-bit signed (X, Y) polar to cartesian operation

● 32-bit unsigned integer to 5:27-bit logarithm

● 5:27-bit logarithm to 32-bit unsigned integer

Each cog can issue one CORDIC instruction per its hub access window (which occurs once every eight clocks) and retrieve the result 55 clocks later via the GETQX and GETQY instructions. For faster throughput cogs can take advantage of the hub access window and CORDIC pipeline to issue a stream of CORDIC instructions interleaved with retrieving corresponding results.

Multiply Use the QMUL instruction to multiply two unsigned 32-bit numbers together and retrieve the CORDIC results with the GETQX and GETQY instructions (for lower and upper long, respectively).

Divide Use the QDIV or QFRAC instruction (either with optional preceding SETQ instruction) to divide a 64-bit numerator by a 32-bit denominator, then retrieve the CORDIC results with the GETQX and GETQY instructions (for quotient and remainder, respectively 18Square Root Use the QSQRT instruction on a 64-bit number and retrieve the square root CORDIC result with the GETQX instruction.

(X, Y) Rotation Use the SETQ instruction followed by the QROTATE instruction to rotate a 32-bit signed Y and X point pair by an unsigned 32-bit angle and retrieve the CORDIC results with the GETQX and GETQY instructions for X and Y, respectively.

(X, Y) to (length, angle) Use the QVECTOR instruction to convert a (X, Y) cartesian coordinate into (length, angle) polar coordinate and retrieve the CORDIC results with the GETQX and GETQY instructions (for length and angle, respectively).

(length, angle) to (X, Y) Use the QROTATE instruction to convert a (length, angle) polar coordinate into (X, Y) cartesian coordinate and retrieve the CORDIC results with the GETQX and GETQY instructions (for X and Y, respectively).

Logarithm Use the QLOG instruction on an unsigned 32-bit integer and retrieve the 5:27-bit logarithm CORDIC result (5-bit exponent and 27-bit mantissa) with the GETQX instruction.

Exponent Use the QEXP instruction on a 5:27-bit logarithm and retrieve the unsigned 32-bit integer CORDIC result with the GETQX instruction.

# Appendix “M” PASM Instructions

-------------------------------------------------------------------------

| Order | #S = immediate (I=1). S = register. #D = immediate (L=1). D = register. - Assembly Syntax - | * Z =( result)=0 |
| --- | --- | --- |
| 1 | NOP | No operation. |
| 2 | ROR D,{#}S {WC/WZ/WCZ} | Rotate right. D = [31:0] of ({D[31:0], D[31:0]} >> S[4:0]). C = last bit shifted out if S[4:0] > 0, else D[0]. * |
| 3 | ROL D,{#}S {WC/WZ/WCZ} | Rotate left. D = [63:32] of ({D[31:0], D[31:0]} << S[4:0]). C = last bit shifted out if S[4:0] > 0, else D[31]. * |
| 4 | SHR D,{#}S {WC/WZ/WCZ} | Shift right. D = [31:0] of ({32'b0, D[31:0]} >> S[4:0]). C = last bit shifted out if S[4:0] > 0, else D[0]. * |
| 5 | SHL D,{#}S {WC/WZ/WCZ} | Shift left. D = [63:32] of ({D[31:0], 32'b0} << S[4:0]). C = last bit shifted out if S[4:0] > 0, else D[31]. * |
| 6 | RCR D,{#}S {WC/WZ/WCZ} | Rotate carry right. D = [31:0] of ({{32{C}}, D[31:0]} >> S[4:0]). C = last bit shifted out if S[4:0] > 0, else D[0]. * |
| 7 | RCL D,{#}S {WC/WZ/WCZ} | Rotate carry left. D = [63:32] of ({D[31:0], {32{C}}} << S[4:0]). C = last bit shifted out if S[4:0] > 0, else D[31]. * |
| 8 | SAR D,{#}S {WC/WZ/WCZ} | Shift arithmetic right. D = [31:0] of ({{32{D[31]}}, D[31:0]} >> S[4:0]). C = last bit shifted out if S[4:0] > 0, else D[0]. * |
| 9 | SAL D,{#}S {WC/WZ/WCZ} | Shift arithmetic left. D = [63:32] of ({D[31:0], {32{D[0]}}} << S[4:0]). C = last bit shifted out if S[4:0] > 0, else D[31]. * |
| 10 | ADD D,{#}S {WC/WZ/WCZ} | Add S into D. D = D + S. C = carry of (D + S). * |
| 11 | ADDX D,{#}S {WC/WZ/WCZ} | Add (S + C) into D, extended. D = D + S + C. C = carry of (D + S + C). Z = Z AND (result == 0). |
| 12 | ADDS D,{#}S {WC/WZ/WCZ} | Add S into D, signed. D = D + S. C = correct sign of (D + S). * |
| 13 | ADDSX D,{#}S {WC/WZ/WCZ} | Add (S + C) into D, signed and extended. D = D + S + C. C = correct sign of (D + S + C). Z = Z AND (result == 0). |
| 14 | SUB D,{#}S {WC/WZ/WCZ} | Subtract S from D. D = D - S. C = borrow of (D - S). * |
| 15 | SUBX D,{#}S {WC/WZ/WCZ} | Subtract (S + C) from D, extended. D = D - (S + C). C = borrow of (D - (S + C)). Z = Z AND (result == 0). |
| 16 | SUBS D,{#}S {WC/WZ/WCZ} | Subtract S from D, signed. D = D - S. C = correct sign of (D - S). * |
| 17 | SUBSX D,{#}S {WC/WZ/WCZ} | Subtract (S + C) from D, signed and extended. D = D - (S + C). C = correct sign of (D - (S + C)). Z = Z AND (result == 0). |
| 18 | CMP D,{#}S {WC/WZ/WCZ} | Compare D to S. C = borrow of (D - S). Z = (D == S). |
| 19 | CMPX D,{#}S {WC/WZ/WCZ} | Compare D to (S + C), extended. C = borrow of (D - (S + C)). Z = Z AND (D == S + C). |
| 20 | CMPS D,{#}S {WC/WZ/WCZ} | Compare D to S, signed. C = correct sign of (D - S). Z = (D == S). |
| 21 | CMPSX D,{#}S {WC/WZ/WCZ} | Compare D to (S + C), signed and extended. C = correct sign of (D - (S + C)). Z = Z AND (D == S + C). |
| 22 | CMPR D,{#}S {WC/WZ/WCZ} | Compare S to D (reverse). C = borrow of (S - D). Z = (D == S). |
| 23 | CMPM D,{#}S {WC/WZ/WCZ} | Compare D to S, get MSB of difference into C. C = MSB of (D - S). Z = (D == S). |
| 24 | SUBR D,{#}S {WC/WZ/WCZ} | Subtract D from S (reverse). D = S - D. C = borrow of (S - D). * |
| 25 | CMPSUB D,{#}S {WC/WZ/WCZ} | Compare and subtract S from D if D >= S. If D => S then D = D - S and C = 1, else D same and C = 0. * |
| 26 | FGE D,{#}S {WC/WZ/WCZ} | Force D >= S. If D < S then D = S and C = 1, else D same and C = 0. * |
| 27 | FLE D,{#}S {WC/WZ/WCZ} | Force D <= S. If D > S then D = S and C = 1, else D same and C = 0. * |
| 28 | FGES D,{#}S {WC/WZ/WCZ} | Force D >= S, signed. If D < S then D = S and C = 1, else D same and C = 0. * |
| 29 | FLES D,{#}S {WC/WZ/WCZ} | Force D <= S, signed. If D > S then D = S and C = 1, else D same and C = 0. * |
| 30 | SUMC D,{#}S {WC/WZ/WCZ} | Sum +/-S into D by C. If C = 1 then D = D - S, else D = D + S. C = correct sign of (D +/- S). * |
| 31 | SUMNC D,{#}S {WC/WZ/WCZ} | Sum +/-S into D by !C. If C = 0 then D = D - S, else D = D + S. C = correct sign of (D +/- S). * |
| 32 | SUMZ D,{#}S {WC/WZ/WCZ} | Sum +/-S into D by Z. If Z = 1 then D = D - S, else D = D + S. C = correct sign of (D +/- S). * |
| 33 | SUMNZ D,{#}S {WC/WZ/WCZ} | Sum +/-S into D by !Z. If Z = 0 then D = D - S, else D = D + S. C = correct sign of (D +/- S). * |
| 34 | TESTB D,{#}S WC/WZ | Test bit S[4:0] of D, write to C/Z. C/Z = D[S[4:0]]. |
| 35 | TESTBN D,{#}S WC/WZ | Test bit S[4:0] of !D, write to C/Z. C/Z = !D[S[4:0]]. |
| 36 | TESTB D,{#}S ANDC/ANDZ | Test bit S[4:0] of D, AND into C/Z. C/Z = C/Z AND D[S[4:0]]. |
| 37 | TESTBN D,{#}S ANDC/ANDZ | Test bit S[4:0] of !D, AND into C/Z. C/Z = C/Z AND !D[S[4:0]]. |
| 38 | TESTB D,{#}S ORC/ORZ | Test bit S[4:0] of D, OR into C/Z. C/Z = C/Z OR D[S[4:0]]. |
| 39 | TESTBN D,{#}S ORC/ORZ | Test bit S[4:0] of !D, OR into C/Z. C/Z = C/Z OR !D[S[4:0]]. |
| 40 | TESTB D,{#}S XORC/XORZ | Test bit S[4:0] of D, XOR into C/Z. C/Z = C/Z XOR D[S[4:0]]. |
| 41 | TESTBN D,{#}S XORC/XORZ | Test bit S[4:0] of !D, XOR into C/Z. C/Z = C/Z XOR !D[S[4:0]]. |
| 42 | BITL D,{#}S {WCZ} | Bits D[S[9:5]+S[4:0]:S[4:0]] = 0. Other bits unaffected. Prior SETQ overrides S[9:5]. C,Z = original D[S[4:0]]. |
| 43 | BITH D,{#}S {WCZ} | Bits D[S[9:5]+S[4:0]:S[4:0]] = 1. Other bits unaffected. Prior SETQ overrides S[9:5]. C,Z = original D[S[4:0]]. |
| 44 | BITC D,{#}S {WCZ} | Bits D[S[9:5]+S[4:0]:S[4:0]] = C. Other bits unaffected. Prior SETQ overrides S[9:5]. C,Z = original D[S[4:0]]. |
| 45 | BITNC D,{#}S {WCZ} | Bits D[S[9:5]+S[4:0]:S[4:0]] = !C. Other bits unaffected. Prior SETQ overrides S[9:5]. C,Z = original D[S[4:0]]. |
| 46 | BITZ D,{#}S {WCZ} | Bits D[S[9:5]+S[4:0]:S[4:0]] = Z. Other bits unaffected. Prior SETQ overrides S[9:5]. C,Z = original D[S[4:0]]. |
| 47 | BITNZ D,{#}S {WCZ} | Bits D[S[9:5]+S[4:0]:S[4:0]] = !Z. Other bits unaffected. Prior SETQ overrides S[9:5]. C,Z = original D[S[4:0]]. |
| 48 | BITRND D,{#}S {WCZ} | Bits D[S[9:5]+S[4:0]:S[4:0]] = RNDs. Other bits unaffected. Prior SETQ overrides S[9:5]. C,Z = original D[S[4:0]]. |
| 49 | BITNOT D,{#}S {WCZ} | Toggle bits D[S[9:5]+S[4:0]:S[4:0]]. Other bits unaffected. Prior SETQ overrides S[9:5]. C,Z = original D[S[4:0]]. |
| 50 | AND D,{#}S {WC/WZ/WCZ} | AND S into D. D = D & S. C = parity of result. * |
| 51 | ANDN D,{#}S {WC/WZ/WCZ} | AND !S into D. D = D & !S. C = parity of result. * |
| 52 | OR D,{#}S {WC/WZ/WCZ} | OR S into D. D = D ¦ S. C = parity of result. * |
| 53 | XOR D,{#}S {WC/WZ/WCZ} | XOR S into D. D = D ^ S. C = parity of result. * |
| 54 | MUXC D,{#}S {WC/WZ/WCZ} | Mux C into each D bit that is '1' in S. D = (!S & D ) ¦ (S & {32{ C}}). C = parity of result. * |
| 55 | MUXNC D,{#}S {WC/WZ/WCZ} | Mux !C into each D bit that is '1' in S. D = (!S & D ) ¦ (S & {32{!C}}). C = parity of result. * |
| 56 | MUXZ D,{#}S {WC/WZ/WCZ} | Mux Z into each D bit that is '1' in S. D = (!S & D ) ¦ (S & {32{ Z}}). C = parity of result. * |
| 57 | MUXNZ D,{#}S {WC/WZ/WCZ} | Mux !Z into each D bit that is '1' in S. D = (!S & D ) ¦ (S & {32{!Z}}). C = parity of result. * |
| 58 | MOV D,{#}S {WC/WZ/WCZ} | Move S into D. D = S. C = S[31]. * |
| 59 | NOT D,{#}S {WC/WZ/WCZ} | Get !S into D. D = !S. C = !S[31]. * |
| 60 | NOT D {WC/WZ/WCZ} | Get !D into D. D = !D. C = !D[31]. * |
| 61 | ABS D,{#}S {WC/WZ/WCZ} | Get absolute value of S into D. D = ABS(S). C = S[31]. * |
| 62 | ABS D {WC/WZ/WCZ} | Get absolute value of D into D. D = ABS(D). C = D[31]. * |
| 63 | NEG D,{#}S {WC/WZ/WCZ} | Negate S into D. D = -S. C = MSB of result. * |
| 64 | NEG D {WC/WZ/WCZ} | Negate D. D = -D. C = MSB of result. * |
| 65 | NEGC D,{#}S {WC/WZ/WCZ} | Negate S by C into D. If C = 1 then D = -S, else D = S. C = MSB of result. * |
| 66 | NEGC D {WC/WZ/WCZ} | Negate D by C. If C = 1 then D = -D, else D = D. C = MSB of result. * |
| 67 | NEGNC D,{#}S {WC/WZ/WCZ} | Negate S by !C into D. If C = 0 then D = -S, else D = S. C = MSB of result. * |
| 68 | NEGNC D {WC/WZ/WCZ} | Negate D by !C. If C = 0 then D = -D, else D = D. C = MSB of result. * |
| 69 | NEGZ D,{#}S {WC/WZ/WCZ} | Negate S by Z into D. If Z = 1 then D = -S, else D = S. C = MSB of result. * |
| 70 | NEGZ D {WC/WZ/WCZ} | Negate D by Z. If Z = 1 then D = -D, else D = D. C = MSB of result. * |
| 71 | NEGNZ D,{#}S {WC/WZ/WCZ} | Negate S by !Z into D. If Z = 0 then D = -S, else D = S. C = MSB of result. * |
| 72 | NEGNZ D {WC/WZ/WCZ} | Negate D by !Z. If Z = 0 then D = -D, else D = D. C = MSB of result. * |
| 73 | INCMOD D,{#}S {WC/WZ/WCZ} | Increment with modulus. If D = S then D = 0 and C = 1, else D = D + 1 and C = 0. * |
| 74 | DECMOD D,{#}S {WC/WZ/WCZ} | Decrement with modulus. If D = 0 then D = S and C = 1, else D = D - 1 and C = 0. * |
| 75 | ZEROX D,{#}S {WC/WZ/WCZ} | Zero-extend D above bit S[4:0]. C = MSB of result. * |
| 76 | SIGNX D,{#}S {WC/WZ/WCZ} | Sign-extend D from bit S[4:0]. C = MSB of result. * |
| 77 | ENCOD D,{#}S {WC/WZ/WCZ} | Get bit position of top-most '1' in S into D. D = position of top '1' in S (0..31). C = (S != 0). * |
| 78 | ENCOD D {WC/WZ/WCZ} | Get bit position of top-most '1' in D into D. D = position of top '1' in S (0..31). C = (S != 0). * |
| 79 | ONES D,{#}S {WC/WZ/WCZ} | Get number of '1's in S into D. D = number of '1's in S (0..32). C = LSB of result. * |
| 80 | ONES D {WC/WZ/WCZ} | Get number of '1's in D into D. D = number of '1's in S (0..32). C = LSB of result. * |
| 81 | TEST D,{#}S {WC/WZ/WCZ} | Test D with S. C = parity of (D & S). Z = ((D & S) == 0). |
| 82 | TEST D {WC/WZ/WCZ} | Test D. C = parity of D. Z = (D == 0). |
| 83 | TESTN D,{#}S {WC/WZ/WCZ} | Test D with !S. C = parity of (D & !S). Z = ((D & !S) == 0). |
| 84 | SETNIB D,{#}S,#N | Set S[3:0] into nibble N in D, keeping rest of D same. |
| 85 | SETNIB {#}S | Set S[3:0] into nibble established by prior ALTSN instruction. |
| 86 | GETNIB D,{#}S,#N | Get nibble N of S into D. D = {28'b0, S.NIBBLE[N]). |
| 87 | GETNIB D | Get nibble established by prior ALTGN instruction into D. |
| 88 | ROLNIB D,{#}S,#N | Rotate-left nibble N of S into D. D = {D[27:0], S.NIBBLE[N]). |
| 89 | ROLNIB D | Rotate-left nibble established by prior ALTGN instruction into D. |
| 90 | SETBYTE D,{#}S,#N | Set S[7:0] into byte N in D, keeping rest of D same. |
| 91 | SETBYTE {#}S | Set S[7:0] into byte established by prior ALTSB instruction. |
| 92 | GETBYTE D,{#}S,#N | Get byte N of S into D. D = {24'b0, S.BYTE[N]). |
| 93 | GETBYTE D | Get byte established by prior ALTGB instruction into D. |
| 94 | ROLBYTE D,{#}S,#N | Rotate-left byte N of S into D. D = {D[23:0], S.BYTE[N]). |
| 95 | ROLBYTE D | Rotate-left byte established by prior ALTGB instruction into D. |
| 96 | SETWORD D,{#}S,#N | Set S[15:0] into word N in D, keeping rest of D same. |
| 97 | SETWORD {#}S | Set S[15:0] into word established by prior ALTSW instruction. |
| 98 | GETWORD D,{#}S,#N | Get word N of S into D. D = {16'b0, S.WORD[N]). |
| 99 | GETWORD D | Get word established by prior ALTGW instruction into D. |
| 100 | ROLWORD D,{#}S,#N | Rotate-left word N of S into D. D = {D[15:0], S.WORD[N]). |
| 101 | ROLWORD D | Rotate-left word established by prior ALTGW instruction into D. |
| 102 | ALTSN D,{#}S | Alter subsequent SETNIB instruction. Next D field = (D[11:3] + S) & $1FF, N field = D[2:0]. D += sign-extended S[17:9]. |
| 103 | ALTSN D | Alter subsequent SETNIB instruction. Next D field = D[11:3], N field = D[2:0]. |
| 104 | ALTGN D,{#}S | Alter subsequent GETNIB/ROLNIB instruction. Next S field = (D[11:3] + S) & $1FF, N field = D[2:0]. D += sign-extended S[17:9]. |
| 105 | ALTGN D | Alter subsequent GETNIB/ROLNIB instruction. Next S field = D[11:3], N field = D[2:0]. |
| 106 | ALTSB D,{#}S | Alter subsequent SETBYTE instruction. Next D field = (D[10:2] + S) & $1FF, N field = D[1:0]. D += sign-extended S[17:9]. |
| 107 | ALTSB D | Alter subsequent SETBYTE instruction. Next D field = D[10:2], N field = D[1:0]. |
| 108 | ALTGB D,{#}S | Alter subsequent GETBYTE/ROLBYTE instruction. Next S field = (D[10:2] + S) & $1FF, N field = D[1:0]. D += sign-extended S[17:9]. |
| 109 | ALTGB D | Alter subsequent GETBYTE/ROLBYTE instruction. Next S field = D[10:2], N field = D[1:0]. |
| 110 | ALTSW D,{#}S | Alter subsequent SETWORD instruction. Next D field = (D[9:1] + S) & $1FF, N field = D[0]. D += sign-extended S[17:9]. |
| 111 | ALTSW D | Alter subsequent SETWORD instruction. Next D field = D[9:1], N field = D[0]. |
| 112 | ALTGW D,{#}S | Alter subsequent GETWORD/ROLWORD instruction. Next S field = ((D[9:1] + S) & $1FF), N field = D[0]. D += sign-extended S[17:9]. |
| 113 | ALTGW D | Alter subsequent GETWORD/ROLWORD instruction. Next S field = D[9:1], N field = D[0]. |
| 114 | ALTR D,{#}S | Alter result register address (normally D field) of next instruction to (D + S) & $1FF. D += sign-extended S[17:9]. |
| 115 | ALTR D | Alter result register address (normally D field) of next instruction to D[8:0]. |
| 116 | ALTD D,{#}S | Alter D field of next instruction to (D + S) & $1FF. D += sign-extended S[17:9]. |
| 117 | ALTD D | Alter D field of next instruction to D[8:0]. |
| 118 | ALTS D,{#}S | Alter S field of next instruction to (D + S) & $1FF. D += sign-extended S[17:9]. |
| 119 | ALTS D | Alter S field of next instruction to D[8:0]. |
| 120 | ALTB D,{#}S | Alter D field of next instruction to (D[13:5] + S) & $1FF. D += sign-extended S[17:9]. |
| 121 | ALTB D | Alter D field of next instruction to D[13:5]. |
| 122 | ALTI D,{#}S | Substitute next instruction's I/R/D/S fields with fields from D, per S. Modify D per S. |
| 123 | ALTI D | Execute D in place of next instruction. D stays same. |
| 124 | SETR D,{#}S | Set R field of D to S[8:0]. D = {D[31:28], S[8:0], D[18:0]}. |
| 125 | SETD D,{#}S | Set D field of D to S[8:0]. D = {D[31:18], S[8:0], D[8:0]}. |
| 126 | SETS D,{#}S | Set S field of D to S[8:0]. D = {D[31:9], S[8:0]}. |
| 127 | DECOD D,{#}S | Decode S[4:0] into D. D = 1 << S[4:0]. |
| 128 | DECOD D | Decode D[4:0] into D. D = 1 << D[4:0]. |
| 129 | BMASK D,{#}S | Get LSB-justified bit mask of size (S[4:0] + 1) into D. D = ($0000_0002 << S[4:0]) - 1. |
| 130 | BMASK D | Get LSB-justified bit mask of size (D[4:0] + 1) into D. D = ($0000_0002 << D[4:0]) - 1. |
| 131 | CRCBIT D,{#}S | Iterate CRC value in D using C and polynomial in S. If (C XOR D[0]) then D = (D >> 1) XOR S, else D = (D >> 1). |
| 132 | CRCNIB D,{#}S | Iterate CRC value in D using Q[31:28] and polynomial in S. Like CRCBIT x 4. Q = Q << 4. Use 'REP #n,#1'+SETQ+CRCNIB+CRCNIB+CRCNIB... |
| 133 | MUXNITS D,{#}S | For each non-zero bit pair in S, copy that bit pair into the corresponding D bits, else leave that D bit pair the same. |
| 134 | MUXNIBS D,{#}S | For each non-zero nibble in S, copy that nibble into the corresponding D nibble, else leave that D nibble the same. |
| 135 | MUXQ D,{#}S | Used after SETQ. For each '1' bit in Q, copy the corresponding bit in S into D. D = (D & !Q) ¦ (S & Q). |
| 136 | MOVBYTS D,{#}S | Move bytes within D, per S. D = {D.BYTE[S[7:6]], D.BYTE[S[5:4]], D.BYTE[S[3:2]], D.BYTE[S[1:0]]}. |
| 137 | MUL D,{#}S {WZ} | D = unsigned (D[15:0] * S[15:0]). Z = (S == 0) ¦ (D == 0). |
| 138 | MULS D,{#}S {WZ} | D = signed (D[15:0] * S[15:0]). Z = (S == 0) ¦ (D == 0). |
| 139 | SCA D,{#}S {WZ} | Next instruction's S value = unsigned (D[15:0] * S[15:0]) >> 16. * |
| 140 | SCAS D,{#}S {WZ} | Next instruction's S value = signed (D[15:0] * S[15:0]) >> 14. In this scheme, $4000 = 1.0 and $C000 = -1.0. * |
| 141 | ADDPIX D,{#}S | Add bytes of S into bytes of D, with $FF saturation. |
| 142 | MULPIX D,{#}S | Multiply bytes of S into bytes of D, where $FF = 1.0 and $00 = 0.0. |
| 143 | BLNPIX D,{#}S | Alpha-blend bytes of S into bytes of D, using SETPIV value. |
| 144 | MIXPIX D,{#}S | Mix bytes of S into bytes of D, using SETPIX and SETPIV values. |
| 145 | ADDCT1 D,{#}S | Set CT1 event to trigger on CT = D + S. Adds S into D. |
| 146 | ADDCT2 D,{#}S | Set CT2 event to trigger on CT = D + S. Adds S into D. |
| 147 | ADDCT3 D,{#}S | Set CT3 event to trigger on CT = D + S. Adds S into D. |
| 148 | WMLONG D,{#}S/P | Write only non-$00 bytes in D[31:0] to hub address {#}S/PTRx. Prior SETQ/SETQ2 invokes cog/LUT block transfer. |
| 149 | RQPIN D,{#}S {WC} | Read smart pin S[5:0] result "Z" into D, don't acknowledge smart pin ("Q" in RQPIN means "quiet"). C = modal result. |
| 150 | RDPIN D,{#}S {WC} | Read smart pin S[5:0] result "Z" into D, acknowledge smart pin. C = modal result. |
| 151 | RDLUT D,{#}S/P {WC/WZ/WCZ} | Read data from LUT address {#}S/PTRx into D. C = MSB of data. * |
| 152 | RDBYTE D,{#}S/P {WC/WZ/WCZ} | Read zero-extended byte from hub address {#}S/PTRx into D. C = MSB of byte. * |
| 153 | RDWORD D,{#}S/P {WC/WZ/WCZ} | Read zero-extended word from hub address {#}S/PTRx into D. C = MSB of word. * |
| 154 | RDLONG D,{#}S/P {WC/WZ/WCZ} | Read long from hub address {#}S/PTRx into D. C = MSB of long. * Prior SETQ/SETQ2 invokes cog/LUT block transfer. |
| 155 | POPA D {WC/WZ/WCZ} | Read long from hub address --PTRA into D. C = MSB of long. * |
| 156 | POPB D {WC/WZ/WCZ} | Read long from hub address --PTRB into D. C = MSB of long. * |
| 157 | CALLD D,{#}S {WC/WZ/WCZ} | Call to S** by writing {C, Z, 10'b0, PC[19:0]} to D. C = S[31], Z = S[30]. |
| 158 | RESI3 | Resume from INT3. (CALLD $1F0,$1F1 WCZ) |
| 159 | RESI2 | Resume from INT2. (CALLD $1F2,$1F3 WCZ) |
| 160 | RESI1 | Resume from INT1. (CALLD $1F4,$1F5 WCZ) |
| 161 | RESI0 | Resume from INT0. (CALLD $1FE,$1FF WCZ) |
| 162 | RETI3 | Return from INT3. (CALLD $1FF,$1F1 WCZ) |
| 163 | RETI2 | Return from INT2. (CALLD $1FF,$1F3 WCZ) |
| 164 | RETI1 | Return from INT1. (CALLD $1FF,$1F5 WCZ) |
| 165 | RETI0 | Return from INT0. (CALLD $1FF,$1FF WCZ) |
| 166 | CALLPA {#}D,{#}S | Call to S** by pushing {C, Z, 10'b0, PC[19:0]} onto stack, copy D to PA. |
| 167 | CALLPB {#}D,{#}S | Call to S** by pushing {C, Z, 10'b0, PC[19:0]} onto stack, copy D to PB. |
| 168 | DJZ D,{#}S | Decrement D and jump to S** if result is zero. |
| 169 | DJNZ D,{#}S | Decrement D and jump to S** if result is not zero. |
| 170 | DJF D,{#}S | Decrement D and jump to S** if result is $FFFF_FFFF. |
| 171 | DJNF D,{#}S | Decrement D and jump to S** if result is not $FFFF_FFFF. |
| 172 | IJZ D,{#}S | Increment D and jump to S** if result is zero. |
| 173 | IJNZ D,{#}S | Increment D and jump to S** if result is not zero. |
| 174 | TJZ D,{#}S | Test D and jump to S** if D is zero. |
| 175 | TJNZ D,{#}S | Test D and jump to S** if D is not zero. |
| 176 | TJF D,{#}S | Test D and jump to S** if D is full (D = $FFFF_FFFF). |
| 177 | TJNF D,{#}S | Test D and jump to S** if D is not full (D != $FFFF_FFFF). |
| 178 | TJS D,{#}S | Test D and jump to S** if D is signed (D[31] = 1). |
| 179 | TJNS D,{#}S | Test D and jump to S** if D is not signed (D[31] = 0). |
| 180 | TJV D,{#}S | Test D and jump to S** if D overflowed (D[31] != C, C = 'correct sign' from last addition/subtraction). |
| 181 | JINT {#}S | Jump to S** if INT event flag is set. |
| 182 | JCT1 {#}S | Jump to S** if CT1 event flag is set. |
| 183 | JCT2 {#}S | Jump to S** if CT2 event flag is set. |
| 184 | JCT3 {#}S | Jump to S** if CT3 event flag is set. |
| 185 | JSE1 {#}S | Jump to S** if SE1 event flag is set. |
| 186 | JSE2 {#}S | Jump to S** if SE2 event flag is set. |
| 187 | JSE3 {#}S | Jump to S** if SE3 event flag is set. |
| 188 | JSE4 {#}S | Jump to S** if SE4 event flag is set. |
| 189 | JPAT {#}S | Jump to S** if PAT event flag is set. |
| 190 | JFBW {#}S | Jump to S** if FBW event flag is set. |
| 191 | JXMT {#}S | Jump to S** if XMT event flag is set. |
| 192 | JXFI {#}S | Jump to S** if XFI event flag is set. |
| 193 | JXRO {#}S | Jump to S** if XRO event flag is set. |
| 194 | JXRL {#}S | Jump to S** if XRL event flag is set. |
| 195 | JATN {#}S | Jump to S** if ATN event flag is set. |
| 196 | JQMT {#}S | Jump to S** if QMT event flag is set. |
| 197 | JNINT {#}S | Jump to S** if INT event flag is clear. |
| 198 | JNCT1 {#}S | Jump to S** if CT1 event flag is clear. |
| 199 | JNCT2 {#}S | Jump to S** if CT2 event flag is clear. |
| 200 | JNCT3 {#}S | Jump to S** if CT3 event flag is clear. |
| 201 | JNSE1 {#}S | Jump to S** if SE1 event flag is clear. |
| 202 | JNSE2 {#}S | Jump to S** if SE2 event flag is clear. |
| 203 | JNSE3 {#}S | Jump to S** if SE3 event flag is clear. |
| 204 | JNSE4 {#}S | Jump to S** if SE4 event flag is clear. |
| 205 | JNPAT {#}S | Jump to S** if PAT event flag is clear. |
| 206 | JNFBW {#}S | Jump to S** if FBW event flag is clear. |
| 207 | JNXMT {#}S | Jump to S** if XMT event flag is clear. |
| 208 | JNXFI {#}S | Jump to S** if XFI event flag is clear. |
| 209 | JNXRO {#}S | Jump to S** if XRO event flag is clear. |
| 210 | JNXRL {#}S | Jump to S** if XRL event flag is clear. |
| 211 | JNATN {#}S | Jump to S** if ATN event flag is clear. |
| 212 | JNQMT {#}S | Jump to S** if QMT event flag is clear. |
| 213 | <empty> {#}D,{#}S | <empty> |
| 214 | <empty> {#}D,{#}S | <empty> |
| 215 | SETPAT {#}D,{#}S | Set pin pattern for PAT event. C selects INA/INB, Z selects =/!=, D provides mask value, S provides match value. |
| 216 | AKPIN {#}S | Acknowledge smart pins S[10:6]+S[5:0]..S[5:0]. Wraps within A/B pins. Prior SETQ overrides S[10:6]. |
| 217 | WRPIN {#}D,{#}S | Set mode of smart pins S[10:6]+S[5:0]..S[5:0] to D, acknowledge smart pins. Wraps within A/B pins. Prior SETQ overrides S[10:6]. |
| 218 | WXPIN {#}D,{#}S | Set "X" of smart pins S[10:6]+S[5:0]..S[5:0] to D, acknowledge smart pins. Wraps within A/B pins. Prior SETQ overrides S[10:6]. |
| 219 | WYPIN {#}D,{#}S | Set "Y" of smart pins S[10:6]+S[5:0]..S[5:0] to D, acknowledge smart pins. Wraps within A/B pins. Prior SETQ overrides S[10:6]. |
| 220 | WRLUT {#}D,{#}S/P | Write D to LUT address {#}S/PTRx. |
| 221 | WRBYTE {#}D,{#}S/P | Write byte in D[7:0] to hub address {#}S/PTRx. |
| 222 | WRWORD {#}D,{#}S/P | Write word in D[15:0] to hub address {#}S/PTRx. |
| 223 | WRLONG {#}D,{#}S/P | Write long in D[31:0] to hub address {#}S/PTRx. Prior SETQ/SETQ2 invokes cog/LUT block transfer. |
| 224 | PUSHA {#}D | Write long in D[31:0] to hub address PTRA++. |
| 225 | PUSHB {#}D | Write long in D[31:0] to hub address PTRB++. |
| 226 | RDFAST {#}D,{#}S | Begin new fast hub read via FIFO. D[31] = no wait, D[13:0] = block size in 64-byte units (0 = max), S[19:0] = block start address. |
| 227 | WRFAST {#}D,{#}S | Begin new fast hub write via FIFO. D[31] = no wait, D[13:0] = block size in 64-byte units (0 = max), S[19:0] = block start address. |
| 228 | FBLOCK {#}D,{#}S | Set next block for when block wraps. D[13:0] = block size in 64-byte units (0 = max), S[19:0] = block start address. |
| 229 | XINIT {#}D,{#}S | Issue streamer command immediately, zeroing phase. |
| 230 | XSTOP | Stop streamer immediately. |
| 231 | XZERO {#}D,{#}S | Buffer new streamer command to be issued on final NCO rollover of current command, zeroing phase. |
| 232 | XCONT {#}D,{#}S | Buffer new streamer command to be issued on final NCO rollover of current command, continuing phase. |
| 233 | REP {#}D,{#}S | Execute next D[8:0] instructions S times. If S = 0, repeat instructions infinitely. If D[8:0] = 0, nothing repeats. |
| 234 | COGINIT {#}D,{#}S {WC} | Start cog selected by D. S[19:0] sets hub startup address and PTRB of cog. Prior SETQ sets PTRA of cog. |
| 235 | QMUL {#}D,{#}S | Begin CORDIC unsigned multiplication of D * S. GETQX/GETQY retrieves lower/upper product. |
| 236 | QDIV {#}D,{#}S | Begin CORDIC unsigned division of {SETQ value or 32'b0, D} / S. GETQX/GETQY retrieves quotient/remainder. |
| 237 | QFRAC {#}D,{#}S | Begin CORDIC unsigned division of {D, SETQ value or 32'b0} / S. GETQX/GETQY retrieves quotient/remainder. |
| 238 | QSQRT {#}D,{#}S | Begin CORDIC square root of {S, D}. GETQX retrieves root. |
| 239 | QROTATE {#}D,{#}S | Begin CORDIC rotation of point (D, SETQ value or 32'b0) by angle S. GETQX/GETQY retrieves X/Y. |
| 240 | QVECTOR {#}D,{#}S | Begin CORDIC vectoring of point (D, S). GETQX/GETQY retrieves length/angle. |
| 241 | HUBSET {#}D | Set hub configuration to D. |
| 242 | COGID {#}D {WC} | If D is register and no WC, get cog ID (0 to 15) into D. If WC, check status of cog D[3:0], C = 1 if on. |
| 243 | COGSTOP {#}D | Stop cog D[3:0]. |
| 244 | LOCKNEW D {WC} | Request a LOCK. D will be written with the LOCK number (0 to 15). C = 1 if no LOCK available. |
| 245 | LOCKRET {#}D | Return LOCK D[3:0] for reallocation. |
| 246 | LOCKTRY {#}D {WC} | Try to get LOCK D[3:0]. C = 1 if got LOCK. LOCKREL releases LOCK. LOCK is also released if owner cog stops or restarts. |
| 247 | LOCKREL {#}D {WC} | Release LOCK D[3:0]. If D is a register and WC, get current/last cog id of LOCK owner into D and LOCK status into C. |
| 248 | QLOG {#}D | Begin CORDIC number-to-logarithm conversion of D. GETQX retrieves log {5'whole_exponent, 27'fractional_exponent}. |
| 249 | QEXP {#}D | Begin CORDIC logarithm-to-number conversion of D. GETQX retrieves number. |
| 250 | RFBYTE D {WC/WZ/WCZ} | Used after RDFAST. Read zero-extended byte from FIFO into D. C = MSB of byte. * |
| 251 | RFWORD D {WC/WZ/WCZ} | Used after RDFAST. Read zero-extended word from FIFO into D. C = MSB of word. * |
| 252 | RFLONG D {WC/WZ/WCZ} | Used after RDFAST. Read long from FIFO into D. C = MSB of long. * |
| 253 | RFVAR D {WC/WZ/WCZ} | Used after RDFAST. Read zero-extended 1..4-byte value from FIFO into D. C = 0. * |
| 254 | RFVARS D {WC/WZ/WCZ} | Used after RDFAST. Read sign-extended 1..4-byte value from FIFO into D. C = MSB of value. * |
| 255 | WFBYTE {#}D | Used after WRFAST. Write byte in D[7:0] into FIFO. |
| 256 | WFWORD {#}D | Used after WRFAST. Write word in D[15:0] into FIFO. |
| 257 | WFLONG {#}D | Used after WRFAST. Write long in D[31:0] into FIFO. |
| 258 | GETQX D {WC/WZ/WCZ} | Retrieve CORDIC result X into D. Waits, in case result not ready. C = X[31]. * |
| 259 | GETQY D {WC/WZ/WCZ} | Retrieve CORDIC result Y into D. Waits, in case result not ready. C = Y[31]. * |
| 260 | GETCT D {WC} | Get CT[31:0] or CT[63:32] if WC into D. GETCT WC + GETCT gets full CT. CT=0 on reset, CT++ on every clock. C = same. |
| 261 | GETRND D {WC/WZ/WCZ} | Get RND into D/C/Z. RND is the PRNG that updates on every clock. D = RND[31:0], C = RND[31], Z = RND[30], unique per cog. |
| 262 | GETRND WC/WZ/WCZ | Get RND into C/Z. C = RND[31], Z = RND[30], unique per cog. |
| 263 | SETDACS {#}D | DAC3 = D[31:24], DAC2 = D[23:16], DAC1 = D[15:8], DAC0 = D[7:0]. |
| 264 | SETXFRQ {#}D | Set streamer NCO frequency to D. |
| 265 | GETXACC D | Get the streamer's Goertzel X accumulator into D and the Y accumulator into the next instruction's S, clear accumulators. |
| 266 | WAITX {#}D {WC/WZ/WCZ} | Wait 2 + D clocks if no WC/WZ/WCZ. If WC/WZ/WCZ, wait 2 + (D & RND) clocks. C/Z = 0. |
| 267 | SETSE1 {#}D | Set SE1 event configuration to D[8:0]. |
| 268 | SETSE2 {#}D | Set SE2 event configuration to D[8:0]. |
| 269 | SETSE3 {#}D | Set SE3 event configuration to D[8:0]. |
| 270 | SETSE4 {#}D | Set SE4 event configuration to D[8:0]. |
| 271 | POLLINT {WC/WZ/WCZ} | Get INT event flag into C/Z, then clear it. |
| 272 | POLLCT1 {WC/WZ/WCZ} | Get CT1 event flag into C/Z, then clear it. |
| 273 | POLLCT2 {WC/WZ/WCZ} | Get CT2 event flag into C/Z, then clear it. |
| 274 | POLLCT3 {WC/WZ/WCZ} | Get CT3 event flag into C/Z, then clear it. |
| 275 | POLLSE1 {WC/WZ/WCZ} | Get SE1 event flag into C/Z, then clear it. |
| 276 | POLLSE2 {WC/WZ/WCZ} | Get SE2 event flag into C/Z, then clear it. |
| 277 | POLLSE3 {WC/WZ/WCZ} | Get SE3 event flag into C/Z, then clear it. |
| 278 | POLLSE4 {WC/WZ/WCZ} | Get SE4 event flag into C/Z, then clear it. |
| 279 | POLLPAT {WC/WZ/WCZ} | Get PAT event flag into C/Z, then clear it. |
| 280 | POLLFBW {WC/WZ/WCZ} | Get FBW event flag into C/Z, then clear it. |
| 281 | POLLXMT {WC/WZ/WCZ} | Get XMT event flag into C/Z, then clear it. |
| 282 | POLLXFI {WC/WZ/WCZ} | Get XFI event flag into C/Z, then clear it. |
| 283 | POLLXRO {WC/WZ/WCZ} | Get XRO event flag into C/Z, then clear it. |
| 284 | POLLXRL {WC/WZ/WCZ} | Get XRL event flag into C/Z, then clear it. |
| 285 | POLLATN {WC/WZ/WCZ} | Get ATN event flag into C/Z, then clear it. |
| 286 | POLLQMT {WC/WZ/WCZ} | Get QMT event flag into C/Z, then clear it. |
| 287 | WAITINT {WC/WZ/WCZ} | Wait for INT event flag, then clear it. Prior SETQ sets optional CT timeout value. C/Z = timeout. |
| 288 | WAITCT1 {WC/WZ/WCZ} | Wait for CT1 event flag, then clear it. Prior SETQ sets optional CT timeout value. C/Z = timeout. |
| 289 | WAITCT2 {WC/WZ/WCZ} | Wait for CT2 event flag, then clear it. Prior SETQ sets optional CT timeout value. C/Z = timeout. |
| 290 | WAITCT3 {WC/WZ/WCZ} | Wait for CT3 event flag, then clear it. Prior SETQ sets optional CT timeout value. C/Z = timeout. |
| 291 | WAITSE1 {WC/WZ/WCZ} | Wait for SE1 event flag, then clear it. Prior SETQ sets optional CT timeout value. C/Z = timeout. |
| 292 | WAITSE2 {WC/WZ/WCZ} | Wait for SE2 event flag, then clear it. Prior SETQ sets optional CT timeout value. C/Z = timeout. |
| 293 | WAITSE3 {WC/WZ/WCZ} | Wait for SE3 event flag, then clear it. Prior SETQ sets optional CT timeout value. C/Z = timeout. |
| 294 | WAITSE4 {WC/WZ/WCZ} | Wait for SE4 event flag, then clear it. Prior SETQ sets optional CT timeout value. C/Z = timeout. |
| 295 | WAITPAT {WC/WZ/WCZ} | Wait for PAT event flag, then clear it. Prior SETQ sets optional CT timeout value. C/Z = timeout. |
| 296 | WAITFBW {WC/WZ/WCZ} | Wait for FBW event flag, then clear it. Prior SETQ sets optional CT timeout value. C/Z = timeout. |
| 297 | WAITXMT {WC/WZ/WCZ} | Wait for XMT event flag, then clear it. Prior SETQ sets optional CT timeout value. C/Z = timeout. |
| 298 | WAITXFI {WC/WZ/WCZ} | Wait for XFI event flag, then clear it. Prior SETQ sets optional CT timeout value. C/Z = timeout. |
| 299 | WAITXRO {WC/WZ/WCZ} | Wait for XRO event flag, then clear it. Prior SETQ sets optional CT timeout value. C/Z = timeout. |
| 300 | WAITXRL {WC/WZ/WCZ} | Wait for XRL event flag, then clear it. Prior SETQ sets optional CT timeout value. C/Z = timeout. |
| 301 | WAITATN {WC/WZ/WCZ} | Wait for ATN event flag, then clear it. Prior SETQ sets optional CT timeout value. C/Z = timeout. |
| 302 | ALLOWI | Allow interrupts (default). |
| 303 | STALLI | Stall Interrupts. |
| 304 | TRGINT1 | Trigger INT1, regardless of STALLI mode. |
| 305 | TRGINT2 | Trigger INT2, regardless of STALLI mode. |
| 306 | TRGINT3 | Trigger INT3, regardless of STALLI mode. |
| 307 | NIXINT1 | Cancel INT1. |
| 308 | NIXINT2 | Cancel INT2. |
| 309 | NIXINT3 | Cancel INT3. |
| 310 | SETINT1 {#}D | Set INT1 source to D[3:0]. |
| 311 | SETINT2 {#}D | Set INT2 source to D[3:0]. |
| 312 | SETINT3 {#}D | Set INT3 source to D[3:0]. |
| 313 | SETQ {#}D | Set Q to D. Use before RDLONG/WRLONG/WMLONG to set block transfer. Also used before MUXQ/COGINIT/QDIV/QFRAC/QROTATE/WAITxxx. |
| 314 | SETQ2 {#}D | Set Q to D. Use before RDLONG/WRLONG/WMLONG to set LUT block transfer. |
| 315 | PUSH {#}D | Push D onto stack. |
| 316 | POP D {WC/WZ/WCZ} | Pop stack (K). D = K. C = K[31]. * |
| 317 | JMP D {WC/WZ/WCZ} | Jump to D. C = D[31], Z = D[30], PC = D[19:0]. |
| 318 | CALL D {WC/WZ/WCZ} | Call to D by pushing {C, Z, 10'b0, PC[19:0]} onto stack. C = D[31], Z = D[30], PC = D[19:0]. |
| 319 | RET {WC/WZ/WCZ} | Return by popping stack (K). C = K[31], Z = K[30], PC = K[19:0]. |
| 320 | CALLA D {WC/WZ/WCZ} | Call to D by writing {C, Z, 10'b0, PC[19:0]} to hub long at PTRA++. C = D[31], Z = D[30], PC = D[19:0]. |
| 321 | RETA {WC/WZ/WCZ} | Return by reading hub long (L) at --PTRA. C = L[31], Z = L[30], PC = L[19:0]. |
| 322 | CALLB D {WC/WZ/WCZ} | Call to D by writing {C, Z, 10'b0, PC[19:0]} to hub long at PTRB++. C = D[31], Z = D[30], PC = D[19:0]. |
| 323 | RETB {WC/WZ/WCZ} | Return by reading hub long (L) at --PTRB. C = L[31], Z = L[30], PC = L[19:0]. |
| 324 | JMPREL {#}D | Jump ahead/back by D instructions. For cogex, PC += D[19:0]. For hubex, PC += D[17:0] << 2. |
| 325 | SKIP {#}D | Skip instructions per D. Subsequent instructions 0..31 get cancelled for each '1' bit in D[0]..D[31]. |
| 326 | SKIPF {#}D | Skip cog/LUT instructions fast per D. Like SKIP, but instead of cancelling instructions, the PC leaps over them. |
| 327 | EXECF {#}D | Jump to D[9:0] in cog/LUT and set SKIPF pattern to D[31:10]. PC = {10'b0, D[9:0]}. |
| 328 | GETPTR D | Get current FIFO hub pointer into D. |
| 329 | GETBRK D WC/WZ/WCZ | Get breakpoint/cog status into D according to WC/WZ/WCZ. See documentation for details. |
| 330 | COGBRK {#}D | If in debug ISR, trigger asynchronous breakpoint in cog D[3:0]. Cog D[3:0] must have asynchronous breakpoint enabled. |
| 331 | BRK {#}D | If in debug ISR, set next break condition to D. Else, set BRK code to D[7:0] and unconditionally trigger BRK interrupt, if enabled. |
| 332 | SETLUTS {#}D | If D[0] = 1 then enable LUT sharing, where LUT writes within the adjacent odd/even companion cog are copied to this cog's LUT. |
| 333 | SETCY {#}D | Set the colorspace converter "CY" parameter to D[31:0]. |
| 334 | SETCI {#}D | Set the colorspace converter "CI" parameter to D[31:0]. |
| 335 | SETCQ {#}D | Set the colorspace converter "CQ" parameter to D[31:0]. |
| 336 | SETCFRQ {#}D | Set the colorspace converter "CFRQ" parameter to D[31:0]. |
| 337 | SETCMOD {#}D | Set the colorspace converter "CMOD" parameter to D[8:0]. |
| 338 | SETPIV {#}D | Set BLNPIX/MIXPIX blend factor to D[7:0]. |
| 339 | SETPIX {#}D | Set MIXPIX mode to D[5:0]. |
| 340 | COGATN {#}D | Strobe "attention" of all cogs whose corresponding bits are high in D[15:0]. |
| 341 | TESTP {#}D WC/WZ | Test IN bit of pin D[5:0], write to C/Z. C/Z = IN[D[5:0]]. |
| 342 | TESTPN {#}D WC/WZ | Test !IN bit of pin D[5:0], write to C/Z. C/Z = !IN[D[5:0]]. |
| 343 | TESTP {#}D ANDC/ANDZ | Test IN bit of pin D[5:0], AND into C/Z. C/Z = C/Z AND IN[D[5:0]]. |
| 344 | TESTPN {#}D ANDC/ANDZ | Test !IN bit of pin D[5:0], AND into C/Z. C/Z = C/Z AND !IN[D[5:0]]. |
| 345 | TESTP {#}D ORC/ORZ | Test IN bit of pin D[5:0], OR into C/Z. C/Z = C/Z OR IN[D[5:0]]. |
| 346 | TESTPN {#}D ORC/ORZ | Test !IN bit of pin D[5:0], OR into C/Z. C/Z = C/Z OR !IN[D[5:0]]. |
| 347 | TESTP {#}D XORC/XORZ | Test IN bit of pin D[5:0], XOR into C/Z. C/Z = C/Z XOR IN[D[5:0]]. |
| 348 | TESTPN {#}D XORC/XORZ | Test !IN bit of pin D[5:0], XOR into C/Z. C/Z = C/Z XOR !IN[D[5:0]]. |
| 349 | DIRL {#}D {WCZ} | DIR bits of pins D[10:6]+D[5:0]..D[5:0] = 0. Wraps within DIRA/DIRB. Prior SETQ overrides D[10:6]. C,Z = DIR bit. |
| 350 | DIRH {#}D {WCZ} | DIR bits of pins D[10:6]+D[5:0]..D[5:0] = 1. Wraps within DIRA/DIRB. Prior SETQ overrides D[10:6]. C,Z = DIR bit. |
| 351 | DIRC {#}D {WCZ} | DIR bits of pins D[10:6]+D[5:0]..D[5:0] = C. Wraps within DIRA/DIRB. Prior SETQ overrides D[10:6]. C,Z = DIR bit. |
| 352 | DIRNC {#}D {WCZ} | DIR bits of pins D[10:6]+D[5:0]..D[5:0] = !C. Wraps within DIRA/DIRB. Prior SETQ overrides D[10:6]. C,Z = DIR bit. |
| 353 | DIRZ {#}D {WCZ} | DIR bits of pins D[10:6]+D[5:0]..D[5:0] = Z. Wraps within DIRA/DIRB. Prior SETQ overrides D[10:6]. C,Z = DIR bit. |
| 354 | DIRNZ {#}D {WCZ} | DIR bits of pins D[10:6]+D[5:0]..D[5:0] = !Z. Wraps within DIRA/DIRB. Prior SETQ overrides D[10:6]. C,Z = DIR bit. |
| 355 | DIRRND {#}D {WCZ} | DIR bits of pins D[10:6]+D[5:0]..D[5:0] = RNDs. Wraps within DIRA/DIRB. Prior SETQ overrides D[10:6]. C,Z = DIR bit. |
| 356 | DIRNOT {#}D {WCZ} | Toggle DIR bits of pins D[10:6]+D[5:0]..D[5:0]. Wraps within DIRA/DIRB. Prior SETQ overrides D[10:6]. C,Z = DIR bit. |
| 357 | OUTL {#}D {WCZ} | OUT bits of pins D[10:6]+D[5:0]..D[5:0] = 0. Wraps within OUTA/OUTB. Prior SETQ overrides D[10:6]. C,Z = OUT bit. |
| 358 | OUTH {#}D {WCZ} | OUT bits of pins D[10:6]+D[5:0]..D[5:0] = 1. Wraps within OUTA/OUTB. Prior SETQ overrides D[10:6]. C,Z = OUT bit. |
| 359 | OUTC {#}D {WCZ} | OUT bits of pins D[10:6]+D[5:0]..D[5:0] = C. Wraps within OUTA/OUTB. Prior SETQ overrides D[10:6]. C,Z = OUT bit. |
| 360 | OUTNC {#}D {WCZ} | OUT bits of pins D[10:6]+D[5:0]..D[5:0] = !C. Wraps within OUTA/OUTB. Prior SETQ overrides D[10:6]. C,Z = OUT bit. |
| 361 | OUTZ {#}D {WCZ} | OUT bits of pins D[10:6]+D[5:0]..D[5:0] = Z. Wraps within OUTA/OUTB. Prior SETQ overrides D[10:6]. C,Z = OUT bit. |
| 362 | OUTNZ {#}D {WCZ} | OUT bits of pins D[10:6]+D[5:0]..D[5:0] = !Z. Wraps within OUTA/OUTB. Prior SETQ overrides D[10:6]. C,Z = OUT bit. |
| 363 | OUTRND {#}D {WCZ} | OUT bits of pins D[10:6]+D[5:0]..D[5:0] = RNDs. Wraps within OUTA/OUTB. Prior SETQ overrides D[10:6]. C,Z = OUT bit. |
| 364 | OUTNOT {#}D {WCZ} | Toggle OUT bits of pins D[10:6]+D[5:0]..D[5:0]. Wraps within OUTA/OUTB. Prior SETQ overrides D[10:6]. C,Z = OUT bit. |
| 365 | FLTL {#}D {WCZ} | OUT bits of pins D[10:6]+D[5:0]..D[5:0] = 0. DIR bits = 0. Wraps within OUTA/OUTB. Prior SETQ overrides D[10:6]. C,Z = OUT bit. |
| 366 | FLTH {#}D {WCZ} | OUT bits of pins D[10:6]+D[5:0]..D[5:0] = 1. DIR bits = 0. Wraps within OUTA/OUTB. Prior SETQ overrides D[10:6]. C,Z = OUT bit. |
| 367 | FLTC {#}D {WCZ} | OUT bits of pins D[10:6]+D[5:0]..D[5:0] = C. DIR bits = 0. Wraps within OUTA/OUTB. Prior SETQ overrides D[10:6]. C,Z = OUT bit. |
| 368 | FLTNC {#}D {WCZ} | OUT bits of pins D[10:6]+D[5:0]..D[5:0] = !C. DIR bits = 0. Wraps within OUTA/OUTB. Prior SETQ overrides D[10:6]. C,Z = OUT bit. |
| 369 | FLTZ {#}D {WCZ} | OUT bits of pins D[10:6]+D[5:0]..D[5:0] = Z. DIR bits = 0. Wraps within OUTA/OUTB. Prior SETQ overrides D[10:6]. C,Z = OUT bit. |
| 370 | FLTNZ {#}D {WCZ} | OUT bits of pins D[10:6]+D[5:0]..D[5:0] = !Z. DIR bits = 0. Wraps within OUTA/OUTB. Prior SETQ overrides D[10:6]. C,Z = OUT bit. |
| 371 | FLTRND {#}D {WCZ} | OUT bits of pins D[10:6]+D[5:0]..D[5:0] = RNDs. DIR bits = 0. Wraps within OUTA/OUTB. Prior SETQ overrides D[10:6]. C,Z = OUT bit. |
| 372 | FLTNOT {#}D {WCZ} | Toggle OUT bits of pins D[10:6]+D[5:0]..D[5:0]. DIR bits = 0. Wraps within OUTA/OUTB. Prior SETQ overrides D[10:6]. C,Z = OUT bit. |
| 373 | DRVL {#}D {WCZ} | OUT bits of pins D[10:6]+D[5:0]..D[5:0] = 0. DIR bits = 1. Wraps within OUTA/OUTB. Prior SETQ overrides D[10:6]. C,Z = OUT bit. |
| 374 | DRVH {#}D {WCZ} | OUT bits of pins D[10:6]+D[5:0]..D[5:0] = 1. DIR bits = 1. Wraps within OUTA/OUTB. Prior SETQ overrides D[10:6]. C,Z = OUT bit. |
| 375 | DRVC {#}D {WCZ} | OUT bits of pins D[10:6]+D[5:0]..D[5:0] = C. DIR bits = 1. Wraps within OUTA/OUTB. Prior SETQ overrides D[10:6]. C,Z = OUT bit. |
| 376 | DRVNC {#}D {WCZ} | OUT bits of pins D[10:6]+D[5:0]..D[5:0] = !C. DIR bits = 1. Wraps within OUTA/OUTB. Prior SETQ overrides D[10:6]. C,Z = OUT bit. |
| 377 | DRVZ {#}D {WCZ} | OUT bits of pins D[10:6]+D[5:0]..D[5:0] = Z. DIR bits = 1. Wraps within OUTA/OUTB. Prior SETQ overrides D[10:6]. C,Z = OUT bit. |
| 378 | DRVNZ {#}D {WCZ} | OUT bits of pins D[10:6]+D[5:0]..D[5:0] = !Z. DIR bits = 1. Wraps within OUTA/OUTB. Prior SETQ overrides D[10:6]. C,Z = OUT bit. |
| 379 | DRVRND {#}D {WCZ} | OUT bits of pins D[10:6]+D[5:0]..D[5:0] = RNDs. DIR bits = 1. Wraps within OUTA/OUTB. Prior SETQ overrides D[10:6]. C,Z = OUT bit. |
| 380 | DRVNOT {#}D {WCZ} | Toggle OUT bits of pins D[10:6]+D[5:0]..D[5:0]. DIR bits = 1. Wraps within OUTA/OUTB. Prior SETQ overrides D[10:6]. C,Z = OUT bit. |
| 381 | SPLITB D | Split every 4th bit of D into bytes. D = {D[31], D[27], D[23], D[19], ...D[12], D[8], D[4], D[0]}. |
| 382 | MERGEB D | Merge bits of bytes in D. D = {D[31], D[23], D[15], D[7], ...D[24], D[16], D[8], D[0]}. |
| 383 | SPLITW D | Split odd/even bits of D into words. D = {D[31], D[29], D[27], D[25], ...D[6], D[4], D[2], D[0]}. |
| 384 | MERGEW D | Merge bits of words in D. D = {D[31], D[15], D[30], D[14], ...D[17], D[1], D[16], D[0]}. |
| 385 | SEUSSF D | Relocate and periodically invert bits within D. Returns to original value on 32nd iteration. Forward pattern. |
| 386 | SEUSSR D | Relocate and periodically invert bits within D. Returns to original value on 32nd iteration. Reverse pattern. |
| 387 | RGBSQZ D | Squeeze 8:8:8 RGB value in D[31:8] into 5:6:5 value in D[15:0]. D = {15'b0, D[31:27], D[23:18], D[15:11]}. |
| 388 | RGBEXP D | Expand 5:6:5 RGB value in D[15:0] into 8:8:8 value in D[31:8]. D = {D[15:11,15:13], D[10:5,10:9], D[4:0,4:2], 8'b0}. |
| 389 | XORO32 D | Iterate D with xoroshiro32+ PRNG algorithm and put PRNG result into next instruction's S. |
| 390 | REV D | Reverse D bits. D = D[0:31]. |
| 391 | RCZR D {WC/WZ/WCZ} | Rotate C,Z right through D. D = {C, Z, D[31:2]}. C = D[1], Z = D[0]. |
| 392 | RCZL D {WC/WZ/WCZ} | Rotate C,Z left through D. D = {D[29:0], C, Z}. C = D[31], Z = D[30]. |
| 393 | WRC D | Write 0 or 1 to D, according to C. D = {31'b0, C). |
| 394 | WRNC D | Write 0 or 1 to D, according to !C. D = {31'b0, !C). |
| 395 | WRZ D | Write 0 or 1 to D, according to Z. D = {31'b0, Z). |
| 396 | WRNZ D | Write 0 or 1 to D, according to !Z. D = {31'b0, !Z). |
| 397 | MODCZ c,z {WC/WZ/WCZ} | Modify C and Z according to cccc and zzzz. C = cccc[{C,Z}], Z = zzzz[{C,Z}]. |
| 398 | MODC c {WC} | Modify C according to cccc. C = cccc[{C,Z}]. |
| 399 | MODZ z {WZ} | Modify Z according to zzzz. Z = zzzz[{C,Z}]. |
| 400 | SETSCP {#}D | Set four-channel oscilloscope enable to D[6] and set input pin base to D[5:2]. |
| 401 | GETSCP D | Get four-channel oscilloscope samples into D. D = {ch3[7:0],ch2[7:0],ch1[7:0],ch0[7:0]}. |
| 402 | JMP #{\}A | Jump to A. If R = 1 then PC += A, else PC = A. "\" forces R = 0. |
| 403 | CALL #{\}A | Call to A by pushing {C, Z, 10'b0, PC[19:0]} onto stack. If R = 1 then PC += A, else PC = A. "\" forces R = 0. |
| 404 | CALLA #{\}A | Call to A by writing {C, Z, 10'b0, PC[19:0]} to hub long at PTRA++. If R = 1 then PC += A, else PC = A. "\" forces R = 0. |
| 405 | CALLB #{\}A | Call to A by writing {C, Z, 10'b0, PC[19:0]} to hub long at PTRB++. If R = 1 then PC += A, else PC = A. "\" forces R = 0. |
| 406 | CALLD PA/PB/PTRA/PTRB,#{\}A | Call to A by writing {C, Z, 10'b0, PC[19:0]} to PA/PB/PTRA/PTRB (per W). If R = 1 then PC += A, else PC = A. "\" forces R = 0. |
| 407 | LOC PA/PB/PTRA/PTRB,#{\}A | Get {12'b0, address[19:0]} into PA/PB/PTRA/PTRB (per W). If R = 1, address = PC + A, else address = A. "\" forces R = 0. |
| 408 | AUGS #n | Queue #n to be used as upper 23 bits for next #S occurrence, so that the next 9-bit #S will be augmented to 32 bits. |
| 409 | AUGD #n | Queue #n to be used as upper 23 bits for next #D occurrence, so that the next 9-bit #D will be augmented to 32 bits. |
| 410 | _RET_ <inst> <ops> | Execute <inst> always and return if no branch. If <inst> is not branching then return by popping stack[19:0] into PC. |
| 411 | IF_NC_AND_NZ <inst> <ops> | Execute <inst> if C = 0 and Z = 0. |
| 412 | IF_NZ_AND_NC <inst> <ops> | Execute <inst> if C = 0 and Z = 0. |
| 413 | IF_GT <inst> <ops> | Execute <inst> if C = 0 and Z = 0, or if 'greater than' after a comparison/subtraction. |
| 414 | IF_A <inst> <ops> | Execute <inst> if C = 0 and Z = 0, or if 'above' after a comparison/subtraction. |
| 415 | IF_00 <inst> <ops> | Execute <inst> if C = 0 and Z = 0. |
| 416 | IF_NC_AND_Z <inst> <ops> | Execute <inst> if C = 0 and Z = 1. |
| 417 | IF_Z_AND_NC <inst> <ops> | Execute <inst> if C = 0 and Z = 1. |
| 418 | IF_01 <inst> <ops> | Execute <inst> if C = 0 and Z = 1. |
| 419 | IF_NC <inst> <ops> | Execute <inst> if C = 0. |
| 420 | IF_GE <inst> <ops> | Execute <inst> if C = 0, or if 'greater than or equal' after a comparison/subtraction. |
| 421 | IF_AE <inst> <ops> | Execute <inst> if C = 0, or if 'above or equal' after a comparison/subtraction. |
| 422 | IF_0X <inst> <ops> | Execute <inst> if C = 0. |
| 423 | IF_C_AND_NZ <inst> <ops> | Execute <inst> if C = 1 and Z = 0. |
| 424 | IF_NZ_AND_C <inst> <ops> | Execute <inst> if C = 1 and Z = 0. |
| 425 | IF_10 <inst> <ops> | Execute <inst> if C = 1 and Z = 0. |
| 426 | IF_NZ <inst> <ops> | Execute <inst> if Z = 0. |
| 427 | IF_NE <inst> <ops> | Execute <inst> if Z = 0, or if 'not equal' after a comparison/subtraction. |
| 428 | IF_X0 <inst> <ops> | Execute <inst> if Z = 0. |
| 429 | IF_C_NE_Z <inst> <ops> | Execute <inst> if C != Z. |
| 430 | IF_Z_NE_C <inst> <ops> | Execute <inst> if C != Z. |
| 431 | IF_DIFF <inst> <ops> | Execute <inst> if C != Z. |
| 432 | IF_NC_OR_NZ <inst> <ops> | Execute <inst> if C = 0 or Z = 0. |
| 433 | IF_NZ_OR_NC <inst> <ops> | Execute <inst> if C = 0 or Z = 0. |
| 434 | IF_NOT_11 <inst> <ops> | Execute <inst> if C = 0 or Z = 0. |
| 435 | IF_C_AND_Z <inst> <ops> | Execute <inst> if C = 1 and Z = 1. |
| 436 | IF_Z_AND_C <inst> <ops> | Execute <inst> if C = 1 and Z = 1. |
| 437 | IF_11 <inst> <ops> | Execute <inst> if C = 1 and Z = 1. |
| 438 | IF_C_EQ_Z <inst> <ops> | Execute <inst> if C = Z. |
| 439 | IF_Z_EQ_C <inst> <ops> | Execute <inst> if C = Z. |
| 440 | IF_SAME <inst> <ops> | Execute <inst> if C = Z. |
| 441 | IF_Z <inst> <ops> | Execute <inst> if Z = 1. |
| 442 | IF_E <inst> <ops> | Execute <inst> if Z = 1, or if 'equal' after a comparison/subtraction. |
| 443 | IF_X1 <inst> <ops> | Execute <inst> if Z = 1. |
| 444 | IF_NC_OR_Z <inst> <ops> | Execute <inst> if C = 0 or Z = 1. |
| 445 | IF_Z_OR_NC <inst> <ops> | Execute <inst> if C = 0 or Z = 1. |
| 446 | IF_NOT_10 <inst> <ops> | Execute <inst> if C = 0 or Z = 1. |
| 447 | IF_C <inst> <ops> | Execute <inst> if C = 1. |
| 448 | IF_LT <inst> <ops> | Execute <inst> if C = 1, or if 'less than' after a comparison/subtraction. |
| 449 | IF_B <inst> <ops> | Execute <inst> if C = 1, or if 'below' after a comparison/subtraction. |
| 450 | IF_1X <inst> <ops> | Execute <inst> if C = 1. |
| 451 | IF_C_OR_NZ <inst> <ops> | Execute <inst> if C = 1 or Z = 0. |
| 452 | IF_NZ_OR_C <inst> <ops> | Execute <inst> if C = 1 or Z = 0. |
| 453 | IF_NOT_01 <inst> <ops> | Execute <inst> if C = 1 or Z = 0. |
| 454 | IF_C_OR_Z <inst> <ops> | Execute <inst> if C = 1 or Z = 1. |
| 455 | IF_Z_OR_C <inst> <ops> | Execute <inst> if C = 1 or Z = 1. |
| 456 | IF_LE <inst> <ops> | Execute <inst> if C = 1 or Z = 1, or if 'less than or equal' after a comparison/subtraction. |
| 457 | IF_BE <inst> <ops> | Execute <inst> if C = 1 or Z = 1, or if 'below or equal' after a comparison/subtraction. |
| 458 | IF_NOT_00 <inst> <ops> | Execute <inst> if C = 1 or Z = 1. |
| 459 | <inst> <ops> | Execute <inst> always. This is the default when no instruction prefix is expressed. |
| 460 | _CLR | C/Z = 0 |
| 461 | _NC_AND_NZ | C/Z = !C AND !Z |
| 462 | _NZ_AND_NC | C/Z = !C AND !Z |
| 463 | _GT | C/Z = !C AND !Z, or 'greater than' after a comparison/subtraction. |
| 464 | _NC_AND_Z | C/Z = !C AND Z |
| 465 | _Z_AND_NC | C/Z = !C AND Z |
| 466 | _NC | C/Z = !C |
| 467 | _GE | C/Z = !C, or 'greater than or equal' after a comparison/subtraction. |
| 468 | _C_AND_NZ | C/Z = C AND !Z |
| 469 | _NZ_AND_C | C/Z = C AND !Z |
| 470 | _NZ | C/Z = !Z |
| 471 | _NE | C/Z = !Z, or 'not equal' after a comparison/subtraction. |
| 472 | _C_NE_Z | C/Z = C NOT_EQUAL_TO Z |
| 473 | _Z_NE_C | C/Z = C NOT_EQUAL_TO Z |
| 474 | _NC_OR_NZ | C/Z = !C OR !Z |
| 475 | _NZ_OR_NC | C/Z = !C OR !Z |
| 476 | _C_AND_Z | C/Z = C AND Z |
| 477 | _Z_AND_C | C/Z = C AND Z |
| 478 | _C_EQ_Z | C/Z = C EQUAL_TO Z |
| 479 | _Z_EQ_C | C/Z = C EQUAL_TO Z |
| 480 | _Z | C/Z = Z |
| 481 | _E | C/Z = Z, or 'equal' after a comparison/subtraction. |
| 482 | _NC_OR_Z | C/Z = !C OR Z |
| 483 | _Z_OR_NC | C/Z = !C OR Z |
| 484 | _C | C/Z = C |
| 485 | _LT | C/Z = C, or 'less than' after a comparison/subtraction. |
| 486 | _C_OR_NZ | C/Z = C OR !Z |
| 487 | _NZ_OR_C | C/Z = C OR !Z |
| 488 | _C_OR_Z | C/Z = C OR Z |
| 489 | _Z_OR_C | C/Z = C OR Z |
| 490 | _LE | C/Z = C OR Z, or 'less than or equal' after a comparison/subtraction. |
| 491 | _SET | C/Z = 1 |
|  |  | C/Z = 1 |