Appendix A: Source Code

The source code below is use to program the robot.

----------------------------------------------------------------------------------------------------------

'{$STAMP BS2}

'{$PBASIC 2.5}

time      VAR    Word(5)                  'declare time as a 16-bits or 2 bytes variable

counter   VAR    Word                     'declare counter as a 16-bits or 2 bytes variable

DO

DO

RCTIME 9, 1, time(0)

LOOP UNTIL time(0) > 1000     'loop until 2ms(the resting state between 2 messages)

PULSIN 9, 0, time(0)

PULSIN 9, 0, time(1)

PULSIN 9, 0, time(2)

PULSIN 9, 0, time(3)

PULSIN 9, 0, time(4)

IF (time(4) < 400) AND (time(3) < 400) AND (time(2) < 400) AND (time(1) < 400) AND (time(0) > 400) THEN

PULSOUT 13, 1000            'digit 2

PULSOUT 12, 500              'forward movement

ELSEIF (time(4) < 400) AND (time(3) < 400) AND (time(2) < 400) AND (time(1) > 400) AND (time(0) > 400) THEN

PULSOUT 13, 500             'digit 4

PULSOUT 12, 500             'left turning

ELSEIF (time(4) < 400) AND (time(3) < 400) AND (time(2) > 400) AND (time(1) < 400) AND (time(0) > 400) THEN

PULSOUT 13, 1000            'digit 6

PULSOUT 12, 1000            'right turning

ELSEIF (time(4) < 400) AND (time(3) < 400) AND (time(2) > 400) AND (time(1) > 400) AND (time(0) > 400) THEN

PULSOUT 13, 500             'digit 8

PULSOUT 12, 1000           'backward movement

ELSEIF (time(4) < 400) AND (time(3) < 400) AND (time(2) < 400) AND (time(1) < 400) AND (time(0) < 400) THEN

PULSOUT 13, 750             'digit 1

PULSOUT 12, 500             'forward pivot left movement

ELSEIF (time(4) < 400) AND (time(3) < 400) AND (time(2) < 400) AND (time(1) > 400) AND (time(0) < 400) THEN

PULSOUT 13, 1000           'digit 3

PULSOUT 12, 750             'forward pivot right movement

ELSEIF (time(4) < 400) AND (time(3) < 400) AND (time(2) > 400) AND (time(1) > 400) AND (time(0) < 400) THEN

PULSOUT 13, 750             'digit 7

PULSOUT 12, 1000           'backward pivot left movement

ELSEIF (time(4) < 400) AND (time(3) > 400) AND (time(2) < 400) AND (time(1) < 400) AND (time(0) < 400) THEN

PULSOUT 13, 500             'digit 9

PULSOUT 12, 750             'backward pivot right movement

ELSEIF (time(4) > 400) AND (time(3) < 400) AND (time(2) < 400) AND (time(1) < 400) AND (time(0) > 400) THEN

FOR counter = 1 TO 100      'program - key

LOW 6                      	  'cruise right

HIGH 7

LOW 4

HIGH 3

NEXT

LOW 7

LOW 3

ELSEIF (time(4) > 400) AND (time(3) < 400) AND (time(2) < 400) AND (time(1) < 400) AND (time(0) < 400) THEN

FOR counter = 1 TO 100      'program + key

LOW 3                                  'cruise left

HIGH 4

LOW 7

HIGH 6

NEXT

LOW 4

LOW 6

ELSEIF (time(4) > 400) AND (time(3) < 400) AND (time(2) < 400) AND (time(1) > 400) AND (time(0) > 400) THEN

FOR counter = 1 TO 100      'vol - key

LOW 7                                  'cruise backward

HIGH 6

LOW 4

HIGH 3

NEXT

LOW 6

LOW 3

ELSEIF (time(4) > 400) AND (time(3) < 400) AND (time(2) < 400) AND (time(1) > 400) AND (time(0) < 400) THEN

FOR counter = 1 TO 100      'vol + key

LOW 6                                  'cruise forward

HIGH 7

LOW 3

HIGH 4

NEXT

LOW 7

LOW 4

ELSEIF (time(4) < 400) AND (time(3) < 400) AND (time(2) > 400) AND (time(1) < 400) AND (time(0) < 400) THEN

PULSOUT 12, 750             'digit 5

PULSOUT 13, 750             'servomotor center point setting or holding the position

PAUSE 20

ENDIF

LOOP

The source code below is to measure the duration of the resting state between 2 messages.

----------------------------------------------------------------------------------------------------------

'{$STAMP BS2}

'{$PBASIC 2.5}

time    		VAR  Word

counter 	VAR  Word

DEBUG 		"Resting State", CR,

"Repetitions  Current Duration", CR,

"___________  ______________"

DO

PULSIN 9, 1, time

IF (time > 1000) THEN

counter = counter + 1

DEBUG CRSRXY, 0, 4,

DEC counter,

CRSRXY, 13, 4,

DEC5 time * 2

ENDIF

LOOP

The source code below will be use to measure the duration of the start pulse of the IR messages.

----------------------------------------------------------------------------------------------------------

'{$STAMP BS2}

'{$PBASIC 2.5}

time      	VAR  Word

counter   	VAR  Word

DEBUG   	"Start pulses", CR,

"Repetitions  Current duration", CR,

"                    (microseconds)  ", CR,

"-----------  ----------------"

DO

PULSIN 9, 0, time

IF (time >975) AND (time<1425) THEN

counter = counter + 1

DEBUG CRSRXY, 0, 4,

DEC counter,

CRSRXY, 13, 4,

DEC5 time * 2

ENDIF

LOOP

The source code below shows a program that use to detect an IR message from SONY IR and record all their data pulses.

----------------------------------------------------------------------------------------------------------

'{$STAMP BS2}

'{$PBASIC 2.5}

time    	VAR    Word(12)

index   	VAR    Nib

DEBUG 	"Time Array", CR,

"PWM Measurements", CR,

"Element Duration,2us", CR,

"_______ ____________"

DO

DO

PULSIN 9, 1, time(0)

LOOP UNTIL time(0) > 1000

PULSIN 9, 0, time(0)

PULSIN 9, 0, time(1)

PULSIN 9, 0, time(2)

PULSIN 9, 0, time(3)

PULSIN 9, 0, time(4)

PULSIN 9, 0, time(5)

PULSIN 9, 0, time(6)

PULSIN 9, 0, time(7)

PULSIN 9, 0, time(8)

PULSIN 9, 0, time(9)

PULSIN 9, 0, time(10)

PULSIN 9, 0, time(11)

FOR index = 0 TO 11

DEBUG CRSRXY, 0, 4 + index, "Time(", DEC index, ")"

DEBUG CRSRXY, 9, 4 + index, DEC time(index)

NEXT

LOOP