Mere Mortal Video Game Driver

Description:

This driver allows simple video games to be created using the Parallax® SX-Tech board or the Parallax® Professional Development Board. The driver handles all video and sound generation using an interrupt routine. Your program does not have to worry about counting cycles or routines that can only take a certain number of cycles. Hence the name “Mere Mortal”.

The only hardware required is 4 resistors and buttons for Left, Right, Up, Down, and Fire. Two players at the same time are supported.

Introduction:

Ever since making the SX-Video modules I have wanted to make a driver program to make games with SX/B for quite awhile. I have finally got it working.

This driver provides a 16 x 11 field of 16 unique tiles. The top line (line 0) has it own unique set of 16 tiles. This allows for scoring on the top line without use 10 of the 16 tiles. The “pong” example program uses quarter cell blocks to create a 32 x 22 field of pixels.

Sound is generated “in the background” also. You specify a tone and duration and the driver will generate the tone for as long as needed, then stop. Your program simply continues as soon as the command is given, it does NOT have to wait for the tone to stop.

Driver Routines:

There are various routines you call to create your game:

LineAddr      FUNC 1                   ' var = LineAddr line

ClearScreen   SUB 1, 2                 ' ClearScreen TileID[, StartLine]

PutTile       SUB 2, 3                 ' PutTile TileID, [address | x,y]

GetTile       FUNC 1, 1, 2             ' var = GetTile [address | x,y]

ScrollUp      SUB 0                    ' Scrolls up lines 1-10 (leaves line 0)

ScrollDown    SUB 0                    ' Scrolls down lines 1-10 (leaves line 0)

ScrollLeft    SUB 0                    ' Scrolls left lines 1-10 (leaves line 0)

ScrollRight   SUB 0                    ' Scrolls right lines 1-10 (leaves line 0)

ValueInc      SUB 1                    ' ValueInc Position ' Must be on line 0

ValueDec      SUB 1                    ' ValueDec Position ' Must be on line 0

Delay         SUB 1, 2                 ' Pause milliseconds

WaitSync      SUB 0                    ' Waits until screen has been drawn

Snd           SUB 2                    ' Snd Freq,Duration

var = LineAddr line

This function returns the screen address of the given line.

ClearScreen tileID[, line]

This routine clears the whole video screen to the tile ID given.

If the optional parameter “line” is given, clearing starts from that line.

PutTile tileID, address | x,y

This routine puts the given tile on the screen.

var = GetTile address | x, y

This routine sets “var” to the tileID that is at the location given.

# ScrollUp

This routine scrolls the screen up. Except the top line.

# ScrollDown

This routine scrolls the screen down. Except the top line.

# ScrollLeft

This routine scrolls the screen left. Except the top line.

# ScrollRight

This routine scrolls the screen right. Except the top line.

ValueInc position

This routine increases a value (score) on the top line.

The position given is the horizontal position of the last (units) digit.

ValueDec position

This routine decreases a value (timer?) on the top line.

The position given is the horizontal position of the last (units) digit.

Delay value

Pauses for “value” milliseconds

# WaitSync

This routine waits unit the video screen has been completely drawn.

This can be used to reduce flicker.

Snd tone, duration

Starts a background sound.