# SPECIFICATION FOR HENGGUAN LED LAMP

## Document No: SPE/HG-PA0BCZJ-C3ZJE2

Model No.:    PA0BCZJ-C3ZJE2

Rev. No.:      A

###### Date:         2007-05-28

Description:

High Power Blue LED For Illumination

High luminous flux output

Dice Material: InGaN

Confirmmed

by Customer:

Date:

| #### Applications ● High luminous flux output for illumination - Exposed pad design for excellent heat transfer - Designed for high current operation Absolute Maximum Ratings (Ta = 25℃) Items; Symbol; Absolute maximum Rating; Unit. Power Dissipation; PD; 10500; mW. Forward Current(DC)*2; IF; 350; mA. Reverse Voltage; VR; 5; V. Peak Forward Current*1; IFP; 400; mA. Operation Temperature; Topr; -40 ~ +85; ℃. Storage Temperature; Tstg; -40 ~ +100; ℃. Lead Soldering Temperature; Tsol; Max.260℃ for 3 sec Max. (3mm from the base of the epoxy bulb). | Notes: - All dimensions in mm tolerance is ±0.2mm unless otherwise noted. - An epoxy meniscus may extend about 1.5mm down the leads. 3. .Burr around bottom of epoxy may be 0.5mm max. |
| --- | --- |

*. Pulse width≦0.1msec duty≦1/10

Typical Electrical & Optical Characteristics ( Ta = 25℃)

| Items | Symbol | Condition | Min. | Typ. | Max. | Unit |
| --- | --- | --- | --- | --- | --- | --- |
| Forward Voltage | VF | IF = 350mA | 27 | 30 | 33 | V |
| Reverse Current | IR | VR = 5V | --- | --- | 10 | μΑ |
| Dominant Wavelength | λD | IF = 350mA | 465 | --- | 470 | nm |
| Luminous Intensity | IV | IF = 350mA | 85 | --- | 90 | lm |
| 50% Power Angle | θ | IF = 350mA | --- | 120 | --- | Deg |

Ranks Combination (IF=350mA) Lamps are sorted to Luminous intensity –Im & Dominant Wavelength – Wd bins shown. E2+ 100 lm Luminous Flux (Lm) 95 lm. 90 lm E3. 85 lm E2. 460nm 465nm 470nm Dominant Wavelength (Wd) * E2+ indicates Luminous Intensity is at E2 bin or above. Important Notes: - All ranks will be included per delivery, rank ratio will be based on the Dices distribution. - Pb content < 1000PPM - Tolerance of measurement of luminous intensity is ±15% - Tolerance of measurement of dominant wavelength ±1.0nm - Tolerance of measurement of forward voltage ±0.05V - Packaging methods are available for seclection, Please refer to PACKAGING STANDARD. - Please refer to LED RELIABILITY TEST STANDARD for reliability test conditions. - Please refer to APPLICATION NOTES for application.

Typical Electrical/ Optical Characteristics Curves (Ta=25°C Unless Otherwise Noted) Revision History. Rev. No; Date; Change Description. A; 2007-05-28; Original version.