' {$STAMP BS2}

' {$PBASIC 2.5}

'

'

' Robot_Sense_Table_Edge.bs2

'

' ------- I/O Definitions ------------

Ping              PIN       15

' ------- Constants ------------------

Trigger           CON       5          ' trigger pulse = 10uSec

Scale             CON       $200       ' raw * 2.00 = uSec

RawToCm           CON       2257       ' 1 / 29.034 (w/ **)

IsHigh            CON       1          ' for PULSOUT

IsLow             CON       0

' ------- Variables ------------------

rawDist           VAR       Word           ' distance as function of lapsed time

cm                VAR       Word

edgeCtr           VAR       Word

distanceCtr       VAR       Word

edgeCtr     =     0

distanceCtr =     0

' ------- Initialization -------------

Reset:

DEBUG CLS,                             ' setup report screen

"Parallax Ping))) Sonar ", CR,

"---------------------- ", CR, CR,

"Time (uS)......     ", CR,

"Centimeters....     "

' ------- Program Code --------------

Main:

DO

GOSUB Get_Sonar

DEBUG CRSRXY, 15, 3,

DEC rawDist, CLREOL,

CRSRXY, 15, 4,

DEC cm, CLREOL

PAUSE 100

GOSUB Move_Robot

LOOP

END

' ------- Subroutines ---------------

Get_Sonar:

Ping = IsLow                            ' make trigger 0-1-0

PULSOUT Ping, Trigger                   ' activate sensor

PULSIN Ping, IsHigh, rawDist            ' measure echo pulse

PAUSE 20

rawDist = rawDist */ Scale              ' convert to uSec

rawDist = rawDist / 2                   ' remove return trip

cm = rawDist ** RawToCm

RETURN

Move_Robot:

IF (cm <= 3 ) THEN

GOSUB Forward

edgeCtr = edgeCtr + 1

ENDIF

IF ( cm > 3 ) THEN

GOSUB BackUp

GOSUB NinetyLeft

GOSUB NinetyLeft

GOSUB Get_Sonar

IF ( cm <= 3 ) THEN

GOSUB Forward

edgeCtr = edgeCtr + 1

IF ( edgeCtr > 2 ) THEN

GOSUB EndProgram

ENDIF

ELSE

GOSUB EndProgram

ENDIF

ENDIF

RETURN

Forward:

PULSOUT 12, 662

PULSOUT 13, 850

PAUSE 20

RETURN

BackUp:

FOR distanceCtr = 0 TO 30

PULSOUT 13, 700

PULSOUT 12, 800

PAUSE 20

NEXT

RETURN

NinetyLeft:

FOR distanceCtr = 0 TO 32

PULSOUT 12, 662

PULSOUT 13, 745

PAUSE 20

NEXT

RETURN

EndProgram:

END