Infrared Sensor Kit (#28019)

Using the S.S.I.R Sensor

This documentation shows how to use the Parallax S.S.I.R board with the Parallax Basic Stamp®

single-board computer.

Packing List

The contents of the Infrared Sensor Kit package include:

- (1) SSIR Infrared Sensor

- (2) 4-40 Machine Screws, 3/8.

- (2) 4-40 Nut, Zinc Plated

- (1) 90 Degree Mounting Bracket

- (1) 10. Servo Extension Cable

Description

The S.S.I.R board is an infrared detection unit. This unit incorporates a standard IR LED with a 40

kHz IR receiver. This unit can be configured for basic object detection.

Size: Width = 15.8 mm, Length = 18.2 mm

Power Requirements: + 5vdc, 2.6 mA

Vdd

Pin

470 Ω

1n5231

Software Interface

'{$STAMP BS2}

DIR0 = 1 .Sets pin 0 to input

Holdbit VAR BIT .Sets up a variable for the echo

Start:

FREQOUT 0,1,38500 .sends out signal to IR LED

Holdbit = in0 .Stores reply

DEBUG 1,"sensor = ",DEC Holdbit,cr .displays results

GOTO Start