' {$STAMP BS2}

' {$PBASIC 2.5}

' =========================================================================

'

'   File....... RoboRocket_Mar05ta.BS2

'   Purpose....

'   Rocketeers.... Andrew (am),  Anna (am), Tyler (tb), Chris (ck), Mollie (md), Tim (tm), Michaella (mw)

'   Rocketeers.... Ben (bl), Jess (jr), Destiny (ds), Nick (nm), Zach (zg)... and Bandit the Rocket Dog

'   Faculty...  L. Mark Kibler (mk)

'   Peanut gallery... Tracy Allen (ta), Phil Kenny (pk),  Bruce Bates (bb) ...

'   E-mail.....

'   Started....

'

'

' =========================================================================

' -----[ Program Description ]---------------------------------------------

' -----[ Revision History ]------------------------------------------------

'

'-- 06 March 2008 (mk, am, mw, bl, ds, jr, nm)

' 1) Changed data sampling time at descent to 1 sec intervals (from 2>10 to 2>100) per (ta)

' 2) Commented out 'GOSUB morse reading' twice in 'main:' subroutine per (ta)

' 3) Shortened duration of 'motormovement' times so we could bench test the program. These will be changed

'    back to their original 1 meter distance (0 TO 681) when we finalize the program.

' 4) The Rocketeers experimented by changing various settings to see what would happen,

'    and marveled at the Morse code light. Bandit the Rocket Dog was little help.

'

'-- 05 Mar 2008  (ta)

' Added increment of sample counter index2.  Displays on debug.  Land flag

' is set by either elapsed time or index2 accumulation.

'

' -- 04 Mar 2008  (ta) Mar04a

' Major restructuring from template, variables, starting point "RocketRobot_Feb29.BS2"

' Streamlined the Sensirion and movement routines with subroutine structure.

' Consolidated variables, named all pins

' This version incorporates the clock, LED and movements, but not yet the datalogger

'

' -----[ I/O Definitions ]-------------------------------------------------

ShtDta  PIN 1        ' bi-directional data with pullup resistor

ShtClk  PIN 0        ' clock (for Sensirion) with pulldown resistor

DSclk    PIN 2      ' DS1302 RTC clock

DSdta    PIN 5      ' DS1302 RTC data line

DSce    PIN 7      ' DS1302 RTC chip enable (high enable)

' note p3, p4, and p6 are unused

TX     PIN 8               ' Transmit Data   --> 27937.4 (RXD)

RTS    PIN 9               ' Request To Send --> 27937.6 (CTS)

RX    PIN 10              ' Receive Data    <-- 27937.5 (TXD)

CTS    PIN 11              ' Clear To Send   <-- 27937.2 (RTS)

ledP    PIN 12      ' indicator LED, positive side

ledN    PIN 13      ' indicator LED, negative side

servoR  PIN 14      ' servo on right side

servoL  PIN 15      ' servo on left side

' -----[ Constants ]-------------------------------------------------------

vBaud           CON     84    ' Serial Baud Rate 9600 bps for Vinculum datalogger

ShtTemp         CON     %00011    ' read temperature command for Sensirion

ShtHumi         CON     %00101    ' read humidity command for Sensirion

Ack             CON     0      ' responses for Sensirion

NoAck           CON     1

DegSym          CON     186     ' degrees symbol for DEBUG

' -----[ Variables ]-------------------------------------------------------

primo         VAR Word

iobyte  VAR primo.BYTE0    ' io to/from datalogger, humidty sensor, & debug

index  VAR primo.NIB2    ' loop counter in some routines

flags  VAR primo.NIB3

ackbit  VAR flags.BIT0    ' acknowledge Sensirion

shtErr   VAR flags.BIT1    ' = 1 if error in sensirion read

dskErr  VAR flags.BIT2    ' =1 if the logger fails to work

land       VAR flags.BIT3       ' =1 after alloted time interval

waldo         VAR Word             ' scratch variable

soSHT  VAR waldo      ' alias as scratch variable for Sensirion

soSHT0  VAR soSHT.BYTE0    ' byte by byte

soSHT1  VAR soSHT.BYTE1

delay  VAR waldo      ' timeout for Sensirion

motormovementtime  VAR waldo  ' alias as timer variable for movements

degC          VAR Word             ' temperature in Celsius*10 from Sensirion

rh            VAR Word             ' humidity in percent*10 from Sensirion

time0         VAR Word

second  VAR time0.BYTE0    ' from DS1302 RTC

minute  VAR time0.BYTE1    ' ditto

time1         VAR Word

hour  VAR time1.BYTE0    ' ditto

index2  VAR time1.BYTE1    ' extra index

buffer   VAR Word                  ' for data returned from logger or RTC

buffer0  VAR buffer.BYTE0  ' will return as bytes

buffer1  VAR buffer.BYTE1

' note the buffer can extend into unassigned space, take care.

' -----[ EEPROM Data ]-----------------------------------------------------

' -----[ Initialization ]--------------------------------------------------

Reset:

DEBUG CR,"Sample time   Sample#   Temp    Humidity (%)",BELL,CR

OUTS =  %0001000000000000   ' indicator led is on. What to do about tx and rts?

'fedcba9876543210

DIRS =  %1111001111111101   ' p1 is input for sensirion data, pa and pb input for rx and cts

PAUSE 10

GOSUB RTC_init  ' reset the clock to 00:00:00

PAUSE 512

HIGH TX                               ' Initialize Transmit Line

LOW RTS                               ' Take Vinculum Out Of Reset

PAUSE 512                               ' Allow 1/2 Second To Settle

LOW ledP        ' LED indicator off - long pulse signals reset

GOSUB SHT_Connection_Reset

' -----[ Program Code ]----------------------------------------------------

Main:

DO

GOSUB RTC_read

IF minute=$01 OR index2>100 THEN land=1   ' set flag to start movements after descent of one minute

index2 = index2 + 1

DEBUG CR,HEX2 hour,":",HEX2 minute,":",HEX2 second, TAB, DEC index2

GOSUB SHT_Measure_Temp

GOSUB SHT_Measure_Humidity

DEBUG TAB,REP "-"\degC.BIT15,DEC ABS degC/10,".",DEC1 ABS degC,degSym

DEBUG TAB, DEC rH/10, ".", DEC1 rH, "%", CR

waldo=degC+5/10

'GOSUB morse_reading

NAP 5

waldo=rh+5/10

'GOSUB morse_reading

IF land THEN GOSUB movement

LOOP

END

' -----[ Subroutines ]-----------------------------------------------------

' -----[ RTC ]-----

RTC_init:

' send $00 to DS1302 control register (command $8E) to enable write

HIGH DSce

SHIFTOUT DSdta,DSclk,LSBFIRST,[$8E,$00]

LOW DSce

' set the clock time using burst command $BE

' second=0, minute=0, hour=0, date31=2, month=3, day7=1, year=08

' also disable further writes: $80 at loc $7

HIGH DSce

SHIFTOUT DSdta,DSclk,LSBFIRST,[$BE,$00,$00,$00,$02,$03,$01,$08,$80]

LOW DSce

RETURN

RTC_read:

' retrieves the time to variable: second, minute, hour

HIGH DSce

SHIFTOUT DSdta,DSclk,LSBFIRST,[$BF]     ' burst mode read $BF command

SHIFTIN DSdta,DSclk,LSBPRE,[second,minute,hour]

LOW DSce

RETURN

' -----[ Sensirion ]-----

SHT_Connection_Reset:

SHIFTOUT ShtDta, ShtClk, LSBFIRST, [$FFF\9]

RETURN

'

' generates SHT1x "start" sequence

'          _____         _____

' ShtData       |_______|

'              ___     ___

' Clock    ___|   |___|   |___

SHT_Start:

INPUT ShtDta    ' let pull-up take line high

LOW ShtClk

HIGH ShtClk

LOW ShtDta

LOW ShtClk

HIGH ShtClk

INPUT ShtDta

LOW ShtClk

RETURN

' sends start sequence & command byte & retrieves two data bytes from Sensirion

SHT_measure:

GOSUB SHT_Start                               ' alert device

GOSUB SHT_Write_Byte                          ' send command (iobyte)

GOSUB SHT_Wait                                ' wait until measurement done

ackBit = Ack                                  ' another read follows

GOSUB SHT_Read_Byte                           ' get MSB

soSHT1 = ioByte

ackBit = NoAck                                ' last read

GOSUB SHT_Read_Byte                           ' get LSB

soSHT0 = ioByte

RETURN

'

' measure temperature:

' -- celcius = soT * 0.01 - 40

SHT_Measure_Temp:

ioByte = ShtTemp                             ' temperature command

GOSUB SHT_measure                         ' send command

degC = soSHT / 10 - 400                           ' convert to tenths C

RETURN

' measure humidity

SHT_Measure_Humidity:

ioByte = ShtHumi                             ' humidity command

GOSUB SHT_measure                             ' send command

rh = (soSHT ** 26542)

rh = rh - ((soSHT ** 3468) * (soSHT ** 3468) + 50 / 100)

rh = rh - 40

rh = ((degC + 250 / 10 - 50) * (soSHT ** 524 + 1) + (rh * 10)) + 5 / 10

RETURN

' sends "ioByte"

' returns "ackBit"

SHT_Write_Byte:

SHIFTOUT ShtDta, ShtClk, MSBFIRST, [ioByte]  ' send byte

SHIFTIN  ShtDta, ShtClk, LSBPRE, [ackBit\1]   ' get ack bit

RETURN

' returns "ioByte"

' sends "ackBit"

SHT_Read_Byte:

SHIFTIN  ShtDta, ShtClk, MSBPRE, [ioByte]    ' get byte

SHIFTOUT ShtDta, ShtClk, LSBFIRST, [ackBit\1] ' send ack bit

INPUT ShtDta                                 ' release data line

RETURN

' wait for sht to finish measurement

' (data line pulled low by SHT within 1/4 second)

SHT_Wait:

INPUT ShtDta                                 ' data line is input

delay = 255                        ' give >1/4 second to finish

DO

delay=delay-1

PAUSE 1

LOOP UNTIL ShtDta=0 OR delay=0

IF delay=0 THEN

GOSUB SHT_Connection_Reset

shtErr=1

ENDIF

RETURN

' -----[ morse code ]-----

Morse_numeral:

iobyte = %11110000011111 >> iobyte & %11111 | %10100000

Morse:

FOR index = iobyte>>5 TO 1

HIGH ledP

PAUSE 192*iobyte.BIT0(index-1)+64

LOW ledP

PAUSE 256

NEXT

PAUSE 512

RETURN

morse_reading:

DEBUG 32

FOR ackbit=1 TO 0

iobyte = waldo DIG ackbit

DEBUG DEC1 iobyte

GOSUB morse_numeral

NEXT

RETURN

mcs CON 3*32+%000  ' dit dit dit

mco CON 3*32+%111 ' dah dah dah

mcn CON 2*32+%10   ' dah dit

mcf CON 4*32+%0010  ' dit dit dah dit

mcb CON 4*32+%1000   ' dah dit dit dit

mcc CON 4*32+%1010  ' dah dit dah dit

mcw CON 3*32+%011  ' dit dah dah

mce CON 1*32+%0     ' dit

message1 DATA mcs,mco,mcs,0  ' sos, startup message for LED

morse_message:

' plays back morse message up to null

' enter with waldo pointing to message start

' e.g. waldo=message1 : GOSUB morse_message

DO

READ waldo,iobyte

IF iobyte=0 THEN EXIT

GOSUB morse

waldo=waldo+1

LOOP

RETURN

' -----[ servos ]-----

movement:

PAUSE 100

iobyte=mcf : GOSUB morse

GOSUB forward

iobyte=mcb : GOSUB morse

GOSUB backward

iobyte=mcc : GOSUB morse

GOSUB turnclockwise

iobyte=mcf : GOSUB morse

GOSUB forward

iobyte=mcb : GOSUB morse

GOSUB backward

iobyte=mcw : GOSUB morse

GOSUB turnCOUNTERclockwise

iobyte=mce : GOSUB morse

PAUSE 1000

RETURN

forward:

FOR motormovementtime = 0 TO 120

PULSOUT servoL, 850                           ' move forward 1

PULSOUT servoR, 650

PAUSE 20

NEXT

RETURN

backward:                                    ' move backward 1/3

FOR motormovementtime = 0 TO 60

PULSOUT servoL, 650

PULSOUT servoR, 850

PAUSE 20

NEXT

RETURN

turnclockwise:                               ' rotate clockwise

FOR motormovementtime = 0 TO 30

PULSOUT servoL, 850

PULSOUT servoR, 850

PAUSE 20

NEXT

RETURN

turnCOUNTERclockwise:                               ' rotate clockwise

FOR motormovementtime = 0 TO 30

PULSOUT servoR, 650

PULSOUT servoL, 650

PAUSE 20

NEXT

RETURN