Imports System.Threading

Public Class Form1

Dim rxdata(2) As Byte   'data from SX

Dim txdata(2) As Byte   'data to SX

Private Sub Form1_Load(ByVal sender As Object, ByVal e As System.EventArgs) Handles Me.Load

CheckForIllegalCrossThreadCalls = False 'this is a test program so we will ignore cross thread errors

SerialPort1.BaudRate = 38400   'set the baud here to whatever

SerialPort1.Open() 'open the port on Form1 load

End Sub

Private Sub SerialPort1_DataReceived(ByVal sender As Object, ByVal e As System.IO.Ports.SerialDataReceivedEventArgs) Handles SerialPort1.DataReceived

Dim indata As String    'string that will hold the header

indata = SerialPort1.ReadLine

If indata = "TX" Then   'if the header is TX read incoming else ignore

Thread.Sleep(2) 'allow time for incoming to reach the buffer

SerialPort1.Read(rxdata, 0, 2)

TextBox1.Text = rxdata(0)   'display incoming in text boxes

TextBox2.Text = rxdata(1)

SerialPort1.ReadExisting() 'clear the serial buffer

SerialPort1.Write(txdata, 0, 2)    'transmit the trackbar values

SerialPort1.ReadExisting() 'clear the serial buffer

End If

End Sub

Private Sub TrackBar1_Scroll(ByVal sender As System.Object, ByVal e As System.EventArgs) Handles TrackBar1.Scroll

txdata(0) = TrackBar1.Value

End Sub

Private Sub TrackBar2_Scroll(ByVal sender As System.Object, ByVal e As System.EventArgs) Handles TrackBar2.Scroll

txdata(1) = TrackBar2.Value

End Sub

End Class

DEVICE SX28,OSCHS3,TURBO,STACKX,OPTIONX

FREQ 50_000_000

BAUD CON "N38400"

TX VAR RB.1

RX VAR RB.0

VAL_1 VAR BYTE

VAL_2 VAR BYTE

VAL_3 VAR BYTE

VAL_4 VAR BYE

PROGRAM MAIN

MAIN:

T_OUT:

SEROUT TX,BAUD,"T"

SEROUT TX,BAUD,"X"

SEROUT TX,BAUD,10

SEROUT TX,BAUD,VAL_1

SEROUT TX,BAUD,VAL_2

SERIN RX,BAUD,VAL_3,100,T_OUT

SERIN RX,BAUD,VAL_4,100,T_OUT

PAUSE 30

VAL_1=VAL_3

VAL_2=VAL_4

GOTO MAIN