Thanks for reading,

I’ve finished basic beta testing for a new file system driver designed exclusively for the propeller chip. The driver is designed to allow the user to access a FAT32 or FAT16 file system on an SD card attached to the propeller chip.

The driver itself contains three software parts – A block driver and a FAT32/16 file system driver and a RTC driver.

The block driver gives the file system driver access to a piece of physical media that can be read and written in 512 bytes chunks. In this case that piece of physical media is a SD card.

The file system driver is the heart of the system and uses the block driver to read and write to a FAT16/32 file system on the physical media. The file system driver is what you the user communicates to.

The RTC driver gives the file system driver access to time and date information so that it can properly apply time stamps on files and folders in the file system.

So, the features the file system has included contain the ability to:

-- Mount any one of the 4 logical partitions on a standard computer file system.

-- Mount a FAT16/32 file system that is located inside of any of the 4 logical partitions.

After the file system is mounted additional commands may be executed. If the file system has not been mounted yet all other commands are disabled.

The block driver will prevent the user from mounting the physical media if the block driver itself cannot initialize the physical media which must occur before mounting the physical media.  Additionally, if a block read or write error occurs then the block driver will automatically un-mount the physical media in order to protect the file system from improper access. Any files that were open at that time are also closed.

If an error occurs then the file system must be remounted. This is done to prevent the file system driver from improperly accessing any file system using possibly outdated environmental variables.

If the file system was successfully mounted then the mount function will return true. If the file system failed to mount the mount function will return false.

All other use functions that have the possibility of failing behave in the same manner. True for success and false for failure. If a function returns false then the file system must be remounted.

However, higher level functions will use the length of a string to report success, failure, or other error formats.

For example, when opening a file the function will return a string of length if the file is found, a string of no length if the file was not found, and false if an error occurred. Because the string pointer is an address other than 0 it is effectively a logical true. Note that the string pointer returned will always be to the same address.

To access files in the file system the driver uses listing functions which are given to the user to find files and folders. The most important of these include “list name”, “list size”, list search”, and “list reset”. Through using these four functions the file system driver is able to find all files and folders in the current directory and properly display them for listing purposes.

The full set of listing functions give the user the ability to:

-- List the next file or directory 8.3 name.

After the next file or directory 8.3 file name is listed then the user can retrieve additional information which includes:

-- The creation date and time.

-- The last access date.

-- The last modification date and time.

-- The file or folder size. (Folders have no size.)

-- The read only, hidden, system, directory, and archive attributes for that file or folder.

All of this functionality is controlled through the “list name” function which lists the next file or folder in the current directory. That said, to start listing from the top of the current directory the “list reset” function allows the user reset the “list name” function so that it will start from the top of the current directory.

Finally, the “list search” function allows the use to search through the current directory for a specific file or folder by comparing 8.3 file names.

So, an 8.3 file name is a file or folder name that has 8 leading characters and 3 trailing characters. The names are stores in all capital letters and must be unique in the current directory. The file system driver will handle the conversion from natural 8.3 file names to standardize 8.3 file names for you.

So, you can enter names in any of these formats:

“name.txt” – maps to “NAME     TXT”

“namedStuff.text” – maps to “NAMEDSTUTEX”

“namesThatAreToLong” – maps to “NAMESTHA   “

The file system driver will grab the first 8 characters for the file or folder name and the next 3 characters only after a period for the extension. Any invalid characters being entering in the 8.3 file name are automatically turned in an underscore. The driver will allow you to enter completely invalid names; however it will always apply the proper rules converting them to proper names. That said, what you enter telling the driver what to do and what it does may differ if your 8.3 file name does not follow naming convention rules.

A good primer on this can be found at http://en.wikipedia.org/wiki/8.3_filename

A good primer on FAT16/32 can also be found at http://en.wikipedia.org/wiki/File_Allocation_Table

Long file names are not supported at this time due to their complexity and irrelevance since the FAT16/32 file system operates without them as they are an extension.

A good primer on Long file names can be found at http://en.wikipedia.org/wiki/Long_filename

Finally, the “list name” function always returns names in the 8.3 file name format in standard form.

Now, the more advanced features of the file system included the ability to:

-- Change the current working directory.

-- Change the attributes for a file or folder.

-- Rename files or folders.

-- Delete files or folders.

-- Create new files or folders.

By using these five functions the user can build their own file and folder structure from the ground up and edit already made files or folders for their use.

Next up are the file access functions. Note that when a file is not open these functions will do nothing and when a file is opened only these functions will do anything and all other functions are disabled.

These functions give the user the ability to:

-- Open a file for reading, writing, or appending (a mode of writing).

-- Close open files and write their last access, modification, and last size data out to the physical media.

-- Seek back and forth throughout a file in an optimized way.

-- Get the current position within a file.

-- Read a character at a time from a file.

-- Write a character at a time from a file.

-- Read a sector (512 bytes) at time from a file.

-- Write a sector (512 bytes) at time from a file.

By using these functions the user has full FAT32/16 file system access.

Last but not least the file system driver supports the ability for the user to query the free space on the card. This function returns then the number of free sectors on the partition dedicated to the FAT32/16 file system. Since this function performs a linear count on all clusters of the FAT it can take a long time to finish executing and may not return for many minutes.

So, the file system driver has the following limitations:

-- The partition must not be more than 1TB in size and must located in the first 1TB of sectors on the physical media. Because SPIN performs signed math the 2TB limit is not possible.

-- The maximum file size is (2GB – 512B). Any file greater than this size can still be access but the bytes above the limit will be ignored.  The file size query will also never return a number above this limit. Because SPIN performs signed math the 4GB limit is not possible.

-- A directory can contain 65536 entries and no more. This included the hidden long file names so the limit may be reached even before 65536 files are found. This is not actually a limit of the file system driver but mandated by Microsoft specifications.

Features that are not supported but may be supported in the future include:

-- FAT32 free space queries and next free cluster optimizations. (For faster FAT32 file system access.)

-- Accessing the volume ID.

-- Setting the file system error flags when an error occurs.

-- Formatting the partition.

Thanks again for reading,