Synthesizing Speech with the Propeller

This chapter assumes you have a Propeller Demo Board, or at least some audio output connections to pins 10 and 11. The featured spectograph program and all code examples can be downloaded from <url here>.

Have you ever wondered how to make a human voice in software? I’ve always been tantalized by the idea, since speech is perhaps the ultimate analog communication protocol. It certainly predates computers by a long time.

In order to synthesize speech, we need to know what’s in it. We could start by looking at some waveforms of recorded speech:

Detail from utterance “To”.

I found this type of looking to be a recipe for madness, as you easily fall into the morass of trying to replicate details which are not important and then missing what matters.

Instead of worrying about waveforms yet, we need to look at the spectrum of speech over time. Here is where our real information is:

This is me counting from 1 to 10.

What you’re looking at is called a “spectograph". This is a 3D representation of a time-varying signal. From left to right you have time. From bottom to top you have frequency. The intensity at each point is the energy of that frequency at that time.

Looking at speech from this perspective gives us the insight we need to go about synthesizing it. In fact, this is what your ear “sees”, rather than a sputtering waveform.

Note the horizontal bands of darkness in the spectrograph. Those are harmonics of my voice’s glottal pitch. I tend to talk at about 110Hz, so there’s nominally about 110Hz of separation between each band. You can see the bands spread out and compress as my voice deviates up and down in tone.

Now notice the concentrations of dark bands distributed vertically. Those are the result of resonances within my vocal tract. As I talk and change the shape of my vocal tract, they move around – some up, some down. Each is a “formant”. The key to speech synthesis is synthesizing these formants.

How can you synthesize formants? This was very challenging for me and I spent a lot of time trying to do it. The first and most obvious approach was to synthesize individual sine waves for each harmonic of the glottal pulse. This was way too much work and computationally impractical. I did, much later, come up with one approach that was really simple, but falls into the “hack” category as it generated discontinuities in the output signal: just generate sine waves at the center frequency of each formant and reset their phases at each glottal pitch interval – like magic, the sine waves diffuse and the pitch harmonics distribute nicely around the formants’ centers, giving you instant vowel sounds! Good enough for a talking watch, but not a musical instrument.

Let’s look at some vowel sounds. Here, I’ll make my glottal pitch very low (like a frog), so that the pitch lines will be highly compressed. This makes it easier to see where the formants are:

Spoken vowels: beet, hat, hot, soap, borrow, ball. Yep, r and l are vowels in synthesis.

Here’s a table of the first four formant frequencies (F1..F4) gleaned from the above vowels:

|  | beet | hat | hot | soap | borrow | ball |
| --- | --- | --- | --- | --- | --- | --- |
| F4 | 3700 | 3400 | 3200 | 3200 | 4700 | 3600 |
| F3 | 3100 | 2500 | 2400 | 2400 | 1500 | 2600 |
| F2 | 2000 | 1700 | 1050 | 950 | 1200 | 850 |
| F1 | 310 | 730 | 750 | 530 | 580 | 560 |

If we can synthesize these formants, we can recreate those vowel sounds.

Now, I’ll introduce the VocalTract object. This is the neatest piece of software I’ve ever written and it’s the kind of program that the Propeller chip was designed for. It synthesizes a human vocal tract in real-time using a simple frame of 13 byte-size parameters. All you have to do is change any parameters of interest within the frame, then tell it how fast to transition to the new frame. It will internally snapshot and queue up each frame you give it, then linearly interpolate each of the 13 parameters from frame to frame, over specified periods. Out comes continuous speech that can be made very expressive, with amazingly little input.

It used to be that synthesis algorithms were king several decades ago. With the advent of big, inexpensive memory, however, rote recording and regurgitation have become the norm for many areas in which algorithmic synthesis would be much more flexible, not to mention more memory-efficient. Many speech synthesizers today use recorded snippets of speech which are concatenated at run-time to create continuous speech. These programs and data often require many megabytes of memory. In contrast, VocalTract is only 0.0013 megabytes in size (334 longs of code + 44 longs of variables per instance).

Below is the initial documentation section from the VocalTract object:

Note the schematic diagram and the parameters associated with each section.

The ASPIRATION section produces turbulence in the vocal tract. This is for breathy sounds, but is always needed for natural sounding speech. The GLOTTAL section produces continuous glottal pulses for voiced speech. The VIBRATO section modulates the glottal pitch for singing, but helps regular speech sound natural, as only robots are known to speak in pure monotone. The pitch parameters are all registered to the musical scale, with 48 steps per octave, making every 4th value a chromatic tone. Note that the ASPIRATION and GLOTTAL sounds are summed before feeding into the formant resonators F1..F4 that make vowel sounds. The NASAL section is an anti-resonator used for M, N, and NG sounds. Lastly, the FRICATION section is for white-noise sounds like T, TH, SH, S, and F that occur at the tongue, teeth, and lips, or after the resonators. The FRICATION output is summed with the NASAL output to make the final product: a 32-bit, 20KHz-updated audio signal.

Though all parameters are 8-bit within a frame, they are interpolated with 24 sub-bits, so intra-frame transitions are very smooth. Also, frames connect seamlessly with the same precision, so there are never any discontinuities in the output signal. One thing a user must be aware of is the likelihood of numerical overflow when volume levels get high. You’ll know if this happens, believe me.

The resonators in VocalTract that make the formants are very simple, but it took a lot of work and luck to figure out how to make them. Most resonator algorithms require transcendental math operations which need extreme precision around the nearly-asymptotic +/-1 sine and cosine points. This is fine for floating-point hardware systems, but a disaster in fixed-point math architectures. Also, you could easily spend more time computing ever-changing resonator coefficients than you would actually computing the resonator state for each output sample. I needed a simpler way to build a resonator. Fortunately, the CORDIC algorithm to my rescue! CORDIC is an ingenious technique developed in the 1950’s by Jack E. Volder which uses binary shifts and adds to perform the transcendental functions. In a nutshell, it can rotate (X,Y) coordinates very efficiently, without the edgy numerical sensitivities of commonly-used resonator algorithms. In each of VocalTract’s resonators, the incoming signal is summed into Y of an (X,Y) point which defines the state of the resonator. Then, that point is CORDIC-rotated by an angle directly proportional to the formant’s center frequency. The resulting Y of the (X,Y) point is passed on to the next resonator. When in-band excitation is received, the (X,Y) point grows further from (0,0) as it rotates around. Out-of-band excitation causes the amplitude to decay to the point where the resonator pretty much just passes its input through to its output. If this wasn’t simple enough, since the step-angle fed to each formant’s CORDIC rotator is directly proportional to the formant’s center frequency, simple linear interpolation is used to slide each formant from one frame’s setting to the next. Goodbye math headaches!

Let’s now recreate those six vowel sounds that I recorded. Here’s all you need to do (see Example1.spin):

CON

_clkmode = xtal1 + pll16x

_xinfreq = 5_000_000

OBJ

v     : "VocalTract"

VAR

byte  aa,ga,gp,vp,vr,f1,f2,f3,f4,na,nf,fa,ff  'vocal tract parameters

PUB start

v.start(@aa, 10, 11, -1)              'start tract, output to pins 10,11

repeat                                'keep repeating talk

gp := rnd(50, 100)                  'set random glottal pitch

vp := rnd(4, 24)                    'set random vibrato pitch

vr := rnd(4, 10)                    'set random vibrato rate

talk                                'talk

PRI talk

setformants(310, 2000, 3100, 3700)    'set formants for bEEt

v.go(1)

aa := 3                               'set breathiness

ga := 30                              'set voiced volume

v.go(100)                             'ramp up

v.go(500)                             'sustain

setformants(730, 1700, 2500, 3400)    'set formants for hAt

v.go(100)                             'transition

v.go(500)                             'sustain

setformants(750, 1050, 2400, 3200)    'set formants for hOt

ga := 12                              'volume down a little

v.go(100)                             'transition

v.go(500)                             'sustain

setformants(530, 950, 2400, 3200)     'set formants for sOAp

v.go(100)                             'transition

v.go(500)                             'sustain

setformants(580, 1200, 1500, 4700)    'set formants for boRRow

v.go(100)                             'transition

v.go(500)                             'sustain

setformants(560, 850, 2600, 3600)     'set formants for baLL

v.go(100)                             'transition

v.go(500)                             'sustain

aa := 0                               'taper down to silence

ga := 0

v.go(100)

v.go(500)                             'pause

PRI setformants(s1, s2, s3, s4)

f1 := s1 * 100 / 1953

f2 := s2 * 100 / 1953

f3 := s3 * 100 / 1953

f4 := s4 * 100 / 1953

PRI rnd(low, high)

return low + ||(?cnt // (high - low + 1))

Recording the audio output of the above program into our spectograph, we get:

You can see the formants transition as they did in the original voice recording. Compare the waveforms in the top panes, as well. They look a little different, but they sound very similar. Speech is a rather inexact phenomenon, thankfully.

By making “ga := 0” in the above program, you will hear the vowels whispered, since there is some breath noise via “aa”. You can tell from listening to the synthesis that the transition from one vowel to another makes nice diphthongs. For example, going from beet to hat makes “yeah”. Vowel speech is pretty straightforward in this way.

How about some breathing sounds? Try modifying the “PRI talk” method in the previous program to be as follows (see Example2.spin):

PRI talk

setformants(880,1075,2350,3050)        'set "inhale" formants

v.go(1)

aa := 30                               'slowly ramp up

v.go(1000)

v.go(500)                              'sustain

aa := 0                                'slowly ramp down

v.go(1500)

setformants(500,800,1100,2325)         'set "exhale" formants

v.go(1)

aa := 10                               'ramp up

v.go(500)

aa := 0                                'slowly ramp down

v.go(2500)

v.go(500)                              'pause between breaths

This should clear up any confusion about the Propeller being able to sleep.

What about consonant sounds? I’ve made a lot of little discoveries about synthesizing consonants by observing spectographs and then experimenting with VocalTract. You can easily do the same. Sometimes it’s necessary to speak very slowly and distinctly into the spectograph in order to see otherwise-subtle phenomena. Looking at the actual waveform can give important hints, too – particularly around plosives (i.e. t, p, and k) where there is silence.

It turns out there is just a handful of basic recipes you need to know in order to make VocalTract pronounce any consonant. The biggest divider among consonants is whether they are voiced or unvoiced. That is, is the glottal pulse going or is there just breathiness? This distinction halves the number of basic consonant recipes.

Say each of these word pairs aloud and note the voiced/unvoiced difference in the underlined consonant:

| Voiced | Unvoiced |
| --- | --- |
| burl | pearl |
| do | to |
| van | fan |
| girl | curl |
| zoo | sue |
| jock | chalk |
| this | thin |
| measure | pressure |

The only difference within these sets of consonants is when your glottal pulses are going. You’ll note that the rest of your vocal tract is in the same exact state when making either sound. This means that controlling the “ga” parameter differently will be the key to making one sound versus the other.

Let’s do “girl” and “curl”. To make these g/c sounds, you can initially set “f1” to zero and set “f2” and “f3” to the average of the following vowels’ “f2” and “f3”. In other words, F1 will sweep upwards rapidly and F2 and F3 will begin in the center and then head down and up, respectively, to their final vowel positions. This is the base recipe for g and c sounds. The difference between g and c is when the glottal pulses start: early for g and late for c.

Substitute the following method for “PRI talk” in our program (see Example3.spin):

PRI talk

setformants(0, 1350, 1350, 4700)      'set formants for G/C before R

v.go(1)

aa := 20                              'set breathiness

v.go(25)                              'make brief G/C turbulence

setformants(580, 1200, 1500, 4700)    'set formants for R

ga := 30                              'voice up for G / skip for C

v.go(150)                             'transition to R

ga := 30                              'sustain / voice up for C

v.go(100)                             'sustain / voice up for C

v.go(200)                             'sustain

setformants(560, 850, 2600, 3600)     'set formants for L

v.go(250)                             'transition

v.go(100)                             'sustain

aa := 0                               'taper down to silence

ga := 0

v.go(100)

v.go(500)                             'pause

You should hear “girl”. If you comment out the first “ga := 30”, you will hear “curl”, as the glottal pulses will start later. Commenting out both “ga := 30” lines will result in a whisper that sounds like it could be “girl” or “curl”.

We can make this say “burl” and “pearl”, as well. For b and p sounds, “f1” and “f2” must be set very low and swept upwards quite rapidly, while “f3” and “f4” can be the same as the following vowel’s. Two changes must be made to our “PRI talk” method to make b and p. First, replace “setformants(0, 1350, 1350, 4700)” with “setformants(0, 200, 1500, 4700)”. Second, replace “v.go(150)” with “v.go(50)” to speed up the transition. Now you should hear “burl”. By commenting out the first “ga := 30”, you should hear “pearl”.

While I won’t go over every consonant recipe here, as I’d have to rediscover them, myself (thereby depriving you of the pleasure), we should cover the nasals m and n. Let’s try to say the word “monster”. That way, we can slip in the s and t sounds to utilize the FRICATION section, as well as the NASAL, completing our tour of VocalTract’s parameters.

First, I’ll say “monster” very slowly in a froggy voice:

After I said “monster”, our two-year-old daughter repeated “monster” unexpectedly. You can see from those pitch lines how shrill her voice is.

Looking at the spectograph, you can see the initial m, followed by the open-mouth o and then the n. After those nasal-vowel-nasal sounds, you can see the tall noise burst of the s, followed by silence, then the brief t burst going into the r. The m and n sounds are enhanced by using the nasal anti-resonator. It sucks up in-band energy, rather than accumulate it, like the formant resonators do. This creates a limited-spectrum vacuum which can make things sound nasally.

By studying the spectograph and incrementally building the word, I made a new “PRI talk” method to say “monster” (see Example4.spin):

PRI talk

setformants(170, 1100, 2600, 3200)    'set formants for M

nf := 2900 * 100 / 1953               'set nasal for M

na := 255

v.go(1)

aa := 10                              'ramp up M

ga := 30

v.go(150)

setformants(700, 900, 2800, 4500)     'set formants for O

na := 0                               'nasal off

v.go(50)                              'transition to O

v.go(150)                             'sustain

setformants(200, 1550, 2500, 4500)    'set formants for N

nf := 1550 * 100 / 1953               'set nasal for N

na := 255

v.go(100)                             'transition to N

v.go(100)                             'sustain

aa := 0                               'ramp down N

ga := 0

v.go(100)

na := 0                               'nasal off

ff := 120                             'set frication for S

v.go(1)

fa := 10                              'ramp up S

v.go(150)

fa := 0                               'ramp down S

v.go(75)

ff := 80                              'set frication for T

v.go(50)                              'sustain silence briefly

fa := 10                              'ramp up T

v.go(20)

setformants(580, 1200, 1500, 4700)    'set formants for R

fa := 0                               'ramp down T

v.go(20)

aa := 10                              'ramp up R

ga := 30

v.go(50)

v.go(150)                             'sustain

aa := 0                               'taper down to silence

ga := 0

v.go(100)

v.go(500)                             'pause

The timing of the s and t sounds took me a lot of trial and error to get tuned right. Even so, you can see that the amount of data involved in synthesizing words is quite low compared to making audio recordings of them (less than 1%).

To utilize VocalTract, it will take some experimentation on your part. Once mastered, though, you will have gained untold insight into the mechanics of speech that you’ll be eager to share with others who will be less interested. As a side benefit, you will be able to add speech to your Propeller applications using minimal resources.

In parting, here is an odd application that uses the StereoSpatializer object to mix and place four separate VocalTract’s into a stereo image. This demonstrates how the Propeller’s multiple processors can be easily put to use in a cooperative fashion to perform complex tasks of trivial value.

Top object called “SpatialBlah” (see SpatialBlah.spin):

CON

_clkmode      = xtal1 + pll16x

_xinfreq      = 5_000_000

voices        = 4                         'voices can be 1..4

buffer_size   = $1000                     'spatializer buffer size

OBJ

v   [voices]  : "Blah"

s             : "StereoSpatializer"

VAR

word  input[4], angle[4], depth[4], knobs 'spatializer parameters

long  buffer[buffer_size]                 'spatializer delay buffer

PUB start | i, r

repeat i from 0 to voices - 1             'start voices and connect

input[i] := v[i].start                  '..to spatializer inputs

knobs := %000_011_100_101                 'start spatializer

s.start(@input, @buffer, buffer_size, 11, -1, 10, -1)

repeat                                    'scan for done voices

repeat i from 0 to voices - 1

if v[i].done                          'if voice done

angle[i] := ?r                      '..set new random angle

depth[i] := ?r & $FFF               '..set new random depth

v[i].go                             '..start new "blah"

Lower object called “Blah” (see Blah.spin):

OBJ

v     : "VocalTract"

VAR

byte  aa,ga,gp,vp,vr,f1,f2,f3,f4,na,nf,fa,ff  'vocal tract parameters

PUB start

v.start(@aa, -1, -1, -1)                  'start tract, no pin outputs

return v.sample_ptr                       'return tract sample pointer

PUB go

'say "blah" randomly

gp := rnd(60, 120)

vp := rnd(4, 48)

vr := rnd(4, 30)

setformants(100, 200, 2800, 3750)

v.go(rnd(100, 1000))

setformants(400, 850, 2800, 3750)

aa := 10

ga := 20

v.go(20)

v.go(80)

setformants(730, 1050, 2500, 3480)

aa := 20

ga := 30

v.go(50)

v.go(rnd(200, 1000))

aa := 0

ga := 0

v.go(100)

PUB done

return v.empty

PRI setformants(s1, s2, s3, s4)

f1 := s1 * 100 / 1953

f2 := s2 * 100 / 1953

f3 := s3 * 100 / 1953

f4 := s4 * 100 / 1953

PRI rnd(low, high)

return low + ||(?cnt // (high - low + 1))