QSX-24

Robotic Servo Controller

REV B 12/11/2008

Features/Specs

| Board Specs | Board Dimensions 2.25” X 2.95” Mounting Hole Dimensions 2” X 2.7” 4 Layer Board Max current rating 16 Amps |
| --- | --- |
| Microcontroller | Parallax Propeller Chip (P8X32A-Q44) |
| Clock Speed | 80 MHz |
| Memory | 64 KB EEPROM (32KB RAM + 32 KB ROM) |
| I/O | 24 Servo Outputs 2 PS/2 Inputs (Keyboard + Mouse) 1 RCA Jack Output 4 I2C Inputs 1 Crystal (Removable) 1 5V output rated at 500mA 1 3.3V output rated at 500mA 8 Pin I/O |
| Power | Max voltage input 9V |
| Indicators | Power LED |
| Programming | Parallax’s Prop Plug (#32201) Parallax’s USB2SER (#28024) |

The QSX-24 is a robotic/servo controller board.  Based off the Parallax Propeller chip #P8X32A the microcontroller contains 8 32-bit processors each with 2K of RAM.  With a HUB RAM of 32Kb which is common to all 8 processors (cogs).

A 5Mhz crystal allows for operation at up to 80Mhz with the correct PLL setting within the chip.  This allows the Propeller to run at 5, 10 , 20 , 40 , & 80 Mhz depending on the PLL setting.

There are two PS/2 ports that allow the use of a keyboard and/or mouse.  As well as a RCA output to display PAL/NTSC video formats.

12 servo outputs on each side allow for a total of 24 servos.  These servo outputs can also serve as inputs for sensors.

Board Description