- Biology; - Electronics; - Aesthetics; - Mechanics. "The BS2-IC is the single most popular BASIC Stamp module. Widely used in educational, hobby, and industrial applications. This module normally has no shortage of program space or I/O pins. Serial PC interface provides enhanced debug features." - Parallax BS2 ROVER (Updated: 10/09/2000) Born: 1999 Weight: ? Power: 4xAA & 1x9v Motors: Standard Hobby Servo Motor (x2) Cost: ~$200 This is my 1st Microcontroller based wheeled robot. His name is BS2 Rover. He's based on the very easy to use Parallax BS2-IC module. Parallax BS2-IC Module Specifications. Microcontroller:; PIC16C57 surface mount. Speed:; 20 MHz / 4,000 instructions per second. EEPROM:; 2K bytes (program and data). Program Length:; 500 lines of PBASIC. RAM (variables):; 32 bytes (6 for I/Os and 26 for variables). Input / Outputs:; 16 (up to 17 RS-232 communication ports). Source / Sink Current:; 20 mA / 25 mA. Serial Communication:; 300-50K baud I/O. Current Requirements:; 7 mA running, 50 uA in sleep. PC Interface:; Serial port. Package:; 24 pin DIP module. Size:; 1 3/16" L x 5/8" W x 3/8" H. Project Area:; Optional carrier board. BS2 Rover - Top; BS2 Rover - Bottom. BS2 Rover - Front; BS2 Rover - Side. Behavior and Observations 10/09/2000 - BS2 Rover isn't a BEAM robot at all. His "brain" is a Parallax Basic Stamp 2 Microcontroller. I'll post the source code next time I update the site. 01/06/2001 - I've added a sensor turret using a servo and a Sharp GP2D05 IR Sensor. Below is the current source code for the roverbot. '{$Stamp BS2} '************************************************************************ ' Brice D. Hornback - BS2 Roverbot !bdh Robot rev_e.bs2 ' 01/06/2001 '************************************************************************ ' ' Hardware Configuration ' ' pin 0 - Left Drive Servo ' pin 1 - Right Drive Servo ' pin 3 - IRODS Vin - Green Wire ' pin 5 - IRODS Vout - Blue Wire ' pin 6 - Sensor Turret CDS Photocell ' pin 7 - Temperature Sensor ' pin 8 - PIEZO Speaker ' pin 10 - LCD Display ' pin 12 - Sensor Turret Servo ' '************************************************************************ 'Initialize Values limit con 10 ' Bright Light N9600 con $4054 ' Baudmode for 9600 bps inverted I con 254 ' Instruction prefix value CLR con 1 ' LCD clear-screen instruction LINE2 con 192 ' Address of 1st char of 2nd line L1_C7 con 135 ' Address of line 1, character 7 fate VAR word 'random what-to-do-next variable choice VAR byte 'selector for branch ticks VAR byte 'amount of time to continue a course mot_L VAR word 'left-motor speed mot_R VAR word 'right-motor speed Rp CON 0 'I/O pin which the right motor is on. change accordingly Lp CON 1 'I/O pin which the left motor is on. change accordingly hafSec con 25 'Approx. number of ticks in a 1/2 second right_servo con 0 left_servo con 1 rcds var word ' OLD: rcds var byte cdsl var word ' CDS Left cdsc var word ' CDS Center cdsr var word ' CDS Right irl var byte irc var byte irr var byte temp var word ' Temperature Value w6=1 ' Bump Sensor Not Detected Value '************************************************************************ 'Initialize Robot serout 10,N9600,[I,CLR] ' Clear the LCD screen pause 1 gosub beep pause 100 gosub beep '************************************************************************ 'MAIN ROUTINE gosub INIT_SENSOR_TURRET start: gosub SCAN_SENSORS pause 800 MOVE: if cdsc < cdsl AND cdsc < cdsr AND in5<>0 then GOFORWARD if cdsl < cdsr AND in5<>0 then GOSPINLEFT if cdsr < cdsl AND in5<>0 then GOSPINRIGHT goto BUMP return GOFORWARD: gosub FORWARD goto start GOSPINLEFT: gosub SPINLEFT goto start GOSPINRIGHT: gosub SPINRIGHT goto start end '************************************************************************ 'SUB-ROUTINES SCAN_SENSORS: gosub CHECK_TEMP gosub SENSOR_SERVO_FORWARD gosub SENSOR_SERVO_LEFT gosub IRODS_DETECT_L gosub CDS_LEFT gosub SENSOR_SERVO_FORWARD gosub SENSOR_SERVO_RIGHT gosub IRODS_DETECT_R gosub CDS_RIGHT gosub SENSOR_SERVO_FORWARD gosub IRODS_DETECT_C gosub CDS_CENTER gosub DISPLAY_CDS return IRODS_DETECT_L: High 3 Pause 3 Low 3 Pause 27 irl=IN5 If IN5=0 Then DETECTED serout 10,N9600,[I,CLR] ' Clear the LCD screen pause 1 serout 10,n9600,["Area Clear"] ' Print message 'pause 800 return IRODS_DETECT_C: High 3 Pause 3 Low 3 Pause 27 irc=IN5 If IN5=0 Then DETECTED 'serout 10,N9600,[I,CLR] ' Clear the LCD screen 'pause 1 'serout 10,n9600,["Area Clear"] ' Print message 'pause 800 return IRODS_DETECT_R: High 3 Pause 3 Low 3 Pause 27 irr=IN5 If IN5=0 Then DETECTED serout 10,N9600,[I,CLR] ' Clear the LCD screen pause 1 serout 10,n9600,["Area Clear"] ' Print message 'pause 800 return DETECTED: ' Object Detected Debug "Object Detected",cr Gosub BEEP goto LCD_IRODS_DETECTED LCD_IRODS_DETECTED: ' Display Object Detected on LCD serout 10,N9600,[I,CLR] ' Clear the LCD screen pause 1 serout 10,n9600,["Object Detected"] ' Print message pause 100 serout 10,N9600,[I,CLR] ' Clear the LCD screen pause 1 gosub random_fate return CHECK_CDS: 'check turret CDS ambiant light high 6 ' discharge capacitor pause 1 ' for 100ms RCTIME 6,1,rcds ' check pot setting return CDS_LEFT: 'check turret CDS ambiant light LEFT high 6 ' discharge capacitor pause 1 ' for 100ms RCTIME 6,1,cdsl ' check pot setting return CDS_CENTER: 'check turret CDS ambiant light CENTER high 6 ' discharge capacitor pause 1 ' for 100ms RCTIME 6,1,cdsc ' check pot setting return CDS_RIGHT: 'check turret CDS ambiant light RIGHT high 6 ' discharge capacitor pause 1 ' for 100ms RCTIME 6,1,cdsr ' check pot setting return DISPLAY_CDS: 'serout 10,N9600,[I,CLR] ' Clear the LCD screen 'pause 1 'serout 10,N9600,[I,LINE2,"Light Level: ",DEC rcds] ' Print message serout 10,N9600,[I,LINE2,"L:",DEC cdsl," C:",DEC cdsc," R:",DEC cdsr] 'pause 800 return SENSOR_SERVO_FORWARD: out12 = in12 for w7=1 to 50 pulsout 12,770 'forward pause 20 next return SENSOR_SERVO_LEFT: for w7=1 to 20 pulsout 12,1200 'left pause 20 next return SENSOR_SERVO_RIGHT: for w7=1 to 20 pulsout 12,350 'right pause 20 next return BEEP: 'Alert Beep Signal low 8 pause 50 high 8 return INIT_SENSOR_TURRET: out12 = in12 for w7=1 to 50 pulsout 12,770 'forward pause 20 next return FORWARD: for b3=1 to 60 pulsout left_servo, 1200 pulsout right_servo, 1200 pause 20 gosub IRODS_DETECT_C gosub CHECKBUMP next return BACKWARDS: for b4=1 to 61 pulsout left_servo, 650 pulsout right_servo, 650 pause 20 next return SPINLEFT: for b3=1 to 20 pulsout left_servo, 1200 pulsout right_servo, 650 pause 20 next return SPINRIGHT: for b4=1 to 20 pulsout left_servo, 650 pulsout right_servo, 1200 pause 20 next return TURNLEFT: for b3=1 to 40 pulsout right_servo, 650 pause 20 next return TURNRIGHT: for b4=1 to 40 pulsout left_servo, 650 pause 20 next return CHECKBUMP: BUTTON 9,1,255,250,w6,1,BUMP return BUMP: serout 10,N9600,[I,CLR] ' Clear the LCD screen pause 1 serout 10,N9600,["Ouch!..."] ' Print message w6=1 gosub BACKWARDS if cdsl > cdsr then GOSPINLEFT GOTO GOSPINRIGHT CHECK_TEMP: 'check temperature sensor high 7 ' discharge capacitor pause 1 ' for 100ms RCTIME 7,1,temp ' check pot setting serout 10,N9600,[I,CLR] ' Clear the LCD screen pause 1 serout 10,n9600,["Temperature Value: "] ' Print message serout 10,N9600,[I,LINE2,"L:",DEC temp] pause 2000 return RANDOM_FATE: random fate 'Pick a Random Number choice = fate & %1111 branch choice,[spinleft,spinleft,backwards,spinright,spinright] return '************************************************************************ END. HOME

NOTICE:  I have tried my best to give credit where possible.  If I have forgotten someone or something,
          or messed something up, please don't have a cow... just send me an email and I'll fix it.  :-)

Quotes used throughout this site were taken from
"Living Machines" by Brosl Hasslacher and Mark Tilden.

COPYRIGHT � 2000-2004 BRICE D. HORNBACK.  ALL RIGHTS RESERVED.
Nervous networks copyrighted � and patented by Mark Tilden
Commercial use of these circuits requires permission of the patent holder.