# A Configurable Propeller Boot-Loader

# Steve Denson (jazzed) July 2008

A Configurable Propeller Boot-Loader	1

Steve Denson (jazzed) July 2008	1

1	Overview	2

2	Boot.spin	3

2.1	Boot Features	3

2.2	The Boot Process	3

2.3	Boot-loader Design	3

3	Bootconf.spin	4

3.1	Configuration Features	4

3.2	Configuration Interface Interaction	4

3.2.1	Scripted Configuration	4

3.2.2	Bootconf Command Reference	6

4	Sdxmodem.spin	8

5	GetConfig.spin Module for Applications	9

5.1	Module Initialization	9

5.2	API for Accessing Configuration Data	10

# Overview

This document describes a configurable boot-loader and other tools for use with Propeller. A boot loader loads and runs a program after power on or reset.

Normally a boot loader will have a configuration command set so that the tool is self-contained. In the case of Propeller and its limited memory, the boot program and the configuration program are mostly separate.

The boot program can interact with the user in simple and convenient ways (getting the base pin number for SD Card or allowing alternative boot methods) to ensure that booting the desired application can succeed.

The separate bootconf.bin boot configuration program allows control over the non-volatile boot configuration saved in EEPROM. This configuration is not lost between power-cycles, but is lost if the boot code is reprogrammed in EEPROM.

Another program that is useful for basic operations is a downloader. Xmodem code is included for downloading programs or other files from your PC via a serial port to the SD card. A PC application like Hyperterm or other xmodem client is required for this.

The three spin embedded applications built from “boot.spin”, “bootconf.spin”, and “sdxmodem.spin” are described in the rest of this document.

In addition to the applications, the “GetConfig.spin” module is provided for other applications to access configuration data.

Today, the boot-loader takes up about 6KB of memory for byte-codes, variables, and stack. This leaves ~26KB for the user boot application byte-code. Another loader is available that would leave most if not all memory free for user boot applications, and may be included at some point. Until an ideal loader is found (which for this author does not exist today), users can consider the memory map below.

Table 1 Boot-loader Memory Map

| Section | Start Addr | End Addr | Notes |
| --- | --- | --- | --- |
| Config Data | $C0 | $1AF | Contains Configuration Data |
| Loader Bytecode | $1B0 | $1178 | Available for user app variables |
| Loader Stack | $7C94 | $7FFF | Available for extra user stack |

“Improvements”

The current boot-loader uses a smaller but slower CRC16 calculation method. The CRC16 result does not follow a standard, but should be just as effective for validating data. A smaller but slower SD card loader is used to load programs now. The previous user application size was about 25KB; it is now a little more than 26KB.

# Boot.spin

The “boot.spin” boot-loader (from boot.zip or the boot directory) will run a spin binary from SD card using configuration information stored in EEPROM.

## Boot Features

- By default the console serial port on pins 30/31 is set to 115200 bps.

- User can specify SD card base-pin at first boot.

- The boot program will load and run an application automatically.

- The boot process can be interrupted to run an application manually.

- The boot loader program takes up about 7K bytes of memory.

- Up to 25K bytes of code/data space is available for applications.

- Standard features of “fsrw.spin” apply (FAT16, 12 letter names, etc…)

- Configuration data is loaded by Propeller ROM, not separate EEPROM code.

## The Boot Process

- After first programming “boot.spin” to EEPROM, the boot image will ask for the base-pin for an attached SD card (the base-pin is the DO pin or first of four connections described in the “sdspiqasm.spin” file). A programmer may enhance this to include individual pins if required.

- The default image to load on the SD card is "bootconf.bin" built from “bootconf.spin” in the “bootconf.zip” archive or bootconf directory. This means "bootconf.bin" must be on the SD card for the configuration program to start up. The “sdxmodem.bin” application should also be added to SD card.

- After any power on reset or soft reboot, the boot code will check for a keystroke from the serial port.

  - If no keystroke is detected, the boot loader will run the predefined program saved in the config space in the variable cfg_sdboot_fname unless the first character in the variable is a space $20.

  - If a "c" is detected, “bootconf.bin” will be run if found on SD card.

  - Any other key character will cause the boot program to display the SD card directory and prompt for the image to boot with “bootfile?”

## Boot-loader Design

The “boot.spin” boot-loader needs to be small. Some software shortcuts are used.

One shortcut example is other applications might use the “GetConfig.spin” module to make accessing configuration fields easier; the boot-loader could use the module too, but would be bloated by doing so, and uses bootdef.spin defines instead.

Another example is the use of the boot-loader’s DAT section to store configuration information. This allows the Propeller boot-rom to load the data for us instead of including EEPROM.spin in the boot-loader. The config address must be right though.

The configuration data structure is made up of two parts: 1) a header and 2) data.

The header should remain fixed and be published as Version-Major 1. The data may be different for various applications and can be controlled by Version-Minor (the lower 16 bits of the version field) and is 1. The first published version is $10001.

# Bootconf.spin

The bootconf program (bootconf.zip or bootconf directory) will let the user change the contents of the EEPROM configuration space.

## Configuration Features

- Provides an interactive interface with a help menu.

- Data is CRC16 protected and allows for checking for valid configuration.

- A fixed header contains size, version, boot-mode, and boot-file name.

- Header can be used to find the CRC16 address if the data-size changes.

- Configuration variable fields can be changed with a step-wise tool.

- Variable name display also provides field length, address, and content.

- Bootconf uses separate EEPROM code to access configuration data.

## Configuration Interface Interaction

A command line interface allows the user to perform various functions.

### Scripted Configuration

Boot configuration may be setup using the Bootconf “C” command. Other commands are provided for utility in the Bootconf menu.

Using the “C” command, the application presents a set of variable names along with expected byte data length, EEPROM configuration addresses, an optional announcement of what the setting means, and the current setting. A sample display is shown below.

At each “>“ prompt for the scripted configuration, the user should enter the value to store in the EEPROM configuration space. If the key entry is keyboard “Enter” or “Return”, the setting will not be changed.

For values that are strings, a “space” and “enter” combination can be used to “erase” the configuration variable content. It is up to the user program to interpret this data.

For values that are numbers such as pin numbers, the pin name and “[#/n]” are shown at the prompt to remind users that a number or the letter “n” are expected for the setting. If letter “n” is set for a pin number field, the user program can interpret this value as not used and a given object should not be used.

The NTSC setting shows choice of “[Y/n]” as a Boolean selection.

The first few fields of the configuration space are read-only header fields.

bootconf> C

cfg_size:4:00C0: F0

cfg_version:4:00C4: 0

cfg_sdpins:4:00C8:

cfg_sdpins:1:C8:SD Card Basepin [#/n] 12>

cfg_baud_rate:4:00CC: 115200>

cfg_sdboot_fname:16:00D0: >

cfg_pingrp1:8:00E0:

cfg_pingrp1:1:E0:TV  Basepin [#/n] n>

cfg_pingrp1:1:E1:VGA Basepin [#/n] n>

cfg_pingrp1:1:E2:NTSC? [Y/n] n>

cfg_pingrp1:1:E4:Mouse DAT [#/n] n>

cfg_pingrp1:1:E5:Mouse CLK [#/n] n>

cfg_pingrp1:1:E6:Keybd DAT [#/n] n>

cfg_pingrp1:1:E7:Keybd CLK [#/n] n>

cfg_pingrp2:8:00E8:

cfg_pingrp2:1:E8:ENET CS  [#/n] n>

cfg_pingrp2:1:E9:ENET SCK [#/n] n>

cfg_pingrp2:1:EA:ENET SI  [#/n] n>

cfg_pingrp2:1:EB:ENET SO  [#/n] n>

cfg_prodname:16:0120: >

cfg_userid:16:0130: >

cfg_passwd:16:0140: >

cfg_hostid:16:0150: >

cfg_ipv4_address:4:0160: 0.0.0.0>

cfg_ipv4_netmask:4:0164: 0.0.0.0>

cfg_ipv4_gateway:4:0168: 0.0.0.0>

cfg_ipv4_dns:4:016C: 0.0.0.0>

cfg_macaddr:16:0170: 00.00.00.00.00.00>

cfg_bootp_fname:44:0180: >

cfg_crc16:4:01AC: 58D5

bootconf>

Figure 1 Bootconf Scripted Output

The “S” command can be used to show the configuration contents. To save or write the configuration, the “W” command can be used. The “q” command will cause the system to reboot, but will prompt the user to write the configuration if necessary. The “L” command can be used at any time the bootconf> prompt appears to load configuration from EEPROM.

Note: while the address for bootconf fields are shown as EEPROM address offsets, the actual working configuration is saved in RAM until written by the user. The bootconf RAM address $C0-$1B0 will never have configuration information.

The key variable is cfg_sdboot_fname. When this shows in the configuration process, enter the SD card file name that you want use for loading after reset.

### Bootconf Command Reference

a: set address for data read/write

The point of the “a” command is to set an address within the configuration space for read or write. This is used in conjunction with “s”, “d”, “r”, and “w” commands.

s: set string data

The point of the “s” command is to specify string data to be written to an address by the “w” command.

d: set number

The point of the “d” command is to specify string data to be written to an address by the “w” command.

r: read one RAM address

This command is used to display information contained in one config RAM address.

w: write one RAM address

This command is used to write information to one config RAM address.

D: dump SD directory

This command is used to show the SD card file contents.

C: start config utility

This command is used to start the scripted configuration tool.

S: show RAM data and config fields

This command is used to display information contained in the configuration RAM.

E: erase EEPROM space

This command is used to erase information contained in the EEPROM configuration.

L: load EEPROM

This command is used to load information from EEPROM configuration to RAM.

N: make new config in RAM

This command creates a new configuration in RAM.

W: write RAM config to EEPROM

This command writes the RAM configuration to EEPROM non-volatile storage.

?: show help and addr/data settings

This shows command help.

bootconf> ?

a: set address for data read/write

s: set string data, d: set number

r: read one RAM address

w: write one RAM address

D: dump SD directory

C: start config utility

S: show RAM data and config fields

E: erase EEPROM space, L: load EEPROM

N: make new config in RAM

W: write RAM config to EEPROM

?: show help and addr/data settings

q: reboot after optionally writing settings

Current r/w settings:

addr $4130, data $00000000

bootconf>

Figure 2 Bootconf Help Menu

q: reboot after optionally writing settings

This command will force Propeller to restart. It will ask user if configuration should be written to EEPROM before reboot. Please avoid writing to EEPROM too often as writing too many times can cause hardware failures.

# Sdxmodem.spin

The sdxmodem program (sdxmodem.zip or sdxmodem directory) lets users download programs to SD card using Hyperterm or xmodem client application.

Bootloader started.

Key hit

HELLO.BIN

SDXMODEM.BIN

BOOTMON.BIN

WWWPROP.BIN

BOOTCONF.BIN

Bootfile? sdxmodem.bin

Boot from SD Card pin 12

File sdxmodem.bin size 6024

SD Card Directory:

HELLO.BIN

SDXMODEM.BIN

BOOTMON.BIN

WWWPROP.BIN

BOOTCONF.BIN

Ready to receive file via XMODEM.

Input File Name:

Figure 3 Sdxmodem Startup

If “sdxmodem.bin” is on the SD card, it can be used to download other programs.

This is done via the serial port. When the bootloader first starts, it checks for a character on the serial port. If a character other than “c” is hit within a second of the “Bootloader Started” announcement, “Key hit” and the SD card contents will be displayed. Enter “sdxmodem.bin” at the “Bootfile?” prompt to get started.

The “sdxmodem.bin” program can also be specified as the program to load with the “bootconf.bin” application in the cfg_sdboot_file variable, but is not recommended.

Downloading a File

After seeing “Input File Name:” and entering the file name, the xmodem negotiation will begin to let you start the download from Hyperterm or your xmodem client.

# GetConfig.spin Module for Applications

A spin module “GetConfig.spin” is available for loading and interpreting configuration data. This module is used by both sdxmodem and bootconf to get the console baud rate. Using the GetConfig.spin module is a two part process where 1) the init method is called to see if data is valid and 2) accessor methods are called to get data.

## Module Initialization

Most spin modules need some initialization (those that do not are generic code or drivers that burden the user to carry around parameters). The GetConfig.spin module needs a call to the “init” method to make the object usable. The “init” method will also return an error code of non-zero if the configuration data appears to be invalid. One way to handle this is to warn the user and halt the program since the system is “broken” at that point.

OBJ

pd : “PinDefs”

fds: “FullDuplexSerial”

cfg: “GetConfig”

PUB main

if cfg.init <> 0

fds.start(pd#SERIAL1_RX, pd#SERIAL1_TX, pd#SERIAL1_MODE, pd#SERIAL1_BAUD)

waitcnt(clkfreq+cnt)

fds.str(string(CR,"EEPROM Config Error",CR))

repeat

rc := fds.start(pd#SERIAL1_RX, pd#SERIAL1_TX, pd#SERIAL1_MODE, cfg.getBaudRate)

waitcnt(clkfreq+cnt)

printf(string(CR,"Serial port started on COG %d"),parm(rc))

Figure 4 Using GetConfig.spin

PUB  init

The init method initializes the module and returns non-zero on error.

## API for Accessing Configuration Data

Once the module is successfully initialized, configuration data may be read with accessor functions. Data is string or numeric. In most cases an accessor ending with “Ptr” suffix indicates a string (the exception is the MAC address which is an oddly sized 6-tuple number stored MSB first). All accessors without “Ptr” suffix return numbers. The list of default accessors with a brief explanation is given below.

PUB  getConfigSize

Config size is a long that we can use for finding the CRC16.

PUB  getVersionMajor

Version major is a word that describes the config data structure header version.

PUB  getVersionMinor

Version minor is a word that describes the config data structure contents version.

PUB  getSdBasePin

SD basepin is a byte that defines the start pin for the SD card interface.

Other SD pins can be defined in this series of 4 bytes if necessary.

PUB  getBaudRate

Baud rate is the rate that programs can use for console i/o.

PUB  getSdBootFilePtr

The SD boot file is the name of the file for the bootloader to run after power up or reset.

PUB  getTvBasePin

TV basepin is the first pin in a series to use for the TV DAC.

PUB  getVgaBasePin

VGA basepin is the first pin in a series to use for the VGA interface.

PUB  getNtscPal

NTSC/PAL is a flag that allows selecting NTSC or PAL for the video standard.

PUB  getExtraVideo

Extra video is just an extra byte in config data.

PUB  getMouseDat

This defines the mouse data interface pin.

PUB  getMouseClk

This defines the mouse clock interface pin.

PUB  getKeybdDat

This defines the keyboard data interface pin

PUB  getKeybdClk

This defines the keyboard clock interface pin.

PUB  getEnetCS

This defines SDIO based Ethernet Chip Select interface pin.

PUB  getEnetSCK

This defines SDIO based Ethernet clock interface pin.

PUB  getEnetSI

This defines SDIO based Ethernet serial in interface pin.

PUB  getEnetSO

This defines SDIO based Ethernet serial out interface pin.

PUB  getProdNamePtr

Product name is a user defined product string.

PUB  getUserIdPtr

User ID is for network or other login identification.

PUB  getPasswdPtr

Password is for network or other login identification.

PUB  getHostIdPtr

Host ID is for network or other login identification.

PUB  getIPv4_addr

IPv4 address is a 32 bit word that defines the device IP address.

PUB  getIPv4_netmask

IPv4 netmask is a 32 bit word that defines the local IP address access range.

PUB  getIPv4_gateway

IPv4 gateway is a 32 bit word that defines the IP address of an internet gateway server.

PUB  getIPv4_dns

IPv4 dns is a 32 bit word that defines the IP address of the domain name server for host/address resolution.

PUB  getMacAddrPtr

A MAC address is a 6 byte word; a pointer is returned so users can deal with it as they like.

PUB  getBootpFnamePtr

The bootp fname is the file name of a remote program to use for booting the device. This is a future feature.

PUB  getCRC16

CRC16 is the data validator for the configuration space. An invalid CRC16 means config data is not usable.