'Read Port WithRC Time  control servo with pot test from JP

' {$STAMP BS2}

' {$PBASIC 2.5}

PAUSE 1000

DEBUG "program Running"

'time VAR Word

'DO

'HIGH 7

'PAUSE 10

'RCTIME 7,1, time

'time = time */ 185

'time = time + 500

'PULSOUT 14, time

'DEBUG HOME, DEC5 time

'LOOP

time VAR Word

DO

HIGH 7

PAUSE 10

RCTIME 7, 1, time

time = time */ 550 ' Scale by 0.724 (X 256 for */).

time = time + 400 ' Offset by 500.

PULSOUT 14, time ' Send pulse to servo.

LOOP