'

' {$STAMP BS2}

' {$PBASIC 2.5}

'

' ==============================================================================

'

'   File....... MAWDBOE_w_CO2_WORKS_OK - June 25, 2010.BS2E

'

'   Purpose.... Humidity, temperature, altitude, and carbon dioxide data logging

'   Rocketeers. Andrew (am), Sean (sd), Dylan (dl), Justin (ja), Mike (mp)

'   Faculty....    Mark Kibler (mk)

'   Peanut gallery... Tracy Allen (ta), Paul Smith (Sylvie) (ps-s)

'   E-mail..... mkibler@alumni.unity.edu

'   Started.... February, 2008 & repurposed June, 2010

'

'

' ==============================================================================

' -----[ Program Description ]---------------------------------------------

'

' ASP-2/MAWDBOE program with a Parallax CO2 sensor: The Air Sampling Probe (ASP-2) measures and records

' temperature, humidity, altitude, CO2 concentration, flight time, sand sample number. The MAWD is a

' PerfectFlite brand "Mini Altimeter With Deployment" feature wire into a Parallax Board of Education.

'

' -----[ Revision History ]------------------------------------------------

'

' -- 08 March 2008 (ta, pk)

' 1) added header section delineating the Vinculum thumb drive record structure

' 2) added subroutines for Vinculum init and file handling

'    V_synchronize, V_check_disk, V_short_commands -- these are initialization

'    V_getdata, V_purge   these two are the same routine, sometimes data simply purged

'    V_open_file, V_close_file,   these are in the main LOOP

' 3) forging the disk, a startup message shows how many tries TO access the disk

' 4) if the data file is opened successfully, "DaedelusCRLF" is written to the file

' 5) added one Word variable to hold more status flags or data.  Buffer is 12 bytes

' 6) LED flashes briefly each time through the loop.

'

'-- 06 March 2008 (mk, am, mw, bl, ds, jr, nm)

' 1) Changed data sampling time at descent to 1 sec intervals (from 2>10 to 2>100) per (ta)

' 2) Commented out 'GOSUB morse reading' twice in 'main:' subroutine per (ta)

' 3) Shortened duration of 'motormovement' times so we could bench test the program. These will be changed

'    back to their original 1 meter distance (0 TO 681) when we finalize the program.

' 4) The Rocketeers experimented by changing various settings to see what would happen,

'    and marveled at the Morse code light. Bandit the Rocket Dog was little help.

'

'-- 05 Mar 2008  (ta)

' Added increment of sample counter index2.  Displays on debug.  Land flag

' is set by either elapsed time or index2 accumulation.

'

' -- 04 Mar 2008  (ta) Mar04a

' Major restructuring from template, variables, starting point "RocketRobot_Feb29.BS2"

' Streamlined the Sensirion and movement routines with subroutine structure.

' Consolidated variables, named all pins

' This version incorporates the clock, LED and movements, but not yet the datalogger

'

'-- 01 July 2009 (mk)

' 1) 'Commented out' (not deleted) robot movement commands (servomotor movements) and inputs

' 2) 'Commented out' Morse code program for servomotor movement - Kept Morse code program for sync and disk tries

'

' -- 9 July 2009, (ta)

' 1) added device pin numbers in () in comments

' 2) made initial debug message a subroutine, "init_message"

' 3) added code to return a data value for MAWD, and to show it on debug screen after %RH

'    initially this is initially a dummy value, 12345 and RETURN, to test program loop and logging

'    added word VARiable "feet", PIN "MAWDin" on p3, and subroutine "MAWD_get"

'    call to MAWD_get inserted in MAIN loop.

' 4) increased size of data string for USB drive to 29 bytes, added field for feet of altitude

' 5) changed DS pins to p7,p6,p5 instead of p2,p5,p7

' 6) added means to synchronize data recording with DS1302 RTC

'    added CONstant "logInterval" (allowed intervals in seconds is 1,2,3,4,5,6,10,12,15,20,or 30)

'    e.g. if logInterval=5 then data will be logged at 5 second intervals

'    added subroutine sync_RTC, and MAIN calls sync_RTC instead of read_RTC

'    new VARiable second0 holds a comparison value to track the change in seconds

'

' -- 10 July 2009, (ta)

' 1) debugged subroutine MAWD_get using 2nd BASIC Stamp to simulate MAWD

'

' -- 25 June 2010, (mk)

' 1) Defined analog-digital converter (ADC) I/O pins for wires that go to the ASP-BOE.

' 2) Added Tracy and Andrew's 'MCP_get:' subroutine

'

' -- 29 June 2010, (mk)

' 1) Defined ALM - PIN 3

' 2) Defined CNTL - PIN 4

'

' -- 30 June 2010 (ta), (mk)

' 1) Condensed VAR 'result0, result1, mVolts0, and mVolts1' into a shorter,

'     more effficient 'MCP_get' subroutine (ta)

' 2) Commented out 'HIGH 4' command which was causing erratic CO2 mV data (mk)

' 3) Added column headings for 'CO2 mV' and 'CO2 ppm' (mk)

' 4) Commented out the 'carbon collect' command that made ALARM status 0 and 1 appear (ck)

' ==========================================================================================

' -----[ I/O Definitions ]-------------------------------------------------

ShtDta  PIN 1        ' bi-directional data with pullup resistor SHTdta=(1)

ShtClk  PIN 0        ' clock (for Sensirion) with pulldown resistor SHTclk=(3)

' power (4)=Vss 4, (8)=Vdd

MAWDin   PIN 15      ' mk changed PIN 3 to PIN 15 - July 11 - input from altitude, 9600 baud inverted, 20 per second, 5 bytes.

DSclk    PIN 2       ' DS1302 RTC clock=(7)    ' ta moved DS pins to 7,6,5 instead of 2,5,7 - mk moved back to 2, 5, 7 - July 11

DSdta    PIN 5       ' DS1302 RTC data line=(6)

DSce     PIN 7       ' DS1302 RTC chip enable (high enable)=(5)

' power to DS1302 Vdd=(1), Vss=(4), Vbat=(8)

TX     PIN 8         ' Transmit Data   --> 27937.4 (RXD) output to Vinculum=(4)

RTS    PIN 9         ' Request To Send --> 27937.6 (CTS) to Vinculum=(6)

RX     PIN 10        ' Receive Data    <-- 27937.5 (TXD) from Vinculum=(5)

CTS    PIN 11        ' Clear To Send   <-- 27937.2 (RTS) from Vinculum=(2)

' power (1)=Vss, (3)=Vdd, (7)=n/c, (8)=ri

' ----- [ MCP I/O definitions ] -------

ALM      PIN 3       ' Wire comes from the CH0 pin on the ADC breadboard to TP1 on the CO2 sensor.

CNTL     PIN 4       ' Wire comes from the CH1 pin on the ADC breadboard to TP3 on the CO2 sensor.

DataOut  PIN 6       ' DataIn and DataOut ADC pins are jump-connected with 1 K ohm resistor.

'   A separate wire runs from each ADC pin. DataIn and DataOut wires

'   are connected into one wire. This single wire goes into p6 on the ASP-BOE.

DataIn   PIN 6       ' DataIn and DataOut ADC pins are jump-connected with 1 K ohm resistor.

'   A separate wire runs from each ADC pin. DataIn and DataOut wires

'   are connected into one wire. This single wire goes into p6 on the ASP-BOE.

CS       PIN 12      ' Wire comes from the CS/SHDN pin on the ADC. It goes to BOE p12.

Clock    PIN 13      ' Wire comes from the CLK pin on the ADC. It goes to BOE p13.

' -----[ Constants ]-------------------------------------------------------

vBaud           CON     84      ' Serial Baud Rate 9600 bps for Vinculum datalogger

ShtTemp         CON     %00011  ' read temperature command for Sensirion

ShtHumi         CON     %00101  ' read humidity command for Sensirion

Ack             CON     0       ' responses for Sensirion

NoAck           CON     1

logInterval     CON     1       ' choose 1,2,3,4,5,6,10,12,15,20,30 seconds

DegSym          CON     186     ' degrees symbol for DEBUG

Cnts2Mv         CON     $0139   ' x 1.22 (To Millivolts)

' -----[ Variables ]-------------------------------------------------------

CO2pp      VAR Word              ' new variable: Tracy - June 11, 2010

primo      VAR Word

iobyte   VAR primo.BYTE0       ' io to/from datalogger, humidty sensor, & debug

index    VAR primo.NIB2        ' loop counter in some routines

unused1  VAR primo.NIB3

secundo    VAR Word

flags    VAR secundo.BYTE0

ackbit  VAR flags.BIT0       ' acknowledge Sensirion

shtErr  VAR flags.BIT1       ' = 1 if error in sensirion read

dskErr  VAR flags.BIT2       ' =1 if the logger fails to work

land    VAR flags.BIT3       ' =1 after alloted time interval

' bits 4 to 7 are unused

second0 VAR secundo.BYTE1

waldo      VAR Word             ' scratch variable

soSHT    VAR waldo            ' alias as scratch variable for Sensirion

soSHT0   VAR soSHT.BYTE0      ' byte by byte

soSHT1   VAR soSHT.BYTE1

delay    VAR waldo            ' timeout for Sensirion

' motormovementtime  VAR waldo

' alias as timer variable for movements

degC       VAR Word             ' temperature in Celsius*10 from Sensirion

rh         VAR Word             ' humidity in percent*10 from Sensirion

feet       VAR Word             ' reading from MAAWD altimeter

time0      VAR Word

second   VAR time0.BYTE0      ' from DS1302 RTC

minute   VAR time0.BYTE1      ' ditto

time1      VAR Word

hour     VAR time1.BYTE0    ' ditto

index2   VAR time1.BYTE1    ' extra index for loop count

buffer     VAR Word           ' for data returned from logger or RTC

buf0     VAR buffer.BYTE0   ' will return as bytes

buf1     VAR buffer.BYTE1

' note the buffer can extend into unassigned space, take care.

CO2mV     VAR    Word         ' one more variable,(at end because didn't know if it would interfere if placed in beginning or middle) August 13, 2010 JA

'---- [ carbon   VAR Word ] ----

'result0    VAR Word           ' Conversion Result CH0

'result1    VAR Byte            ' Conversion Result CH1

'mVolts0    VAR Word           ' Result0 --> mVolts

'mVolts1    VAR Byte            ' Result1 --> mVolts

' -----[ EEPROM Data ]-----------------------------------------------------

'

'

' -----[ Thumb Drive Data Format ]-----------------------------------------

'

' data format with RTC clock time included

'

' 2 bytes hour

' 1 byte ":" <---- colon time separator

' 2 bytes minute

' 1 byte ":" <-- okay

' 2 bytes second

' 1 byte ","   <--- comma as field separator

' 1 byte sign, either - or space

' 3 bytes temperature integer part

' 1 byte "."

' 1 byte temperature decimal part

' 1 byte ","

' 3 bytes humidity integer part

' 1 byte "."

' 1 byte humidity decimal part

' 1 byte ","

' 5 byte altitude from MAWD

' 1 byte CR   <---- CRLF as record separator

' 1 byte LF

' 1 byte CR for the Vinculum terminator, this does not appear in the data file

' =

' 29 bytes total for Vinculum character count, 23 bytes per record on Thummb drive.

' -----[ Initialization ]--------------------------------------------------

Reset:

'OUTS =  %0001001000000000   ' indicator led is on. rts (p9) high to start, tx (p8) low to start.

'fedcba9876543210 - commented out by JA, June 14, 2010

'DIRS =  %1111001111111101   ' p1 is input for sensirion data, pa and pb input for rx and cts - commented out by JA,

' June 14, 2010

OUTS = %0001001000000000      ' indicator led is on. rts (p9) high to start, tx (p8) low to start.

'fedcba9876543210 - TA Program fix added by JA, June 14, 2010

DIRS = %1111001111110101      ' p1 is input for sensirion data, pa and pb

' TA Program fix added by JA, June 14, 2010

' input FOR rx AND cts AND p3 FOR CO2

DEBUG CR, "Initializing...",BELL,CR, CR  ' mk changed 'wait' to 'initializing' July 11

GOSUB init_Message

PAUSE 10

GOSUB RTC_init                           ' reset the clock to 00:00:00

PAUSE 1000

GOSUB RTC_init                           ' mk added 08 March 2008

PAUSE 512

HIGH TX                                  ' Initialize Transmit Line

LOW RTS                                  ' Take Vinculum Out Of Reset

PAUSE 512                                ' Allow 1/2 Second To Settle

' LOW ledP                                ' LED indicator off - long pulse signals reset

GOSUB V_synchronize                      ' get ready to send data to thumb drive

DEBUG CR,"sync tries: ",DEC waldo

iobyte=waldo : GOSUB morse_numeral       ' show the tries count for debugging

GOSUB V_short_commands                   ' enable short command mode on Vinculum/thumb drive

GOSUB V_Check_Drive

DEBUG CR,"disk tries: ",DEC waldo, CR

iobyte=waldo : GOSUB morse_numeral       ' show the tries count for debugging

' either print disk not found, else write

IF dskerr=1 THEN

DEBUG " not found"

ELSE

GOSUB V_open_datafile

SEROUT TX\CTS, vBaud, [ $8,$20,0,0,0,10,CR,"ASP data",CR, LF]

GOSUB V_purge

GOSUB V_close_datafile

ENDIF

DEBUG CR,"Time (sec)  Sample#   CO2(mV) CO2(ppm) Temp(C) Humidity(%)  Altitude(ft)",BELL,CR

GOSUB SHT_Connection_Reset

PAUSE 256

' -----[ Program Code ]----------------------------------------------------

Main:

DO

' PULSOUT ledP,50

' GOSUB RTC_read

GOSUB RTC_sync

' mk removed " minute =$01 OR " from the next line, right after IF command - 01 July 2009

' IF index2>100 THEN land=1   ' set flag to start movements after descent of one minute

index2 = index2 + 1

DEBUG CR,HEX2 hour,":",HEX2 minute,":",HEX2 second, TAB, DEC index2

GOSUB MAWD_get

GOSUB SHT_Measure_Temp

GOSUB SHT_Measure_Humidity

GOSUB Carbon_collect

GOSUB MCP_get

DEBUG TAB,REP "-"\degC.BIT15,DEC ABS degC/10,".",DEC1 ABS degC,degSym

DEBUG TAB, DEC rH/10, ".", DEC1 rH, "%"

DEBUG TAB, TAB, DEC feet, CR  ' mk added a second tab - July 11

GOSUB V_open_datafile  ' thumb drive

GOSUB V_write_record   ' one data record

GOSUB V_close_datafile  ' close the file

' IF land THEN GOSUB movement - mk commented out - 01 July 2009

LOOP

END

' -----[ Subroutines ]-----------------------------------------------------

'

' -----[ RTC ]-----

RTC_init:  ' send $00 to DS1302 control register (command $8E) to enable write

HIGH DSce

SHIFTOUT DSdta,DSclk,LSBFIRST,[$8E,$00]

LOW DSce

' set the clock time using burst command $BE

' second=0, minute=0, hour=0, date31=4, month=7, day7=9, year=2009

' also disable further writes: $80 at loc $7

HIGH DSce

SHIFTOUT DSdta,DSclk,LSBFIRST,[$BE,$00,$00,$00,$04,$07,$09,$09,$80]

LOW DSce

RETURN

rtc_Sync:

' paces data logging to the number of seconds specified

' in CONstant "logInterval"

buffer=loginterval*200  ' this variable is used as a temporary counter

DO:DO

GOSUB RTC_read

buffer=buffer-1       ' this countdown is an escape from loop in case RTC fails

LOOP UNTIL second<>second0 OR buffer=0  '

second0=second          ' change, update old value for tracking

LOOP UNTIL second.NIB1*10+second.NIB0//logInterval=0 OR buffer=0

' wait for log interval to arrive

' DEBUG CR,?buffer

RETURN

RTC_read:

' retrieves the time to variable: second, minute, hour

HIGH DSce

SHIFTOUT DSdta,DSclk,LSBFIRST,[$BF]

' burst mode read $BF command

SHIFTIN DSdta,DSclk,LSBPRE,[second,minute,hour]

LOW DSce

RETURN

' -----[ Vinculum/Thumb Drive]-----

'

' refer to email discussion around email at:

' http://forums.parallax.com/forums/default.aspx?f=6&p=7&m=247882#m252750

V_purge:

V_Get_Data:

SERIN RX\RTS, vbaud,128, V_Timeout, [STR buf0\12]

V_Timeout:                ' timeout is the normal exit, buffer not full

RETURN

V_synchronize:

' Send 'E' to start the synchronization process.

' Expect 'E' and a CR for the response, timing per pk

waldo=0                   ' count tries

DO

GOSUB V_purge           ' clear the vinculum tx data buffer

PAUSE 256               ' wait about 1/4 second

SEROUT TX\CTS, vbaud, ["E", CR]           ' Sync Command Character

GOSUB V_get_Data                          ' Get Response

waldo=waldo+1

LOOP UNTIL (buf0="E" AND buf1=CR) OR waldo=8

GOSUB V_purge                             ' Purge Buffer of leftover junk

SEROUT TX\CTS, vbaud, ["e", CR]           ' next Sync Command Character

GOSUB V_get_Data                          ' Get Response/purge

IF (buf0<>"e" OR buf1<>CR) THEN waldo=waldo + 1

IF waldo>7 THEN dskerr=1

' returns with number of tries in waldo

' returns with dskerr=1 if cant sync

RETURN

V_Check_Drive:

waldo=0

DO

GOSUB V_purge

SEROUT TX\CTS, vbaud, [CR]

GOSUB V_get_Data

IF buf0="N" AND buf1="D" THEN

waldo=9                      ' error: disk not found

EXIT                         ' no point to test more

ENDIF

waldo=waldo+1

PAUSE 100

LOOP UNTIL buf0=">" OR waldo=8

IF waldo>7 THEN dskerr=1

' return with number of tries in waldo

' waldo=9 means it got the no disk error

' dskerr flag=1 is tries>7

RETURN

V_short_commands:

SEROUT TX\CTS, vbaud, ["SCS", CR]

GOSUB v_get_Data                 ' purge

RETURN

V_Open_DataFile:

GOSUB V_purge                    ' purge receive buffer

SEROUT TX\CTS, vbaud, [$09, $20, "datafile.txt", CR]

GOSUB V_purge      ' purge receive buffer

RETURN

v_Close_DataFile:

GOSUB V_purge                      ' Purge Receive Buffer

SEROUT TX\CTS, vbaud, [$0A, $20, "datafile.txt", CR]

GOSUB V_purge             ' purge receive buffer

RETURN

V_write_record:

' Write data with time to flash drive

' Added 'CO2 ppm' - June 11, 2010 - Tracy, Dylan, Justin

LOW RTS

SEROUT TX\CTS, vBaud, [ $8,$20,0,0,0,41,CR, HEX2 hour, ":" , HEX2 minute, ":", HEX2 second, "," , degC.BIT15*13+32,DEC3 ABS (degC/10),".", DEC1 ABS degC,",",

DEC3 (RH/10),".",DEC1 RH,",", DEC5 feet,",",DEC5 CO2pp, ",",DEC5 CO2mV,CR, LF] 'Tracey says: "note that this adds one comma and one 5 digit number,and bumps the total number of bytes per record up from 35 to 41" August 13,2010 JA

HIGH RTS

GOSUB V_purge

RETURN

' -----[ Sensirion ]-----

SHT_Connection_Reset:

SHIFTOUT ShtDta, ShtClk, LSBFIRST, [$FFF\9]

RETURN

'

' generates SHT1x "start" sequence

'          _____         _____

' ShtData       |_______|

'              ___     ___

' Clock    ___|   |___|   |___

'

'

SHT_Start:

INPUT ShtDta            ' let pull-up take line high

LOW ShtClk

HIGH ShtClk

LOW ShtDta

LOW ShtClk

HIGH ShtClk

INPUT ShtDta

LOW ShtClk

RETURN

' sends start sequence & command byte & retrieves two data bytes from Sensirion

SHT_measure:

GOSUB SHT_Start         ' alert device

GOSUB SHT_Write_Byte    ' send command (iobyte)

GOSUB SHT_Wait          ' wait until measurement done

ackBit = Ack            ' another read follows

GOSUB SHT_Read_Byte     ' get MSB

soSHT1 = ioByte

ackBit = NoAck          ' last read

GOSUB SHT_Read_Byte     ' get LSB

soSHT0 = ioByte

RETURN

'

' measure temperature:

' -- celsius = soT * 0.01 - 40

SHT_Measure_Temp:

ioByte = ShtTemp        ' temperature command

GOSUB SHT_measure       ' send command

degC = soSHT / 10 - 400 ' convert to tenths C

RETURN

' measure humidity

SHT_Measure_Humidity:

ioByte = ShtHumi        ' humidity command

GOSUB SHT_measure       ' send command

rh = (soSHT ** 26542)

rh = rh - ((soSHT ** 3468) * (soSHT ** 3468) + 50 / 100)

rh = rh - 40

rh = ((degC + 250 / 10 - 50) * (soSHT ** 524 + 1) + (rh * 10)) + 5 / 10

RETURN

' sends "ioByte"

' returns "ackBit"

SHT_Write_Byte:

SHIFTOUT ShtDta, ShtClk, MSBFIRST, [ioByte]   ' send byte

SHIFTIN  ShtDta, ShtClk, LSBPRE, [ackBit\1]   ' get ack bit

RETURN

' returns "ioByte"

' sends "ackBit"

SHT_Read_Byte:

SHIFTIN  ShtDta, ShtClk, MSBPRE, [ioByte]     ' get byte

SHIFTOUT ShtDta, ShtClk, LSBFIRST, [ackBit\1] ' send ack bit

INPUT ShtDta                                  ' release data line

RETURN

' wait for sht to finish measurement

' (data line pulled low by SHT within 1/4 second)

SHT_Wait:

INPUT ShtDta   ' data line is input

delay = 255    ' give >1/4 second to finish

DO

delay=delay-1

PAUSE 1

LOOP UNTIL ShtDta=0 OR delay=0

IF delay=0 THEN

GOSUB SHT_Connection_Reset

shtErr=1

ENDIF

RETURN

' -----[ Carbon dioxide ]-----

' Added June 11, 2010 - Tracy, Dylan, Justin

Carbon_collect:

HIGH 4                'This activates the Heat Switch (HSW)

' <--- this does not really need to be repeated, but it doesn't hurt

CO2pp = IN3         ' read the CO2 level.  Later, we will make this do more than just the alarm status

' DEBUG TAB, DEC CO2pp  ' show the CO2 level ALARM STATUS 0 or 1

RETURN

' -----[ MCP3202 ADC ]-----

' Added June 14, 2010 by Andrew from Tracy

MCP_get:

'read channel 1 first, potentiometer, will NOT be logged

HIGH 4                'This activates the Heat Switch (HSW)

LOW CS                                          ' Enable ADC

SHIFTOUT DataIn, Clock, MSBFIRST, [ %1111\4]     ' Select CH1, Single-Ended

SHIFTIN DataOut, Clock, MSBPOST, [ CO2pp\12]   ' Read ADC

HIGH CS                                         ' Disable ADC

CO2pp = CO2pp */ Cnts2Mv         ' Convert To Millivolts

DEBUG TAB, DEC CO2pp             ' shows the potentiometer voltage

' read channel 0, CO2 voltage, variable CO2pp will be logged on flash drive

CO2mV=CO2pp                      ' added right after ADC acquires mV reading from potentiometer as requested, August 13, 2010 JA

LOW CS                                          ' Enable ADC

SHIFTOUT DataIn, Clock, MSBFIRST, [ %1101\4]     ' Select CH0, Single-Ended

SHIFTIN DataOut, Clock, MSBPOST, [ CO2pp\12]   ' Read ADC

HIGH CS                                         ' Disable ADC

CO2pp = CO2pp */ Cnts2Mv                    ' Convert To Millivolts

DEBUG TAB, DEC CO2pp             ' show the CO2 voltage

' mVolts1 writes mV for CO2 potientiometer

RETURN

' -----[ Morse code ]-----

Morse_numeral:

iobyte = %11110000011111 >> iobyte & %11111 | %10100000

Morse:

FOR index = iobyte>>5 TO 1

'  HIGH ledP

PAUSE 192*iobyte.BIT0(index-1)+64

'  LOW ledP

PAUSE 256

NEXT

PAUSE 512

RETURN

morse_reading:

DEBUG 32

FOR ackbit=1 TO 0

iobyte = waldo DIG ackbit

DEBUG DEC1 iobyte

GOSUB morse_numeral

NEXT

RETURN

mcs CON 3*32+%000  ' dit dit dit

mco CON 3*32+%111  ' dah dah dah

mcn CON 2*32+%10   ' dah dit

mcf CON 4*32+%0010 ' dit dit dah dit

mcb CON 4*32+%1000 ' dah dit dit dit

mcc CON 4*32+%1010 ' dah dit dah dit

mcw CON 3*32+%011  ' dit dah dah

mce CON 1*32+%0    ' dit

mcv CON 4*32+%0001 ' dit dit dit dah

morse_message:

' plays back morse message up to null

' enter with waldo pointing to message start

' e.g. waldo=message1 : GOSUB morse_message

' message1 DATA mcs,mco,mcs,0  ' sos, example message for LED

DO

READ waldo,iobyte

IF iobyte=0 THEN EXIT

GOSUB morse

waldo=waldo+1

LOOP

RETURN

init_message:

'DEBUG CR, "ARLISS - A Rocket Launch for International Student Satellites", CR

DEBUG CR,"ARLISS Team New Hampshire - June, 2010", CR, CR

DEBUG "Temperature, humidity, carbon dioxide, and altitude data logging program",CR, CR

DEBUG "With sincere thanks to Dr. Tracy Allen and Paul Smith",CR, CR

DEBUG "And our friends at Parallax Inc. - www.parallax.com",  CR,CR

' DEBUG "__________________________________________________________________________ ", CR

RETURN

MAWD_get:

' alternative code to acquire MAWD data, uses DEC5 instead of STR in SERIN

PULSIN MAWDin,1,iobyte   ' the purpose of this is to synchronize with the MAWD timing.

PAUSE 10                 ' this assures the SERIN will start waiting in the silent period before

' a MAWD xmission

feet=0

SERIN MAWDin,84,64,MAWDnot,[DEC5 feet]

MAWDnot:

RETURN

'The end - Mark – August 14, 2010