Servo Control by STAMP Cheat Sheet

John@Kauffmans.org		2005 March 02

PBASIC code:

FOR counter = start TO end

PULSOUT pin# , duration

PAUSE z

NEXT

| Servo | Diagram Below | Pulse | PBASIC Sample above |
| --- | --- | --- | --- |
| Angle of arm | A | Width of Pulse Accepts 1-2 mS | Duration 500 = 90° = 3:00 750= 0° = 12:00 1000 = -90° = 9:00 Value on left will be multiplied by clock tick of microcontroller. BS2 stamp = 2 µS , so result is 1-2 mS |
| Connection of white wire | n/a | Pin # | pin# |
| Duration arm stays at setting | C | Number of pulses | Counter … end Depending on number of lines in the loop. BS2: 2 lines ≈ 50mS |
| Time between pulses received (Fixed in the servo at 20mS) | D | Time between pulses | Z Needs to result in 20mS. Code varies with microcontroller speed, from 5 to 20. Adjust for smooth servo action |