Smith Team WHOI

4/1/2011

Liaison’s suggestion

For example, get a TIP142. Connect the valve between the TIP142 collector and supply +24V. Connect supply ground to the emitter of the TIP142. Connect your micro's GPIO pin to the TIP142 base with 1K ohms in series to limit current to 5 mA. Ground microcontroller to the 24V ground. Then when micro puts 5 mA into the TIP142, the TIP142 will multiply it by 1000 and sink 5 A from the valve

Components

Solenoid Valve – TIP142

- Power supply from 24 VDC wall-wart

- Resistance (measured) - 78Ω

- Current -  0.29 A (doesn’t specify if min or max)

Transistor TIP142 – Darlington pair power transistor

- HFE = 1000 for IC = 5A and VCE = 4 V

- Collector Peak Current – 20 A

Protection Diode 1N4007

- VRRM = 1000

- IF(AV) average rectified forward current = 1.0 A

- ISFM non-repetitive peak forward surge current = 30 A

Example calculations for choosing NPN Transistor

Reference: http://www.kpsec.freeuk.com/trancirc.htm

1. Choose resistor that meets these requirements: Ic(max) and hFE(min).

2. The transistor's maximum collector current Ic(max) must be greater than the load current Ic.

| load current Ic = | supply voltage Vs |
| --- | --- |
|  | load resistance RL |

3. The transistor's minimum current gain hFE(min) must be at least five times the load current Ic divided by the maximum output current from the IC (chip)

| hFE(min) > 5 × | load current Ic |
| --- | --- |
|  | max. IC current |

4. Calculate an approximate value for the base resistor:

| RB = | Vc × hFE | where Vc = IC supply voltage (in a simple circuit with one supply this is Vs) |
| --- | --- | --- |
|  | 5 × Ic |  |

Example Calculations:

IC = 24V / 78 Ω = 0.307 A

hFE = 5 x (0.307 A / 5 A) = 0.307

| RB = | (24) × (1000) = 960 Ω |
| --- | --- |
|  | 5 × (5) |

* Suggested to use hFE of 1000

Questions

- Were incorrect methods or values used for calculating IC (max) and hFE (min)?

- Does the size of the diode only depend on peak current of the solenoid valve?

Temperature Sensor TH-44004

- Resistance = 2252 Ω

- Specifications: http://www.omega.com/ppt/pptsc.asp?ref=TH-44000-NPT

A/D Converter

- 12-bit resolution

- +/- 1 LSB max DNL

- Serial interface (modes  0,0 and 1,1)

- 4 input channels

- Power requirements: 2.7V to +5.5 VDC

Humidity Sensor HM1500LF

- Resistance = Ω

- Specifications: http://www.futureelectronics.com/en/technologies/semiconductors/analog/sensors/humidity-dew/Pages/2376010-HM1500LF.aspx

A/D Converter

- 12-bit resolution

- +/- 1 LSB max DNL

- Serial interface (modes  0,0 and 1,1)

- 4 input channels

- Power requirements: 2.7V to +5.5 VDC

Pressure Sensor PX309 Series

- Resistance =  Ω

- Specifications: http://www.omega.com/ppt/pptsc.asp?ref=PX309-100MV

A/D Converter

- 12-bit resolution

- +/- 1 LSB max DNL

- Serial interface (modes  0,0 and 1,1)

- 4 input channels

- Power requirements: 2.7V to +5.5 VDC