/*

ReceiveBeacon

Purpose:

Receive beacons from a Base Station or BeaconSimulator and provides the Vector to the Base Station to the robot over SPI or Serial.

Arduino Pin Usage:

**Digital**

-Serial-

Pin 1  UART RX (Radio to Arduino) will monitor the Radio Communications

Pin 2  UART TX (Arduino to Robot) will send vector heading to robot

-SPI-

Pin 10 SS_PIN

Pin 11 MOSI_PIN

Pin 12 MISO_PIN

Pin 13 SCK_PIN

-Radio to Arduino-

RSSI   Analog Pin 0

PDN    Digital Pin 8

TX-RX  Digital Pin 9

Data   RX - Digital Pin 0

VIN    5V

GND    GND

-Settings-

Voltage Reference for divider - Using GND and Digital Pin 7 and feeding divided voltage to AREF.  DPin 7 is set to logic High

LED      Digital Pin 6 (GoodPacketPin)

BeaSim   Digital Pin 4 - Beacon Sim pin to allow checkout of SPI Interface

-Status-

LED      Digital Pin 6  Output  - to a 1k resister then and LED to Ground

-How it works-

-SPI and UART interfaces-

The Beacon Receiver is a standard slave SPI interface.  Here are the Op codes

200 - Ambigous Vector robot should wait or move until a valid vector is available

201 - SPI read has pulled last valid vector.  Poll again in a second and see if a valid vector is now available.  This will update

0-179 - Valid vectors

The Robot will listen on the TX line

200 - Ambigous Vector robot should wait or move until a valid vector is available

0-179 - Valid vectors

History:

Initial Date: Sept 27 2010

Author: Brian Sanders

Project: Robotics Challenge

10/27/2010  Versions: 1.0 Initial

02/08/2011  1.1 Cleaned up code

02/13/2011  1.2 Added SPI capabilities

02/15/2011  1.3 Added SPI multi-read support and 201 and 200 code definitions

01/25/2012  1.4 Updated for Arudino 1.0 and to support a constant beacon simulator. Cleaned up Code.

02/10/2012  1.5 Cleaned up code and fixed bugs pertaining to many invalid vector reports

02/15/2012  1.6 Added LED status like feature for static vector receipt

------------------------------------------------------------------------------------------------*/

#define SCK_PIN   13

#define MISO_PIN  12

#define MOSI_PIN  11

#define SS_PIN    10

//Pin Definitions

const int RSSIInPin = 0;        // Analog input pin that the RSSI is attached to

const int GoodPacketLEDPin = 6; // Flashing LED on this pin indicates a good direction

const int PDNPin = 8;           // Power on the transceiver

const int TXRXPin = 9;          // Set the mode of the transceiver

const int BeaconSimPin = 4;    // Pulled high to allow simulated beacon and SPI checkout

// Defining sampling size

const int filterSamples = 15; //number of reading to average

int sensSmoothRSSI [filterSamples];   // array for holding raw sensor values for sensor1

//Variables

int prevVector = 0;        // used to keep track of previous recoreded vector

int LEDStatus = 0;         // Flips on/off/on when a valid vectors are received

int maxRSSI = 0;           // highest RSSI on a particular scan

long lastReportTime = 0;  //millis function uses a long

int simValue = 0;          // Simulator reported value

int const historySize = 180;    // history for determining a 0-360 deg scan at 2 deg resolution

int historyIndex = 0;             // initialized to an invalid index

int historyRSSI [historySize];    // array to record a full scan

int historyRSSIMaxIndex = 0;      // index into historyRSSI where the RSSImax is observed during a scan

int lastGoodVector = 0;

//values for determining the current and previous headings

int previousReportVector = 0;

const float receiveThreshold = 0.98; // Percentage of max RSSI value to signal sholders of beam.

void setup() {

// initialize serial communications at 9600 bps:

Serial.begin(9600);

//Define Interface Pins

pinMode(TXRXPin, OUTPUT);

pinMode(PDNPin, OUTPUT);

pinMode(BeaconSimPin, INPUT);

pinMode(GoodPacketLEDPin, OUTPUT);

analogReference(EXTERNAL);

//Configure Radio for operation

digitalWrite(TXRXPin, 0);    // Keep Transceiver in Receive

digitalWrite(PDNPin, 1);     // Keep Radio powered ON

//Setup this Arduino as a SPI device

SlaveInit();

}

void loop() {

//Determine if the Beacon Sim Mode is selected.  This proves that communication to the Robot is possible.

int beaconSim = digitalRead(BeaconSimPin);

if (beaconSim == 1){

++simValue;

if (simValue > 179) {

simValue = 0;

}

SPDR = simValue ;

Serial.println(simValue);

delay(3000);

}

else

{

int currentRSSI = 0;

int RSSIRawData = 0;

// Find the packet Header

if(Serial.find("FF1")){

// Get the signal strength in anticipation of Vector

for (int i=0; i<filterSamples; i++){

RSSIRawData = analogRead(RSSIInPin);                        // read RSSI from Radio

currentRSSI = digitalSmooth(RSSIRawData, sensSmoothRSSI);

}

//Grab the Vector

int currentVector = Serial.read();

// Insure the next character is valid "U" confirming a properly received packet

if (Serial.find("U")){

// Allow for a simulator transmitter which does not have a time varying vector output.

// if the last report was more than 3 seconds ago simply pass the last received vector as the current vector

if((millis() - lastReportTime) > 3000)

{

//Print out to the Serial port that the vector is static and briefly blink the LED for visual indication

//Serial.println("Static Vector");

lastReportTime = millis();

printVector(currentVector);

LEDStatus =1;

digitalWrite(GoodPacketLEDPin, LEDStatus);

delay(750);

LEDStatus =0;

digitalWrite(GoodPacketLEDPin, LEDStatus);

}

else {

recordCurrentVector(currentRSSI, currentVector);

}

}

}

}

}

/*-------------------------------------------------------------------------------------------------------------

recordCurrentVector adds received RSSI and logs them according to their Vector

-------------------------------------------------------------------------------------------------------------*/

void recordCurrentVector(int currentRSSI, int currentVector){

// Populate the historyRSSI[0-179] to index and record corresponding RSSI in two deg incriments

// enter the currentRSSI into the approriate location in the vector

historyRSSI[currentVector] = currentRSSI;

//int candidateCurrentVector;

/*

Serial.print("\t\t");

Serial.println(currentVector);

Serial.print("\t");

Serial.println(currentRSSI);

*/

// keep track of vector with highest RSSI value

if (currentRSSI > maxRSSI){

historyRSSIMaxIndex = currentVector;   // save the index (vector) of the max RSSI

maxRSSI = currentRSSI ;                // save the max RSSI value for later comparison

}

// determine the completion of a scan cycle by looking for the vector roll over.  currentVector should be smaller than prev

if ( (currentVector + 5) < prevVector  ){

//do scan analysis

//use the historyRSSImaxIndex and calculate the threshold above and below maxRSSI point

int peakRSSIVector =0;

peakRSSIVector = findPeak(historyRSSIMaxIndex);

// Average the upper and lower thresholdsto find the center

int vector = reasonableVector(peakRSSIVector);

//Make vector available to SPI and Serial

printVector(vector);

// Clear out buffers and prepare for new scan

clearData();

}

else

{

prevVector= currentVector;

}

}

/*-------------------------------------------------------------------------------------------------------------

printVector provides the current vector to the serial or SPI output

-------------------------------------------------------------------------------------------------------------*/

void printVector(int vector)

{

SPDR = vector;

Serial.println(vector);

previousReportVector = vector;

prevVector = vector;

// If SPI has been read then set to 201 indicating that SPI has been previously fetched

if (SPDR == 0) {

SPDR = 201;

}

}

/*-------------------------------------------------------------------------------------------------------------

reasonableVector insures that the received vector has changed not "much" since the last valid vector before

passing vector on to be made available.

-------------------------------------------------------------------------------------------------------------*/

int reasonableVector(int candidateCurrentVector)

{

//broken down to account for abs funtion uniquness

int deltaSinceLastUpdate = 0;

deltaSinceLastUpdate = abs (candidateCurrentVector - lastGoodVector);

int currentReportVector;

/*

Serial.println("Finding deltaSinceLastUpdate candidateCurrentVector");

Serial.print(deltaSinceLastUpdate);

Serial.print(":");

Serial.print(candidateCurrentVector);

Serial.print(":");

Serial.print(previousReportVector);

Serial.print(":");

Serial.println(candidateCurrentVector);

*/

// determine if this reading is within 20 steps of last report

if (abs(deltaSinceLastUpdate) < 20){

currentReportVector = candidateCurrentVector;

// Change the LED status to indicate a complete scan and vector is valid

++LEDStatus;

if (LEDStatus >1) {

LEDStatus =0;

}

digitalWrite(GoodPacketLEDPin, LEDStatus);

}

else

{

//Value of 200 means that a the vector is not currently valid

currentReportVector = 200;

}

lastGoodVector = candidateCurrentVector;

return currentReportVector;

}

/*-------------------------------------------------------------------------------------------------------------

findPeak startes from the Vector which had the highest RSSI signal and serarches higher and lower from that

point finding the Vector at which the RSSI reaches a threshold value indicating the sides of the received

signal peak.  The average of the higher and lower peaks is returned as the current vector.

-------------------------------------------------------------------------------------------------------------*/

int findPeak(int RSSIMaxIndex){

//Start from the max RSSI and find the upper and lower vectors just above the threshold value.  Average those two and return

//the center vector.

int lowerThresholdIndex = RSSIMaxIndex;        //index of max RSSI location

int indexValueofhistoryRSSI = historyRSSI[lowerThresholdIndex];  // Start scanning at the Max RSSI Index

int thresholdValue = int(maxRSSI*receiveThreshold);              // Calculate RSSI Value to stop search

//Scan lower in vectors until thresholdValue is found and skip zero values (no reports)

while(thresholdValue <= indexValueofhistoryRSSI && lowerThresholdIndex > 1)

{

--lowerThresholdIndex;  // = lowerThresholdIndex - 1;

indexValueofhistoryRSSI= historyRSSI[lowerThresholdIndex];

//make sure to skip Zeros - seems to be a limitation in the while Arduino preventing the additional condition check

while (indexValueofhistoryRSSI == 0  && lowerThresholdIndex > 1) {

--lowerThresholdIndex;

indexValueofhistoryRSSI= historyRSSI[lowerThresholdIndex];

}

}

// Scan higher in vectors until thresholdValue is found and skip zero value (no reports)

int higherThresholdIndex = RSSIMaxIndex;        //index of max RSSI location

indexValueofhistoryRSSI = historyRSSI[higherThresholdIndex];  // Start scanning at the Max RSSI Index

while((thresholdValue <= indexValueofhistoryRSSI || indexValueofhistoryRSSI == 0 ) && higherThresholdIndex <  historySize - 1 )

{

++higherThresholdIndex;

indexValueofhistoryRSSI= historyRSSI[higherThresholdIndex];

//make sure to skip Zeros - seems to be a limitation in the while Arduino preventing the additional condition check i the while loop

while (indexValueofhistoryRSSI == 0 && higherThresholdIndex <  historySize - 1) {

++higherThresholdIndex;

indexValueofhistoryRSSI= historyRSSI[higherThresholdIndex];

}

}

int candidateCurrentVector = int((higherThresholdIndex + lowerThresholdIndex)/2);

/*

Serial.println("Finding the index higher lower max and then currentVector");

Serial.print(higherThresholdIndex);

Serial.print(":");

Serial.print(lowerThresholdIndex);

Serial.print(":");

Serial.print(RSSIMaxIndex);

Serial.print(":");

Serial.println(candidateCurrentVector);

*/

return candidateCurrentVector;

}

/*-------------------------------------------------------------------------------------------------------------

clearData resets variables and data structures to start a new scan

-------------------------------------------------------------------------------------------------------------*/

void clearData(){

//End cycle and start recording a new cycle

maxRSSI = 0;

historyRSSIMaxIndex = 0;

historyIndex = 0;

//currentVector = 0;

prevVector = 0;

//Update the last report timer

lastReportTime = millis();

//Reset the RSSI Vector history for the next scan

for(int i = 0; i < historySize; i++){

historyRSSI [i]=0;

}

}

/*-------------------------------------------------------------------------------------------------------------

digitalSmooth provides an averaged value when sampling the RSSI analog input pin.

-------------------------------------------------------------------------------------------------------------*/

//Credit for digitalSmooth function http://www.arduino.cc/playground/Main/DigitalSmooth

int digitalSmooth(int rawIn, int *sensSmoothArray){     // "int *sensSmoothArray" passes an array to the function - the asterisk indicates the array name is a pointer

int j, k, temp, top, bottom;

long total;

static int i;

// static int raw[filterSamples];

static int sorted[filterSamples];

boolean done;

i = (i + 1) % filterSamples;    // increment counter and roll over if necc. -  % (modulo operator) rolls over variable

sensSmoothArray[i] = rawIn;                 // input new data into the oldest slot

// Serial.print("raw = ");

for (j=0; j<filterSamples; j++){     // transfer data array into anther array for sorting and averaging

sorted[j] = sensSmoothArray[j];

}

done = 0;                // flag to know when we're done sorting

while(done != 1){        // simple swap sort, sorts numbers from lowest to highest

done = 1;

for (j = 0; j < (filterSamples - 1); j++){

if (sorted[j] > sorted[j + 1]){     // numbers are out of order - swap

temp = sorted[j + 1];

sorted [j+1] =  sorted[j] ;

sorted [j] = temp;

done = 0;

}

}

}

/*

for (j = 0; j < (filterSamples); j++){    // print the array to debug

Serial.print(sorted[j]);

Serial.print("   ");

}

Serial.println();

*/

// throw out top 15% and bottom 60% of samples - limit to throw out at least one from top and bottom

bottom = max(((filterSamples * 60)  / 100), 1);

top = min((((filterSamples * 85) / 100) + 1  ), (filterSamples - 1));   // the + 1 is to make up for asymmetry caused by integer rounding

k = 0;

total = 0;

for ( j = bottom; j< top; j++){

total += sorted[j];  // total remaining indices

k++;

// Serial.print(sorted[j]);

// Serial.print("   ");

}

//  Serial.println();

//  Serial.print("average = ");

//  Serial.println(total/k);

return total / k;    // divide by number of samples

}

/*-------------------------------------------------------------------------------------------------------------

SlaveInit initializes the Arduino to act as a slave SPI device to a master SPI device requesting data

-------------------------------------------------------------------------------------------------------------*/

void SlaveInit(void) {

// Set MISO output, all others input

pinMode(SCK_PIN, INPUT);

pinMode(MOSI_PIN, INPUT);

pinMode(MISO_PIN, OUTPUT);

pinMode(SS_PIN, INPUT);

// Enable SPI

SPCR = B00000000;

SPCR = (1<<SPE);

}