### INTRODUCTION

This project demonstrates how a battery works.  A battery consists of two electrodes and an electrolyte.  I experimented with different electrodes and the electrolytes.  This has an effect on the electrical voltage and current produced by a battery.

Experiments

| 1. | Measure Voltages Produced by Different Materials |
| --- | --- |
| 2. | Measure Voltages using Different Electrolytes |
| 3. | Measure Electrical Currents for Different Areas of Electrodes |
| 4. | Determine How Many Lemons Are Needed to Run a Digital Clock |
| 5. | Measure the Electrical Current for Different Strengths of Electrolyte |

#### Experiment 1

### Voltage for different Pairs of Electrodes

| -+ | Alum-inum | Carbon | Zinc | Iron | ###### Silver |
| --- | --- | --- | --- | --- | --- |
| Copper | 0.65 | -0.09 | 0.96 | 0.43 | 0.09 |
| Silver | 0.57 | -0.11 | 0.97 | 0.49 |  |
| Iron | 0.15 | -0.55 | 0.49 |  |  |
| Zinc | -0.42 | -1.00 |  |  |  |
| Carbon | 0.71 |  |  |  |  |

Conclusion:  Zinc with Carbon, Silver or Copper produce the largest voltages.

## Experiment 2

## Voltages Produced by Different Electrolytes

| Electrolyte | Volts |  | Electrolyte | Volts |
| --- | --- | --- | --- | --- |
| Lemon Juice | 0.97 |  | Vinegar | 0.97 |
| Salt Water | 0.75 |  | Onion | 0.84 |
| Lemon | 0.86 |  | Banana | 0.84 |
| Potato | 0.91 |  |  |  |

Conclusion:  Lemon juice and vinegar produce the highest voltages.

Experiment 3

#### Current versus Electrode Length

| Length (Inches) | 0.0 | 0.5 | 1.0 | 1.5 | 2.0 | 2.5 | 3.0 |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Current (mA) | 0.0 | 0.7 | 1.5 | 1.8 | 2.2 | 3.3 | 3.9 |

## Conclusion:  The current increased with more of the electrodes in the electrolyte.

## Experiment 4

##### How Many Lemons are Needed to Run a Digital Clock

##### Conclusion:  It takes two lemons with copper and zinc electrodes to power a digital clock.  One lemon does not produce enough voltage.

##### Experiment 5

Electrical Current for Different Strengths of Electrolyte

| % | 0 | 1 | 2 | 3 | 4 | 5 | 10 | 15 | 20 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| mA | 0.0 | 0.6 | 0.8 | 1.0 | 1.2 | 1.4 | 1.5 | 1.7 | 1.9 |

Conclusion:  Current increases with a greater percentage of lemon juice.