ZX81 Emulator on HYDRA

Specification

Version: 0.1

Status: Draft

Date: 08 January 2008

Author: Gianni Metitieri

Reviewed by: <names>

Document Information

| Document Identification |  |
| --- | --- |
| Project/ID |  |
| Document/ID |  |
| Access Path |  |

| Document History |  |  |  |  |
| --- | --- | --- | --- | --- |
| Version | Date | Status | Remarks | Author(s) |
| 0.1 |  | Draft |  |  |
| 0.9 |  | Review |  |  |
| 1.0 |  | Final |  |  |

| References |  |
| --- | --- |
| File | Link |

Table of Contents

1	Summary	5

2	Background Overview	6

2.1	Hardware Overview	6

2.2	Z80 CPU	6

2.2.1	Registers	6

2.2.2	Opcodes	6

2.2.3	Interrupts	6

2.3	Ferranti ULA	6

2.3.1	Video	6

2.3.2	Cassette Ear / Microphone	6

2.4	Memory	6

2.4.1	ROM	6

2.4.2	RAM	6

3	Prototyping	7

3.1	Prototype Aims	7

3.2	Prototype Specification	7

3.3	Prototype Iteration one	7

3.4	Prototype Iteration two	7

3.5	Prototype Iteration three	7

3.6	Prototype Iteration four (Final prototype)	7

3.7	Prototype Class implementation	7

3.7.1	ZX81 Emu	7

3.7.2	ZX80 CPU	7

3.7.3	ZX81 Video	7

3.8	Final Prototype testing	7

3.8.1	ZX81 Emulator	7

3.9	Prototype Evaluation	7

4	Design	8

4.1	Design Overview	8

4.2	Z80 CPU Class	8

4.3	ZX81 Emu Class	8

4.4	ZX81 Video Class	8

4.5	Debug Window Class	8

4.6	Hydra Graphics Class	8

4.7	Hydra VGA Class	8

4.8	Hydra TV PAL Class	8

4.9	Hydra Keyboard Class	8

4.10	Designed class structure	8

5	Implementation	10

5.1	Implementation Overview	10

5.2	Z80 CPU	10

5.3	ZX81 Emu	10

5.4	ZX81 Video	10

5.5	Optimisation	10

5.5.1	Code optimisation	10

5.5.2	Class structure optimisations	10

5.5.3	Graphical optimisations	10

6	User Interface	11

6.1	Application Interface	11

6.2	Debugging Interface	11

7	Project Management	12

7.1	Risk Assessment	12

7.2	Time Management	12

7.3	Project plan	12

8	Conclusion	13

8.1	Achievements	13

9	Bibliography	14

A	Appendixes	15

A.1	???	15

A.1.1	???	15

# Summary

The Sinclair ZX81 home computer, released by Sinclair Research in 1981, was the follow up to the company's ZX80. The case was black, with a membrane keyboard; the machine's distinctive appearance was the work of industrial designer Rick Dickinson. Video output was to a television set, and saving and loading programs was via an ordinary home audio tape recorder to audio cassette.

# Background Overview

## Hardware Overview

The ZX81 contains (depending on RAM type) four or five chips; ROM, CPU, SRAM, and a Ferranti Gate array (or ULA – Uncommitted Logic Array). The ROM occupies addresses 0–8191 (but also addresses 8192–16383, due to minimal decoding hardware). The 1 KB SRAM is placed at address 16384 (but repeats up to address 32767). Unless more than 16 KB RAM is installed, this upper 32 KB area is mirroring the lower 32 KB (except for code execution).

## Z80 CPU

Lorem ipsum dolor sit amet consectetuer ut wisi enim ad minimum veniam quis nostrud exerci.

### Registers

The Z80 has the following registers:

I, R, A, F, BC, DE, HL, IX, IY, SP, PC, AF’, BC’, DE’ and HL’.

The R register is for memory refresh. It is increased by 1 after every instruction, whereby the CB, DD, DDCB, ED, FD and FDCB prefixes are viewed as separate instructions. Bit 7 is never modified, but it can be changed with LD A,R.

The F register is the flag register. It has the following structure:

Bit 7:	The S flag: if the result of the operation is negative, this flag is high. This is a copy of the MSB of the result of the operation.

Bit 6:	The Z flag: if the result is zero, this flag is high.

Bit 5:	This bit has no official name. Throughout this document it is referred to as ‘b5’. It is a copy of bit 5 of the result of the operation.

Bit 4:	The H flag. This is the carry from bit 3 to bit 4 of the operation. Used with DAA instruction.

Bit 3:	This bit has no official name. Throughout this document it is referred to as ‘b3’. It is a copy of bit 3 of the result of the operation.

Bit 2:	The P/V flag. It contains the overflow (high if two’s complements result does not fit in register) or the parity (parity of number of high bits in result of operation). See description of instructions for which one.

Bit 1:	The N flag. It is high if the last operation was an subtraction, otherwise it was an addition. Used with DAA instruction.

Bit 0:	The C flag. If the result of the operation does not fit in the register, this bit is high..

### Opcodes

There are four different opcode prefixes: CB, DD, ED and FD. If an opcode begins the DD prefix, the IX register is used in stead of the HL register and likewise the FD prefix results in use of the IY register in stead of the HL register. If HL refers to memory address, (e.g. LD A, (HL) ), an offset d is added to the opcode (e.g. LD A, (IX + d) ). If the instruction uses the H or L register, the high order byte or the low order byte of the IX or IY register is used. There are two exceptions to this rule, though: the EXX and EX DE, HL instructions. These instructions cannot be changed with a DD or FD prefix. If the instruction does not use the HL, H or L register, the instruction is executed as without the DD or FD prefix (except for the DDCB and FDCB opcode prefixes). Multiple DDs or FDs after each other operate like NOPs. If a instruction is preceded by an ED, a new set of instructions is used (see Opcode list). If the opcode is not listed, it will operate as two NOPs. Similarly the CB, FDCB and the DDCB select a new instruction set, though every possible instruction has a function and is listed.

### Interrupts

There are two types of interrupts: mask-able and non mask-able, and the are two flip-flops associated with interrupts: IFF1 and IFF2. DI resets both and EI sets both. If IFF1  is set then mask-able interrupts are accepted. When an non mask-able interrupt occurs, the IFF1 is reset, so disabling mask-able interrupts. When the CPU returns from a non mask-able interrupt (with RETN) the IFF2 is copied into IFF1 (and thus restored). Only IFF2 can be read (with LD A, I and LD A, R  IFF2 is copied into the P/V flag).

## Ferranti ULA

The ZX81 Sinclair Logic Chip (usually called the ULA) as shown in FIG 1 with all relevant connections including the isolation resistors R.

### Video

The ULA contains a 6.5 MHz crystal oscillator and a frequency divider which generates horizontal sync pulses at the video output and NMI pulses on the NMI output. The HSYNC and the NMI outputs can be controlled with the following I/O operations.

| 1. | OUT FD,A | turns off the NMI generator |
| --- | --- | --- |
| 2. | OUT FE,A | turns on the NMI generator |
| 3. | IN A,FE | turns off the HSYNC generator (only if NMI is off) |
| 4. | OUT FF,A | turns on the HSYNC generator |

The ULA video output switches between 3 voltage levels. It is normally at the +5V white level for blank lines. Characters patterns are displayed as black pixels when the level is +2.5V . The narrow horizontal sync pulses and wide vertical sync pulses are 0V level as shown in FIG 2 waveform. These logic levels are reduced with a resistor divider to 1V, 0.5V and 0V (UK/US) at the input of the TV RF modulator.

white _  _____    _____    _________//__________//____             ____//_____

black _   |   |__|     |__|   |             |         |<--400us-->|          |

sync  _   |<-------64us------>|<---64us---->|         |_____//____|          |

display line         blank        blank   vert sync     blank

FIG 2 - VIDEO LEVELS

The HSYNC pulses are 5 usec wide with 64 usec between HSYNC pulses. The VSYNC is 400 usec wide with 16.6 msec or 20 msec between VSYNC pulses. VSYNC is used to synchronize the TV vertical oscillator and start the raster scan at the top of the screen. This occurs when IN A,FE (used for scanning the keyboard) clamps the video output to the SYNC level. 400us later OUT FF,A releases SYNC to enable the 64 us HSYNC pulses. The HSYNC pulses continue to be generated independed of the CPU until the next VSYNC. The CPU executes the application code during the blank lines at the top and bottom of the screen while the NMI generator interrupts the CPU every 64 us and increments a blank line counter to determine if it is time for the VIDEO DISPLAY of VSYNC routines.

.

### Cassette Ear / Microphone

Lorem ipsum dolor sit amet consectetuer ut wisi enim ad minimum veniam quis nostrud exerci.

## Memory

Lorem ipsum dolor sit amet consectetuer ut wisi enim ad minimum veniam quis nostrud exerci.

### ROM

Lorem ipsum dolor sit amet consectetuer ut wisi enim ad minimum veniam quis nostrud exerci.

### RAM

Lorem ipsum dolor sit amet consectetuer ut wisi enim ad minimum veniam quis nostrud exerci.

# Prototyping

Lorem ipsum dolor sit amet consectetuer ut wisi enim ad minimum veniam quis nostrud exerci.

## Prototype Aims

Lorem ipsum dolor sit amet consectetuer ut wisi enim ad minimum veniam quis nostrud exerci.

## Prototype Specification

Lorem ipsum dolor sit amet consectetuer ut wisi enim ad minimum veniam quis nostrud exerci.

## Prototype Iteration one

Lorem ipsum dolor sit amet consectetuer ut wisi enim ad minimum veniam quis nostrud exerci.

## Prototype Iteration two

Lorem ipsum dolor sit amet consectetuer ut wisi enim ad minimum veniam quis nostrud exerci.

## Prototype Iteration three

Lorem ipsum dolor sit amet consectetuer ut wisi enim ad minimum veniam quis nostrud exerci.

## Prototype Iteration four (Final prototype)

Lorem ipsum dolor sit amet consectetuer ut wisi enim ad minimum veniam quis nostrud exerci.

## Prototype Class implementation

Lorem ipsum dolor sit amet consectetuer ut wisi enim ad minimum veniam quis nostrud exerci.

### ZX81 Emu

Lorem ipsum dolor sit amet consectetuer ut wisi enim ad minimum veniam quis nostrud exerci.

### ZX80 CPU

Lorem ipsum dolor sit amet consectetuer ut wisi enim ad minimum veniam quis nostrud exerci.

### ZX81 Video

Lorem ipsum dolor sit amet consectetuer ut wisi enim ad minimum veniam quis nostrud exerci.

## Final Prototype testing

Lorem ipsum dolor sit amet consectetuer ut wisi enim ad minimum veniam quis nostrud exerci.

### ZX81 Emulator

Lorem ipsum dolor sit amet consectetuer ut wisi enim ad minimum veniam quis nostrud exerci.

## Prototype Evaluation

Lorem ipsum dolor sit amet consectetuer ut wisi enim ad minimum veniam quis nostrud exerci.

# Design

## Design Overview

Lorem ipsum dolor sit amet consectetuer ut wisi enim ad minimum veniam quis nostrud exerci.

## Z80 CPU Class

Lorem ipsum dolor sit amet consectetuer ut wisi enim ad minimum veniam quis nostrud exerci.

## ZX81 Emu Class

Lorem ipsum dolor sit amet consectetuer ut wisi enim ad minimum veniam quis nostrud exerci.

## ZX81 Video Class

Lorem ipsum dolor sit amet consectetuer ut wisi enim ad minimum veniam quis nostrud exerci.

## Debug Window Class

Lorem ipsum dolor sit amet consectetuer ut wisi enim ad minimum veniam quis nostrud exerci.

## Hydra Graphics Class

Lorem ipsum dolor sit amet consectetuer ut wisi enim ad minimum veniam quis nostrud exerci.

## Hydra VGA Class

Lorem ipsum dolor sit amet consectetuer ut wisi enim ad minimum veniam quis nostrud exerci.

## Hydra TV PAL Class

Lorem ipsum dolor sit amet consectetuer ut wisi enim ad minimum veniam quis nostrud exerci.

## Hydra Keyboard Class

The description of the business process requirements provides an analysis of the to-be state.

## Designed class structure

Lorem ipsum dolor sit amet consectetuer ut wisi enim ad minimum veniam quis nostrud exerci.

# Implementation

## Implementation Overview

Lorem ipsum dolor sit amet consectetuer ut wisi enim ad minimum veniam quis nostrud exerci.

## Z80 CPU

Lorem ipsum dolor sit amet consectetuer ut wisi enim ad minimum veniam quis nostrud exerci.

## ZX81 Emu

Lorem ipsum dolor sit amet consectetuer ut wisi enim ad minimum veniam quis nostrud exerci.

## ZX81 Video

Lorem ipsum dolor sit amet consectetuer ut wisi enim ad minimum veniam quis nostrud exerci.

## Optimisation

Lorem ipsum dolor sit amet consectetuer ut wisi enim ad minimum veniam quis nostrud exerci.

### Code optimisation

Lorem ipsum dolor sit amet consectetuer ut wisi enim ad minimum veniam quis nostrud exerci.

### Class structure optimisations

Lorem ipsum dolor sit amet consectetuer ut wisi enim ad minimum veniam quis nostrud exerci.

### Graphical optimisations

Lorem ipsum dolor sit amet consectetuer ut wisi enim ad minimum veniam quis nostrud exerci.

# User Interface

## Application Interface

Lorem ipsum dolor sit amet consectetuer ut wisi enim ad minimum veniam quis nostrud exerci.

## Debugging Interface

Lorem ipsum dolor sit amet consectetuer ut wisi enim ad minimum veniam quis nostrud exerci.

# Project Management

## Risk Assessment

Lorem ipsum dolor sit amet consectetuer ut wisi enim ad minimum veniam quis nostrud exerci.

## Time Management

Lorem ipsum dolor sit amet consectetuer ut wisi enim ad minimum veniam quis nostrud exerci.

## Project plan

Lorem ipsum dolor sit amet consectetuer ut wisi enim ad minimum veniam quis nostrud exerci.

# Conclusion

## Achievements

Lorem ipsum dolor sit amet consectetuer ut wisi enim ad minimum veniam quis nostrud exerci.

# Bibliography

Propeller Manual

Jeff Martin, 2006, Parallax

Game Programming for the Propeller Powered HYDRA

Andrè LaMothe, 2006, Nurve Networks

Z80 Programming

Rod Zaks, 1979, Sybex

Understanding your ZX81 ROM

Ian Logan, 1981, Melbourne House Publishers

Applying UML and Patterns

Craig Larman. 1998, PrenticeHall PTR

- Appendix - Instructions Descriptions

  - 8 bit Load Group

| Mnemonic | Symbolic Operation |  | Flags S Z F5 H F3 P/V N C | Opcode 76 543 210 | Hex | No. of Bytes | No. of M Cycles | No. of T States | Comments |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| LD r, r’ | r  r’ |  | • • • • • • • • | 01 r r’ |  | 1 | 1 | 4 | r, r’ Reg. |
| LD p, p’* | p  p’ |  | • • • • • • • • | 11 011 101 01 p p’ | DD | 2 | 2 | 8 | 000 B 001 C |
| LD q, q’* | q  q’ |  | • • • • • • • • | 11 111 101 01 q q’ | FD | 2 | 2 | 8 | 010 D 011 E |
| LD r, n | r  n |  | • • • • • • • • | 00 r 110  n  |  | 2 | 2 | 7 | 100 H 101 L |
| LD p, n* | p  n |  | • • • • • • • • | 11 011 101 00 p 110  n  | DD | 3 | 3 | 11 | 111 A p, p’ Reg. |
| LD q, n* | q  n |  | • • • • • • • • | 11 111 101 00 q 110  n  | FD | 3 | 3 | 11 | 000 B 001 C 010 D |
| LD r, (HL) | r  (HL) |  | • • • • • • • • | 01 r 110 |  | 1 | 2 | 7 | 011 E |
| LD r, (IX + d) | r  (IX + d) |  | • • • • • • • • | 11 011 101 01 r 110  d  | DD | 3 | 5 | 19 | 100 IXH 101 IXL 111 A |
| LD r, (IY + d) | r  (IY + d) |  | • • • • • • • • | 11 111 101 01 r 110  d  | FD | 3 | 5 | 19 | q, q’ Req. 000 B |
| LD (HL), r | (HL)  r |  | • • • • • • • • | 01 110 r |  | 1 | 2 | 7 | 001 C |
| LD (IX + d), r | (IX + d)  r |  | • • • • • • • • | 11 011 101 01 110 r  d  | DD | 3 | 5 | 19 | 010 D 011 E 100 IYH |
| LD (IY + d), r | (IY + d)  r |  | • • • • • • • • | 11 111 101 01 110 r  d  | FD | 3 | 5 | 19 | 101 IYL 111 A |
| LD (HL), n | (HL)  n |  | • • • • • • • • | 00 110 110  n  | 36 | 2 | 3 | 10 |  |
| LD (IX + d), n | (IX + d)  n |  | • • • • • • • • | 11 011 101 00 110 110  d   n  | DD 36 | 4 | 5 | 19 |  |
| LD (IY + d), n | (IY + d)  n |  | • • • • • • • • | 11 111 101 00 110 110  d   n  | FD 36 | 4 | 5 | 19 |  |
| LD A, (BC) | A  (BC) |  | • • • • • • • • | 00 001 010 | 0A | 1 | 2 | 7 |  |
| LD A, (DE) | A  (DE) |  | • • • • • • • • | 00 011 010 | 1A | 1 | 2 | 7 |  |
| LD A, (nn) | A  (nn) |  | • • • • • • • • | 00 111 010  n   n  | 3A | 3 | 4 | 13 |  |
| LD (BC), A | (BC)  A |  | • • • • • • • • | 00 000 010 | 02 | 1 | 2 | 7 |  |
| LD (DE), A | (DE)  A |  | • • • • • • • • | 00 010 010 | 12 | 1 | 2 | 7 |  |
| LD (nn), A | (nn)  A |  | • • • • • • • • | 00 110 010  n   n  | 32 | 3 | 4 | 13 |  |
| LD A, I | A  I |  |    0  IFF2 0 • | 11 101 101 01 010 111 | ED 57 | 2 | 2 | 9 |  |
| LD A, R | A  R |  |    0  IFF2 0 • | 11 101 101 01 011 111 | ED 5F | 2 | 2 | 9 | R is read after it is increased. |
| LD I, A | I  A |  | • • • • • • • • | 11 101 101 01 000 111 | ED 47 | 2 | 2 | 9 |  |
| LD R, A | R  A |  | • • • • • • • • | 11 101 101 01 001 111 | ED 4F | 2 | 2 | 9 | R is written after it is increased. |
| Notes: |  | r, r’ means any of the registers A, B, C, D, E, H, L. p, p’ means any of the registers A, B, C, D, E, IXH, IXL. q, q’ means any of the registers A, B, C, D, E, IYH, IYL. ddL, ddH refer to high order and low order eight bits of the register respectively. * means unofficial instruction. |  |  |  |  |  |  |  |
| Flag Notation: |  | • = flag is not affected, 0 = flag is reset, 1 = flag is set,  = flag is set according to the result of the operation, IFF2 = the interrupt flip-flop 2 is copied. |  |  |  |  |  |  |  |

  - 16 bit Load Group

| Mnemonic | Symbolic Operation |  | Flags S Z F5 H F3 P/V N C | Opcode 76 543 210 | Hex | No. of Bytes | No. of M Cycles | No. of T States | Comments |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| LD dd, nn | dd  nn |  | • • • • • • • • | 00 dd0 001  n   n  |  | 3 | 3 | 10 | dd Pair 00 BC 01 DE |
| LD IX, nn | IX  nn |  | • • • • • • • • | 11 011 101 00 110 001  n   n  | DD 21 | 4 | 4 | 14 | 02 HL 03 SP |
| LD IY, nn | IY  nn |  | • • • • • • • • | 11 111 101 00 110 001  n   n  | FD 21 | 4 | 4 | 14 |  |
| LD HL, (nn) | L  (nn) H  (nn+1) |  | • • • • • • • • | 00 101 010  n   n  | 2A | 3 | 5 | 16 |  |
| LD dd, (nn) | ddL  (nn) ddH  (nn+1) |  | • • • • • • • • | 11 101 101 01 dd1 011  n   n  | ED | 4 | 6 | 20 |  |
| LD IX, (nn) | IXL  (nn) IXH  (nn+1) |  | • • • • • • • • | 11 011 101 00 101 010  n   n  | DD 2A | 4 | 6 | 20 |  |
| LD IY, (nn) | IYL  (nn) IYH  (nn+1) |  | • • • • • • • • | 11 111 101 00 101 010  n   n  | FD 2A | 4 | 6 | 20 |  |
| LD (nn), HL | (nn)  L (nn+1)  H |  | • • • • • • • • | 00 100 010  n   n  | 22 | 3 | 5 | 16 |  |
| LD (nn), dd | (nn)  ddL (nn+1)  ddH |  | • • • • • • • • | 11 101 101 01 dd0 011  n   n  | DD | 4 | 6 | 20 |  |
| LD (nn), IX | (nn)  IXL (nn+1)  IXH |  | • • • • • • • • | 11 011 101 00 100 010  n   n  | DD 22 | 4 | 6 | 20 |  |
| LD (nn), IY | (nn)  IYL (nn+1)  IYH |  | • • • • • • • • | 11 111 101 00 100 010  n   n  | FD 22 | 4 | 6 | 20 |  |
| LD SP, HL | SP  HL |  | • • • • • • • • | 11 111 001 | F9 | 1 | 1 | 6 |  |
| LD SP, IX | SP  IX |  | • • • • • • • • | 11 011 101 11 111 001 | DD F9 | 2 | 2 | 10 |  |
| LD SP, IY | SP  IY |  | • • • • • • • • | 11 111 101 11 111 001 | FD F9 | 2 | 2 | 10 |  |
| PUSH qq | SP  SP - 1 (SP)  qqH SP  SP - 1 (SP)  qqL |  | • • • • • • • • | 11 qq0 101 |  | 1 | 3 | 11 | qq Pair 00 BC 01 DE 10 HL |
| PUSH IX | SP  SP - 1 (SP)  IXH SP  SP - 1 (SP)  IXL |  | • • • • • • • • | 11 011 101 11 100 101 | DD E5 | 2 | 4 | 15 | 11 AF |
| PUSH IY | SP  SP - 1 (SP)  IYH SP  SP - 1 (SP)  IYL |  | • • • • • • • • | 11 111 101 11 100 101 | FD E5 | 2 | 4 | 15 |  |
| POP qq | (SP)  qqL SP  SP + 1 (SP)  qqH SP  SP + 1 |  | • • • • • • • • | 11 qq0 001 |  | 1 | 3 | 10 |  |
| POP IX | (SP)  IXL SP  SP + 1 (SP)  IXH SP  SP + 1 |  | • • • • • • • • | 11 011 101 11 100 001 | DD E1 | 2 | 4 | 14 |  |
| POP IY | (SP)  IYL SP  SP + 1 (SP)  IYH SP  SP + 1 |  | • • • • • • • • | 11 111 101 11 100 001 | FD E1 | 2 | 4 | 14 |  |
| Notes: |  | dd is any of the register pair BC, DE, HL, SP. qq is any of the register pair BC, DE, HL, AF. |  |  |  |  |  |  |  |
| Flag Notation: |  | • = flag is not affected, 0 = flag is reset, 1 = flag is set,  = flag is set according to the result of the operation. |  |  |  |  |  |  |  |

  - Exchange, Block Transfer and Search Groups

| Mnemonic | Symbolic Operation |  | Flags S Z F5 H F3 P/V N C | Opcode 76 543 210 | Hex | No.of Bytes | No.of M Cycles | No.of T States | Comments |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| EX DE, HL | DE  HL |  | • • • • • • • • | 11 101 011 | EB | 1 | 1 | 4 |  |
| EX AF, AF’ | AF  AF’ |  | • • • • • • • • | 00 001 000 | 08 | 1 | 1 | 4 |  |
| EXX | BC  BC’ DE  DE’ HL  HL’ |  | • • • • • • • • | 11 011 001 | D9 | 1 | 1 | 4 |  |
| EX (SP), HL | (SP+1)  H (SP)  L |  | • • • • • • • • | 11 100 011 | E3 | 1 | 5 | 19 |  |
| EX (SP), IX | (SP+1)  IXH (SP)  IXL |  | • • • • • • • • | 11 011 101 11 100 011 | DD E3 | 2 | 6 | 23 |  |
| EX (SP), IY | (SP+1)  IYH (SP)  IYL |  | • • • • • • • • | 11 111 101 11 100 011 | FD E3 | 2 | 6 | 23 |  |
| LDI | (DE)  (HL) DE  DE + 1 HL  HL + 1 BC  BC - 1 |  | • • 1 0 2 3 0 • | 11 101 101 10 100 000 | ED A0 | 2 | 4 | 16 |  |
| LDIR | (DE)  (HL) DE  DE + 1 HL  HL + 1 BC  BC - 1 repeat until: BC = 0 |  | • • 1 0 2 0 0 • | 11 101 101 10 110 000 | ED B0 | 2 2 | 5 4 | 21 16 | if BC  0 if BC = 0 |
| LDD | (DE)  (HL) DE  DE - 1 HL  HL - 1 BC  BC - 1 |  | • • 1 0 2 3 0 • | 11 101 101 10 101 000 | ED A8 | 2 | 4 | 16 |  |
| LDDR | (DE)  (HL) DE  DE - 1 HL  HL - 1 BC  BC - 1 repeat until: BC = 0 |  | • • 1 0 2 0 0 • | 11 101 101 10 111 000 | ED B8 | 2 2 | 5 4 | 21 16 | if BC  0 if BC = 0 |
| CPI | A - (HL) HL  HL + 1 BC  BC -1 |  | 4 4 5 4 6 3 1 • | 11 101 101 10 100 001 | ED A1 | 2 | 4 | 16 |  |
| CPIR | A - (HL) HL  HL + 1 BC  BC -1 Repeat until: A = (HL) or BC = 0 |  | 4 4 5 4 6 3 1 • | 11 101 101 10 110 001 | ED B1 | 2 2 | 5 4 | 21 16 | if BC  0 and A  (HL). if BC = 0 or A = (HL) |
| CPD | A - (HL) HL  HL - 1 BC  BC -1 |  | 4 4 5 4 6 3 1 • | 11 101 101 10 101 001 | ED A9 | 2 | 4 | 16 |  |
| CPDR | A - (HL) HL  HL - 1 BC  BC -1 Repeat until: A = (HL) or BC = 0 |  | 4 4 5 4 6 3 1 • | 11 101 101 10 111 001 | ED B9 | 2 2 | 5 4 | 21 16 | if BC  0 and A  (HL). if BC = 0 or A = (HL) |
| Notes: |  | 1 F5 is a copy of bit 1 of A + last transferred byte, thus (A + (HL))1 2 F3 is a copy of bit 3 of A + last transferred byte, thus (A + (HL))3 3 P/V flag is 0 if the result of BC - 1 = 0, otherwise P/V = 1. 4 These flags are set as in CP (HL) 5 F5 is copy of bit 1 of A - last compared address - H, thus (A - (HL) - H)1. H is as in F after the comparison. 6 F3 is copy of bit 3 of A - last compared address - H, thus (A - (HL) - H)3. H is as in F after the comparison. |  |  |  |  |  |  |  |
| Flag Notation: |  | • = flag is not affected, 0 = flag is reset, 1 = flag is set,  = flag is set according to the result of the operation. |  |  |  |  |  |  |  |

  - 8 bit Arithmetic and Logical Group

| Mnemonic | Symbolic Operation |  | Flags S Z F5 H F3 P/V N C | Opcode 76 543 210 | Hex | No.of Bytes | No.of M Cycles | No.of T States | Comments |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| ADD A, r | A  A + r |  |      V 0  | 10 000 r |  | 1 | 1 | 4 | r Reg. p Reg. |
| ADD A, p* | A  A + p |  |      V 0  | 11 011 101 10 000 p | DD | 2 | 2 | 8 | 000 B 000 B 001 C 001 C |
| ADD A, q* | A  A + q |  |      V 0  | 11 111 101 10 000 q | FD | 2 | 2 | 8 | 010 D 010 D 011 E 011 E |
| ADD A, n | A  A + n |  |      V 0  | 11 000 110  n  |  | 2 | 2 | 8 | 100 H 100 IXH 101 L 101 IXH |
| ADD A, (HL) | A  A + (HL) |  |      V 0  | 10 000 110 |  | 1 | 2 | 7 | 111 A 111 A |
| ADD A, (IX + d) | A  A + (IX + d) |  |      V 0  | 11 011 101 10 000 110  d  | DD | 3 | 5 | 19 |  |
| ADD A, (IY + d) | A  A + (IY + d) |  |      V 0  | 11 111 101 10 000 110  d  | FD | 3 | 5 | 19 |  |
| ADC A, s | A  A + s + CY |  |      V 0  | 001 |  |  |  |  | s is any of r, n, (HL), |
| SUB A, s | A  A - s |  |      V 1  | 010 |  |  |  |  | (IX+d), (IY+d), p, q |
| SBC A, s | A  A - s - CY |  |      V 1  | 011 |  |  |  |  | as shown for the ADD |
| AND s | A  A AND s |  |    1  P 0 0 | 100 |  |  |  |  | instruction. The |
| OR s | A  A OR s |  |    0  P 0 0 | 110 |  |  |  |  | underlined bits replace |
| XOR s | A  A XOR s |  |    0  P 0 0 | 101 |  |  |  |  | the underlined bits in |
| CP s | A - s |  |   1  1 V 1  | 111 |  |  |  |  | the ADD set. |
| INC r | r  r + 1 |  |      V 0 • | 00 r 100 |  | 1 | 1 | 4 |  |
| INC p* | p  p + 1 |  |      V 0 • | 11 011 101 00 p 100 | DD | 2 | 2 | 8 | q Reg. 000 B |
| INC q* | q  q + 1 |  |      V 0 • | 11 111 101 00 q 100 | FD | 2 | 2 | 8 | 001 C 010 D |
| INC (HL) | (HL)  (HL) + 1 |  |      V 0 • | 00 110 100 |  | 1 | 3 | 11 | 011 E |
| INC (IX + d) | (IX + d)  (IX + d) + 1 |  |      V 0 • | 11 011 101 00 110 100  d  | DD | 3 | 6 | 23 | 100 IYH 101 IYL 111 A |
| INC (IY + d) | (IY + d)  (IY + d) + 1 |  |      V 0 • | 11 111 101 00 110 100  d  | FD | 3 | 6 | 23 |  |
| DEC m | m  m - 1 |  |      V 1 • | 101 |  |  |  |  | m is any of r, p, q, (HL), (IX+d), (IY+d), as shown for the INC instruction. DEC same format and states as INC. Replace 100 with 101 in opcode. |
| Notes: |  | 1 F5 and F3 are copied from the operand (s), not from the result of (A - s). The V symbol in the P/V flag column indicates that the P/V flags contains the overflow of the operation. Similarly the P symbol indicates parity. r means any of the registers A, B, C, D, E, H, L. p means any of the registers A, B, C, D, E, IXH, IXL. q means any of the registers A, B, C, D, E, IYH, IYL. ddL, ddH refer to high order and low order eight bits of the register respectively. CY means the carry flip-flop. * means unofficial instruction. |  |  |  |  |  |  |  |
| Flag Notation: |  | • = flag is not affected, 0 = flag is reset, 1 = flag is set,  = flag is set according to the result of the operation. |  |  |  |  |  |  |  |

  - 16 bit Arithmetic Group

| Mnemonic | Symbolic Operation |  | Flags S Z F5 H F3 P/V N C | Opcode 76 543 210 | Hex | No.of Bytes | No.of M Cycles | No.of T States | Comments |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| ADD HL, ss | HL  HL + ss |  | • • 2 2 2 • 0 1 | 00 ss1 001 |  | 1 | 3 | 11 | ss Reg. |
| ADC HL, ss | HL  HL + ss + CY |  | 1 1 2 2 2 V1 0 1 | 11 101 101 01 ss1 010 | ED | 2 | 4 | 15 | 00 BC 01 DE |
| SBC HL, ss | HL  HL - ss - CY |  | 1 1 2 2 2 V1 1 1 | 11 101 101 01 ss0 010 | ED | 2 | 4 | 15 | 10 HL 11 SP |
| ADD IX, pp | IX  IX + pp |  | • • 2 2 2 • 0 1 | 11 011 101 00 pp1 001 | DD | 2 | 4 | 15 | pp Reg. |
| ADD IY, rr | IY  IY + rr |  | • • 2 2 2 • 0 1 | 11 111 101 00 rr1 001 | FD | 2 | 4 | 15 | 00 BC 01 DE |
| INC ss | ss  ss + 1 |  | • • • • • • • • | 00 ss0 011 |  | 1 | 1 | 6 | 10 IX |
| INC IX | IX  IX + 1 |  | • • • • • • • • | 11 011 101 00 100 011 | DD 23 | 2 | 2 | 10 | 11 SP |
| INC IY | IY  IY + 1 |  | • • • • • • • • | 11 111 101 00 100 011 | FD 23 | 2 | 2 | 10 | rr Reg. 00 BC |
| DEC ss | ss  ss - 1 |  | • • • • • • • • | 00 ss1 011 |  | 1 | 1 | 6 | 01 DE |
| DEC IX | IX  IX - 1 |  | • • • • • • • • | 11 011 101 00 101 011 | DD 2B | 2 | 2 | 10 | 10 IY 11 SP |
| DEC IY | IY  IY - 1 |  | • • • • • • • • | 11 111 101 00 101 011 | FD 2B | 2 | 2 | 10 |  |
| Notes: |  | The V symbol in the P/V flag column indicates that the P/V flags contains the overflow of the operation. ss means any of the registers BC, DE, HL, SP. pp means any of the registers BC, DE, IX, SP. rr means any of the registers BC, DE, IY, SP. 16 bit additions are performed by first adding the two low order eight bits, and then the two high order eight bits. 1 Indicates the flag is affected by the 16 bit result of the operation. 2 Indicates the flag is affected by the 8 bit addition of the high order eight bits. CY means the carry flip-flop. |  |  |  |  |  |  |  |
| Flag Notation: |  | • = flag is not affected, 0 = flag is reset, 1 = flag is set,  = flag is set according to the result of the operation. |  |  |  |  |  |  |  |

  - General Purpose Arithmetic and CPU Control Groups

| Mnemonic | Symbolic Operation |  | Flags S Z F5 H F3 P/V N C | Opcode 76 543 210 | Hex | No.of Bytes | No.of M Cycles | No.of T States | Comments |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| DAA | Converts A into packed BCD following add or subtract with BCD operands. __ |  |      P •  | 00 100 111 | 27 | 1 | 1 | 4 |  |
| CPL | A  A |  | • • 1 1 1 • 1 • | 00 101 111 | 2F | 1 | 1 | 4 | One’s complement. |
| NEG4 | A  0 - A __ |  |      V 1  | 11 101 101 01 000 100 | ED 44 | 2 | 2 | 8 | Two’s complement. |
| CCF | CY  CY |  | • • 1 2 1 • 0  | 00 111 111 | 3F | 1 | 1 | 4 | Complement carry flag. |
| SCF | CY  1 |  | • • 1 0 1 • 0 1 | 00 110 111 | 37 | 1 | 1 | 4 |  |
| NOP | No operations |  | • • • • • • • • | 00 000 000 | 00 | 1 | 1 | 4 |  |
| HALT | CPU halted |  | • • • • • • • • | 01 110 110 | 76 | 1 | 1 | 4 |  |
| DI3 | IFF1  0 IFF2  0 |  | • • • • • • • • | 11 110 011 | F3 | 1 | 1 | 4 |  |
| EI3 | IFF1  1 IFF2  1 |  | • • • • • • • • | 11 111 011 | FB | 1 | 1 | 4 |  |
| IM 04 | Set interrupt mode 0 |  | • • • • • • • • | 11 101 101 01 000 110 | ED 46 | 2 | 2 | 8 |  |
| IM 14 | Set interrupt mode 1 |  | • • • • • • • • | 11 101 101 01 010 110 | ED 56 | 2 | 2 | 8 |  |
| IM 24 | Set interrupt mode 2 |  | • • • • • • • • | 11 101 101 01 011 110 | ED 5E | 2 | 2 | 8 |  |
| Notes: |  | The V symbol in the P/V flag column indicates that the P/V flags contains the overflow of the operation. Similarly the P symbol indicates parity. 1 F5 and F3 are a copy of bit 5 and 3 of register A _ 2 H contains the previous carry state (after instruction H  C) 3 No interrupts are issued directly after a DI or EI. 4 This instruction has other unofficial opcodes, see Opcodes list. CY means the carry flip-flop. |  |  |  |  |  |  |  |
| Flag Notation: |  | • = flag is not affected, 0 = flag is reset, 1 = flag is set,  = flag is set according to the result of the operation. |  |  |  |  |  |  |  |

  - Rotate and Shift Group

| Mnemonic | Symbolic Operation |  | Flags S Z F5 H F3 P/V N C | Opcode 76 543 210 | Hex | No. of Bytes | No. of M Cycles | No. of T States | Comments |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| RLCA |  |  | • •  0  • 0  | 00 000 111 | 07 | 1 | 1 | 4 |  |
| RLA |  |  | • •  0  • 0  | 00 010 111 | 17 | 1 | 1 | 4 |  |
| RRCA |  |  | • •  0  • 0  | 00 001 111 | 0F | 1 | 1 | 4 |  |
| RRA |  |  | • •  0  • 0  | 00 011 111 | 1F | 1 | 1 | 4 |  |
| RLC r |  |  |    0  P 0  | 11 001 011 00 000 r | CB | 2 | 2 | 8 | r Reg. 000 B |
| RLC (HL) |  |  |    0  P 0  | 11 001 011 00 000 110 | CB | 2 | 4 | 15 | 001 C 010 D |
| RLC (IX + d) |  |  |    0  P 0  | 11 011 101 11 001 011  d  00 000 110 | DD CB | 4 | 6 | 23 | 011 E 100 H 101 L 111 A |
| RLC (IY + d) |  |  |    0  P 0  | 11 111 101 11 001 011  d  00 000 110 | FD CB | 4 | 6 | 23 |  |
| LD r,RLC (IX + d)* | r  (IX + d) RLC r (IX + d)  r |  |    0  P 0  | 11 011 101 11 001 011  d  00 000 r | DD CB | 4 | 6 | 23 |  |
| LD r,RLC (IY + d)* | r  (IY + d) RLC r (IY + d)  r |  |    0  P 0  | 11 111 101 11 001 011  d  00 000 r | FD CB | 4 | 6 | 23 |  |
| RL m |  |  |    0  P 0  | 010 |  |  |  |  | Instruction format |
| RRC m |  |  |    0  P 0  | 001 |  |  |  |  | and states are the |
| RR m |  |  |    0  P 0  | 011 |  |  |  |  | same as RLC. |
| SLA m |  |  |    0  P 0  | 100 |  |  |  |  | Replace 000 with |
| SLL m* |  |  |    0  P 0  | 110 |  |  |  |  | new number. |
| SRA m |  |  |    0  P 0  | 101 |  |  |  |  |  |
| SRL m |  |  |    0  P 0  | 111 |  |  |  |  |  |
| RLD |  |  |    0  P 0 • | 11 101 101 01 101 111 | ED 6F | 2 | 5 | 18 |  |
| RRD |  |  |    0  P 0 • | 11 101 101 01 100 111 | ED 67 | 2 | 5 | 18 |  |
| Notes: |  | The P symbol in the P/V flag column indicates that the P/V flags contains the parity of the result. r means any of the registers A, B, C, D, E, H, L. * means unofficial instruction. CY means the carry flip-flop. |  |  |  |  |  |  |  |
| Flag Notation: |  | • = flag is not affected, 0 = flag is reset, 1 = flag is set,  = flag is set according to the result of the operation. |  |  |  |  |  |  |  |

  - Bit Manipulation Group

| Mnemonic | Symbolic Operation |  | Flags S Z F5 H F3 P/V N C | Opcode 76 543 210 | Hex | No. of Bytes | No. of M Cycles |  | No. of T States | Comments |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| BIT b, r | _ Z  rb |  | 1  2 1 3 4 0 • | 11 001 011 01 b r | CB | 2 | 2 | 8 |  | r Reg. 000 B |
| BIT b, (HL) | ___ Z  (HL)b |  | 1  2 1 3 4 0 • | 11 001 011 01 b 110 | CB | 2 | 3 | 12 |  | 001 C 010 D |
| BIT b, (IX + d)5 | _____ Z  (IX + d)b |  | 1  2 1 3 4 0 • | 11 011 101 11 001 011  d  01 b 110 | DD CB | 4 | 5 | 20 |  | 011 E 100 H 101 L 111 A |
| BIT b, (IY + d)5 | _____ Z  (IY + d)b |  | 1  2 1 3 4 0 • | 11 111 101 11 001 011  d  01 b 110 | FD CB | 4 | 5 | 20 |  |  |
| SET b, r | rb  1 |  | • • • • • • • • | 11 001 011 11 b r | CB | 2 | 2 | 8 |  | b Bit. 000 0 001 1 |
| SET b, (HL) | (HL)b  1 |  | • • • • • • • • | 11 001 011 11 b 110 | CB | 2 | 4 | 15 |  | 010 2 011 3 |
| SET b, (IX + d) | (IX + d)b  1 |  | • • • • • • • • | 11 011 101 11 001 011  d  11 b 110 | DD CB | 4 | 6 | 23 |  | 100 4 101 5 110 6 111 7 |
| SET b, (IY + d) | (IY + d)b  1 |  | • • • • • • • • | 11 111 101 11 001 011  d  11 b 110 | FD CB | 4 | 6 | 23 |  |  |
| LD r,SET b, (IX + d)* | r  (IX + d) rb  1 (IX + d)  r |  | • • • • • • • • | 11 011 101 11 001 011  d  11 b r | DD CB | 4 | 6 | 23 |  |  |
| LD r,SET b, (IY + d)* | r  (IY + d) rb  1 (IY + d)  r |  | • • • • • • • • | 11 111 101 11 001 011  d  11 b r | FD CB | 4 | 6 | 23 |  |  |
| RES b, m | mb  0 m  r, (HL), (IX+d), (IY+d) |  | • • • • • • • • | 10 |  |  |  |  |  | To form new opcode replace 11 of SET b, s with 10. Flags and states are the same. |
| Notes: |  | The notation mb indicates bit b (0 to 7) of location m. BIT instructions are performed by an bitwise AND. 1 S is set if b = 7 and Z = 0 2 F5 is set if b = 5 and Z = 0 3 F3 is set if b = 3 and Z = 0 4 P/V is set like the Z flag 5 This instruction has other unofficial opcodes * means unofficial instruction. |  |  |  |  |  |  |  |  |
| Flag Notation: |  | • = flag is not affected, 0 = flag is reset, 1 = flag is set,  = flag is set according to the result of the operation. |  |  |  |  |  |  |  |  |

  - Input and Output Groups

| Mnemonic | Symbolic Operation |  | Flags S Z F5 H F3 P/V N C | Opcode 76 543 210 | Hex | No.of Bytes | No.of M Cycles | No.of T States | Comments |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| IN A, (n) | A  (n) |  | • • • • • • • • | 11 011 011  n  | DB | 2 | 3 | 11 | r Reg. 000 B |
| IN r, (C) | r  (C) |  |    0  P 0 • | 11 101 101 01 r 000 | ED | 2 | 3 | 12 | 001 C 010 D |
| IN (C)* or IN F, (C)* | Just affects flags, value is lost. |  |    0  P 0 • | 11 101 101 01 110 000 | ED 70 | 2 | 3 | 12 | 011 E 100 H |
| INI | (HL)  (C) HL  HL + 1 B  B - 1 |  | 1 1 1 3 1 X 2 3 | 11 101 101 10 100 010 | ED A2 | 2 | 4 | 16 | 101 L 111 A |
| INIR | (HL)  (C) HL  HL + 1 B  B - 1 Repeat until B = 0 |  | 0 1 0 3 0 X 2 3 | 11 101 101 10 110 010 | ED B2 | 2 2 | 5 4 | 21 16 | if B  0 if B = 0 |
| IND | (HL)  (C) HL  HL - 1 B  B - 1 |  | 1 1 1 4 1 X 2 4 | 11 101 101 10 101 010 | ED AA | 2 | 4 | 16 |  |
| INDR | (HL)  (C) HL  HL - 1 B  B - 1 Repeat until B = 0 |  | 0 1 0 4 0 X 2 4 | 11 101 101 10 111 010 | ED BA | 2 2 | 5 4 | 21 16 | if B  0 if B = 0 |
| OUT (n), A | (n)  A |  | • • • • • • • • | 11 010 011  n  | D3 | 2 | 3 | 11 |  |
| OUT (C), r | (C)  r |  | • • • • • • • • | 11 101 101 01 r 001 | ED | 2 | 3 | 12 |  |
| OUT (C), 0* | (C)  0 |  | • • • • • • • • | 11 101 101 01 110 001 | ED 71 | 2 | 3 | 12 |  |
| OUTI | (C)  (HL) HL  HL + 1 B  B - 1 |  | 1 1 1 X 1 X X X | 11 101 101 10 100 011 | ED A3 | 2 | 4 | 16 |  |
| OTIR | (C)  (HL) HL  HL + 1 B  B - 1 Repeat until B = 0 |  | 0 1 0 X 0 X X X | 11 101 101 10 110 011 | ED B3 | 2 2 | 5 4 | 21 16 | if B  0 if B = 0 |
| OUTD | (C)  (HL) HL  HL - 1 B  B - 1 |  | 1 1 1 X 1 X X X | 11 101 101 10 101 011 | ED AB | 2 | 4 | 16 |  |
| OTDR | (C)  (HL) HL  HL - 1 B  B - 1 Repeat until B = 0 |  | 0 1 0 X 0 X X X | 11 101 101 10 111 011 | ED BB | 2 2 | 5 4 | 21 16 | if B  0 if B = 0 |
| Notes: |  | The V symbol in the P/V flag column indicates that the P/V flags contains the overflow of the operation. Similarly the P symbol indicates parity. r means any of the registers A, B, C, D, E, H, L. 1 flag is affected by the result of B  B - 1 as in DEC B. 2 N is a copy bit 7 of the last value from the input (C). 3 this flag contains the carry of ( ( (C + 1) AND 255) + (C) ) 4 this flag contains the carry of ( ( (C - 1) AND 255) + (C) ) * means unofficial instruction. |  |  |  |  |  |  |  |
| Flag Notation: |  | • = flag is not affected, 0 = flag is reset, 1 = flag is set, X = flag is unknown,  = flag is set according to the result of the operation. |  |  |  |  |  |  |  |

  - Jump Group

| Mnemonic | Symbolic Operation |  | Flags S Z F5 H F3 P/V N C | Opcode 76 543 210 | Hex | No.of Bytes | No.of M Cycles | No.of T States | Comments |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| JP nn | PC  nn |  | • • • • • • • • | 11 000 011  n   n  | C3 | 3 | 3 | 10 |  |
| JP cc, nn | if cc is true, PC  nn |  | • • • • • • • • | 11 ccc 010  n   n  |  | 3 | 3 | 10 | ccc Condition 000 NZ 001 Z 010 NC 011 C 100 PO 101 PE 110 P |
| JR e | PC  PC + e |  | • • • • • • • • | 00 011 000  e - 2  | 18 | 2 | 3 | 12 | 111 M |
| JR ss, e | if ss is true PC  PC + e |  | • • • • • • • • | 00 ss 000  e - 2  |  | 2 2 | 3 2 | 12 7 | if ss is true if ss is false |
| JP HL | PC  HL |  | • • • • • • • • | 11 101 001 | E9 | 1 | 1 | 4 |  |
| JP IX | PC  IX |  | • • • • • • • • | 11 011 101 11 101 001 | DD E9 | 2 | 2 | 8 | ss Condition 111 C 110 NC |
| JP IY | PC  IY |  | • • • • • • • • | 11 111 101 11 101 001 | FD E9 | 2 | 2 | 8 | 101 Z 100 NZ |
| DJNZ e | B  B - 1 if B  0 PC  PC + e |  | • • • • • • • • | 00 010 000  e - 2  | 10 | 2 2 | 2 3 | 8 13 | if B = 0 if B  0 |
| Notes: |  | e is a signed two-complement number in the range <-126, 129> e - 2 in the opcode provides an effective number of PC + e as PC incremented by 2 prior to the addition of e. |  |  |  |  |  |  |  |
| Flag Notation: |  | • = flag is not affected, 0 = flag is reset, 1 = flag is set,  = flag is set according to the result of the operation. |  |  |  |  |  |  |  |

  - Call and Return Group

| Mnemonic | Symbolic Operation |  | Flags S Z F5 H F3 P/V N C | Opcode 76 543 210 | Hex | No.of Bytes | No.of M Cycles | No.of T States | Comments |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CALL nn | SP  SP - 1 (SP)  PCH SP  SP - 1 (SP)  PCL PC  nn |  | • • • • • • • • | 11 001 101  n   n  | CD | 3 | 5 | 17 |  |
| CALL cc, nn | if cc is true, SP  SP - 1 (SP)  PCH SP  SP - 1 (SP)  PCL PC  nn |  | • • • • • • • • | 11 ccc 100  n   n  |  | 3 3 | 3 5 | 10 17 | if cc is false if cc is true |
| RET | PCL  (SP) SP  SP + 1 PCH  (SP) SP  SP + 1 |  | • • • • • • • • | 11 001 001 | C9 | 1 | 3 | 10 |  |
| RET cc | if cc is true, PCL  (SP) SP  SP + 1 PCH  (SP) SP  SP + 1 |  | • • • • • • • • | 11 ccc 000 |  | 1 1 | 1 3 | 5 11 | if cc is false if cc is true |
| RETI2 | PCL  (SP) SP  SP + 1 PCH  (SP) SP  SP + 1 |  | • • • • • • • • | 11 101 101 01 001 101 | ED 4D | 2 | 4 | 14 | cc Condition 000 NZ 001 Z 010 NC 011 C |
| RETN1,2 | PCL  (SP) SP  SP + 1 PCH  (SP) SP  SP + 1 IFF1  IFF2 |  | • • • • • • • • | 11 101 101 01 000 101 | ED 45 | 2 | 4 | 14 | 100 PO 101 PE 110 P 111 M |
| RST p | SP  SP - 1 (SP)  PCH SP  SP - 1 (SP)  PCL PC  p |  | • • • • • • • • | 11 t 111 |  | 1 | 3 | 11 | t p 000 0h 001 8h 010 10h 011 18h 100 20h 101 28h 110 30h 111 38h |
| Notes: |  | 1 This instruction has other unofficial opcodes, see Opcode list. 2 Instruction also IFF1  IFF2 |  |  |  |  |  |  |  |
| Flag Notation: |  | • = flag is not affected, 0 = flag is reset, 1 = flag is set,  = flag is set according to the result of the operation. |  |  |  |  |  |  |  |

- Appendix