{  TESTING STEPPER IP1 and IP2 STATUS }

{{ This BS2 code works well and received "0" and/or "1" depending on the state of the switches IP1 and IP2.

---------------------BS2 Code Sample that DO work well---------

DO

SEROUT MotorSerIn,Baud2400, ["{V}"]

SERIN  MotorSerOut,Baud2400, [WAIT("["), BIN iP1, BIN iP2]

DEBUG "ip1 = ",DEC ip1,"  ", "ip2 = ", DEC ip2, CR

LOOP

' When there is - a.) "NO CONTACT - PIN NOT PUllED DOWN TO GND the BINATY out - put is = 1

'                 b.) "WITH CONTACT - PIN pulled down to GND the BINARY out -put is = 0      }

{{  Propeller - StepperMotor"Little Step-U"motor driver"   - 7/8/08     with DEBUG

Little Step-U connected as follows: Serial in  -  --> P6

Serial out -  --> P5

Busy       -  --> P4

IP1        -  --> P3

IP2        -  --> P27  }}

------------------ Stepper code starts here ----------------

CON

_clkmode = xtal1 + pll16x

_xinfreq = 5_000_000

VAR

OBJ

'

Stepper: "FullDuplexSerial"

BS2    : "BS2_Functions"

PUB Main | a,b

BS2.start(31,30)

Stepper.start(5,6,1,2400)         ‘stepper rx,tx,mode,baudrate -

Repeat

' this code works well

waitcnt(clkfreq/10 +cnt)

Stepper.Str(string("{E35}"))

Stepper.str(string("{V}"))

waitcnt(clkfreq/100 + cnt)                    '  until here

If Stepper.Rxcheck == "["

a:=Stepper.Rx - "0"

if stepper.Rx <> ","

abort

b:=stepper.Rx -"0"

if Stepper.Rx <> "]"

abort

'

BS2.Debug_Str(13)

BS2.Debug_Bin(a,1)

BS2.Debug_Str(13)                        '

BS2.Debug_Bin(b,1)