The Handbook of BASIC Stamp Supercomputing

First Edition September 2009

Table of Contents

Cover Photo											1

Introduction											2

Contents											3

Evolutionary											7

Why Build a Stamp Supercomputer?								7

About the Stamp BS1  										7

List of Features											8

Configuration & Computing Power								8

Overall Design  											9

Design Similarities to the BSS									9

Skyscraper Tower Construction									10

Skyscraper Personal Space									11

Protective Ground Field - EMI/RFI Shielding							11

SSS Schematics											12

Building Enumeration Circuits									12

Decoupling Capacitors										13

Additional BS1 Schematics									13

BS1 without a Brown out Detector							13

BS1 Serial Adapter									13

Stamp 1 Project Board Schematic rev C							14

The SIP BS1 module rev. A details							14

OEM BS1 Stamp Rev. A									15

BS1 IC Module schematic Rev B								15

Brownout Detector										16

SIP form BS1 rev. b with pin details								16

BS1 version D board										16

Wiring												17

Supercomputer Assembly Instructions								17

Testing												18

Parts List & Cost Guide (2009) - Basic Stamp SEED						18

Control Panel										18

Deterministic RC Circuits								18

Peripherals										19

Optional Base Station Bstat								19

Other Components, Tools and Supplies							19

Output												19

Power LED											19

Banner												19

Control Panel with Input Array									20

Wiring the Control Panel Pushbutton with LED							20

Wiring & Reading Toggle Switches/ LEDs							20

– Assorted Example Circuits								21

Switch Software											21

All Computers - Pin Assignments									21

Computer 1 Port Assignments									21

Computer 2 through 9 Port Assignments (8 computers)						22

Computer 10 Port Assignments (1 computer)							22

Global Computer Ports (Availability Depends on Projects)						22

Powering Up the Boards										22

Running a Single Board										22

Peripherals											22

Programming Section (SEED) Artificial Intelligence						23

Life Form – Self Evolving Enumerated Deterministic Programming

How the Code Works										24

A Summary of Artificial Intelligence Software for the Stamp SEED Supercomputer			25

in 256 Bytes of BS1 EEPROM

Artificial Intelligence Software									25

Code Posting											25

Sample Screens											25

Only One Program										25

Life Form											25

Unique												25

Life Span											25

Birth												25

Self Enumerating										25

Self Deterministic										25

Memory & Recall										25

Napping & Random Dreaming									25

Dream Watching										26

Sleep & Fuzzy Clock										26

Talking to Neighbors										26

Listening to Neighbors										26

Getting to Know Neighbors									26

Remembering Neighbors									26

Thinking & Working										26

End of Life											26

Personalities											26

Restrictions											26

Becoming Sick											26

Personified Maladies									26

Psychotic Breakdown									26

Fitful Sleep										27

Sudden Death										27

Brain Amnesia										27

No Resuscitating Situation								27

Alternate Identity Syndrome								27

Stroke											27

Stuttering										27

Code Fit											27

Processor Type											27

Changing the Code										27

Flow Control											27

Running the Code & Watching Evolution								28

The Story of Supercomputer Life – Comments by Dr. Humanoido					28

Life form Screens										28

Stamp_ai.bs1 Code Listing									29

Introduction										29

Features										29

Life Cycle Performance									29

Phases of SEED’s Minuscule AI								30

History											31

Phase 0 	Preparing the Enumeration Pins						31

Typical Pin	1 Resistance Code Chart							32

Phase I 	Formation and Birthing Process						32

Variable Examples							33

Phase II 	Personal Introduction, Flowchart						34

Some Program Elements						34

Phase III	Self Enumeration - Creating a Self Aware Identity			34

Phase IV 	Thought Monitor							34

Phase V 	Self Determination							34

Phase VI 	Early Memory – Remembering Self Identity				35

Phase VII	Nap									35

Phase VIII	Dream									35

Phase IX	Developing a Sense of Time (Fuzzy Clock)				36

Phase X		Sleep									36

Phase XI	Conversation – Introduction to Neighbors				37

Phase XII	Conversation – Listen to Neighbors					37

Phase XIII	Memorizing the Identity of Neighbors					37

Phase XIV	Long Term Memory – Recall Identity of Neighbors			37

Phase XV	Finding Purpose in Life – Do Work/Thinking				37

Phase XVI	Speak Plan Piezo Language						38

Phase XVII	End of Life Cycle – Suspended Animation/Hybernation			38

Self Modifying Code									38

Personal Sick Leave									39

What happens when the program ends? Does the life form die?					40

How to save or preserve the state of the life form?						40

Multiple Ways to Program a Stamp Supercomputer – Wait then Retrieve Method			40

The Software – Self Modifying Code								40

The Language											41

Brain Scanning											41

EEPROM Write Limitation									41

Computer Enumerating Pin Circuits – Theory and Practicality					41

Circuit												42

Part List for Enumerating Pin Circuit								42

SDEP.BS1 Program										42

Syntax - POT Pin, Scale, Variable								43

Variable											43

Pot											43

Finding the best Scale value								43

EEPROM (a directive at compile time, not at run time)						43

Base Station (Bstat) 										45

History												45

Wireless Communications Transceiver								45

Transmitter and Receiver Specifications								46

Transmitter Supply Current									46

Receiver Supply Current										46

Theory of Operation										47

PDN												47

RSSI (receiver only)										47

Calibration											47

Source Code BASIC Stamp® 1 Example Programs						47

TxCode_v_1.0.bs1									48

RxCode_v_1.0.bs1									48

Power Distribution										48

Power Draw											48

Piezo Speakers and Current Draw								48

Sleep Nap and Wake										49

Demo Software											49

transmit.bs1										49

Receive.bs1										49

Software Section										50

SERIN Options of Driving the BS1							50

SEROUT Options									50

Paralleled Demo Programs									51

Computer 1 Network Code								51

Computer 2 Network Code								51

Computer 3 Network Code								51

Piezo Speaker Test Programs									52

First Program										52

Second Program									52

Piezo Language (PLAN) Expanded Option							52

PLAN Command List										53

Audible Command List									53

Representations										53

Hardware Control									53

Applications Section: Ideas and Details								53

Application 1: Twinkle Power Lights! 								53

twinkle.bs1										54

Application 2: Adding Push Buttons and Toggle							54

Application 3: Build a Jacob’s Ladder of Sound! 							54

Application 4: Solving the Riddle of Pullup and Pulldown						54

Application 5: Using a Pin for both Input and Output - Sharing Pins (reference NV38)  		54

Circuit to Display Status of I/O Pin - Detect Switch Input, Output to LED				55

Application 6: Create a Physically Rising Chromatic Scale  					55

Application 7: Create Ten-Note Chords								55

Application 8: Build a Bstat Base Station  							55

Application 9: Add a Parallax 2x16 Serial Green Screen						56

Application 10: Add a Parallax 2x16 Parallel Green Screen					56

Application 11: Demonstrating the Hidden Low Power Mode					56

Application 12: Long Term Memory Using EEPROM						57

WRITE.bs1 Program									58

LPM Discovery! (Hidden Low Power Mode)							59

Application 13: Handling RAM as Short Term Memory						59

Application 14: Setting the Ports									60

Application 15: Developing Stamp Supercomputer Math						61

Mathematical Functions List, Math Examples						61

Application 16: Mystery of the Vanishing EEPROM						62

Application 17: Setting the Format of Pins							62

Application 18: Creating a Variable or Pin Alias							62

Application 19: How to reference a specific bit within a byte or word variable			62

Application 20: Determining Program Memory							62

Application 21: Performing Memory Dump							63

Application 22: Transmitting and Receiving Serial Messages Between BS1 Computers		63

Serial Transmitter Code									63

Syntax & Function Study								63

SERIN.bs1 Program									63

Application 23: Build a Minuscule Stamp Supercomputer						64

Minuscule Stamp Supercomputer Code							64

Application 24: Serial Communications by Computer Name					64

Transmitter Code									64

Receiver Code										65

Application 25: One Talks and Many Will Listen							65

Application 26: Number Talking and Listening, number_talker.bs1 Program			65

number_listener.bs1 Program								65

Application 27: Direct Addressing by Personal Name						65

Chip_talker.bs1 Program								65

Chip_listener.bs1 Program								66

Application 28: Build Your Own Basic Stamp One Serial Interface					66

Application 29: Build a TriCore Stamp Supercomputer						66

Simplified Stamp TriCore Supercomputer STS schematic						67

Self Enumeration Schematic									67

TriCore Supercomputer AI Software Listing, TriCore_AI.bs1 Program				67

Application 30: Putting Your Supercomputer to Sleep, Sleep.bs1 Program				71

Single Processor Specifications by Parallax							71

Actual Conditions Single Processor								71

Actual Conditions Ten Processors								71

Ten Processor Specifications									71

Comparative Speed										72

Upgrades and Improvements									72

Cocooning for Transport										72

Cocooning Procedure										72

Coming Out of the Cocoon									72

Resources and Links										73

Parallax Basic Stamp 1 Applications Source							73

A Growing family of Supercomputers								73

The Basic Stamp Supercomputer family 								73

Stamp Baby Supercomputer Announcement 06.10.09						73

A Very Short Guide to PBASIC 1 Instructions - Programming the BS1 Supercomputer		74

By Request - Interview with Dr. Humanoido							75

Project Developer - the Series of Basic Stamp Supercomputers				75

Basic Stamp 1 Instruction List									76

PBASIC Programming Guidelines for the Stamp 1							83

Control Codes and Printing Characters								85

Sources												86

The Completed Stamp Seed Supercomputer							87