Schematic:

Communications Region in BoeBotControl.cs

#region Communication

/**

*  Handshake happens here.  192 causes the Boe-Bot's program to restart.  It sends a 1,

*  PC replies with 2, then Boe-Bot replies with 3.  If Boe-Bot want's to restart handshake,

*  it can send a 1.

*/

private bool WaitForConnect()

{

byte[] buf = new byte[5];

buf[1] = 0;

buf[2] = 192;

SendPackets(ref buf);

do

{

if (!running)

{

break;

}

buf[2] = (byte)2;

SendPackets(ref buf);

} while (buf[2] != (byte)3);

msgCnt = (byte)buf[1];

return true;

//callback("*" + buf[0] + "-" + buf[1] + "-" + buf[2] + "-" + buf[3] + "-" + buf[4] + "*");

}

/**

*  Now, packet is set repeatedly until reply is received from the Boe-Bot.

*/

private bool SendPackets(ref byte[] packet)

{

if (packet.Length != 5) throw new SystemException("Invalid packet length!");

packet[0] = 255;                                            // Start byte.

packet[1] = msgCnt;                                         // Message index, incremened by Boe-Bot.

// Must be used in next message that is

// sent.

while (_serialPort.BytesToRead < 5)

{

_serialPort.Write(packet, 0, 5);

while (_serialPort.BytesToWrite > 0) Thread.Sleep(1);

}

_serialPort.Read(packet, 0, 5);                             // Get Boe-Bot's reply.

msgCnt = packet[1];                                         // Get next message count.

if (digFlag)                                                // If EnableDigitalSensors = true,

{                                                           // Boe-Bot replies with digital sensor

//                                                      // values.

// Use active-high convention

//

irLeft = !((packet[3] & 2) > 0);

irRight = !((packet[3] & 1) > 0);

wLeft = !((packet[3] & 8) > 0);

wRight = !((packet[3] & 4) > 0);

}

if (packet[2] == 1) handshake = false;                      // Boe-Bot requests reconnect.

//callback("^" + bytes + "@" + abytes + "#" + aabytes + "*");// + packet[0] + "-" + packet[1] + "-" + packet[2] + "-" + packet[3] + "-" + packet[4] + "*\r\n");

return true;

}

#endregion

0000

Communication part of BoeBotControlforMSRS.bs2

'Wait for the eb500 radio to be ready.

PAUSE 1000

'Wait for the EB500 Bluetooth connection to be established.

DO UNTIL IN5 = 1: LOOP

GOSUB Ram_Clear

Request_Connect:

pointer = RequestConnect

GOSUB Get_Packet

SEROUT 1, 84, [STR buffer \5]

Confirm_Connect:

pointer = ConnectionGranted

GOSUB Get_Packet

Wait_For_Confirm:

SEROUT 1, 84, [255, 0, 1, 0, 0]

SERIN 0, 84, 20, Wait_For_Confirm, [WAITSTR buffer \ 2,

buffer2, buffer3,

buffer4]

Request_Packet:

msgIndex = msgIndex + 1

pointer = RequestCommand

GOSUB Get_Packet

SEROUT 1, 84, [STR buffer \5]