'TRACY - The CO2 sensor is wired like this:

'From CNTL (HSW)pin to BOE p4

'From ALR pin       to BOE p3

'From Vin pin       to BOE Vin

'From Vss pin       to BOE Vss

'NOTES:

'Separate wires are soldered to the CO2 sensor at TP1 and TP3. They ARE NOT ATTACHED to anything but air...

'The BOE is otherwise NOT WIRED to the stand-alone breadboard

'p3 and p4 are unassigned in the program

' {$STAMP BS2e}

' {$PBASIC 2.5}

' ==============================================================================

'

'   File....... MAWDBOE_w_CO2_June.24.2010.BS2

'

' ==============================================================================

' -----[ I/O Definitions ]-------------------------------------------------

ShtDta  PIN 1        ' bi-directional data with pullup resistor SHTdta=(1)

ShtClk  PIN 0        ' clock (for Sensirion) with pulldown resistor SHTclk=(3)

' power (4)=Vss 4, (8)=Vdd

MAWDin   PIN 15      ' mk changed PIN 3 to PIN 15 - July 11 - input from altitude, 9600 baud inverted, 20 per second, 5 bytes.

DSclk    PIN 2      ' DS1302 RTC clock=(7)    ' ta moved DS pins to 7,6,5 instead of 2,5,7 - mk moved back to 2, 5, 7 - July 11

DSdta    PIN 5      ' DS1302 RTC data line=(6)

DSce    PIN 7      ' DS1302 RTC chip enable (high enable)=(5)

' power to DS1302 Vdd=(1), Vss=(4), Vbat=(8)

TX     PIN 8               ' Transmit Data   --> 27937.4 (RXD) output to Vinculum=(4)

RTS    PIN 9               ' Request To Send --> 27937.6 (CTS) to Vinculum=(6)

RX    PIN 10              ' Receive Data    <-- 27937.5 (TXD) from Vinculum=(5)

CTS    PIN 11              ' Clear To Send   <-- 27937.2 (RTS) from Vinculum=(2)

' power (1)=Vss, (3)=Vdd, (7)=n/c, (8)=ri

' ----- [start MCP I/O definitions] -------

CS              PIN     6                        ' Chip Select  (MCP3202.1)

Clock           PIN     14                         ' Clock        (MCP3202.7)

DataOut         PIN     13                         ' Pin 6 on ADC (MCP3202.6)

DataIn          PIN     12                         ' Pin 5 on ADC (MCP3202.5)

' -----[ Constants ]-------------------------------------------------------

vBaud           CON     84    ' Serial Baud Rate 9600 bps for Vinculum datalogger

ShtTemp         CON     %00011    ' read temperature command for Sensirion

ShtHumi         CON     %00101    ' read humidity command for Sensirion

Ack             CON     0      ' responses for Sensirion

NoAck           CON     1

logInterval   CON      1   ' choose 1,2,3,4,5,6,10,12,15,20,30 seconds

DegSym          CON     186     ' degrees symbol for DEBUG

Cnts2Mv         CON     $0139   ' x 1.22 (To Millivolts)

' -----[ Variables ]-------------------------------------------------------

CO2pp        VAR Word    ' <--- new variable!! Tracy - June 11, 2010

primo         VAR Word

iobyte  VAR primo.BYTE0    ' io to/from datalogger, humidty sensor, & debug

index  VAR primo.NIB2    ' loop counter in some routines

unused1  VAR primo.NIB3

secundo  VAR Word

flags  VAR secundo.BYTE0

ackbit  VAR flags.BIT0    ' acknowledge Sensirion

shtErr   VAR flags.BIT1    ' = 1 if error in sensirion read

dskErr  VAR flags.BIT2    ' =1 if the logger fails to work

land       VAR flags.BIT3       ' =1 after alloted time interval

' bits 4 to 7 are unused

second0  VAR secundo.BYTE1

waldo         VAR Word             ' scratch variable

soSHT  VAR waldo      ' alias as scratch variable for Sensirion

soSHT0  VAR soSHT.BYTE0    ' byte by byte

soSHT1  VAR soSHT.BYTE1

delay  VAR waldo      ' timeout for Sensirion

' motormovementtime  VAR waldo  ' alias as timer variable for movements

degC          VAR Word             ' temperature in Celsius*10 from Sensirion

rh            VAR Word             ' humidity in percent*10 from Sensirion

feet         VAR Word         ' reading from MAAWD altimeter

time0         VAR Word

second  VAR time0.BYTE0    ' from DS1302 RTC

minute  VAR time0.BYTE1    ' ditto

time1      VAR Word

hour    VAR time1.BYTE0    ' ditto

index2    VAR time1.BYTE1    ' extra index for loop count

buffer   VAR Word                  ' for data returned from logger or RTC

buf0  VAR buffer.BYTE0  ' will return as bytes

buf1  VAR buffer.BYTE1

' note the buffer can extend into unassigned space, take care.

'carbon   VAR Word

result0         VAR     Word                      ' Conversion Result CH0

result1         VAR     Byte                      ' Conversion Result CH1

mVolts0         VAR     Word                      ' Result0 --> mVolts

mVolts1         VAR     Byte                      ' Result1 --> mVolts

' -----[ EEPROM Data ]-----------------------------------------------------

' -----[ Thumb Drive Data Format ]-----------------------------------------

'

' data format with RTC clock time included

'

' 2 bytes hour

' 1 byte ":" <---- colon time separator

' 2 bytes minute

' 1 byte ":" <-- okay

' 2 bytes second

' 1 byte ","   <--- comma as field separator

' 1 byte sign, either - or space

' 3 bytes temperature integer part

' 1 byte "."

' 1 byte temperature decimal part

' 1 byte ","

' 3 bytes humidity integer part

' 1 byte "."

' 1 byte humidity decimal part

' 1 byte ","

' 5 byte altitude from MAWD

' 1 byte CR   <---- CRLF as record separator

' 1 byte LF

' 1 byte CR for the Vinculum terminator, this does not appear in the data file

' =

' 29 bytes total for Vinculum character count, 23 bytes per record on Thummb drive.

' -----[ Initialization ]--------------------------------------------------

Reset:

'OUTS =  %0001001000000000   ' indicator led is on. rts (p9) high to start, tx (p8) low to start.

'fedcba9876543210 - commented out by JA, June 14, 2010

'DIRS =  %1111001111111101   ' p1 is input for sensirion data, pa and pb input for rx and cts - commented out by JA,

' June 14, 2010

OUTS = %0001001000000000 ' indicator led is on. rts (p9) high to start, tx (p8) low to start.

'fedcba9876543210 - TA Program fix added by JA, June 14, 2010

DIRS = %1111001111110101 ' p1 is input for sensirion data, pa and pb

' TA Program fix added by JA, June 14, 2010nput FOR rx AND cts AND p3 FOR CO2

DEBUG CR, "Initializing...",BELL,CR, CR  ' mk changed 'wait' to 'initializing' July 11

GOSUB init_Message

PAUSE 10

GOSUB RTC_init  ' reset the clock to 00:00:00

PAUSE 1000

GOSUB RTC_init '----- mk added 08 March 2008-------

PAUSE 512

HIGH TX                               ' Initialize Transmit Line

LOW RTS                               ' Take Vinculum Out Of Reset

PAUSE 512                               ' Allow 1/2 Second To Settle

' LOW ledP        ' LED indicator off - long pulse signals reset

GOSUB V_synchronize           ' get ready to send data to thumb drive

DEBUG CR,"sync tries: ",DEC waldo

iobyte=waldo : GOSUB morse_numeral   ' show the tries count for debugging

GOSUB V_short_commands  ' enable short command mode on Vinculum/thumb drive

GOSUB V_Check_Drive

DEBUG CR,"disk tries: ",DEC waldo, CR

iobyte=waldo : GOSUB morse_numeral   ' show the tries count for debugging

' either print disk not found, else write

IF dskerr=1 THEN

DEBUG " not found"

ELSE

GOSUB V_open_datafile

SEROUT TX\CTS, vBaud, [ $8,$20,0,0,0,10,CR,"ASP data",CR, LF]

GOSUB V_purge

GOSUB V_close_datafile

ENDIF

DEBUG CR,"Time (sec)   Sample#  CO2(PPM) Temp(C) Humidity(%)   Altitude(ft)",BELL,CR

GOSUB SHT_Connection_Reset

PAUSE 256

' -----[ Program Code ]----------------------------------------------------

Main:

DO

'    PULSOUT ledP,50

' GOSUB RTC_read

GOSUB RTC_sync

' mk removed " minute =$01 OR " from the next line, right after IF command - 01 July 2009

' IF index2>100 THEN land=1   ' set flag to start movements after descent of one minute

index2 = index2 + 1

DEBUG CR,HEX2 hour,":",HEX2 minute,":",HEX2 second, TAB, DEC index2

GOSUB MAWD_get

GOSUB SHT_Measure_Temp

GOSUB SHT_Measure_Humidity

GOSUB Carbon_collect

GOSUB MCP_get

DEBUG TAB,REP "-"\degC.BIT15,DEC ABS degC/10,".",DEC1 ABS degC,degSym

DEBUG TAB, DEC rH/10, ".", DEC1 rH, "%"

DEBUG TAB, TAB, DEC feet, CR  ' mk added a second tab - July 11

GOSUB V_open_datafile  ' thumb drive

GOSUB V_write_record   ' one data record

GOSUB V_close_datafile  ' close the file

'    IF land THEN GOSUB movement - mk commented out - 01 July 2009

LOOP

END

' -----[ Subroutines ]-----------------------------------------------------

' -----[ RTC ]-----

RTC_init:

' send $00 to DS1302 control register (command $8E) to enable write

HIGH DSce

SHIFTOUT DSdta,DSclk,LSBFIRST,[$8E,$00]

LOW DSce

' set the clock time using burst command $BE

' second=0, minute=0, hour=0, date31=4, month=7, day7=9, year=2009

' also disable further writes: $80 at loc $7

HIGH DSce

SHIFTOUT DSdta,DSclk,LSBFIRST,[$BE,$00,$00,$00,$04,$07,$09,$09,$80]

LOW DSce

RETURN

rtc_Sync:

' paces data logging to the number of seconds specified in CONstant "logInterval"

buffer=loginterval*200  ' this variable is used as a temporary counter

DO:DO

GOSUB RTC_read

buffer=buffer-1   ' this countdown is an escape from loop in case RTC fails

LOOP UNTIL second<>second0 OR buffer=0  '

second0=second   ' change, update old value for tracking

LOOP UNTIL second.NIB1*10+second.NIB0//logInterval=0 OR buffer=0  ' wait for log interval to arrive

' DEBUG CR,?buffer

RETURN

RTC_read:

' retrieves the time to variable: second, minute, hour

HIGH DSce

SHIFTOUT DSdta,DSclk,LSBFIRST,[$BF]     ' burst mode read $BF command

SHIFTIN DSdta,DSclk,LSBPRE,[second,minute,hour]

LOW DSce

RETURN

' -----[ Vinculum/Thumb Drive]-----

' refer to email discussion around email http://forums.parallax.com/forums/default.aspx?f=6&p=7&m=247882#m252750

V_purge:

V_Get_Data:

SERIN RX\RTS, vbaud,128, V_Timeout, [STR buf0\12]

V_Timeout:  ' timeout is the normal exit, buffer not full

RETURN

V_synchronize:

'  Send 'E' to start the synchronization process.

'  Expect 'E' and a CR for the response, timing per pk

waldo=0   ' count tries

DO

GOSUB V_purge   ' clear the vinculum tx data buffer

PAUSE 256     ' wait about 1/4 second

SEROUT TX\CTS, vbaud, ["E", CR]        ' Sync Command Character

GOSUB V_get_Data                        ' Get Response

waldo=waldo+1

LOOP UNTIL (buf0="E" AND buf1=CR) OR waldo=8

GOSUB V_purge                     ' Purge Buffer of leftover junk

SEROUT TX\CTS, vbaud, ["e", CR]        ' next Sync Command Character

GOSUB V_get_Data                        ' Get Response/purge

IF (buf0<>"e" OR buf1<>CR) THEN waldo=waldo + 1

IF waldo>7 THEN dskerr=1

' returns with number of tries in waldo

' returns with dskerr=1 if cant sync

RETURN

V_Check_Drive:

waldo=0

DO

GOSUB V_purge

SEROUT TX\CTS, vbaud, [CR]

GOSUB V_get_Data

IF buf0="N" AND buf1="D" THEN

waldo=9   ' error: disk not found

EXIT     ' no point to test more

ENDIF

waldo=waldo+1

PAUSE 100

LOOP UNTIL buf0=">" OR waldo=8

IF waldo>7 THEN dskerr=1

' return with number of tries in waldo

' waldo=9 means it got the no disk error

' dskerr flag=1 is tries>7

RETURN

V_short_commands:

SEROUT TX\CTS, vbaud, ["SCS", CR]

GOSUB v_get_Data                       ' purge

RETURN

V_Open_DataFile:

GOSUB V_purge      ' purge receive buffer

SEROUT TX\CTS, vbaud, [$09, $20, "datafile.txt", CR]

GOSUB V_purge      ' purge receive buffer

RETURN

v_Close_DataFile:

GOSUB V_purge                      ' Purge Receive Buffer

SEROUT TX\CTS, vbaud, [$0A, $20, "datafile.txt", CR]

GOSUB V_purge             ' purge receive buffer

RETURN

V_write_record:

' Write data with time to flash drive

' Added 'CO2 ppm' - June 11, 2010 - Tracy, Dylan, Justin

LOW RTS

SEROUT TX\CTS, vBaud, [ $8,$20,0,0,0,35,CR, HEX2 hour, ":" , HEX2 minute, ":", HEX2 second, "," , degC.BIT15*13+32,DEC3 ABS (degC/10),".", DEC1 ABS degC,",",DEC3 (RH/10),".",DEC1 RH,",", DEC5 feet,",",DEC5 CO2pp, CR, LF]

HIGH RTS

GOSUB V_purge

RETURN

' -----[ Sensirion ]-----

SHT_Connection_Reset:

SHIFTOUT ShtDta, ShtClk, LSBFIRST, [$FFF\9]

RETURN

'

' generates SHT1x "start" sequence

'          _____         _____

' ShtData       |_______|

'              ___     ___

' Clock    ___|   |___|   |___

SHT_Start:

INPUT ShtDta    ' let pull-up take line high

LOW ShtClk

HIGH ShtClk

LOW ShtDta

LOW ShtClk

HIGH ShtClk

INPUT ShtDta

LOW ShtClk

RETURN

' sends start sequence & command byte & retrieves two data bytes from Sensirion

SHT_measure:

GOSUB SHT_Start                               ' alert device

GOSUB SHT_Write_Byte                          ' send command (iobyte)

GOSUB SHT_Wait                                ' wait until measurement done

ackBit = Ack                                  ' another read follows

GOSUB SHT_Read_Byte                           ' get MSB

soSHT1 = ioByte

ackBit = NoAck                                ' last read

GOSUB SHT_Read_Byte                           ' get LSB

soSHT0 = ioByte

RETURN

'

' measure temperature:

' -- celsius = soT * 0.01 - 40

SHT_Measure_Temp:

ioByte = ShtTemp                             ' temperature command

GOSUB SHT_measure                         ' send command

degC = soSHT / 10 - 400                           ' convert to tenths C

RETURN

' measure humidity

SHT_Measure_Humidity:

ioByte = ShtHumi                             ' humidity command

GOSUB SHT_measure                             ' send command

rh = (soSHT ** 26542)

rh = rh - ((soSHT ** 3468) * (soSHT ** 3468) + 50 / 100)

rh = rh - 40

rh = ((degC + 250 / 10 - 50) * (soSHT ** 524 + 1) + (rh * 10)) + 5 / 10

RETURN

' sends "ioByte"

' returns "ackBit"

SHT_Write_Byte:

SHIFTOUT ShtDta, ShtClk, MSBFIRST, [ioByte]  ' send byte

SHIFTIN  ShtDta, ShtClk, LSBPRE, [ackBit\1]   ' get ack bit

RETURN

' returns "ioByte"

' sends "ackBit"

SHT_Read_Byte:

SHIFTIN  ShtDta, ShtClk, MSBPRE, [ioByte]    ' get byte

SHIFTOUT ShtDta, ShtClk, LSBFIRST, [ackBit\1] ' send ack bit

INPUT ShtDta                                 ' release data line

RETURN

' wait for sht to finish measurement

' (data line pulled low by SHT within 1/4 second)

SHT_Wait:

INPUT ShtDta                       ' data line is input

delay = 255                        ' give >1/4 second to finish

DO

delay=delay-1

PAUSE 1

LOOP UNTIL ShtDta=0 OR delay=0

IF delay=0 THEN

GOSUB SHT_Connection_Reset

shtErr=1

ENDIF

RETURN

' -----[ Carbon dioxide ]-----

' Added June 11, 2010 - Tracy, Dylan, Justin

Carbon_collect:

HIGH 4 'This activates the Heat Switch (HSW)   ' <--- this does not really need to be repeated, but it doesn't hurt

CO2pp = IN3           ' read the CO2 level.  Later, we will make this do more than just the alarm status

DEBUG TAB, DEC CO2pp   ' show the CO2 level

RETURN

' -----[ MCP3202 ADC ]-----

------------------------------------------------------------------------------------------------------------------------------------

MCP_get:

LOW CS                                          ' Enable ADC

SHIFTOUT DataIn, Clock, MSBFIRST, [%1101\4]     ' Select CH0, Single-Ended

' Commenting SHIFTOUT, out * TURNS 'HIGH VOLTAGE' (5.006 - 4096) OFF *

' When the next SHIFTOUT below (Select CH1) is commented out,

' both 'HIGH VOLTAGE' and 'LOW VOLTAGE (0.055 - 255) appear as 0.0 - 0

SHIFTIN DataOut, Clock, MSBPOST, [result0\12]    ' Read ADC

' Commenting this SHIFTIN, out makes HIGH voltage (5.006 - 4096)

' appear as really low voltage (0.015 - 13.) 0.005 - 215 remains same.

' When the next SHIFTIN below (Read ADC)) is commented out,

' both 'HIGH VOLTAGE' (0.015 - 13) remains and 'LOW VOLTAGE'

' appears as 0.0 – 0

------------------------------------------------------------------------------------------------------------------------------------------

HIGH CS                                         ' Disable ADC

mVolts0 = result0 */ Cnts2Mv                    ' Convert To Millivolts

LOW CS                                          ' Enable ADC

SHIFTOUT DataIn, Clock, MSBFIRST, [%1111\4]     ' Select CH1, Single-Ended * THIS TURNS VOLTAGE ON AND OFF *

SHIFTIN DataOut, Clock, MSBPOST, [result1\12]   ' Read ADC

HIGH CS                                         ' Disable ADC

mVolts1 = result1 */ Cnts2Mv                    ' Convert To Millivolts

DEBUG HOME, CRSRXY, 11, 0, DEC result0, CLREOL, ' Displays voltages & digital value

CRSRXY, 11, 1, DEC mVolts0 DIG 3,   ' for both channels

".", DEC3 mVolts0,

CRSRXY, 11, 2, DEC result1, CLREOL,

CRSRXY, 11, 3, DEC mVolts1 DIG 3,

".", DEC3 mVolts1

PAUSE 100

RETURN

' -----[ morse code ]-----

Morse_numeral:

iobyte = %11110000011111 >> iobyte & %11111 | %10100000

Morse:

FOR index = iobyte>>5 TO 1

'  HIGH ledP

PAUSE 192*iobyte.BIT0(index-1)+64

'  LOW ledP

PAUSE 256

NEXT

PAUSE 512

RETURN

morse_reading:

DEBUG 32

FOR ackbit=1 TO 0

iobyte = waldo DIG ackbit

DEBUG DEC1 iobyte

GOSUB morse_numeral

NEXT

RETURN

mcs CON 3*32+%000  ' dit dit dit

mco CON 3*32+%111 ' dah dah dah

mcn CON 2*32+%10   ' dah dit

mcf CON 4*32+%0010  ' dit dit dah dit

mcb CON 4*32+%1000   ' dah dit dit dit

mcc CON 4*32+%1010  ' dah dit dah dit

mcw CON 3*32+%011  ' dit dah dah

mce CON 1*32+%0     ' dit

mcv CON 4*32+%0001  ' dit dit dit dah

morse_message:

' plays back morse message up to null

' enter with waldo pointing to message start

' e.g. waldo=message1 : GOSUB morse_message

' message1 DATA mcs,mco,mcs,0  ' sos, example message for LED

DO

READ waldo,iobyte

IF iobyte=0 THEN EXIT

GOSUB morse

waldo=waldo+1

LOOP

RETURN

init_message:

'DEBUG CR, "Rocket Launch for International Student Satellites", CR

DEBUG CR, "ARLISS Team New Hampshire - June, 2010", CR, CR

'DEBUG "Humidity, temperature, and altitude data logging program.",CR, CR

'DEBUG "With sincere thanks to our many friends on the Parallax forum.", CR, CR, CR

'DEBUG "_______________________________________________________________ ", CR

RETURN

MAWD_get:

' alternative code to acquire MAWD data, uses DEC5 instead of STR in SERIN

PULSIN MAWDin,1,iobyte   ' the purpose of this is to synchronize with the MAWD timing.

PAUSE 10    ' this assures the SERIN will start waiting in the silent period before a MAWD xmission

feet=0

SERIN MAWDin,84,64,MAWDnot,[DEC5 feet]

MAWDnot:

RETURN