Pin Out:

RX/P31 is not attached if code has been secured.

PropPlug is inserted with badge up

uSD Card is attached to P00-P03

Securing your code*:

- Remove Power

- Short VDD & VSS

- Connect positive of power supply** to VDD

- Connect negative of power supply to RX

- Remove power supply

*Warning once secured can not be undone.  If you may need to change your code write in a back door method to do this through another pin.

**Power supply needs to be strong enough to supply at least 250mA with a voltage greater then 1V.