HO Switching Display Track Schematic

/\

/  l \

/   l   \

l

l

North

IR 5      _+__________

*                    \

\

IR 4      _+___________\____                                           ________________+__      IR 3

*                       /\      \                                         /                                 *

3        \                                     /

IR 1      _+_________________\_________________/__________________+__      IR 2

*			   /\			/\			    *

2                               1

Key:

Infra Red System Identification:

+   IR Emitters

*  IR Detectors

Turnout Identification:

Noted as:  1, 2, & 3 - - - Miniature Servo Actuated

Starting Position:	IR 1

Activity:

1.	 Start at IR 1 - - run to IR 2 &  then stop, change turnout position, pause 2  seconds & reverse.

2.	Run through turnout 2 and 3 to IR 5, stop, pause 2 seconds & reverse and run back to IR 2.

- At IR 2, stop, pause seconds, change turnout to straight, start and run

back to IR 1 then stop.

The remainder of the activity would mirror this through the other sidings.

Each of the other sidings will incorporate electromagnetic de-couplers, operated by

a relay board, actuated with a ULN2803 chip.

Pinouts for the OEM BS2 Stamp are as follows:

Pin:		0	“H” Bridge connection

- “H” Bridge connection

- IR Emitters - - (All 5 emitters driven from this pin through the

base of a TIP-120 darlington transistor located in the cathode lead.)

- IR Receiver Input - - - irDetect1

- IR Receiver Input - - - irDetect2

- IR Receiver Input - - - irDetect3

- IR Receiver Input - - - irDetect 4

- IR Receiver Input - - - irDetect5

- Open

- Open

- Open

- Open

- Open

- Turnout  #3 Servo

- Turnout #2 Servo

- Turnout #1 Servo

The plan is to utilize Pins 8, 9 and 10 for the de-coupling relay control.