' {$STAMP BS2p}

' {$PBASIC 2.5}

' -----[ I/O Definitions ]-------------------------------------------------

'Temp I/0

Reset        CON     8

Alarm        CON     9

Sensor       CON     10

TX           PIN     11              ' serial output to LCD

T9600        CON     240

' Vibration I/0

Dio          PIN     15              ' data to/from module

Clk          PIN     14              ' clock output

CS           PIN     13              ' active-low chip select

' Flex force I/0

sensorPin    CON     12              ' Flexiforce sensor circuit

' -----[ Variables ]-------------------------------------------------------

' temp variables

temperature  VAR    Word

tempL        VAR    temperature.LOWBYTE

tempH        VAR    temperature.HIGHBYTE

pec          VAR    Byte

Celsius      VAR    Byte            'Word

CelsiusDec   VAR    Byte            'Word

Fahrnheit    VAR    Byte            'Word

FahrnheitDec VAR    Byte            'Word

w            VAR    Byte

x            VAR    Byte

y            VAR    Byte

z            VAR    Byte

' Vibration

axis         VAR    Nib             ' axis selection

rvCount      VAR    Word            ' ref voltage adc counts

axCount      VAR    Word            ' axis voltage adc counts

mVolts       VAR    Word            ' millivolts

gForce       VAR    Word            ' axis g-force

dValue       VAR    Word            ' display value

dPad         VAR    Nib             ' display pad

'Flexiforce

rawForce     VAR     Word           ' Stores raw output

'-----[ Main  ]----------------------------------------------------

' Three major subs and Four different states.  Buttons 0 - 2 will serve

'for readout of data such as temperature, acceleration, and force.

'Buttons 3-6 will put the device in four different frequency states

'50Hz, 60Hz, 70Hz, 90Hz.

DO

IF IN4=0 THEN

AUXIO

HIGH 8

MAINIO

GOSUB FortyHz

ELSEIF IN5=0 THEN

AUXIO

HIGH 8

MAINIO

GOSUB FiftyHZ

ELSEIF IN6=0 THEN

AUXIO

HIGH 8

MAINIO

GOSUB SixtyHZ

ELSEIF IN7=0 THEN

AUXIO

HIGH 8

MAINIO

GOSUB SeventyHZ

ELSE

AUXIO

LOW 8

MAINIO

PAUSE 50

ENDIF

LOOP

'subs =======================================================

FortyHz:

DO  WHILE IN4=0

AUXIO

FREQOUT 9 , 5000 , 40

MAINIO

IF (IN0 =0)THEN                    'Temp

AUXIO

HIGH 10

MAINIO

GOSUB TempReadOut

ELSEIF (IN1 =0)THEN                  'Accel

AUXIO

HIGH 10

MAINIO

GOSUB Accelerometer

ELSEIF (IN2 = 0) THEN                'flex

AUXIO

HIGH 10

MAINIO

GOSUB WeitghMesurement

ELSE

AUXIO

LOW 10

MAINIO

PAUSE 100

ENDIF

LOOP

RETURN

'========================================================================

FiftyHz:

DO WHILE IN5 =0

AUXIO

FREQOUT 9, 5000, 50

MAINIO

IF (IN0 =0)THEN                    'Temp

AUXIO

HIGH 10

MAINIO

GOSUB TempReadOut

ELSEIF (IN1 =0)THEN                  'Accel

AUXIO

HIGH 10

MAINIO

GOSUB Accelerometer

ELSEIF (IN2 = 0) THEN                'flex

AUXIO

HIGH 10

MAINIO

GOSUB WeitghMesurement

ELSE

AUXIO

LOW 10

MAINIO

PAUSE 100

ENDIF

LOOP

RETURN

'==========================================================================

SixtyHz:

DO WHILE IN6 =0

AUXIO

FREQOUT 9, 5000, 60

MAINIO

IF (IN0 =0)THEN                  'Temp

AUXIO

HIGH 10

MAINIO

GOSUB TempReadOut

ELSEIF (IN1 =0)THEN                  'Accel

AUXIO

HIGH 10

MAINIO

GOSUB Accelerometer

ELSEIF (IN2 = 0) THEN                'flex

AUXIO

HIGH 10

MAINIO

GOSUB WeitghMesurement

ELSE

AUXIO

LOW 10

MAINIO

PAUSE 100

ENDIF

LOOP

RETURN

'============================================================================

SeventyHz:

DO WHILE IN7 =0

AUXIO

FREQOUT 9, 5000, 70

MAINIO

IF (IN0 =0)THEN                   'Temp

AUXIO

HIGH 10

MAINIO

GOSUB TempReadOut

ELSEIF (IN1 =0)THEN                 'Accel

AUXIO

HIGH 10

MAINIO

GOSUB Accelerometer

ELSEIF (IN2 = 0) THEN               'flex

AUXIO

HIGH 10

MAINIO

GOSUB WeitghMesurement

ELSE

AUXIO

LOW 10

MAINIO

PAUSE 100

ENDIF

LOOP

RETURN

'================================================================

'Nested Subs**************************************************************

' Temp sub=======================================================

TempReadOut:

' -----[ Constants ]-------------------------------------------------------

baud            CON     240

xslave          CON     $35          'slave address

LcdBaud         CON     T9600

LcdCls          CON     $0C          ' clear LCD (use PAUSE 5 after)

LcdBLoff        CON     $12          ' backlight off

LcdOn1          CON     $16          ' LCD on; cursor off, blink off

LcdLine1        CON     $80          ' move to line 1, column 0

LcdLine2        CON     $94          ' move to line 2, column 0

' -----[ EEPROM Data ]-----------------------------------------------------

PEC0   DATA $00,$07,$0E,$09,$1C,$1B,$12,$15,$38,$3F,$36,$31,$24,$23,$2A,$2D

PEC1   DATA $70,$77,$7E,$79,$6C,$6B,$62,$65,$48,$4F,$46,$41,$54,$53,$5A,$5D

PEC2   DATA $E0,$E7,$EE,$E9,$FC,$FB,$F2,$F5,$D8,$DF,$D6,$D1,$C4,$C3,$CA,$CD

PEC3   DATA $90,$97,$9E,$99,$8C,$8B,$82,$85,$A8,$AF,$A6,$A1,$B4,$B3,$BA,$BD

PEC4   DATA $C7,$C0,$C9,$CE,$DB,$DC,$D5,$D2,$FF,$F8,$F1,$F6,$E3,$E4,$ED,$EA

PEC5   DATA $B7,$B0,$B9,$BE,$AB,$AC,$A5,$A2,$8F,$88,$81,$86,$93,$94,$9D,$9A

PEC6   DATA $27,$20,$29,$2E,$3B,$3C,$35,$32,$1F,$18,$11,$16,$03,$04,$0D,$0A

PEC7   DATA $57,$50,$59,$5E,$4B,$4C,$45,$42,$6F,$68,$61,$66,$73,$74,$7D,$7A

PEC8   DATA $89,$8E,$87,$80,$95,$92,$9B,$9C,$B1,$B6,$BF,$B8,$AD,$AA,$A3,$A4

PEC9   DATA $F9,$FE,$F7,$F0,$E5,$E2,$EB,$EC,$C1,$C6,$CF,$C8,$DD,$DA,$D3,$D4

PEC10  DATA $69,$6E,$67,$60,$75,$72,$7B,$7C,$51,$56,$5F,$58,$4D,$4A,$43,$44

PEC11  DATA $19,$1E,$17,$10,$05,$02,$0B,$0C,$21,$26,$2F,$28,$3D,$3A,$33,$34

PEC12  DATA $4E,$49,$40,$47,$52,$55,$5C,$5B,$76,$71,$78,$7F,$6A,$6D,$64,$63

PEC13  DATA $3E,$39,$30,$37,$22,$25,$2C,$2B,$06,$01,$08,$0F,$1A,$1D,$14,$13

PEC14  DATA $AE,$A9,$A0,$A7,$B2,$B5,$BC,$BB,$96,$91,$98,$9F,$8A,$8D,$84,$83

PEC15  DATA $DE,$D9,$D0,$D7,$C2,$C5,$CC,$CB,$E6,$E1,$E8,$EF,$FA,$FD,$F4,$F3

' -----[ Initialization ]--------------------------------------------------

Init:

LOW Reset

INPUT Reset

DEBUG CLS,                                    ' setup report screen

"MLX90614 Infra Red Thermometer", CR,

"=============================", CR,CR,

' "Kelvin.........       ", CR,

"Celsius........       ", CR,

"Fahrnheit......       ", CR,

"PEC test.......       ", CR

' -----[ Program Code ]----------------------------------------------------

getTemperature:

DO WHILE IN0= 0

SEROUT Sensor,baud,[0,"!TEMR",xslave,$07]

SERIN Sensor,baud,1000,PECfail,[tempL,tempH,pec]

checkPEC:

z=0

y = xslave<<1+0

GOSUB calculateCRC

y = $07

GOSUB calculateCRC

y = xslave<<1+1

GOSUB calculateCRC

y = tempL

GOSUB calculateCRC

y = tempH

GOSUB calculateCRC

IF z<>pec THEN PECfail

GOTO convertTemperatures

calculateCRC:

w=z^y

READ w,z

RETURN

convertTemperatures:

'Kelvin = (temperature/100)*2

'KelvinDec = temperature*2

IF (temperature*2) < 27315 THEN overWordsize

Celsius = (temperature/100*2)-273

CelsiusDec = (temperature*2)-27315

Fahrnheit= (Celsius */ $01CD) + 32        ' $01CD represents 1.8 * 256 in HEX

FahrnheitDec = (CelsiusDec */ $01CD) + 32 ' $01CD represents 1.8 * 256 in HEX

GOTO displayTemperatures

overWordsize:

Celsius = ((27315-(temperature*2))/100)

CelsiusDec = (27315-(temperature*2))

DEBUG CRSRXY, 23, 4,"-",DEC Celsius,".",DEC2 CelsiusDec, CLREOL

displayTemperatures:

SEROUT TX, LcdBaud, [LcdBLoff, LcdOn1, LcdCls]

PAUSE 250

SEROUT TX, LcdBaud, [DEC Celsius,".",DEC2 CelsiusDec, " Celsius"]

SEROUT TX, LcdBaud, [(LcdLine2 ),DEC Fahrnheit,".",DEC2 FahrnheitDec, " Fahrnheit" ]'line 2

'DEBUG CRSRXY, 23, 3,DEC Kelvin,".",DEC1 KelvinDec,CLREOL

DEBUG CRSRXY, 23, 3,DEC Celsius,".",DEC2 CelsiusDec, CLREOL

DEBUG CRSRXY, 23, 4,DEC Fahrnheit,".",DEC2 FahrnheitDec, CLREOL

DEBUG CRSRXY, 23, 5,"Pass",CLREOL

PAUSE 500 '250

GOTO getTemperature

PECfail:

DEBUG CRSRXY, 23, 3,CLREOL,CR

DEBUG CRSRXY, 23, 4,CLREOL,CR

DEBUG CRSRXY, 23, 5,"Fail",CLREOL,CR

PAUSE 500 '250

GOTO getTemperature

LOOP

RETURN

' =========================================================================

'Accelerometer=============================================================

Accelerometer:

' -----[ Constants ]-------------------------------------------------------

XAxis CON 0        ' adc channels

YAxis CON 1

ZAxis CON 2

VRef  CON 3

Cnt2Mv CON $CE4C   ' counts to millivolts

' 0.80586 with **

GfCnv CON $3852    ' g-force conversion

' 0.22 with **

' -----[ Initialization ]--------------------------------------------------

HIGH CS

DEBUG CLS,

"=========================", CR,

"H48C 3-Axis Accelerometer", CR,

"=========================", CR,

CR,

"       Count Volts G ", CR,

"       ----- ----- -----", CR,

"VRef ", CR,

" X ", CR,

" Y ", CR,

" Z "

' -----[ Program Code ]----------------------------------------------------

DO WHILE IN1= 0

Main:

'FOR axis = XAxis TO ZAxis        loop through each axis

axis = ZAxis

GOSUB Get_H48C                    ' REAd vRef & axis counts

dValue = rvCount                  ' display vRef count

'DEBUG CRSRXY, 6, 6

'GOSUB RJ_Print

dValue = axCount                  ' display axis count

'DEBUG CRSRXY, 6, (7 + axis)

'GOSUB RJ_Print

mVolts = rvCount ** Cnt2Mv        ' convert vref to mv

'DEBUG CRSRXY, 13, 6,             ' display

'DEC (mVolts / 1000), ".",

'DEC3 mVolts

mVolts = axCount ** Cnt2Mv        ' convert axis to mv

'DEBUG CRSRXY, 13, (7 + axis),

'  DEC (mVolts / 1000), ".",

'  DEC3 mVolts

' calculate g-force

' -- "gForce" is signed word

IF (axCount >= rvCount) THEN

gForce = (axCount - rvCount) ** GfCnv        ' positive g-force

ELSE

gForce = -((rvCount - axCount) ** GfCnv)     ' negative g-force

ENDIF

SEROUT TX, LcdBaud, [LcdBLoff, LcdOn1, LcdCls]

PAUSE 500

SEROUT TX, LcdBaud, [DEC1 (ABS(gForce) / 100), ".", DEC2 ABS(gForce) ]

DEBUG CRSRXY, 20, (7 + axis),                  ' display g-force

" " + (gForce.BIT15 * 13),

DEC1 (ABS(gForce) / 100), ".",

DEC2 ABS(gForce)

PAUSE 500 '50 '200

LOOP

RETURN

GOTO Main

' -----[ Subroutines ]-----------------------------------------------------

' Reads VRef and selected H48C axis through an MCP3204 ADC

' -- pass axis (0 - 2) in "axis"

' -- returns reference voltage counts in "rvCount"

' -- returns axis voltage counts in "axCounts"

Get_H48C:

LOW CS

SHIFTOUT Dio, Clk, MSBFIRST, [%11\2, VRef\3]       ' select vref register

SHIFTIN Dio, Clk, MSBPOST, [rvCount\13]            ' read ref voltage counts

HIGH CS

PAUSE 1

LOW CS

SHIFTOUT Dio, Clk, MSBFIRST, [%11\2, axis\3]       ' select axis

SHIFTIN Dio, Clk, MSBPOST, [axCount\13]            ' read axis voltage counts

HIGH CS

RETURN

' =========================================================================

'Weitgh====================================================================

WeitghMesurement:

Measure:

DO WHILE IN2=0

HIGH sensorPin                        ' Discharge the capacitor

PAUSE 250

RCTIME sensorPin,1,rawForce           ' Measure RC charge time

DEBUG CLS

DEBUG HOME, "Flexiforce raw output = ", DEC rawForce,CR

SEROUT TX, LcdBaud, [LcdBLoff, LcdOn1, LcdCls]

PAUSE 250

SEROUT TX, LcdBaud, [DEC rawForce ]

GOTO Measure

LOOP

RETURN

' =========================================================================