' {$STAMP BS2pe}

' {$PBASIC 2.5}

' -----[ Program Description ]---------------------------------------------

' This program measures temperature using the Dallas Semiconductor DS1620

' temperature sensor. Resolution is 0.5 degrees Celsius.

' -----[ I/O Definitions ]-------------------------------------------------

DQ CON 3 ' DS1620.1 (data I/O)

Clock CON 2 ' DS1620.2

Reset CON 1 ' DS1620.3

' -----[ Constants ]-------------------------------------------------------

RdTmp CON $AA ' read temperature

WrHi CON $01 ' write TH (high temp)

WrLo CON $02 ' write TL (low temp)

RdHi CON $A1 ' read TH

RdLo CON $A2 ' read TL

RdCntr CON $A0 ' read counter

RdSlope CON $A9 ' read slope

StartC CON $EE ' start conversion

StopC CON $22 ' stop conversion

WrCfg CON $0C ' write config register

RdCfg CON $AC ' read config register

DegSym CON 186 ' degrees symbol

' -----[ Variables ]-------------------------------------------------------

tempIn VAR Word ' raw temperature

sign VAR tempIn.BIT8 ' 1 = negative temperature

tC VAR Word ' Celsius

' -----[ Initialization ]-------------------------------------------------

Setup:

HIGH Reset ' alert the DS1620

SHIFTOUT DQ, Clock, LSBFIRST, [WrCfg, %10] ' use with CPU; free-run

LOW Reset

PAUSE 10

'-------[Setting T_Hi]-------------------------------------------------------

HIGH Reset                                  ' Tells the DS1620 that a command is coming.

SHIFTOUT DQ, Clock, LSBFIRST, [WrHi,$34\9]  ' Command to set DS1620 TH to 32.

LOW Reset                                   ' Completes the command cycle.

PAUSE 10

'-------[Setting T_Lo]-------------------------------------------------------

HIGH Reset                                  ' Tells the DS1620 that a command is coming.

SHIFTOUT DQ, Clock, LSBFIRST, [WrLo,$32\9]  ' Command to set DS1620 TH to 31.

LOW Reset                                   ' Completes the command cycle.

PAUSE 10

'------------------------------------------

HIGH Reset

SHIFTOUT DQ, Clock, LSBFIRST, [StartC] ' start conversions

LOW Reset

Init_Graph:

DEBUG "!RSET", CR ' clear graph

DEBUG "!SPAN 25,35", CR ' disply 70 - 80 degrees

DEBUG "!AMUL 0.1", CR ' convert from tenths

DEBUG "!CLMM" ' clear min/max values

DEBUG "!TMAX 3600", CR ' 1 hour scale

DEBUG "!PNTS 60", CR ' graph every second

DEBUG "!MAXS", CR ' stop when graph full

DEBUG "!TSMP ON", CR ' enable time stamping

DEBUG "!TITL Temperature", CR ' set window title

DEBUG "!USRS Temperature in Celcius", CR ' graph legend

DEBUG "!CLRM", CR ' clear messages

DEBUG "Reset and monitoring temperature", CR' message box

DEBUG "!PLOT ON", CR ' enable plotting

' -----[ Program Code ]----------------------------------------------------

Main:

'DO

GOSUB Read_DS1620 ' get the temperature

tempIn.BYTE1 = -sign ' extend sign bit

tC = tempIn * 5 ' convert to tenths

DEBUG DEC tC, CR ' send to Stamp Plot Lite

PAUSE 990 ' wait about 1 second

GOTO Main ' do it again

' -----[ Subroutines ]-----------------------------------------------------

Read_DS1620:

HIGH Reset ' alert the DS1620

SHIFTOUT DQ, Clock, LSBFIRST, [RdTmp] ' give command to read temp

SHIFTIN DQ, Clock, LSBPRE, [tempIn\9] ' read it in

LOW Reset ' release the DS1620

RETURN