ARLISS “Night Before” Checklist

- Charge Ni-Cd battery

- Charge Beast battery

- Charge ARLISS transmitter (leave with Ken Biba)

- Set Beauty transmit time to 30 seconds (Jess)

- Check Beauty batteries and replace if needed

- Check Garmin GPS batteries and replace if needed

- Check Trek GPS batteries and replace if needed

- Check 9-volt lithium MAWD battery

- Check all backup batteries voltage

- Check BOE program for integrity by running for five minutes and checking data file, THEN: Remove flash drive and clear data file

- Secure surgical tubing across 9-volt battery

- Check walkie-talkie batteries

Pre-Flight Checklist

- Check all batteries for voltage.

- Check all nuts, bolts, connections …

- Fold main parachute and install in deployment bag.

- Attach pilot chute to deployment bag. Set both aside.

Arming Sequence

- Duct tape flash drive securely.

- Tape ARLISS transmitter battery to bulkhead.

- Arm Beast.

- Arm ARLISS transmitter.

- Check Beauty for reception.

- Have ARLISS check their receiver.

- Install 12-volt battery in ALT15K.

- Zip-tie ALT15K battery.

- Arm MAWD. Listen for double beeps.

- Arm BOE.

- Attach main parachute to surgical tubing.

- Tighten quick link!

- Install ASP in payload bay (upside down?)