'

' {$STAMP BS2}

' {$PBASIC 2.5}

'

' ==============================================================================

'

'   File....... ASP-2 with CO2 - August 12, 2010.BS2E

'

'   Purpose.... Carbon dioxide, humidity, temperature, altitude, and elapsed time data logging

'   Rocketeers. Andrew (am), Sean (sd), Dylan (dl), Justin (ja), Mike (mp)

'   Faculty....    Mark Kibler (mk)

'   Peanut gallery... Tracy Allen (ta), Paul Smith (Sylvie) (ps-s)

'   E-mail..... mkibler@alumni.unity.edu

'   Started.... February, 2008 - Repurposed June, 2010

'

'

' ==============================================================================

' -----[ Program Description ]---------------------------------------------

'

' ASP-2/MAWDBOE program with a Parallax CO2 sensor: The Air Sampling Probe (ASP-2) measures and records

' temperature, humidity, altitude, CO2 concentration, elapsed time, and the sample number. The MAWD is a

' PerfectFlite brand "Mini Altimeter With Deployment" wired into a Parallax Board of Education.

'

' -----[ Revision History ]------------------------------------------------

'

' -- 08 March 2008 (ta, pk)

' 1) added header section delineating the Vinculum thumb drive record structure

' 2) added subroutines for Vinculum init and file handling

'    V_synchronize, V_check_disk, V_short_commands -- these are initialization

'    V_getdata, V_purge   these two are the same routine, sometimes data simply purged

'    V_open_file, V_close_file,   these are in the main LOOP

' 3) forging the disk, a startup message shows how many tries TO access the disk

' 4) if the data file is opened successfully, "DaedelusCRLF" is written to the file

' 5) added one Word variable to hold more status flags or data.  Buffer is 12 bytes

' 6) LED flashes briefly each time through the loop.

'

'-- 06 March 2008 (mk, am, mw, bl, ds, jr, nm)

' 1) Changed data sampling time at descent to 1 sec intervals (from 2>10 to 2>100) per (ta)

' 2) Commented out 'GOSUB morse reading' twice in 'main:' subroutine per (ta)

' 3) Shortened duration of 'motormovement' times so we could bench test the program. These will be changed

'    back to their original 1 meter distance (0 TO 681) when we finalize the program.

' 4) The Rocketeers experimented by changing various settings to see what would happen,

'    and marveled at the Morse code light. Bandit the Rocket Dog was little help.

'

'-- 05 Mar 2008  (ta)

' Added increment of sample counter index2.  Displays on debug.  Land flag

' is set by either elapsed time or index2 accumulation.

'

' -- 04 Mar 2008  (ta) Mar04a

' Major restructuring from template, variables, starting point "RocketRobot_Feb29.BS2"

' Streamlined the Sensirion and movement routines with subroutine structure.

' Consolidated variables, named all pins

' This version incorporates the clock, LED and movements, but not yet the datalogger

'

'-- 01 July 2009 (mk)

' 1) 'Commented out' robot movement commands (servomotor movements) and inputs

' 2) 'Commented out' Morse code program for servomotor movement - Kept Morse code program for sync and disk tries

'

' -- 9 July 2009, (ta)

' 1) added device pin numbers in () in comments

' 2) made initial debug message a subroutine, "init_message"

' 3) added code to return a data value for MAWD, and to show it on debug screen after %RH

'    initially this is initially a dummy value, 12345 and RETURN, to test program loop and logging

'    added word VARiable "feet", PIN "MAWDin" on p3, and subroutine "MAWD_get"

'    call to MAWD_get inserted in MAIN loop.

' 4) increased size of data string for USB drive to 29 bytes, added field for feet of altitude

' 5) changed DS pins to p7,p6,p5 instead of p2,p5,p7

' 6) added means to synchronize data recording with DS1302 RTC

'    added CONstant "logInterval" (allowed intervals in seconds is 1,2,3,4,5,6,10,12,15,20,or 30)

'    e.g. if logInterval=5 then data will be logged at 5 second intervals

'    added subroutine sync_RTC, and MAIN calls sync_RTC instead of read_RTC

'    new VARiable second0 holds a comparison value to track the change in seconds

'

' -- 10 July 2009, (ta)

' 1) debugged subroutine MAWD_get using 2nd BASIC Stamp to simulate MAWD

'

' -- 25 June 2010, (mk)

' 1) Defined analog-digital converter (ADC) I/O pins for wires that go to the ASP-BOE.

' 2) Added Tracy and Andrew's 'MCP_get:' subroutine

'

' -- 29 June 2010, (mk)

' 1) Defined ALM - PIN 3

' 2) Defined CNTL - PIN 4

'

' -- 30 June 2010 (ta), (mk)

' 1) Condensed VAR 'result0, result1, mVolts0, and mVolts1' into a shorter,

'     more effficient 'MCP_get' subroutine (ta)

' 2) Commented out 'HIGH 4' command which was causing erratic CO2 mV data (mk)

' 3) Added column headings for 'CO2 mV' and 'CO2 ppm' (mk)

' 4) Commented out the 'carbon collect' command that made ALARM status 0 and 1 appear (ck)

' ==========================================================================================

' -----[ I/O Definitions ]-------------------------------------------------

ShtDta  PIN 1        ' bi-directional data with pullup resistor SHTdta=(1)

ShtClk  PIN 0        ' clock (for Sensirion) with pulldown resistor SHTclk=(3)

' power (4)=Vss 4, (8)=Vdd

MAWDin   PIN 15      ' mk changed PIN 3 to PIN 15 - July 11 - input from altitude, 9600 baud inverted, 20 per second, 5 bytes.

DSclk    PIN 2       ' DS1302 RTC clock=(7)    ' ta moved DS pins to 7,6,5 instead of 2,5,7 - mk moved back to 2, 5, 7 - July 11

DSdta    PIN 5       ' DS1302 RTC data line=(6)

DSce     PIN 7       ' DS1302 RTC chip enable (high enable)=(5)

' power to DS1302 Vdd=(1), Vss=(4), Vbat=(8)

TX     PIN 8         ' Transmit Data   --> 27937.4 (RXD) output to Vinculum=(4)

RTS    PIN 9         ' Request To Send --> 27937.6 (CTS) to Vinculum=(6)

RX     PIN 10        ' Receive Data    <-- 27937.5 (TXD) from Vinculum=(5)

CTS    PIN 11        ' Clear To Send   <-- 27937.2 (RTS) from Vinculum=(2)

' power (1)=Vss, (3)=Vdd, (7)=n/c, (8)=ri

' ----- [ MCP I/O definitions ] -------

ALM      PIN 3       ' Wire comes from the CH0 pin on the ADC breadboard to TP1 on the CO2 sensor.

CNTL     PIN 4       ' Wire comes from the CH1 pin on the ADC breadboard to TP3 on the CO2 sensor.

DataOut  PIN 6       ' DataIn and DataOut ADC pins are jump-connected with 1 K ohm resistor.

'   A separate wire runs from each ADC pin. DataIn and DataOut wires

'   are connected into one wire. This single wire goes into p6 on the ASP-BOE.

DataIn   PIN 6       ' DataIn and DataOut ADC pins are jump-connected with 1 K ohm resistor.

'   A separate wire runs from each ADC pin. DataIn and DataOut wires

'   are connected into one wire. This single wire goes into p6 on the ASP-BOE.

CS       PIN 12      ' Wire comes from the CS/SHDN pin on the ADC. It goes to BOE p12.

Clock    PIN 13      ' Wire comes from the CLK pin on the ADC. It goes to BOE p13.

' -----[ Constants ]-------------------------------------------------------

vBaud           CON     84      ' Serial Baud Rate 9600 bps for Vinculum datalogger

ShtTemp         CON     %00011  ' read temperature command for Sensirion

ShtHumi         CON     %00101  ' read humidity command for Sensirion

Ack             CON     0       ' responses for Sensirion

NoAck           CON     1

logInterval     CON     1       ' choose 1,2,3,4,5,6,10,12,15,20,30 seconds

DegSym          CON     186     ' degrees symbol for DEBUG

Cnts2Mv         CON     $0139   ' x 1.22 (To Millivolts)

' -----[ Variables ]-------------------------------------------------------

CO2pp      VAR Word              ' new variable: Tracy - June 11, 2010

primo      VAR Word

iobyte   VAR primo.BYTE0       ' io to/from datalogger, humidty sensor, & debug

index    VAR primo.NIB2        ' loop counter in some routines

unused1  VAR primo.NIB3

secundo    VAR Word

flags    VAR secundo.BYTE0

ackbit  VAR flags.BIT0       ' acknowledge Sensirion

shtErr  VAR flags.BIT1       ' = 1 if error in sensirion read

dskErr  VAR flags.BIT2       ' =1 if the logger fails to work

land    VAR flags.BIT3       ' =1 after alloted time interval

' bits 4 to 7 are unused

second0 VAR secundo.BYTE1

waldo      VAR Word             ' scratch variable

soSHT    VAR waldo            ' alias as scratch variable for Sensirion

soSHT0   VAR soSHT.BYTE0      ' byte by byte

soSHT1   VAR soSHT.BYTE1

delay    VAR waldo            ' timeout for Sensirion

' motormovementtime  VAR waldo

' alias as timer variable for movements

degC       VAR Word             ' temperature in Celsius*10 from Sensirion

rh         VAR Word             ' humidity in percent*10 from Sensirion

feet       VAR Word             ' reading from MAAWD altimeter

time0      VAR Word

second   VAR time0.BYTE0      ' from DS1302 RTC

minute   VAR time0.BYTE1      ' ditto

time1      VAR Word

hour     VAR time1.BYTE0      ' ditto

index2   VAR time1.BYTE1      ' extra index for loop count

buffer     VAR Word             ' for data returned from logger or RTC

buf0     VAR buffer.BYTE0     ' will return as bytes

buf1     VAR buffer.BYTE1

' note the buffer can extend into unassigned space, take care.

'---- [ carbon   VAR Word ] ----

'result0    VAR Word            ' Conversion Result CH0

'result1    VAR Byte            ' Conversion Result CH1

'mVolts0    VAR Word            ' Result0 --> mVolts

'mVolts1    VAR Byte            ' Result1 --> mVolts

' -----[ EEPROM Data ]-----------------------------------------------------

'

'

' -----[ Thumb Drive Data Format ]-----------------------------------------

'

' data format with RTC clock time included

'

' 2 bytes hour

' 1 byte ":" <---- colon time separator

' 2 bytes minute

' 1 byte ":" <-- okay

' 2 bytes second

' 1 byte ","   <--- comma as field separator

' 1 byte sign, either - or space

' 3 bytes temperature integer part

' 1 byte "."

' 1 byte temperature decimal part

' 1 byte ","

' 3 bytes humidity integer part

' 1 byte "."

' 1 byte humidity decimal part

' 1 byte ","

' 5 byte altitude from MAWD

' 1 byte CR   <---- CRLF as record separator

' 1 byte LF

' 1 byte CR for the Vinculum terminator, this does not appear in the data file

' =

' 29 bytes total for Vinculum character count, 23 bytes per record on Thummb drive.

' -----[ Initialization ]--------------------------------------------------

Reset:

'OUTS =  %0001001000000000   ' indicator led is on. rts (p9) high to start, tx (p8) low to start.

' fedcba9876543210 - commented out by JA, June 14, 2010

'DIRS =  %1111001111111101   ' p1 is input for sensirion data, pa and pb input for rx and cts - commented out by JA,

' June 14, 2010

OUTS = %0001001000000000     ' indicator led is on. rts (p9) high to start, tx (p8) low to start.

' fedcba9876543210 - TA Program fix added by JA, June 14, 2010

DIRS = %1111001111110101     ' p1 is input for sensirion data, pa and pb

' TA Program fix added by JA, June 14, 2010

' input FOR rx AND cts AND p3 FOR CO2

DEBUG CR, "Initializing...",BELL,CR, CR  ' mk changed 'wait' to 'initializing' July 11

GOSUB init_Message

PAUSE 10

GOSUB RTC_init                           ' reset the clock to 00:00:00

PAUSE 1000

GOSUB RTC_init                           ' mk added 08 March 2008

PAUSE 512

HIGH TX                                  ' Initialize Transmit Line

LOW RTS                                  ' Take Vinculum Out Of Reset

PAUSE 512                                ' Allow 1/2 Second To Settle

' LOW ledP                                ' LED indicator off - long pulse signals reset

GOSUB V_synchronize                      ' get ready to send data to thumb drive

DEBUG CR,"sync tries: ",DEC waldo

iobyte=waldo : GOSUB morse_numeral       ' show the tries count for debugging

GOSUB V_short_commands                   ' enable short command mode on Vinculum/thumb drive

GOSUB V_Check_Drive

DEBUG CR,"disk tries: ",DEC waldo, CR

iobyte=waldo : GOSUB morse_numeral       ' show the tries count for debugging

' either print disk not found, else write

IF dskerr=1 THEN

DEBUG " not found"

ELSE

GOSUB V_open_datafile

SEROUT TX\CTS, vBaud, [ $8,$20,0,0,0,10,CR,"ASP data",CR, LF]

GOSUB V_purge

GOSUB V_close_datafile

ENDIF

DEBUG CR,"Time (sec)  Sample#   CO2(mV) CO2(ppm) Temp(C) Humidity(%)  Altitude(ft)",BELL,CR

GOSUB SHT_Connection_Reset

PAUSE 256

' -----[ Program Code ]----------------------------------------------------

Main:

DO

' PULSOUT ledP,50

' GOSUB RTC_read

GOSUB RTC_sync

' mk removed " minute =$01 OR " from the next line, right after IF command - 01 July 2009

' IF index2>100 THEN land=1   ' set flag to start movements after descent of one minute

index2 = index2 + 1

DEBUG CR,HEX2 hour,":",HEX2 minute,":",HEX2 second, TAB, DEC index2

GOSUB MAWD_get

GOSUB SHT_Measure_Temp

GOSUB SHT_Measure_Humidity

GOSUB Carbon_collect

GOSUB MCP_get

DEBUG TAB,REP "-"\degC.BIT15,DEC ABS degC/10,".",DEC1 ABS degC,degSym

DEBUG TAB, DEC rH/10, ".", DEC1 rH, "%"

DEBUG TAB, TAB, DEC feet, CR  ' mk added a second tab - July 11

GOSUB V_open_datafile  ' thumb drive

GOSUB V_write_record   ' one data record

GOSUB V_close_datafile  ' close the file

' IF land THEN GOSUB movement - mk commented out - 01 July 2009

LOOP

END

' -----[ Subroutines ]-----------------------------------------------------

'

' -----[ RTC ]-----

RTC_init:  ' send $00 to DS1302 control register (command $8E) to enable write

HIGH DSce

SHIFTOUT DSdta,DSclk,LSBFIRST,[$8E,$00]

LOW DSce

' set the clock time using burst command $BE

' second=0, minute=0, hour=0, date31=4, month=7, day7=9, year=2009

' also disable further writes: $80 at loc $7

HIGH DSce

SHIFTOUT DSdta,DSclk,LSBFIRST,[$BE,$00,$00,$00,$04,$07,$09,$09,$80]

LOW DSce

RETURN

rtc_Sync:

' paces data logging to the number of seconds specified

' in CONstant "logInterval"

buffer=loginterval*200  ' this variable is used as a temporary counter

DO:DO

GOSUB RTC_read

buffer=buffer-1       ' this countdown is an escape from loop in case RTC fails

LOOP UNTIL second<>second0 OR buffer=0  '

second0=second          ' change, update old value for tracking

LOOP UNTIL second.NIB1*10+second.NIB0//logInterval=0 OR buffer=0

' wait for log interval to arrive

' DEBUG CR,?buffer

RETURN

RTC_read:

' retrieves the time to variable: second, minute, hour

HIGH DSce

SHIFTOUT DSdta,DSclk,LSBFIRST,[$BF]

' burst mode read $BF command

SHIFTIN DSdta,DSclk,LSBPRE,[second,minute,hour]

LOW DSce

RETURN

' -----[ Vinculum/Thumb Drive]-----

'

' refer to email discussion around email at:

' http://forums.parallax.com/forums/default.aspx?f=6&p=7&m=247882#m252750

V_purge:

V_Get_Data:

SERIN RX\RTS, vbaud,128, V_Timeout, [STR buf0\12]

V_Timeout:                ' timeout is the normal exit, buffer not full

RETURN

V_synchronize:

' Send 'E' to start the synchronization process.

' Expect 'E' and a CR for the response, timing per pk

waldo=0                   ' count tries

DO

GOSUB V_purge           ' clear the vinculum tx data buffer

PAUSE 256               ' wait about 1/4 second

SEROUT TX\CTS, vbaud, ["E", CR]           ' Sync Command Character

GOSUB V_get_Data                          ' Get Response

waldo=waldo+1

LOOP UNTIL (buf0="E" AND buf1=CR) OR waldo=8

GOSUB V_purge                             ' Purge Buffer of leftover junk

SEROUT TX\CTS, vbaud, ["e", CR]           ' next Sync Command Character

GOSUB V_get_Data                          ' Get Response/purge

IF (buf0<>"e" OR buf1<>CR) THEN waldo=waldo + 1

IF waldo>7 THEN dskerr=1

' returns with number of tries in waldo

' returns with dskerr=1 if cant sync

RETURN

V_Check_Drive:

waldo=0

DO

GOSUB V_purge

SEROUT TX\CTS, vbaud, [CR]

GOSUB V_get_Data

IF buf0="N" AND buf1="D" THEN

waldo=9                      ' error: disk not found

EXIT                         ' no point to test more

ENDIF

waldo=waldo+1

PAUSE 100

LOOP UNTIL buf0=">" OR waldo=8

IF waldo>7 THEN dskerr=1

' return with number of tries in waldo

' waldo=9 means it got the no disk error

' dskerr flag=1 is tries>7

RETURN

V_short_commands:

SEROUT TX\CTS, vbaud, ["SCS", CR]

GOSUB v_get_Data                 ' purge

RETURN

V_Open_DataFile:

GOSUB V_purge                    ' purge receive buffer

SEROUT TX\CTS, vbaud, [$09, $20, "datafile.txt", CR]

GOSUB V_purge                    ' purge receive buffer

RETURN

v_Close_DataFile:

GOSUB V_purge                      ' Purge Receive Buffer

SEROUT TX\CTS, vbaud, [$0A, $20, "datafile.txt", CR]

GOSUB V_purge                      ' purge receive buffer

RETURN

V_write_record:

' Write data with time to flash drive

' Added 'CO2 ppm' - June 11, 2010 - Tracy, Dylan, Justin

LOW RTS

SEROUT TX\CTS, vBaud, [ $8,$20,0,0,0,35,CR, HEX2 hour, ":" , HEX2 minute, ":", HEX2 second, "," , degC.BIT15*13+32,DEC3 ABS (degC/10),".", DEC1 ABS degC,",",DEC3 (RH/10),".",DEC1 RH,",", DEC5 feet,",",DEC5 CO2pp, CR, LF]

HIGH RTS

GOSUB V_purge

RETURN

' -----[ Sensirion ]-----

SHT_Connection_Reset:

SHIFTOUT ShtDta, ShtClk, LSBFIRST, [$FFF\9]

RETURN

'

' generates SHT1x "start" sequence

'          _____         _____

' ShtData       |_______|

'              ___     ___

' Clock    ___|   |___|   |___

'

'

SHT_Start:

INPUT ShtDta            ' let pull-up take line high

LOW ShtClk

HIGH ShtClk

LOW ShtDta

LOW ShtClk

HIGH ShtClk

INPUT ShtDta

LOW ShtClk

RETURN

' sends start sequence & command byte & retrieves two data bytes from Sensirion

SHT_measure:

GOSUB SHT_Start         ' alert device

GOSUB SHT_Write_Byte    ' send command (iobyte)

GOSUB SHT_Wait          ' wait until measurement done

ackBit = Ack            ' another read follows

GOSUB SHT_Read_Byte     ' get MSB

soSHT1 = ioByte

ackBit = NoAck          ' last read

GOSUB SHT_Read_Byte     ' get LSB

soSHT0 = ioByte

RETURN

' measure temperature:

' -- celsius = soT * 0.01 - 40

SHT_Measure_Temp:

ioByte = ShtTemp        ' temperature command

GOSUB SHT_measure       ' send command

degC = soSHT / 10 - 400 ' convert to tenths C

RETURN

' measure humidity

SHT_Measure_Humidity:

ioByte = ShtHumi        ' humidity command

GOSUB SHT_measure       ' send command

rh = (soSHT ** 26542)

rh = rh - ((soSHT ** 3468) * (soSHT ** 3468) + 50 / 100)

rh = rh - 40

rh = ((degC + 250 / 10 - 50) * (soSHT ** 524 + 1) + (rh * 10)) + 5 / 10

RETURN

' sends "ioByte"

' returns "ackBit"

SHT_Write_Byte:

SHIFTOUT ShtDta, ShtClk, MSBFIRST, [ioByte]   ' send byte

SHIFTIN  ShtDta, ShtClk, LSBPRE, [ackBit\1]   ' get ack bit

RETURN

' returns "ioByte"

' sends "ackBit"

SHT_Read_Byte:

SHIFTIN  ShtDta, ShtClk, MSBPRE, [ioByte]     ' get byte

SHIFTOUT ShtDta, ShtClk, LSBFIRST, [ackBit\1] ' send ack bit

INPUT ShtDta                                  ' release data line

RETURN

' wait for sht to finish measurement

' (data line pulled low by SHT within 1/4 second)

SHT_Wait:

INPUT ShtDta   ' data line is input

delay = 255    ' give >1/4 second to finish

DO

delay=delay-1

PAUSE 1

LOOP UNTIL ShtDta=0 OR delay=0

IF delay=0 THEN

GOSUB SHT_Connection_Reset

shtErr=1

ENDIF

RETURN

' -----[ Carbon dioxide ]-----

' Added June 11, 2010 - Tracy, Dylan, Justin

Carbon_collect:

HIGH 4                ' This activates the Heat Switch (HSW)

' <--- this does not really need to be repeated, but it doesn't hurt

CO2pp = IN3          ' read the CO2 level.  Later, we will make this do more than just the alarm status

' DEBUG TAB, DEC CO2pp  ' show the CO2 level ALARM STATUS 0 or 1

RETURN

' -----[ MCP3202 ADC ]-----

' Added June 14, 2010 by Andrew from Tracy

MCP_get:

' read channel 1 first, potentiometer, will NOT be logged

HIGH 4               ' This activates the Heat Switch (HSW)

LOW CS                                          ' Enable ADC

SHIFTOUT DataIn, Clock, MSBFIRST, [ %1111\4]  ' Select CH1, Single-Ended

SHIFTIN DataOut, Clock, MSBPOST, [ CO2pp\12]  ' Read ADC

HIGH CS                                         ' Disable ADC

CO2pp = CO2pp */ Cnts2Mv         ' Convert To Millivolts

DEBUG TAB, DEC CO2pp             ' shows the potentiometer voltage

' read channel 0, CO2 voltage, variable CO2pp will be logged on flash drive

LOW CS                                            ' Enable ADC

SHIFTOUT DataIn, Clock, MSBFIRST, [ %1101\4]   ' Select CH0, Single-Ended

SHIFTIN DataOut, Clock, MSBPOST, [ CO2pp\12]   ' Read ADC

HIGH CS                                          ' Disable ADC

CO2pp = CO2pp */ Cnts2Mv                         ' Convert To Millivolts

DEBUG TAB, DEC CO2pp                             ' show the CO2 voltage

' mVolts1 writes mV for CO2 potientiometer

RETURN

' -----[ Morse code ]-----

Morse_numeral:

iobyte = %11110000011111 >> iobyte & %11111 | %10100000

Morse:

FOR index = iobyte>>5 TO 1

' HIGH ledP

PAUSE 192*iobyte.BIT0(index-1)+64

' LOW ledP

PAUSE 256

NEXT

PAUSE 512

RETURN

morse_reading:

DEBUG 32

FOR ackbit=1 TO 0

iobyte = waldo DIG ackbit

DEBUG DEC1 iobyte

GOSUB morse_numeral

NEXT

RETURN

mcs CON 3*32+%000  ' dit dit dit

mco CON 3*32+%111  ' dah dah dah

mcn CON 2*32+%10   ' dah dit

mcf CON 4*32+%0010 ' dit dit dah dit

mcb CON 4*32+%1000 ' dah dit dit dit

mcc CON 4*32+%1010 ' dah dit dah dit

mcw CON 3*32+%011  ' dit dah dah

mce CON 1*32+%0    ' dit

mcv CON 4*32+%0001 ' dit dit dit dah

morse_message:

' plays back morse message up to null

' enter with waldo pointing to message start

' e.g. waldo=message1 : GOSUB morse_message

' message1 DATA mcs,mco,mcs,0  ' sos, example message for LED

DO

READ waldo,iobyte

IF iobyte=0 THEN EXIT

GOSUB morse

waldo=waldo+1

LOOP

RETURN

init_message:

'DEBUG CR, "ARLISS - A Rocket Launch for International Student Satellites", CR

DEBUG CR,"ARLISS Team New Hampshire - June, 2010", CR, CR

DEBUG "Carbon dioxide, temperature, humidity, altitude, and elapsed time data logging program",CR, CR

DEBUG "With sincere thanks to Dr. Tracy Allen and Paul Smith",CR, CR

DEBUG "And our friends at Parallax Inc. - www.parallax.com",  CR,CR

' DEBUG "__________________________________________________________________________ ", CR

RETURN

MAWD_get:

' alternative code to acquire MAWD data, uses DEC5 instead of STR in SERIN

PULSIN MAWDin,1,iobyte   ' the purpose of this is to synchronize with the MAWD timing.

PAUSE 10                 ' this assures the SERIN will start waiting in the silent period before

' a MAWD xmission

feet=0

SERIN MAWDin,84,64,MAWDnot,[DEC5 feet]

MAWDnot:

RETURN

'The end - Mark - August 12, 2010