Software used is PARALLAX  BASIC STAMP EDITOR 2.5.2

Coding for  MMA7455 3-AXIS ACCELEROMETER MODULE

' {$STAMP BS2e}

' {$PBASIC 2.5}

'How TO Use:

' • With power initially off, connect VIN TO 5VDC (the same voltage

'   powering the sTAMP).  Connect GND TO Vss.

'

' • Connect P0, P1, AND P2 ON the Stamp directly TO the CS, DATA, AND CLK

'   pins ON the Digital 3-Axis Accelerometer module.

'

' • Power ON the Stamp.  Download AND run this code ON the Stamp.

'

' • Acceleration values will stream back TO the computer, AND can be viewed

'   on the DEBUG terminal.

'

' • The offset values FOR each axis can be calibrated by placing the device

'   ON a flat horizontal surface AND adjusting the corresponding constants

'   UNTIL the values FOR each axis READ (WHILE in 2g mode):

'        X = 0   (0g)

'        Y = 0   (0g)

'        Z = 63  (+1g)

'   The values already present are FOR demonstration purposes AND can

'   be easily modified TO fine tune your own device. Keep in mind that

'   the offset values are in 1/2 Bit increments, so FOR example, TO offset

'   an axis by 5 counts, the corresponding offset would need TO be increased

'   by a value of 10.  See the MMA7455L device datasheet FOR more

'   information.

'Offset values FOR each axis:

XCal      CON 25    'VAR Word

YCal      CON 50    'VAR word

ZCal      CON 0     'VAR Word

CLKPin    PIN 0     ' Clock Pin

DATAPin   PIN 1     ' Data Pin

CSPin     PIN 2     ' Chip Select Pin

XOUTL     CON $00   ' 10 bits output value X LSB             XOUT[7]  XOUT[6]  XOUT[5]  XOUT[4]  XOUT[3]  XOUT[2]  XOUT[1]  XOUT[0]

XOUTH     CON $01   ' 10 bits output value X MSB             --       --       --       --       --       --       XOUT[9]  XOUT[8]

YOUTL     CON $02   ' 10 bits output value Y LSB             YOUT[7]  YOUT[6]  YOUT[5]  YOUT[4]  YOUT[3]  YOUT[2]  YOUT[1]  YOUT[0]

YOUTH     CON $03   ' 10 bits output value Y MSB             --       --       --       --       --       --       YOUT[9]  YOUT[8]

ZOUTL     CON $04   ' 10 bits output value Z LSB             ZOUT[7]  ZOUT[6]  ZOUT[5]  ZOUT[4]  ZOUT[3]  ZOUT[2]  ZOUT[1]  ZOUT[0]

ZOUTH     CON $05   ' 10 bits output value Z MSB             --       --       --       --       --       --       ZOUT[9]  ZOUT[8]

XOUT8     CON $06   ' 8 bits output value X                  XOUT[7]  XOUT[6]  XOUT[5]  XOUT[4]  XOUT[3]  XOUT[2]  XOUT[1]  XOUT[0]

YOUT8     CON $07   ' 8 bits output value Y                  YOUT[7]  YOUT[6]  YOUT[5]  YOUT[4]  YOUT[3]  YOUT[2]  YOUT[1]  YOUT[0]

ZOUT8     CON $08   ' 8 bits output value Z                  ZOUT[7]  ZOUT[6]  ZOUT[5]  ZOUT[4]  ZOUT[3]  ZOUT[2]  ZOUT[1]  ZOUT[0]

STATUS    CON $09   ' Status registers                       --       --       --       --       --       PERR     DOVR     DRDY

DETSRC    CON $0A   ' Detection source registers             LDX      LDY      LDZ      PDX      PDY      PDZ      INT1     INT2

TOUT      CON $0B   ' "Temperature output value" (Optional)  TMP[7]   TMP[6]   TMP[5]   TMP[4]   TMP[3]   TMP[2]   TMP[1]   TMP[0]

'         CON $0C   ' (Reserved)                             --       --       --       --       --       --       --       --

I2CAD     CON $0D   ' I2C device address I                   2CDIS    DAD[6]   DAD[5]   DAD[4]   DAD[3]   DAD[2]   DAD[1]   DAD[0]

USRINF    CON $0E   ' User information (Optional)            UI[7]    UI[6]    UI[5]    UI[4]    UI[3]    UI[2]    UI[1]    UI[0]

WHOAMI    CON $0F   ' "Who am I" value (Optional)            ID[7]    ID[6]    ID[5]    ID[4]    ID[3]    ID[2]    ID[1]    ID[0]

XOFFL     CON $10   ' Offset drift X value (LSB)             XOFF[7]  XOFF[6]  XOFF[5]  XOFF[4]  XOFF[3]  XOFF[2]  XOFF[1]  XOFF[0]

XOFFH     CON $11   ' Offset drift X value (MSB)             --       --       --       --       --       XOFF[10] XOFF[9]  XOFF[8]

YOFFL     CON $12   ' Offset drift Y value (LSB)             YOFF[7]  YOFF[6]  YOFF[5]  YOFF[4]  YOFF[3]  YOFF[2]  YOFF[1]  YOFF[0]

YOFFH     CON $13   ' Offset drift Y value (MSB)             --       --       --       --       --       YOFF[10] YOFF[9]  YOFF[8]

ZOFFL     CON $14   ' Offset drift Z value (LSB)             ZOFF[7]  ZOFF[6]  ZOFF[5]  ZOFF[4]  ZOFF[3]  ZOFF[2]  ZOFF[1]  ZOFF[0]

ZOFFH     CON $15   ' Offset drift Z value (MSB)             --       --       --       --       --       ZOFF[10] ZOFF[9]  ZOFF[8]

MCTL      CON $16   ' Mode control                           LPEN     DRPD     SPI3W    STON     GLVL[1]  GLVL[0]  MOD[1]   MOD[0]

INTRST    CON $17   ' Interrupt latch reset                  --       --       --       --       --       --       CLRINT2  CLRINT1

CTL1      CON $18   ' Control 1                              --       THOPT    ZDA      YDA      XDA      INTRG[1] INTRG[0] INTPIN

CTL2      CON $19   ' Control 2                              --       --       --       --       --       DRVO     PDPL     LDPL

LDTH      CON $1A   ' Level detection threshold limit value  LDTH[7]  LDTH[6]  LDTH[5]  LDTH[4]  LDTH[3]  LDTH[2]  LDTH[1]  LDTH[0]

PDTH      CON $1B   ' Pulse detection threshold limit value  PDTH[7]  PDTH[6]  PDTH[5]  PDTH[4]  PDTH[3]  PDTH[2]  PDTH[1]  PDTH[0]

PW        CON $1C   ' Pulse duration value                   PD[7]    PD[6]    PD[5]    PD[4]    PD[3]    PD[2]    PD[1]    PD[0]

LT        CON $1D   ' Latency time value                     LT[7]    LT[6]    LT[5]    LT[4]    LT[3]    LT[2]    LT[1]    LT[0]

TW        CON $1E   ' Time window for 2nd pulse value        TW[7]    TW[6]    TW[5]    TW[4]    TW[3]    TW[2]    TW[1]    TW[0]

'         CON $1F   ' (Reserved)                             --       --       --       --       --       --       --       --

XAccel    VAR Word  ' Variables to store incoming RAW data from the accelerometer

YAccel    VAR Word

ZAccel    VAR Word

Address   VAR Word  ' Variables for reading and writing data to the acclerometer

SendData  VAR Byte

ReceiveData VAR Byte

'

'         MCTL - Mode control register

'   +---------------------------------------+

'   ¦ D7 ¦ D6 ¦ D5 ¦ D4 ¦ D3 ¦ D2 ¦ D1 ¦ D0 ¦

'   +----+----+----+----+----+----+----+----¦

'   ¦ -- ¦DRPD¦SPI3¦STON¦GLVL¦GLVL¦MODE¦MODE¦

'   +---------------------------------------+

'

'

'   D7       - don't care          0

'

'   D6(DRPD) - DATA ready status   0 - OUTPUT TO INT1 PIN

'                                  1 - is NOT OUTPUT TO INT1 PIN

'

'   D5(SPI3W)- Wire Mode           0 - SPI is 4-wire mode

'                                  1 - SPI is 3-wire mode

'

'   D4(STON) - Self Test           0 - NOT enabled

'                                  1 - enabled

'

'   D3(GLVL[1]) - g-SELECT        00 - 8g ; 16 LSB/g in 8-Bit format

'   D2(GLVL[0]) - g-SELECT        10 - 4g ; 32 LSB/g in 8-Bit format

'                                 01 - 2g ; 64 LSB/g in 8-Bit format

'

'                                         ; Note: When reading g in 10-Bit

'                                         ;       format, resolution is fixed

'                                         ;       at 64 LSB/g

'                   10-Bit g register

'   +-------------------------------------------------+

'   ¦ D9 ¦ D8 ¦ D7 ¦ D6 ¦ D5 ¦ D4 ¦ D3 ¦ D2 ¦ D1 ¦ D0 ¦

'   +-------------------------------------------------+

'   ¦<------------------------------------->¦            ; These 8 bits are READ in 8g mode

'        ¦<------------------------------------->¦       ; These 8 bits are READ in 4g mode

'             ¦<------------------------------------->¦  ; These 8 bits are READ in 2g mode

'

'   D1(MODE[1]) - Mode SELECT     00 - Standby

'   D0(MODE[0]) - Mode SELECT     01 - Measurement

'                                 10 - Level Detection

'                                 11 - Pulse Detection

Main:

Address = MCTL: SendData = %01100101: GOSUB DataOut  'Set the Mode control register

'DATA ready status is NOT OUTPUT TO INT1 PIN

'3-wire SPI mode

'Self Test NOT enabled

'+/-2g sensitivity mode

'Measurement mode

Address = XOFFL: SendData = XCal& $FF: GOSUB DataOut 'Write X-Axis Calibration Value ; LOWBYTE

Address = XOFFH: SendData = XCal >> 8: GOSUB DataOut '                               ; HIGHBYTE

Address = YOFFL: SendData = YCal& $FF: GOSUB DataOut 'Write Y-Axis Calibration Value ; LOWBYTE

Address = YOFFH: SendData = YCal >> 8: GOSUB DataOut '                               ; HIGHBYTE

Address = ZOFFL: SendData = ZCal& $FF: GOSUB DataOut 'Write Z-Axis Calibration Value ; LOWBYTE

Address = ZOFFH: SendData = ZCal >> 8: GOSUB DataOut '                               ; HIGHBYTE

DEBUG CLS                                           'Clear Display

GOSUB TextField                                     'Draw TEXT field (stationary TEXT that does not get updated)

ReadDataLoop:

Address = XOUT8:GOSUB DataIn                        'Read in X-Axis Acceleration Value

XAccel = ReceiveData|($FF00*ReceiveData.BIT7)          'Sign extend the two's complement byte so

'negative numbers can be properly displayed

Address = YOUT8:GOSUB DataIn                        'Read in Y-Axis Acceleration Value

YAccel = ReceiveData|($FF00*ReceiveData.BIT7)          'Sign extend the two's complement byte so

'negative numbers can be properly displayed

Address = ZOUT8:GOSUB DataIn                        'Read in Z-Axis Acceleration Value

ZAccel = ReceiveData|($FF00*ReceiveData.BIT7)          'Sign extend the two's complement byte so

'negative numbers can be properly displayed

DEBUG CRSRXY,50,3, SDEC XAccel, "  ",               'Display the RAW X, Y, and Z Accelerometer values

CRSRXY,50,4, SDEC YAccel, "  ",

CRSRXY,50,5, SDEC ZAccel, "  "

GOTO ReadDataLoop

DataOut:

LOW CSPin                                              'Pull chip select pin low to start transmission

SHIFTOUT DATAPin, CLKPin, MSBFIRST, [(Address|%1000000)<<1] 'Select register Address

SHIFTOUT DATAPin, CLKPin, MSBFIRST, [SendData]         'Write value to Address

HIGH CSPin                                             'End transmission

RETURN

DataIn:

LOW CSPin                                              'Pull chip select pin low to start transmission

SHIFTOUT DATAPin, CLKPin, MSBFIRST, [Address<<1]       'Select register Address

SHIFTIN DATAPin, CLKPin, MSBPRE, [ReceiveData]         'Read value from Address

HIGH CSPin                                             'End transmission

RETURN

TextField:

DEBUG CRSRXY,40,0,"MMA7455 3-Axis Accelerometer BS2 DEMO #1",   'Display all stationary TEXT that does not get updated

CRSRXY,48,2,"8-Bit 2g Mode ; 64 LSB/g",

CRSRXY,48,3, "X=",

CRSRXY,48,4, "Y=",

CRSRXY,48,5, "Z="

RETURN

' -----[ Title ]-----------------------------------------------------------

'

'   File...... RXM-SG GPS Demo V1.0.bsp

'   Purpose... Demonstrate Decoding Data From RXM-SG GPS Module

'   Author.... Parallax, Inc.

'   E-mail.... support@parallax.com

'   Started... 08-16-2010

'   Updated...

'

'   {$STAMP BS2p}

'   {$PBASIC 2.5}

' -----[ Program Description ]---------------------------------------------

' 1) Connect the TX output of the Parallax RXM-SG GPS Module to P0 of the

'    BS2p module.

' 2) Provide common ground and a regulated 5V for the GPS Module.  If

'    using the Board of Education you can get 5V from VDD.

' 3) Download this program to the BS2p module and watch the debug screen.

' NOTE: Be sure the battery and antenna are installed before powering up

'       the GPS Module.

' This code is NOT compatible with the PMB-648 & PMB-688 by default.

' Unlike the PMB-648 & PMB-688, the RXM-SG GPS Module runs at 9600 bps by

' default.

' Reads NMEA data string from GPS receiver and parses data.  GPS string is

' buffered into scratchpad RAM with SPSTR modifier.  Once in SPRAM, data is

' parsed out based on its position.

'

' $GPRMC,POS_UTC,POS_STAT,LAT,LAT_D,LON,LON_D,SPD,HDG,DATE,MAG_VAR,MAG_REF,*CC

'

'  POS_UTC  - UTC of position. Hours, minutes and seconds. (hhmmss)

'  POS_STAT - Position status. (A = Data valid, V = Data invalid)

'  LAT      - Latitude (ddmm.ffff)

'  LAT_D    - Latitude direction. (N = North, S = South)

'  LON      - Longitude (dddmm.ffff)

'  LON_D    - Longitude direction (E = East, W = West)

'  SPD      - Speed over ground. (knots) (0.0 - 999.9)

'  HDG      - Heading/track made good (degrees True) (x.x)

'  DATE     - Date (ddmmyy)

'  MAG_VAR  - Magnetic variation (degrees) (x.x)

'  MAG_REF  - Magnetic variation (E = East, W = West)

'  *CC      - Checksum

' This program also reads the $GPGGA string and parses it for altitude.

' -----[ Revision History ]------------------------------------------------

' -----[ I/O Definitions ]-------------------------------------------------

GPSpin          PIN     0               ' GPS serial input

' -----[ Constants ]-------------------------------------------------------

T9600           CON     240             ' Baud rate for GPS (typical)

MoveTo          CON     2               ' DEBUG positioning command

ClrRt           CON     11              ' Clear line right of cursor

EST             CON     -5              ' Eastern Standard Time

CST             CON     -6              ' Central Standard Time

MST             CON     -7              ' Mountain Standard Time

PST             CON     -8              ' Pacific Standard Time

EDT             CON     -4              ' Eastern Daylight Time

CDT             CON     -5              ' Central Daylight Time

MDT             CON     -6              ' Mountain Daylight Time

PDT             CON     -7              ' Pacific Daylight Time

UTCfix          CON     PDT             ' For Rocklin, CA

Comma           CON     ","             ' Comma

DegSym          CON     176             ' Degrees symbol for report

MinSym          CON      39             ' Minutes symbol

SecSym          CON      34             ' Seconds symbol

' -----[ Variables ]-------------------------------------------------------

index           VAR     Byte            ' Index into GPS data in SPRAM

flags           VAR     Byte            ' Holds bit values

valid           VAR     Flags.BIT3      ' Is data valid?

numSats         VAR     Byte            ' Number of satellites

tmHrs           VAR     Byte            ' Time fields

tmMins          VAR     Byte

tmSecs          VAR     Byte

latDeg          VAR     Byte            ' Latitude

latMin          VAR     Byte

latSec          VAR     Word

latNS           VAR     flags.BIT0      ' 0 = N

longDeg         VAR     Byte            ' Longitude

longMin         VAR     Byte

longSec         VAR     Word

longEW          VAR     flags.BIT1      ' 0 = E

speed           VAR     Word            ' In tenths of mph***************

altitude        VAR     Word            ' In feet************************

day             VAR     Byte            ' Day of month, 1 - 31

month           VAR     flags.NIB1      ' Month, 1 - 12

year            VAR     Byte            ' Year, 00 - 99

char            VAR     Byte            ' Byte pulled from SPRAM

workVal         VAR     Word            ' For numeric conversions

eeAddr          VAR     workVal         ' Pointer to EE data

field           VAR     Nib             ' Field #

fldWidth        VAR     field           ' Width of field

' -----[ EEPROM Data ]-----------------------------------------------------

NotValid        DATA    "No", 0

IsValid         DATA    "Yes", 0

DaysInMon       DATA    31, 28, 31, 30, 31, 30, 31, 31, 30, 31, 30, 31

MonNames        DATA    "JAN",0,"FEB",0,"MAR",0,"APR",0,"MAY",0,"JUN",0

DATA    "JUL",0,"AUG",0,"SEP",0,"OCT",0,"NOV",0,"DEC",0

' -----[ Initialization ]--------------------------------------------------

Initialize:

PAUSE 250                             ' Let DEBUG open

DEBUG CLS                             ' Clear the screen

DEBUG "RXM-SG GPS Demo V1.0", CR

DEBUG "======================="

Draw_Ruler:

FOR index = 0 TO 65

IF (index = 0) THEN Print_Ones

IF (index // 10) > 0 THEN Print_Ones

DEBUG MoveTo, (7 + index), 3, DEC1 (index / 10)

Print_Ones:

DEBUG MoveTo, (7 + index), 4, DEC1 (index // 10)

Print_Ticks:

IF (index // 10) > 0 THEN Next_Digit

DEBUG MoveTo, (7 + index), 5, "|"

Next_Digit:

NEXT

Draw_Data_Labels:

DEBUG MoveTo, 0,  9, "Signal Valid: "

DEBUG MoveTo, 0, 11, "  Local Time: "

DEBUG MoveTo, 0, 12, "  Local Date: "

DEBUG MoveTo, 0, 14, "    Latitude: "

DEBUG MoveTo, 0, 15, "   Longitude: "

DEBUG MoveTo, 0, 16, "    Altitude: "

DEBUG MoveTo, 0, 17, "       Speed: "

' -----[ Program Code ]----------------------------------------------------

Main:

' Wait for $GPRMC string and store data in SPRAM

SERIN GPSpin, T9600, 3000, No_GPS_Data, [WAIT("GPRMC,"), SPSTR 65]

GOSUB Parse_GPS_Data                  ' Extract data from SPRAM

Show_GPMRC_String:

DEBUG MoveTo, 0, 6, "$GPRMC,"         ' Print header

index = 0                             ' Start at position UTC

Print_GPRMC_Char:                       ' Print the $GPRMC data string

GET index, char                       ' Get char from SPRAM

DEBUG char                            ' Display it

IF char = "*" THEN Print_Checksum     ' Look for checksum indicator

index = index + 1                     ' Point to next char

GOTO Print_GPRMC_Char

Print_Checksum:

GET (index + 1), char                 ' Get first checksum char

DEBUG char                            ' Display

GET (index + 2), char                 ' Get second checksum char

DEBUG char, ClrRt                     ' Display, clear to end of line

Show_Report:

DEBUG MoveTo, 14, 9                   ' Was the signal valid?

LOOKUP valid, [NotValid, IsValid], eeAddr' Get answer from EE

GOSUB Print_Z_String                  ' Print it

DEBUG " (", DEC numSats, " Satellites)"

DEBUG ClrRt                           ' Clear end of line

IF (valid = 0) THEN Signal_Not_Valid

Get_Altitude:

SERIN GPSpin, T9600, 2000, Signal_Is_Valid, [WAIT("GPGGA,"), SPSTR 75]

index = 45                            ' Altitude

GOSUB Mixed_To_Tenths                 ' Convert "xxx.x" To number

altitude = workVal

index = 38 : fldWidth = 2             ' Number of sats

GOSUB String_To_Value

numSats = workVal

Show_GPGGA_String:

DEBUG MoveTo, 0, 7, "$GPGGA,"         ' Print header

index = 0                             ' Start at position UTC

Print_GPGGA_Char:                       ' Print the $GPRMC data string

GET index, char                       ' Get char from SPRAM

DEBUG char                            ' Display it

IF char = "*" THEN Print_Chexum       ' Look for checksum indicator

index = index + 1                     ' Point to next char

GOTO Print_GPGGA_Char

Print_Chexum:

GET (index + 1), char                 ' Get first checksum char

DEBUG char                            ' Display

GET (index + 2), char                 ' Get second checksum char

DEBUG char, ClrRt                     ' Display, clear to end of line

Signal_Is_Valid:

DEBUG MoveTo, 14, 11, DEC2 tmHrs, ":", DEC2 tmMins, ":", DEC2 tmSecs

DEBUG MoveTo, 14, 12, DEC2 day, " "

eeAddr = (month - 1) * 4 + MonNames           ' get address of month name

GOSUB Print_Z_String                          ' print it

DEBUG " 20", DEC2 year

DEBUG MoveTo, 14, 14, " ", DEC2 latDeg, DegSym, " ", DEC2 latMin, MinSym, " "

DEBUG DEC2 (latSec / 10), ".", DEC1 (latSec // 10), SecSym, " "

DEBUG "N" + (latNS * 5)

DEBUG MoveTo, 14, 15, DEC3 longDeg, DegSym, " ", DEC2 longMin, MinSym, " "

DEBUG DEC2 (longSec / 10), ".", DEC1 (longSec // 10), SecSym, " "

DEBUG "E" + (longEW * 18)

DEBUG MoveTo, 14, 16, DEC (altitude / 10), ".", DEC1 (altitude //10), " Meters", ClrRt

DEBUG MoveTo, 14, 17, DEC (speed / 10), ".", DEC1 (speed // 10), " MPH "

GOTO Main

Signal_Not_Valid:

DEBUG MoveTo, 14, 11, "?", ClrRt      ' Clear all fields

DEBUG MoveTo, 14, 12, "?", ClrRt

DEBUG MoveTo, 14, 14, "?", ClrRt

DEBUG MoveTo, 14, 15, "?", ClrRt

DEBUG MoveTo, 14, 16, "?", ClrRt

DEBUG MoveTo, 14, 17, "?", ClrRt

GOTO Main

' -----[ Subroutines ]-----------------------------------------------------

No_GPS_Data:

DEBUG CLS, "Error: No GPS data detected"

PAUSE 2500

GOTO Initialize                       ' Try again

Parse_GPS_Data:

index = 0 : fldWidth = 2              ' UTC hours

GOSUB String_To_Value

tmHrs = workVal

index = 2 : fldWidth = 2              ' UTC minutes

GOSUB String_To_Value

tmMins = workVal

index = 4 : fldWidth = 2              ' UTC seconds

GOSUB String_To_Value

tmSecs = workVal

index = 13 : fldWidth = 2             ' Latitude degrees

GOSUB String_To_Value

latDeg = workVal

index = 15 : fldWidth = 2             ' Latitude minutes

GOSUB String_To_Value

latMin = workVal

index = 18 : fldWidth = 4             ' Latitude fractional minutes

GOSUB String_To_Value

latSec = workVal ** $0F5C             ' x 0.06 --> tenths of seconds

index = 25 : fldWidth = 3             ' Longitude degrees

GOSUB String_To_Value

longDeg = workVal

index = 28 : fldWidth = 2             ' Longitude minutes

GOSUB String_To_Value

longMin = workVal

index = 31 : fldWidth = 4             ' Longitude fractional minutes

GOSUB String_To_Value

longSec = workVal ** $0F5C            ' x 0.06 --> tenths of seconds

' Get non-numeric data

Get_Valid:

GET 11, char

valid = 1                             ' Assume valid

IF (char = "A") THEN Get_Lat_Dir      ' It is, so skip

valid = 0                             ' Set to 0 if not valid

Get_Lat_Dir:

latNS = 0                             ' Assume North

GET 19, char                          ' Check it

IF (char = "N") THEN Get_Long_Dir     ' Confirm

latNS = 1                             ' Set to 1 if South

Get_Long_Dir:

longEW = 0                            ' Assume East

GET 33, char                          ' Check it

IF (char = "E") THEN Get_Speed        ' Confirm

longEW = 1                            ' Set to 1 if West

' Get variable length data

Get_Speed:

index = 38

GOSUB Mixed_To_Tenths                 ' Convert "xxx.x" To number

' speed = workVal                     ' Speed in knots (tenths)

speed = workVal + (workVal ** $2699)  ' x 1.1507771555 for mph

' Get date

' -- past variable data, so we need to use field search

Get_Date:

field = 8                             ' Set field to find

GOSUB Move_To_Field                   ' Go get position

PUT 125, index                        ' Save date position

fldWidth = 2

GOSUB String_To_Value

day = workVal                         ' UTC day, 1 - 31

GET 125, index                        ' Get stored position

index = index + 2 : fldWidth = 2

GOSUB String_To_Value

month = workVal                       ' UTC month, 1 - 12

GET 125, index                        ' Get stored position

index = index + 4 : fldWidth = 2

GOSUB String_To_Value

year = workVal                        ' UTC year, 0 - 99

' Adjust date for local position

Correct_Local_Time_Date:

workVal = tmHrs + UTCfix              ' Add UTC offset

IF (workVal < 24) THEN Adjust_Time    ' Midnight crossed?

workVal = UTCfix                      ' Yes, so adjust date

BRANCH workVal.BIT15, [Location_Leads, Location_Lags]

Location_Leads:                         ' East of Greenwich

day = day + 1                         ' No, move to next day

eeAddr = DaysInMon * (month - 1)      ' Get days in month

READ eeAddr, char

IF (day <= char) THEN Adjust_Time     ' In same month?

month = month + 1                     ' No, move to next month

day = 1                               ' First day

IF (month < 13) THEN Adjust_Time      ' In same year?

month = 1                             ' No, set to January

year = year + 1 // 100                ' Add one to year

GOTO Adjust_Time

Location_Lags:                          ' West of Greenwich

day = day - 1                         ' Adjust day

IF (day > 0) THEN Adjust_Time         ' Same month?

month = month - 1

IF (month > 0) THEN Adjust_Time       ' Same year?

month = 1                             ' No, set to January

eeAddr = DaysInMon * (month - 1)

READ eeAddr, day                      ' Get new day

year = year + 99 // 100               ' Set to previous year

Adjust_Time:

tmHrs = tmHrs + (24 + UTCfix) // 24   ' Localize hours

RETURN

' *********************************************

' Convert string data (nnnn) to numeric value

' -- index    - location of first digit in data

' -- fldWidth - width of data field (1 to 5)

' -- workVal  - returns numeric value of field

' *********************************************

String_To_Value:

workVal = 0

IF (fldWidth = 0) OR (fldWidth > 5) THEN String_To_Value_Done

Get_Field_Digit:

GET index, char                       ' Get digit from field

workVal = workVal + (char - "0")      ' Convert, add into value

fldWidth = fldWidth - 1               ' Decrement field width

IF (fldWidth = 0) THEN String_To_Value_Done

workVal = workVal * 10                ' Shift result digits left

index = index + 1                     ' Point to next digit

GOTO Get_Field_Digit

String_To_Value_Done:

RETURN

' *****************************************************

' Convert string data (nnn.n) to numeric value (tenths)

' -- index   - location of first digit in data

' -- workVal - returns numeric value of field

' *****************************************************

Mixed_To_Tenths:

workVal = 0

Get_Mixed_Digit:

GET index, char                       ' Read digit from speed field

IF (char = ".") THEN Get_Mixed_Last   ' Skip decimal point

workVal = (workVal + (char - "0")) * 10' Add digit, move data left

index = index + 1                     ' Point to next digit

GOTO Get_Mixed_Digit

Get_Mixed_Last:

GET (index + 1), char

workVal = workVal + (char - "0")      ' Speed in knots

RETURN

' ************************************************************

' Find field location after variable-length data (i.e., speed)

' -- field - field number

' -- index - returns position of first digit in field

' ************************************************************

Move_To_Field:

index = 0

IF (field = 0) THEN Move_To_Field_Done' If zero, we're there

Get_Char:

GET index, char                       ' Get char from SPRAM

IF (char = Comma) THEN Found_Comma    ' Is it a comma?

index = index + 1                     ' No, move to next char

GOTO Get_Char

Found_Comma:

field = field - 1                     ' Was comma, dec field coutner

index = index + 1                     ' Point to next char

IF (field = 0) THEN Move_To_Field_Done' If field = 0, we're there

GOTO Get_Char

Move_To_Field_Done:

RETURN

' *********************************************

' Print Zero-terminated string stored in EEPROM

' -- eeAddr - starting character of string

' *********************************************

Print_Z_String:

READ eeAddr, char                     ' Get char from EE

IF (char = 0) THEN Print_Z_String_Done' If zero, we're done

DEBUG char                            ' Print the char

eeAddr = eeAddr + 1                   ' Point to the next one

GOTO Print_Z_String

Print_Z_String_Done:

RETURN

GPS tracking code of RXM-SG-GPS Module

' -----[ Title ]-----------------------------------------------------------

'

'   File...... RXM-SG GPS Demo V1.0.bsp

'   Purpose... Demonstrate Decoding Data From RXM-SG GPS Module

'   Author.... Parallax, Inc.

'   E-mail.... support@parallax.com

'   Started... 08-16-2010

'   Updated...

'

'   {$STAMP BS2p}

'   {$PBASIC 2.5}

' -----[ Program Description ]---------------------------------------------

' 1) Connect the TX output of the Parallax RXM-SG GPS Module to P0 of the

'    BS2p module.

' 2) Provide common ground and a regulated 5V for the GPS Module.  If

'    using the Board of Education you can get 5V from VDD.

' 3) Download this program to the BS2p module and watch the debug screen.

' NOTE: Be sure the battery and antenna are installed before powering up

'       the GPS Module.

' This code is NOT compatible with the PMB-648 & PMB-688 by default.

' Unlike the PMB-648 & PMB-688, the RXM-SG GPS Module runs at 9600 bps by

' default.

' Reads NMEA data string from GPS receiver and parses data.  GPS string is

' buffered into scratchpad RAM with SPSTR modifier.  Once in SPRAM, data is

' parsed out based on its position.

'

' $GPRMC,POS_UTC,POS_STAT,LAT,LAT_D,LON,LON_D,SPD,HDG,DATE,MAG_VAR,MAG_REF,*CC

'

'  POS_UTC  - UTC of position. Hours, minutes and seconds. (hhmmss)

'  POS_STAT - Position status. (A = Data valid, V = Data invalid)

'  LAT      - Latitude (ddmm.ffff)

'  LAT_D    - Latitude direction. (N = North, S = South)

'  LON      - Longitude (dddmm.ffff)

'  LON_D    - Longitude direction (E = East, W = West)

'  SPD      - Speed over ground. (knots) (0.0 - 999.9)

'  HDG      - Heading/track made good (degrees True) (x.x)

'  DATE     - Date (ddmmyy)

'  MAG_VAR  - Magnetic variation (degrees) (x.x)

'  MAG_REF  - Magnetic variation (E = East, W = West)

'  *CC      - Checksum

' This program also reads the $GPGGA string and parses it for altitude.

' -----[ Revision History ]------------------------------------------------

' -----[ I/O Definitions ]-------------------------------------------------

GPSpin          PIN     0               ' GPS serial input

' -----[ Constants ]-------------------------------------------------------

T9600           CON     240             ' Baud rate for GPS (typical)

MoveTo          CON     2               ' DEBUG positioning command

ClrRt           CON     11              ' Clear line right of cursor

EST             CON     -5              ' Eastern Standard Time

CST             CON     -6              ' Central Standard Time

MST             CON     -7              ' Mountain Standard Time

PST             CON     -8              ' Pacific Standard Time

EDT             CON     -4              ' Eastern Daylight Time

CDT             CON     -5              ' Central Daylight Time

MDT             CON     -6              ' Mountain Daylight Time

PDT             CON     -7              ' Pacific Daylight Time

UTCfix          CON     PDT             ' For Rocklin, CA

Comma           CON     ","             ' Comma

DegSym          CON     176             ' Degrees symbol for report

MinSym          CON      39             ' Minutes symbol

SecSym          CON      34             ' Seconds symbol

' -----[ Variables ]-------------------------------------------------------

index           VAR     Byte            ' Index into GPS data in SPRAM

flags           VAR     Byte            ' Holds bit values

valid           VAR     Flags.BIT3      ' Is data valid?

numSats         VAR     Byte            ' Number of satellites

tmHrs           VAR     Byte            ' Time fields

tmMins          VAR     Byte

tmSecs          VAR     Byte

latDeg          VAR     Byte            ' Latitude

latMin          VAR     Byte

latSec          VAR     Word

latNS           VAR     flags.BIT0      ' 0 = N

longDeg         VAR     Byte            ' Longitude

longMin         VAR     Byte

longSec         VAR     Word

longEW          VAR     flags.BIT1      ' 0 = E

speed           VAR     Word            ' In tenths of mph***************

altitude        VAR     Word            ' In feet************************

day             VAR     Byte            ' Day of month, 1 - 31

month           VAR     flags.NIB1      ' Month, 1 - 12

year            VAR     Byte            ' Year, 00 - 99

char            VAR     Byte            ' Byte pulled from SPRAM

workVal         VAR     Word            ' For numeric conversions

eeAddr          VAR     workVal         ' Pointer to EE data

field           VAR     Nib             ' Field #

fldWidth        VAR     field           ' Width of field

' -----[ EEPROM Data ]-----------------------------------------------------

NotValid        DATA    "No", 0

IsValid         DATA    "Yes", 0

DaysInMon       DATA    31, 28, 31, 30, 31, 30, 31, 31, 30, 31, 30, 31

MonNames        DATA    "JAN",0,"FEB",0,"MAR",0,"APR",0,"MAY",0,"JUN",0

DATA    "JUL",0,"AUG",0,"SEP",0,"OCT",0,"NOV",0,"DEC",0

' -----[ Initialization ]--------------------------------------------------

Initialize:

PAUSE 250                             ' Let DEBUG open

DEBUG CLS                             ' Clear the screen

DEBUG "RXM-SG GPS Demo V1.0", CR

DEBUG "======================="

Draw_Ruler:

FOR index = 0 TO 65

IF (index = 0) THEN Print_Ones

IF (index // 10) > 0 THEN Print_Ones

DEBUG MoveTo, (7 + index), 3, DEC1 (index / 10)

Print_Ones:

DEBUG MoveTo, (7 + index), 4, DEC1 (index // 10)

Print_Ticks:

IF (index // 10) > 0 THEN Next_Digit

DEBUG MoveTo, (7 + index), 5, "|"

Next_Digit:

NEXT

Draw_Data_Labels:

DEBUG MoveTo, 0,  9, "Signal Valid: "

DEBUG MoveTo, 0, 11, "  Local Time: "

DEBUG MoveTo, 0, 12, "  Local Date: "

DEBUG MoveTo, 0, 14, "    Latitude: "

DEBUG MoveTo, 0, 15, "   Longitude: "

DEBUG MoveTo, 0, 16, "    Altitude: "

DEBUG MoveTo, 0, 17, "       Speed: "

' -----[ Program Code ]----------------------------------------------------

Main:

' Wait for $GPRMC string and store data in SPRAM

SERIN GPSpin, T9600, 3000, No_GPS_Data, [WAIT("GPRMC,"), SPSTR 65]

GOSUB Parse_GPS_Data                  ' Extract data from SPRAM

Show_GPMRC_String:

DEBUG MoveTo, 0, 6, "$GPRMC,"         ' Print header

index = 0                             ' Start at position UTC

Print_GPRMC_Char:                       ' Print the $GPRMC data string

GET index, char                       ' Get char from SPRAM

DEBUG char                            ' Display it

IF char = "*" THEN Print_Checksum     ' Look for checksum indicator

index = index + 1                     ' Point to next char

GOTO Print_GPRMC_Char

Print_Checksum:

GET (index + 1), char                 ' Get first checksum char

DEBUG char                            ' Display

GET (index + 2), char                 ' Get second checksum char

DEBUG char, ClrRt                     ' Display, clear to end of line

Show_Report:

DEBUG MoveTo, 14, 9                   ' Was the signal valid?

LOOKUP valid, [NotValid, IsValid], eeAddr' Get answer from EE

GOSUB Print_Z_String                  ' Print it

DEBUG " (", DEC numSats, " Satellites)"

DEBUG ClrRt                           ' Clear end of line

IF (valid = 0) THEN Signal_Not_Valid

Get_Altitude:

SERIN GPSpin, T9600, 2000, Signal_Is_Valid, [WAIT("GPGGA,"), SPSTR 75]

index = 45                            ' Altitude

GOSUB Mixed_To_Tenths                 ' Convert "xxx.x" To number

altitude = workVal

index = 38 : fldWidth = 2             ' Number of sats

GOSUB String_To_Value

numSats = workVal

Show_GPGGA_String:

DEBUG MoveTo, 0, 7, "$GPGGA,"         ' Print header

index = 0                             ' Start at position UTC

Print_GPGGA_Char:                       ' Print the $GPRMC data string

GET index, char                       ' Get char from SPRAM

DEBUG char                            ' Display it

IF char = "*" THEN Print_Chexum       ' Look for checksum indicator

index = index + 1                     ' Point to next char

GOTO Print_GPGGA_Char

Print_Chexum:

GET (index + 1), char                 ' Get first checksum char

DEBUG char                            ' Display

GET (index + 2), char                 ' Get second checksum char

DEBUG char, ClrRt                     ' Display, clear to end of line

Signal_Is_Valid:

DEBUG MoveTo, 14, 11, DEC2 tmHrs, ":", DEC2 tmMins, ":", DEC2 tmSecs

DEBUG MoveTo, 14, 12, DEC2 day, " "

eeAddr = (month - 1) * 4 + MonNames           ' get address of month name

GOSUB Print_Z_String                          ' print it

DEBUG " 20", DEC2 year

DEBUG MoveTo, 14, 14, " ", DEC2 latDeg, DegSym, " ", DEC2 latMin, MinSym, " "

DEBUG DEC2 (latSec / 10), ".", DEC1 (latSec // 10), SecSym, " "

DEBUG "N" + (latNS * 5)

DEBUG MoveTo, 14, 15, DEC3 longDeg, DegSym, " ", DEC2 longMin, MinSym, " "

DEBUG DEC2 (longSec / 10), ".", DEC1 (longSec // 10), SecSym, " "

DEBUG "E" + (longEW * 18)

DEBUG MoveTo, 14, 16, DEC (altitude / 10), ".", DEC1 (altitude //10), " Meters", ClrRt

DEBUG MoveTo, 14, 17, DEC (speed / 10), ".", DEC1 (speed // 10), " MPH "

GOTO Main

Signal_Not_Valid:

DEBUG MoveTo, 14, 11, "?", ClrRt      ' Clear all fields

DEBUG MoveTo, 14, 12, "?", ClrRt

DEBUG MoveTo, 14, 14, "?", ClrRt

DEBUG MoveTo, 14, 15, "?", ClrRt

DEBUG MoveTo, 14, 16, "?", ClrRt

DEBUG MoveTo, 14, 17, "?", ClrRt

GOTO Main

' -----[ Subroutines ]-----------------------------------------------------

No_GPS_Data:

DEBUG CLS, "Error: No GPS data detected"

PAUSE 2500

GOTO Initialize                       ' Try again

Parse_GPS_Data:

index = 0 : fldWidth = 2              ' UTC hours

GOSUB String_To_Value

tmHrs = workVal

index = 2 : fldWidth = 2              ' UTC minutes

GOSUB String_To_Value

tmMins = workVal

index = 4 : fldWidth = 2              ' UTC seconds

GOSUB String_To_Value

tmSecs = workVal

index = 13 : fldWidth = 2             ' Latitude degrees

GOSUB String_To_Value

latDeg = workVal

index = 15 : fldWidth = 2             ' Latitude minutes

GOSUB String_To_Value

latMin = workVal

index = 18 : fldWidth = 4             ' Latitude fractional minutes

GOSUB String_To_Value

latSec = workVal ** $0F5C             ' x 0.06 --> tenths of seconds

index = 25 : fldWidth = 3             ' Longitude degrees

GOSUB String_To_Value

longDeg = workVal

index = 28 : fldWidth = 2             ' Longitude minutes

GOSUB String_To_Value

longMin = workVal

index = 31 : fldWidth = 4             ' Longitude fractional minutes

GOSUB String_To_Value

longSec = workVal ** $0F5C            ' x 0.06 --> tenths of seconds

' Get non-numeric data

Get_Valid:

GET 11, char

valid = 1                             ' Assume valid

IF (char = "A") THEN Get_Lat_Dir      ' It is, so skip

valid = 0                             ' Set to 0 if not valid

Get_Lat_Dir:

latNS = 0                             ' Assume North

GET 19, char                          ' Check it

IF (char = "N") THEN Get_Long_Dir     ' Confirm

latNS = 1                             ' Set to 1 if South

Get_Long_Dir:

longEW = 0                            ' Assume East

GET 33, char                          ' Check it

IF (char = "E") THEN Get_Speed        ' Confirm

longEW = 1                            ' Set to 1 if West

' Get variable length data

Get_Speed:

index = 38

GOSUB Mixed_To_Tenths                 ' Convert "xxx.x" To number

' speed = workVal                     ' Speed in knots (tenths)

speed = workVal + (workVal ** $2699)  ' x 1.1507771555 for mph

' Get date

' -- past variable data, so we need to use field search

Get_Date:

field = 8                             ' Set field to find

GOSUB Move_To_Field                   ' Go get position

PUT 125, index                        ' Save date position

fldWidth = 2

GOSUB String_To_Value

day = workVal                         ' UTC day, 1 - 31

GET 125, index                        ' Get stored position

index = index + 2 : fldWidth = 2

GOSUB String_To_Value

month = workVal                       ' UTC month, 1 - 12

GET 125, index                        ' Get stored position

index = index + 4 : fldWidth = 2

GOSUB String_To_Value

year = workVal                        ' UTC year, 0 - 99

' Adjust date for local position

Correct_Local_Time_Date:

workVal = tmHrs + UTCfix              ' Add UTC offset

IF (workVal < 24) THEN Adjust_Time    ' Midnight crossed?

workVal = UTCfix                      ' Yes, so adjust date

BRANCH workVal.BIT15, [Location_Leads, Location_Lags]

Location_Leads:                         ' East of Greenwich

day = day + 1                         ' No, move to next day

eeAddr = DaysInMon * (month - 1)      ' Get days in month

READ eeAddr, char

IF (day <= char) THEN Adjust_Time     ' In same month?

month = month + 1                     ' No, move to next month

day = 1                               ' First day

IF (month < 13) THEN Adjust_Time      ' In same year?

month = 1                             ' No, set to January

year = year + 1 // 100                ' Add one to year

GOTO Adjust_Time

Location_Lags:                          ' West of Greenwich

day = day - 1                         ' Adjust day

IF (day > 0) THEN Adjust_Time         ' Same month?

month = month - 1

IF (month > 0) THEN Adjust_Time       ' Same year?

month = 1                             ' No, set to January

eeAddr = DaysInMon * (month - 1)

READ eeAddr, day                      ' Get new day

year = year + 99 // 100               ' Set to previous year

Adjust_Time:

tmHrs = tmHrs + (24 + UTCfix) // 24   ' Localize hours

RETURN

' *********************************************

' Convert string data (nnnn) to numeric value

' -- index    - location of first digit in data

' -- fldWidth - width of data field (1 to 5)

' -- workVal  - returns numeric value of field

' *********************************************

String_To_Value:

workVal = 0

IF (fldWidth = 0) OR (fldWidth > 5) THEN String_To_Value_Done

Get_Field_Digit:

GET index, char                       ' Get digit from field

workVal = workVal + (char - "0")      ' Convert, add into value

fldWidth = fldWidth - 1               ' Decrement field width

IF (fldWidth = 0) THEN String_To_Value_Done

workVal = workVal * 10                ' Shift result digits left

index = index + 1                     ' Point to next digit

GOTO Get_Field_Digit

String_To_Value_Done:

RETURN

' *****************************************************

' Convert string data (nnn.n) to numeric value (tenths)

' -- index   - location of first digit in data

' -- workVal - returns numeric value of field

' *****************************************************

Mixed_To_Tenths:

workVal = 0

Get_Mixed_Digit:

GET index, char                       ' Read digit from speed field

IF (char = ".") THEN Get_Mixed_Last   ' Skip decimal point

workVal = (workVal + (char - "0")) * 10' Add digit, move data left

index = index + 1                     ' Point to next digit

GOTO Get_Mixed_Digit

Get_Mixed_Last:

GET (index + 1), char

workVal = workVal + (char - "0")      ' Speed in knots

RETURN

' ************************************************************

' Find field location after variable-length data (i.e., speed)

' -- field - field number

' -- index - returns position of first digit in field

' ************************************************************

Move_To_Field:

index = 0

IF (field = 0) THEN Move_To_Field_Done' If zero, we're there

Get_Char:

GET index, char                       ' Get char from SPRAM

IF (char = Comma) THEN Found_Comma    ' Is it a comma?

index = index + 1                     ' No, move to next char

GOTO Get_Char

Found_Comma:

field = field - 1                     ' Was comma, dec field coutner

index = index + 1                     ' Point to next char

IF (field = 0) THEN Move_To_Field_Done' If field = 0, we're there

GOTO Get_Char

Move_To_Field_Done:

RETURN

' *********************************************

' Print Zero-terminated string stored in EEPROM

' -- eeAddr - starting character of string

' *********************************************

Print_Z_String:

READ eeAddr, char                     ' Get char from EE

IF (char = 0) THEN Print_Z_String_Done' If zero, we're done

DEBUG char                            ' Print the char

eeAddr = eeAddr + 1                   ' Point to the next one

GOTO Print_Z_String

Print_Z_String_Done:

RETURN